WORKPAPER DISPOSITION FOR

Lighting Occupancy Sensor Controls

California Public Utilities Commission, Energy Division

September 6, 2013 (revised)

Revision Notes, May 3, 2013: The embedded workbook containing savings values for all measures was revised to correct errors where incorrect annual hours of use were being used to calculate savings.

Revision Notes, May 31, 2013: The embedded workbook containing savings values for all measures was revised to correct the following errors:

- Refrigerated Warehouse values were not available in previous versions. Those values are not included in the workbook.

- The weighted commercial values (Com) were incorrect. The previous values were those that appeared in the DEER 2005 database for measure (D03-03 Occupancy Sensor Small Area <200 Square Feet). The Com value for this measure consisted of weighted results for Primary School, Hospital, Hotel, Motel, Nursing Home and Small Office only. The Com results in the revised embedded workbook include are weighted to consider results for all DEER building types. These include the non-simulated results as well as the results from the DEER simulations for D03-003 (small area sensors) and D03-004 (large area sensors). Refer to Table 3 for a listing of which building types are supported by DEER simulation results and which are supported by the alternative methods incorporated into the embedded workbook.

NOTE: Savings values for individual building types have not changed from the previous versions of this disposition. Only the weighted results for the “Com” building type have changed. Electricity (kWh) savings values for the “Com” building type in the revised workbook are between 14% and 18% less, depending on IOU, compared to the previous workbook; demand savings are 20%-24% less, and; negative gas savings are reduced 9%-11%. Refer to Table A below for a comparison of original to revised savings for the “Com” weighted building type.

Table A - Comparison of Original to Revised Disposition

Revision Notes, September 6, 2013: The embedded workbook containing savings values for all measures was revised in consideration of recent input from utilities.

Revision to direct impact demand savings:

The disposition calculates the direct impact demand savings for buildings not covered in DEER as:

kW/kWcontrolled = CDF * Reduction Factor * Weekday Open Fraction

where:

CDF is the DEER coincident demand factor

Reduction Factor is the DEER total hours of use reduction. For DEER building types not covered by the DEER measure, this reduction factor is based on Energy Division staff’s best estimate of a reduction factor for a covered building type that would be equivalent to a non-covered building type.

Weekday Open Fraction is the DEER allocation of total usage reduction assigned to the weekday open period. The DEER methodology allocates the total usage reduction into for periods: Weekday Open; Weekday Closed; Weekend Open; Weekend Closed. Values for non-covered building types align with the same equivalent mappings of covered building types for determination of the Reduction Factor described above.

SCE staff recommended revising the demand savings calculation to not include the Weekday Open Fraction. Energy Division staff had a web meeting with utility staff and agreed to investigate further. Staff believes that removing the Weekday Open Fraction would like result in overestimation of savings for the non-covered DEER building types. Savings for the covered DEER building types were developed for the 2005 version of DEER through a modeling approach. This approach is fully described in the 2005 DEER documentation. The DEER approach assumes that, for all building types, not all areas of the building would benefit from the installation of occupancy sensors. For example, it is assumed for office buildings that no savings from occupancy sensors would occur in the lobby. In schools, no savings would occur in maintenance shops, kitchens or corridors.

In response to this concern, the ex ante review team developed a spreadsheet that calculates the overall building demand impact of occupancy sensors, for buildings not currently in DEER, that is similar to the approach used for DEER savings. The general approach is to weight together, across a single building type, the peak schedules and connected lighting loads, to determine a whole building peak direct impact peak reduction. The analysis shows that, for some building types, applying the “weekday open fraction” significantly underestimates peak demand impacts. However, there are some building types for which the new method results in lower demand impacts than were previously published in the disposition. The full development of the peak demand savings is included in the workbook “DEER2005+2008-EFLH+CDF+OccSensor_September2013-v2a.xlsx”.

Ex Ante Savings Table Format

Utilities requested that the table of energy savings ex ante values included in the disposition be revised to match the format of ex ante values included with READI. The ex ante review team has updated the savings table to match the READI format. The revised table is included in the ex ante workbook, “Occ_Sens_Savings_Revised_6September2013.xlsx”, embedded at the end of this disposition.

Refer to Table 1 for a list of currently submitted IOU workpapers that cover lighting occupancy sensor controls.

Table 1 – Lighting Control Workpapers

| Workpaper ID | Workpaper Title | Date |
| --- | --- | --- |
| SDG&E |  |  |
| WPSCNRLG0016 | Occupancy Sensors: Wall or Ceiling Mounted | 06/5/2012 |
| SCE |  |  |
| SCE13LG076 | Integrated Linear Fluorescent Occupancy Sensor | 05/24/2012 |
| SCE13LG025 | Occupancy Sensors, Wall or Ceiling Mounted | 04/10/2012 |
| SCE13LG107 | Battery- and Self-Powered Wireless Occupancy Sensors – Ceiling or Wall Mounted | 04/12/2012 |
| SCE13LG020 | Wall Mounted Occupancy Sensors – Multifamily and Hospitality | 04/25/2012 |
| PG&E |  |  |
| PGECOLTG120 | Wall or Ceiling Occupancy Sensors | 08/28/2012 |
| PGECOLTG134 | Fixture Integrated Occupancy Sensors – Fixtures ≥ 150 W | 08/28/2012 |
| PGECOLTG135 | Fixture Integrated Occupancy Sensors – Fixtures <150 W | 08/28/2012 |
| PGECOLTG130 | Occupancy Sensors: Multifamily (Wall-box) | 08/28/2012 |
| PGE3PLTG187 | Fixture Integrated Occupancy Sensors – Fixtures <150 W, Direct Install | 08/30/2012 |
| PGE3PLTG186 | Fixture Integrated Occupancy Sensors – Fixtures ≥ 150 W, Direct Install | 08/30/2012 |

Workpaper Disposition:

Disposition Summary

Energy Division staff recommends the following revisions:

- Savings values for the DEER measures D03-003 or D03-004 shall be used when available. Values are not available for all building types. Savings values for other building types shall be the values specified in item 2, below.

- The attached workbook contains the savings values for all building types not covered in READI by DEER measures D03-003 or D03-004.

Control Savings Estimation Summary

Where not listed in DEER, occupancy sensor savings shall be calculated according to staff direction provided to PG&E in review of the Modified Lighting Calculator . This review document has been embedded in this disposition at the end of the document. Table 3 is taken from the Modified Lighting Calculator Disposition and lists the DEER space types that are covered in the DEER occupancy sensor measures along with their lighting use reduction factors. Table 3  also includes acceptable reduction factors for all other space types not covered by DEER.

Demand direct impact reductions have been determined by multiplying together the total usage reduction, the fraction of that reduction that occurs during the peak demand period and the DEER coincident demand factor for the DEER building type.

Occupancy sensor measures do have HVAC interactive effects, but DEER does not include interactive effects for occupancy sensor measures at this time. At this time, occupancy sensor measures shall use the DEER HVAC interactive effects for lighting power reductions.

NTG review

The NTG ratios of the workpapers of Table 1 present some inconsistencies. Energy Division staff wish to remind that the NTG ratios to be used by all IOUs are the ratios available in the READI database. They are presented in Table 2. In the case of a “Direct Install” measure ED recommends to use the “Com-Default>2yrs” ratio and not the “NonRes-sAll-mLtgCtrl-htr” which is for a hard to reach market.

Table 2 - NTG Ratios from READI

| NTG ID | Measure Type | NTG | Measure delivery mechanism |
| --- | --- | --- | --- |
| NonRes-sAll-mLtgCtrl | Lighting controls | 0.6 | PreRebDown |
| NonRes-sAll-mLtgCtrl-htr | Lighting controls | 0.89 | DirInstall |
| NonRes-sAll-mOccSens | Occupancy Sensors | 0.6 | PreRebDown |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | 0.6 | All |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | 0.55 | All |

Table 3 - Lighting Use Reduction Factors for Occupancy Sensor

| DEER Building Type | Activity Area Type | DEER Occupancy Sensor? | Non-DEER Reference Building Type | Non-DEER Reference Activity Area Type | Reduction |
| --- | --- | --- | --- | --- | --- |
| Assembly | Auditorium | NO | Education - Secondary School | Classroom/Lecture | 0.20 |
| Assembly | Office | NO | Education - Community College | Office (General) | 0.16 |
| Assembly | All Other | NO | Education - Community College | Office (General) | 0.16 |
| Education - Primary School | Classroom/Lecture | YES |  |  | 0.20 |
| Education - Primary School | Dining Area | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Primary School | Exercising Centers and Gymnasium | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Primary School | Kitchen and Food Preparation | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Primary School | All Other | NO | Education - Community College | Office (General) | 0.16 |
| Education - Secondary School | Classroom/Lecture | YES |  |  | 0.20 |
| Education - Secondary School | Computer Room (Instructional/PC Lab) | YES |  |  | 0.20 |
| Education - Secondary School | Dining Area | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Secondary School | Exercising Centers and Gymnasium | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Secondary School | Kitchen and Food Preparation | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Secondary School | Office (General) | NO | Education - Community College | Office (General) | 0.16 |
| Education - Secondary School | All Other | NO | Education - Community College | Office (General) | 0.16 |
| Education - Community College | Classroom/Lecture | YES |  |  | 0.20 |
| Education - Community College | Computer Room (Instructional/PC Lab) | YES |  |  | 0.20 |
| Education - Community College | Dining Area | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Community College | Kitchen and Food Preparation | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education - Community College | Office (General) | YES |  |  | 0.16 |
| Education - Community College | All Other | NO | Education - Community College | Office (General) | 0.16 |
| Education – University | Classroom/Lecture | YES |  |  | 0.20 |
| Education – University | Comm/Ind Work (General Low Bay) | NO | Education - Community College | Office (General) | 0.16 |
| Education – University | Computer Room (Instructional/PC Lab) | YES |  |  | 0.20 |
| Education – University | Corridor (Dormitory) | NO | Health/Medical - Nursing Home | Corridor | 0.16 |
| Education – University | Dining Area | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education – University | Hotel/Motel Guest Room (Dormitory) | NO | Lodging – Hotel | Hotel/Motel Guest Room (incl. toilets) | 0.10 |
| Education – University | Kitchen and Food Preparation | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Education – University | Office (General) | YES |  |  | 0.16 |
| Education – University | All Other | NO | Education – University | Office (General) | 0.16 |
| Education - Relocatable Classroom | Classroom/Lecture | YES |  |  | 0.20 |
| Grocery | Comm/Ind Work (Loading Dock) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Grocery | Office (General) | NO | Office – Large | Office (Executive/Private) | 0.30 |
| Grocery | Refrigerated (Food Preparation) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Grocery | Refrigerated (Walk-in Cooler) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Grocery | Refrigerated (Walk-in Freezer) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Grocery | Retail Sales Grocery | NO | - none - | - none - | 0.00 |
| Grocery | All Other | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Health/Medical – Hospital | Dining Area | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Health/Medical – Hospital | Kitchen and Food Preparation | NO | Health/Medical – Hospital | Office (General) | 0.16 |
| Health/Medical – Hospital | Laboratory Medical | NO | Health/Medical – Hospital | Office (General) | 0.16 |
| Health/Medical – Hospital | Medical and Clinical Care (12 Hr) | NO | Health/Medical – Hospital | Office (General) | 0.16 |
| Health/Medical – Hospital | Medical and Clinical Care (24 Hr) | NO | Health/Medical – Hospital | Office (General) | 0.16 |
| Health/Medical – Hospital | Patient Rooms | NO | - none - | - none - | 0.00 |
| Health/Medical – Hospital | Office (General) | YES |  |  | 0.16 |
| Health/Medical – Hospital | All Other | NO | Health/Medical – Hospital | Office (General) | 0.16 |
| Health/Medical - Nursing Home | Corridor | YES |  |  | 0.16 |
| Health/Medical - Nursing Home | Dining Area | YES |  |  | 0.16 |
| Health/Medical - Nursing Home | Patient Room | NO | - none - | - none - | 0.00 |
| Health/Medical - Nursing Home | Kitchen and Food Preparation | NO | Health/Medical - Nursing Home | Dining Area | 0.16 |
| Health/Medical - Nursing Home | Office (General) | YES |  |  | 0.16 |
| Health/Medical - Nursing Home | All Other | NO | Health/Medical - Nursing Home | Office (General) | 0.16 |
| Lodging – Hotel | Bar Cocktail Lounge | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Hotel | Corridor | YES |  |  | 0.16 |
| Lodging – Hotel | Dining Area | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Hotel | Hotel/Motel Guest Room (incl. toilets) | YES |  |  | 0.10 |
| Lodging – Hotel | Kitchen and Food Preparation | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Hotel | Laundry | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Hotel | Lobby (Hotel) | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Hotel | Office (General) | YES |  |  | 0.16 |
| Lodging – Hotel | All Other | NO | Lodging – Hotel | Office (General) | 0.16 |
| Lodging – Motel | Corridor | YES |  |  | 0.16 |
| Lodging – Motel | Hotel/Motel Guest Room (incl. toilets) | YES |  |  | 0.10 |
| Lodging – Motel | Laundry | NO | Lodging – Motel | Office (General) | 0.16 |
| Lodging – Motel | Office (General) | YES |  |  | 0.16 |
| Lodging – Motel | All Other | NO | Lodging – Motel | Office (General) | 0.16 |
| Manufacturing - Bio/Tech | Comm/Ind Work (High Tech Bio Tech Lab) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Bio/Tech | Computer Room (Mainframe/Server) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Bio/Tech | Conference Room | NO | Office – Large | Conference Room | 0.16 |
| Manufacturing - Bio/Tech | Corridor | NO | Office – Large | Corridor | 0.10 |
| Manufacturing - Bio/Tech | Dining Area | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Bio/Tech | Kitchen and Food Preparation | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Bio/Tech | Office (General) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Bio/Tech | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Light Industrial | Comm/Ind Work (General High Bay) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Manufacturing - Light Industrial | Storage (Unconditioned) | NO | Office – Small | Mechanical/Electrical Room | 0.16 |
| Manufacturing - Light Industrial | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Office – Large | Conference Room | YES |  |  | 0.16 |
| Office – Large | Copy Room (photocopying equipment) | YES |  |  | 0.10 |
| Office – Large | Corridor | YES |  |  | 0.10 |
| Office – Large | Lobby(Office Reception/Waiting) | NO | Office – Large | Corridor | 0.10 |
| Office – Large | Mechanical/Electrical Room | YES |  |  | 0.16 |
| Office – Large | Office (Executive/Private) | YES |  |  | 0.30 |
| Office – Large | Office (Open Plan) | YES |  |  | 0.15 |
| Office – Large | Restrooms | YES |  |  | 0.26 |
| Office – Large | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Office – Small | Conference Room | YES |  |  | 0.16 |
| Office – Small | Copy Room (photocopying equipment) | YES |  |  | 0.10 |
| Office – Small | Corridor | YES |  |  | 0.10 |
| Office – Small | Lobby (Office Reception/Waiting) | NO | Office – Small | Corridor | 0.10 |
| Office – Small | Mechanical/Electrical Room | YES |  |  | 0.16 |
| Office – Small | Office (Executive/Private) | YES |  |  | 0.30 |
| Office – Small | Restrooms | YES |  |  | 0.10 |
| Office – Small | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Sit-Down | Dining Area | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Sit-Down | Kitchen and Food Preparation | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Sit-Down | Lobby (Main Entry and Assembly) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Sit-Down | Restrooms | NO | Office – Large | Restrooms | 0.26 |
| Restaurant - Sit-Down | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Fast-Food | Dining Area | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Fast-Food | Kitchen and Food Preparation | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Fast-Food | Lobby (Main Entry and Assembly) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Fast-Food | Restrooms | NO | Office – Large | Office (Open Plan) | 0.15 |
| Restaurant - Fast-Food | All Other | NO | Office – Large | Restrooms | 0.26 |
| Retail - 3-Story Large | Office (General) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail - 3-Story Large | Retail Sales and Wholesale Showroom | NO | - none - | - none - | 0.00 |
| Retail - 3-Story Large | Storage (Conditioned) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Retail - 3-Story Large | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail - Single-Story Large | Auto Repair Workshop | NO | - none - | - none - | 0.00 |
| Retail - Single-Story Large | Kitchen and Food Preparation | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail - Single-Story Large | Office (General) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail - Single-Story Large | Retail Sales and Wholesale Showroom | NO | - none - | - none - | 0.00 |
| Retail - Single-Story Large | Storage (Conditioned) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail - Single-Story Large | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail – Small | Retail Sales and Wholesale Showroom | NO | - none - | - none - | 0.00 |
| Retail – Small | Storage (Conditioned) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Retail – Small | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| Storage – Conditioned | Storage (Conditioned) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Storage – Unconditioned | Storage (Unconditioned) | NO | Office – Large | Mechanical/Electrical Room | 0.16 |
| Storage - Refrigerated Warehouse | Comm/Ind Work (Loading Dock) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Storage - Refrigerated Warehouse | Office (Executive/Private) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Storage - Refrigerated Warehouse | Refrigerated (Cooled Storage) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Storage - Refrigerated Warehouse | Refrigerated (Food Preparation) | NO | Office – Large | Office (Open Plan) | 0.15 |
| Storage - Refrigerated Warehouse | All Other | NO | Office – Large | Office (Open Plan) | 0.15 |
| All Non-DEER | All Non-DEER | NO | - none - | - none - | 0.15 |

Reference(s):

1 - Control Savings Calculator (Revised September 6, 2013)

2 – Occupancy Savings Demand Impact Calculations

3 - Modified Lighting Calculator Preliminary Disposition

MODIFIED_LIGHTING_CALCULATOR_V2 7.xlsx submitted by PG&E in September 2012

PRELIMINARY REVIEW DISPOSITION FOR “Modified Lighting Calculator”, CPUC, November 2012