Work Paper SCE13AP003

Revision 1

Southern California Edison Company

Energy Star Clothes Washer

# At-a-Glance Summary

| Applicable Measure Codes: | AP-98665, AP-61333, AP-12634 |
| --- | --- |
| Measure Description: | Top-loading or front-loading clothes washers that meet Energy Star or Energy Star Most Efficient requirements |
| Base Case Description: | Top-loading or front-loading clothes washers meeting Federal and Title 20 requirements (minimum Modified Energy Factor of 1.26 and maximum Water Factor of 8.5) |
| Energy Impact Common Units: | Unit |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 11 years (Appl-EffCW, ComLau-EffCW) |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Res: 0.31 (Res-sAll-mCW) Non-res: 0.60 (Com-Default>2yrs) |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13AP003.0 | No | 6/15/2012 | Jason Wang/SCE | Adapted PG&E 13-14 WP to SCE template |
| SCE13AP003.1 | Yes | 7/7/2014 | Jason Wang/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Adapted PG&E WP to SCE template. -Added measure for Energy Star Most Efficient > 2/5 cubic ft. -Updated all savings. -Changed load shape. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The base case is a top-loading or front-loading clothes washer that meets Federal efficiency requirements. The measure case is a top-loading or front-loading clothes washer that meets Energy Star or Energy Star Most Efficient requirements.

The measure case clothes washer can be used in one of these locations:

- Residential single family home or double-wide mobile home (AP-98665, AP-12634)

- Multi-family common area,for use by multiple tenants (AP-98665, AP-12634)

- Commercial building, typically a coin-operated machine inside a Laundromat (AP-61333)

See Table 1 for measures and solution codes.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| AP-98665 | Energy Star Clothes Washer |
| AP-61333 | Energy Star Clothes Washer, Coin-Operated |
| AP-12634 | Energy Star Most Efficient Clothes Washer > 2.5 cubic ft |

### 1.1.1 Measure Requirements

Energy Star Eligibility Requirements

- The clothes container volume must be ≥ 1.6 and ≤ 6.0 cubic ft.

- Configurations other than a front-loading or top-loading design do not qualify.

- Combination washer-dryers and residential clothes washers with an optional dry cycle do not qualify.

- Efficiency requirements are listed in Table 2. Note that this work paper only offers the ESME measure for residential models greater than 2.5 cubic ft.

Table 2 Energy Star (ES) and Energy Star Most Efficient (ESME) Efficiency Requirements

| Clothes Washer Type | Size (cubic ft) | Energy Star 6.1 (effective 2/1/13) |  | Energy Star Most Efficient 2014 (effective 1/1/14–12/30/14) |  |
| --- | --- | --- | --- | --- | --- |
|  |  | MEF | WF | MEF | WF |
| Residential | ≤ 2.5 | ≥ 2.0 | ≤ 6.0 | ≥ 2.4 | ≤ 4.5 |
|  | > 2.5 |  |  | ≥ 3.2 | ≤ 3.0 |
| Commercial | Any | ≥ 2.2 | ≤ 4.5 | N/A | N/A |

Energy Star Definitions

Modified Energy Factor (MEF) is the quotient of the capacity of the clothes container, C, divided by the total clothes washer energy consumption per cycle, with such energy consumption expressed as the sum of the machine electrical energy consumption, M, the hot water energy consumption, E, and the energy required for removal of the remaining moisture in the wash load, D. The water heating energy (E) may be from a gas or electric water heater.

MEF=CM+E+D

Integrated Modified Energy Factor (IMEF) is similar to the MEF but also considers the combined low-power mode energy consumption, L.

IMEF=CM+E+D+L

Water Factor (WF) is the quotient of the total weighted per-cycle water consumption, Q, divided by the capacity of the clothes washer, C.

WF=QC

Lists of qualifying products for Energy Star and Energy Star Most Efficient are available on the Energy Star web site.

## 1.2 Technical Description

Over 80% of households in the U.S. have clothes washers [423]. High-efficiency clothes washers can have front- or top-loading configurations; however, most high-efficiency washers are front-loading, using a horizontal-axis design.  A significant amount of the energy used for clothes washing is used for heating the water, so machines that use less water are usually more energy-efficient. Horizontal-axis clothes washers tumble clothes through a much smaller pool of water than conventional (top-loading) models, therefore saving more energy. High-efficiency machines also have more efficient motors, spinning clothes two to three times faster than the conventional machines. This removes more water from the clothes which reduces the amount of time and energy required to dry them.

## 1.3 Measure Application Type

The install type is Replace on Burnout (ROB).

The delivery method is Financial Support – Downstream Incentive – Deemed.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

These measures are not in DEER 2014.

Table 3 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | N/A |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net-to-Gross Ratios

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 4 below.

Table 4 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-sAll-mCW | Clothes washer MEF 10% > Energy Star | Res | Any | PreRebDown | 0.31 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 5 below.

Table 5 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 6 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | AppPlug |
| Measure Case UseSubCats | Laundry |
| Measure Case TechGroups | Clean_equip |
| Measure Case TechTypes | ClothesWash |
| Base Case TechGroups | Clean_equip |
| Base Case TechTypes | ClothesWash |

### 1.4.2 Codes and Standards Analysis

Table 7 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2014) | Section 1605.1(p) | January 1, 2007 |
| DOE | Code of Federal Regulations, 10 CFR 430.32(g) | January 1, 2007 |

Title 20 (2014): The California Appliance Efficiency Regulations [422] require that residential and commercial clothes washers manufactured on or after the dates indicated below meet minimum efficiency requirements for MEF and WF:

- “Compact clothes washer” means a clothes washer of less than 1.6 cubic ft in clothes container compartment capacity.

- “Standard clothes washer” means a clothes washer of 1.6 cubic ft or more in clothes container compartment capacity.

- “Commercial clothes washer” means a soft mount front-loading or soft mount top-loading clothes washer with clothes container compartment no greater than 3.5 cubic ft for horizontal-axis clothes washers, or no greater than 4.0 cubic ft for vertical-axis clothes washers, that is designed for use in (1) applications where the occupants of more than one household will be using it, such as multi-family housing common areas and coin laundries; or (2) other commercial applications.

- “Soft mount clothes washer” means a clothes washer that does not require mechanical fastening to a floor for proper operating performance under typical commercial clothes washer applications.

Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: The DOE federal minimum efficiency requirements match the Title 20 values shown above.

    - Non-DEER Study Review

2009 Residential Appliance Saturation Survey (RASS) [428]: The 2009 RASS study was used to estimate the residential electric and gas water heater and dryer combinations for statewide residential homes.  This distribution was used to weight various water-heating and drying energy source combinations to obtain a weighted average savings for kW, kWh and therms. The residential distribution from RASS was used for multifamily common area and non-residential units because no other comparable source was available for those applications.

Department of Energy Technical Support Documents [429, 430]: The Department of Energy (DOE) division of Energy Efficiency and Renewable Energy (EERE) has issued residential and commercial Technical Support Documents (TSDs) addressing clothes washer equipment standards. The purpose of the Technical Support Documents is to provide technical analyses for the DOE Final Rules on appliance energy conservation standards. These most recent residential clothes washer Technical Support Document was published in April 2012. This document includes per cycle energy use (in kWh) for various clothes washer efficiencies, an engineering analysis, unit shipment analysis as well as the number of cycles per year for the residential sector. The energy consumption values from the residential TSD were used to develop per cycle energy savings for residential, nonresidential, and multifamily components of the clothes washer program. The clothes washer program issues rebates for residential-sized units for all sectors, so the residential per-cycle energy consumption is appropriate for all sectors in the program. The clothes washer energy use is given in kWh, and is disaggregated by machine use, domestic hot water use, and dryer use (energy to remove moisture from the clothes). The TSD assumed an average dryer usage factor of 0.91, meaning that 91% of washer loads are dried in a clothes dryer. Upon investigation, it appears as though the dryer energy usage is exclusively for the removal of moisture from the load and does not account for the dryer motor energy to spin the drum. No sources could be located that provided a credible estimate of dryer machine energy usage.

## 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted. Table 8 below identifies the value/methodology used for the measures in this work paper.

Table 8 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| Appl-EffCW | Residential | Appliances | High Efficiency Clothes Washers | 11 | 3.7 |
| ComLau-EffCW | Commercial | Appliances | High Efficiency Clothes Washer (CEE Tiers 1,2,3) | 11 | 3.7 |

# Section 2. Energy Savings & Demand Reduction Calculations

## 2.1. Electric Savings Estimation Methodologies

DOE Technical Support Documents

The 2012 DOE Residential Clothes Washers TSD provides the following data for front-loading and top-loading clothes washers of various efficiencies (see Table 9 and Table 10):

- MEF and IMEF

- Volume

- Energy use per cycle (kWh) for machine, drying, and water heating

Table 9 Top-Loading Standard Capacity Energy Use per Cycle

| Work Paper Measure | MEF | IMEF* | Volume (ft3) | Energy Use (kWh/cycle) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Machine | Dryer | Water Heat |
| Base Case | 1.26 | 0.84 | 3.09 | 0.279 | 2.16 | 1.24 |
| N/A | 1.40 | 0.98 | 3.38 | 0.281 | 2.43 | 0.74 |
| N/A | 1.72 | 1.29 | 3.38 | 0.228 | 1.69 | 0.69 |
| N/A | 1.80 | 1.37 | 3.76 | 0.082 | 1.41 | 1.26 |
| N/A | 1.80 | 1.37 | 3.76 | 0.082 | 1.41 | 1.26 |
| N/A | 1.80 | 1.37 | 3.76 | 0.082 | 1.41 | 1.25 |
| ES | 2.00 | 1.57 | 3.86 | 0.082 | 1.38 | 0.99 |
| N/A | 2.26 | 1.83 | 3.96 | 0.077 | 1.41 | 0.67 |
| N/A | 2.47 | 2.04 | 4.34 | 0.082 | 1.39 | 0.66 |

*IMEF was calculated using the equation y = 0.988 * MEF - 0.4051, from Chapter 5 of the TSD

Table 10 Front-Loading Standard Capacity Energy Use per Cycle

| Work Paper Measure | MEF | IMEF* | Volume | Energy Use (kWh/cycle) |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Machine | Dryer | Water Heat |
| From the TSD: |  |  |  |  |  |  |
| N/A | 1.72 | 1.41 | 3.00 | 0.113 | 1.31 | 0.69 |
| N/A | 1.72 | 1.41 | 3.00 | 0.113 | 1.31 | 0.69 |
| N/A | 1.72 | 1.41 | 3.00 | 0.113 | 1.31 | 0.70 |
| N/A | 1.80 | 1.49 | 3.00 | 0.113 | 1.31 | 0.60 |
| ES | 2.00 | 1.66 | 3.30 | 0.163 | 1.42 | 0.40 |
| N/A | 2.20 | 1.84 | 3.41 | 0.154 | 1.34 | 0.36 |
| N/A | 2.40 | 2.02 | 3.60 | 0.164 | 1.41 | 0.20 |
| N/A | 2.60 | 2.20 | 3.83 | 0.167 | 1.37 | 0.21 |
| N/A | 2.89 | 2.46 | 3.90 | 0.155 | 1.38 | 0.04 |
| Calculated separately because this MEF was not covered by the TSD: |  |  |  |  |  |  |
| ESME** | 3.32 | 2.85 | 4.24 | 0.149 | 1.19 | 0.15 |

*IMEF was calculated using the equation y = 0.8979 * MEF - 0.1311, from Chapter 5 of the TSD

**ESME models are front-loading only.

Determination of Average MEF

The MEFs for the base case and measure cases are shown in Table 11. Even though the minimum required MEF for ESME is 3.2, a market average of 3.32 was determined by averaging MEFs from the ESME qualified products list.

Table 11 Average MEFs

| Measure | MEF requirement | MEF used in calculatons |  | Source |
| --- | --- | --- | --- | --- |
|  |  | Top-Loading | Front-loading |  |
| Base Case | ≥1.26 | 1.26 | 1.26 (uses the top-loading energy usage) | Title 20 requirements |
| ES | ≥2.0 | 2.0 | 2.0 | Energy Star specifications |
| ESME | ≥3.2 | N/A; all ESME models are front-loading | 3.32 | ESME Qualified Products List |

Calculations on TSD Data

The base case and ES case data were already available from the TSD (Table 9 and Table 10), so no additional calculations were needed. However, since the ESME average MEF was higher than those provided in the TSD, data from the ESME qualified products list were used to find the IMEF, average volume, and energy use for a 3.32 MEF clothes washer; see the last row of Table 10. The following example shows the calculation process:

- Find total energy use per cycle:

Total Energy Use kWh= Volume [cubic ft]IMEFcubic ftkWh

Total Energy Use, MEF 3.32=4.24 cubic ft2.85 cubic ftkWh=1.49 kWh

- Find the energy use breakdown. According to DOE tests on several clothes washers with rated efficiency values meeting the ESME criteria, the breakdown is:

- Machine energy use: 10%

- Dryer energy use: 80%

- Water heater energy use: 10%

Component Energy Use kWh=10%*Total Energy Use[kWh]

Machine Energy Use, MEF 3.32=10%*1.49 kWh=0.149 kWh

Dryer Energy Use, MEF 3.32=80%*1.49 kWh=1.19 kWh

Water Heater Energy Use, MEF 3.32=10%*1.49 kWh=0.149 kWh

- Normalize energy usage by the volume of the base case clothes washer (top-loading clothes washer, 1.26 MEF, 3.09 cubic ft). High capacity clothes washers consume more energy, but are generally more efficient because larger loads can be washed per cycle. In order to not penalize high capacity clothes washers, energy use for equivalently sized loads is determined and compared:

Normalized Energy Use=Energy Use*3.09 cubic ftAverage Measure Volume

Machine Normalized Energy Use=0.149 kWh*3.09 cubic ft4.24 cubic ft=0.108 kWh

Dryer Normalized Energy Use=1.19 kWh*3.09 cubic ft4.24 cubic ft=0.868 kWh

Water Heater Normalized Energy Use=0.149 kWh*3.09 cubic ft4.24 cubic ft=0.108 kWh

Residential Appliance Saturation Survey 2009 – Fuel Conversion and Population Weighting

The energy usage calculated so far assumes the use of electric drying and water heating. Machine energy usage is assumed to be electrical, but water heating and drying can be accomplished using either natural gas or electricity. Therefore it is necessary to calculate savings for each fuel combination:

- Electric water heater (EWH), 	Electric dryer (ED)

- Electric water heater, 		Gas dryer (GD)

- Gas-fired water heater (GWH), 	Electric dryer

- Gas-fired water heater, 		Gas dryer

For scenarios 2, 3, and 4, energy usage for gas appliances is converted from kWh to therms. Per the TSD, the following correction factors are used in the conversion because gas dryers and gas-fired water heaters have a lower Energy Factor than electric dryers and water heaters:

Gas Dryer therms=Electric Dryer kWh*0.0341thermskWh*1.12

Gas Dryer therms=0.868*0.0341thermskWh*1.12=0.033 therms

Gas WH therms=Electric WH kWh*0.0341thermskWh*100% (EWH Efficiency)75% (GWH Efficiency)

Gas WH therms=0.108*0.0341thermskWh*100%75%=0.005 therms

Population Weighting

To merge the 4 fuel combinations into a single savings value, they were weighted using PG&E and SCE distribution data from RASS:

Table 12 PG&E and SCE Water Heater and Dryer Fuel Population Weights

|  | Gas Water Heater | Electric Water Heater |
| --- | --- | --- |
| Gas Dryer | 60.37% | 0.13% |
| Electric Dryer | 38.13% | 1.37% |

Per cycle usageBase or Measure=EWH/ED kWh or therm*1.37%

+EWH/GD kWh or therm*0.13%

+GWH/ED kWh or therm*38.13%

+GWH/GD kWh or therm*60.37%

Next, the measure case is subtracted from the base case to obtain savings:

Per cycle savingstop or front=Per cycle usageBase-Per cycle usageMeasure

Market Share

The savings are then weighted based on estimated 2012 market share of top-loading and front-loading machines, from the TSD:

Table 13 Clothes Washer Market Share

| Clothes Washer Configuration | 2012 Market Share |
| --- | --- |
| Top-Loading | 50% |
| Front-Loading | 50% |

Per cycle savingsfinal=(Per cycle savingstop*50%)+(Per cycle savingsfront*50%)

These per cycle values are multiplied by the number of cycles per year, to obtain annual savings.

Table 14 Clothes Washer Annual Usage

| Clothes Washer Type | Cycles/year | Source |
| --- | --- | --- |
| Residential Single Family and Mulitfamily in-unit | 295 | 2012 Residential TSD [429] |
| Multifamily common area | 1241 | 2009 Commercial TSD [430] |
| Nonresidential | 2190 | 2009 Commercial TSD [430] |

Annual savings= Per cycle savingsfinal×Cycles per year

Interactive Effects

Since DEER interactive effects specific to clothes washers were unavailable, the HVAC interactive effects factors for CFLs were used. The interactive effects factors were selected for “IOU territory” (weighted by climate zone) and the “Existing” building vintage. The factors for PG&E and SCE were averaged to create one statewide value.  The “all Res” building type was used for the in-unit clothes washer measures, and the “Small Office” building type was used for the non-residential and multifamily common area measures.

Table 15 PG&E and SCE Averaged CFL HVAC Interactive Effects Factors

| Clothes Washer Type | Building Type | Statewide Average HVAC Factors |  |  |
| --- | --- | --- | --- | --- |
|  |  | kW/kW | kWh/kWh | therm/kWh |
| Residential Single Family and Mulitfamily in-unit | Res | 1.37 | 1.05 | -0.022 |
| Multifamily common area, Non-residential | Small Office | 1.27 | 1.10 | -0.003 |

The whole building impacts were calculated by applying CFL interactive effects factors to the end use impacts.

Annual savingsfinal= Annual savings*Interactive Effects

## 2.2. Demand Reduction Estimation Methodologies

Demand savings can be achieved through reductions in machine energy use, domestic hot water use and dryer energy use depending on the fuel source for the domestic hot water and dryer.  Electric demand savings are attributable to all domestic hot water / dryer fuel combinations since all units have electrical machine usage.

Demand savings were calculated by applying a peak kW factor to the electrical energy savings. Peak kW factor is defined as the ratio of peak kW demand to annual kWh reduction. Load shape data was used to develop peak kW factors. Peak kW factors are used to allocate annual energy usage into utility costing periods and to provide estimates of peak demand based on cost period energy usage. Based on the E3 Calculator, using the load shape DEER:Res_ClothesDishWasher, the July to September average residential peak kW factor for clothes washers is 0.000154 kW/kWh. The peak kW factor is averaged for the months of July to September because these months coincide with the DEER peak periods.

Figure 1 E3 Calculator “Input” Tab

Figure 2 E3 Calculator “Loads” Tab

## 2.3. Gas Energy Savings Estimation Methodologies

Gas energy savings for clothes washers are present in machines with gas water heaters and/or gas dryers. Gas water heaters are assumed to have an efficiency of 75% per the 2012 residential clothes washer TSD. Gas energy savings were calculated using the methodology detailed in Section 2.1.

See Attachment 2 for all calculations. See Attachment 1 for a list of all savings.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the HorizAxisClothesWasher-RC load shape.  See Table 16 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 16 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential Single Family | Residential | HorizAxisClothesWasher-RC |
| Residential Multi-family | Residential | HorizAxisClothesWasher-RC |
| Residential Mobile Home - Double-Wide | Residential | HorizAxisClothesWasher-RC |
| Misc – Commercial | Residential | HorizAxisClothesWasher-RC |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

Cost data was gathered from retailers in 2012 and the average cost by CEE Tier was determined. The base case cost is the average cost for non-qualifying products: $464.16. See Table 17.

Table 17 Clothes Washer Price Data

| Measure | CEE Tier | Min. MEF | Average Volume (cubic ft) | Average Cost |
| --- | --- | --- | --- | --- |
| Base Case | non-qualifying | < 2.0 | 3.5 | $464.16 |
| ES | 1 | 2.0 | 3.7 | $700.76 |
| N/A | 2 | 2.2 | 3.9 | $802.05 |
| ESME | 3 | 2.4 | 4.1 | $956.11 |

## 4.2 Measure Case Cost

The CEE Tier 1 average cost was used to determine the ES measure cost, and the CEE Tier 3 average cost was used to determine the ESME measure cost. Since the average volumes for CEE Tiers 1 and 3 (3.7 and 4.1 cubic ft) are different from the base case (3.5 cubic ft), a regression analysis was used to find the measure cost for a 3.5 cubic ft unit.

Table 18 GMC/IMC and Measure Cost

| Measure | Average Volume (ft3) | Base Case Cost | Average Measure Cost | GMC/IMC | GMC/IMC Adjusted by Volume | Adjusted Measure Cost |
| --- | --- | --- | --- | --- | --- | --- |
| ES | 3.7 | $464.16 | $700.76 | $236.60 | $160.51 | $624.67 |
| ESME | 4.1 | $464.16 | $956.11 | $154.05 | $283.59 | $747.75 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

In the case of ROB the equipment being replaced is assumed to have failed in place or is past its useful life, so the customer is in the situation of having to purchase new equipment.  The customer is then faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.

Gross measure cost is represented by the equation below:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

The GMC is shown in Table 18.

### 4.3.2 Incremental Measure Cost

For ROB applications, the incremental measure cost is the same as the gross measure cost.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |
| [423] | 2009 RECS Survey Data |
| [428] | Residential Appliance Saturation Survey |
| [429] | U.S. Department of Energy Technical Support Document: Residential Clothes Washers (2012) |
| [430] | U.S. Department of Energy Technical Support Document: Commercial Clothes Washers (2009) |
| [436] | 2014 DEER EUL Table - Update |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |