Work Paper PGE3PAGR114

Revision 0

Pacific Gas & Electric Company

Customer Energy Solutions

Milk Pre Cooler

Measure Code A2

EnSave Inc.

# At a Glance Summary

| Applicable Measure Codes: | A2 |
| --- | --- |
| Measure Description: | Milk Pre-Cooler (PC) |
| Energy Impact Common Units: | Milk Pre-Cooler Installed. |
| Base Case Description: | No Milk Pre-Cooler |
| Base Case Energy Consumption: | Source: EnSave Calculations |
| Measure Energy Consumption: | Source: EnSave Calculations |
| Energy Savings ((Base Case – Measure) x Interactive Effects) | Source: EnSave Calculations |
| Costs Common Units: | Milk Pre-Cooler Installed. |
| Base Case Equipment Cost ($/unit): | $0 |
| Measure Equipment Cost ($/unit): | $3,444, per Measure Cost Data PC PGE3PAGR114 Milk Pre Cooler |
| Measure Incremental Cost ($/unit): | $3,444, per Measure Cost Data PC PGE3PAGR114 Milk Pre Cooler |
| Effective Useful Life (years): | 15 years, per SPTdata_ format-v0.97 |
| Program Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | NTGR = 0.6: Agric-Default>2yrs “All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years” (DEER2011_NTGR_2012-05-16.xls Cell T58) Source2: DEER 2011 V4.01 |

# At-A-Glance Measure List

|  |  |  |  |  |  |  |  | 1st Baseline |  |  |  |  |  |  |  |  | 2nd Baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Code | Version Source | Measure Description | Measure Application Type | Building Type | Building Vintage | Climate Zone | Unit Definition | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | (EUL) LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | KW Peak Electric Demand Reduction |  | KWh Electric Savings | THM Gas Savings | LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | GRR_ kW |  | GRR_ kWh |  | GRR_ therm |  | NTG |  | Implementation Method [DI, DD, I] |  | ISR |  |  |
| A2 | Calculations | Milk Pre Cooler | ROB | BCR | ALL | ALL | Each | 0 | 16,205 | 0 | 15 | $0 | $2,950 | $494 | $3,444 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 1 |  | 1 |  | 0.6 |  | I |  | 1 |  |  |

# Work Paper Approvals

The following Manager(s) approved this workpaper through the PG&E Electronic Data Routing System under Routing Requisition # _______________

Grant Brohard Manager, Technical Product Support

_____________________ Manager, _______________ Products

Jeff Gleeson Manager, Customized & Industrial Products

_____________________ Manager, _______________ Products

Ila Homsher Manager, 3rd Party Programs

_____________________ Manager, _______________ Channel Engagement

# Document Revision History

| Revision # | Date | Original short form work paper. | Author (Company) |
| --- | --- | --- | --- |
| Revision 1 | 4/4/08 | Updated for initial DEEP 2009-2011 Submission | EnSave, Inc. |
| Revision 2 | 5/16/08 | Updated to standard Long Form. | EnSave, Inc. |
| Revision 3 | 8/13/09 | Updated for revised DEEP 2009-2011 post Bridge Funding Period. | EnSave, Inc. |
| Revision 4 | 11/13/09 | Updated EUL and NTG. | EnSave, Inc. |
| Revision 5 | 11/17/09 | Updated with references to the most recent EM&V study conducted on the 2004-2005 CA Multi Measure Farm Program. | EnSave, Inc. |
| Revision 6 | 11/23/09 | Updated Sections 1-4 to include more complete reference source descriptions. | EnSave, Inc. |
| Revision 7 | 12/9/09 | Updated kWh and kW savings based on per 00 gallon savings factor from the most recent EM&V study conducted on the 2004-2005 CA Multi Measure Farm Program. | EnSave, Inc. |
| Revision 8 | 12/22/09 | Corrected common unit definitions; updated kW savings based on revised on-peak coincidence factors. | EnSave, Inc. |
| Revision 9 | 2/11/10 | Redefined common unit definitions and associated data and descriptions. | EnSave, Inc. |
| Revision 10 | 2/25/10 | Revised unit abbreviations (minor edits). | EnSave, Inc. |
| Revision 11 | 3/1/10 | Updated AAG Format | EnSave, Inc. |
| Revision 12 | 2/3/12 | Updated measure equipment costs, measure incremental costs, measure installed costs, and installation labor costs. Changed costs common units. | EnSave, Inc. |
| Revision 13 | 6/8/12 | Updated work paper for 2013-2014. | EnSave, Inc. |
| Revision 14 | 2/8/2013 | Updated from hybrid measure based on 100 gallons per day of milk production to deemed measure based on average kWh savings per Pre Cooler installed. Includes changes at: - At a Glance Summary - Section 2.1 Energy Savings Estimation Methodologies - Section 2.2 Demand Reduction Estimation Methodologies Updated format for both Summary At A Glance pages. Updated NTG value and reference at A-A-G pages and Section 1.7. Added signature page. Removed embedded files, now to be separate files, updated file names & references. Various formatting updates. | EnSave, Inc. |
| PGE3PAGR114 Revision 0 | 2/14/2013 | Re-named from WPenNRPR0003, Rev 14 as per PG&E review request | EnSave, Inc. |

# Table of Contents

At a Glance Summary	i

At-A-Glance Measure List	ii

Work Paper Approvals	iii

Document Revision History	iv

Table of Contents	v

Section 1. General Measure & Baseline Data	1

1.1 Product Measure Description & Background	1

1.2 Product Technical Description	1

1.3 Measure Application Type	1

1.4	Product Base Case and Measure Case Information	1

1.4.1DEER Base Case and Measure Case Information	1

1.4.2 Codes & Standards Requirements Base Case and Measure Information	2

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	2

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	2

Section 2. Calculation Methods	2

2.1 Energy Savings Estimation Methodologies	2

2.2. Demand Reduction Estimation Methodologies	3

Section 3. Load Shapes	3

3.1 Base Cases Load Shapes	3

3.2 Measure Load Shapes	3

Section 4. Base Case & Measure Costs	3

4.1 Base Cases Costs	3

4.2 Measure Costs	3

4.3 Incremental & Full Measure Costs	4

References	4

# Section 1. General Measure & Baseline Data

## 1.1 Product Measure Description & Background

Milk Pre-Coolers are milk to well-water heat exchangers, typically of the plate and frame design.

They save energy by pre cooling milk, prior to it being cooled by the refrigeration system1, by using well water which is typically roughly 30 degrees colder than the incoming milk.

## 1.2 Product Technical Description

A Milk Pre-Cooler device is an efficient heat exchanger that uses cool water to reduce the temperature of the milk before it enters the bulk tank reducing the compressor run time and saving energy.

## 1.3 Measure Application Type

This measure is considered ROB (replace on burnout) as there is only a single baseline used for savings evaluation.

## Product Base Case and Measure Case Information

## 1.4.1DEER Base Case and Measure Case Information

Table 1. DEER USE and Technology Table

Net-to-Gross ratio for this work paper is based on DEER2011_NTGR_2012-05-16.xls from DEER Database for Energy-Efficient Resources; Version 2011 4.01. Error: Reference source not found2 below shows the description of the applicable Net-to-Gross ratio for this measure.

Table 2: Net-to-Gross Ratios

| NTGR_ID | Description | Sector | NTGR | Documentation |
| --- | --- | --- | --- | --- |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | 0.6 | 2011 DEER Update Report - Section 15 Table 15-3 T58 |

Effective Useful Life: DEER Version and Impact IDs

The Effective Useful Life estimates are taken from  SPTdata_ format-v0.97 for “Milk Pre-Cooler” at line 21, Index #6 and match the intended measure.

| Building type | Bldg Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- |
| Ag | n/a | n/a | 15 | 5.0 | DEER2011 | n/a |

## 1.4.2 Codes & Standards Requirements Base Case and Measure Information

There are no applicable state or federal codes that dictate or restrict the energy saving potential of this measure.

## 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

An EM&V study was completed for this measure as part of the California Multi Measure Farm Program by kW Engineering, and was summarized in a report dated March 15, 20072. This study focused on the installation of high efficiency options for five measures associated with milking at small, independent dairies. Evaluation results are based on calculations completed using data collected through end-use metering.

## 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

There are no further data or calculations provided for the support of the measures in this workpaper.

# Section 2. Calculation Methods

## 2.1 Energy Savings Estimation Methodologies

Value: 16,205 annual kWh / Pre Cooler installed.

Value was derived using the equation below for the California 2006-2008 Dairy Energy Efficiency Program and averaging the results across all installations.

Well water milk pre-coolers are generally sized to cool milk to within 3-5 degrees F of the incoming water temperature3, for steady flow conditions.  For intermittent flow conditions, as when the milk transfer pump is not set up with a variable speed drive, the temperature difference between outgoing milk and incoming well water is assumed to be 12 degrees F (see MTPVSD Workpaper for discussion of temperature differences).

The equation used for this measure is as follows:

The .68 factor for kWh savings per 100 gallons of daily milk production is per EM&V for the 2004-2005 California Multi Measure Farm Program (by KW Engineering, dated March 15, 2007), Exhibit 3, page 10.

This annual 248.2 kWh savings per 100 gallons per day of milk production is then multiplied by the average daily milk production per Pre Cooler unit, 65.29 hundred gallons per day, in the DEEP Measures Installed 06-08 PC WPenNRPR0003 data provided in the file listed in the References section.

This value is also seen in the Excel object referenced above.

Pre-coolers are assumed utilized every day of the year, owing to the nature of dairy operations.

## 2.2. Demand Reduction Estimation Methodologies

Value: 0.0 peak kW

Pre-cooling the milk prior to it entering the refrigeration system reduces the kWh consumption to cool the milk to the set point temperature. However, the existing electric refrigeration system is unchanged by this measure. Thus the peak demand kW when the refrigeration system is running is the same as the peak demand kW prior to the installation of the Pre Cooler Unit.

# Section 3. Load Shapes

## 3.1 Base Cases Load Shapes

Base case Load Shape is not applicable since the base case is no pre cooler.

## 3.2 Measure Load Shapes

Measure is an agricultural measure so the agricultural load shape is assumed in the E3 calculator

# Section 4. Base Case & Measure Costs

## 4.1 Base Cases Costs

Base case costs are not applicable since the base case is no pre cooler.

## 4.2 Measure Costs

Value: $3,444 per Pre Cooler installed.

The measure cost is derived as the average cost taken from a sample of 25 installations in DEEP 2006-2012.

| Measure Code | Measure Application Type | Baseline | Equipment Cost | Labor / Installation Cost | Maintenance / Other Cost | Total Measure Case Cost |
| --- | --- | --- | --- | --- | --- | --- |
| A2 | ROB | Average cost for Milk Pre-Cooler units installed | $2,950 | $494 | n/a | $3,444 |

All costs are noted as $ per measure unit

Note: Material costs from sample were found to be $2,950 per Pre Cooler installed. Labor and installation costs from sample were found to be $494. Chiller installations were not included in sample.

Data used to determine average costs is found in referenced Excel file.

## 4.3 Incremental & Full Measure Costs

Full measure costs are used for this measure since the base case is always no milk pre cooler.

# References

- www.dairyfarmenergy.com, , DLTech, Inc. company website

- kW Engineering, “Evaluation, Measurement and Verification Report California Multi Measure Farm Program 1354-04 and 1360-04”, prepared for California Public Utilities Commission, March 15, 2007

- http://www.delaval.com/Dairy_Knowledge/EfficientCooling/Cooling_Technology.htm, Delaval company website

- Lund, John, “Geothermal Heat Pump Utilization in the United States,” Geo Heat Center

- “DEEP Measures Installed 06-08 PC PGE3PAGR114” Excel-based analysis of measure installations in 2006-2008.

- “Measure Cost Data PC PGE3PAGR114” – Measure cost information from DEEP 2006-2012, NYSERDA, MAESTRO, and NY Disaster Relief programs.

- SPTdata_ format-v0.97 Excel file

DEER2011_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2011 4.01 found at :http://www.deeresources.com/index.php?option=com_content&view=article&id=68&Itemid=60 Under: DEER2011 Update Documentation linked at: DEER2011 Update Net-To-Gross table

Cells: (Table 15-3 T58)