Work Paper SCE13HC051

Revision 1

Southern California Edison Company

Prescriptive Window Retrofit

# At-a-Glance Summary

| Applicable Measure Codes: | BE-92103 |
| --- | --- |
| Measure Description: | 2013 Title 24 Prescriptive glass/window assembly with U-factor of 0.32 and SHGC of 0.25 |
| Base Case Description: | Standard (2005 DEER) double-pane clear glass/window assembly with U-factor of 0.67 and SHGC of 0.79 |
| Energy Impact Common Units: | Square footage of window |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | Source: Updated 2014 EUL Table / 20-years |
| Measure Application Type: | Retrofit (RET) |
| Net-to-Gross Ratios: | DEER 2014 Support Table NTGR / 0.55 |
| Important Comments: | Measure to comply with 2013 Title-24’s minimum prescriptive requirements. Unit on measure per window area (sqft) deviate from DEER units of “100 sqft” of window area. This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE15HC051.0 | No | 06/21/2012 | Jason Wang/SCE | Original work paper for 2013 PC |
| SCE15HC051.1 | Yes | 03/28/2014 | Andres Fergadiotti/SCE | Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. Estimated measure savings using ED’s recommended GLASS-TYPE-CODE methodology. Energy Analysis includes SFM and MFM building types. SFM and MFM prototypes were generated using latest MASControl software application. |

# Section 1. General Measure & Baseline Data

This work paper details energy savings on energy efficiency measure consisting of retrofitting standard (2005 DEER) double pane clear glass/window assembly with high performance glass/window assembly meeting minimum 2013 Title-24 prescriptive requirements.

## 1.1 Measure Description & Background

Base Case

The base case is a standard double-pane clear window based on 2005 DEER Update Study – Single Family Home Prototype Characteristics using 1993-2001 Vintage with typical glass/window assembly U-factor of 0.67 and Solar Heat Gain Coefficient (SHGC) of 0.79.

Measure Case

The measure case is a high performance glass/window assembly that meets the minimum 2013 Energy Standard prescriptive compliance approach for newly constructed residential buildings with maximum allowable U-factor of 0.32 and SHGC of 0.25.  The measure can be met by various alternatives - using a double and/or triple pane glass window with Low-E clear and/or tint and/or double electrochromic glass window.  Energy savings on the measure are estimated using a double pane Low-E window/glass assembly.  See Section 2 for additional performance documentation on Base case and Measure case windows.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| BE-92103 | Prescriptive Window U-0.32/SHGC 0.25 replacing Standard Window U-0.67/SHGC 0.79 |

## 1.2 Technical Description

Prescriptive glass/window assembly with U-0.32/SHGC 0.25 replacing a Standard glass/window assembly with U-0.67/SHGC 0.79

## 1.3 Measure Application Type

The delivery mechanism is Down-Stream Incentive – Deemed.

The install type is Retrofit (RET)

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | Yes (SFM) |
| Deviation from DEER | Change in unit from “100 sq ft” to “Square Foot” |
| DEER Version | 2014 DEER |
| DEER Run ID and Measure Name (Sample) | N/A |

Bases for estimating energy and demand impacts on the measured were based on 2014 DEER Residential Single Family (SFM) and Multi-Family (MFM) DOE2.2 prototypes in which performance parameters on fenestration (windows) were adjusted to meet measure criteria on both base case and measure case.

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Building Envelope |
| Measure Case UseSubCats | Fenestration |
| Measure Case TechGroups | Fenestration |
| Measure Case TechTypes | Manufactured Window |
| Base Case TechGroups | Fenestration |
| Base Case TechTypes | Manufactured Window |

### 1.4.2 Codes and Standards Analysis

Per 2013 Building Energy Efficiency Standards [355], installed fenestration products, under the performance and prescriptive compliance approaches for newly constructed residential buildings, shall have an area weighted average U-factor and SHGC no greater than the applicable value in TABLE 150.1-A  per Table 7 of this report and shall be determined in accordance with Sections 110.6(a)2 of the Standard with maximum U-factor of 0.32 and SHGC of 0.25 applicable to all SCE’s climate zones.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards – Table 150.1-A | July 1, 2014 |

Table 7 Abstract of Table 150.1-A with Climate Zones 1 through 6

### 1.4.3 Non-DEER Study Review

N/A

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2014 program cycle on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, DEER2014-EUL-table-update_2014-02-05.xlsx [436], was consulted. Table 8 below identifies the value/methodology used for the measures in this work paper.

Table 8 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| BS-Win | Residential | Building Envelope | High Performance Windows | 20 | 6.7 |

# Section 2. Energy Savings & Demand Reduction Calculations

Energy and demand savings on the measure were estimated using comparative (parametric) runs between the base case and measure case window/glass assemblies using the 2014 DEER Residential Single Family (SFM) and Multi-Family (MFM) DOE2.2 prototypes with prototypes generated using the latest MASControl software application.

Methodology for estimating energy savings is based on the GLASS-TYPE-CODE method within the eQuest/DOE2.2 energy modeling software.  The GLASS-TYPE-CODE method selects a glazing from the DOE library which contains pre-determined performance data on window/glass assemblies including center of glass and glass plus window frame heat transfer coefficient “U-factor”, Solar Heat Gain Coefficient (“SHGC”), and Visible Transmittance (“VT”) among others.  Performance parameters on both measure case and base case window/glass assemblies and selected GLASS-TYPE-CODE utilized in the energy analysis are summarized below:

Base Case Fenestration:

U-0.67/SHGC 0.79 [Double Clear]

GLASS-TYPE-CODE: 2000 [Double Clear]|U-0.57/U-0.62 (Center G./Alum w/break) | SHGC 0.76

Measure Case Fenestration:

U-0.32/SHGC 0.25 [Double Low-E (e2=0.04) Tint]

GLASS-TYPE-CODE: 2667 [Double Low-E (e2=0.04)|U-0.29/U-0.33 (Center G./Alum w/break)| SHGC 0.29

Energy savings on the measure were normalized based on total square footage of window in corresponding building (home).  For instance, the total energy and demand impacts as estimated by the building energy model were divided by the total window area as indicated by the building energy model’s LV-D report.  Based on measure characteristics and building weight documentation per climate zone, only the vintage “Built before 1978” was used in the analysis.  This assumption is based on the highest number of building stokes per IOU Weights by Climate Zone & Vintage as documented in DEER14 Database Definition-v3.xlsm per Attachment 2 in this report. Energy simulations utilize latest CTZ2010 weather data.

The measure assumes that the energy impacts on the 2nd baseline are zero (o) since the measure brings fenestration (windows) up to Title-24 prescriptive requirements.

Attachment 1 contains detail information on measure savings for both SFM and MFM building types per climate zones under SCE’s territory.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the Window_Tint-Ret load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential Single Family | Small_Office | Window_Tint-Ret |
| Residential Multi-family | Small_Office | Window_Tint-Ret |

# Section 4. Base Case & Measure Costs

Measure cost is expected to vary per glass/window assembly type which may include double or triple pane glass with Low-E clear or tint or double electrochromic glass.  Estimated measure cost information including Measure Material and Labor Costs are based on Residential Cost Data (RSMeans Residential Cost Data 2010 – [408]) documentation assuming high performance double-pane Low-E glass window.

Base Case cost was based on DEER05 documentation.

## 4.1 Base Case Cost

Base Case cost from DEER05 is $16.41 per square foot of window.

## 4.2 Measure Case Cost

## Measure cost is $32.10 per square foot of window [408].

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

Per the E3, the gross measure cost is the cost to install an energy efficient measure.  In the case of RET, REA, and REF, GMC means the full cost of the measure to purchase and install.

In the case of RET, REA, and REF, the customer is making a conscience decision to replace existing, working equipment before the useful life of the equipment has expired.  Since this is a discretionary choice by the consumer, the cost invoked is the full cost of the equipment and installation of the energy efficient equipment.

RET measures have two baseline periods for energy savings because the measure has an applicable code and/or standard baseline. At the expiration of the RUL period and the start of the EUL-RUL period, the base case is assumed to jump from the customer baseline to the code baseline.

Measure cost is estimated as $32.10 per square foot of window and Labor cost as $8.80 per square foot of window.

For RET (1st Baseline), the gross measure cost is the full measure cost:

GMC 	= (Measure Equipment Cost + Measure Labor Cost)

For RET (2nd Baseline), the gross measure cost is:

GMC 	= (Measure Equipment Cost – Code Equipment Cost)

Note that on the above expression, the measure case labor and base case labor are assumed to be the same value

Therefore:

1st Baseline GMC = $32.10 + $8.80 = $40.90

2nd Baseline GMC = $32.10 - $16.41 = $15.69

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  While IMC has a straight forward definition, depending on the install type the equation does vary.  The incremental cost is only used to help determine program incentives.  It is not affected by the first and second baseline periods and may differ from the cost used for cost effectiveness calculations.

The incremental cost is the same as the 2nd Baseline GMC: $15.69.

# Attachments

# References

[351]

[355]

[213]

[31]

[408]

[436]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |