Work Paper PGE3PAGR116

Revision 0

Pacific Gas & Electric Company

Customer Energy Solutions

Milk Vacuum Pump VSD
(Dairy Farm Equipment)

Measure Code P312

EnSave Inc.

# At a Glance Summary

| Applicable Measure Codes: | P312 |
| --- | --- |
| Measure Description: | Milk Vacuum Pump Variable Speed Drive (VPVSD). |
| Energy Impact Common Units: | Milk Vacuum Pump Variable Speed Drive Installed. |
| Base Case Description: | No Milk Vacuum Pump Variable Speed Drive Installed. |
| Base Case Energy Consumption: | Source: EnSave Calculations |
| Measure Energy Consumption: | Source: EnSave Calculations |
| Energy Savings ((Base Case – Measure) x Interactive Effects) | Source: EnSave Calculations |
| Costs Common Units: | Milk Vacuum Pump Variable Speed Drive Installed. |
| Base Case Equipment Cost ($/unit): | $0 |
| Measure Equipment Cost ($/unit): | $5,048, per Measure Cost Data VPVSD PGE3PAGR116 |
| Measure Incremental Cost ($/unit): | $5,048, per Measure Cost Data VPVSD PGE3PAGR116 |
| Effective Useful Life (years): | 15 years, per SPTdata_ format-v0.97 |
| Program Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | NTGR = 0.6: Agric-Default>2yrs “All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years” (DEER2011_NTGR_2012-05-16.xls Cell T58) Source2: DEER 2011 V4.01 |

# At-A-Glance Measure List

|  |  |  |  |  |  |  |  | 1st Baseline |  |  |  |  |  |  |  |  | 2nd Baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Code | Version Source | Measure Description | Measure Application Type | Building Type | Building Vintage | Climate Zone | Unit Definition | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | (EUL) LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | KW Peak Electric Demand Reduction |  | KWh Electric Savings | THM Gas Savings | LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | GRR_ kW |  | GRR_ kWh |  | GRR_ therm |  | NTG |  | Implementation Method [DI, DD, I] |  | ISR |  |  |
| P312 | Calculations | Milk Vacuum Pump VSD | ROB | BCR | ALL | ALL | Each | 0 | 43,687 | 0 | 15 | $0 | $3,871 | $1,177 | $5,048 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 1 |  | 1 |  | 0.6 |  | I |  | 1 |  |  |

# Work Paper Approvals

The following Manager(s) approved this workpaper through the PG&E Electronic Data Routing System under Routing Requisition # _______________

Grant Brohard Manager, Technical Product Support

_____________________ Manager, _______________ Products

Jeff Gleeson Manager, Customized & Industrial Products

_____________________ Manager, _______________ Products

Ila Homsher Manager, 3rd Party Programs

_____________________ Manager, _______________ Channel Engagement

# Document Revision History

| Revision 1 | 4/4/08 | Original short form work paper. | Author (Company) |
| --- | --- | --- | --- |
| Revision 2 | 8/26/08 | Updated to standard Long Form. | EnSave, Inc. |
| Revision 3 | 11/13/09 | Updated equipment cost, NTG, kWh, kW, EUL to agree with 2008 DEER and E3 | EnSave, Inc. |
| Revision 4 | 11/17/09 | Updated Section 1.4 with more recent EM&V study realization rates. | EnSave, Inc. |
| Revision 5 | 11/23/09 | Updated Sections 1-4 to include more complete reference source descriptions. | EnSave, Inc. |
| Revision 6 | 12/22/09 | Corrected common unit definition; edited text descriptions. | EnSave, Inc. |
| Revision 7 | 3/10/10 | Redefined and corrected common unit definitions and associated data and text descriptions; updated At A Glance table format. | EnSave, Inc. |
| Revision 8 | 2/3/12 | Updated measure equipment costs, measure incremental costs, measure installed costs, and installation labor costs. Changed costs common units. | EnSave, Inc. |
| Revision 9 | 6/8/12 | Updated work paper for 2013-2014 | EnSave, Inc. |
| Revision 10 | 2/8/2013 | Updated from hybrid measure based on pump horsepower (HP) to deemed measure based on average kWh savings per Vacuum Pump Variable Speed Drive Installed. Includes changes at: - At a Glance Summary - Section 2.1 Energy Savings Estimation Methodologies - Section 2.2 Demand Reduction Estimation Methodologies Updated format for both Summary At A Glance pages. Updated NTG value and reference at A-A-G pages and Section 1.7. Added signature page. Removed embedded files, now to be separate files, updated file names & references. Various formatting updates. | EnSave, Inc. |
| PGE3PAGR116 Revision 0 | 2/14/2013 | Re-named from WPenNRPR0006, Rev 10 as per PG&E review request | EnSave, Inc. |

# Table of Contents

At a Glance Summary	i

At-A-Glance Measure List	ii

Work Paper Approvals	iii

Document Revision History	iv

Table of Contents	v

Section 1. General Measure & Baseline Data	1

1.1 Product Measure Description & Background	1

1.2 Product Technical Description	1

1.3 Measure Application Type	1

1.4	Product Base Case and Measure Case Information	1

1.4.1	DEER Base Case and Measure Case Information	1

1.4.2 Codes & Standards Requirements Base Case and Measure Information	2

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	2

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	2

Section 2. Calculation Methods	2

2.1 Electric Energy Savings Estimation Methodologies	2

2.2. Demand Reduction Estimation Methodologies	3

Section 3. Load Shapes	4

3.1 Base Cases Load Shapes	4

3.2 Measure Load Shapes	4

Section 4. Base Case & Measure Costs	4

4.1 Base Cases Costs	4

4.2 Measure Costs	4

4.3 Incremental & Full Measure Costs	4

References	5

# Section 1. General Measure & Baseline Data

## 1.1 Product Measure Description & Background

A vacuum is needed to attach a milking unit to a cow’s udder and to help draw out the cow’s milk.  When milking units are attached to cows’ udders, less vacuum is needed than when the milking units are off the cows.  A milking vacuum pump variable speed drive (VSD) unit controls the speed of the vacuum pump motor, slowing it down when milking units are attached to udders, thus reducing the demand and energy being used.

## 1.2 Product Technical Description

A vacuum pump variable speed drive (VSD)  gauges the amount of vacuum suction needed in the parlor and adjusts the speed of the pump motor to deliver no more than is actually needed.  The energy savings is from the reduced demand of the vacuum pump.

## 1.3 Measure Application Type

This measure is considered ROB (replace on burnout) as there is only a single baseline used for savings evaluation.

## Product Base Case and Measure Case Information

## DEER Base Case and Measure Case Information

Table 1. DEER USE and Technology Table

Net-to-Gross ratio for this work paper is based on DEER2011_NTGR_2012-05-16.xls from DEER Database for Energy-Efficient Resources; Version 2011 4.01. Error: Reference source not found2 below shows the description of the applicable Net-to-Gross ratio for this measure.

Table 2: Net-to-Gross Ratios

| NTGR_ID | Description | Sector | NTGR | Documentation |
| --- | --- | --- | --- | --- |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | 0.6 | 2011 DEER Update Report - Section 15 Table 15-3 T58 |

Effective Useful Life: DEER Version and Impact IDs

The Effective Useful Life estimates are taken from  SPTdata_ format-v0.97 for “Milk Vacuum Pump Variable Speed Drive” at line 23, Index #8 and match the intended measure.

| Building type | Bldg Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- |
| Ag | n/a | n/a | 15 | 5.0 | DEER2011 | n/a |

## 1.4.2 Codes & Standards Requirements Base Case and Measure Information

There are no applicable state or federal codes that dictate or restrict the energy saving potential of this measure.

## 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

An EM&V study was completed for this measure as part of the California Multi Measure Farm Program by kW Engineering, and was summarized in a report dated March 15, 20072. This study focused on the installation of high efficiency options for five measures associated with milking at small, independent dairies. Evaluation results are based on calculations completed using data collected through end-use metering.

## 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

There are no further data or calculations provided for the support of the measures in this workpaper.

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

Value: 43,687 annual kWh /  Vacuum Pump Variable Speed Drive Installed.

Value was derived using the equation below for the California 2006-2008 Dairy Energy Efficiency Program and averaging the results across all installations.

The equation used for this measure is as follows:

Where:

HP = vacuum pump motor horsepower

.25 = HP required per milking unit owing to the VSD

MU = number of milking units

N = constant (typically .88) capturing both .746 kW/HP and 1/85% typical motor efficiency; .746/.85 = .88

DRhr = daily run hours of vacuum pump motor

DY = days milking per year

Illustrative site-specific example (which approximates, but does not match precisely the average site value discerned from the 2006-2008 Dairy Energy Efficiency Program): for a milking parlor with 20 milking units powered by a 15 HP vacuum motor used 16 hours per day and 365 days per year,

Derivation of the actual 2006-2008 Dairy Energy Efficiency Program average of 3,084 annual kWh savings / HP is provided as an Excel file in the References section.

This annual 3,084 kWh savings / HP is then multiplied by the average horsepower (HP) of the existing vacuum pump, 14.166 HP, in the DEEP Measures Installed 06-08 VPVSD Excel file listed in the References section.

This value is also seen in the Excel object referenced above.

Note that the equation has not been adjusted to reflect EM&V results because:

-	The EM&V for the 2004-2005 California Multi Measure Farm Program (by KW Engineering, dated March 15, 2007) discerned a gross realization rate of 100% for kWh savings (Exhibit 2, page 2).

-	The EM&V for the 2002-2003 California Variable Speed Drive Farm Program (by KW Engineering, dated June 9, 2004) discerned a gross realization rate of 97% kWh savings (Table 1, page 1).

## 2.2. Demand Reduction Estimation Methodologies

Value: 0.0 peak kW savings

There is no anticipated demand reduction associated with this measure.

While the average demand (kW) during milking operation will be reduced as a result of this measure, the peak demand (kW) remains unchanged. Prior to the installation of the VSD the motor is assumed to run at full load. After installation of the VSD the motor speed will be adjusted according to the load on the milking system during milking operations. However, during the washing operation after each milking the motor is typically run at full speed thus the peak demand kW during this washing operation is similar to the peak value prior to installation of the VSD.

# Section 3. Load Shapes

## 3.1 Base Cases Load Shapes

Base case Load Shape is not applicable since the base case is no milk vacuum pump VSD.

## 3.2 Measure Load Shapes

Measure is an agricultural measure so the agricultural load shape is assumed in the E3 calculator.

# Section 4. Base Case & Measure Costs

## 4.1 Base Cases Costs

Base case costs are not applicable since the base case is no VSD.

## 4.2 Measure Costs

Value: $5,048 per Milk Vacuum Pump Variable Speed Drive installed.

The measure cost is derived as the average cost taken from a sample of 25 installations in DEEP 2010-2012.

| Measure Code | Measure Application Type | Baseline | Equipment Cost | Labor / Installation Cost | Maintenance / Other Cost | Total Measure Case Cost |
| --- | --- | --- | --- | --- | --- | --- |
| P312 | ROB | Average cost for Milk Vacuum Pump Variable Speed Drive units installed | $3,871 | $1,177 | n/a | $5,048 |

All costs are noted as $ per measure unit

Note: Material costs from sample were found to be $3,871 per Milk Vacuum Pump Variable Speed Drive installed. Labor and installation costs from sample were found to be $1,177.

Data used to determine average costs is found in referenced Excel file.

## 4.3 Incremental & Full Measure Costs

Full measure costs are used for this measure since the base case is always no VSD.

# References

- Database for Energy Efficient Resources (DEER) Measure ID D03-982

- kW Engineering, Inc., Evaluation, Measurement and Verification Report California Variable Speed Drive Farm Program 243ABC-02 – June 9, 2004

- kW Engineering, Inc., Evaluation, Measurement and Verification Report California Multi Measure Farm Program 1354-04 and 1360-04 – March 15, 2007

- “DEEP Measures Installed 06-08 VPVSD PGE3PAGR116” - Excel-based derivation of milk vacuum pump VSD measure data per 2006-2008 installations.

- “Measure Cost Data VPVSD PGE3PAGR116” – Measure cost information from DEEP 2006-2012, NYSERDA, MAESTRO, and NY Disaster Relief programs.

- SPTdata_ format-v0.97 Excel file

DEER2011_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2011 4.01 found at :http://www.deeresources.com/index.php?option=com_content&view=article&id=68&Itemid=60 Under: DEER2011 Update Documentation linked at: DEER2011 Update Net-To-Gross table

Cells: (Table 15-3 T58)