Work Paper PGE3PAGR118

Revision 0

Pacific Gas & Electric Company

Customer Energy Solutions

Milk Transfer Pump Variable Speed Drive

Measure Code P313

EnSave Inc.

# At a Glance Summary

| Applicable Measure Codes: | P313 |
| --- | --- |
| Measure Description: | Milk Transfer Pump Variable Speed Drive (MTPVSD). |
| Energy Impact Common Units: | Milk Transfer Pump Variable Speed Drive Installed. |
| Base Case Description: | No MTPVSD |
| Base Case Energy Consumption: | Source: EnSave Calculations |
| Measure Energy Consumption: | Source: EnSave Calculations |
| Energy Savings ((Base Case – Measure) x Interactive Effects) | Source: EnSave Calculations |
| Costs Common Units: | Milk Transfer Pump Variable Speed Drive Installed. |
| Base Case Equipment Cost ($/unit): | $0 |
| Measure Equipment Cost ($/unit): | $2,052, per Measure Cost Data MTPVSD PGE3PAGR118 |
| Measure Incremental Cost ($/unit): | $2,052, per Measure Cost Data MTPVSD PGE3PAGR118 |
| Effective Useful Life (years): | 15 years, per SPTdata_ format-v0.97 |
| Program Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | NTGR = 0.6: Agric-Default>2yrs “All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years” (DEER2011_NTGR_2012-05-16.xls Cell T58) Source2: DEER 2011 V4.01 |

# At-A-Glance Measure List

|  |  |  |  |  |  |  |  | 1st Baseline |  |  |  |  |  |  |  |  | 2nd Baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Code | Version Source | Measure Description | Measure Application Type | Building Type | Building Vintage | Climate Zone | Unit Definition | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | (EUL) LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | KW Peak Electric Demand Reduction |  | KWh Electric Savings | THM Gas Savings | LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | GRR_ kW |  | GRR_ kWh |  | GRR_ therm |  | NTG |  | Implementation Method [DI, DD, I] |  | ISR |  |  |
| P313 | Calculations | Milk Transfer Pump Variable Speed Drive (MTPVSD) | ROB | BCR | ALL | ALL | Each | 0 | 9,777 | 0 | 15 | $0 | $1,469 | $583 | $2,052 | 0 |  | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 |  | 1 |  | 1 |  | 0.6 |  | I |  | 1 |  |  |

# Work Paper Approvals

The following Manager(s) approved this workpaper through the PG&E Electronic Data Routing System under Routing Requisition # _______________

Grant Brohard Manager, Technical Product Support

_____________________ Manager, _______________ Products

Jeff Gleeson Manager, Customized & Industrial Products

_____________________ Manager, _______________ Products

Ila Homsher Manager, 3rd Party Programs

_____________________ Manager, _______________ Channel Engagement

# Document Revision History

| Revision # | Date | Original short form work paper. | Author (Company) |
| --- | --- | --- | --- |
| Revision 1 | 4/04/08 | Updated Short form work paper | EnSave, Inc. |
| Revision 2 | 5/16/08 | Updated to Long Form | EnSave, Inc. |
| Revision 3 | 8/13/09 | Updated for revised DEEP 2009-2011 post Bridge Funding Period | EnSave, Inc. |
| Revision 4 | 11/17/09 | Updated with references to the most recent EM&V study conducted on the 2004-2005 CA Multi Measure Farm Program. | EnSave, Inc. |
| Revision 5 | 11/24/09 | Updated Sections 1-4 to include more complete reference source descriptions. | EnSave, Inc. |
| Revision 6 | 12/9/09 | Updated kWh and kW savings based on per 00 gallon savings factor from the most recent EM&V study conducted on the 2004-2005 CA Multi Measure Farm Program. | EnSave, Inc. |
| Revision 7 | 12/22/09 | Corrected common unit definitions; corrected EUL; updated kW savings based on revised on-peak coincidence factors. | EnSave, Inc. |
| Revision 8 | 2/11/10 | Redefined common unit definitions and associated data and descriptions. | EnSave, Inc. |
| Revision 9 | 2/25/10 | Revised unit abbreviations (minor edits). | EnSave, Inc. |
| Revision 10 | 3/1/10 | Updated At A Glance table format. | EnSave, Inc. |
| Revision 11 | 3/5/10 | Minor text corrections and clarifications. | EnSave, Inc. |
| Revision 12 | 2/3/12 | Updated measure equipment costs, measure incremental costs, measure installed costs, and installation labor costs. Changed costs common units. | EnSave, Inc. |
| Revision 13 | 6/08/12 | Updated work paper for 2013-2014. | EnSave, Inc. |
| Revision 14 | 2/8/2013 | Updated from hybrid measure based on 100 gallons per day of milk production to deemed measure based on average kWh savings per Milk Transfer Pump Variable Speed Drive Installed. Includes changes at: - At a Glance Summary - Section 2.1 Energy Savings Estimation Methodologies - Section 2.2 Demand Reduction Estimation Methodologies Updated format for both Summary At A Glance pages. Updated NTG value and reference at A-A-G pages and Section 1.7. Added signature page. Removed embedded files, now to be separate files, updated file names & references. Various formatting updates. | EnSave, Inc. |
| PGE3PAGR118 Revision 0 | 2/14/2013 | Re-named from WPenNRPR0004, Rev 14 as per PG&E review request | EnSave, Inc. |

# Table of Contents

At a Glance Summary	i

At-A-Glance Measure List	ii

Work Paper Approvals	iii

Document Revision History	iv

Table of Contents	v

Section 1. General Measure & Baseline Data	1

1.1 Product Measure Description & Background	1

1.2 Product Technical Description	1

1.3 Measure Application Type	1

1.4	Product Base Case and Measure Case Information	1

1.4.1 DEER Base Case and Measure Case Information	1

1.4.2 Codes & Standards Requirements Base Case and Measure Information	2

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	2

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	2

Section 2. Calculation Methods	2

2.1 Electric Energy Savings Estimation Methodologies	2

2.2. Demand Reduction Estimation Methodologies	3

Section 3. Load Shapes	3

3.1 Base Cases Load Shapes	3

3.2 Measure Load Shapes	4

Section 4. Base Case & Measure Costs	4

4.1 Base Cases Costs	4

4.2 Measure Costs	4

4.3 Incremental & Full Measure Costs	4

References	5

# Section 1. General Measure & Baseline Data

## 1.1 Product Measure Description & Background

The milk transfer pump variable speed drive (MTPVSD) measure is only valid if used in conjunction with a once-through well water-to-milk pre cooler.  The measure saves energy by providing a steadier flow of milk through the pre cooler, resulting in more effective precooling of the milk.

## 1.2 Product Technical Description

Milk is typically collected in relatively small (10-30 gallon) receiver tanks in the milking parlor1.  When the level of the milk in the receiver tank reaches a high level sensor, the pump is turned on (at full speed) and the tank is quickly emptied.  A typical cycle might be 10 seconds on and 30 seconds off, meaning that the pump is on 1/4th of the time, and when it is on, its flow rate is four times the average incoming milk flow rate.  This high flow rate increases the temperature at which the milk leaves the pre-cooler, and increases the cooling load on the refrigeration system.

## 1.3 Measure Application Type

This measure is considered ROB (replace on burnout) as there is only a single baseline used for savings evaluation.

## Product Base Case and Measure Case Information

## 1.4.1 DEER Base Case and Measure Case Information

Table 1. DEER USE and Technology Table

Net-to-Gross ratio for this work paper is based on DEER2011_NTGR_2012-05-16.xls from DEER Database for Energy-Efficient Resources; Version 2011 4.01. 2 below shows the description of the applicable Net-to-Gross ratio for this measure.

Table 2: Net-to-Gross Ratios

| NTGR_ID | Description | Sector | NTGR | Documentation |
| --- | --- | --- | --- | --- |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | 0.6 | 2011 DEER Update Report - Section 15 Table 15-3 T58 |

Effective Useful Life: DEER Version and Impact IDs

The Effective Useful Life estimates are taken from  SPTdata_ format-v0.97 for “Milk Transfer Pump Variable Speed Drive” at line 22, Index #7 and match the intended measure.

| Building type | Bldg Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- |
| Ag | n/a | n/a | 15 | 5.0 | DEER2011 | n/a |

## 1.4.2 Codes & Standards Requirements Base Case and Measure Information

There are no applicable state or federal codes that dictate or restrict the energy saving potential of this measure.

## 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

An EM&V study was completed for this measure as part of the California Multi Measure Farm Program by kW Engineering, and was summarized in a report dated March 15, 20072. This study focused on the installation of high efficiency options for five measures associated with milking at small, independent dairies. Evaluation results are based on calculations completed using data collected through end-use metering.

## 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

There are no further data or calculations provided for the support of the measures in this workpaper.

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

Value: 9,777 annual kWh/ Milk Transfer Pump Variable Speed Drive Installed.

Value was derived using the equation below for the California 2006-2008 Dairy Energy Efficiency Program and averaging the results across all installations.

Well water to milk precoolers are typically designed for about a 4F approach temperature difference3 (the difference between the temperature of the milk as it exits the precooler, and the temperature of the well water as it enters the precooler).  However, when the milk flow rate is twice the design flow rate, the approach temperature difference goes up to over 12F, based on a simple log mean temperature difference (LMTD) model of a counter flow heat exchanger.  So, without an MTPVSD, the precooler is assumed to pre cool the milk to within 12 F of the incoming well water, while with an MTPVSD, the precooler is able to cool the milk to within 4F of the incoming well water. Thus there is an effective improvement in the effectiveness of the precooler of 12F – 4F = 8F.

There are numerous factors affecting the actual savings achieved in a particular installation, including the internal volume of the plate cooler, the on/off cycle of the milk transfer pump without a VSD, the remaining flow variability of the milk transfer pump once it’s been equipped with a VSD, the entry/clean/milk/exit cycle of the cows.

The equation used for this measure is as follows:

The factor of .39 kWh savings per day per 100 gallons of daily milk production is per EM&V for the 2004-2005 California Multi Measure Farm Program (by KW Engineering, dated March 15, 2007), Exhibit 5, page 12.

This annual 142.35 kWh savings / 100 gallons per day of milk production is then multiplied by the average daily milk production per Milk Transfer Pump Variable Speed Drive Installed, 68.68  hundred gallons per day, in the DEEP Measures Installed 06-08 MTPVSD Excel file listed in the References section.

This value is also seen in the Excel object referenced above.

Milk transfer pump variable speed drives are assumed utilized every day of the year, owing to the nature of dairy operations.

## 2.2. Demand Reduction Estimation Methodologies

Value 0.0 peak kW savings

There is no anticipated demand reduction associated with this measure.

While the average demand (kW) during milking operation will be reduced as a result of this measure, the peak demand (kW) remains unchanged. Prior to the installation of the VSD the motor is assumed to run at full load. After installation of the VSD the motor speed will be adjusted according to the load on the milking system during milking operations. However, during the washing operation after each milking the motor is typically run at full speed thus the peak demand kW during this washing operation is similar to the peak value prior to installation of the VSD.

# Section 3. Load Shapes

## 3.1 Base Cases Load Shapes

Base case Load Shape is not applicable since the base case is no MTPVSD.

## 3.2 Measure Load Shapes

Measure is an agricultural measure so the agricultural load shape is assumed in the E3 calculator

# Section 4. Base Case & Measure Costs

## 4.1 Base Cases Costs

Base case costs are not applicable since the base case is no MTPVSD.

## 4.2 Measure Costs

Value: $2,052 per Milk Transfer Pump Variable Speed Drive installed.

The measure cost is derived as the average cost taken from a sample of 25 installations in DEEP 2009-2012.

| Measure Code | Measure Application Type | Baseline | Equipment Cost | Labor / Installation Cost | Maintenance / Other Cost | Total Measure Case Cost |
| --- | --- | --- | --- | --- | --- | --- |
| P313 | ROB | Average cost for Milk Transfer Pump Variable Speed Drive units installed | $1,469 | $583 | n/a | $2,052 |

All costs are noted as $ per measure unit

Note: Material costs from sample were found to be $1,469 per Milk Transfer Pump Variable Speed Drive installed. Labor and installation costs from sample were found to be $583.

Data used to determine average costs is found in referenced Excel file.

## 4.3 Incremental & Full Measure Costs

Full measure costs are used for this measure since the base case is always no MTPVSD.

# References

- www.dairyfarmenergy.com, DLTech, Inc. company website

- kW Engineering, “Evaluation, Measurement and Verification Report California Multi Measure Farm Program 1354-04 and 1360-04”, prepared for California Public Utilities Commission, March 15, 2007

- http://www.delaval.com/Dairy_Knowledge/EfficientCooling/Cooling_Technology.htm, Delaval company website

- “DEEP Measures Installed 06-08 MTPVSD PGE3PAGR118” Excel-based analysis of measure installations in 2006-2008.

- “Measure Cost Data MTPVSD PGE3PAGR118” – Measure cost information from DEEP 2006-2012, NYSERDA, MAESTRO, and NY Disaster Relief programs.

- SPTdata_ format-v0.97 Excel file

DEER2011_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2011 4.01 found at :http://www.deeresources.com/index.php?option=com_content&view=article&id=68&Itemid=60 Under: DEER2011 Update Documentation linked at: DEER2011 Update Net-To-Gross table

Cells: (Table 15-3 T58)