Work Paper SCE13CC001

Revision 2

Southern California Edison Company

Commercial Reach-In Refrigerators and Freezers

# At-a-Glance Summary

| Applicable Measure Codes: | FS-13121 FS-44686 FS-16170 FS-54809 FS-21276 FS-58112 FS-28291 FS-61432 FS-30794 FS-68882 FS-31425 FS-77373 FS-38598 FS-86848 FS-42962 FS-93044 |
| --- | --- |
| Measure Description: | Energy efficient commercial reach-in solid and glass door refrigerators and freezers, in both vertical and chest configurations |
| Base Case Description: | Reach-in solid and glass door refrigerators and freezers governed by code. |
| Energy Impact Common Units: | Per unit |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 12 years (Cook-SDRef) |
| Measure Application Type: | Early Retirement (RET), & Replace on Burnout (ROB). |
| Net-to-Gross Ratios: | 0.6 (Com-Default>2yrs) |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13CC001.0 | Yes | 04/06/2012 | John Rossi/EMCOR | - Updated to 2013-2014 Template. |
| SCE13CC001.1 | Yes | 10/03/2013 | Alfredo Gutierrez/SCE | - Updated savings value (kWh and kW) for Glass-Door Reach-In Refrigerators with volume between 30-49 ft3 - Savings values (kWh and kW) have been fixed for the following measures: - < 15 cubic feet Glass-Door Reach-In Freezer (FS-16170) - 15 – 29 cubic feet Glass-Door Reach-In Freezer (FS-38598) - The following measures have been updated with a new installation type (RET) and delivery method (Midstream): - < 15 cubic feet Glass-Door Reach-In Refrigerator (FS-77373) - 15 – 29 cubic feet Glass-Door Reach-In Refrigerator (FS-28291) - 30 – 49 cubic feet Glass-Door Reach-In Refrigerator (FS-68882) - ≥ 50 cubic feet Glass-Door Reach-In Refrigerator (FS-44686) |
| SCE13CC001.2 | No | 06/23/2014 | Alfredo Gutierrez/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Updated work paper template. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the purchase of new or replacement energy efficient commercial reach-in solid and glass door refrigerators and freezers, in both vertical and chest configurations. In all categories, the refrigeration system shall be built-in (packaged).

The following table lists the core measures associated with this work paper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| FS-13121 | < 15 cubic feet Solid-Door Reach-In Refrigerator |
| FS-42962 | 15 – 29 cubic feet Solid-Door Reach-In Refrigerator |
| FS-93044 | 30 – 49 cubic feet Solid-Door Reach-In Refrigerator |
| FS-31425 | ≥ 50 cubic feet Solid-Door Reach-In Refrigerator |
| FS-21276 | < 15 cubic feet Solid-Door Reach-In Freezer |
| FS-61432 | 15 – 29 cubic feet Solid-Door Reach-In Freezer |
| FS-86848 | 30 – 49 cubic feet Solid-Door Reach-In Freezer |
| FS-54809 | ≥ 50 cubic feet Solid-Door Reach-In Freezer |
| FS-77373 | < 15 cubic feet Glass-Door Reach-In Refrigerator |
| FS-28291 | 15 – 29 cubic feet Glass-Door Reach-In Refrigerator |
| FS-68882 | 30 – 49 cubic feet Glass-Door Reach-In Refrigerator |
| FS-44686 | ≥ 50 cubic feet Glass-Door Reach-In Refrigerator |
| FS-16170 | < 15 cubic feet Glass-Door Reach-In Freezer |
| FS-38598 | 15 – 29 cubic feet Glass-Door Reach-In Freezer |
| FS-58112 | 30 – 49 cubic feet Glass-Door Reach-In Freezer |
| FS-30794 | ≥ 50 cubic feet Glass-Door Reach-In Freezer |

Units with remote refrigeration systems do not qualify. Used or rebuilt equipment is not eligible. Each installed equipment must meet minimum prescriptive criteria to qualify for a per unit rebate. This measure is only applicable to Non-residential building types. This measure is not approved for installation in Residential Single Family, Residential Multi-Family, or Residential Mobile Home – Double Wide building types. This measure is eligible in all SCE climate zones.

## 1.2 Technical Description

Reach-in refrigerators and freezers are typically used to house perishable goods, such as soft drinks in supermarkets or other commercial areas. They are typically constructed from stainless steel and have a thick layer of insulation in the walls, doors, and floor. The cabinets have one to three doors, made of either glass or steel, and come in a variety of sizes with storage capabilities up to 72 cubic feet. These units are known as self-contained as they have an internal compressor for cooling rather than relying on an external compressor.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The delivery methods for the measures in this work paper are Financial Support – Down Stream Incentive – Deemed and Financial Support – Midstream Incentive –Deemed.

The program type is Replace-On-Burnout (ROB) and Retrofit (RET). In the early retirement approach, functional units will be identified through the Energy Efficiency Mid-Stream Bottling Pilot program which targets glass-door reach-in refrigerators. These units will be targeted for participation through the pilot program and will be retired prematurely. The pilot program will provide incentives to the contractor for this to occur.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

The 2014 DEER database does contain energy star refrigerators and freezers as well as recycled refrigerators and freezers. The DEER database, however, does not contain solid or glass door reach-in refrigerators or freezers.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | Measures are not in DEER |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | All | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID* | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

*Denotes that the column is taken from the DEER GSIA Table.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Commercial Refrigeration |
| Measure Case UseSubCats | Refrigerated Display |
| Measure Case TechGroups | Refrigerated Storage |
| Measure Case TechTypes | Reach-In Storage |
| Base Case TechGroups | Refrigerated Storage |
| Base Case TechTypes | Reach-In Storage |

### 1.4.2 Codes and Standards Analysis

The 2013 Title 24 [355] code does not apply in this case because solid door reach-in refrigerators, solid door reach-in freezers, glass door reach-in refrigerators, and glass door reach-in freezers do not fall under the CEC Title 24, Building Energy Efficiency Standards or the Department of Energy (DOE) energy regulations.

The 2014 Title 20 [422] code does not apply to the measures within this work paper. As such, Federal Code, 10 CFR 431.66, would cover these measures.

Per the Federal Code:

Each commercial refrigerator, freezer, and refrigerator-freezer with a self-contained condensing unit designed for holding temperature applications manufactured on or after January 1, 2010, shall have a daily energy consumption (in kilowatt hours per day) that does not exceed the following:

Table 6 Base Case - Federal Requirements for Reach-In Refrigerators and Freezers

| Equipment Description (cubic feet) | Daily Energy Usage (kWh/day) |
| --- | --- |
| Solid-Door Reach-In Refrigerator | 0.100V+2.04* |
| Solid-Door Reach-In Freezer | 0.400V+1.38 |
| Glass-Door Reach-In Refrigerator | 0.120V+3.34 |
| Glass-Door Reach-In Freezer | 0.750V+4.10 |

*V is the internal volume in cubic feet.

As of January 1, 2010, the new ENERGY STAR® Version 2.0 specification [A] is effective, establishing the measure case equipment efficiencies. The following table lists the applicable codes. Measure case energy use requirements for qualifying equipment are shown in Table 7. As of January 1, 2010, the ENERGY STAR® specification was changed to the new Version 2.0 specification [A]. For the 2013 to 2014 program cycle, the new ENERGY STAR® Version 2.0 specification will be used as the requirement for the measures.

Table 7 Measure Case - Energy Star® Version 2.0 Requirements for Reach-In Refrigerators and Freezers

| Equipment Description (cubic feet) | Daily Energy Usage (kWh/day) |  |
| --- | --- | --- |
| Solid-Door Reach-In Refrigerator |  |  |
| 0 < V < 15 | 0.089V + 1.411* |  |
| 15 V < 30 | 0.037V + 2.200 |  |
| 30 V < 50 | 0.056V + 1.635 |  |
| 50 V | 0.060V + 1.416 |  |
| Solid-Door Reach-In Freezer |  |  |
| 0 < V < 15 | 0.250V + 1.250 |  |
| 15 V < 30 | 0.400V – 1.000 |  |
| 30 V < 50 | 0.163V + 6.125 |  |
| 50 V | 0.158V + 6.333 |  |
| Glass-Door Reach-In Refrigerator |  |  |
| 0 < V < 15 | 0.118V + 1.382 |  |
| 15 V < 30 | 0.140V + 1.050 |  |
| 30 V < 50 | 0.088V + 2.625 |  |
| 50 V | 0.110V + 1.500 |  |
| Glass-Door Reach-In Freezer |  |  |
| 0 < V < 15 |  | 0.607V + 0.893 |
| 15 V < 30 |  | 0.733V - 1.000 |
| 30 V < 50 |  | 0.250V + 13.500 |
| 50 V |  | 0.450V + 3.500 |

*V is the internal volume in cubic feet.

Table 8 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | N/A |
| Title 20 (2014) | N/A | N/A |
| U.S. DOE Code of Federal Regulations | 10 CFR 431.66 | January 1, 2010 to Present |
| ENERGY STAR 2.1 | Commercial Refrigerators and Freezers Program Requirements | January 1, 2010 to Present |

### 1.4.3 Non-DEER Study Review

No additional studies were used in the creation of this work paper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted. Table 9 below identifies the value/methodology used for the measures in this work paper.

Table 9 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| Cook-SDRef | Commercial | Refrigeration | Commercial Reach-In Refrigerator / Freezer | 12 | 4 |

# Section 2. Energy Savings & Demand Reduction Calculations

The annual energy savings are summarized in Table 10 through Table 13. The nominal size and internal volume range are based on standard widths and each standard width may include multiple (French style) doors and/or multiple sections. The calculated annual energy usage was based on 365 days per year operation, and the daily energy consumption for typical units was determined by applying the code and measure maximum daily energy consumption as shown in Table 6 and Table 7.

A sample calculation showing the daily and annual energy savings of a glass-door refrigerator is provided below.

Glass-Door Reach-In Refrigerator with total volume less than 15 cubic feet (0 < V < 15 ft3):

Volume (V) = 10 ft3

Measure Case Daily Energy Usage 	= 0.118 * V + 1.382 (See Table 7)

= 0.118 * 10 + 1.382

= 2.56 kWh / day

Measure Case Annual Energy Usage	= (Daily Energy Usage) * 365 days per year

= 2.56 kWh / day * 365 days / year

= 934 kWh / year

Base Case Daily Energy Usage 	= 0.120 * V + 3.34 (See Table 6)

= 0.120 * 10 + 3.34

= 4.54 kWh / day

Base Case Annual Energy Usage	= (Daily Energy Usage) * 365 days per year

= 4.54 kWh / day * 365 days / year

= 1657 kWh / year

Annual Energy Savings	= Base Case Annual Usage – Measure Case Annual Usage

= 1657 – 934 kWh / year

= 723 kWh / year (see Table 12)

Complete calculation methodologies for both energy savings and demand reductions are in the calculation worksheet [B].

Table 10 through Table 13 below show the energy savings associated with commercial solid and glass reach-in refrigerators and freezers.

Table 10 Energy Savings for Commercial Solid-Door Reach-in Refrigerators

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 3.04 | 4.44 | 6.44 | 9.24 |
| Measure Unit Daily Energy Use (kWh/day) | 2.30 | 3.09 | 4.10 | 5.74 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1110 | 1621 | 2351 | 3373 |
| Measure Unit Annual Energy Use (kWh/yr) | 840 | 1127 | 1496 | 2094 |
| Annual Energy Savings (kWh/yr) | 270 | 493 | 854 | 1279 |

Table 11 Energy Savings for Commercial Solid-Door Reach-in Freezers

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 5.38 | 10.98 | 18.98 | 30.18 |
| Measure Unit Daily Energy Use (kWh/day) | 3.75 | 8.6 | 13.30 | 17.71 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1964 | 4008 | 6928 | 11016 |
| Measure Unit Annual Energy Use (kWh/yr) | 1369 | 3139 | 4853 | 6464 |
| Annual Energy Savings (kWh/yr) | 595 | 869 | 2074 | 4552 |

Table 12 Energy Savings for Commercial Glass-Door Reach-in Refrigerators

|  | Glass-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 4.54 | 6.22 | 8.62 | 11.98 |
| Measure Unit Daily Energy Use (kWh/day) | 2.56 | 4.41 | 6.50 | 9.42 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1657 | 2270 | 3146 | 4373 |
| Measure Unit Annual Energy Use (kWh/yr) | 934 | 1610 | 2371 | 3438 |
| Annual Energy Savings (kWh/yr) | 723 | 661 | 775 | 934 |

Table 13 Energy Savings for Commercial Glass-Door Reach-in Freezers

|  | Glass-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 11.6 | 22.1 | 37.1 | 58.1 |
| Measure Unit Daily Energy Use (kWh/day) | 6.96 | 16.59 | 24.50 | 35.90 |
| Base Case Unit Annual Energy Use (kWh/yr) | 4234 | 8067 | 13542 | 21207 |
| Measure Unit Annual Energy Use (kWh/yr) | 2541 | 6056 | 8943 | 13104 |
| Annual Energy Savings (kWh/yr) | 1693 | 2010 | 4599 | 8103 |

## 2.2. Demand Reduction Estimation Methodologies

The demand reduction estimations are obtained by dividing the daily energy usage (Table 10 through Table 13) by 24 hours, as the units are plugged in 24/7.

The demand reduction estimation is summarized in Table 14 through Table 17.

A sample calculation showing the peak demand reduction associate with a glass-door reach-in refrigerator for the 2013-2014 program years is provided below.

Glass-Door Reach-In Refrigerator with total volume less than 15 cubic feet (0 < V < 15 ft3):

Volume (V) = 10 ft3

Measure Case Peak Demand Reduction	= Measure Daily Energy Usage / 24 hrs

= 2.56 / 24

= 0.107 kW

Base Case Peak Demand Reduction 		= Base Daily Energy Usage / 24 hrs

= 4.54 / 24

= 0.189 kW

Demand Reduction Savings	= (Base Reduction – Measure Reduction)

= 0.189 kW – 0.107 kW

= 0.083 kW

Peak Demand Reduction Savings	= Reduction Savings * 0.9

= 0.083 *0.9

= 0.074 kW

A multiplier of 0.9 is used to conservatively estimate the peak demand reduction during the DEER peak period for the particular climate zone.

Table 14 through Table 17 show the demand reductions associated with commercial glass and solid door reach-in refrigerators and freezers.

Table 14 Demand Reduction for Commercial Solid-Door Reach-in Refrigerators

|  | Solid-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.127 | 0.185 | 0.268 | 0.385 |
| Measure Unit Demand (kW) | 0.096 | 0.129 | 0.171 | 0.239 |
| Demand Reduction (kW) | 0.031 | 0.056 | 0.098 | 0.146 |
| Peak Demand Reduction (kW) | 0.028 | 0.051 | 0.088 | 0.131 |

Table 15 Demand reduction for Commercial Solid-Door Reach-in Freezers

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.224 | 0.458 | 0.791 | 1.258 |
| Measure Unit Demand (kW) | 0.156 | 0.358 | 0.554 | 0.738 |
| Demand Reduction (kW) | 0.068 | 0.099 | 0.237 | 0.520 |
| Peak Demand Reduction (kW) | 0.061 | 0.089 | 0.213 | 0.468 |

Table 16 Demand reductions for Commercial Glass-Door Reach-in Refrigerators

|  | Glass-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.189 | 0.259 | 0.359 | 0.499 |
| Measure Unit Demand (kW) | 0.107 | 0.184 | 0.271 | 0.393 |
| Demand Reduction (kW) | 0.083 | 0.075 | 0.088 | 0.107 |
| Peak Demand Reduction (kW) | 0.074 | 0.068 | 0.079 | 0.096 |

Table 17 Demand reductions for Commercial Glass-Door Reach-in Freezers

|  | Glass-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.483 | 0.921 | 1.546 | 2.421 |
| Measure Unit Demand (kW) | 0.290 | 0.691 | 1.021 | 1.496 |
| Demand Reduction (kW) | 0.193 | 0.230 | 0.525 | 0.925 |
| Peak Demand Reduction (kW) | 0.174 | 0.207 | 0.473 | 0.833 |

Please note that for all RET measures in this work paper, the savings have been mapped to the savings associated with ROB measures. The current market for existing reach-in refrigerators is unknown, so the savings are based on above code required equipment until further research can identify the current market mix of existing equipment. Once this information is known, the work paper will be updated to incorporate the true baseline found from market research.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available. Therefore, the closest load shape chosen for this measure is the Refrigeration load shape. See Table 18 for a list of all Building Types and Load Shapes. See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 18 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | Agricultural | Refrigeration |
| Assembly | Assembly | Refrigeration |
| Education - Primary School | K_thru_12_School | Refrigeration |
| Education - Secondary School | K_thru_12_School | Refrigeration |
| Education - Relocatable Classroom | K_thru_12_School | Refrigeration |
| Education - Community College | College_University | Refrigeration |
| Education - University | College_University | Refrigeration |
| Grocery | Grocery_Store | Refrigeration |
| Food Store | Food_Store | Refrigeration |
| Health/Medical - Hospital | Hospital | Refrigeration |
| Health/Medical - Nursing Home | Medical_Clinic | Refrigeration |
| Health/Medical - Clinic | Small_Office | Refrigeration |
| Lodging - Hotel | Hotel_Motel | Refrigeration |
| Lodging - Guest Rooms | Hotel_Motel | Refrigeration |
| Lodging - Motel | Hotel_Motel | Refrigeration |
| Manufacturing - Bio/Tech | Industrial | Refrigeration |
| Manufacturing - Light Industrial | Industrial | Refrigeration |
| Industrial | Industrial | Refrigeration |
| Misc - Commercial | Misc._Commercial | Refrigeration |
| Office – Large | Large_Office | Refrigeration |
| Office – Small | Small_Office | Refrigeration |
| Restaurant - Fast-Food | Fast_Food_Restaurant | Refrigeration |
| Restaurant - Sit-Down | Sit_Down_Restaurant | Refrigeration |
| Retail - Multistory Large | Large_Retail_Store | Refrigeration |
| Retail – Single-Story Large | Large_Retail_Store | Refrigeration |
| Retail – Small | Small_Retail_Store | Refrigeration |
| Storage - Conditioned | Storage_Building | Refrigeration |
| Storage - Unconditioned | Storage_Building | Refrigeration |
| Transportation - Communication – Utilities | Trans_Comm_Util | Refrigeration |
| Warehouse - Refrigerated | Refrigerated_Warehouse | Refrigeration |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

There is no additional labor or maintenance cost when comparing the measure to the base case for solid and glass door reach-in refrigerators and solid door reach-in freezers. The cost includes both material and labor, and applies to ROB and RET installations, which expects to have the same cost for all customers. The estimated equipment cost is based on manufacturer list prices and applying an industry-standard 44% discount to the published list prices [C]. The base case cost can be symbolically written as:

Base Case Cost = Base Case Equipment Cost + Base Case Installation Cost

The purchase prices for the Base Case solid- and glass-door, reach-in refrigerators and freezers are listed in Table 19.

Table 19 Base Case Refrigerator and Freezer Material and Labor Costs

| Typical Appliance Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Typical Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators | $ 1,284.00 | $ 2,883.15 | $ 3,802.03 | $ 4,526.32 |
| Solid-Door Reach-In Freezers | $ 773.84 | $ 3,467.04 | $ 5,151.16 | $ 7,469.68 |
| Glass-Door Reach-In Refrigerators | $ 1270.00 | $ 3,340.20 | $ 4,788.00 | $ 6,915.84 |
| Glass-Door Reach-In Freezers (Energy Star 1.0)** | $ 1,281.20 | $ 5,212.19 | $ 7,401.52 | $ 10,304.40 |

## 4.2 Measure Case Cost

The measure costs include equipment and labor, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 44% discount to the manufacturer published list prices [C]. The Measure Cost is symbolically written as:

Measure Cost = Measure Equipment Cost + Measure Installation Cost

Table 20 summarizes the purchase price for the Measure solid- and glass-door, reach-in refrigerators and freezers. Solid-door refrigerator and freezers purchase price are shown in Table 20.

Table 20 Measure Refrigerator and Freezer Material and Labor Costs

| Typical Appliance Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Typical Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators (Energy Star 2.0) | $ 2,376.00 | $ 4,293.88 | $ 5,770.73 | $ 7,249.60 |
| Solid-Door Reach-In Freezers (Energy Star 2.0) | $ 1,031.44 | $ 4,830.22 | $ 6,707.87 | $ 9,437.71 |
| Glass-Door Reach-In Refrigerators (Energy Star 2.0) | $ 1,373.60 | $ 4,204.00 | $ 5,864.11 | $ 8,464.80 |
| Glass-Door Reach-In Freezers (Energy Star 2.0) | $ 1,306.68 | $ 5,336.22 | $ 7,615.72 | $ 11,203.70 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB measures, DEER defines Full Measure Cost as the sum of the equipment measure cost and the labor cost to install the measure. The CPUC Standard Practice Manual recognizes additional costs that may be included in the TRC calculations such as annual maintenance costs [E]. However, in keeping with the DEER, this work paper adopts its minimalist definition. The Full Measure cost can be symbolically written as:

Full Measure Cost = Measure Equipment Cost + Measure Installation Labor Cost

The Gross Measure Cost (GMC) for ROB measures can be written as:

Gross Measure Cost = Measure Equipment Cost – Base Case Equipment Cost

Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Calculations [B].

The table below summarizes the GMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as ROB. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

Table 21 Gross Measure Refrigerator and Freezer Costs for ROB

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | $ 1,092.00 | $ 1,410.73 | $ 1,968.70 | $ 2,723.28 |
| Solid-Door Reach-In Freezers Incremental Cost | $ 257.60 | $ 1,363.18 | $ 1,556.71 | $ 1,968.03 |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 103.60 | $ 863.80 | $ 1,076.11 | $ 1,548.96 |
| Glass-Door Reach-In Freezers Incremental Cost | $ 25.48 | $ 124.04 | $ 214.20 | $ 899.30 |

For RET measures, the GMC is written as:

Gross Measure Cost = Measure Equipment Cost +Measure Labor Cost

The table below gives the 1st baseline GMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as RET. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

Table 22 1st Baseline Gross Measure Refrigerator and Freezer Costs for RET

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | N/A | N/A | N/A | N/A |
| Solid-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 1,373.60 | $ 4,204.00 | $ 5,864.11 | $ 8,464.80 |
| Glass-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |

The table below gives the 2nd baseline GMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as RET. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

Table 23 2nd Baseline Gross Measure Refrigerator and Freezer Costs for RET

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | N/A | N/A | N/A | N/A |
| Solid-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 103.60 | $ 863.80 | $ 1,076.11 | $ 1,548.96 |
| Glass-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |

### 4.3.2 Incremental Measure Cost

For ROB measures, the Incremental Measure Cost is the difference between the Full Measure Cost and the Full Base Case Cost and it can be symbolically written as:

Incremental Measure Cost = Full Measure Cost – Full Base Case Cost

Given that the installation labor cost is expected to be the same for the Measure and Base Case, the Incremental Measure Cost can be rewritten as:

Incremental Measure Cost = Measure Equipment Cost – Base Case Equipment Cost

For ROB measures, the Incremental Measure Cost (IMC) is equal to the Gross Measure Cost (GMC) as shown in Table 21.

For the RET measures, the incremental cost is represented by the equation below:

IMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

*Note: Unless stated otherwise the measure case labor and base case labor are assumed to be the same value reducing the equation to the following:

IMC 	= Measure Equipment Cost – Base Case Equipment Cost

Table 23 below gives the incremental cost for solid- and glass-door, reach-in refrigerators and freezers claimed as RET. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

# Attachments

- Reach-In Refrigerator and Freezer Savings

- Energy Star 2.0 Requirements

- Cost Calculation References

- Reach-In Savings Calculations

# References

[31]

[351]

[355]

[422]

[436]

[A] Attachment 2-Energy Star 2.0 Requirements

[B] Attachment 1-Reach-In Refrigerator and Freezer Calculations

[C] Attachment 3-Cost Calculation References

[D] Attachment 4-Reach-In Savings Calculations

[E] CPUC California Standard Practice Manual: Economic Analysis of Demand-Side Programs and Projects, page 18. California Public Utilities Commission, October 2001.

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |