Work Paper SCE13RN008

Revision 1

Southern California Edison Company

New Low and Medium Temperature Display Cases with Doors

# At-a-Glance Summary

| Applicable Measure Codes: | RF-68765, RF-75389 |
| --- | --- |
| Measure Description: | New refrigeration display case with doors, low or medium temperature |
| Base Case Description: | Existing refrigeration display case without doors, low or medium temperature |
| Energy Impact Common Units: | Linear Feet of Case |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 12 years (EUL ID: GrocDisp-FixtDoors) |
| Measure Application Type: | Replace-on-Burnout (ROB) |
| Net-to-Gross Ratios: | 0.6 (NTG ID: Com-Default>2yrs) 0.85 (NTG ID: Com-Default-HTR-di) |
| Important Comments: | - This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13RN008.0 | No | 05/11/2012 | Yin Yin Wu/BASE Energy, Inc. Chris Fernandez/BASE Energy, Inc. | This is the original work paper for the bridge cycle 2013-2014 |
| SCE13RN008.1 | Yes | 4/17/2014 | Thomas Tseng (SCE) Jason Wang (SCE) Jason Ochs, EIT (PECI) Jim Wyatt (PG&E) | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. - Combined SCE and PG&E work papers. - Incorporated 2012 Federal Code Updates for Refrigerated Case Baselines. - Changed install type to ROB |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is a reach-in display case, either low or medium temperature (LT or MT), with glass doors, low or no anti-sweat heating (ASH), electronically commutated fan motors (ECMs), and LED lighting.

The base case is an existing open vertical or multi-deck display case, either LT or MT, without doors.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| RF-68765 | Medium Temperature Reach-in Display Case |
| RF-75389 | Low Temperature Reach-in Display Case |

Measure Requirements

- The new display case must have:

  - Double-pane glass doors with heat-reflective treatment or gas fill

  - ECM fan motors (evaporator and/or condenser)

  - LED lighting

  - Low or no ASH:  The need for ASH depends on the display case characteristics, operating conditions, and ambient conditions. If ASH is used, ASH controls are recommended.

    - For MT cases, heat is allowed in the frame but not the door.

    - For LT cases, heat is allowed in both the frame and the door, but total heating power must be ≤ 50 W/ft of case length.

- The new display case length must be equal to or shorter than the original display case length.

- Self-contained and remote cases are eligible.

- Specialty deli cases, custom coolers/freezers, and walk-in boxes with reach-in doors are not eligible

- Display case replacements that are part of large-scale store remodels and any new construction projects should be revised to be custom measures. Large-scale remodels are defined as any project involving 50% of the linear feet of refrigerated casework or 32 linear feet of casework replacements, whichever is less.

## 1.2 Technical Description

This work paper defines a LT case as maintaining an evaporator temperature between -40°F and 0°F, and a MT case as maintaining an evaporator temperature between 0°F and 35°F. These types of display cases can be found in grocery stores. Medium temperature display cases are used to stock dairy, deli, fish, and meat. Low-temperature display cases are used to stock frozen food and ice cream.

The main benefit of this measure is the reduction in total cooling load due to reduced air infiltration. The infiltration load refers to the transfer of warm and moist air from the sales area to the refrigerated space. The infiltration load accounts for more than 80% of the total cooling load of a multi-deck display case [459]. Therefore, any reduction in infiltration load can significantly reduce refrigeration compressor power demand and energy usage. It should be noted that installing cases with glass doors may increase the anti-sweat heater (ASH) load.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The delivery methods are:

- Financial Support - Down Stream Incentive – Deemed

- Financial Support – On-bill Finance – loan

- Financial Support – Direct Install

- Partnership – Down-Stream Incentive – Deemed

- Partnership – On-bill Finance – Loan

The application type is Replace-on-Burnout (ROB)

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

DEER 2014 includes legacy measure D03-207, which replaces existing open MT multi-deck cases with new MT multi-deck cases with glass doors, ECM motors, and T8 (with electronic ballast) lighting. The DEER savings are not used for several reasons:

- Reach-in display cases available on the market today have higher efficiencies than what is modeled in the DEER 2005 prototype for D03-207. Features such as LED lighting, low/no ASH, and ECM motors are becoming standard and in some cases required by Federal Appliance Standards. Therefore, most of the energy savings for this measure are from using doors to prevent infiltration loss. The magnitude of infiltration reduction in D03-207 aligns with analyses found in recent studies [446].

- Power densities for lighting, ASH, and fans in D03-207 do not align with current practice and the 2012 Federal Efficiency Standards. In fact, the D03-207 model has higher lighting and ASH power densities in the measure case that in the base case. This is not a mistake; in 2005 when the measure was initially created, additional fluorescent case lights were installed to keep the product illuminated and heat was run through the doors and frames to prevent condensation and frost build up. Since then, and in large part driven by the Federal Standards, LED lighting has become economically viable in new cases, resulting in better illumination of products while at the same time using approximately the same, if not less, power. ASH can also be eliminated through the use of more advanced doors and frames. Each of these components directly draws power as well as indirectly interacts with compressor power consumption by contributing heat to the refrigerated space.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | Yes (Modified) |
| Deviation from DEER | DEER prototypes were modified in order to reflect current code requirements where applicable |
| DEER Version | DEER 2014 |
| DEER Run ID and Measure Name (Sample) | D03-207 |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID* | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

*Denotes that the column is taken from the DEER GSIA Table.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Commercial Refrigeration |
| Measure Case UseSubCats | Refrigerated Display |
| Measure Case TechGroups | Refrigerated Storage |
| Measure Case TechTypes | Vertical (multi-deck) Display Fixture |
| Base Case TechGroups | Refrigerated Storage |
| Base Case TechTypes | Vertical (multi-deck) Display Fixture |

### 1.4.2 Codes and Standards Analysis

Title 20 (2014) [422]

This measure does not fall under Title 20 of the California Energy Regulations. Title 20, Section 1605.3 covers new appliances sold or offered for sale in California, but does not apply to vertical open refrigerated display cases.

Title 24 (2013) [355]

Title 24 Section 120.6(b)3 requires controls on display case lighting.

Federal Standards [393]

Federal Standard 10 CFR §431.66 provides requirements for maximum daily energy consumption for MT and LT display cases, with or without doors (VCT or VOP). This standard does not prescribe requirements for specific case components such as evaporator fan motors, case lighting, insulation, evaporator coil, or air curtain.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| FS 10 CFR | §431.66 | Current as of 7/29/14 |
| Title 24 (2013) | Section 120.6(b)3 | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

Numerous studies have been conducted on the energy savings associated with the measures described in this paper. One commonly cited study conducted by researchers at Southern California Edison’s Research and Thermal Test Center found that after retrofitting doors to an open display case the cooling load attributable to infiltration was reduced by 68%. This conclusion was based on lab testing conducted with static ambient temperature and humidity levels [446].

Another report based on lab testing concluded that the cooling load of open cases decreased 66% versus cases without doors or night covers and 53% when compared to an open case with night covers in place at night. The same report discussed the results of a field trial that measured savings of 26% for the store’s entire refrigeration system when several medium and low temperature cases were retrofitted with doors. Though this study focuses on a door retrofit the principles of load reduction are the same [447]. Case replacements as described in this workpaper also include component upgrades for lighting and motors.

The American Society of Heating, Refrigerating, and Air-Conditioning Engineers (ASHRAE) paid for a research project (RP-1402) to compare energy and sales results of open multi-deck vs. reach-in case for medium temperature applications. The display cases tested held alcoholic beverages and dairy and were located in 2 stores in Kansas. The report summary showed an 18% reduction in energy, using calculated compressor savings and measured refrigeration load [450].

The State of California Air Resources Board has estimated that adding doors to all viable open vertical refrigerated display cases in the state would reduce energy consumption by more than 1,000 GWH, or 5% of the total energy consumption attributed to the grocery sector [451].

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| GrocDisp-FixtDoors | Non-Residential | ComRefrig | New case with Doors | 12 | 4 |

# Section 2. Energy Savings & Demand Reduction Calculations

## 2.1 Electric Energy Savings Estimation Methodologies

The following assumptions were made for the calculations of this work paper:

- The building simulation models were generated from the DEER measure analysis tool within eQUEST 3-61R. The DEER model simulates a grocery store with a multiplex-compressor system for the refrigerated display cases. Since single-compressor systems are generally slightly less efficient than multiplex-compressor systems, only multiplex systems have been analyzed as a conservative estimate of savings. To be conservative, it is assumed that the generated energy savings of this work paper will also be applied to display cases with single-compressor systems.

- This work paper applies to display cases located within a space with heating, cooling, and humidity controls. The unit energy savings are represented per linear-foot of the display case. The building simulation models were generated for a grocery store. Since the heat gain to a display case mainly depends on the temperature maintained for the display case and the surrounding space temperature, it is assumed that the building types would not have significant impact on the energy savings. Thus, the resulting savings of the grocery store model is applied to all other building types considered in this work paper.

Simulation Assumptions

The DEER grocery store prototype, vintage 2002-2005, was used as the starting point for the simulations in this work paper. Since the prototype did not exactly match the measures in this work paper, several adjustments were made:

- Spec sheets for several open and closed MT and LT display cases were studied to help develop appropriate baselines and measure cases. See Attachment 8 for the spec sheets and Attachment 2 for a summary.

- To better align with current grocery store characteristics, humidity controls were implemented into the model. The humidity set point was adjusted to 45% to match industry standard practice and more meaningfully simulate HVAC interactions [460].

- MT baseline: The DEER prototype includes open multi-deck MT cases. These were updated to reflect 2012 Federal Code requirements so they could be used as the baseline. Based on the spec sheets, the MT baseline has a mixture of LED and T8 lighting, has ECM fan motors, and uses no ASH in either door or frame.

- MT measure: The MT measure case has LED lighting, ECM fan motors, and ASH in the frame only. Based on industry practice, additional case lighting was incorporated to improve product visibility.

- LT baseline: The LT cases in the DEER prototype already have doors. Therefore, they were modified “backward” into 2012 Federal Code-compliant open LT cases. The LT baseline has LED lighting, ECM fan motors, and ASH in the frame only.

- LT measure: The LT measure case has LED lighting, ECM fan motors, and ASH in both the door and frame.

Several key input parameters are shown in Table 8; note that these values are averages across the different types of cases in a grocery store. Also note that the units are different for the base and measure cases (kW/ft and kW/door). See Attachment 5 for a full list of eQUEST input parameters.

Table 8 Average Energy Modeling Input Parameters

| Component | Medium Temperature |  | Low Temperature |  |
| --- | --- | --- | --- | --- |
|  | Baseline: Open Multi Deck | Measure: Reach in Multi Deck | Baseline: Open Multi Deck | Measure: Reach in Multi Deck |
| Evaporator Fan Power | 0.0147 (kW/ft) | 0.0150 (kW/dr) | 0.01788 (kW/ft) | 0.01976 (kW/dr) |
| Lighting Power | 0.0369 (kW/ft) | 0.02810 (kW/dr) | 0.0179 (kW/ft) | 0.0478 (kW/dr) |
| Anti-Sweat Power | 0.0000 (kW/ft) | 0.01783 (kW/dr) | 0.04525 (kW/ft) | 0.09568 (kW/dr) |
| Infiltration Load | 1058 (Btu/hr-ft) | 153.33 (Btu/hr-dr) | 1550 (Btu/hr-ft) | 65.84 (Btu/hr-dr) |
| Conduction Load | 77.8 (Btu/hr-ft) | 273.33 (Btu/hr-dr) | 325 (Btu/hr-ft) | 600 (Btu/hr-dr) |
| Total Line-Up Length | 128 (ft) | 50 (doors) | 283 (ft) | 109 (doors) |

Savings Calculations

Once simulations were completed and energy consumption (electricity and natural gas) for all cases were tabulated, the total energy savings were calculated. These calculations are in Attachments 6 “LT Results” and 7 “MT Results”. The unit energy savings, in kWh/yr-ft for electricity and therm/yr-ft for natural gas, were calculated by dividing the total energy savings by the total line-up length of the display cases. Dividing the length of cases by the number of doors in the DEER grocery prototype yielded an approximate unit door length of 2.58 ft per door.

## 2.2. Demand Reduction Estimation Methodologies

By utilizing eQUEST’s hourly report function, the average peak demand outputs were calculated based on the CPUC defined peak demand periods specific to each climate zone. The peak hours are in Attachment 9 “DEER 2014 Peak Demand Hours by Climate Zone” and a few examples are in Table 9. Similar to the energy savings, the unit demand reduction, in kW/ft, was calculated by dividing the total demand reduction by the total line-up length of the display cases. All simulations were run using weather files adopted by the 2013 update of Title 24. All calculations are in Attachments 6 and 7.

The DEER peak period is based on the following criteria:

- Occurs between June 1st and September 30th

- Does not include weekends or holidays

- Has the highest value for

  - Average temperature over the three day period

  - The average temperature from noon to 6 p.m. over the three day period

  - The peak temperature over the three day period

Table 9 Hour Specifications for Peak Demand Calculations

| CZ | Start Month | Start Day | End Month | End Day | Start Hour | End Hour |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 9 | 16 | 9 | 18 | 15 | 17 |
| 2 | 7 | 8 | 7 | 10 | 15 | 17 |
| 3 | 7 | 8 | 7 | 10 | 15 | 17 |
| 4 | 9 | 1 | 9 | 3 | 15 | 17 |
| 5 | 9 | 8 | 9 | 10 | 15 | 17 |
| 6 | 9 | 1 | 9 | 3 | 15 | 17 |

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the Refrigeration load shape.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Assembly | Assembly | Refrigeration |
| Grocery | Grocery_Store | Refrigeration |
| Food Store | Food_Store | Refrigeration |
| Restaurant - Fast-Food | Fast_Food_Restaurant | Refrigeration |
| Restaurant - Sit-Down | Sit_Down_Restaurant | Refrigeration |
| Retail - Multistory Large | Retail - Multistory Large | Refrigeration |
| Retail - Single-Story Large | Retail - Single-Story Large | Refrigeration |
| Retail – Small | Retail - Small | Refrigeration |

# Section 4. Base Case & Measure Costs

In the following section, various price fluctuations are not addressed in the equations, such as future costs due to inflation in labor, future costs due to deflation in material cost, and other variables that cannot be accurately described at this time.

## 4.1 Base Case Cost

Base case costs were compiled using the Federal energy standards life cycle analysis data (Attachment 3). See Table 11.

## 4.2 Measure Case Cost

Measure case costs were compiled using the Federal energy standards life cycle analysis data (Attachment 3). See Table 11.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB measures, the Gross Measure Cost (GMC) is represented by the equation below:

GMC = (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

Since the measure and base case labor are the same, the equation reduces to the following:

GMC = Measure Equipment Cost – Base Case Equipment Cost

Table 11 GMC and IMC

| Measure | Base Case Equipment Cost | Measure Case Equipment Cost | Base and Measure Labor Cost | GMC/IMC |
| --- | --- | --- | --- | --- |
| Medium Temperature Reach-in Display Case | $593.82 | $915.77 | $206.33 | $321.95 |
| Low Temperature Reach-in Display Case | $837.05 | $1,048.32 | $206.33 | $211.27 |

*All costs noted as $ per measure unit

**For Cost Data See Attachment 3“Cost Information”

### 4.3.2 Incremental Measure Cost

For ROB measures, the Incremental Measure Cost (IMC) is the same as the GMC. See IMCs in Table 11.
Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [393] | Code of Federal Regulations Title 10 - Energy |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |
| [436] | 2014 DEER EUL Table - Update |
| [446] | Performance and Energy Impact of Installing Glass Doors on an Open Vertical Deli/Dairy Display Case |
| [447] | Vertical Display Case Cabinets without and with Doors |
| [450] | Comparison of Vertical Display Cases: Energy and Productivity Impacts of Glass Doors Versus Open Vertical Display Cases |
| [451] | Inventory of Direct and Indirect GHG Emissions from Stationary Air Conditioning and Refrigeration Sources, with Special Emphasis on Retail Food Refrigeration and Unitary Air Conditioning |
| [459] | ASHRAE Refrigeration Fundamentals |
| [460] | Isolated Sub-Dehumidification Strategies in Large Supermarkets and Grocery Stores |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |