Work Paper PGECOFST100

#### Commercial Combination Oven

Revision # 5

Pacific Gas & Electric Company

Customer Energy Solutions

Commercial Combination Oven-Electric and Gas

Measure Codes F100, F101

# At a Glance Summary — Electric Combination Ovens

| Applicable Measure Codes: | HA16 | F100 | HA19 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Combination Oven 16-28 pan capacity (Electric) | Commercial Combination Oven < 15 pan capacity (Electric) | Commercial Combination Oven > 29 pan capacity (Electric) |
| Energy Impact Common Units: | Combination Oven | Combination Oven | Combination Oven |
| Base Case Description: | Source: PG&E Calculations Existing Electric Combination Oven | Source: PG&E Calculations Existing Electric Combination Oven | Source: PG&E Calculations Existing Electric Combination Oven |
| Base Case Energy Consumption: | Source: PG&E Calculations 30,879 kWh/yr | Source: PG&E Calculations 39,347 kWh/yr | Source: PG&E Calculations 60,590 kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations 19,382 kWh/yr | Source: PG&E Calculations 24,273 kWh/yr | Source: PG&E Calculations 38,581 kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations 11,497 kWh/yr | Source: PG&E Calculations 15,074 kWh/yr | Source: PG&E Calculations 22,009 kWh/yr |
| Costs Common Units: | Source: PG&E Calculations Combination Oven | Source: PG&E Calculations Combination Oven | Source: PG&E Calculations Combination Oven |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations $9,137 | Source: PG&E Calculations $15,024 | Source: PG&E Calculations $19,610 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations $10,705 | Source: PG&E Calculations $16,608 | Source: PG&E Calculations $26,658 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations $1,568 | Source: PG&E Calculations $1,584 | Source: PG&E Calculations $7,048 |
| Effective Useful Life (years): | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL |
| Program Type: | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). |
| Net-to-Gross Ratios: | Source: 2014 DEER Com-Default>2yrs | Source: 2014 DEER Com-Default>2yrs | Source: 2014 DEERCom-Default>2yrs |
| Important Comments: |  |  |  |

# At a Glance Summary – Gas Convection Ovens

| Applicable Measure Codes: | HA48 | F101 | HA49 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Combination Oven 16-28 pan capacity (Gas) | Commercial Combination Oven < 15 pan capacity (Gas) | Commercial Combination Oven > 29 pan capacity (Gas) |
| Energy Impact Common Units: | Combination Oven | Combination Oven | Combination Oven |
| Base Case Description: | Source: PG&E Calculations Existing Gas Combination Oven | Source: PG&E Calculations Existing Gas Combination Oven | Source: PG&E Calculations Existing Gas Combination Oven |
| Base Case Energy Consumption: | Source: PG&E Calculations 1,572 Therms/yr | Source: PG&E Calculations 2,087 Therms/yr | Source: PG&E Calculations 3,134 Therms/yr |
| Measure Energy Consumption: | Source: PG&E Calculations 774 Therms/yr | Source: PG&E Calculations 967 Therms/yr | Source: PG&E Calculations 1,561 Therms/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations 798 Therms/yr | Source: PG&E Calculations 1,120 Therms/yr | Source: PG&E Calculations 1,573 Therms/yr |
| Costs Common Units: | Source: PG&E Calculations Combination Oven | Source: PG&E Calculations Combination Oven | Source: PG&E Calculations Combination Oven |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations $11,157 | Source: PG&E Calculations $16,807 | Source: PG&E Calculations $22,730 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations $14,746 | Source: PG&E Calculations $20,168 | Source: PG&E Calculations $30,620 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations $3,588 | Source: PG&E Calculations $3,361 | Source: PG&E Calculations $7,890 |
| Effective Useful Life (years): | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL |
| Program Type: | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). |
| Net-to-Gross Ratios: | Source: 2014 DEER Com-Default>2yrs | Source: 2014 DEER Com-Default>2yrs | Source: 2014 DEER Com-Default>2yrs |
| Important Comments: |  |  |  |

|  |  |  |  |  |  |  |  | 1st Baseline |  |  |  |  |  |  |  | 2nd Baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Code | Version Source | Measure Description | Measure Application Type | Building Type | Building Vintage | Climate Zone | Unit Definition | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | (EUL) LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | GRR_ kW | GRR_ kWh | GRR_ therm | NTG | Implementation Method [DI, DD, I] | ISR |
| HA16 |  | Commercial Combination Oven 16-28 pan capacity (Electric) | ROB, NC | ANY | ANY | ANY | each | 2.7 | 11,497 | 0 | 12 | 9,137 | 10,705 | 0 | 1,568 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |
| F100 |  | Commercial Combination Oven < 15 pan capacity (Electric) | ROB, NC | ANY | ANY | ANY | each | 3.5 | 15,074 | 0 | 12 | 15,024 | 16,608 | 0 | 1,584 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |
| HA19 |  | Commercial Combination Oven > 29 pan capacity (Electric) | ROB, NC | ANY | ANY | ANY | each | 5.0 | 22,009 | 0 | 12 | 19,610 | 26,658 | 0 | 7,048 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |
| HA48 |  | Commercial Combination Oven 16-28 pan capacity (Gas) | ROB, NC | ANY | ANY | ANY | each | 0 | 0 | 798 | 12 | 11,157 | 14,746 | 0 | 3,588 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |
| F101 |  | Commercial Combination Oven < 15 pan capacity (Gas) | ROB, NC | ANY | ANY | ANY | each | 0 | 0 | 1,120 | 12 | 16,807 | 20,168 | 0 | 3,361 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |
| HA49 |  | Commercial Combination Oven > 29 pan capacity (Gas) | ROB, NC | ANY | ANY | ANY | each | 0 | 0 | 1,573 | 12 | 22,730 | 30,620 | 0 | 7,890 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | 1 | 1 | 0.60 | I | 1 |

# At-A-Glance Measure List: Combination Ovens

# Work Paper Approvals

| Grant Brohard Manager, Technical Product Support | Date |
| --- | --- |
| Carol Weiner Manager, Appliance Products | Date |

# Document Revision History

Revision # 		Date 		         Description		         Author (Company)

| Revision 0 | 12/11/2007 | Original work paper -- Commercial Combination Ovens PGECOFST100 R0.doc | David Zabrowski (Fisher-Nickel, inc.) |
| --- | --- | --- | --- |
| Revision 1 | 6/1/09 | Changes to EUL, NTG language and references, costs updated | David Zabrowski, Lauren Mills (Fisher-Nickel, inc.), Steve Blanc PG&E |
| Revision 2 | 2/10/2010 | Update to DEER 2009-11 NTG file | David Zabrowski (Fisher-Nickel, inc.), Steve Blanc PG&E |
| Revision 3 | 7/15/2010 | Update language for NTG and EUL per ED comments | Charlene Spoor (PG&E) |
| Revision 4 | 5/31/2012 8/22/2012 | Updated NTG, EUL, IMC, and savings analysis Broke down savings for different sizes for SCG/SCE Updated BLD, CZ and VIN to ANY per READI requirements | David Zabrowski (Fisher-Nickel, inc.) Charlene Spoor (PG&E) |
| Revision 5 | 3/13/14 | Updated format for July 1, 2014 submittal. Updated measure codes as needed. | Charlene Spoor CLCi (PG&E) |

# Table of Contents

At a Glance Summary — Electric Combination Ovens	ii

At a Glance Summary – Gas Convection Ovens	iii

At-A-Glance Measure List: Combination Ovens	iv

Work Paper Approvals	v

Document Revision History	vi

Table of Contents	vii

List of Tables	viii

Section 1. General Measure & Baseline Data	1

1.1 Product Measure Description & Background	1

1.2 Product Technical Description	2

1.3 Measure Application Types	2

1.4 Product Base Case and Measure Case Data	3

1.4.1 DEER Base Case and Measure Case Information	3

1.4.2 Codes & Standards Requirements Base Case and Measure Information	4

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	5

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	5

1.4.5 Time of use Adjustment Factor	6

1.5 Summary of Inputs for Savings Calculations……………………………………………...6

Section 2. Calculation Methods	7

2.1 Electric Energy Savings Estimation Methodologies	7

2.2. Demand Reduction Estimation Methodologies	11

2.3. Gas Energy Savings Estimation Methodologies	11

Section 3. Load Shapes	15

3.1 Base Case Load Shapes	15

3.2 Measure Load Shapes	15

Section 4. Base Case & Measure Case Costs……………………………………………………16

4.1 Base Cases Costs	16

4.2 Measure Costs	16

4.3 Incremental & Full Measure Costs	16

Index……………………………………………………………………………………………..17

Appendix…………………………………………………………………………………………18

References	21

# List of Tables

Table 1. Idle Rate Requirements for Commercial Combination Oven/Steamers.	1

Table 2. Measure Application Type	1

Table 3. DEER Use and Technology Table……………………………………………………….3

Table 4. DEER 2014 Net-to-Gross Ratios	3

Table 5. DEER 2014 Effective Useful Life	.4

Table 6. Baseline ASTM Test Results for Commercial Combination Oven	6

Table 7. Measure Case ASTM Test Results for Commercial Combination Ovens	6

Table 8. Summary of Inputs for Savings Calculations	7

Table 9. Commercial 12-Pan Electric Combination Oven Cost Effectiveness Example.	8

Table 10. Commercial 20-Pan Electric Combination Oven Cost Effectiveness Example.	9

Table 11.Commercial 40-Pan Electric Combination Oven Cost Effectiveness Example	10

Table 12. Commercial 12-Pan Gas Combination Oven Cost Effectiveness Example…………..12

Table 13. Commercial 20-Pan Gas Combination Oven Cost Effectiveness Example…………..13

Table 14. Commercial 40-Pan Gas Combination Oven Cost Effectiveness Example…………..14

# Section 1. General Measure & Baseline Data

## 1.1 Product Measure Description & Background

This work paper documents the rationale for the Energy Efficient Commercial Combination Oven (Electric and Gas) measures as listed in the Commercial Food Service Catalog. The Commercial Food Service Catalog is part of Pacific Gas and Electric Company’s Customer Energy Efficiency Program. PG&E offers incentives to non-residential customers for installing qualifying lighting, refrigeration, air-conditioning, food service, and agricultural equipment.

Catalog Description –

HA16, F100, and HA19: The electric combination oven/steamer must have a tested steam mode cooking energy efficiency of ≥50% and convection mode cooking energy efficiency of ≥70% utilizing American Society for Testing and Materials (ASTM) Standard F2861 and meet the idle rate requirements in Table 1.

HA48, F101, and HA49: The gas combination oven/steamer must have a tested steam mode cooking energy efficiency of ≥38% and convection mode cooking energy efficiency of ≥44% utilizing American Society for Testing and Materials (ASTM) Standard F2861, and meet the idle rate requirements in Table 1.

SCE 1, SCE 2, SCG 1, SCG2: Southern California Gas and Electric will be further separating out the measure sizes for their programs.

Program Restrictions and Guidelines

Terms and Conditions

This measure includes new commercial electric or gas combination ovens that meet the qualifications listed in Table 1. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance has a cooking-energy efficiency that meets the requirements in Table 1.

F100 and F101:  This rebate is downstream provided to the customer at the time of sale upon receipt of application and invoice.  This is not a direct install program.

Table 1. Idle Rate Requirements for Commercial Combination Oven/Steamers.

| Measure Code | Combi Oven Type | Steam Mode Idle Energy Rate | Convection Mode Idle Energy Rate |
| --- | --- | --- | --- |
| F100 | Electric Combi <15 pan capacity* | ≤ 5.0 kW | ≤ 2.0 kW |
| HA16 | Electric Combi 16−28 pan capacity* | ≤ 6.0 kW | ≤ 2.5 kW |
| HA19 | Electric Combi >28 pan capacity* | ≤ 9.0 kW | ≤ 4.0 kW |
| F101 | Gas Combi < 15 pan capacity* | ≤ 15,000 Btu/h | ≤ 8,000 Btu/h |
| HA48 | Gas Combi 16−28 pan capacity* | ≤ 18,000 Btu/h | ≤ 10,000 Btu/h |
| HA49 | Gas Combi > 28 pan capacity* | ≤ 28,000 Btu/h | ≤ 16,000 Btu/h |

*Combination oven/steamer pan capacity on based on the maximum capacity of full-size 2 ½-inch deep hotel pans. This must be consistent with the number of pans used to meet the energy-efficiency qualifications per ASTM F2861.

Market Applicability

This measure is applicable to any commercial cooking application, including (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

## 1.2 Product Technical Description

An oven can be simply described as a fully enclosed, insulated chamber used to heat food. Commercial combination ovens offer even more options with their ability to add steam to the oven cavity. In addition to baking and roasting, a combination oven is also capable of steaming, proofing and reheating various food products. Foods can be cooked in a convection oven dry heat only mode, a steam only mode, and a combination of dry heat and steam modes. The programmability of combination ovens also allows food to be cooked partially in one mode at a certain temperature, and then finished in another mode and at a separate temperature. For example, a turkey can be cooked in combination mode at low temperature for several hours, and then stepped to a higher temperature in dry heat mode to finish. With competition rising among equipment manufacturers, new designs that incorporate timesaving features via sophisticated control packages are being introduced.

Combination ovens are available in a variety of sizes ranging from 6-pan countertop models to 40-pan roll-in models.  Combination oven sizes are based on the capacity to accommodate 12 x 20 x 21/2-inch hotel pans. Half-size models can accommodate one column of hotel pans and 9 x 13-inch (half-size) sheet pans, while full-size models can accommodate two columns of hotel pans and 18 x 26-inch (full-size) sheet pans.

Oven performance is determined by applying the ASTM Standard Test Method for the Performance of Combination Ovens in Various Modes (F2861). The ASTM standard test method is considered to be the industry standard for quantifying the energy consumption, efficiency and cooking performance of combination ovens.

## 1.3 Measure Application Types

Table 2. Measure Application Type

| Code | Description | Comment |
| --- | --- | --- |
| ER | Early retirement | measure applied while existing equipment still viable, or retrofit of existing equipment |
| ROB | Replace on Burnout | measure applied when existing equipment fails or maintenance requires replacement |
| NC | New Construction | measure applied during construction design phase as an alternative to a code-compliant standard design |

Since there are no EM&V studies on the useful life of combination ovens and it is standard practice in the commercial foodservice industry to purchase equipment only when it is needed (e.g., replacement or additional capacity), this measure is focused on ROB and NC applications only.

## 1.4 Product Base Case and Measure Case Data

### 1.4.1 DEER Base Case and Measure Case Information

The DEER database does not contain information on energy use or savings for an energy-efficient electric or gas combination oven measure. The only reference in DEER for Commercial cooking equipment is for Estimated Useful Life.

Table 3. DEER Use and Technology Table

- Hours of Operation

- This measure would follow the hours of operation for quick serve and full serve restaurants as noted in the ASTM standards.  For this measure annual hours of operation are considered 4380.

- 12 hrs /day * 365 day/ yr = 4380 hrs

- Base Case & Measure Case Costs

The base case and measure case costs are calculated an found in section 1.4.4.

Net-to-Gross Assumptions

The 2013-2014 DEER NTGR Values file does not specifically list commercial food service appliances. The default used for non-residential measures is 0.6.

Table 4 summarizes all applicable DEER based Net-to-Gross ratios for programs that may be used by this measure.

Table 4. DEER 2014 Net-to-Gross Ratios

| Program Approach | NTG |
| --- | --- |
| All Com-Default>=2yrs | 0.6 |

- Effective Useful Lives

The 2014 DEER database shows a EUL of 12 years and an RUL of 4 years for all cooking appliance measures, including electric and gas combination ovens.

Table 5. DEER Effective Useful Life

| Measure Code | Building Type | Building Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HA18/F100/HA19 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |
| HA48/F101/HA49 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |

In Service Rate/ First Year Installation Rate

- The in-service rate was not found in DEER or any supporting documentation. We have therefore assumed the ISR is 1.0 for all measures based on engineering judgment.

- See section 1.1 Terms and Conditions and Market Applicability to reference the type of program delivery mechanism and customer status used to determine this entry.

### 1.4.2 Codes & Standards Requirements Base Case and Measure Information

#### California Title 20

State of California Title 20 Appliance Efficiency Regulation has a category for cooking appliances, but combination ovens are not included.

#### California Title 24

There are no State of California Title 24 Efficiency Regulation requirements for combination ovens.

#### Federal

#### There are no Federal energy efficiency requirements for Combination Ovens.

#### ASTM Standards

Prior to 2010, the ASTM Standard Test Method for the Performance of Combination Ovens (F1639-05) was the benchmark for quantifying estimated energy use and cooking performance of combination ovens and was used to estimate the energy consumption of the base case and measure equipment.

Subsequent testing found that the cooking-energy efficiency test in F1639-05 may not be the most representative of combination oven energy usage. Furthermore, it was determined that the method could provide inconsistent results when comparing one model to another as it was originally based on first generation combination oven models, which have changed considerably since the last revision to the test method. The new test method for the enhanced performance of combination ovens in different operation modes examines the cooking performance in convection and steam modes, using a testing methodology that has been well established in the ASTM test methods for steam cookers (F1484) and convection ovens (F1496), while adapting the methodology to work specifically for combination ovens. This method was ratified by ASTM in 2010 as F2861-10. With F2861 effectively replacing the outdated F1639 for evaluating commercial combination ovens, ASTM plans to withdraw F1639 from active status in 2012.

In 2011, ASTM F2861-10 was used to estimate the energy consumption of base case and measure equipment.

### 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

There were no specific EM&V studies identified that addressed cooking measures in the commercial sector.

### 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

The Food Service Technology Center conducted an assessment of major commercial cooking appliance technologies in 2002, which included a chapter on ovens. The study showed that electric oven efficiency varies from 50 to 80% and gas oven efficiency varies from 10% to 50%, based on oven type (convection, combination, conventional, etc.).

Since the current Title 20 regulations do not include a minimum performance requirement for combi ovens, the base case for existing models of electric and gas combination ovens was determined from the Food Service Technology Center database of appliance performance.

This Work Paper uses ASTM Standard Test Method for the Performance of Combination Ovens in Various Modes (F2861) for calculation of energy use and demand, based on testing in an approved and qualified laboratory. In the absence of mandatory regulations for testing commercial combination ovens, there is little incentive on the part of equipment manufacturers to have their baseline equipment tested. Therefore, the ASTM performance parameters for baseline equipment were drawn from a sample of economy grade equipment tested by the Food Service Technology Center and is summarized in Table 6.

Table 6. Baseline ASTM Test Results for Commercial Combination Ovens.

| Combination Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Combi < 15 pan capacity* | 3,000 W oven mode 10,000 W steam mode | 65% oven mode 40% steam mode | 80 lb/h oven mode 100 lb/h steam mode |
| Electric Combi 15−28 pan capacity* | 3,750 W oven mode 12,500 W steam mode | 65% oven mode 40% steam mode | 100 lb/h oven mode 150 lb/h steam mode |
| Electric Combi > 28 pan capacity* | 5,250 W oven mode 18,000 W steam mode | 65% oven mode 40% steam mode | 275 lb/h oven mode 350 lb/h steam mode |
| Gas Combi < 15 pan capacity* | 15,000 Btu/h oven mode 45,000 Btu/h steam mode | 35% oven mode 20% steam mode | 80 lb/h oven mode 100 lb/h steam mode |
| Gas Combi 15−28 pan capacity* | 20,000 Btu/h oven mode 60,000 Btu/h steam mode | 35% oven mode 20% steam mode | 100 lb/h oven mode 150 lb/h steam mode |
| Gas Combi > 28 pan capacity* | 30,000 Btu/h oven mode 80,000 Btu/h steam mode | 35% oven mode 20% steam mode | 275 lb/h oven mode 350 lb/h steam mode |

*Based on the ASTM F2861.

The measure case data was drawn from data generated by The PG&E Food Service Technology Center in San Ramon, the Southern California Gas Company Foodservice Equipment Center in Downey and the Southern California Edison Foodservice Technology Center in Irwindale. The lab-based test data was used to establish a measure case level that effectively differentiated between standard-efficiency models and energy-efficient models. The performance parameters used to determine the energy consumption for the measure case is summarized in Table 7.

Table 7. Measure Case ASTM Test Results for Commercial Combination Ovens.

| Combination Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Combi < 15 pan capacity* | 2,000 W oven mode 5,000 W steam mode | 70% oven mode 50% steam mode | 100 lb/h oven mode 120 lb/h steam mode |
| Electric Combi 15−28 pan capacity* | 2,500 W oven mode 6,000 W steam mode | 70% oven mode 50% steam mode | 125 lb/h oven mode 200 lb/h steam mode |
| Electric Combi > 28 pan capacity* | 4,000 W oven mode 9,000 W steam mode | 70% oven mode 50% steam mode | 325 lb/h oven mode 400 lb/h steam mode |
| Gas Combi < 15 pan capacity* | 8,000 Btu/h oven mode 15,000 Btu/h steam mode | 44% oven mode 38% steam mode | 100 lb/h oven mode 120 lb/h steam mode |
| Gas Combi 15−28 pan capacity* | 10,000 Btu/h oven mode 18,000 Btu/h steam mode | 44% oven mode 38% steam mode | 125 lb/h oven mode 200 lb/h steam mode |
| Gas Combi > 28 pan capacity* | 16,000 Btu/h oven mode 28,000 Btu/h steam mode | 44% oven mode 38% steam mode | 325 lb/h oven mode 400 lb/h steam mode |

*Based on the ASTM F2861.

# 1.4.5 Time of use Adjustment Factor

The TOU adjustment factor for all non A/C measures is 0.

1.5 Summary of inputs for Savings Calculations

Table 8. Summary of Inputs for Savings Calculations

| Input Variable | Variations | Base Case 1 Average Value | Base Case 2 Average Value | Measure Case Average Value F100 | Measure Case Average Value F101 | Refer- ence Section |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Savings* | None | N/A | N/A | 15074 | N/A | N/A |
| Gas Savings* | None | N/A | N/A | N/A | 1120 | 1.4.4 |
| Hours of operation | None | 8,760 | N/A | 4380 | 4380 | 1.4.1 |
| Full Cost | None | $30.17 | N/A | 16608 | 20168 | 1.4.4 |
| Increment Cost | None | $5.58 | N/A | 1584 | 3361 | 1.4.4 |
| EUL /RUL | None | 12 | N/A | 12 | 12 | 1.4.1 |
| NTG | None | 0.6 | N/A | 0.6 | 0.6 | 1.4.1 |
| ISR | None | 1 | N/A | 1 | 1 | 1.4.1 |

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of combination ovens is ASTM Standard Test Method for the Performance of Combination ovens (F2861). Tables 9-11 show examples of the calculation results for electric combination ovens based on data obtained from applying the ASTM F2861 test method.

Table 9. Commercial 12-Pan Electric Combination Oven Cost Effectiveness Example

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 3.00 | 1.50 |
| Convection Idle Energy Rate (kW) | 3.00 | 2.00 |
| Convection Cooking Energy Efficiency (%) | 65% | 70% |
| Convection Production Capacity (lbs/hr) | 80 | 100 |
| Steam Idle Energy Rate (kW) | 10.00 | 5.00 |
| Steam Cooking Energy Efficiency (%) | 40% | 50% |
| Steam Production Capacity (lbs/hr) | 100 | 120 |
| Average Water Consumption Rate (gal/h) | 30 | 20 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 200 | 200 |
| Percentage Time in Steam Mode | 50% | 50% |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| ASTM Steam Mode Energy to Food (kWh/lb) | 0.0308 | 0.0308 |
| Daily Energy Consumption (kWh) | 84.6 | 53.1 |
| Average Demand (kW) | 7.1 | 4.4 |
| Estimated Demand Reduction (kW) | - | 2.7 |
| Annual Energy Consumption (kWh) | 30,879 | 19,382 |
| Estimated Energy Savings (kWh/yr) | - | 11,497 |
| Annual Water Consumption (gal) | 65,700 | 43,800 |
| Estimated Water Savings (gal) | - | 21,900 |
| Annual Energy Cost ($) | $4,014 | $2,520 |
| Estimated Cost Savings ($/yr) | - | $1,494 |
| Annual Water/Sewer Cost ($) a | $439 | $293 |
| Estimated Water/Sewer Cost Savings ($/yr) a | - | $146 |
| Incremental Measure Cost b | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) c | 12 years | 12 years |

a  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.  (1 CCF = 748 gallons)

b Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on 2011 DEER EUL estimates.

Table 10. Commercial 20-Pan Electric Combination Oven Cost Effectiveness Example

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 3.75 | 2.00 |
| Convection Idle Energy Rate (kW) | 3.75 | 2.50 |
| Convection Cooking Energy Efficiency (%) | 65% | 70% |
| Convection Production Capacity (lbs/hr) | 100 | 125 |
| Steam Idle Energy Rate (kW) | 12.50 | 6.00 |
| Steam Cooking Energy Efficiency (%) | 40% | 50% |
| Steam Production Capacity (lbs/hr) | 150 | 200 |
| Average Water Consumption Rate (gal/h) | 40 | 25 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 250 | 250 |
| Percentage Time in Steam Mode | 50% | 50% |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| ASTM Steam Mode Energy to Food (kWh/lb) | 0.0308 | 0.0308 |
| Daily Energy Consumption (kWh) | 107.8 | 66.5 |
| Average Demand (kW) | 9.0 | 5.5 |
| Estimated Demand Reduction (kW) | - | 3.5 |
| Annual Energy Consumption (kWh) | 39,347 | 24,273 |
| Estimated Energy Savings (kWh/yr) | - | 15,074 |
| Annual Water Consumption (gal) | 87,600 | 54,750 |
| Estimated Water Savings (gal) | - | 32,850 |
| Annual Energy Cost ($) | $5,115 | $3,155 |
| Estimated Cost Savings ($/yr) | - | $1,960 |
| Annual Water/Sewer Cost ($) a | $586 | $366 |
| Estimated Water/Sewer Cost Savings ($/yr) a | - | $220 |
| Incremental Measure Cost b | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) c | 12 years | 12 years |

a  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.  (1 CCF = 748 gallons)

b Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on 2011 DEER EUL estimates.

Table 11. Commercial 40-Pan Electric Combination Oven Cost Effectiveness Example

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 5.63 | 3.00 |
| Convection Idle Energy Rate (kW) | 5.25 | 4.00 |
| Convection Cooking Energy Efficiency (%) | 65% | 70% |
| Convection Production Capacity (lbs/hr) | 275 | 325 |
| Steam Idle Energy Rate (kW) | 18.00 | 9.00 |
| Steam Cooking Energy Efficiency (%) | 40% | 50% |
| Steam Production Capacity (lbs/hr) | 350 | 400 |
| Average Water Consumption Rate (gal/h) | 70 | 30 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 400 | 400 |
| Percentage Time in Steam Mode | 50% | 50% |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| ASTM Steam Mode Energy to Food (kWh/lb) | 0.0308 | 0.0308 |
| Daily Energy Consumption (kWh) | 166.0 | 105.7 |
| Average Demand (kW) | 13.8 | 8.8 |
| Estimated Demand Reduction (kW) | - | 5.0 |
| Annual Energy Consumption (kWh) | 60,590 | 38,581 |
| Estimated Energy Savings (kWh/yr) | - | 22,009 |
| Annual Water Consumption (gal) | 153,300 | 65,700 |
| Estimated Water Savings (gal) | - | 87,600 |
| Annual Energy Cost ($) | $7,877 | $5,016 |
| Estimated Cost Savings ($/yr) | - | $2,861 |
| Annual Water/Sewer Cost ($) a | $1,025 | $439 |
| Estimated Water/Sewer Cost Savings ($/yr) a | - | $586 |
| Incremental Measure Cost b | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) c | 12 years | 12 years |

a  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.  (1 CCF = 748 gallons)

b Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on 2011 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = Convection Mode((LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)]) + Steam Mode(lbfood*Efood/Efficiency + IdleRate*(ton - lbsfood/PC - tp)) + nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (kWh/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking based on the ASTM test method |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (kW) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (kWh) |

## 2.2. Demand Reduction Estimation Methodologies

A combination oven’s actual contribution to a building’s peak demand may vary significantly depending on its usage pattern in relation to that of other electric equipment in the facility (operating schedule, appliance on time, etc.). The probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance. Therefore, it has been assumed that the probable contribution to the building’s peak demand is equal to the appliance’s average demand.

The demand reduction estimation is based on measured data for standard efficiency electric combination ovens and for high-efficiency combination ovens. The measured data are derived from tests conducted under ASTM Standard Test Method for the Performance of Combination ovens (F2861).

ASTM F2861 provides standard conditions under which combination oven energy use is measured. The estimated demand reduction of 2.7 kW for models with less than 15 pan capacity, 3.5 kW for models with a pan capacity between 16 and 28 pans and 5.0 kW for models with a capacity greater than 29 pans is based on data from tests of standard efficiency and high efficiency electric combination ovens. Applying a Coincidence Factor of 0.9 per the DEER methodology yields a Demand Savings of 2.4 kW for models with less than 15 pan capacity, 3.2 kW models with a pan capacity between 16 and 28 pans and 4.5 kW for models with a capacity greater than 29 pans.

## 2.3. Gas Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of combination ovens is ASTM Standard Test Method for the Performance of Combination Ovens (F2861). Table 12-14 shows an example of the calculation results under ASTM F2861.

Table 12. Commercial 12-Pan Gas Combination Oven Cost Effectiveness Example.

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 18,000 | 13,000 |
| Convection Idle Energy Rate (Btu/h) | 15,000 | 8,000 |
| Convection Heavy Load Cooking Energy Efficiency (%) | 35% | 44% |
| Convection Production Capacity (lbs/hr) | 80 | 100 |
| Steam Idle Energy Rate (Btu/h) | 45,000 | 15,000 |
| Steam Cooking Energy Efficiency (%) | 20% | 38% |
| Steam Production Capacity (lbs/hr) | 100 | 120 |
| Average Water Consumption Rate (gal/h) | 30 | 20 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 200 | 200 |
| Percentage Time in Steam Mode | 50% | 50% |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (Btu/lb) | 250 | 250 |
| ASTM Steam Mode Energy to Food (Btu/lb) | 105 | 105 |
| Daily Energy Consumption (Btu) | 430,679 | 212,075 |
| Annual Energy Consumption (therms) a | 1,572 | 774 |
| Estimated Energy Savings (therms/yr) | - | 798 |
| Annual Water Consumption (gal) | 65,700 | 43,800 |
| Estimated Water Savings (gal) | - | 21,900 |
| Annual Energy Cost ($) | $1,572 | $774 |
| Estimated Cost Savings ($/yr) | - | $798 |
| Annual Water/Sewer Cost ($) b | $439 | $293 |
| Estimated Water/Sewer Cost Savings ($/yr) b | - | $146 |
| Incremental Measure Cost c | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) d | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.

(1 CCF = 748 gallons)

c Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

d. The estimated useful life is based on 2011 DEER EUL estimates.

Table 13. Commercial 20-Pan Gas Combination Oven Cost Effectiveness Example.

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 22,000 | 16,000 |
| Convection Idle Energy Rate (Btu/h) | 20,000 | 10,000 |
| Convection Heavy Load Cooking Energy Efficiency (%) | 35% | 44% |
| Convection Production Capacity (lbs/hr) | 100 | 125 |
| Steam Idle Energy Rate (Btu/h) | 60,000 | 18,000 |
| Steam Cooking Energy Efficiency (%) | 20% | 38% |
| Steam Production Capacity (lbs/hr) | 150 | 200 |
| Average Water Consumption Rate (gal/h) | 40 | 25 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 250 | 250 |
| Percentage Time in Steam Mode | 50% | 50% |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (Btu/lb) | 250 | 250 |
| ASTM Steam Mode Energy to Food (Btu/lb) | 105 | 105 |
| Daily Energy Consumption (Btu) | 571,911 | 264,812 |
| Annual Energy Consumption (therms) a | 2,087 | 967 |
| Estimated Energy Savings (therms/yr) | - | 1,120 |
| Annual Water Consumption (gal) | 87,600 | 54,750 |
| Estimated Water Savings (gal) | - | 32,850 |
| Annual Energy Cost ($) | $2,087 | $967 |
| Estimated Cost Savings ($/yr) | - | $1,120 |
| Annual Water/Sewer Cost ($) b | $586 | $366 |
| Estimated Water/Sewer Cost Savings ($/yr) b | - | $220 |
| Incremental Measure Cost c | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) d | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.

(1 CCF = 748 gallons)

c Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX A.

d. The estimated useful life is based on 2011 DEER EUL estimates

Table 14. Commercial 40-Pan Gas Combination Oven Cost Effectiveness Example.

| Performance | Baseline Model | Energy-Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 32,000 | 24,000 |
| Convection Idle Energy Rate (Btu/h) | 30,000 | 16,000 |
| Convection Heavy Load Cooking Energy Efficiency (%) | 35% | 44% |
| Convection Production Capacity (lbs/hr) | 275 | 325 |
| Steam Idle Energy Rate (Btu/h) | 80,000 | 28,000 |
| Steam Cooking Energy Efficiency (%) | 20% | 38% |
| Steam Production Capacity (lbs/hr) | 350 | 400 |
| Average Water Consumption Rate (gal/h) | 70 | 30 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 400 | 400 |
| Percentage Time in Steam Mode | 50% | 50% |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| Water/Sewer Cost ($/CCF) | $5.00 | $5.00 |
| ASTM Convection Mode Energy to Food (Btu/lb) | 250 | 250 |
| ASTM Steam Mode Energy to Food (Btu/lb) | 105 | 105 |
| Daily Energy Consumption (Btu) | 858,575 | 427,553 |
| Annual Energy Consumption (therms) a | 3,134 | 1,561 |
| Estimated Energy Savings (therms/yr) | - | 1,573 |
| Annual Water Consumption (gal) | 153,300 | 65,700 |
| Estimated Water Savings (gal) | - | 87,600 |
| Annual Energy Cost ($) | $3,134 | $1,561 |
| Estimated Cost Savings ($/yr) | - | $1,573 |
| Annual Water/Sewer Cost ($) b | $1,025 | $439 |
| Estimated Water/Sewer Cost Savings ($/yr) b | - | $586 |
| Incremental Measure Cost c | - | SEE APPENDIX A |
| Estimated Useful Life (EUL) d | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b  Water and wastewater cost are based on a rate of $2.00/CCF water and $3.00/CCF wastewater.

(1 CCF = 748 gallons)

c Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX A.

d. The estimated useful life is based on 2011 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = Convection((LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)]) + Steam((LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)])  
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (Btu/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (Btu/lb) = Btu/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (Btu/h) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (Btu) |

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Case Load Shapes

The base case load shape would be expected to follow a typical non-residential foodservice end use load shape.

Commercial combination oven load shapes  differ among food service facilities (quick service, casual dining, hotels, college, schools, hospitals, etc.) depending on daily menu variations, hours of operation, serving periods, day-of-week, and facility location (city downtown, suburban mall, access to interstate highways, etc.). Consequently, applicable average TOU and hourly load shapes for combination ovens are unavailable. The ASTM Standard Test Method used to generate energy use data is based on hours of use and operating state (preheat, idle, and heavy-load cooking). Generally, combination ovens are used to prepare food within a few days to a few hours before it is served, so loads tend to not necessarily be coincident with regular meal periods (breakfast, lunch, and dinner). Between meal periods combination ovens may be used to prepare ingredients for either the next meal period or for menu items to be served the next several days (in which case the ingredients are refrigerated immediately after cooking).

## 3.2 Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for this measure is determined by the E3 calculator based on the applicable non-residential market sector and the foodservice end-use.

The electric demand profile for the high-efficiency electric combination oven is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high-efficiency combination oven will use less energy and have a lower demand profile.

The gas load profile for the high efficiency gas combination oven is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high efficiency combination oven will use less energy.

Section 4. Base Case & Measure Costs

High-efficiency combination ovens typically have a higher list price than standard efficiency combination ovens. However, high-efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high-efficiency combination ovens exhibit better uniformity and higher production rates that increase their cost-effectiveness.

Equipment prices for these work papers were compiled from a number of sources including, Autoquotes, equipment sales reps and manufacturer sources. Since equipment pricing in food service is closely held information and prices vary widely according to buying volume and other factors, we cannot list the sources for prices specifically.

## 4.1 Base Cases Costs

The Base Case costs include only the equipment. High efficiency combination ovens require no additional labor or maintenance compared to base case combination ovens. Since this measure is applicable for ROB and NEW installations, the installation and maintenance costs are expected to be the same for the customer. The estimated equipment costs for both Base and Measure cases are based on recent list cost data for electric and gas combination ovens and applying an industry-standard 50% discount to the manufacturer published list prices.

## 4.2 Measure Costs

The Measure costs include only the equipment, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data (see Appendix A).

## 4.3 Incremental & Full Measure Costs

Incremental measure costs are used in the analysis.

Index

DEER Measure Cost Data Users Guide, 4

load shape, 17, 18

Measure Load Shape, 18

New Construction, ii, iii, 4

Replace on Burnout, ii, iii, 4

load shape, 15

Measure Load Shape, 15, 16

New Construction, ii, iii

Replace on Burnout, ii, iii

Appendix A

Equipment Cost Data for Electric Combination Ovens

| Group | Make | Model | Hotel Pan Capacity | List Price ($) | Cost ($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | BKI | C 1.06 | 6 | $18,200 | $9,100 |
| Baseline | BKI | C 2.10 | 20 | $35,984 | $17,992 |
| Baseline | BKI | K 1.06 | 6 | $20,720 | $10,360 |
| Baseline | Blodgett | BCM-101 | 10 | $24,900 | $12,450 |
| Baseline | Blodgett | BCM-102 | 20 | $30,000 | $15,000 |
| Baseline | Blodgett | BCP-102 | 20 | $33,000 | $16,500 |
| Baseline | Blodgett | BCP-61 | 6 | $21,000 | $10,500 |
| Baseline | Electrolux | 102 | 20 | $33,200 | $16,600 |
| Baseline | Eloma | G 10-11E | 10 | $19,950 | $9,975 |
| Baseline | Eloma | G 6-11E | 6 | $15,200 | $7,600 |
| Baseline | Henny Penny | ECC-615 | 6 | $17,708 | $8,854 |
| Baseline | Piper | HME 102P | 20 | $27,240 | $13,620 |
| Baseline | Piper | HME102X | 20 | $33,447 | $16,724 |
| Baseline | Rational | CM 101 | 10 | $14,130 | $7,065 |
| Baseline | Rational | CM 102 | 20 | $19,610 | $9,805 |
| Baseline | Rational | CM 201 | 20 | $27,900 | $13,950 |
| Baseline | Rational | CM 202 | 40 | $39,220 | $19,610 |
| Baseline | Rational | CM 61 | 6 | $9,450 | $4,725 |
| Baseline | Rational | CM 62 | 12 | $15,090 | $7,545 |
| Baseline | Vulcan | CEM10U | 10 | $24,382 | $12,191 |
| Baseline | Vulcan | CEM6 | 6 | $17,992 | $8,996 |
| Energy Efficient | Alto Shaam | 12.20 ES | 20 | $28,660 | $14,330 |
| Energy Efficient | Alto Shaam | 20.20 ES | 40 | $42,750 | $21,375 |
| Energy Efficient | Alto Shaam | 7.14 ES | 14 | $17,063 | $8,532 |
| Energy Efficient | Cleveland | OEB 10.20 | 20 | $32,400 | $16,200 |
| Energy Efficient | Cleveland | OEB 12.20 | 24 | $41,890 | $20,945 |
| Energy Efficient | Cleveland | OEB 20.20 | 40 | $62,830 | $31,415 |
| Energy Efficient | Cleveland | OES 10.10 | 10 | $25,170 | $12,585 |
| Energy Efficient | Cleveland | OES 10.20 | 20 | $31,480 | $15,740 |
| Energy Efficient | Cleveland | OES 12.20 | 24 | $40,630 | $20,315 |
| Energy Efficient | Cleveland | OES 20.20 | 40 | $60,230 | $30,115 |
| Energy Efficient | Cleveland | OES 6.20 | 14 | $25,480 | $12,740 |
| Energy Efficient | Henny Penny | ESC-610 | 5 | $16,025 | $8,013 |
| Energy Efficient | Hobart | CE10FD | 20 | $26,990 | $13,495 |
| Energy Efficient | Hobart | CE20FD | 40 | $54,638 | $27,319 |
| Energy Efficient | Hobart | CE20HD | 20 | $41,244 | $20,622 |
| Energy Efficient | Piper | HME 061X | 6 | $17,930 | $8,965 |
| Energy Efficient | Rational | SCC 102 | 20 | $22,790 | $11,395 |
| Energy Efficient | Rational | SCC 201 | 20 | $32,860 | $16,430 |
| Energy Efficient | Rational | SCC 202 | 40 | $46,130 | $23,065 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Cost Data for Gas Combination Ovens

| Group | Make | Model | Hotel Pan Capacity | List Price ($) | Cost ($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | Electrolux | 102 | 20 | $41,400 | $20,700 |
| Baseline | Eloma | G 6-11G | 6 | $19,450 | $9,725 |
| Baseline | Groen | CBE-10G | 10 | $31,480 | $15,740 |
| Baseline | Hobart | CG10FI | 20 | $33,729 | $16,865 |
| Baseline | Piper | HMG 072X | 14 | $32,068 | $16,034 |
| Baseline | Piper | HMG 102P | 20 | $33,102 | $16,551 |
| Baseline | Piper | HMG 102X | 20 | $39,309 | $19,655 |
| Baseline | Rational | CM 101G | 10 | $17,530 | $8,765 |
| Baseline | Rational | CM 102G | 20 | $23,970 | $11,985 |
| Baseline | Rational | CM 201G | 20 | $30,170 | $15,085 |
| Baseline | Rational | CM 202G | 40 | $45,460 | $22,730 |
| Baseline | Rational | CM 61G | 6 | $13,530 | $6,765 |
| Baseline | Rational | CM 62G | 12 | $19,830 | $9,915 |
| Energy Efficient | Alto Shaam | 10.20 ESG | 18 | $34,722 | $17,361 |
| Energy Efficient | Alto Shaam | 12.18 ESG | 18 | $34,398 | $17,199 |
| Energy Efficient | Alto Shaam | 20.20 ESG | 40 | $51,131 | $25,566 |
| Energy Efficient | Alto Shaam | 7.14 ESG | 14 | $24,644 | $12,322 |
| Energy Efficient | Cleveland | OGB 10.20 | 20 | $40,180 | $20,090 |
| Energy Efficient | Cleveland | OGB 12.20 | 24 | $54,520 | $27,260 |
| Energy Efficient | Cleveland | OGB 20.20 | 40 | $70,770 | $35,385 |
| Energy Efficient | Cleveland | OGB 6.20 | 14 | $32,130 | $16,065 |
| Energy Efficient | Cleveland | OGS 10.20 | 20 | $40,120 | $20,060 |
| Energy Efficient | Cleveland | OGS 12.20 | 24 | $54,150 | $27,075 |
| Energy Efficient | Cleveland | OGS 20.20 | 40 | $70,020 | $35,010 |
| Energy Efficient | Cleveland | OGS 6.20 | 14 | $31,700 | $15,850 |
| Energy Efficient | Henny Penny | GCC-615 | 6 | $21,058 | $10,529 |
| Energy Efficient | Henny Penny | GSC-115 | 10 | $27,354 | $13,677 |
| Energy Efficient | Rational | SCC 102G | 20 | $28,450 | $14,225 |
| Energy Efficient | Rational | SCC 201G | 20 | $36,150 | $18,075 |
| Energy Efficient | Rational | SCC 202G | 40 | $53,040 | $26,520 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Incremental Cost Data for Energy Efficient Combination Ovens*

| Combination Oven Type | Baseline Unit Price | Energy Efficient Unit Price | Incremental Price Difference | Baseline Unit Cost | Energy Efficient Unit Cost | Incremental Measure Cost (IMC) |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Combi < 15 pan capacity* | $18,274 | $21,411 | $3,137 | $9,137 | $10,705 | $1,568 |
| Electric Combi 15−28 pan capacity* | $30,048 | $33,216 | $3,168 | $15,024 | $16,608 | $1,584 |
| Electric Combi > 28 pan capacity* | $39,220 | $53,316 | $14,096 | $19,610 | $26,658 | $7,048 |
| Gas Combi < 15 pan capacity* | $22,315 | $29,491 | $7,177 | $11,157 | $14,746 | $3,588 |
| Gas Combi 15−28 pan capacity* | $33,613 | $40,336 | $6,723 | $16,807 | $20,168 | $3,361 |
| Gas Combi > 28 pan capacity* | $45,460 | $61,240 | $15,780 | $22,730 | $30,620 | $7,890 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

# References

American Society for Testing and Materials. Standard Test Method for the Performance of Combination Ovens in Different Modes. ASTM Designation F2861, in Annual Book of ASTM Standards, West Conshohocken, PA

DEER2014_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: DEER2014 Update Net-To-Gross table

EUL_Summary_10-1-08.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: EUL/RUL Values Cells: (D82)

2005 California Energy Commission (CEC) Title 20 Appliance Efficiency Regulations, CEC 400-2005-012, p. 69.

American Society for Testing and Materials. Standard Test Method for the Performance of Combination Ovens. ASTM Designation F1639, in Annual Book of ASTM Standards, West Conshohocken, PA.

American Society for Testing and Materials. Standard Test Method for the Performance of Steam Cookers. ASTM Designation F1484, in Annual Book of ASTM Standards, West Conshohocken, PA.

American Society for Testing and Materials. Standard Test Method for the Performance of Convection Ovens. ASTM Designation F1496, in Annual Book of ASTM Standards, West Conshohocken, PA.

Fisher-Nickel, inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 7-1 to 7-31.

Food Service Technology Center, Unpublished. Combination oven performance data.

Food Service Technology Center, Unpublished. Combination oven performance data.

Food Service Technology Center, Unpublished. Combination oven performance data.

2004-2005 Database for Energy Efficiency Resources (DEER) Update Study Final Report, pp. 3-15 to 3-18, table 3-14.

Food Service Technology Center, Unpublished. Combination oven performance data.

Food Service Technology Center, Unpublished. Combination oven performance data.

Food Service Technology Center, Unpublished. Combination oven performance data.

AutoQuotes electronic catalog for foodservice equipment and supplies http://www.aqnet.com/ .