Work Paper PGECOFST101

#### Commercial Convection Oven

Revision # 5

Pacific Gas & Electric Company

Customer Energy Solutions

Commercial Convection Oven-Electric and Gas

Measure CodeHA55, F187, HA38, HA64, F188, HA54

# At a Glance Summary

| Applicable Measure Codes: | HA55 | F187 | HA38 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Half-Size Convection Oven (Electric) | Commercial Full-Size Convection Oven (Electric) | Commercial Large Full-Size Convection Oven (Electric) |
| Energy Impact Common Units: | Convection Oven | Convection Oven | Convection Oven |
| Base Case Description: | Source: PG&E Calculations Existing Half-Size Electric Convection Oven | Source: PG&E Calculations Existing Full-Size Electric Convection Oven | Source: PG&E Calculations Existing Large Full-Size Electric Convection Oven |
| Base Case Energy Consumption: | Source: PG&E Calculations 9,709 kWh/yr | Source: PG&E Calculations 12,191 kWh/yr | Source: PG&E Calculations 14,527kWh/yr |
| Measure Energy Consumption: | Source: PG&E Calculations 7,191kWh/yr | Source: PG&E Calculations 9,417 kWh/yr | Source: PG&E Calculations 10,914kWh/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations 2,518 kWh/yr | Source: PG&E Calculations 2,774 kWh/yr | Source: PG&E Calculations 3,613 kWh/yr |
| Costs Common Units: | Source: PG&E Calculations Convection Oven | Source: PG&E Calculations Convection Oven | Source: PG&E Calculations Convection Oven |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations $3,605 | Source: PG&E Calculations $4,108 | Source: PG&E Calculations $7,300 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations $4,398 | Source: PG&E Calculations $5,115 | Source: PG&E Calculations $7,949 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations $793 | Source: PG&E Calculations $1,007 | Source: PG&E Calculations $649 |
| Effective Useful Life (years): | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL |
| Program Type: | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). |
| Net-to-Gross Ratios: | Source: 2014 DEER Com Default >2 yrs 0.6 | Source: 2014 DEER Com Default >2 yrs 0.6 | Source: 2014 DEER Com Default >2 yrs 0.6 |

# At a Glance Summary (Continued)

| Applicable Measure Codes: | HA64 | F188 | HA54 |
| --- | --- | --- | --- |
| Measure Description: | Commercial Half-Size Convection Oven (Gas) | Commercial Full-Size Convection Oven (Gas) | Commercial Large Full-Size Convection Oven (Gas) |
| Energy Impact Common Units: | Convection Oven | Convection Oven | Convection Oven |
| Base Case Description: | Source: PG&E Calculations Existing Half-Size Gas Convection Oven | Source: PG&E Calculations Existing Full-Size Gas Convection Oven | Source: PG&E Calculations Existing Large Full-Size Gas Convection Oven |
| Base Case Energy Consumption: | Source: PG&E Calculations 769 Therms/yr | Source: PG&E Calculations 1,052 Therms/yr | Source: PG&E Calculations 1,205 Therms/yr |
| Measure Energy Consumption: | Source: PG&E Calculations 543 Therms/yr | Source: PG&E Calculations 695 Therms/yr | Source: PG&E Calculations 749 Therms/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations 226 Therms/yr | Source: PG&E Calculations 357 Therms/yr | Source: PG&E Calculations 456 Therms/yr |
| Costs Common Units: | Source: PG&E Calculations Convection Oven | Source: PG&E Calculations Convection Oven | Source: PG&E Calculations Convection Oven |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations $3,614 | Source: PG&E Calculations $4,349 | Source: PG&E Calculations $N/A |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations $4,104 | Source: PG&E Calculations $5,526 | Source: PG&E Calculations $N/A |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations $490 | Source: PG&E Calculations $1,177 | Source: PG&E Calculations $N/A |
| Effective Useful Life (years): | 12 years -- Source: www.Deerresources.com EUL | 12 years -- Source: www.Deerresources.com EUL | 12 years -- Source: www.Deerresources.com EUL |
| Program Type: | Replace on Burnout (ROB), and New Construction (NC). Replace on Burnout (ROB), and New Construction (NC). Replace on Burnout (ROB), and New Construction (NC). |  |  |
| Net-to-Gross Ratios: | Source: 2014 DEER Com Default > 2 yrs 0.6 | Source: 2014 DEER Com Default >2yrs 0.6 | Source: 2014 DEER Com Default >2 yrs 0.6 |
| Important Comments: |  |  |  |

# At-A-Glance Measure List

|  |  |  |  |  |  |  |  | 1st Baseline |  |  |  |  |  |  |  | 2nd Baseline |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Measure Code | VersionSource | Measure Description | Measure Application Type | Building Type | Building Vintage | Climate Zone | Unit Definition | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | (EUL) LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | KW Peak Electric Demand Reduction | KWh Electric Savings | THM Gas Savings | LIFE CYCLE | Base Case Cost ($/unit) | Measure Cost ($/unit) | Labor Cost ($/unit) | IMC Incremental Measure Cost ($/unit) | GRR_kW | GRR_kWh | GRR_thm | NTG | Implementation Method [DI, DD, I] | ISR |
| HA55 | Calculations | Commercial Half-Size Convection Oven (Electric) | ROB | ANY | ANY | ANY | each | 0.52 | 2,518 | 0 | 12 | 3,605 | 4,398 | 0 | 793 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |
| F187 | Calculations | Commercial Full-Size Convection Oven (Electric) | ROB | ANY | ANY | ANY | each | 0.57 | 2,774 | 0 | 12 | 4,108 | 5,115 | 0 | 1007 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |
| HA38 | Calculations | Commercial Large Full-Size Convection Oven (Electric) | ROB | ANY | ANY | ANY | each | 0.75 | 3,613 | 0 | 12 | 7,300 | 7,949 | 0 | 649 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |
| HA64 | Calculations | Commercial Half-Size Convection Oven (Gas) | ROB | ANY | ANY | ANY | each | 0 | 0 | 226 | 12 | 3,614 | 4,104 | 0 | 490 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |
| F188 | Calculations | Commercial Full-Size Convection Oven (Gas) | ROB | ANY | ANY | ANY | each | 0 | 0 | 408 | 12 | 4,349 | 5,635 | 0 | 1286 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |
| HA54 | Calculations | Commercial Large Full-Size Convection Oven (Gas) | ROB | ANY | ANY | ANY | each | 0 | 0 | 456 | 12 | 0 | 0 | 0 | 0 |  |  |  |  |  |  |  |  | 1 | 1 | 1 | 0.6 | I | 1 |

# Work Paper Approvals

The following Manager(s) approved this workpaper through the PG&E Electronic Data Routing System under Routing Requisition # _______________

Grant Brohard Manager, Technical Product Support

Carolyn Weiner Manager, Appliance Products

# Document Revision History

Revision # 		Date 		         Description		         Author (Company)

| Revision 0 | 12/12/2007 | Original work paper: Commercial Convection Oven PGECOFST101 R0.doc | David Zabrowski (Fisher-Nickel, Inc.) |
| --- | --- | --- | --- |
| Revision 1. | 6/1/09. | Changes to EUL, NTG language and references, costs updated. | David Zabrowski, Lauren Mills (Fisher-Nickel, inc.), Steve Blanc PG&E. |
| Revision 2 | 3/31/2010 | Updated pricing and incremental cost, Changes to align with new Energy Star specification, Update to DEER 2009-11 NTG file | David Zabrowski (Fisher-Nickel, inc.), Steve Blanc PG&E |
| Revision 3 | 7/27/2011 | Updated measure codes for 2010-12 program, included ED comments and UES reduction in savings by 30% in sections 2.3 and 2.3 | Charlene Spoor PG&E (CLCI) |
| Revision 4 | 6/5/2012 8/22/2012 | Updated NTG, EUL, IMC, and savings analysis Updated BLD, CZ, VIN to ANY per READI tool requirements | David Zabrowski (Fisher-Nickel, inc.) Charlene Spoor (PG&E) |
| Revision 5 | 3/14/14 | Updated T&C’s to match savings in Energy Star V 2.1, updated cost and savings data for new level. New WP template. | David Zabrowdki (Fni) Charlene Spoor (PG&E) |

# Table of Contents

At a Glance Summary	ii

At a Glance Summary (Continued)	iii

At-A-Glance Measure List	iv

Work Paper Approvals	v

Document Revision History	vi

Table of Contents	vii

List of Tables	viii

Section 1. General Measure & Baseline Data	1

1.1 Product Measure Description & Background	1

1.2 Product Technical Description	2

1.3 Measure Application Types	3

1.4 Product Base Case and Measure Case Data	3

1.4.1 DEER Base Case and Measure Case Information	4

1.4.2 Codes & Standards Requirements Base Case and Measure Information	4

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	4

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	5

1.4.5 Time of Use Adjustment Factors…………………………………………………………5

1.5 Summary of Inputs for Savings Calculations	5

1.6 Net-to-Gross Ratios for Different Program Strategies	5

Section 2. Calculation Methods	5

2.1 Electric Energy Savings Estimation Methodologies	5

2.2. Demand Reduction Estimation Methodologies	9

2.3. Gas Energy Savings Estimation Methodologies	9

Section 3. Load Shapes	12

3.1 Base Cases Load Shapes	12

3.2 Measure Load Shapes	12

Section 4. Base Case & Measure Costs	13

4.1 Base Cases Costs	13

4.2 Measure Costs	13

4.3.1 Gross Measure Costs…………………………………………………………………….13

4.3.2 Incremental & Full Measure Costs	13

Appendix A	15

Appendix B	17

References	20

# List of Tables

Table 1. Energy Efficiency Requirements for Commercial Convection Ovens.	2

Table 2. Measure Application Type…………………………………………………………….....3

Table 3. DEER Use and Technology Table……………………………………………………….3

Table 4. DEER Net-to-Gross Ratios………………………………………………………………4

Table 5. DEER Estimated Useful Life…………………………………………………………….4

Table 6. Baseline ASTM test results for Commercial Convection Ovens.	6

Table 7. Measure Case ASTM test results for Commercial Convection Ovens.	6

Table 8. Summary of Inputs……………………………………………………………………….7

Table 9. Commercial Electric Half-Size Convection Oven Cost-Effectiveness Example.	8

Table 10. Commercial Electric Full-Size Convection Oven Cost-Effectiveness Example.	9

Table 11. Commercial Electric Large Full-Size Convection Oven Cost-Effectiveness Example	10

Table 12. Commercial Gas Half-Size Convection Oven Cost Effectiveness Example.	11

Table 13. Commercial Gas Full-Size Convection Oven Cost Effectiveness Example.	12

Table 14. Commercial Gas Large Full-Size Convection Oven Cost Effectiveness Example.	13

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the rationale for the Energy-Efficient Commercial Convection Oven (Electric and Gas) measures as listed in the Commercial Food Service Catalog. The Commercial Food Service Catalog is part of Pacific Gas and Electric Company’s Customer Energy Efficiency Program. PG&E offers incentives to non-residential customers for installing qualifying lighting, refrigeration, air-conditioning, food service, and agricultural equipment. All qualifying models must be listed in the California Energy Commissions Database and must meet the following minimum criteria.

Catalog Description –

HA55: The half-size electric oven must meet ENERGY STAR® specifications or have a tested heavy load potato cooking energy efficiency ≥ 71% and idle rate ≤ 1.0 kW utilizing ASTM Standard F1496.

F187: The full-size (≤ 5 pans) electric oven must meet ENERGY STAR® specifications or have a tested heavy load potato cooking energy efficiency ≥ 71% and idle rate ≤ 1.6 kW utilizing ASTM Standard F1496.

HA38: The large full-size (> 5 pans) electric oven must have a tested heavy load potato cooking energy efficiency ≥ 73% and idle rate ≤ 1.9 kW utilizing ASTM Standard F1496.

HA64: The half-size gas oven must have a tested heavy load potato cooking energy efficiency ≥ 46% and idle rate ≤ 8,000 Btu/h utilizing ASTM Standard F1496.

F188: The full-size (≤ 5 pans) gas oven must meet ENERGY STAR® specifications or have a tested heavy load cooking energy efficiency ≥ 46% and idle rate ≤ 12,000 Btu/h utilizing ASTM Standard F1496.

HA54: The large full-size (> 5 pans) gas oven must have a tested heavy load potato cooking energy efficiency ≥ 46% and idle rate ≤ 13,000 Btu/h utilizing ASTM Standard F1496.

SCE1, SCE2, SCG3, SCG4:  Southern California Edison and Southern California Gas have further broken down the measure by size, separating out half size and large full size for both electric and gas technologies.

Program Restrictions and Guidelines

Terms and Conditions

HA55, F187, HA38, HA64, F188, HA54:  The rebate is downstream provided to the customer as the time of sale upon receipt of application and invoice. With the exception of HA55 provided by 3P this is not a direct install program.

Market Applicability

This measure is applicable to any commercial cooking application, including (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities. Installations across all climate zones, building types and building vintages in the PG&E Service Territory may apply.

This measure includes new commercial electric or gas convection ovens that are ENERGY STAR® qualified or meet the qualifications listed in Table 1. ENERGY STAR® maintains an updated list of qualifying products and specifications at www.energystar.gov. Consult with the manufacturer or manufacturer’s representative to determine if a non-ENERGY STAR® qualified model meets the efficiency requirements in Table 1.Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance has a cooking-energy efficiency that meets the requirements in Table 1.

Table 1 Energy Efficiency Requirements for Commercial Convection Ovens.

| Measure Code | Convection oven Type | Cooking-Energy Efficiency* | Idle Energy Rate |
| --- | --- | --- | --- |
| F187 | Electric Half-Size Convection Ovens | ≥ 71% | ≤ 1.0 kW |
|  | Electric Full-Size (≤ 5 pans) Convection Ovens | ≥ 71% | ≤ 1.6 kW |
|  | Electric Full-Size (> 5 pans) Convection Ovens | ≥ 73% | ≤ 1.9 kW |
| F188 | Gas Half-Size Convection Ovens | ≥ 46% | ≤ 8,000 Btu/h |
|  | Gas Full-Size (≤ 5 pans) Convection Ovens | ≥ 46% | ≤ 13,000 Btu/h |
|  | Gas Full-Size (> 5 pans) Convection Ovens | ≥ 46% | ≤ 13,000 Btu/h |

*Based on the heavy-load potato test in ASTM F1496.

## 1.2 Product Technical Description

Commercial convection ovens are the most widely used appliances in the food service industry. Many food service operations rely heavily on the versatility of ovens. Operators can cook varieties of foods in large quantities with a single appliance. An oven can be simply described as a fully enclosed, insulated chamber used to heat food. With competition rising among equipment manufacturers, new designs that incorporate timesaving features via sophisticated control packages are being introduced.

Ovens represent the largest appliance category in terms of the types of units manufactured of any of the major cooking equipment categories. This versatility and diversity mean that they can be found in almost any type of food service operation. A recent US study showed that 95% of commercial (non-institutional) operations reported using at least one type of oven; 98% of noncommercial (institutional) operations reported the same. The percentage of operations, commercial and institutional, using general bake ovens was 52% and 56%, respectively. Fifty percent of the operations in the commercial sector reported using convection ovens as compared to 83% of noncommercial operations.

Convection oven performance is determined by applying the ASTM Standard Test Method for the Performance of Convection Ovens (F1496). The ASTM standard test method is considered to be the industry standard for quantifying the energy consumption, efficiency and cooking performance of convection ovens.

In 2013, ASTM F1496 was updated to accommodate a wider range of oven cavity sizes. The Energy Star v2.1 specification for commercial ovens now references the updated ASTM standard in addition to tightening the idle rate requirements for gas full-size convection ovens.

## 1.3 Measure Application Types

The DEER Measure Cost Data Users Guide found on www.deeresources.com under DEER2014 Database Format hyperlink, DEER2014, spreadsheet SPTdata_format-V0.97.xls, defines the terms as follows:

Table 2 Measure Application Type

Identifies the measure application type in the Measure Implementation table in DEER2014

| Code | Description | Comment |
| --- | --- | --- |
| ER | Early retirement | measure applied while existing equipment still viable, or retrofit of existing equipment |
| ROB | Replace on Burnout | measure applied when existing equipment fails or maintenance requires replacement |
| NC | New Construction | measure applied during construction design phase as an alternative to a code-compliant standard design |

Since there are no EM&V studies on the useful life of convection ovens and it is standard practice in the commercial foodservice industry to purchase equipment only when it is needed (e.g., replacement or additional capacity), this measure is focused on ROB and NC applications only.

## 1.4 Product Base Case and Measure Case Data

## 1.4.1 DEER Base Case and Measure Case Information

Table 3 DEER Use and Technology Table

The DEER database does not contain information on energy use or savings for an energy-efficient electric or gas convection oven measure. The only reference in DEER for Commercial cooking equipment is for Estimated Useful Life.

- Hours of Operation

- This measure would follow the hours of operation for quick serve and full serve restaurants as noted in the ASTM standards.  For this measure annual hours of operation are considered 4380.

- 12 hrs /day * 365 day/ yr = 4380 hrs

- Base Case & Measure Case Costs

The base case and measure case costs are calculated and found in section 1.4.4.

Net-to-Gross Assumptions

The 2013-2014 DEER NTGR Values file does not specifically list commercial food service appliances. The default used for non-residential measures is 0.6.

Table 4 summarizes all applicable DEER based Net-to-Gross ratios for programs that may be used by this measure.

Table 4. DEER 2014 Net-to-Gross Ratios

| Program Approach | NTG |
| --- | --- |
| All Com-Default>=2yrs | 0.6 |

- Effective Useful Lives

The 2014 DEER database shows a EUL of 12 years and an RUL of 4 years for all cooking appliance measures, including electric and gas convection ovens.

Table 5. DEER Effective Useful Life

| Measure Code | Building Type | Building Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| HA55/F187/HA38 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |
| HA64/F188/HA54 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |

In Service Rate/ First Year Installation Rate

- The in-service rate was not found in DEER or any supporting documentation. We have therefore assumed the ISR is 1.0 for all measures based on engineering judgment.

- See section 1.1 Terms and Conditions and Market Applicability to reference the type of program delivery mechanism and customer status used to determine this entry.

## 1.4.2 Codes & Standards Requirements Base Case and Measure Information

California Title 20: California Title 20 Appliance Efficiency Standards does require reporting the convection oven idle energy rate per ASTM F1496 for the CEC Appliance Database, but the standard contains no minimum performance requirement.

California Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: These measures do not fall under Federal DOE or EPA Energy Regulations.

ASTM Standards: ASTM Standard Test Method for the Performance of Convection ovens (F1496) is applicable for estimating energy use and cooking performance. It was used to estimate the energy consumption of the base case and measure equipment.

## 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

There were no specific EM&V studies identified that addressed cooking measures in the commercial sector.

## 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

This Work Paper uses ASTM Standard Test Method for the Performance of Convection Ovens (F1496) for calculation of energy use and demand, based on testing in an approved and qualified laboratory. In the absence of mandatory regulations for testing commercial ovens, there is little incentive on the part of equipment manufacturers to have their baseline equipment tested. Therefore, the ASTM performance parameters for baseline equipment were drawn from a sample of economy grade equipment tested by the Food Service Technology Center and is summarized in Table 6.

Table 6 Baseline ASTM Test Results for Commercial Convection Ovens.

| Convection Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Half-Size | 1,500 W | 65% | 45 lb/h |
| Electric Full-Size | 2,000 W | 65% | 70 lb/h |
| Electric Large Full-Size | 2,500 W | 65% | 100 lb/h |
| Gas Half-Size | 12,000 Btu/h | 30% | 45 lb/h |
| Gas Full-Size | 18,000 Btu/h | 30% | 70 lb/h |
| Gas Large Full-Size | 21,000 Btu/h | 30% | 100 lb/h |

*Based on the Heavy-load potato test in ASTM F1496.1

The measure case data was drawn from the list of commercial convection ovens that have been tested by The PG&E Food Service Technology Center in San Ramon, the Southern California Gas Company Foodservice Equipment Center in Downey and the Southern California Edison Foodservice Technology Center in Irwindale. The complete list of energy-efficient convection ovens as of May 30, 2012 is in Appendix A and the averages are summarized in Table 7.

Table 7 Measure Case ASTM Test Results for Commercial Convection Ovens.

| Convection Oven Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Half-Size | 880 W | 72% | 53 lb/h |
| Electric Full-Size | 1,400 W | 73% | 82 lb/h |
| Electric Large Full-Size | 1,730 W | 76% | 137 lb/h |
| Gas Half-Size | 8,500 Btu/h | 45% | 83 lb/h |
| Gas Full-Size | 10,870 Btu/h | 48% | 81 lb/h |
| Gas Large Full-Size | N/A | N/A | N/A |

*Based on the Heavy-load potato test in ASTM F1496.1

The Food Service Technology Center conducted an assessment of major commercial cooking appliance technologies in 2002, which included a chapter on ovens.  The study showed that oven efficiency varies from 10% to 80%, based on primary fuel source and oven type (convection, combination, conventional, etc.). A further Food Service Technology Center study included convection oven performance data, which suggests a baseline efficiency of 65% for electric convection ovens and 30% gas convection ovens, respectively.

Since the current Title 20 regulations do not include a minimum performance requirement for convection ovens, the base case for existing models of electric convection ovens was determined from Table 3-2 in the Food Service Technology Center study.Error: Reference source not found

# 1.5 Summary of Inputs for Saving Calculations

Table 8. Summary of Inputs for Savings Calculations

| Input Variable | Variations | Base Case E Average Value | Base Case G Average Value | Measure Case Average Value F187 | Measure Case Average Value F188 | Refer- ence Section |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Savings* | None | N/A | N/A | 2774 | N/A | N/A |
| Gas Savings* | None | N/A | N/A | N/A | 408 | 1.4.4 |
| Hours of operation | None | 4,380 | 4380 | 4380 | 4380 | 1.4.1 |
| Full Cost | None | $4,108 | $4,349 | $5,115 | $5,635 | 1.4.4 |
| Increment Cost | None | N/A | N/A | $1,007 | $1,286 | 1.4.4 |
| EUL /RUL | None | 12 | N/A | 12 | 12 | 1.4.1 |
| NTG | None | 0.6 | N/A | 0.6 | 0.6 | 1.4.1 |
| ISR | None | 1 | N/A | 1 | 1 | 1.4.1 |

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of convection ovens is ASTM Standard Test Method for the Performance of Convection ovens (F1496). Tables 9 - 11 show examples of the calculation results for electric convection ovens based on ASTM F1496 test results.

Table 9. Commercial Electric Half-Size Convection Oven Cost-Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.00 | 0.90 |
| Idle Energy Rate (kW) | 1.5 | 0.88 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 72% |
| Production Capacity (lbs/hr) | 45 | 53 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 26.6 | 19.7 |
| Average Demand (kW) | 2.2 | 1.6 |
| Estimated Demand Reduction (kW) | - | 0.6 |
| Annual Energy Consumption (kWh) | 9,709 | 7,191 |
| Estimated Energy Savings (kWh/yr) | - | 2,518 |
| Annual Energy Cost ($) | $1,262 | $935 |
| Estimated Cost Savings ($/yr) | - | $327 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on the 2014 DEER EUL estimates.

Table 10. Commercial Electric Full-Size Convection Oven Cost-Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.50 | 1.00 |
| Idle Energy Rate (kW) | 2.00 | 1.40 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 73% |
| Production Capacity (lbs/hr) | 70 | 82 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 33.4 | 25.8 |
| Average Demand (kW) | 2.8 | 2.2 |
| Estimated Demand Reduction (kW) | - | 0.6 |
| Annual Energy Consumption (kWh) | 12,191 | 9,417 |
| Estimated Energy Savings (kWh/yr) | - | 2,774 |
| Annual Energy Cost ($) | $1,585 | $1,224 |
| Estimated Cost Savings ($/yr) | - | $361 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on the 2014 DEER EUL estimates.

Table 11. Commercial Electric Large Full-Size Convection Oven Cost-Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 1.70 | 1.20 |
| Idle Energy Rate (kW) | 2.50 | 1.73 |
| Heavy Load Cooking Energy Efficiency (%) | 65% | 76% |
| Production Capacity (lbs/hr) | 100 | 137 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.0732 | 0.0732 |
| Daily Energy Consumption (kWh) | 39.8 | 29.9 |
| Average Demand (kW) | 3.3 | 2.5 |
| Estimated Demand Reduction (kW) | - | 0.7 |
| Annual Energy Consumption (kWh) | 14,527 | 10,914 |
| Estimated Energy Savings (kWh/yr) | - | 3,613 |
| Annual Energy Cost ($) | $1,889 | $1,419 |
| Estimated Cost Savings ($/yr) | - | $470 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on the 2014 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (kWh/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking based on ASTM F1496 |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (kW) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (kWh) |

## 2.2. Demand Reduction Estimation Methodologies

An oven’s actual contribution to a building’s peak demand may vary significantly depending on its usage pattern in relation to that of other electric equipment in the facility (operating schedule, appliance on time, etc.). The probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance. Therefore, it has been assumed that the probable contribution to the building’s peak demand is equal to the appliance’s average demand.

The demand reduction estimation is based on measured data for standard efficiency electric convection ovens and for high-efficiency convection ovens. The measured data are derived from tests conducted under ASTM Standard Test Method for the Performance of Convection ovens (F1496).

ASTM F1496 provides standard conditions under which convection oven energy use is measured. The estimated demand reduction of 580 Watts for a Half-Size oven, 630 Watts for a Full-Size oven and 830 Watts for a Large Full-Size oven is based on data from tests of standard efficiency and high efficiency convection ovens. Applying a Coincidence Factor of 0.9 per the DEER methodology yields a Demand Savings of 520 Watts for a Half-Size oven, 570 Watts for a Full-Size oven and 750 Watts for a Large Full-Size oven.

## 2.3. Gas Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of convection ovens is ASTM Standard Test Method for the Performance of Convection ovens (F1496). Tables 12 - 14 show examples of the calculation results under ASTM F1496.

Table 12. Commercial Gas Half-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 13,000 | 7,500 |
| Idle Energy Rate (Btu/hr) | 12,000 | 8,500 |
| Cooking-Energy Efficiency (%) | 30% | 45% |
| Production Capacity (lb/hr) | 45 | 55 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 210,667 | 148,739 |
| Annual Energy Consumption (therms)a | 769 | 543 |
| Estimated Energy Savings (therms/yr) | - | 226 |
| Annual Energy Cost ($) | $769 | $543 |
| Estimated Cost Savings ($/yr) | - | $226 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)c | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

c The estimated useful life is based on 2014 DEER EUL estimates.

Table 13. Commercial Gas Full-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 19,000 | 10,094 |
| Idle Energy Rate (Btu/hr) | 18,000 | 10,870 |
| Cooking-Energy Efficiency (%) | 30% | 48% |
| Production Capacity (lb/hr) | 70 | 81 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 288,119 | 176,480 |
| Annual Energy Consumption (therms)a | 1,052 | 644 |
| Estimated Energy Savings (therms/yr) | - | 408 |
| Annual Energy Cost ($) | $1,052 | $644 |
| Estimated Cost Savings ($/yr) | - | $408 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)c | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

c The estimated useful life is based on 2014 DEER EUL estimates.

Table 14. Commercial Gas Large Full-Size Convection Oven Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 21,000 | 11,000 |
| Idle Energy Rate (Btu/hr) | 21,000 | 13,000 |
| Cooking-Energy Efficiency (%) | 30% | 46% |
| Production Capacity (lb/hr) | 100 | 100 |
| Operating Hours/Day | 12 | 12 |
| Operating Days/Year | 365 | 365 |
| Number of Preheats per Day | 1 | 1 |
| Pounds of Food Cooked per Day | 100 | 100 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 250 | 250 |
| Daily Energy Consumption (Btu) | 330,083 | 205,098 |
| Annual Energy Consumption (therms)a | 1,205 | 749 |
| Estimated Energy Savings (therms/yr) | - | 456 |
| Annual Energy Cost ($) | $1,205 | $749 |
| Estimated Cost Savings ($/yr) | - | $456 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)c | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

c The estimated useful life is based on 2014 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (Btu/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (Btu/lb) = Btu/pound of energy absorbed by food product during cooking based on ASTM F1496 |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (Btu/h) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (Btu) |

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio for electricity.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Cases Load Shapes

The base case would be expected to follow a typical nonresidential foodservice end use load shape.

Commercial convection oven load shapes  differ among food service facilities (quick service, casual dining, hotels, college, schools, hospitals etc) depending on daily menu variations, hours of operation, serving periods, day-of-week, and facility location (city downtown, suburban mall, access to interstate highways, etc.). Consequently, applicable average TOU and hourly load shapes for convection ovens are unavailable. The ASTM Standard Test Method used to generate energy use data is based on hours of use and operating state (preheat, idle, and heavy-load cooking). Generally, convection ovens are used to prepare food within a few days to a few hours before it is served, so loads tend to not necessarily be coincident with regular meal periods (breakfast, lunch, and dinner). Between meal periods convection ovens may be used to prepare ingredients for either the next meal period or for menu items to be served the next several days (in which case the ingredients are refrigerated immediately after cooking).

## 3.2 Measure Load Shapes

The measure load shape for the electric measure is determined by the E3 calculator based on the applicable nonresidential market sector and the foodservice end-use.

The electric demand profile for the high-efficiency electric convection oven is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high-efficiency convection oven will use less energy and have a slightly lower demand profile.

The gas load profile for the high efficiency gas convection oven is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high efficiency convection oven will use less energy.

# Section 4. Base Case & Measure Costs

High-efficiency convection ovens typically have a higher list price than standard efficiency convection ovens. However, high-efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high-efficiency convection ovens exhibit better baking uniformity and higher production rates that increase their cost-effectiveness.

Equipment prices for these work papers were compiled from a number of sources including, Autoquotes (autoquotes.com), equipment sales reps and manufacturer sources. Since equipment pricing in food service is closely held information and prices vary widely according to buying volume and other factors, we cannot list the sources for prices specifically.

## 4.1 Base Cases Costs

The Base Case costs include only the equipment. High efficiency convection ovens require no additional labor or maintenance compared to base case convection ovens. Since this measure is applicable for ROB and NC installations, the installation and maintenance costs are expected to be the same for the customer. The estimated equipment cost is based on recent list cost data for electric and gas convection ovens and applying an industry-standard 50% discount to the manufacturer published list prices.

## 4.2 Measure Costs

The Measure costs include only the equipment, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 50% discount to the manufacturer published list prices (see Appendix B).

## 4.3 Incremental & Full Measure Costs

Incremental measure costs are used in the analysis.

Index

Daily Energy Consumption, 11

DEER Measure Cost Data Users Guide, 3

load shape, 12

Measure Load Shape, 12, 13

New Construction, ii, iii, 3

Replace on Burnout, ii, iii, 3

Retrofit, ii, iii

# Appendix A

ASTM Qualifying Equipment Data for Electric Convection Ovens

| Company | Model Number | Size | Energy Efficiency (%) | Idle Energy Rate (Btu/h or kW) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| APW Wyott | BCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Bakers Pride | BCO-E | Full-Size | 72% | 1.46 | 71.5 |
| BKI | COM-E | Full-Size | 72% | 1.46 | 71.5 |
| Blodgett | CTB | Half-Size | 72% | 1.01 | 52 |
| Blodgett | HV-100E | Full-Size | 70% | 1.45 | 92.7 |
| Blodgett | Mark-V | Full-Size | 74% | 1.46 | 98.6 |
| Blodgett | SHO-E | Full-Size | 73% | 1.6 | 90 |
| Blodgett | XL-50E | Half-Size | 74% | 0.63 | 51 |
| Blodgett | Zephaire 240E | Full-Size | 75% | 1.44 | 101 |
| Blodgett | Zephaire E | Full-Size | 71% | 1.57 | 87 |
| CPG | CPG-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Imperial | ICVE | Full-Size | 72% | 1.56 | 85 |
| Moffat | E32D | Full-Size | 78% | 0.98 | 82 |
| Moffat | E32MS | Full-Size | 80% | 0.8 | 77 |
| Moffat | E35MS | Large-Size | 73% | 1.55 | 120 |
| NDG | NDG-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Revent | 7802 | Full-Size | 75% | 1.1 | 76 |
| Revent | 7803 | Large-Size | 79% | 1.9 | 153 |
| Short Order | SO-DCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Southbend | SLES/10 | Full-Size | 75% | 1.55 | 84 |
| Star Manufacturing | ECOF-AP | Full-Size | 70% | 1.36 | 85 |
| Star Manufacturing | ECSF-ES | Full-Size | 76% | 1.6 | 96 |
| Tri-Star | TSCO-E | Full-Size | 72% | 1.46 | 71.5 |
| Wells | M4200-3 | Half-Size | 71% | 0.99 | 55.2 |
| Average Performance |  | Half-Size | 72% | 0.88 | 53 |
| Average Performance |  | Full-Size | 73% | 1.40 | 82 |
| Average Performance |  | Large-Size | 76% | 1.725 | 137 |

ASTM Qualifying Equipment Data for Gas Convection Ovens

| Company | Model Number | Size | Energy Efficiency (%) | Idle Energy Rate (Btu/h or kW) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| American Range | M-1 | Full-Size | 46% | 10842 | 101 |
| American Range | MSD | Full-Size | 46% | 11714 | 85 |
| Blodgett | DFG100 | Full-Size | 46% | 11200 | 84 |
| Blodgett | SHO-G | Full-Size | 46% | 10000 | 86 |
| Blodgett | Zephaire 240G | Full-Size | 46% | 12000 | 78 |
| Blue Seal | G32D5 | Full-Size | 47% | 12000 | 61 |
| Duke Manufacturing | E-101G | Full-Size | 53% | 10000 | 82 |
| Garland | MCO-GS-10 ESS | Full-Size | 46% | 11600 | 90 |
| Hobart | HGC5 | Full-Size | 48% | 11850 | 80 |
| Hobart | HGC5DX | Full-Size | 48% | 11097 | 90 |
| Imperial | ICVG-1 | Full-Size | 46% | 11400 | 82 |
| Lang | GCSF | Full-Size | 46% | 8530 | 97 |
| Moffat | G32D5 | Full-Size | 47% | 12000 | 61 |
| Vulcan | SG4D | Full-Size | 50% | 11360 | 89 |
| Vulcan | VC4G | Full-Size | 48% | 11850 | 80 |
| Vulcan | VC6G | Full-Size | 48% | 11097 | 90 |
| Vulcan | GCO2D | Half-Size | 53% | 4293 | 47 |
| Wolf | WKGD1 | Full-Size | 48% | 11850 | 80 |
| Average Performance |  | Half-Size | 53% | 4293 | 47 |
| Average Performance |  | Full-Size | 48% | 10870 | 81 |

# Appendix B

Equipment Cost Data for Electric Convection Ovens

| Group | Make | Model | Size | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | Alto-Shaam | ASC-4E | Full-Size | $5,354 | $2,677 |
| Baseline | Doyon | JA6SL | Large Full-Size | $14,600 | $7,300 |
| Baseline | Duke | 5/9-E3V | Half-Size | $6,826 | $3,413 |
| Baseline | Duke | 613QE1* | Full-Size | $9,404 | $4,702 |
| Baseline | Duke | E101E* | Full-Size | $6,818 | $3,409 |
| Baseline | Garland | MCOE5 | Half-Size | $8,710 | $4,355 |
| Baseline | Hobart | HEC5 | Full-Size | $6,499 | $3,250 |
| Baseline | Lang | ECOH | Half-Size | $8,240 | $4,120 |
| Baseline | Market Forge | 3000 | Full-Size | $7,956 | $3,978 |
| Baseline | Market Forge | 2600HE | Full-Size | $8,788 | $4,394 |
| Baseline | Moffat | E311MS | Half-Size | $3,900 | $1,950 |
| Baseline | Montague | EK12A | Full-Size | $10,696 | $5,348 |
| Baseline | Vulcan | ECO2D | Half-Size | $8,370 | $4,185 |
| Baseline | Vulcan | VC4EC | Full-Size | $10,210 | $5,105 |
| Energy Efficient | Bakers Pride | BCO-E1 | Full-Size | $10,864 | $5,432 |
| Energy Efficient | Blodgett | CTB | Half-Size | $8,470 | $4,235 |
| Energy Efficient | Blodgett | Mark V | Full-Size | $12,775 | $6,388 |
| Energy Efficient | Blodgett | Xcel-50 | Half-Size | $9,120 | $4,560 |
| Energy Efficient | Blodgett | Zephaire E | Full-Size | $9,405 | $4,703 |
| Energy Efficient | Moffat | E35 | Full-Size | $11,915 | $5,958 |
| Energy Efficient | Moffat | E35 | Large Full-Size | $11,915 | $5,958 |
| Energy Efficient | Revent | 7803 | Large Full-Size | $19,882 | $9,941 |
| Energy Efficient | Southbend | SLES/10 | Full-Size | $6,188 | $3,094 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Cost Data for Gas Convection Ovens

| Group | Make | Model | Size | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- | --- |
| Baseline | Blodgett | DFG-50 | Half-Size | $10,155 | $5,078 |
| Baseline | Duke | E-101G | Full-Size | $6,818 | $3,409 |
| Baseline | Garland | G5L-N | Half-Size | $5,492 | $2,746 |
| Baseline | Lang | GCOF-T | Full-Size | $8,640 | $4,320 |
| Baseline | Market Forge | 2900 | Full-Size | $8,414 | $4,207 |
| Baseline | Market Forge | 2500HEC | Full-Size | $9,707 | $4,854 |
| Baseline | Montague | 115A | Full-Size | $12,566 | $6,283 |
| Baseline | Montague | R85A | Full-Size | $10,488 | $5,244 |
| Baseline | Southbend | BGS | Full-Size | $5,378 | $2,689 |
| Baseline | Southbend | GH-20SC | Half-Size | $6,037 | $3,019 |
| Baseline | US Range | SDG-1 | Full-Size | $7,567 | $3,784 |
| Energy Efficient | Blodgett | DFG-100 | Full-Size | $14,015 | $7,008 |
| Energy Efficient | Blodgett | Zephaire-240G | Full-Size | $11,340 | $5,670 |
| Energy Efficient | Duke | E102-G | Full-Size | $12,718 | $6,359 |
| Energy Efficient | Garland | MCO-GS-10-ESS | Full-Size | $10,582 | $5,291 |
| Energy Efficient | Hobart | HGC20 | Half-Size | $5,786 | $2,893 |
| Energy Efficient | Imperial Range | ICV | Full-Size | $8,546 | $4,273 |
| Energy Efficient | Vulcan | GCOD | Half-Size | $10,630 | $5,315 |
| Energy Efficient | Vulcan | SG4D | Full-Size | $13,050 | $6,525 |
| Energy Efficient | Vulcan | VC4GD | Full-Size | $10,210 | $5,105 |
| Energy Efficient | Wolf | WKGD1 | Full-Size | $9,700 | $4,850 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Incremental Cost Data for Energy Efficient Convection Ovens*

| Convection Oven Type | Baseline Unit Price | Energy Efficient Unit Price | Incremental Price Difference | Baseline Unit Cost | Energy Efficient Unit Cost | Incremental Measure Cost (IMC) |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Half-Size | $7,209 | $8,795 | $1,586 | $3,605 | $4,398 | $793 |
| Electric Full-Size | $8,216 | $10,229 | $2,014 | $4,108 | $5,115 | $1,007 |
| Electric Large Full-Size | $14,600 | $15,899 | $1,299 | $7,300 | $7,949 | $649 |
| Gas Half-Size | $7,228 | $8,208 | $980 | $3,614 | $4,104 | $490 |
| Gas Full-Size | $8,697 | $11,270 | $2,573 | $4,349 | $5,635 | $1,286 |
| Gas Large Full-Size | N/A | N/A | N/A | N/A | N/A | N/A |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

N/A: Pricing not available.

# References

American Society for Testing and Materials. Standard Test Method for the Performance of Convection Ovens. ASTM Designation F1496, in Annual Book of ASTM Standards, West Conshohocken, PA.

2009 Energy Star® Program Requirements for Commercial Ovens. http://www.energystar.gov/ia/partners/product_specs/program_reqs/Commercial_Ovens_Program_Requirements.pdf

Fisher-Nickel, Inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 3-1 to 3-27.

The DEER Measure Cost Data Users Guide found on www.deeresources.com under DEER2011 Database Format hyperlink, DEER2011 for 13-14, spreadsheet SPTdata_format-V0.97.xls.

2008 Database for Energy-Efficient Resources Version 2008.2.05  December 16, 2008; www.deeresources.com – EUL/RUL (Effective/Remaining Useful Life) Values --EUL/RUL Values and Summary Documentation (Updated 10 October 2008) Excel Spreadsheet --http://www.deeresources.com/deer0911planning/downloads/EUL_Summary_10-1-08.xls (Non-Residential/Cooking )

DEER2014_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: DEER2014 Update Net-To-Gross table

EUL_Summary_10-1-08.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: EUL/RUL Values Cells: (D82)

2005 California Energy Commission (CEC) Title 20 Appliance Efficiency Regulations, CEC 400-2005-012, p. 69.

Fisher-Nickel, Inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 3-1 to 3-27.

PG&E, J. Blessent, 1994. Development and Application of a Uniform Testing Procedure for Convection Ovens. Pacific Gas and Electric Company Department of Research and Development Report 008.1-94-12, Table 3-2, pp. 3-2 to 3-3.

2004-2005 Database for Energy Efficiency Resources (DEER) Update Study Final Report, pp. 3-15 to 3-18.