Work Paper PGECOFST102

#### Commercial Fryer-Electric and Gas

Revision # 5

Pacific Gas & Electric Company

Customer Energy Solutions

Commercial Fryer-Electric and Gas

Measure Codes F205, F206

# At a Glance Summary

| Applicable Measure Codes: | F205 | F206 |
| --- | --- | --- |
| Measure Description: | Commercial Fryer (Electric) | Commercial Fryer (Gas) |
| Energy Impact Common Units: | Fryer Vat | Fryer Vat |
| Base Case Description: | Source: PG&E Calculations Existing Electric Fryer | Source: PG&E Calculations Existing Gas Fryer |
| Base Case Energy Consumption: | Source: PG&E Calculations 18,177 kWh/yr | Source: PG&E Calculations 1,688 Therms/yr |
| Measure Energy Consumption: | Source: PG&E Calculations 15,111 kWh/yr | Source: PG&E Calculations 879 Therms/yr |
| Energy Savings (Base Case – Measure) | Source: PG&E Calculations 3,066 kWh/yr | Source: PG&E Calculations 809 Therms/yr |
| Costs Common Units: Source: PG&E Calculations | Fryer | Fryer |
| Base Case Equipment Cost ($/unit): | Source: PG&E Calculations $4,108 | Source: PG&E Calculations $3,367 |
| Measure Equipment Cost ($/unit): | Source: PG&E Calculations $4,876 | Source: PG&E Calculations $4,384 |
| Measure Incremental Cost ($/unit): | Source: PG&E Calculations $769 | Source: PG&E Calculations $1,017 |
| Effective Useful Life (years): | 12 years -- Source: www.Deeresources.com EUL | 12 years -- Source: www.Deeresources.com EUL |
| Program Type: | Replace on Burnout (ROB), and New Construction (NC). | Replace on Burnout (ROB), and New Construction (NC). |
| Net-to-Gross Ratios: | Source: 2014 DEER Com Default >= 2 yrs 0.6 | Source: 2014 DEER Com Default>= 2 yrs 0.6 |

- Work Paper Approvals

The following Manager(s) approved this workpaper through the PG&E Electronic Data Routing System under Routing Requisition # _______________

Grant Brohard Manager, Technical Product Support

Carolyn Weiner Manager, Appliance Products

# Document Revision History

Revision # 		Date 		         Description		         Author (Company)

| Revision 0 | 12/11/2007 | Original work paper: Commercial Fryer - Electric and Gas PGECOFST102 R0.doc | David Zabrowski (Fisher-Nickel, Inc.) |
| --- | --- | --- | --- |
| Revision 1 | 6/1/09 | Changes to EUL, NTG language and references, costs updated, corrections to tables and some language | David Zabrowski, Lauren Mills (Fisher-Nickel, inc.), Steve Blanc PG&E |
| Revision 2 | 3/31/2010 | Updated pricing and incremental cost, Update to DEER 2009-11 NTG file | David Zabrowski (Fisher-Nickel, inc.), Steve Blanc PG&E |
| Revision 3 | 7/15/2010 | Updated EUL and NTG language per ED comments | Charlene Spoor (PG&E) |
|  | 2/17/2011 7/17/2011 | Updated incremental costs Reduced UES by 30% per ED Request | David Zabrowski (Fisher-Nickel, inc.) Charlene Spoor (PG&E) |
| Revision 4 | 6/13/2012 8/22/2012 | Updated NTG, EUL and savings analysis. Combined FST102 and FST114 Changed BLD, CZ and VIN to ANY per READI requirements | David Zabrowski, Kong Sham (Fisher-Nickel, inc.) Charlene Spoor (PG&E) |
| Revision 5 | 3/20/2014 | Updated to new WP template for 7/1/14 filing | Charlene Spoor (PG&E) |

# Table of Contents

At a Glance Summary	ii

Work Paper Approvals	iii

Document Revision History	iv

Table of Contents	v

List of Tables	vi

Section 1. General Measure & Baseline Data	1

1.1 Measure Description & Background	1

1.2 Product Technical Description	2

1.3 Measure Application Types	3

1.4 Product Base Case and Measure Case Data	3

Hours of Operation	3

This Work Paper assumes a 14-hour daily operating period over 365 days per year, or 5,110 hours per year.	3

Base Case & Measure Case Costs	3

Effective Useful Lives	4

The in-service rate was not found in DEER or any supporting documentation. We have therefore assumed the ISR is 1.0 for all measures based on engineering judgment.	4

See section 1.1 Terms and Conditions and Market Applicability to reference the type of program delivery mechanism and customer status used to determine this entry.	4

1.4.2 Codes & Standards Requirements Base Case and Measure Information	4

1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	5

1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases	5

Section 2. Calculation Methods	7

2.1 Electric Energy Savings Estimation Methodologies	7

2.2. Demand Reduction Estimation Methodologies	8

2.3. Gas Energy Savings Estimation Methodologies	8

Section 3. Load Shapes	10

3.1 Base Cases Load Shapes	10

3.2 Measure Load Shapes	10

Section 4. Base Case & Measure Costs	11

4.1 Base Cases Costs	11

4.2 Measure Costs	11

4.3 Incremental & Full Measure Costs	11

Appendix A	13

Appendix B	17

References	21

# List of Tables

Table 1. Energy Efficiency Requirements for Commercial Fryers.	2

Table 2. Measure Application Type	3

Table 3. DEER Use and Technology Table……………………………………………………….3

Table 4. DEER Net To Gross Ratios……………………………………………………………...4

Table 5. DEER Effective Useful Life……………………………………………………………..4

Table 6. Baseline ASTM test results for Commercial Fryers.	5

Table 7. Measure ASTM test results for Commercial Fryers.	5

Table 8. Summary of Inputsfor Savings Calculations	6

Table 9. Commercial Electric Fryer Cost Effectiveness Example.	7

Table 10. Commercial Gas Fryer Cost Effectiveness Example.	9

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the rational for the Energy Efficient Commercial Fryer (Electric and Gas) measure as listed in the Commercial Food Service Catalog. The Commercial Food Service Catalog is part of Pacific Gas and Electric Company’s Customer Energy Efficiency Program. PG&E offers incentives to non-residential customers for installing qualifying lighting, refrigeration, air-conditioning, food service, and agricultural equipment.

Catalog Descriptions

F205: The commercial electric fryer (vat width < 18-inches) must meet ENERGY STAR® specifications for energy efficiency or must have a tested heavy load cooking energy efficiency of 80% and an idle energy rate less than or equal to 1,000 W, utilizing ASTM Standard F1361.  The commercial electric large vat fryer (vat width ≥ 18-inches) must meet ENERGY STAR® specifications for energy efficiency or must have a tested heavy load cooking energy efficiency of 80% and an idle energy rate less than or equal to 1,100 W, utilizing ASTM Standard F2144.  Multiple vat configurations are paid per qualifying vat.

F206: The commercial gas fryer (vat width < 18-inches) must meet ENERGY STAR specifications for energy efficiency or must have a tested heavy load cooking energy efficiency of 50% and an idle energy rate less than or equal to 9,000 Btu/h, utilizing ASTM Standard F1361. The commercial gas large vat fryer (vat width ≥ 18-inches) must meet ENERGY STAR® specifications for energy efficiency or must have a tested heavy load cooking energy efficiency of 80% and an idle energy rate less than or equal to 12,000 Btu/h, utilizing ASTM Standard F2144. Multiple vat configurations are paid per qualifying vat.

Program Restrictions and Guidelines

Terms and Conditions

F 205, F206: The rebate is downstream provided to the customer as the time of sale upon receipt of application and invoice. This is not a direct install program.

This measure includes new commercial electric and gas fryers that are ENERGY STAR® qualified or meet the qualifications listed in Table 1. ENERGY STAR® maintains an updated list of qualifying products and specifications at www.energystar.gov. Consult with the manufacturer or manufacturer’s representative to determine if a non-ENERGY STAR® qualified model meets the efficiency requirements in Table 1. Used or rebuilt equipment is not eligible. Customers must provide proof that the appliance has a cooking-energy efficiency that meets the requirements in Table 1.

Table 1. Energy Efficiency Requirements for Commercial Fryers.

| Measure Code | Fryer Type | Cooking-Energy Efficiency* | Idle Energy Rate |
| --- | --- | --- | --- |
| F205 | Electric Fryers (vat width <18 in.) | ≥ 80% | ≤ 1,000 W |
|  | Electric Fryers (vat width ≥18 in.) | ≥ 80% | ≤ 1,100 W |
| F206 | Gas Fryers (vat width <18 in.) | ≥ 50% | ≤ 9,000 Btu/h |
|  | Gas Fryers(vat width ≥18 in.) | ≥ 50% | ≤ 12,000 Btu/h |

*Based on the Heavy-load French fry test

Market Applicability

This measure is applicable to any commercial cooking application, including (but not limited to) casual dining and quick service restaurants, hotels, motels, schools, colleges and recreational facilities.

## 1.2 Product Technical Description

Commercial fryers are among the most common pieces of cooking equipment in commercial food service facilities. Recent advances in equipment design have produced fryers that operate more efficiently, quickly, safely and conveniently. Energy efficient commercial fryers reduce energy consumption primarily through advanced burner and heat exchanger design, and through the application of advanced controls and insulation. With the availability of ENERGY STAR® rated models of fryers, it is straightforward to differentiate between high efficiency and standard efficiency models.

All fryers share a common basic design. The fry vat contains a sufficient amount of oil so that the cooking food is essentially supported by displacement of the oil rather than by the bottom of the vessel. Fryers are most often compared on the basis of width and energy-input rating. Taken together, these two numbers suggest the approximate amount of food a fryer can prepare in a given time, which is one of the most important factors in choosing the proper fryer for a kitchen. Fryer with a fry vat less than 18-inches wide are evaluated using ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361).

Large vat fryers have frypots ranging from 18 x 14 inches to 34 x 34 inches. The most common of these larger vat sizes is the 18 x 18 inches. Large vat fryers are becoming more common in restaurants as they replace smaller (14-inch) fryers to increase production capability while maximizing the available space in the kitchen. This category has historically been driven by the lowest first cost and, until recently, has traditionally not employed energy-efficient technologies. Large vat fryer performance is determined by applying the ASTM Standard Test Method for the Performance of Large Vat Fryers (F2144).

Energy efficient fryers that have earned the ENERGY STAR® qualification offer shorter cook times and higher production rates. Frypot insulation reduces standby losses resulting in a lower idle energy rate. The ASTM standard test methods are considered the industry standard for quantifying the efficiency and performance of open deep fat fryers.

## 1.3 Measure Application Types

The DEER Measure Cost Data Users Guide found on www.deeresources.com under DEER2014 Database Format hyperlink, DEER2014, spreadsheet SPTdata_format-V0.97.xls, defines the terms as follows:

Table 2 Measure Application Type

Identifies the measure application type in the Measure Implementation table in DEER2014

| Code | Description | Comment |
| --- | --- | --- |
| ER | Early retirement | measure applied while existing equipment still viable, or retrofit of existing equipment |
| ROB | Replace on Burnout | measure applied when existing equipment fails or maintenance requires replacement |
| NC | New Construction | measure applied during construction design phase as an alternative to a code-compliant standard design |

Since there are no EM&V studies on the useful life of convection ovens and it is standard practice in the commercial foodservice industry to purchase equipment only when it is needed (e.g., replacement or additional capacity), this measure is focused on ROB and NC applications only.

## 1.4 Product Base Case and Measure Case Data

1.4.1 DEER Base Case and Measure Case Information

Table 3. DEER Use and Technology Table

The DEER database does not contain information on energy use or savings for an energy-efficient electric or gas convection oven measure. The only reference in DEER for Commercial cooking equipment is for Estimated Useful Life.

- Hours of Operation

- This Work Paper assumes a 14-hour daily operating period over 365 days per year, or 5,110 hours per year.

- Base Case & Measure Case Costs

The base case and measure case costs are calculated and found in section 1.4.4.

Net-to-Gross Assumptions

The 2013-2014 DEER NTGR Values file does not specifically list commercial food service appliances. The default used for non-residential measures is 0.6.

Table 4 summarizes all applicable DEER based Net-to-Gross ratios for programs that may be used by this measure.

Table 4. DEER 2014 Net-to-Gross Ratios

| Program Approach | NTG |
| --- | --- |
| All Com-Default>=2yrs | 0.6 |

- Effective Useful Lives

The 2014 DEER database shows a EUL of 12 years and an RUL of 4 years for all cooking appliance measures, including electric and gas convection ovens.

Table 5. DEER Effective Useful Life

| Measure Code | Building Type | Building Vintage | Climate Zone | EUL (yrs) | RUL (yrs) | DEER Version | Impact IDs |
| --- | --- | --- | --- | --- | --- | --- | --- |
| F205 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |
| F206 | ANY | ANY | PGE | 12 | N/A | DEER2014 | Cooking Equipment |

In Service Rate/ First Year Installation Rate

- The in-service rate was not found in DEER or any supporting documentation. We have therefore assumed the ISR is 1.0 for all measures based on engineering judgment.

- See section 1.1 Terms and Conditions and Market Applicability to reference the type of program delivery mechanism and customer status used to determine this entry.

## 1.4.2 Codes & Standards Requirements Base Case and Measure Information

California Title 20: These measures do not fall under Title 20 of the California Appliance Efficiency Standards and does require reporting the performance the Fryer idle energy rate per ASTMF1361 and ASTM 2144 for the CEC Appliance Database, but the standard contains no minimum performance requirement.

California Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: These measures do not fall under Federal DOE or EPA Energy Regulations.

ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361) and Large Vat Fryer (2144) are applicable for estimating energy use and cooking performance. They are  used to estimate the energy consumption of the base case and measure equipment.

## 1.4.3 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

There were no specific EM&V studies identified that addressed cooking measures in the commercial sector.

## 1.4.4 Assumptions and Calculations from other sources—Base and Measure Cases

This Work Paper uses ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361) and the ASTM Standard Test Method for the Performance of Large Vat Fryers (F2144) for calculation of energy use and demand, based on testing in an approved and qualified laboratory. In the absence of mandatory regulations for testing commercial fryers, there is little incentive on the part of equipment manufacturers to have their baseline equipment tested. Therefore, the ASTM performance parameters for baseline equipment were drawn from a sample of economy grade equipment tested by the Food Service Technology Center and is summarized in Table 6.

Table 6. Baseline ASTM test results for Commercial Fryers.

| Fryer Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Fryers | 1,200 W | 75% | 71 lb/h |
| Gas Fryers | 17,000 Btu/h | 35% | 75 lb/h |

*Based on the Heavy-load French fry test in ASTM F1361.1

The measure case data was drawn from the list of commercial fryers that have been tested by IOU testing laboratories as of April 20, 2012. The complete list is in Appendix A and the averages are summarized in Table 7.

Table 7. Measure ASTM test results for Commercial Fryers.

| Fryer Type | Idle Energy Rate | Cooking-Energy Efficiency* | Production Capacity* |
| --- | --- | --- | --- |
| Electric Fryers | 860 W | 85% | 71 lb/h |
| Gas Fryers | 6,371 Btu/h | 57% | 75 lb/h |

*Based on the Heavy-load French fry test in ASTM F1361.1

The Food Service Technology Center conducted an assessment of major commercial cooking appliance technologies, which included a chapter on fryers. Since commercial fryers are currently not covered by state or national codes, the base case for existing models of fryers was determined from Table 2.1 in the Food Service Technology Center assessment.

This Work Paper assumes a 14-hour daily operating period over 365 days per year, or 5,110 hours per year. This 14-hour operating day is consistent with typical Quick Service Restaurant (QSR) operating hours, which range from 14 to 24 hours per day. Since energy efficient fryers represent a fairly significant initial cost investment over standard fryers, this measure is targeted for heavy-use operations that regularly produce a large volume (e.g., 150 pounds per day per fryer) of fried food. This Work Paper is based on the calculation methods in ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361), which uses measured data under preheat, idle, and heavy-load cooking conditions.

1.5 Summary of Inputs for Savings Calculations

Table 8. Summary of Inputs for Savings Calculations

| Input Variable | Variations | Base Case E Average Value | Base Case G Average Value | Measure Case Average Value F205 | Measure Case Average Value F206 | Refer- ence Section |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Savings* | None | N/A | N/A | 3066 | N/A | N/A |
| Gas Savings* | None | N/A | N/A | N/A | 809 | 1.4.4 |
| Hours of operation | None | 5110 | 5110 | 5110 | 5110 | 1.4.1 |
| Full Cost | None | $4108 | $3367 | $4876 | $4384 | 1.4.4 |
| Increment Cost | None | N/A | N/A | $769 | $1017 | 1.4.4 |
| EUL /RUL | None | 12 | 12 | 12 | 12 | 1.4.1 |
| NTG | None | 0.6 | 0.6 | 0.6 | 0.6 | 1.4.1 |
| ISR | None | 1 | 1 | 1 | 1 | 1.4.1 |

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

The industry standards for energy use and cooking performance of fryers are ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361) and ASTM Standard Test Method for the Performance of Large Vat Fryers (FF2144). Table 9 shows an example of the calculation results for electric fryers based on ASTM test results.

Table 9 Commercial Electric Fryer Cost Effectiveness Example.

| Performance | Baseline Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (kWh) | 2.40 | 1.90 |
| Idle Energy Rate (kW) | 1.20 | 0.86 |
| Heavy Load Cooking Energy Efficiency (%) | 75% | 85% |
| Production Capacity (lbs/hr) | 71 | 71 |
| Operating Hours/Day | 14 | 14 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 150 | 150 |
| Electric Cost ($/kWh) | $0.13 | $0.13 |
| ASTM Energy to Food (kWh/lb) | 0.167 | 0.167 |
| Daily Energy Consumption (kWh) | 49.8 | 41.4 |
| Average Demand (kW) | 3.6 | 3.0 |
| Estimated Demand Reduction (kW) | - | 0.6 |
| Annual Energy Consumption (kWh) | 18,177 | 15,111 |
| Estimated Energy Savings (kWh/yr) | - | 3,066 |
| Annual Energy Cost ($) | 2,363 | 1,964 |
| Estimated Cost Savings ($/yr) | - | $399 |
| Incremental Measure Costa | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)b | 12 years | 12 years |

a Incremental measure cost was determined through communications with local manufacturers and distributors to determine the retail cost to purchase a qualifying model over the baseline standard.

b The estimated useful life is based on the 2014 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (kWh/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (kWh/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (kW) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (kWh) |

## 2.2. Demand Reduction Estimation Methodologies

A fryer’s actual contribution to a building’s peak demand may vary significantly depending on its usage pattern in relation to that of other electric equipment in the facility (operating schedule, appliance on time, etc.). The probability of an appliance drawing its average rate during the period that the building peak is set is significantly higher than for any other input rate for that appliance. Therefore, it has been assumed that the probable contribution to the building’s peak demand is equal to the appliance’s average demand.

The demand reduction estimation is based on measured data for standard efficiency electric fryers (no fry pot insulation and mechanical or simple electronic controls) and for high efficiency fryers that meet Energy Star requirements (greater than 80% cooking efficiency, fry pot insulation, and advanced electronic controls). The measured data are derived from tests conducted under ASTM Standard Test Method for the Performance of Open Deep Fat Fryers (F1361) and ASTM Standard Test Method for the Performance of Large Vat Fryers (F2144).

The ASTM test methods provide standard conditions under which fryer energy use is measured. The estimated demand reduction of 600 Watts is based on data from tests of standard efficiency and high efficiency fryers. Applying a Coincidence Factor of 0.9 per the DEER methodology, yields a Demand Savings of 540 Watts.

## 2.3. Gas Energy Savings Estimation Methodologies

The industry standard for energy use and cooking performance of deep fat fryers is ASTM Standard Test Method for the Performance of Fryers (F1361) and ASTM Standard Test Method for the Performance of Large Vat Fryers (F2144). Table 10 shows an example of the calculation results for gas fryers under ASTM test results.

Table 10. Commercial Gas Fryer Cost Effectiveness Example.

| Performance | Base Model | Energy Efficient Model |
| --- | --- | --- |
| Preheat Time (min) | 15 | 15 |
| Preheat Energy (Btu) | 18,500 | 16,000 |
| Idle Energy Rate (Btu/hr) | 17,000 | 6,371 |
| Cooking-Energy Efficiency (%) | 35% | 55% |
| Production Capacity (lb/hr) | 75 | 75 |
| Operating Hours/Day | 14 | 14 |
| Operating Days/Year | 365 | 365 |
| Pounds of Food Cooked per Day | 150 | 150 |
| Gas Cost ($/therm) | $1.00 | $1.00 |
| ASTM Energy to Food (Btu/lb) | 570 | 570 |
| Daily Energy Consumption (Btu) | 462,536 | 240,859 |
| Annual Energy Consumption (therms)a | 1,688 | 879 |
| Estimated Energy Savings (therms/yr) | - | 809 |
| Annual Energy Cost ($) | $ 1,688 | $879 |
| Estimated Cost Savings ($/yr) | - | $809 |
| Incremental Measure Costb | - | SEE APPENDIX B |
| Estimated Useful Life (EUL)c | 12 years | 12 years |

a 1 therm = 100,000 Btu.

b Incremental measure cost was determined through comparison of an average of published pricing listed in APPENDIX B.

c The estimated useful life is based on the 2014 DEER EUL estimates.

Daily Energy Consumption Calculation and Definitions

EDAY = (LBFOOD x EFOOD) ÷ EFFICIENCY + [IDLERATE x (TON - LBFOOD/PC – nP x TP/60)] 
+ nP x EP

Where:

| EDAY = | Calculated Daily Energy Consumption (Btu/day) |
| --- | --- |
| LBFOOD = | Estimated Pounds of Food Cooked per Day |
| EFOOD = | ASTM Energy to Food (Btu/lb) = kWh/pound of energy absorbed by food product during cooking |
| EFFICIENCY = | Measured Heavy Load Cooking Energy Efficiency % |
| IDLE RATE = | Measured Idle Energy Rate (Btu/h) |
| TON = | Estimated Operating Hours/Day |
| PC = | Measured Production Capacity (lbs/hr) |
| TP = | Estimated Preheat Time (min) |
| nP = | Estimated Number of preheats/Day |
| EP = | Measured Preheat Energy (kWh) |

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Cases Load Shapes

The base case load shape would be expected to follow a typical nonresidential foodservice end use load shape.

Commercial fryer load shapes  differ among food service facilities (quick service, casual dining, hotels, college, schools, hospitals etc) depending on daily menu variations, hours of operation, serving periods, day-of-week, and facility location (city downtown, suburban mall, access to interstate highways, etc.). Consequently, applicable average TOU and hourly load shapes for fryers are unavailable. The ASTM Standard Test Method used to generate energy use data for evaluation against the Energy Star program is based on hours of use and food cooking condition (idle, light, and heavy). Generally, fryers are used to prepare food shortly before it is served, so loads tend to increase shortly before and during regular meal periods (breakfast, lunch, and dinner). Between meal periods fryer use tends to be either idle or light.

## 3.2 Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for this measure is determined by the E3 calculator based on the applicable nonresidential market sector and the foodservice end-use.

The electric demand profile for the high efficiency electric fryer is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high efficiency fryer will use less energy and have a slightly lower demand profile.

The gas load profile for the high-efficiency gas fryer is expected to be the same as the Base Case. The profile will vary as explained in Section 3.1. The Measure Load Shape for the high-efficiency fryer will use less energy.

# Section 4. Base Case & Measure Costs

High efficiency fryers typically list for more than standard efficiency fryers. However, high efficiency designs are often bundled with other features such as all stainless steel construction and high quality components and controls. In addition to lower operating costs, high efficiency fryers exhibit higher production rates and shorter recovery times than Baseline model fryers and in some cases may eliminate the need for a backup fryer.

Equipment prices for these work papers were compiled from a number of sources including, Autoquotes, equipment sales reps and manufacturer sources. Since equipment pricing in food service is closely held information and prices vary widely according to buying volume and other factors, we cannot list the sources for prices specifically.

## 4.1 Base Cases Costs

The Base Case costs include only the equipment. High efficiency fryers require no additional labor or maintenance compared to base case fryers. Since this measure is applicable for ROB and NC installations, the installation and maintenance costs are expected to be the same for the customer. The estimated equipment cost is based on recent list cost data for electric and gas fryers and applying an industry-standard 50% discount to the manufacturer published list prices.

## 4.2 Measure Costs

The Measure costs include only the equipment, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 50% discount to the manufacturer published list prices (see Appendix B).

## 4.3 Incremental & Full Measure Costs

Incremental costs are used in the analysis.

Index

DEER Measure Cost Data Users Guide, 3

load shape, 9

Measure Load Shape, 9, 10

New Construction, ii, 3

Replace on Burnout, ii, 3

# Appendix A

ASTM Qualifying Equipment Data for Electric Fryers

| Fryer Size (inches) | Company | Model Number | Idle Energy Rate (kW) | Cooking-Energy Efficiency (%) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| 14 | Frymaster | LOV/ BIELA*14 | 0.79 | 86% | 68 |
| 14 | Frymaster | BIRE*14 | 0.82 | 84% | 68 |
| 14 | Frymaster | BK*17C | 0.89 | 87% | 70 |
| 14 | Frymaster | BK*17RC | 0.89 | 87% | 70 |
| 14 | Frymaster | BKFP*17C | 0.89 | 87% | 70 |
| 14 | Frymaster | BKFP*17RC | 0.89 | 87% | 70 |
| 14 | Frymaster | Protector™ / FPEL*14 | 0.79 | 86% | 68 |
| 14 | Frymaster | Protector™/ FPEL*17 | 0.81 | 86% | 77 |
| 14 | Frymaster | Protector™/ FPEL*22 | 0.81 | 87% | 83 |
| 14 | Frymaster | H*14 | 0.97 | 81% | 68 |
| 14 | Frymaster | H*17TC | 0.77 | 84% | 72 |
| 14 | Frymaster | RE*14 | 0.82 | 84% | 68 |
| 14 | Frymaster | RE*17 | 0.89 | 87% | 70 |
| 14 | Frymaster | RE*22 | 0.86 | 84% | 76 |
| 18 | Frymaster | EH117 | 1.2 | 83% | 84 |
| 20 | Frymaster | FPC228 | 1.47 | 85% | 128 |
| 14 | Giles Enterprises, Inc. | GBF-50 | 0.83 | 82% | 66 |
| 14 | Giles Enterprises, Inc. | GEF-400 | 0.70 | 86% | 53 |
| 20 | Giles Enterprises, Inc. | EOF-10 | 1.12 | 82% | 115 |
| 24 | Giles Enterprises, Inc. | EOF-24 | 0.95 | 83% | 150 |
| 18.5 | Giles Enterprises, Inc. | GEF-560 | 0.83 | 89% | 77 |
| 18.5 | Giles Enterprises, Inc. | GEF-720 | 0.93 | 85% | 96 |
| 14 | Henny Penny | Evolution Elite/ EEE-14* | 0.81 | 85% | 64 |
| 14 | Henny Penny | LVE | 0.81 | 85% | 64 |
| 14 | Henny Penny | OEA-32* (14.4kW) | 0.91 | 80% | 63 |
| 14 | Henny Penny | OEA-32* (22kW) | 0.75 | 83% | 69 |
| 14 | Henny Penny | OFE-32* (14.4kW) | 0.91 | 80% | 63 |
| 14 | Henny Penny | OFE-32* (22kW) | 0.75 | 83% | 69 |
| 14 | Henny Penny | OFE-32* Split Vat | 0.70 | 82% | 70 |
| 18 | Henny Penny | OEA-34* | 1.08 | 86% | 84 |
| 18 | Henny Penny | OFE-34* | 1.08 | 86% | 84 |
| 14 | Hobart | HF50 | 0.63 | 85% | 72 |
| 14 | Motion Technology, Inc. | AutoFry/ MTI-10XL | 0.96 | 83% | 34 |
| 14 | Motion Technology, Inc. | AutoFry/MTI-40C | 0.94 | 92% | 58 |
| 14 | Motion Technology, Inc. | AutoFry/MTI-40E | 0.94 | 92% | 58 |
| 14 | Perfect Fry Company | RapidFry/ PFA7200 | 0.59 | 86% | 35 |
| 14 | Perfect Fry Company | PFC5700 | 0.53 | 81% | 29 |
| 14 | Pitco | BKSEH50SPC | 0.93 | 82% | 69 |
| 14 | Pitco | MEII | 0.93 | 82% | 69 |
| 14 | Pitco | SE148 SEF148 SFSE148 | 0.99 | 85% | 77 |
| 14 | Pitco | SEH50 | 0.93 | 82% | 69 |
| 18 | Pitco | SE18 | 1.29 | 85% | 105 |
| 20 | Pitco | SEM20 | 1.58 | 88% | 92 |
| 18 | Pitco | SFSE1814 | 1.07 | 84% | 111 |
| 18 | Ultrafryer Systems | E-15-18 | 1.2 | 91% | 117 |
| 18 | Ultrafryer Systems | E-20-18 | 1.08 | 83% | 93 |
| 14 | Vulcan | ER50 | 0.63 | 85% | 72 |
| Average Performance |  |  | 0.91 | 85% | 96 |

ASTM Qualifying Equipment Data for Gas Fryers

| Fryer Size (inches) | Company | Model Number | Idle Energy Rate (Btu/lb) | Cooking-Energy Efficiency (%) | Production Capacity (lbs/h) |
| --- | --- | --- | --- | --- | --- |
| 14 | Alto-Shaam | ASF-60G | 4,072 | 66% | 82 |
| 22 | Alto-Shaam | ASF-75G | 4,825 | 71% | 134 |
| 14 | Dean | Decathlon/ HD*50G | 8,386 | 52% | 72 |
| 18 | Dean | HDx60G | 10,242 | 52% | 107 |
| 18 | Dean | HDx63G | 17,411 | 51% | 99 |
| 14 | Frymaster | LOV/BIGLA*30 | 4,382 | 56% | 68 |
| 14 | Frymaster | BIH*55 | 5,604 | 55% | 69 |
| 14 | Frymaster | BK*55C | 5,604 | 55% | 69 |
| 14 | Frymaster | BK*55RC | 5,604 | 55% | 69 |
| 14 | Frymaster | BK55C | 5,604 | 55% | 69 |
| 14 | Frymaster | BK55RC | 5,604 | 55% | 69 |
| 14 | Frymaster | BKFP*55C | 5,604 | 55% | 69 |
| 14 | Frymaster | BKFP*55RC | 5,604 | 55% | 69 |
| 14 | Frymaster | Protector/FPGL*30 | 4,382 | 56% | 68 |
| 14 | Frymaster | FPH50 | 6,400 | 50% | 57 |
| 14 | Frymaster | H*55 | 5,604 | 55% | 69 |
| 14 | Frymaster | H55 | 5,604 | 55% | 69 |
| 14 | Frymaster | MJH*50 | 6,400 | 50% | 57 |
| 14 | Frymaster | MPH*55 | 5,604 | 55% | 69 |
| 14 | Frymaster | PMJH*50 | 6,400 | 50% | 57 |
| 14 | Frymaster | YSCFH*14G | 8,386 | 52% | 72 |
| 18 | Frymaster | HD1814 | 14,579 | 52% | 128 |
| 14 | Giles Enterprises, Inc. | GGF-400 | 3,862 | 54% | 36 |
| 18.5 | Giles Enterprises, Inc. | GGF-720 | 5,645 | 61% | 80 |
| 14 | Henny Penny | Evolution Elite /EEG-14* | 5,790 | 54% | 64 |
| 14 | Henny Penny | LVG | 5,790 | 54% | 64 |
| 14 | Henny Penny | OFG-32* | 7,040 | 50% | 62 |
| 14 | Henny Penny | OGA-32* | 7,040 | 50% | 62 |
| 18 | Henny Penny | OFG-34* | 23,652 | 51% | 84 |
| 18 | Henny Penny | OGA-34* | 23,652 | 51% | 84 |
| 14 | Hobart | HK45 | 4,636 | 65% | 78 |
| 19.5 | Hobart | HK65 | 4,838 | 66% | 91 |
| 19.5 | Hobart | HK85 | 6,075 | 66% | 100 |
| 14 | Keating | Incredible Flying Machine/ IFM 14 | 4,700 | 57% | 57 |
| 14 | Paloma | PF-*S | 3,420 | 62% | 62 |
| 14 | Pitco | BKSGH50SPC | 8,510 | 54% | 67 |
| 14 | Pitco | MGII | 8,510 | 54% | 67 |
| 14 | Pitco | SGC | 4,899 | 50% | 55 |
| 14 | Pitco | SGH50 | 8,510 | 54% | 67 |
| 14 | Pitco | SSH55 SFSSH55 SSHF55 | 8,140 | 55% | 72 |
| 14 | Pitco | SSH60 SFSSH60 SSHF60 | 8,705 | 58% | 76 |
| 14 | Pitco | SSH60R SFSSH60R SSHF60R | 8,446 | 52% | 74 |
| 14 | Pitco | Frialator/ VF35 | 7,349 | 50% | 59 |
| 20 | Pitco | SGM20 SFSGM20 SGMF20 | 11,206 | 57% | 90 |
| 24 | Pitco | SGM24 SFSGM24 SGMF24 | 15,356 | 52% | 146 |
| 18 | Pitco | SSH60W SFSSH60W SSHF60W | 10,669 | 57% | 76 |
| 18 | Pitco | SSH75 SFSSH75 SSHF75 | 11,226 | 61% | 99 |
| 18 | Pitco | SSH75R SFSSH75R SSHF75R | 11,819 | 56% | 107 |
| 14 | Ultrafryer Systems | Par 3 /C-P30-14 | 4,120 | 60% | 60 |
| 14 | Ultrafryer Systems | Par 3/ F-P30-14 B-P30-14 | 4,180 | 65% | 73 |
| 14 | Ultrafryer Systems | Par 3/ PDEX 1V 1-14 | 4,180 | 65% | 73 |
| 18 | Ultrafryer Systems | F-P20-18 B-P20-18 | 6,281 | 59% | 67 |
| 18 | Ultrafryer Systems | F-P25-18 B-P25-18 | 5,710 | 59% | 89 |
| 18 | Ultrafryer Systems | F-P30-18 B-P30-18 | 5,993 | 61% | 120 |
| 18 | Ultrafryer Systems | P40-18 | 4,909 | 74% | 104 |
| 14 | Vulcan | VK45 | 4,636 | 65% | 78 |
| 19.5 | Vulcan | VK65 | 4,838 | 66% | 91 |
| 19.5 | Vulcan | VK85 | 6,075 | 66% | 100 |
| Average Performance |  |  | 7,169 | 57% | 84 |

# Appendix B

Equipment prices for these work papers were compiled from a number of sources including, Autoquotes, equipment sales reps and manufacturer sources. Since equipment pricing in food service is closely held information and prices vary widely according to buying volume and other factors, we cannot list the sources for prices specifically.

Equipment Cost Data for Electric Fryers

| Size | Group | Model | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- |
| Fryer | Baseline | B1 | $5,845 | $2,923 |
| Fryer | Baseline | B2 | $5,845 | $2,923 |
| Fryer | Baseline | B3 | $9,543 | $4,772 |
| Fryer | Baseline | B4 | $8,879 | $4,440 |
| Fryer | Baseline | B5 | $10,090 | $5,045 |
| Fryer | Baseline | B6 | $9,515 | $4,758 |
| Fryer | Baseline | B7 | $10,364 | $5,182 |
| Fryer | Baseline | B8 | $4,861 | $2,431 |
| Fryer | Baseline | B9 | $5,481 | $2,741 |
| Fryer | Baseline | B10 | $4,891 | $2,446 |
| Large Vat | Baseline | B11 | $9,386 | $4,693 |
| Large Vat | Baseline | B12 | $10,550 | $5,275 |
| Large Vat | Baseline | B13 | $13,821 | $6,911 |
| Large Vat | Baseline | B14 | $7,870 | $3,935 |
| Large Vat | Baseline | B15 | $8,216 | $4,108 |
| Large Vat | Baseline | B16 | $6,156 | $3,078 |
| Large Vat | Baseline | B17 | $6,218 | $3,109 |
| Large Vat | Baseline | B18 | $7,204 | $3,602 |
| Large Vat | Baseline | B19 | $8,031 | $4,016 |
| Large Vat | Baseline | B20 | $9,938 | $4,969 |
| Large Vat | Baseline | B21 | $9,810 | $4,905 |
| Fryer | Energy Efficient | EE1 | $6,850 | $3,425 |
| Fryer | Energy Efficient | EE2 | $9,800 | $4,900 |
| Fryer | Energy Efficient | EE3 | $7,920 | $3,960 |
| Fryer | Energy Efficient | EE4 | $10,830 | $5,415 |
| Fryer | Energy Efficient | EE5 | $8,696 | $4,348 |
| Fryer | Energy Efficient | EE6 | $11,580 | $5,790 |
| Fryer | Energy Efficient | EE7 | $4,414 | $2,207 |
| Fryer | Energy Efficient | EE8 | $17,762 | $8,881 |
| Fryer | Energy Efficient | EE9 | $15,806 | $7,903 |
| Fryer | Energy Efficient | EE10 | $15,804 | $7,902 |
| Fryer | Energy Efficient | EE11 | $9,400 | $4,700 |
| Fryer | Energy Efficient | EE12 | $8,548 | $4,274 |
| Fryer | Energy Efficient | EE13 | $7,926 | $3,963 |
| Fryer | Energy Efficient | EE14 | $8,524 | $4,262 |
| Fryer | Energy Efficient | EE15 | $6,566 | $3,283 |
| Fryer | Energy Efficient | EE16 | $7,576 | $3,788 |
| Fryer | Energy Efficient | EE17 | $7,854 | $3,927 |
| Large Vat | Energy Efficient | EE18 | $9,692 | $4,846 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Cost Data for Gas Fryers

| Size | Group | Model | List Price ($) | Cost($)* |
| --- | --- | --- | --- | --- |
| Fryer | Baseline | B1 | $3,950 | $1,975 |
| Fryer | Baseline | B2 | $3,350 | $1,675 |
| Fryer | Baseline | B3 | $6,890 | $3,445 |
| Fryer | Baseline | B4 | $9,950 | $4,975 |
| Fryer | Baseline | B5 | $2,435 | $1,218 |
| Fryer | Baseline | B6 | $3,570 | $1,785 |
| Fryer | Baseline | B7 | $5,026 | $2,513 |
| Fryer | Baseline | B8 | $7,791 | $3,896 |
| Fryer | Baseline | B9 | $8,342 | $4,171 |
| Fryer | Baseline | B10 | $5,504 | $2,752 |
| Fryer | Baseline | B11 | $4,538 | $2,269 |
| Fryer | Baseline | B12 | $8,591 | $4,296 |
| Fryer | Baseline | B13 | $5,822 | $2,911 |
| Fryer | Baseline | B14 | $5,000 | $2,500 |
| Fryer | Baseline | B15 | $8,920 | $4,460 |
| Fryer | Baseline | B16 | $5,738 | $2,869 |
| Fryer | Baseline | B17 | $8,059 | $4,030 |
| Fryer | Baseline | B18 | $7,819 | $3,910 |
| Fryer | Baseline | B19 | $8,059 | $4,030 |
| Fryer | Baseline | B20 | $4,419 | $2,210 |
| Fryer | Baseline | B21 | $4,982 | $2,491 |
| Fryer | Baseline | B22 | $10,063 | $5,032 |
| Fryer | Baseline | B23 | $5,082 | $2,541 |
| Fryer | Baseline | B24 | $5,729 | $2,865 |
| Fryer | Baseline | B25 | $7,393 | $3,697 |
| Fryer | Baseline | B26 | $3,772 | $1,886 |
| Fryer | Baseline | B27 | $6,654 | $3,327 |
| Fryer | Baseline | B28 | $6,047 | $3,024 |
| Fryer | Baseline | B29 | $7,687 | $3,844 |
| Fryer | Baseline | B30 | $7,920 | $3,960 |
| Fryer | Baseline | B31 | $6,860 | $3,430 |
| Fryer | Baseline | B32 | $7,556 | $3,778 |
| Fryer | Baseline | B33 | $2,628 | $1,314 |
| Fryer | Baseline | B34 | $3,264 | $1,632 |
| Large Vat | Baseline | B35 | $5,086 | $2,543 |
| Large Vat | Baseline | B36 | $6,249 | $3,125 |
| Large Vat | Baseline | B37 | $10,382 | $5,191 |
| Large Vat | Baseline | B38 | $8,652 | $4,326 |
| Large Vat | Baseline | B39 | $6,986 | $3,493 |
| Large Vat | Baseline | B40 | $7,826 | $3,913 |
| Large Vat | Baseline | B41 | $5,778 | $2,889 |
| Large Vat | Baseline | B42 | $6,686 | $3,343 |
| Large Vat | Baseline | B43 | $9,510 | $4,755 |
| Large Vat | Baseline | B44 | $5,596 | $2,798 |
| Large Vat | Baseline | B45 | $8,071 | $4,036 |
| Large Vat | Baseline | B46 | $6,670 | $3,335 |
| Large Vat | Baseline | B47 | $4,842 | $2,421 |
| Large Vat | Baseline | B48 | $6,306 | $3,153 |
| Large Vat | Baseline | B49 | $7,277 | $3,639 |
| Large Vat | Baseline | B50 | $9,670 | $4,835 |
| Large Vat | Baseline | B51 | $4,570 | $2,285 |
| Large Vat | Baseline | B52 | $7,687 | $3,844 |
| Large Vat | Baseline | B53 | $11,188 | $5,594 |
| Large Vat | Baseline | B54 | $17,540 | $8,770 |
| Large Vat | Baseline | B55 | $8,816 | $4,408 |
| Large Vat | Baseline | B56 | $9,708 | $4,854 |
| Large Vat | Baseline | B57 | $3,010 | $1,505 |
| Large Vat | Baseline | B58 | $3,950 | $1,975 |
| Fryer | Energy Efficient | EE1 | $11,170 | $5,585 |
| Fryer | Energy Efficient | EE2 | $6,278 | $3,139 |
| Fryer | Energy Efficient | EE3 | $10,968 | $5,484 |
| Fryer | Energy Efficient | EE4 | $10,100 | $5,050 |
| Fryer | Energy Efficient | EE5 | $6,466 | $3,233 |
| Fryer | Energy Efficient | EE6 | $5,562 | $2,781 |
| Fryer | Energy Efficient | EE7 | $10,538 | $5,269 |
| Fryer | Energy Efficient | EE8 | $9,796 | $4,898 |
| Fryer | Energy Efficient | EE9 | $10,764 | $5,382 |
| Fryer | Energy Efficient | EE10 | $9,004 | $4,502 |
| Fryer | Energy Efficient | EE11 | $7,284 | $3,642 |
| Fryer | Energy Efficient | EE12 | $7,284 | $3,642 |
| Large Vat | Energy Efficient | EE13 | $11,945 | $5,973 |
| Large Vat | Energy Efficient | EE14 | $7,340 | $3,670 |
| Large Vat | Energy Efficient | EE15 | $7,854 | $3,927 |
| Large Vat | Energy Efficient | EE16 | $7,932 | $3,966 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

Equipment Incremental Cost Data for Energy Efficient Fryers*

| Category | Baseline Unit Price | Energy Efficient Unit Price | Incremental Price Difference | Baseline Unit Cost | Energy Efficient Unit Cost | Incremental Measure Cost (IMC) |
| --- | --- | --- | --- | --- | --- | --- |
| Electric Fryer | $8,215 | $9,753 | $1,537 | $4,108 | $4,876 | $769 |
| Gas Fryer | $6,733 | $8,768 | $2,034 | $3,367 | $4,384 | $1,017 |

*Estimated purchase price and Incremental Measure Cost (IMC) were based on an industry-standard 50% discount off the manufacturer’s list price.

# References

American Society for Testing and Materials. Standard Test Method for the Performance of Open Deep Fat Fryers. ASTM Designation F1361, in Annual Book of ASTM Standards, West Conshohocken, PA.

American Society for Testing and Materials. Standard Test Method for the Performance of Large Vat Fryers. ASTM Designation F2144, in Annual Book of ASTM Standards, West Conshohocken, PA.

2003 Energy Star® Program Requirements for Commercial Fryers. http://www.energystar.gov/index.cfm?c=fryers.pr_crit_fryers

The DEER Measure Cost Data Users Guide found on www.deeresources.com under DEER2011 Database Format hyperlink, DEER2011 for 13-14, spreadsheet SPTdata_format-V0.97.xls.

2008 Database for Energy-Efficient Resources Version 2008.2.05  December 16, 2008; www.deeresources.com – EUL/RUL (Effective/Remaining Useful Life) Values --EUL/RUL Values and Summary Documentation (Updated 10 October 2008) Excel Spreadsheet --http://www.deeresources.com/deer0911planning/downloads/EUL_Summary_10-1-08.xls (Non-Residential/Cooking )

DEER2014_NTGR_2012-05-16.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: DEER2014 Update Net-To-Gross table

EUL_Summary_10-1-08.xls from  DEER Database for Energy-Efficient Resources; Version 2014 1.0 found at: http://www.deeresources.com/2014 Under: DEER2014 Update Documentation linked at: EUL/RUL Values Cells: (D82)

2005 California Energy Commission (CEC) Title 20 Appliance Efficiency Regulations, CEC 400-2005-012, p. 69.

Fisher-Nickel, inc., D. Fisher, et al., 2002. Commercial Cooking Appliance Technology Assessment. Food Service Technology Center Report 5011.02.26, pp. 2-1 to 2-24.

2004-2005 Database for Energy Efficiency Resources (DEER) Update Study Final Report, pp. 3-15 to 3-18, table 3-14.

AutoQuotes electronic catalog for foodservice equipment and supplies http://www.aqnet.com/