Work Paper PGECOREF111

Vending Machine Controller

Revision 5

Pacific Gas & Electric Company

Customer Energy Solutions

Vending Machine

Controller

# Measure Code R86, R98

PGECOREF111 R5 Vending Machine Controller

PG&E is using the SCE work paper Work Paper SCE15CS005.2 ex-ante values for PG&E measure codes R86 and R98.

The measure mapping is as follows:

PG&E Measure code R86 = SCE code RF-75217

PG&E Measure code R98 = SCE code LT-85945

Work Paper SCE13CS005

Revision 2

Southern California Edison

Beverage Merchandise Controller

For Work Paper Reviewer Use Only

List all major comments that occurred during the review. This table may only be removed during management review.

| Major Comment | Reviewer Name | Date | Outcome/Resolution |
| --- | --- | --- | --- |
| E.g. Please remove measure LT-12345 (LD123) from this work paper because it is no longer eligible for incentives. | Reviewer 1 | 6/1/15 | E.g. Comment incorporated. LT-12345 was removed. |

# At-a-Glance Summary

| Measure Codes | RF-75217, LT-85945, RF-48900, RF-56733, RF-65065, RF-70008 |
| --- | --- |
| Measure Description | An occupancy sensor based control for reducing lighting and refrigeration consumption of vending machines and beverage merchandise coolers |
| Base Case Description | No controls in place |
| Units | Sensor |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | Plug-VendCtrler: 5 years |
| Measure Installation Type | Retrofit Add-on (REA) |
| Net-to-Gross Ratio | Refer to Section 1.4.1 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/10/11 | Sean Gouw/SCE | Original workpaper from WPSCNRCS0005.0, updated to new template |
|  | 06/19/12 | Alfredo Gutierrez/SCE | Added the following DEER 2005 measures: - Cold Vending Machine Controls (D03-912) - Snack Vending Machine Controls (D03-913) |
| 1 | 06/19/14 | Jack Melnyk/SCE | - Updated Tables titles to agree with 2014 CA T20 - Updated savings for RF-75217 and LT-85945 Updated the E3BT for Clinic to be Small _Office |
| 2 | 10/29/15 | Jason Wang (SCE) | - Updated the HOU for DEER2016 - Updated Title 20 requirements for canned vending machines |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | - Self-contained beverage merchandise cooler with an occupancy sensor-based beverage merchandise controller (BMC) - Self-contained vending machine with an occupancy sensor-based vending machine controller (VMC). |
| Existing Condition | - Beverage merchandise cooler without BMC - Vending machine without VMC |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | RF-48900 |  | Double Door Beverage Merchandise Cooler Control |
|  |  | RF-56733 |  | One Door Beverage Merchandise Cooler Control |
|  |  | RF-65065 |  | One Door Under Counter Beverage Merchandise Cooler Control |
|  |  | RF-70008 |  | Triple Door Beverage Merchandise Cooler Control |
|  |  | RF-75217 |  | Cold Vending Machine Controls |
|  |  | LT-85945 |  | Snack Vending Machine Controls |

Eligibility Requirements

Building types:

- All measures are eligible for all non-residential building types.

- The vending machine measures are also eligible for the Residential Multi-family and Residential Mobile Home - Double-Wide building types.

Existing beverage merchandise coolers must:

- Have either glass sliding doors or glass pull-open doors

- Have a self-contained condensing unit

- Be used to maintain temperatures for non-perishable products

BMCs must:

- Shut down the beverage merchandise cooler when occupancy/traffic drops below a certain threshold

- Periodically power the cooler back on at a maximum increment of every 4 hours to reasonably maintain product temperatures

- Be rated for 120 VAC applications

For cold vending machines and VMCs (RF-75217) the Express program has the following requirements:

- The refrigerated vending machines must contain only non-perishable bottled and canned beverages. If a vending machine is being installed, the equipment must be Energy Star qualified.

- The controller must turn off the lights and the compressor when the surrounding area is unoccupied for 15 minutes to 1 hour, or if there is no sales traffic for a given period of time.

- Control logic must periodically power up the machine at regular intervals to maintain product temperature and provide compressor protection. Refurbished vending machines that include this option are eligible.

## 1.2 Technical Description

A beverage merchandise cooler is a commercial reach-in refrigerator with transparent doors and a self-contained condensing unit. The beverage merchandise cooler maintains product temperatures for non-perishable goods. It typically has lighting to illuminate the product.

A BMC is a device with a passive infrared occupancy sensor, a duplex receptacle, and a power cord for connecting the device to 120V power. The energy savings are achieved by shutdown of the cooler when there are no occupants present. During the unoccupied periods, the controller measures ambient air temperature outside of the case and periodically turns on the cooler to maintain preset product temperatures. The controller does not interrupt cooling cycles when shutting down the cooler.

A vending machine is similar to a beverage merchandise cooler but dispenses product through a coin or bill-operated interface. Snack vending machines are typically not refrigerated. Vending machine controllers shut down compressors and/or lighting during periods of non-use.

## 1.3 Installation Types and Delivery Mechanisms

The delivery mechanisms are:

  - Financial Support - Direct Install

  - Financial Support – Downstream Deemed

The program type/install type is Retrofit Add-On (REA).

Notes regarding the REA install type

The Non Residential Direct Install program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is a form which information for measures installed in all building types related to the Non Residential Direct Install program.  The PLF contains the following fields: Service Account Address, Measures Proposed/Installed, Product Make/Model, Install Locations (detailed to define separate spaces/floors, as well as specific locations within the space including but not limited to:  Bathrooms, Hallways, Meeting Rooms, Offices, Warehouse, etc.).

Customers are solicited to participate primarily through field visits.  Contractors conduct energy consultation and provide recommendation that can help eligible customers use less energy.  If the customer agrees, the Direct Install contractor will help them complete an authorization form and schedule an installation appointment.

SCE can also provide photos to show measure functionality and a sample close up photos to substantiate the measure base case where applicable.  These photos would be part of the required project package.  The project package is identified with the Service Account Number and attached to SCE’s SMART database (SCE Project Tracking System) at each specific project level. The above described information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | DEER2016 does not contain these measures. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |

Notes regarding the Direct Install incentive method

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

- Business Size: Customer must have less than ten employees

- Language: Customer’s primary language spoken is not English

- Geography: Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area, the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.

The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties.

The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| Plug-VendCtrler | Vending Machine Controller | Com | Vending | 5 | 1.7 |

### 1.4.2 Codes and Standards Analysis

This specific measure of adding controls on both vending machines and beverage merchandise coolers is not regulated under 2015 California Appliance Efficiency Regulations (Title 20) [493] or the Code of Federal Regulations Title 10 [393].  Both CA Title 20 and CFR Title 10 do contain standards for daily energy use for both types of equipment as well as Title 20 including a requirement for vending machines to have manufacturer-equipped hardware/software capable of automated low-power mode operation. However, these requirements do not apply to the beverage coolers targeted for this measure.

For beverage merchandise coolers the maximum daily energy consumption set up by the federal requirements in CFR Title 10 Part 431 Subpart C §431.66 (b) are identical to the requirements in CA Title 20 Table A-4. For purposes of the calculations in Section 2, CA Title 20 is referenced.

For canned vending machines the maximum daily energy consumption set up by the federal requirements in CFR Title 10 Part 431 Subpart Q §431.296 (b) are identical to the requirements in CA Title 20 Table A-8. For purposes of the calculations in Section 2, CA Title 20 is referenced.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Table A-4 Maximum daily energy consumption (kWh) for refrigerators with transparent doors designed for pull-down applications | January 1, 2010 |
| Title 20 (2015) | Table A-8 Standards for Refrigerated Canned and Bottled Beverage Vending Machines | August 31, 2012 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

### 1.5.1 2004-2005 DEER Update Study Final Report [26]

The 2004-2005 DEER report documented the assumptions used in the development of DEER 2005 measures. It was used to reference typical operating characteristics of vending machine controllers, as well as their maximum sleep time duration.

## 1.6 Data Quality and Future Data Needs

No future data needs are anticipated at this time.

# Section 2. Calculation Methodology

## The energy savings for the measures in this workpaper will be derived from four different calculations.

- The reduction in lighting use from turning off an interior display light

- The reduction in lighting use from turning off a backlit display

- The reduction in refrigeration time for Title 20 defined refrigerated canned and bottled beverage vending machines

- The reduction in refrigeration time for commercial refrigerators, commercial refrigerator-freezers, and commercial freezers

Table 1 below depicts which measures rely on which of the energy calculations listed above.

Table 1 Measure Calculations

| Solution Code | Measure | Interior Display Light | Backlit Signage | Vending Machine | Com Refrig/Freezer |
| --- | --- | --- | --- | --- | --- |
| RF-48900 | Double Door Beverage Merchandise Cooler Control |  |  |  | Yes |
| RF-56733 | One Door Beverage Merchandise Cooler Control |  |  |  | Yes |
| RF-65065 | One Door Under Counter Beverage Merchandise Cooler Control |  |  |  | Yes |
| RF-70008 | Triple Door Beverage Merchandise Cooler Control |  |  |  | Yes |
| RF-75217 | Cold Vending Machine Controls |  | Yes | Yes |  |
| LT-85945 | Snack Vending Machine Controls | Yes |  |  |  |

Interactive Effects

Interactive effects will not be applied to the energy savings derived for the measures in the workpaper. The function of occupancy based control of vending machines and beverage cabinets is to turn the equipment off when the building is not occupied. To apply interactive effect savings, the assumption that the HVAC equipment is run the same when facility is closed as it is when it is open would have to be made. The most likely situation would be the interactive effects somewhere between the DEER derived values and none at all. To ensure conservative savings estimates, interactive effects will not be applied.

Demand Reduction

As discussed in the 2004-2005 DEER Update Study Final Report [26] on Vending Machine Control (VMC), the VMC is expected to operate primarily during off-peak hours and therefore no demand reduction will occur during the DEER peak period. This statement can be applied to the BMC due to its similar operation and application.

While there will be some savings due to occupancy during the DEER defined peak, there is no substantial evidence available to apply lighting coincident diversity factors to vending and beverage cabinet control. It is determined that the demand reduction by the vending and beverage machine control is Zero.

## (1) Energy savings for the reduction in lighting use from turning off an interior display light

## In uncooled snack vending machines, a display light typically illuminates the products inside the vending machine. For purposes of these calculations the illumination will be assumed to be provided by one 2 foot T8 linear fluorescent lamp, fixture F2ILL at 20 Watts from Appendix B Table of Standard Fixture Wattages and Sample Lighting Table [297].

To estimate the specific energy savings, the DEER2016 linear fluorescent lighting effective full load operating hours will be used to represent when the unit is assumed to be enabled. The savings for occupancy based vending control will occur in a load profile complimentary to an 8760 load shape. For example, in a small office the DEER2016 occupancy sensor based lighting effective full load operating hours is 1,760 hours, therefore the time the vending machine control would turn the internal lights off would be 8,760 hours minus 1,760  hours, resulting in 7,000 hours of off time. See Equation 1 below for the general form.

Equation 1

Energy Saving=8,760 hrs-DEER OccSnsrBldgHrs×Fixture Wattage

Detailed Calculation results located in calculation template in the Attachment Section.

## (2) Energy savings for the reduction in lighting use from turning off a backlit display

## The logic for the interior display lighting the backlit display will be calculated with the same strategy as the interior display light. The only deviation will be the wattage controlled. According to E Source tech Update TU-96-7 [392], the typical backlit display for a cooled beverage vending machine consists of two 5 foot linear fluorescent lamps. The two options available for this application are T5 and T8 lamps. Using the Table of Standard Fixture Wattages the T5 fixture (F52PL) consumes 78 Watts and the T8 fixture (F52ILL) consumes 72 Watts. For purposes of the calculation, the T8 fixture at 72 watts will be used. Refer to Equation 1 for the general form of the savings calculation.

## (3) Energy savings for the reduction in refrigeration time for Title 20 defined refrigerated canned and bottled beverage vending machines

In the 2004-2005 DEER Update Study Final Report on VMC, a maximum of 4 hours of “sleep mode” per day is applied when there are no occupants present in the area of the vending machine. This maximum sleep mode duration is adopted as the usage reduction rate in this work paper.

## The base case energy usage is established by using the CEC Title 20 requirements [493] for refrigerated canned and bottled beverage vending machines, Section 1605.3 Table A-8. Requirements are provided for:

- Class A: A refrigerated bottled or canned beverage vending machine that is fully cooled, and is not a combination vending machine

- Class B: Any refrigerated bottled of canned beverage vending machine not considered to be Class A, and is not a combination vending machine.

## Class A was chosen because its requirements are more stringent. The maximum daily energy consumption (kWh) is given in Equation 2. The Federal Energy Management Program (FEMP) used a 21 ft3 refrigerated volume when calculating efficiency vending machine savings [494], so this value is used for the work paper.

Equation 2

Maximum Daily Energy Consumption kWh=0.055*V+2.56

Where V=Refrigerated volume (ft3)

Maximum Daily Energy Consumption kWh=0.055*21+2.56

Maximum Daily Energy Consumption kWh=3.715 kWh

The 4 hours of “sleep mode” per day will be used to develop a usage reduction rate that can be applied to the base case consumption to develop avoided kWh savings.

Equation 3

Usage Reduction Rate =(4 hrs÷24hrs)×100%

Usage Reduction Rate =16.67%

To develop the annual kWh savings, the daily energy consumption multiplied across 365 days for an annual kWh usage with the 16.67% reduction rate applied for the avoided usage.

Equation 4

Annual Reduction kWh=Daily Energy Consumption kWh×365 days×Usage Reduction Rate

Annual Reduction kWh=3.715 kWh×365 days ×16.67%

Annual Reduction kWh=226.04 kWh/yr

## 4) Energy savings for the reduction in refrigeration time for commercial refrigerators, commercial refrigerator-freezers, and commercial freezers

Similar to calculations for the beverage vending machines, a “sleep mode” of four hours will be used to estimate avoided usage. The calculation is illustrated in again in Equation 5.

Equation 5

Usage Reduction Rate =(4 hrs÷24hrs)×100%

Usage Reduction Rate =16.67%

The base case energy usage is established by using the CEC Title 20 (T-20) requirements [493] for glass door reach-in refrigerators, Section 1605.1 Table A-4.

Equation 6

Maximum Daily Energy Consumption kWh=0.12× V+3.34

Where V=Refrigerator volume

The annual savings are illustrated by Equation 7:

Equation 7

Annual Reduction kWh=Daily Energy Consumption kWh×365 days×Usage Reduction Rate

The energy savings calculations details for the targeted beverage cooler types are summarized in

Table 2.

Table 2 Annual Energy Savings for BMC on Beverage Coolers

| Description | Under Counter | Single Door | Double Door | Triple Door |
| --- | --- | --- | --- | --- |
| Measure Name | One Door Under Counter Beverage Merchandise Cooler Control | One Door Beverage Merchandise Cooler Control | Double Door Beverage Merchandise Cooler Control | Triple Door Beverage Merchandise Cooler Control |
| Nominal Volume Range (cubic feet) | <15 | 15-29 | 29-49 | >49 |
| Typical Volume (cubic feet) | 10 | 24 | 44 | 72 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1657.10 | 2270.30 | 3146.30 | 4372.70 |
| Usage Reduction Rate (%) | 16.67% | 16.67% | 16.67% | 16.67% |
| Measure Case Energy Savings (kWh/yr) | 276.18 | 378.38 | 524.38 | 728.78 |

It is worth noting that, for beverage coolers with volumes greater than 49 cubic feet, the power source for the built-in compressor can be a 208/240VAC single phase circuit. The BMC described in this workpaper, which is rated for 120VAC applications, may not be used for this application. For energy savings claims for such coolers, a BMC specification will need to be submitted for validation purposes.

## Summary of Energy Savings Calculations

Final results are located in the attached calculation spreadsheet.

Table 3 Measure Calculation Summary

| Solution Code | Applicable Code Reference | Interior Display Light | Backlit Signage | Vending Machine | Com Refrig/Freezer |
| --- | --- | --- | --- | --- | --- |
| RF-48900 | Double Door Beverage Merchandise Cooler Control |  |  |  | 524.38 kWh |
| RF-56733 | One Door Beverage Merchandise Cooler Control |  |  |  | 378.38 kWh |
| RF-65065 | One Door Under Counter Beverage Merchandise Cooler Control |  |  |  | 276.18 kWh |
| RF-70008 | Triple Door Beverage Merchandise Cooler Control |  |  |  | 728.78 kWh |
| RF-75217 | Cold Vending Machine Controls |  | 0.072kW * Hours Off per year | 226.04 kWh |  |
| LT-85945 | Snack Vending Machine Controls | 0.02kW * Hours Off per year |  |  |  |

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | Occupancy Sensor | Misc._Commercial |
| Education - Primary School | Occupancy Sensor | K_thru_12_School |
| Education - Secondary School | Occupancy Sensor | K_thru_12_School |
| Education - Relocatable Classroom | Occupancy Sensor | K_thru_12_School |
| Education - Community College | Occupancy Sensor | K_thru_12_School |
| Education - University | Occupancy Sensor | K_thru_12_School |
| Grocery | Occupancy Sensor | Misc._Commercial |
| Health/Medical - Hospital | Occupancy Sensor | Misc._Commercial |
| Health/Medical - Nursing Home | Occupancy Sensor | Misc._Commercial |
| Lodging - Hotel | Occupancy Sensor | Hotel_Motel |
| Lodging - Guest Rooms | Occupancy Sensor | Hotel_Motel |
| Lodging - Motel | Occupancy Sensor | Hotel_Motel |
| Manufacturing - Bio/Tech | Occupancy Sensor | Industrial |
| Manufacturing - Light Industrial | Occupancy Sensor | Industrial |
| Office – Large | Occupancy Sensor | Large_Office |
| Office – Small | Occupancy Sensor | Small_Office |
| Restaurant - Fast-Food | Occupancy Sensor | Misc._Commercial |
| Restaurant - Sit-Down | Occupancy Sensor | Misc._Commercial |
| Retail - Multistory Large | Occupancy Sensor | Large_Retail_Store |
| Retail - Single-Story Large | Occupancy Sensor | Large_Retail_Store |
| Retail - Small | Occupancy Sensor | Small_Retail_Store |
| Storage - Conditioned | Occupancy Sensor | Misc._Commercial |
| Storage - Unconditioned | Occupancy Sensor | Misc._Commercial |
| Warehouse - Refrigerated | Occupancy Sensor | Misc._Commercial |
| Residential Multi-family | Occupancy Sensor | Misc._Commercial |
| Residential Mobile Home - Double-Wide | Occupancy Sensor | Misc._Commercial |

# Section 4. Costs

## 4.1 Base Case Cost

The base case cost is $0 because these measures are voluntary retrofits to existing equipment.

## 4.2 Measure Case Cost

The measure case equipment costs are the retail costs of control units (controller, occupancy sensor, and mounting equipment), from a major manufacturer. The estimated time required to mount the sensor and plug in the controller is 1 hour, at the DEER2016 labor rate “NR-MISC” of $67.88/hr.

| Measure Code | Applicable Code Reference | Equipment Cost | Labor Cost |
| --- | --- | --- | --- |
| RF-48900 | Double Door Beverage Merchandise Cooler Control | $169.00 | $67.88 |
| RF-56733 | One Door Beverage Merchandise Cooler Control |  |  |
| RF-65065 | One Door Under Counter Beverage Merchandise Cooler Control |  |  |
| RF-70008 | Triple Door Beverage Merchandise Cooler Control |  |  |
| RF-75217 | Cold Vending Machine Controls | $189.00 |  |
| LT-85945 | Snack Vending Machine Controls | $160.00 |  |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure Code | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| RF-48900 | REA | $236.88 | $236.88 | N/A |
| RF-56733 | REA |  |  |  |
| RF-65065 | REA |  |  |  |
| RF-70008 | REA |  |  |  |
| RF-75217 | REA | $256.88 | $256.88 | N/A |
| LT-85945 | REA | $227.88 | $227.88 | N/A |

# Attachments

- SCE Savings

- PG&E Savings

# References

[26]

[297]

[392]

[393]

[493]

[494]