Work Paper SCE13AP001

Revision 3

Southern California Edison

Energy Efficient Refrigerators

# At-a-Glance Summary

| Measure Codes | See Section 1. |
| --- | --- |
| Measure Description | Refrigerator that is 10% (Energy Star) or 15% (Energy Star Most Efficient) more efficient than the federal standard |
| Base Case Description | Refrigerator that meets the federal efficiency standards |
| Units | Refrigerator |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | Appl-ESRefg: 14 years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Res-Default>2: 0.55 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 6/27/12 | Jenny Roecks (PG&E) | PGECOAPP124 Energy Efficient Refrigerators R0.doc PG&E 2010-2012 measure values included. Values for statewide workpaper (PG&E and SCE) for 2013-2014 filing also included. Includes measure impacts for Federal Standard + 20% as well as Federal Standard + 30% efficiency levels. Compact Refrigerators included using SCE methodology (and DEER interactive effects) |
| 0 | 8/24/12 | Steve Blanc (PG&E) | PGECOAPP124 Energy Efficient Refrigerators R0.doc Corrected for Dates and ED reqts |
| 1 | 5/8/14 | Jia Huang (PG&E) | PGECOAPP124 Energy Efficient Refrigerators R1.docx Updated with new workpaper template. Updated NTG. |
| 2 | 1/14/15 | Jia Huang (PG&E) | PGECOAPP124 Energy Efficient Refrigerators R2.docx Updated Savings values for 9/14/2014 Federal code change and DEER 2015. |
| 3 | 11/24/15 | Jason Wang (SCE) | - PG&E is no longer using this work paper so it is SCE-only - Removed CEE Tier 3 measures - Removed freezer measures - Did not change savings values - Updated costs using DEER and WO017 |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Refrigerator that is 10% (Energy Star) or 15% (Energy Star Most Efficient) more efficient than the federal standard |
| Existing Condition | N/A |
| Code/Standard | Refrigerator that meets the federal efficiency standards |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | AP-87223 |  | Energy Star - Compact (less than 7.75 ft3) with no Freezer Refrigerator |
|  |  | AP-40945 |  | Energy Star with no Freezer (11 - 23 ft3) Refrigerator |
|  |  | AP-79111 |  | Energy Star - Compact (less than 7.75 ft3) Refrigerator/Freezer |
|  |  | AP-90011 |  | Energy Star Bottom Mount Freezer, no Ice - Compact (less than 7.75 ft3) Refrigerator |
|  |  | AP-39992 |  | Energy Star Bottom Mount Freezer, no Ice - Small (8-16.5 ft3) Refrigerator |
|  |  | AP-68842 |  | Energy Star Bottom Mount Freezer, no Ice - Large (16.5 ft3 or greater) Refrigerator |
|  |  | AP-10114 |  | Energy Star Bottom Mount Freezer, with Ice - Large (16.5 ft3 or greater) Refrigerator |
|  |  | AP-29187 |  | Energy Star Top Mount Freezer, no Ice - Compact (less than 7.75 ft3) Refrigerator |
|  |  | AP-19411 |  | Energy Star Top Mount Freezer, no Ice - Small (10-15 ft3) Refrigerator |
|  |  | AP-29410 |  | Energy Star Top Mount Freezer, no Ice - Medium (15-20 ft3) Refrigerator |
|  |  | AP-45521 |  | Energy Star Top Mount Freezer, no Ice - Large (20 ft3 or greater) Refrigerator |
|  |  | AP-39094 |  | Energy Star Top Mount Freezer, with Ice - Medium (15-20 ft3) Refrigerator |
|  |  | AP-74421 |  | Energy Star Side Mount Freezer, no Ice - Medium (15-23 ft3) Refrigerator |
|  |  | AP-98888 |  | Energy Star Side Mount Freezer, no Ice - Large (23 ft3 or greater) Refrigerator |
|  |  | AP-80244 |  | Energy Star Side Mount Freezer, with Ice - Medium (15-23 ft3) Refrigerator |
|  |  | AP-12333 |  | Energy Star Side Mount Freezer, with Ice - Large (23 ft3 or greater) Refrigerator |
|  |  | AP-80112 |  | ES Most Efficient - Average Size Refrigerator/Freezer |
|  |  | AP-94456 |  | ES Most Efficient Bottom Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-29677 |  | ES Most Efficient Bottom Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-82565 |  | ES Most Efficient Bottom Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator |
|  |  | AP-65945 |  | ES Most Efficient Bottom Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-37895 |  | ES Most Efficient Bottom Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-39976 |  | ES Most Efficient Bottom Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator |
|  |  | AP-19658 |  | ES Most Efficient Top Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-75456 |  | ES Most Efficient Top Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-94675 |  | ES Most Efficient Top Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator |
|  |  | AP-78564 |  | ES Most Efficient Top Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-26754 |  | ES Most Efficient Top Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-10098 |  | ES Most Efficient Top Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator |
|  |  | AP-13536 |  | ES Most Efficient Side Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-47785 |  | ES Most Efficient Side Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-47658 |  | ES Most Efficient Side Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator |
|  |  | AP-95467 |  | ES Most Efficient Side Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator |
|  |  | AP-50694 |  | ES Most Efficient Side Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator |
|  |  | AP-59865 |  | ES Most Efficient Side Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator |

SCE Program Restrictions and Guidelines

Customer must have electricity distributed by SCE to the installation address.

Terms and Conditions: Energy Star Mid- and Full-Sized Refrigerators, Compact Refrigerators

- Must be on the ENERGY STAR qualifying list for relevant rebate.

- ENERGY STAR qualified refrigerators are at least 10% more efficient than models that simply meet the federal minimum standard for energy efficiency.

- Refrigerators smaller than 7.75 cubic feet are classified as compact refrigerators.

- Keg/beverage centers and wine coolers/chillers do not qualify.

Terms and Conditions: Energy Star Most Efficient Full-Sized Refrigerators

- Must be on the ENERGY STAR qualifying list and be indicated as meeting the ENERGY STAR Most Efficient criteria for 2016.

- ENERGY STAR Most Efficient qualified refrigerators are at least 15% more efficient than models that simply meet the federal minimum standard for energy efficiency. They must also use less than or equal to 637 kWh per year. Top freezers do not have to meet the 15% requirement; only the ≤ 637 kWh/year requirement applies to them.

- Compact refrigerators, all-refrigerators, and freezers do not qualify.

- Keg/beverage centers and wine coolers/chillers do not qualify.

Market Applicability

- Building types: Residential Single Family, Residential Multi-family, Residential Mobile Home - Double-Wide

- All SCE climate zones

## 1.2 Technical Description

Qualifying refrigerators are more efficient due to improvements in insulation and compressors. All refrigerator models must meet the size requirements mentioned in each of the measure names.

## 1.3 Installation Types and Delivery Mechanisms

The installation type is Replace-on-Burnout (ROB).

The delivery mechanism is Financial Support: Down-Stream Incentive – Deemed.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

SCE’s size requirements for refrigerators do not exactly align with DEER measures. Therefore, the rated energy savings obtained from the DOE equations are multiplied by DEER basis factors to obtain claimed savings for SCE’s size ranges. See Section 2 for more details.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | Yes |
| Scaled DEER measure | Yes |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | DEER assumes different size ranges and efficiency levels. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| Appl-ESRefg | High Efficiency Refrigerator | Res | AppPlug | 14 | N/A |

### 1.4.2 Codes and Standards Analysis

Title 20 2015 [493]: These measures fall under Section 1605.1 of Title 20 of the California Energy Regulations. These State Standards are based on the Federal Regulations for Refrigerators (see “Federal Standards” below).

Title 24: These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards: These measures fall under Federal DOE regulations.  The National Appliance Energy Conservation Act (NAECA) dictates minimum energy consumption standards for both commercial and non-commercial refrigerators and freezers. These standards are as follows:

| Product Class | Equations for Maximum Energy Use (kWh/yr) |  |
| --- | --- | --- |
|  | Based on AV (cu ft) | Based on av (L) |
| 1. Refrigerator-freezers and refrigerators other than all-refrigerators with manual defrost. | 7.99AV + 225.0 | 0.282av + 225.0 |
| 1A. All-refrigerators—manual defrost. | 6.79AV + 193.6 | 0.240av + 193.6 |
| 2. Refrigerator-freezers—partial automatic defrost | 7.99AV + 225.0 | 0.282av + 225.0 |
| 3. Refrigerator-freezers—automatic defrost with top-mounted freezer without an automatic icemaker. | 8.07AV + 233.7 | 0.285av + 233.7 |
| 3-BI. Built-in refrigerator-freezer—automatic defrost with top-mounted freezer without an automatic icemaker. | 9.15AV + 264.9 | 0.323av + 264.9 |
| 3I. Refrigerator-freezers—automatic defrost with top-mounted freezer with an automatic icemaker without through-the-door ice service. | 8.07AV + 317.7 | 0.285av + 317.7 |
| 3I-BI. Built-in refrigerator-freezers—automatic defrost with top-mounted freezer with an automatic icemaker without through-the-door ice service. | 9.15AV + 348.9 | 0.323av + 348.9 |
| 3A. All-refrigerators—automatic defrost. | 7.07AV + 201.6 | 0.250av + 201.6 |
| 3A-BI. Built-in All-refrigerators—automatic defrost. | 8.02AV + 228.5 | 0.283av + 228.5 |
| 4. Refrigerator-freezers—automatic defrost with side-mounted freezer without an automatic icemaker. | 8.51AV + 297.8 | 0.301av + 297.8 |
| 4-BI. Built-In Refrigerator-freezers—automatic defrost with side-mounted freezer without an automatic icemaker. | 10.22AV + 357.4 | 0.361av + 357.4 |
| 4I. Refrigerator-freezers—automatic defrost with side-mounted freezer with an automatic icemaker without through-the-door ice service. | 8.51AV + 381.8 | 0.301av + 381.8 |
| 4I-BI. Built-In Refrigerator-freezers—automatic defrost with side-mounted freezer with an automatic icemaker without through-the-door ice service. | 10.22AV + 441.4 | 0.361av + 441.4 |
| 5. Refrigerator-freezers—automatic defrost with bottom-mounted freezer without an automatic icemaker. | 8.85AV + 317.0 | 0.312av + 317.0 |
| 5-BI. Built-In Refrigerator-freezers—automatic defrost with bottom-mounted freezer without an automatic icemaker. | 9.40AV + 336.9 | 0.332av + 336.9 |
| 5I. Refrigerator-freezers—automatic defrost with bottom-mounted freezer with an automatic icemaker without through-the-door ice service. | 8.85AV + 401.0 | 0.312av + 401.0 |
| 5I-BI. Built-In Refrigerator-freezers—automatic defrost with bottom-mounted freezer with an automatic icemaker without through-the-door ice service. | 9.40AV + 420.9 | 0.332av + 420.9 |
| 5A. Refrigerator-freezer—automatic defrost with bottom-mounted freezer with through-the-door ice service. | 9.25AV + 475.4 | 0.327av + 475.4 |
| 5A-BI. Built-in refrigerator-freezer—automatic defrost with bottom-mounted freezer with through-the-door ice service. | 9.83AV + 499.9 | 0.347av + 499.9 |
| 6. Refrigerator-freezers—automatic defrost with top-mounted freezer with through-the-door ice service. | 8.40AV + 385.4 | 0.297av + 385.4 |
| 7. Refrigerator-freezers—automatic defrost with side-mounted freezer with through-the-door ice service. | 8.54AV + 432.8 | 0.302av + 432.8 |
| 7-BI. Built-In Refrigerator-freezers—automatic defrost with side-mounted freezer with through-the-door ice service. | 10.25AV + 502.6 | 0.362av + 502.6 |
| 8. Upright freezers with manual defrost. | 5.57AV + 193.7 | 0.197av + 193.7 |
| 9. Upright freezers with automatic defrost without an automatic icemaker. | 8.62AV + 228.3 | 0.305av + 228.3 |
| 9I. Upright freezers with automatic defrost with an automatic icemaker. | 8.62AV + 312.3 | 0.305av + 312.3 |
| 9-BI. Built-In Upright freezers with automatic defrost without an automatic icemaker. | 9.86AV + 260.9 | 0.348av + 260.9 |
| 9I-BI. Built-in upright freezers with automatic defrost with an automatic icemaker. | 9.86AV + 344.9 | 0.348av + 344.9 |
| 10. Chest freezers and all other freezers except compact freezers. | 7.29AV + 107.8 | 0.257av + 107.8 |
| 10A. Chest freezers with automatic defrost. | 10.24AV + 148.1 | 0.362av + 148.1 |
| 11. Compact refrigerator-freezers and refrigerators other than all-refrigerators with manual defrost. | 9.03AV + 252.3 | 0.319av + 252.3 |
| 11A.Compact all-refrigerators—manual defrost. | 7.84AV + 219.1 | 0.277av + 219.1 |
| 12. Compact refrigerator-freezers—partial automatic defrost | 5.91AV + 335.8 | 0.209av + 335.8 |
| 13. Compact refrigerator-freezers—automatic defrost with top-mounted freezer. | 11.80AV + 339.2 | 0.417av + 339.2 |
| 13I. Compact refrigerator-freezers—automatic defrost with top-mounted freezer with an automatic icemaker. | 11.80AV + 423.2 | 0.417av + 423.2 |
| 13A. Compact all-refrigerators—automatic defrost. | 9.17AV + 259.3 | 0.324av + 259.3 |
| 14. Compact refrigerator-freezers—automatic defrost with side-mounted freezer. | 6.82AV + 456.9 | 0.241av + 456.9 |
| 14I. Compact refrigerator-freezers—automatic defrost with side-mounted freezer with an automatic icemaker. | 6.82AV + 540.9 | 0.241av + 540.9 |
| 15. Compact refrigerator-freezers—automatic defrost with bottom-mounted freezer. | 11.80AV + 339.2 | 0.417av + 339.2 |
| 15I. Compact refrigerator-freezers—automatic defrost with bottom-mounted freezer with an automatic icemaker. | 11.80AV + 423.2 | 0.417av + 423.2 |
| 16. Compact upright freezers with manual defrost. | 8.65AV + 225.7 | 0.306av + 225.7 |
| 17. Compact upright freezers with automatic defrost. | 10.17AV + 351.9 | 0.359av + 351.9 |
| 18. Compact chest freezers. | 9.25AV + 136.8 | 0.327av + 136.8 |

AV= adjusted volume in cubic feet; av = adjusted volume in liters.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Section 1605.1(a) | July 1, 2015 |
| DOE | Standards for Residential Refrigerators and Freezers | September 15, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

### 1.5.1 Process Evaluation of Southern California Edison’s 2006-2008 Home Energy Efficiency Rebate (HEER) Program

In 2009 KEMA published the Process Evaluation of Southern California Edison’s 2006-2008 Home Energy Efficiency Rebate (HEER) Program: Final Report. SCE offered Energy Star refrigerator rebates through their HEER program in 2006-2008 and are currently offering Energy Star refrigerator rebates for the 2010-2012 program cycle.  The KEMA study evaluated customer awareness of, and interest in, the SCE HEER program.  This report did not provide information pertinent to the energy savings, NTG, EUL or cost estimates for this workpaper.

### 1.5.1 Refrigerator Market Profile

The EPA’s ENERGY STAR also published a Refrigerator Market Profile in 2009 to give energy efficiency program designers and policy makers an idea of market opportunities and savings potential for energy efficient refrigerators.  The energy savings potential identified in the report was based on the difference in energy consumption between the installed refrigerator base leading up to 2009 and the Energy Star refrigerator specification.  The savings potential from this study cannot be easily compared to the savings identified in this workpaper because the workpaper measures estimate the difference in energy consumption between a refrigerator that is more efficient than Energy Star and the federal standard for refrigerator energy consumption.  Many refrigerators comprising the installed base in the ENERGY STAR report do not meet current federal standard.

## 1.6 Data Quality and Future Data Needs

No future data needs are anticipated.

# Section 2. Calculation Methodology

## 2.1 Electric Energy Savings Estimation Methodologies

Energy Star Refrigerators

DEER 2015 contains savings impacts for standard sized refrigerators in the following size categories: mini, small, medium, large, and very large. However, in order to retain consistency with the previous program cycle, the IOU measures for standard sized refrigerators are split into at most three size categories (same as DEER 2011): large, medium, and/or small. Therefore, the DEER 2015 impacts are not directly applicable and savings impacts are recalculated with IOU size categories. The following outlines the methodology to crosswalk the DEER 2015 size categories to the IOU size categories.

The average total volume of the size category is taken as the average of the lower and upper range of the applicable size category. The total volume is crosswalked to adjusted volume for the applicable refrigerator type using the methodology in the DOE’s Technical Support Document. Using the adjusted volume, the federal mandated maximum annual unit energy consumption can be derived from the energy standard equations in section 1.4.2. Similarly, the annual unit energy consumption of an Energy Star refrigerator can be calculated by applying the adjusted volume to Energy Star’s equation for determining eligibility. The difference between the federal code UEC and Energy Star UEC is the rated energy savings. The rated energy savings is converted to claimed energy savings by applying DEER basis factors. DEER basis factors are taken from DEER 2014: “RE-Appl-RefgCond-basis”. Below is an example for refrigerators with bottom mounted freezers without icemaker (climate zone 1, >= 16.5 cubic feet):

kWh savings=federal code UEC-Energy Star UEC*(DEER basis factor)_kWh

kW savings=federal code UEC-Energy Star UEC*(DEER basis factor)_kW

therm savings=federal code UEC-Energy Star UEC*(DEER basis factor)_therm

kWh savings=543-489*0.677=36.7 kWh

kW savings=543-489*0.0000866=0.0047 kW

therm savings=543-489*-0.028=-1.52 therms

See file “Refrigerator Calculations – DEER 2015.xlsx” for calculations.

Energy Star Most Efficient Refrigerators

Savings impacts for Energy Star Most Efficient standard sized refrigerators are calculated the same way as for Energy Star refrigerators except that the total volume is calculated as an average of volumes in the Energy Star qualified products list.

Compact Refrigerators

Energy Saving impacts for Energy Star compact refrigerators is derived by taking an average of DEER 2015 (Tier 1) savings impacts for small and mini compact refrigerator categories.

## 2.2. Demand Reduction Estimation Methodologies

Demand savings are obtained either directly from DEER 2015 or by applying the applicable DEER basis factor for kW savings to the delta between the rated federal code UEC and the rated measure UEC.

kW savings=federal code UEC-measure UEC*(DEER basis factor)_kW

See Section 2.1 for details.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Mobile Home - Double-Wide | DEER:RefgFrzr_HighEff | RES |
| Residential Multi-family | DEER:RefgFrzr_HighEff | RES |
| Residential Single Family | DEER:RefgFrzr_HighEff | RES |

# Section 4. Costs

## 4.1 Base Case Cost

Since the work paper measures do not exactly match DEER2016 measures, the DEER2016 Refrigerator and Freezer cost workbook (based on WO017 cost models) was used to calculate work paper costs (see Attachment 1, tab “RefgFrzr Technology.”) The following costs were out of scope of the cost models and were therefore assigned certain values:

- Measure “Energy Star with no Freezer (11 - 23 ft3) Refrigerator” was assigned the base case cost for “Energy Star Top Mount Freezer, no Ice - Small (10-15 ft3) Refrigerator”

- Compact models were assigned a base case cost of $175, from the Energy Star Residential Refrigerator Calculator.

- Measure “ES Most Efficient - Average Size Refrigerator/Freezer” was calculated as the average of Energy Star Most Efficient bottom, top and side mount freezer (large size) base case costs.

The labor cost for non-compact models is 1.5 hours at an hourly rate of $48.60 (DEER Labor Rate ID “R-App”), for a total of $72.90. The labor cost for compact models is 0.5 hours at an hourly rate of $48.60 (DEER Labor Rate ID “R-App”), for a total of $24.30.

## 4.2 Measure Case Cost

The DEER2016 Refrigerator and Freezer cost workbook was used to calculate work paper costs. The following costs were out of scope of the cost models and were therefore assigned certain values:

- Measure “Energy Star with no Freezer (11 - 23 ft3) Refrigerator” was assigned the measure case cost for “Energy Star Top Mount Freezer, no Ice - Small (10-15 ft3) Refrigerator”

- Compact models were assigned a measure case cost of $185, based on a $10 incremental cost from the Energy Star Appliance Savings Calculator.

- Measure “ES Most Efficient - Average Size Refrigerator/Freezer” was calculated as the average of Energy Star Most Efficient bottom, top and side mount freezer (large size) measure case costs.

The labor costs are the same as in the base case.

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| Energy Star - Compact (less than 7.75 ft3) with no Freezer Refrigerator | ROB | $10.00 | $10.00 | N/A |
| Energy Star with no Freezer (11 - 23 ft3) Refrigerator | ROB | $17.00 | $17.00 | N/A |
| Energy Star - Compact (less than 7.75 ft3) Refrigerator/Freezer | ROB | $10.00 | $10.00 | N/A |
| Energy Star Bottom Mount Freezer, no Ice - Compact (less than 7.75 ft3) Refrigerator | ROB | $10.00 | $10.00 | N/A |
| Energy Star Bottom Mount Freezer, no Ice - Small (8-16.5 ft3) Refrigerator | ROB | $21.00 | $21.00 | N/A |
| Energy Star Bottom Mount Freezer, no Ice - Large (16.5 ft3 or greater) Refrigerator | ROB | $25.00 | $25.00 | N/A |
| Energy Star Bottom Mount Freezer, with Ice - Large (16.5 ft3 or greater) Refrigerator | ROB | $30.00 | $30.00 | N/A |
| Energy Star Top Mount Freezer, no Ice - Compact (less than 7.75 ft3) Refrigerator | ROB | $10.00 | $10.00 | N/A |
| Energy Star Top Mount Freezer, no Ice - Small (10-15 ft3) Refrigerator | ROB | $17.00 | $17.00 | N/A |
| Energy Star Top Mount Freezer, no Ice - Medium (15-20 ft3) Refrigerator | ROB | $19.00 | $19.00 | N/A |
| Energy Star Top Mount Freezer, no Ice - Large (20 ft3 or greater) Refrigerator | ROB | $22.00 | $22.00 | N/A |
| Energy Star Top Mount Freezer, with Ice - Medium (15-20 ft3) Refrigerator | ROB | $22.00 | $22.00 | N/A |
| Energy Star Side Mount Freezer, no Ice - Medium (15-23 ft3) Refrigerator | ROB | $24.00 | $24.00 | N/A |
| Energy Star Side Mount Freezer, no Ice - Large (23 ft3 or greater) Refrigerator | ROB | $28.00 | $28.00 | N/A |
| Energy Star Side Mount Freezer, with Ice - Medium (15-23 ft3) Refrigerator | ROB | $26.00 | $26.00 | N/A |
| Energy Star Side Mount Freezer, with Ice - Large (23 ft3 or greater) Refrigerator | ROB | $31.00 | $31.00 | N/A |
| ES Most Efficient - Average Size Refrigerator/Freezer | ROB | $34.67 | $34.67 | N/A |
| ES Most Efficient Bottom Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $32.00 | $32.00 | N/A |
| ES Most Efficient Bottom Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $38.00 | $38.00 | N/A |
| ES Most Efficient Bottom Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $42.00 | $42.00 | N/A |
| ES Most Efficient Bottom Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $47.00 | $47.00 | N/A |
| ES Most Efficient Bottom Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $52.00 | $52.00 | N/A |
| ES Most Efficient Bottom Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $27.00 | $27.00 | N/A |
| ES Most Efficient Top Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $25.00 | $25.00 | N/A |
| ES Most Efficient Top Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $29.00 | $29.00 | N/A |
| ES Most Efficient Top Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $33.00 | $33.00 | N/A |
| ES Most Efficient Top Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $37.00 | $37.00 | N/A |
| ES Most Efficient Top Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $41.00 | $41.00 | N/A |
| ES Most Efficient Top Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $45.00 | $45.00 | N/A |
| ES Most Efficient Side Mount Freezer, no Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $31.00 | $31.00 | N/A |
| ES Most Efficient Side Mount Freezer, no Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $37.00 | $37.00 | N/A |
| ES Most Efficient Side Mount Freezer, no Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $39.00 | $39.00 | N/A |
| ES Most Efficient Side Mount Freezer, with Ice - Medium (18.0 ft3 or less) Refrigerator | ROB | $42.00 | $42.00 | N/A |
| ES Most Efficient Side Mount Freezer, with Ice - Large (18.1-22.5 ft3) Refrigerator | ROB | $47.00 | $47.00 | N/A |
| ES Most Efficient Side Mount Freezer, with Ice - X-Large (greater than 22.5 ft3) Refrigerator | ROB | $50.00 | $50.00 | N/A |

# Attachments

# References

[493]