Work Paper SCE13CC001

Revision 3

Southern California Edison

Commercial Reach-In Refrigerators and Freezers

# At-a-Glance Summary

| Measure Codes | FS-13121, FS-44686, FS-16170, FS-54809, FS-21276, FS-58112, FS-28291, FS-61432, FS-30794, FS-68882, FS-31425, FS-77373, FS-38598, FS-86848, FS-42962, FS-93044 |
| --- | --- |
| Measure Description | Energy efficient commercial reach-in solid and glass door refrigerators and freezers, in both vertical and chest configurations |
| Base Case Description | Reach-in solid and glass door refrigerators and freezers governed by code. |
| Units | Unit |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | 12 years (Cook-SDRef) |
| Measure Installation Type | Early Retirement (RET), & Replace on Burnout (ROB). |
| Net-to-Gross Ratio | 0.6 (Com-Default>2yrs), 0.6 (Ind-Default>2yrs), 0.6 (Agric-Default>2yrs) |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/06/2012 | John Rossi/EMCOR | - Updated to 2013-2014 Template. |
| 1 | 10/03/2013 | Alfredo Gutierrez/SCE | - Updated savings value (kWh and kW) for Glass-Door Reach-In Refrigerators with volume between 30-49 ft3 - Savings values (kWh and kW) have been fixed for the following measures: - < 15 cubic feet Glass-Door Reach-In Freezer (FS-16170) - 15 – 29 cubic feet Glass-Door Reach-In Freezer (FS-38598) - The following measures have been updated with a new installation type (RET) and delivery method (Midstream): - < 15 cubic feet Glass-Door Reach-In Refrigerator (FS-77373) - 15 – 29 cubic feet Glass-Door Reach-In Refrigerator (FS-28291) - 30 – 49 cubic feet Glass-Door Reach-In Refrigerator (FS-68882) - ≥ 50 cubic feet Glass-Door Reach-In Refrigerator (FS-44686) |
| 2 | 06/23/2014 | Alfredo Gutierrez/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Updated work paper template. |
| 3 | 01/14/2016 | Ajay Wadhera/Solaris | -New template update for 2016 program year -WP effective from 1/1/2016 thru 12/31/2016 -Removed SCE building types -No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the purchase of new or replacement energy efficient commercial reach-in solid and glass door refrigerators and freezers, in both vertical and chest configurations. In all categories, the refrigeration system shall be built-in (packaged).

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Energy efficient reach-in solid and glass door refrigerators and freezers |
| Existing Condition | Existing reach-in solid and glass door refrigerators and freezers |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | FS-13121 | N/A | < 15 cubic feet Solid-Door Reach-In Refrigerator |
| N/A | N/A | FS-42962 | N/A | 15 – 29 cubic feet Solid-Door Reach-In Refrigerator |
| N/A | N/A | FS-93044 | N/A | 30 – 49 cubic feet Solid-Door Reach-In Refrigerator |
| N/A | N/A | FS-31425 | N/A | ≥ 50 cubic feet Solid-Door Reach-In Refrigerator |
| N/A | N/A | FS-21276 | N/A | < 15 cubic feet Solid-Door Reach-In Freezer |
| N/A | N/A | FS-61432 | N/A | 15 – 29 cubic feet Solid-Door Reach-In Freezer |
| N/A | N/A | FS-86848 | N/A | 30 – 49 cubic feet Solid-Door Reach-In Freezer |
| N/A | N/A | FS-54809 | N/A | ≥ 50 cubic feet Solid-Door Reach-In Freezer |
| N/A | N/A | FS-77373 | N/A | < 15 cubic feet Glass-Door Reach-In Refrigerator |
| N/A | N/A | FS-28291 | N/A | 15 – 29 cubic feet Glass-Door Reach-In Refrigerator |
| N/A | N/A | FS-68882 | N/A | 30 – 49 cubic feet Glass-Door Reach-In Refrigerator |
| N/A | N/A | FS-44686 | N/A | ≥ 50 cubic feet Glass-Door Reach-In Refrigerator |
| N/A | N/A | FS-16170 | N/A | < 15 cubic feet Glass-Door Reach-In Freezer |
| N/A | N/A | FS-38598 | N/A | 15 – 29 cubic feet Glass-Door Reach-In Freezer |
| N/A | N/A | FS-58112 | N/A | 30 – 49 cubic feet Glass-Door Reach-In Freezer |
| N/A | N/A | FS-30794 | N/A | ≥ 50 cubic feet Glass-Door Reach-In Freezer |

Units with remote refrigeration systems do not qualify. Used or rebuilt equipment is not eligible. Each installed equipment must meet minimum prescriptive criteria to qualify for a per unit rebate. This measure is only applicable to Non-residential building types. This measure is not approved for installation in Residential Single Family, Residential Multi-Family, or Residential Mobile Home – Double Wide building types. This measure is eligible in all SCE climate zones.

## 1.2 Technical Description

Reach-in refrigerators and freezers are typically used to house perishable goods, such as soft drinks in supermarkets or other commercial areas. They are typically constructed from stainless steel and have a thick layer of insulation in the walls, doors, and floor. The cabinets have one to three doors, made of either glass or steel, and come in a variety of sizes with storage capabilities up to 72 cubic feet. These units are known as self-contained as they have an internal compressor for cooling rather than relying on an external compressor.

## 1.3 Installation Types and Delivery Mechanisms

The program type is Replace-On-Burnout (ROB) and Retrofit (RET). In the early retirement approach, functional units will be identified through the Energy Efficiency Mid-Stream Bottling Pilot program which targets glass-door reach-in refrigerators. These units will be targeted for participation through the pilot program and will be retired prematurely. The pilot program will provide incentives to the contractor for this to occur.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |

The delivery methods for the measures in this work paper are Financial Support – Down Stream Incentive – Deemed and Financial Support – Midstream Incentive –Deemed.

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

The 2014 DEER database does contain energy star refrigerators and freezers as well as recycled refrigerators and freezers. The DEER database, however, does not contain solid or glass door reach-in refrigerators or freezers.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | No |
| DEER Operating Hours | No |
| DEER eQUEST Prototypes | No |
| DEER Version | N/A |
| Reason for Deviation from DEER | N/A |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | All | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| Cook-SDRef | Commercial Reach-In Refrigerator / Freezer | Com | ComRefrig | 12 | 4 |
| Cook-SDFreez | Commercial Reach-In Refrigerator / Freezer | Com | ComRefrig | 12 | 4 |

### 1.4.2 Codes and Standards Analysis

The 2013 Title 24 [355] code does not apply in this case because solid door reach-in refrigerators, solid door reach-in freezers, glass door reach-in refrigerators, and glass door reach-in freezers do not fall under the CEC Title 24, Building Energy Efficiency Standards or the Department of Energy (DOE) energy regulations.

The 2014 Title 20 [422] code does not apply to the measures within this work paper. As such, Federal Code, 10 CFR 431.66, would cover these measures.

Per the Federal Code:

Each commercial refrigerator, freezer, and refrigerator-freezer with a self-contained condensing unit designed for holding temperature applications manufactured on or after January 1, 2010, shall have a daily energy consumption (in kilowatt hours per day) that does not exceed the following:

Base Case - Federal Requirements for Reach-In Refrigerators and Freezers

| Equipment Description (cubic feet) | Daily Energy Usage (kWh/day) |
| --- | --- |
| Solid-Door Reach-In Refrigerator | 0.100V+2.04* |
| Solid-Door Reach-In Freezer | 0.400V+1.38 |
| Glass-Door Reach-In Refrigerator | 0.120V+3.34 |
| Glass-Door Reach-In Freezer | 0.750V+4.10 |

As of January 1, 2010, the new ENERGY STAR® Version 2.0 specification [A] is effective, establishing the measure case equipment efficiencies. The following table lists the applicable codes. Measure case energy use requirements for qualifying equipment are shown in table below. As of January 1, 2010, the ENERGY STAR® specification was changed to the new Version 2.0 specification [A]. For the 2013 to 2014 program cycle, the new ENERGY STAR® Version 2.0 specification will be used as the requirement for the measures.

Measure Case - Energy Star® Version 2.0 Requirements for Reach-In Refrigerators and Freezers

| Equipment Description (cubic feet) | Daily Energy Usage (kWh/day) |
| --- | --- |
| Solid-Door Reach-In Refrigerator |  |
| 0 < V < 15 | 0.089V + 1.411* |
| 15 V < 30 | 0.037V + 2.200 |
| 30 V < 50 | 0.056V + 1.635 |
| 50 V | 0.060V + 1.416 |
| Solid-Door Reach-In Freezer |  |
| 0 < V < 15 | 0.250V + 1.250 |
| 15 V < 30 | 0.400V – 1.000 |
| 30 V < 50 | 0.163V + 6.125 |
| 50 V | 0.158V + 6.333 |
| Glass-Door Reach-In Refrigerator |  |
| 0 < V < 15 | 0.118V + 1.382 |
| 15 V < 30 | 0.140V + 1.050 |
| 30 V < 50 | 0.088V + 2.625 |
| 50 V | 0.110V + 1.500 |
| Glass-Door Reach-In Freezer |  |
| 0 < V < 15 | 0.607V + 0.893 |
| 15 V < 30 | 0.733V - 1.000 |
| 30 V < 50 | 0.250V + 13.500 |
| 50 V | 0.450V + 3.500 |
| Equipment Description (cubic feet) | Daily Energy Usage (kWh/day) |

*V is the internal volume in cubic feet.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | N/A |
| Title 20 (2014) | N/A | N/A |
| U.S. DOE Code of Federal Regulations | 10 CFR 431.66 | January 1, 2010 to Present |
| ENERGY STAR 2.1 | Commercial Refrigerators and Freezers Program Requirements | January 1, 2010 to Present |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

### 1.5.1 Non-DEER Study Review

No additional studies were used in the creation of this work paper.

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

The annual energy savings are summarized in tables shown below. The nominal size and internal volume range are based on standard widths and each standard width may include multiple (French style) doors and/or multiple sections. The calculated annual energy usage was based on 365 days per year operation, and the daily energy consumption for typical units was determined by applying the code and measure maximum daily energy consumption as shown in section 1.4.2 tables.

A sample calculation showing the daily and annual energy savings of a glass-door refrigerator is provided below.

Glass-Door Reach-In Refrigerator with total volume less than 15 cubic feet (0 < V < 15 ft3):

Volume (V) = 10 ft3

Measure Case Daily Energy Usage 	= 0.118 * V + 1.382

= 0.118 * 10 + 1.382

= 2.56 kWh / day

Measure Case Annual Energy Usage	= (Daily Energy Usage) * 365 days per year

= 2.56 kWh / day * 365 days / year

= 934 kWh / year

Base Case Daily Energy Usage 	= 0.120 * V + 3.34 (See Table 6)

= 0.120 * 10 + 3.34

= 4.54 kWh / day

Base Case Annual Energy Usage	= (Daily Energy Usage) * 365 days per year

= 4.54 kWh / day * 365 days / year

= 1657 kWh / year

Annual Energy Savings	= Base Case Annual Usage – Measure Case Annual Usage

= 1657 – 934 kWh / year

= 723 kWh / year

Complete calculation methodologies for both energy savings and demand reductions are in the calculation worksheet [B].

Tables below show the energy savings associated with commercial solid and glass reach-in refrigerators and freezers.

Energy Savings for Commercial Solid-Door Reach-in Refrigerators

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 3.04 | 4.44 | 6.44 | 9.24 |
| Measure Unit Daily Energy Use (kWh/day) | 2.30 | 3.09 | 4.10 | 5.74 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1110 | 1621 | 2351 | 3373 |
| Measure Unit Annual Energy Use (kWh/yr) | 840 | 1127 | 1496 | 2094 |
| Annual Energy Savings (kWh/yr) | 270 | 493 | 854 | 1279 |

Energy Savings for Commercial Solid-Door Reach-in Freezers

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 5.38 | 10.98 | 18.98 | 30.18 |
| Measure Unit Daily Energy Use (kWh/day) | 3.75 | 8.6 | 13.30 | 17.71 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1964 | 4008 | 6928 | 11016 |
| Measure Unit Annual Energy Use (kWh/yr) | 1369 | 3139 | 4853 | 6464 |
| Annual Energy Savings (kWh/yr) | 595 | 869 | 2074 | 4552 |

Energy Savings for Commercial Glass-Door Reach-in Refrigerators

|  | Glass-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 4.54 | 6.22 | 8.62 | 11.98 |
| Measure Unit Daily Energy Use (kWh/day) | 2.56 | 4.41 | 6.50 | 9.42 |
| Base Case Unit Annual Energy Use (kWh/yr) | 1657 | 2270 | 3146 | 4373 |
| Measure Unit Annual Energy Use (kWh/yr) | 934 | 1610 | 2371 | 3438 |
| Annual Energy Savings (kWh/yr) | 723 | 661 | 775 | 934 |

Energy Savings for Commercial Glass-Door Reach-in Freezers

|  | Glass-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Daily Energy Use (kWh/day) | 11.6 | 22.1 | 37.1 | 58.1 |
| Measure Unit Daily Energy Use (kWh/day) | 6.96 | 16.59 | 24.50 | 35.90 |
| Base Case Unit Annual Energy Use (kWh/yr) | 4234 | 8067 | 13542 | 21207 |
| Measure Unit Annual Energy Use (kWh/yr) | 2541 | 6056 | 8943 | 13104 |
| Annual Energy Savings (kWh/yr) | 1693 | 2010 | 4599 | 8103 |

## 2.2. Demand Reduction Estimation Methodologies

The demand reduction estimations are obtained by dividing the daily energy usage by 24 hours, as the units are plugged in 24/7.

The demand reduction estimation is summarized in the tables shown below.

A sample calculation showing the peak demand reduction associate with a glass-door reach-in refrigerator for the 2013-2014 program years is provided below.

Glass-Door Reach-In Refrigerator with total volume less than 15 cubic feet (0 < V < 15 ft3):

Volume (V) = 10 ft3

Measure Case Peak Demand Reduction	= Measure Daily Energy Usage / 24 hrs

= 2.56 / 24

= 0.107 kW

Base Case Peak Demand Reduction 		= Base Daily Energy Usage / 24 hrs

= 4.54 / 24

= 0.189 kW

Demand Reduction Savings			= (Base Reduction – Measure Reduction)

= 0.189 kW – 0.107 kW

= 0.083 kW

Peak Demand Reduction Savings		= Reduction Savings * 0.9

= 0.083 *0.9

= 0.074 kW

A multiplier of 0.9 is used to conservatively estimate the peak demand reduction during the DEER peak period for the particular climate zone.

Tables below show the demand reductions associated with commercial glass and solid door reach-in refrigerators and freezers.

Demand Reduction for Commercial Solid-Door Reach-in Refrigerators

|  | Solid-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.127 | 0.185 | 0.268 | 0.385 |
| Measure Unit Demand (kW) | 0.096 | 0.129 | 0.171 | 0.239 |
| Demand Reduction (kW) | 0.031 | 0.056 | 0.098 | 0.146 |
| Peak Demand Reduction (kW) | 0.028 | 0.051 | 0.088 | 0.131 |

Demand reduction for Commercial Solid-Door Reach-in Freezers

|  | Solid-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.224 | 0.458 | 0.791 | 1.258 |
| Measure Unit Demand (kW) | 0.156 | 0.358 | 0.554 | 0.738 |
| Demand Reduction (kW) | 0.068 | 0.099 | 0.237 | 0.520 |
| Peak Demand Reduction (kW) | 0.061 | 0.089 | 0.213 | 0.468 |

Demand reductions for Commercial Glass-Door Reach-in Refrigerators

|  | Glass-Door Reach-In Refrigerators (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.189 | 0.259 | 0.359 | 0.499 |
| Measure Unit Demand (kW) | 0.107 | 0.184 | 0.271 | 0.393 |
| Demand Reduction (kW) | 0.083 | 0.075 | 0.088 | 0.107 |
| Peak Demand Reduction (kW) | 0.074 | 0.068 | 0.079 | 0.096 |

Demand reductions for Commercial Glass-Door Reach-in Freezers

|  | Glass-Door Reach-In Freezers (ENERGY STAR® Version 2.0) |  |  |  |
| --- | --- | --- | --- | --- |
| Description | Under- Counter | Single- Door | Double- Door | Triple- Door |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | < 15 | 15-29 | 30-49 | 49 < |
| Typical Volume Used in Energy Calculations (ft3) | 10 | 24 | 44 | 72 |
| Base Case Unit Demand (kW) | 0.483 | 0.921 | 1.546 | 2.421 |
| Measure Unit Demand (kW) | 0.290 | 0.691 | 1.021 | 1.496 |
| Demand Reduction (kW) | 0.193 | 0.230 | 0.525 | 0.925 |
| Peak Demand Reduction (kW) | 0.174 | 0.207 | 0.473 | 0.833 |

Please note that for all RET measures in this work paper, the savings have been mapped to the savings associated with ROB measures. The current market for existing reach-in refrigerators is unknown, so the savings are based on above code required equipment until further research can identify the current market mix of existing equipment. Once this information is known, the work paper will be updated to incorporate the true baseline found from market research.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | Refrigeration | Assembly |
| Education - Primary School | Refrigeration | K_thru_12_School |
| Education - Secondary School | Refrigeration | K_thru_12_School |
| Education - Relocatable Classroom | Refrigeration | K_thru_12_School |
| Education - Community College | Refrigeration | College_University |
| Education - University | Refrigeration | College_University |
| Grocery | Refrigeration | Grocery_Store |
| Health/Medical - Hospital | Refrigeration | Hospital |
| Health/Medical – Nursing Home | Refrigeration | Medical_Clinic |
| Lodging - Hotel | Refrigeration | Hotel_Motel |
| Lodging - Guest Rooms | Refrigeration | Hotel_Motel |
| Lodging - Motel | Refrigeration | Hotel_Motel |
| Manufacturing - Bio/Tech | Refrigeration | Industrial |
| Manufacturing - Light Industrial | Refrigeration | Industrial |
| Office – Large | Refrigeration | Large_Office |
| Office – Small | Refrigeration | Small_Office |
| Restaurant - Fast-Food | Refrigeration | Fast_Food_Restaurant |
| Restaurant - Sit-Down | Refrigeration | Sit_Down_Restaurant |
| Retail - Multistory Large | Refrigeration | Large_Retail_Store |
| Retail – Single-Story Large | Refrigeration | Large_Retail_Store |
| Retail – Small | Refrigeration | Small_Retail_Store |
| Storage - Conditioned | Refrigeration | Storage_Building |
| Storage - Unconditioned | Refrigeration | Storage_Building |
| Warehouse - Refrigerated | Refrigeration | Refrigerated_Warehouse |

# Section 4. Costs

## 4.1 Base Case Cost

There is no additional labor or maintenance cost when comparing the measure to the base case for solid and glass door reach-in refrigerators and solid door reach-in freezers. The cost includes both material and labor, and applies to ROB and RET installations, which expects to have the same cost for all customers. The estimated equipment cost is based on manufacturer list prices and applying an industry-standard 44% discount to the published list prices [C]. The base case cost can be symbolically written as:

Base Case Cost = Base Case Equipment Cost + Base Case Installation Cost

The purchase prices for the Base Case solid- and glass-door, reach-in refrigerators and freezers are listed below.

Base Case Refrigerator and Freezer Material and Labor Costs

| Typical Appliance Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Typical Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators | $ 1,284.00 | $ 2,883.15 | $ 3,802.03 | $ 4,526.32 |
| Solid-Door Reach-In Freezers | $ 773.84 | $ 3,467.04 | $ 5,151.16 | $ 7,469.68 |
| Glass-Door Reach-In Refrigerators | $ 1270.00 | $ 3,340.20 | $ 4,788.00 | $ 6,915.84 |
| Glass-Door Reach-In Freezers (Energy Star 1.0)** | $ 1,281.20 | $ 5,212.19 | $ 7,401.52 | $ 10,304.40 |

## 4.2 Measure Case Cost

The measure costs include equipment and labor, as explained in Section 4.1. The estimated equipment cost is based on recent list cost data and applying an industry-standard 44% discount to the manufacturer published list prices [C]. The Measure Cost is symbolically written as:

Measure Cost = Measure Equipment Cost + Measure Installation Cost

Table below summarizes the purchase price for the Measure solid- and glass-door, reach-in refrigerators and freezers. Solid-door refrigerator and freezers purchase price are shown in table below.

Measure Refrigerator and Freezer Material and Labor Costs

| Typical Appliance Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Typical Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators (Energy Star 2.0) | $ 2,376.00 | $ 4,293.88 | $ 5,770.73 | $ 7,249.60 |
| Solid-Door Reach-In Freezers (Energy Star 2.0) | $ 1,031.44 | $ 4,830.22 | $ 6,707.87 | $ 9,437.71 |
| Glass-Door Reach-In Refrigerators (Energy Star 2.0) | $ 1,373.60 | $ 4,204.00 | $ 5,864.11 | $ 8,464.80 |
| Glass-Door Reach-In Freezers (Energy Star 2.0) | $ 1,306.68 | $ 5,336.22 | $ 7,615.72 | $ 11,203.70 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

The table below summarizes the FMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as ROB. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

Full Measure Refrigerator and Freezer Costs for ROB

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | $ 1,092.00 | $ 1,410.73 | $ 1,968.70 | $ 2,723.28 |
| Solid-Door Reach-In Freezers Incremental Cost | $ 257.60 | $ 1,363.18 | $ 1,556.71 | $ 1,968.03 |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 103.60 | $ 863.80 | $ 1,076.11 | $ 1,548.96 |
| Glass-Door Reach-In Freezers Incremental Cost | $ 25.48 | $ 124.04 | $ 214.20 | $ 899.30 |

The table below gives the 1st baseline FMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as RET. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

1st Baseline Full Measure Refrigerator and Freezer Costs for RET

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | N/A | N/A | N/A | N/A |
| Solid-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 1,373.60 | $ 4,204.00 | $ 5,864.11 | $ 8,464.80 |
| Glass-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |

The table below gives the 2nd baseline FMC for the solid- and glass-door, reach-in refrigerators and freezers claimed as RET. Note that DEER climate zone dependent costs are not applied to the base costs shown in this summary table. The DEER climate zone multipliers are used in the Cost Analysis section of the Calculations [B].

2nd Baseline Full Measure Refrigerator and Freezer Costs for RET

| Description | Under-Counter | Single-Door | Double-Door | Triple-Door |
| --- | --- | --- | --- | --- |
| Nominal Size | 1 door | 1 door | 2 doors | 3 doors |
| Nominal Volume Range (ft3) | 0 < V < 15 | 15 ≤ V < 30 | 30 ≤ V < 50 | 50 ≤ V |
| Solid-Door Reach-In Refrigerators Incremental Cost | N/A | N/A | N/A | N/A |
| Solid-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |
| Glass-Door Reach-In Refrigerators Incremental Cost | $ 103.60 | $ 863.80 | $ 1,076.11 | $ 1,548.96 |
| Glass-Door Reach-In Freezers Incremental Cost | N/A | N/A | N/A | N/A |

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| < 15 cubic feet Solid-Door Reach-In Refrigerator | ROB | $ 1,092.00 | $ 1,092.00 | N/A |
| 15 – 29 cubic feet Solid-Door Reach-In Refrigerator | ROB | $ 1,410.73 | $ 1,410.73 | N/A |
| 30 – 49 cubic feet Solid-Door Reach-In Refrigerator | ROB | $ 1,968.70 | $ 1,968.70 | N/A |
| ≥ 50 cubic feet Solid-Door Reach-In Refrigerator | ROB | $ 2,723.28 | $ 2,723.28 | N/A |
| < 15 cubic feet Solid-Door Reach-In Freezer | ROB | $ 257.60 | $ 257.60 | N/A |
| 15 – 29 cubic feet Solid-Door Reach-In Freezer | ROB | $ 1,363.18 | $ 1,363.18 | N/A |
| 30 – 49 cubic feet Solid-Door Reach-In Freezer | ROB | $ 1,556.71 | $ 1,556.71 | N/A |
| ≥ 50 cubic feet Solid-Door Reach-In Freezer | ROB | $ 1,968.03 | $ 1,968.03 | N/A |
| < 15 cubic feet Glass-Door Reach-In Refrigerator | ROB | $ 103.60 | $ 103.60 | N/A |
| 15 – 29 cubic feet Glass-Door Reach-In Refrigerator | ROB | $ 863.80 | $ 863.80 | N/A |
| 30 – 49 cubic feet Glass-Door Reach-In Refrigerator | ROB | $ 1,076.11 | $ 1,076.11 | N/A |
| ≥ 50 cubic feet Glass-Door Reach-In Refrigerator | ROB | $ 1,548.96 | $ 1,548.96 | N/A |
| < 15 cubic feet Glass-Door Reach-In Freezer | ROB | $ 25.48 | $ 25.48 | N/A |
| 15 – 29 cubic feet Glass-Door Reach-In Freezer | ROB | $ 124.04 | $ 124.04 | N/A |
| 30 – 49 cubic feet Glass-Door Reach-In Freezer | ROB | $ 214.20 | $ 214.20 | N/A |
| ≥ 50 cubic feet Glass-Door Reach-In Freezer | ROB | $ 899.30 | $ 899.30 | N/A |
| < 15 cubic feet Solid-Door Reach-In Refrigerator | RET | N/A | N/A | N/A |
| 15 – 29 cubic feet Solid-Door Reach-In Refrigerator | RET | N/A | N/A | N/A |
| 30 – 49 cubic feet Solid-Door Reach-In Refrigerator | RET | N/A | N/A | N/A |
| ≥ 50 cubic feet Solid-Door Reach-In Refrigerator | RET | N/A | N/A | N/A |
| < 15 cubic feet Solid-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| 15 – 29 cubic feet Solid-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| 30 – 49 cubic feet Solid-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| ≥ 50 cubic feet Solid-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| < 15 cubic feet Glass-Door Reach-In Refrigerator | RET | $ 103.60 | $ 1,373.60 | $ 103.60 |
| 15 – 29 cubic feet Glass-Door Reach-In Refrigerator | RET | $ 863.80 | $ 4,204.00 | $ 863.80 |
| 30 – 49 cubic feet Glass-Door Reach-In Refrigerator | RET | $ 1,076.11 | $ 5,864.11 | $ 1,076.11 |
| ≥ 50 cubic feet Glass-Door Reach-In Refrigerator | RET | $ 1,548.96 | $ 8,464.80 | $ 1,548.96 |
| < 15 cubic feet Glass-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| 15 – 29 cubic feet Glass-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| 30 – 49 cubic feet Glass-Door Reach-In Freezer | RET | N/A | N/A | N/A |
| ≥ 50 cubic feet Glass-Door Reach-In Freezer | RET | N/A | N/A | N/A |

# Attachments

- Reach-In Refrigerator and Freezer Savings

- Energy Star 2.0 Requirements

- Cost Calculation References

- Reach-In Savings Calculations

# References

[355]

[422]

[A] Attachment 2-Energy Star 2.0 Requirements

[B] Attachment 1-Reach-In Refrigerator and Freezer Calculations

[C] Attachment 3-Cost Calculation References

[D] Attachment 4-Reach-In Savings Calculations

[E] CPUC California Standard Practice Manual: Economic Analysis of Demand-Side Programs and Projects, page 18. California Public Utilities Commission, October 2001.