Work Paper SCE13HC001

Revision 2

Southern California Edison

Energy Star Room Air Conditioners

# At-a-Glance Summary

| Measure Codes | AC-60931 Energy Star Room Air Conditioner DX Equipment |
| --- | --- |
| Measure Description | The measure for this work paper involves the purchase and installation of residential room air conditioners (RAC) that meet Energy Star RAC requirements (ES-RAC). |
| Base Case Description | The base case for this work paper is the minimum Title 20 code efficiency RAC (C-RAC). |
| Units | Per Unit (i.e. per ES-RAC) |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | 9 years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Res-sAll-mHVAC-RmAC-dn = 0.36, Res-Default>2 = 0.55, Res-Default-HTR-di = 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/09/2012 | Joseph Ling/AESC | Original work paper for 2013 PC |
|  | 04/15/2013 | Andres Fergadiotti/SCE | MIDI Program Description |
| 1 | 03/17/2014 | Briana Rogers/AESC | - Work paper has been updated to reflect change in Code effective June 1, 2014. |
|  | 03/17/2014 | Cassie Cuaresma /SCE | - Work paper updated for reporting period, effective 7/1/2014 – 12/31/2014. |
| 2 | 01/27/2016 | Eduardo Munoz/SCE | -New template update for 2016 program year -WP effective from 1/1/2016 thru 12/31/2016 -No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## Measure Description & Background

This work paper details the measure for the purchase and installation of residential room air conditioners (RAC) that meet Energy Star RAC requirements (ES-RAC) instead of minimum

Code efficiency RACs (C-RAC) requirement.  Thus, a C-RAC is the base case and purchase of an ES-RAC is the measure case for this work paper.  Installation costs are presumed to be identical.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Purchase and installation of residential room air conditioners (RAC) that meet Energy Star RAC requirements (ES-RAC). |
| Existing Condition | Minimum Title 20 code efficiency RAC (C-RAC). |
| Code/Standard | Title 20 code efficiency RAC (C-RAC). |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | AC-60931 | N/A | Energy Star Room Air Conditioner DX Equipment |

The capacity and the rated efficiency of the ES-RAC shall be documented in the rebate application for this measure to facilitate future tracking and Measurement and Evaluation efforts.

ES-RAC’s are defined as having a minimum of 10% energy efficiency improvement over minimum DOE requirements.

### Middle Income Direct Install (MIDI) Program Requirements

Ruling R.09-11-014 ordering #55 ordered the Investor Owned Utilities (IOUs) to establish MIDI programs in 2013 – 2014 to explore expansion of eligibility to improve the program’s comprehensiveness.

The CPUC Final Decision on 2013-2014 EE Applications authorizes the IOUs may execute various trials/pilots under the Energy Upgrade California Program.  The EUC Program is authorized to conduct additional pilots/trials to test technologies, applications or other programmatic design and marketing possibilities. The learning from such activities is expected to contribute to the program innovation and further learning.

The Local MIDI Program will be offered by SCE and SoCalGas to eligible customers residing in Single Family (SF) and Multi Family (MF) properties (MF common areas excluded) served by SCE and SoCalGas.  Participants must be income eligible (between 201% and 250% of FPG).

## Technical Description

Title 20 [51] established the following definition in Section 1602(c) Air Conditioners,

“Room Air Conditioner” (RAC) means a factory-encased air conditioner that is designed:

(1) as a unit for mounting in a window, through a wall, or as a console, and

(2) for delivery without ducts of conditioned air to an enclosed space.

## 1.3 Installation Types and Delivery Mechanisms

The program/install type for the above measure is:

- Replace on Burn-out (ROB)

The delivery methods that are available for these measures are:

- Financial Support - Down-Stream Incentive – Deemed

- Financial Support - Direct Install

- Partnership - Down-Stream Incentive – Deemed

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

This specific measure is not included in the Database for Energy Efficient Resources (DEER) 2014 Version 4.0 [49].  While DEER does include residential package rooftop AC, package terminal AC, SEER rated package rooftop, SEER rated package rooftop AC, and split or packaged DX AC refrigerant system, none of the residential DEER measures address equipment that provides conditioned air to an enclosed space without the use of ducts.  Therefore, DEER values will not be used as the energy savings and demand reduction estimates.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | No |
| DEER Operating Hours | No |
| DEER eQUEST Prototypes | No |
| DEER Version | N/A |
| Reason for Deviation from DEER | DEER does not contain this type of measure. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-sAll-mHVAC-RmAC-dn | Energy Star Room AC and HP | Res | Any | PreRebDown | 0.36 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer homes.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer homes as HTR. The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria.

SCE’s Multi-Family Energy Efficiency Rebate (MFEER) program addresses the ongoing concern with “split incentives”, where the residents are not the owners of the property, so they lack incentive to improve their energy usage. Similarly, the property owners do not live on-site and pay higher utility expenses due to inefficient appliances, thus lack any incentive to upgrade. The MFEER is designed to drive this customer segment toward participation by offering property owners a variety of energy efficiency measures and services.  The MFEER program will offer and track measure installations in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums.  Measures offered via direct install activities in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums will receive the HTR NTG.  Other measures in the MFEER program will receive default NTG (NTGR_ID: Res-Default>2), unless otherwise specified in DEER.

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Res-RAC-SCE | Residential Room AC; Annual Installation Rate | Res | Any | NonUpStrm | 0.96 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| HV-RAC-ES | Room AC - Energy Star | Res | HVAC | 9 | 3 |

### 1.4.2 Codes and Standards Analysis

The RAC Standards took effect January 1, 1990 and were later modified October 1, 2000.  These RAC Standards are summarized in the State of California Code Of Regulations, Title 20 [356], which are herein referenced.  The RAC Standards effective June 1, 2014 set the requirements for computer room air conditioners (C-RACs).

As stated in Section 1605.1 (b), the EER of room air conditioners and room air-conditioning heat pumps that are manufactured on or after the effective dates shown shall be not less than the applicable values provided in table below.  The Minimum EER or COP column, Effective June 1, 2014, lists the new code requirements for C-RAC units.

1605.1 (b) Table B-2 Standards for Room Air Conditioners and Room Air-Conditioning Heat Pumps

| Appliance | Louvered Sides | Cooling Capacity (Btu/hr) | Minimum EER or COP |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Effective January 1, 1990 | Effective October 1, 2000 | Effective June 1, 2014 |
| Room Air Conditioner | Yes | < 6,000 | 8.0 | 9.7 | 11.0 |
| Room Air Conditioner | Yes | ≥ 6,000 - 7,999 | 8.5 | 9.7 | 11.0 |
| Room Air Conditioner | Yes | ≥ 8,000 - 13,999 | 9.0 | 9.8 | 10.9 |
| Room Air Conditioner | Yes | ≥ 14,000 - 19,999 | 8.8 | 9.7 | 10.7 |
| Room Air Conditioner | Yes | ≥ 20,000-24,999 | 8.2 | 8.5 | 9.4 |
| Room Air Conditioner | Yes | ≥ 25,000 | 8.2 | 8.5 | 9.0 |
| Room Air Conditioner | No | < 6,000 | 8.0 | 9.0 | 10.0 |
| Room Air Conditioner | No | ≥ 6,000 - 7,999 | 8.5 | 9.0 | 10.0 |
| Room Air Conditioner | No | ≥ 8,000 - 10,999 | 8.5 | 8.5 | 9.6 |
| Room Air Conditioner | No | ≥ 11,000 - 13,999 | 8.5 | 8.5 | 9.5 |
| Room Air Conditioner | No | ≥ 14,000 - 19,999 | 8.5 | 8.5 | 9.3 |
| Room Air Conditioner | No | ≥ 20,000 | 8.2 | 8.5 | 9.4 |
| Room Air Conditioning Heat Pump | Yes | < 20,000 | 8.5 | 9.0 | 9.8 |
| Room Air Conditioning Heat Pump | Yes | ≥ 20,000 | 8.5 | 8.5 | 9.3 |
| Room Air Conditioning Heat Pump | No | < 14,000 | 8.0 | 8.5 | 9.3 |
| Room Air Conditioning Heat Pump | No | ≥ 14,000 | 8.0 | 8.0 | 8.7 |
| Casement-Only Room Air Conditioner | Either | Any | * | 8.7 | 9.5 |
| Casement-Slider Room Air Conditioner | Either | Any | * | 9.5 | 10.4 |
| *Casement-only room air conditioners and casement-slider room air conditioners are not separate product classes under standards effective January 1, 1990. Such appliances, if manufactured before October 1, 2000, are subject to the applicable standards in Table B-2 for the other room air conditioners and room air-conditioning heat pumps based on capacity and the presence or absence of louvered sides. |  |  |  |  |  |

Energy Star announced labels for RACs in October 1996 [329].  ES-RACs are defined as having a minimum of 10% energy efficiency improvement over minimum Energy Code requirements.

The Federal Standard (effective June 1, 2014) and Energy Star Energy Efficiency Ratio (EER) requirements are detailed in table below.

ENERGY STAR Qualified Room Air Conditioner (RAC) Eligibility

| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| --- | --- | --- | --- | --- |
| < 6,000 | ≥ 11.0 | ≥12.1 | ≥ 10.0 | ≥11.0 |
| 6,000 to 7,999 |  |  |  |  |
| 8,000 to 10,999 | ≥ 10.9 | ≥12.0 | ≥ 9.6 ≥ 9.5 ≥ 9.4 | ≥10.6 |
| 11,000 to 13,999 |  |  | ≥ 9.5 | ≥10.5 |
| 14,000 to 19,999 | ≥ 10.7 | ≥11.8 | ≥ 9.3 | ≥10.2 |
| ≥ 20,000 | ≥ 9.4 | ≥10.3 | ≥ 9.4 | ≥10.3 |
| Casement | Federal Standard EER |  | ENERGY STAR EER |  |
| Casement-only | ≥ 9.5 |  | ≥ 10.5 |  |
| Casement-slider | ≥ 10.4 |  | ≥ 11.4 |  |
| REVERSE CYCLE (Heat Pump) |  |  |  |  |
| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| < 14,000 | n/a | n/a | ≥ 9.3 | ≥ 10.2 |
| ≥ 14,000 |  |  | ≥ 9.7 | ≥ 10.7 |
| < 20,000 | ≥ 9.8 | ≥ 10.8 | n/a | n/a |
| ≥ 20,000 | ≥ 9.3 | ≥ 10.2 | n/a | n/a |

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2014) | Section 1605.1 (b) | June 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

### 1.5.1 Non-DEER Study Review

To determine both energy savings and demand reduction, this work paper utilizes the Residential Retrofit High Impact Measure Evaluation Report [329] prepared by the CADMUS group, which provides RAC modeled annual energy usage and peak demand reduction in climate zone 6, 8, 9 and 10 based on the Title 20 code effective as of October 1, 2000.  The results of the study were updated to reflect the new Federal code, effective June 1, 2014.  As the CADMUS group did not account for RAC estimated annual energy usage and peak demand reduction in climate zones 13, 14, 15 and 16, the California Statewide Residential Appliance Saturation Study (RASS) [172] results by climate zone are used to develop estimated annual energy and peak demand savings for the remaining climate zones that are not addressed in the Residential Retrofit High Impact Measure Evaluation Report.

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

The Residential Retrofit High Impact Measure Evaluation Report prepared by the CADMUS group, is utilized to obtain estimated annual energy usage and demand reduction in climate zone 6, 8, 9 and 10 based on the Federal code, effective in October 2000.  According to the report, energy usage (kWh) and demand reduction (kW) were determined through a combination of on-site metering and lab tests.  Metering was conducted on a total of 102 participant RAC units for 90 days during the months of June through September 2009. Two meters were used for each unit, the “Watts up? PRO.Net” (recording demand readings), and the “HOBO U12-12” (recording indoor temperature and humidity).

Hourly regression models were developed to estimate hourly energy use as a function of outdoor temperature. The results of the demand at each hour were then summed to estimate the modeled annual kWh savings. As mentioned above, the models were based on metered data from June through September. Behavioral changes that occur outside that time period, and that are not directly captured by change in outdoor temperature are not represented in these results [329].

Furthermore, in an effort to better understand how RAC performance and efficiency varies under different environmental conditions, four room air conditioners were tested in a laboratory.

As the final evaluation concluded in 2010, the study took into consideration the Federal code effective as of October 2000.  An EER of 10.8 was assumed for ENERGY STAR units and an EER of 9.8 was assumed for non-ENERGY STAR units. This is based on a 10,000 btu/h unit.  Table below presents the modeled annual energy usage (kWh) for an ES-RAC and the mean peak demand reduction (kW) from replacing a standard efficiency RAC to a program qualified ENERGY STAR unit.

RAC Modeled Annual Energy Usage (kWh), Estimated Annual Energy and Peak Demand Savings (Climate Zone 6, 8, 9 and 10)

| Climate Zone | Annual Energy Usage (kWh/unit) | Mean Unit Peak Demand Savings (kW/unit) |
| --- | --- | --- |
| 6 | 201 | 0.014 |
| 7 | 240 | 0.015 |
| 8 | 333 | 0.034 |
| 9 | 485 | 0.041 |
| 10 | 592 | 0.063 |

Results from the Residential Retrofit High Impact Measure Evaluation Report [329] have been amended to reflect the new Federal code effective June 1, 2014.  An EER of 12.0 was assumed for ENERGY STAR units and an EER of 10.9 was assumed for non-ENERGY STAR units.  For consistency purposes, the efficiencies are based on a 10,000 btu/h unit with louvered sides.

Table below summarizes the calculated energy usage (kWh) of an ES-RAC for Climate Zone 6, 7, 8, 9 and 10 based on the updated Title 20 code requirements.  Table 10 also provides the total annual energy savings (kWh/unit) and the mean peak demand reduction (kW/unit) from replacing a standard efficiency RAC to a program qualified ENERGY STAR unit.

Energy Star RAC Estimated Annual Energy Usage (kWh), Estimated Annual Energy and Peak Demand Savings (Climate Zone 6, 8, 9 and 10)

| Climate Zone | Annual Energy Usage (kWh/unit) | Estimated Annual Energy Savings (kWh/unit) | Estimated Energy Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| 6 | 180.90 | 18.26 | 0.0137 |
| 7 | 216.00 | 21.80 | 0.0147 |
| 8 | 299.70 | 30.24 | 0.0334 |
| 9 | 436.50 | 44.05 | 0.0402 |
| 10 | 532.80 | 53.77 | 0.0618 |

The estimated annual energy savings and demand reduction for the remaining climate zones are extrapolated by using RASS Energy Use Consumption (EUC) kWh values for RAC [172]. Table below provides the RASS EUCs by climate zone.

RASS EUCs for RACs by climate zone

|  | CZ 1 | CZ 2 | CZ 3 | CZ 4 | CZ 5 | CZ 6 | CZ 7 | CZ 8 | CZ 9 | CZ 10 | CZ 11 | CZ 12 | CZ 13 | CZ 14 | CZ 15 | CZ 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RASS EUC for RAC | 0 | 56 | 20 | 89 | 8 | 82 | 66 | 161 | 237 | 244 | 375 | 199 | 391 | 428 | 329 | 126 |

As shown in table below, RASS EUCs are utilized to estimate energy savings (kWh/unit) and peak demand savings (kW/unit).  The RASS EUCs were used to determine a scaling factor calculated as Residential HIM Savings/RASS EUC.  Based on climatic parallels, energy and demand savings for Climate Zones 11, 13, 14, 15 are derived from the Climate Zone 10 Residential HIM/RASS EUC scaling factor.  Similarly, estimated energy and demand savings for Climate Zones 12 and 16 are determined via the Climate Zone 8 scaling factor.  Remaining Climate Zones 1, 2, 3, 4 and 5 are calculated in a similar manner using Climate Zone 6 as a parallel. See “RoomAC RASS Extrapolation CZ 1-16.xls” in the Attachments section for more details.

Estimated Annual Energy Savings & Demand Reduction

| Climate Zone | Sources | Estimated Annual Energy Savings (kWh/unit) | Estimated Energy Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| 1 | RASS EUC scaling factor | 0.00 | 0.000 |
| 2 | RASS EUC scaling factor | 12.47 | 0.00938 |
| 3 | RASS EUC scaling factor | 4.45 | 0.00335 |
| 4 | RASS EUC scaling factor | 19.81 | 0.0149 |
| 5 | RASS EUC scaling factor | 1.78 | 0.00134 |
| 6 | Residential Retrofit HIM evaluation Report | 18.26 | 0.0137 |
| 7 | Residential Retrofit HIM evaluation Report | 21.80 | 0.0147 |
| 8 | Residential Retrofit HIM evaluation Report | 30.24 | 0.0334 |
| 9 | Residential Retrofit HIM evaluation Report | 44.05 | 0.0402 |
| 10 | Residential Retrofit HIM evaluation Report | 53.77 | 0.0618 |
| 11 | RASS EUC scaling factor | 82.64 | 0.09498 |
| 12 | RASS EUC scaling factor | 37.38 | 0.0412 |
| 13 | RASS EUC scaling factor | 86.16 | 0.09903 |
| 14 | RASS EUC scaling factor | 94.32 | 0.1084 |
| 15 | RASS EUC scaling factor | 72.50 | 0.0833 |
| 16 | RASS EUC scaling factor | 23.67 | 0.0261 |

The demand reduction calculations/estimates must consider the peak demand period as specific summer weekday periods delineated by climate zone, 2:00 PM to 5:00 PM as set forth in CPUC Decision D06-06-063.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | DEER:HVAC_Eff_AC | RES |
| Residential Multi-family | DEER:HVAC_Eff_AC | RES |
| Residential Mobile Home – Double-Wide | DEER:HVAC_Eff_AC | RES |

# Section 4. Costs

## 4.1 Base Case Cost

The base case equipment cost is the purchase price of a unit that meets minimum Federal and State of California appliance standards.  Installed costs for a 10.9 EER Room Air Conditioner, 8,000-13,999 Btu/h, with louvers is estimated at $498 [375].  As labor cost is estimated at $67.61 (see Measure Case Cost section for details), the base material cost is $430.39.

## 4.2 Measure Case Cost

The Evaluation of Energy-Efficiency Standards for Room Air Conditioners in the US [375] estimates the installed cost of a 12.0 EER Room Air Conditioner, 8,000-13,999 Btu/h, with louvers at $605.  DEER provides a Measure Material Cost for RAC-RoomAC-ES of $537.39 [215].  Therefore, the installation cost is the difference between the installed cost and the measure material cost, $67.61.

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| Room Air Conditioner | ROB | $107.00 | $107.00 | N/A |

# Attachments

# References

[51]

[172]

[215]

[329]

[356]

[375]