Work Paper SCE13HC001

Revision 1

Southern California Edison Company

Energy Star Room Air Conditioners

# At-a-Glance Summary

| Applicable Measure Codes: | AC-60391 Energy Star Room Air Conditioner DX Equipment |
| --- | --- |
| Measure Description: | The measure for this work paper involves the purchase and installation of residential room air conditioners (RAC) that meet Energy Star RAC requirements (ES-RAC). |
| Base Case Description: | The base case for this work paper is the minimum Title 20 code efficiency RAC (C-RAC). |
| Energy Impact Common Units: | Per Unit (i.e. per ES-RAC) |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 9 years |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | 0.36 |
| Important Comments: |  |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13HC001.0 | No | 04/09/2012 | Joseph Ling/AESC | Original work paper for 2013 PC |
|  | No | 04/15/2013 | Andres Fergadiotti/SCE | MIDI Program Description |
| SCE13HC001.1 | Yes | 03/17/2014 | Briana Rogers/AESC | - Work paper has been updated to reflect change in Code effective June 1, 2014. |
|  | No | 03/17/2014 | Cassie Cuaresma /SCE | - Work paper updated for reporting period, effective 7/1/2014 – 12/31/2014. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the measure for the purchase and installation of residential room air conditioners (RAC) that meet Energy Star RAC requirements (ES-RAC) instead of minimum

Code efficiency RACs (C-RAC) requirement.  Thus, a C-RAC is the base case and purchase of an ES-RAC is the measure case for this work paper.  Installation costs are presumed to be identical.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| AC-60391 | Energy Star Room Air Conditioner DX Equipment |

The capacity and the rated efficiency of the ES-RAC shall be documented in the rebate application for this measure to facilitate future tracking and Measurement and Evaluation efforts.

ES-RAC’s are defined as having a minimum of 10% energy efficiency improvement over minimum DOE requirements.

### Middle Income Direct Install (MIDI) Program Requirements

Ruling R.09-11-014 ordering #55 ordered the Investor Owned Utilities (IOUs) to establish MIDI programs in 2013 – 2014 to explore expansion of eligibility to improve the program’s comprehensiveness.

The CPUC Final Decision on 2013-2014 EE Applications authorizes the IOUs may execute various trials/pilots under the Energy Upgrade California Program.  The EUC Program is authorized to conduct additional pilots/trials to test technologies, applications or other programmatic design and marketing possibilities. The learning from such activities is expected to contribute to the program innovation and further learning.

The Local MIDI Program will be offered by SCE and SoCalGas to eligible customers residing in Single Family (SF) and Multi Family (MF) properties (MF common areas excluded) served by SCE and SoCalGas.  Participants must be income eligible (between 201% and 250% of FPG).

## 1.2 Technical Description

Title 20 [51] established the following definition in Section 1602(c) Air Conditioners,

“Room Air Conditioner” (RAC) means a factory-encased air conditioner that is designed:

(1) as a unit for mounting in a window, through a wall, or as a console, and

(2) for delivery without ducts of conditioned air to an enclosed space.

## 1.3 Measure Application Type

The program/install type for the above measure is:

- Replace on Burn-out (ROB)

The delivery methods that are available for these measures are:

- Financial Support - Down-Stream Incentive – Deemed

- Financial Support - Direct Install

- Partnership - Down-Stream Incentive – Deemed

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

This specific measure is not included in the Database for Energy Efficient Resources (DEER) 2014 Version 4.0 [49].  While DEER does include residential package rooftop AC, package terminal AC, SEER rated package rooftop, SEER rated package rooftop AC, and split or packaged DX AC refrigerant system, none of the residential DEER measures address equipment that provides conditioned air to an enclosed space without the use of ducts.  Therefore, DEER values will not be used as the energy savings and demand reduction estimates.

DEER 2014 also does not contain Incremental Measure Cost data for this measure.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this type of measure |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-sAll-mHVAC-RmAC-dn | Energy Star Room AC and HP | Res | Any | PreRebDown | 0.36 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |
| Res-Default-HTG-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Res-RAC-SCE | Residential Room AC; Annual Installation Rate | Res | Any | NonUpStrm | 0.96 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex-ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this work paper |
| --- | --- |
| Measure Case UseCategory | HVAC |
| Measure Case UseSubCats | Space Cooling |
| Measure Case TechGroups | Dx AC Equipment |
| Measure Case TechTypes | Room AC |
| Base Case TechGroups | Dx AC Equipment |
| Base Case TechTypes | Room AC |

### 1.4.2 Codes and Standards Analysis

The RAC Standards took effect January 1, 1990 and were later modified October 1, 2000.  These RAC Standards are summarized in the State of California Code Of Regulations, Title 20 [356], which are herein referenced.  The RAC Standards effective June 1, 2014 set the requirements for computer room air conditioners (C-RACs).

As stated in Section 1605.1 (b), the EER of room air conditioners and room air-conditioning heat pumps that are manufactured on or after the effective dates shown shall be not less than the applicable values provided in Table 6.  The Minimum EER or COP column, Effective June 1, 2014, lists the new code requirements for C-RAC units.

Table 6 - 1605.1 (b) Table B-2 Standards for Room Air Conditioners and Room Air-Conditioning Heat Pumps

| Appliance | Louvered Sides | Cooling Capacity (Btu/hr) | Minimum EER or COP |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Effective January 1, 1990 | Effective October 1, 2000 | Effective June 1, 2014 |
| Room Air Conditioner | Yes | < 6,000 | 8.0 | 9.7 | 11.0 |
| Room Air Conditioner | Yes | ≥ 6,000 - 7,999 | 8.5 | 9.7 | 11.0 |
| Room Air Conditioner | Yes | ≥ 8,000 - 13,999 | 9.0 | 9.8 | 10.9 |
| Room Air Conditioner | Yes | ≥ 14,000 - 19,999 | 8.8 | 9.7 | 10.7 |
| Room Air Conditioner | Yes | ≥ 20,000-24,999 | 8.2 | 8.5 | 9.4 |
| Room Air Conditioner | Yes | ≥ 25,000 | 8.2 | 8.5 | 9.0 |
| Room Air Conditioner | No | < 6,000 | 8.0 | 9.0 | 10.0 |
| Room Air Conditioner | No | ≥ 6,000 - 7,999 | 8.5 | 9.0 | 10.0 |
| Room Air Conditioner | No | ≥ 8,000 - 10,999 | 8.5 | 8.5 | 9.6 |
| Room Air Conditioner | No | ≥ 11,000 - 13,999 | 8.5 | 8.5 | 9.5 |
| Room Air Conditioner | No | ≥ 14,000 - 19,999 | 8.5 | 8.5 | 9.3 |
| Room Air Conditioner | No | ≥ 20,000 | 8.2 | 8.5 | 9.4 |
| Room Air Conditioning Heat Pump | Yes | < 20,000 | 8.5 | 9.0 | 9.8 |
| Room Air Conditioning Heat Pump | Yes | ≥ 20,000 | 8.5 | 8.5 | 9.3 |
| Room Air Conditioning Heat Pump | No | < 14,000 | 8.0 | 8.5 | 9.3 |
| Room Air Conditioning Heat Pump | No | ≥ 14,000 | 8.0 | 8.0 | 8.7 |
| Casement-Only Room Air Conditioner | Either | Any | * | 8.7 | 9.5 |
| Casement-Slider Room Air Conditioner | Either | Any | * | 9.5 | 10.4 |
| *Casement-only room air conditioners and casement-slider room air conditioners are not separate product classes under standards effective January 1, 1990. Such appliances, if manufactured before October 1, 2000, are subject to the applicable standards in Table B-2 for the other room air conditioners and room air-conditioning heat pumps based on capacity and the presence or absence of louvered sides. |  |  |  |  |  |

Energy Star announced labels for RACs in October 1996 [329].  ES-RACs are defined as having a minimum of 10% energy efficiency improvement over minimum Energy Code requirements.

The Federal Standard (effective June 1, 2014) and Energy Star Energy Efficiency Ratio (EER) requirements are detailed in Table 7.

Table 7 ENERGY STAR Qualified Room Air Conditioner (RAC) Eligibility

| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| --- | --- | --- | --- | --- |
| < 6,000 | ≥ 11.0 | ≥12.1 | ≥ 10.0 | ≥11.0 |
| 6,000 to 7,999 |  |  |  |  |
| 8,000 to 10,999 | ≥ 10.9 | ≥12.0 | ≥ 9.6 ≥ 9.5 ≥ 9.4 | ≥10.6 |
| 11,000 to 13,999 |  |  | ≥ 9.5 | ≥10.5 |
| 14,000 to 19,999 | ≥ 10.7 | ≥11.8 | ≥ 9.3 | ≥10.2 |
| ≥ 20,000 | ≥ 9.4 | ≥10.3 | ≥ 9.4 | ≥10.3 |
| Casement | Federal Standard EER |  | ENERGY STAR EER |  |
| Casement-only | ≥ 9.5 |  | ≥ 10.5 |  |
| Casement-slider | ≥ 10.4 |  | ≥ 11.4 |  |
| REVERSE CYCLE (Heat Pump) |  |  |  |  |
| Capacity (Btu/Hr) | Federal Standard EER, with louvered sides | ENERGY STAR EER, with louvered sides | Federal Standard EER, without louvered sides | ENERGY STAR EER, without louvered sides |
| < 14,000 | n/a | n/a | ≥ 9.3 | ≥ 10.2 |
| ≥ 14,000 |  |  | ≥ 9.7 | ≥ 10.7 |
| < 20,000 | ≥ 9.8 | ≥ 10.8 | n/a | n/a |
| ≥ 20,000 | ≥ 9.3 | ≥ 10.2 | n/a | n/a |

### 1.4.3 Non-DEER Study Review

To determine both energy savings and demand reduction, this work paper utilizes the Residential Retrofit High Impact Measure Evaluation Report [329] prepared by the CADMUS group, which provides RAC modeled annual energy usage and peak demand reduction in climate zone 6, 8, 9 and 10 based on the Title 20 code effective as of October 1, 2000.  The results of the study were updated to reflect the new Federal code, effective June 1, 2014.  As the CADMUS group did not account for RAC estimated annual energy usage and peak demand reduction in climate zones 13, 14, 15 and 16, the California Statewide Residential Appliance Saturation Study (RASS) [172] results by climate zone are used to develop estimated annual energy and peak demand savings for the remaining climate zones that are not addressed in the Residential Retrofit High Impact Measure Evaluation Report.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 8 below identifies the value/methodology used for the measures in this work paper.

Table 8 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| HV-RAC-ES | Residential | HVAC | Room AC – Energy Star | 9 | 3 |

# Section 2. Energy Savings & Demand Reduction Calculations

The Residential Retrofit High Impact Measure Evaluation Report prepared by the CADMUS group, is utilized to obtain estimated annual energy usage and demand reduction in climate zone 6, 8, 9 and 10 based on the Federal code, effective in October 2000.  According to the report, energy usage (kWh) and demand reduction (kW) were determined through a combination of on-site metering and lab tests.  Metering was conducted on a total of 102 participant RAC units for 90 days during the months of June through September 2009. Two meters were used for each unit, the “Watts up? PRO.Net” (recording demand readings), and the “HOBO U12-12” (recording indoor temperature and humidity).

Hourly regression models were developed to estimate hourly energy use as a function of outdoor temperature. The results of the demand at each hour were then summed to estimate the modeled annual kWh savings. As mentioned above, the models were based on metered data from June through September. Behavioral changes that occur outside that time period, and that are not directly captured by change in outdoor temperature are not represented in these results [329].

Furthermore, in an effort to better understand how RAC performance and efficiency varies under different environmental conditions, four room air conditioners were tested in a laboratory.

As the final evaluation concluded in 2010, the study took into consideration the Federal code effective as of October 2000.  An EER of 10.8 was assumed for ENERGY STAR units and an EER of 9.8 was assumed for non-ENERGY STAR units. This is based on a 10,000 btu/h unit.  Table 9 presents the modeled annual energy usage (kWh) for an ES-RAC and the mean peak demand reduction (kW) from replacing a standard efficiency RAC to a program qualified ENERGY STAR unit.

Table 9 RAC Modeled Annual Energy Usage (kWh), Estimated Annual Energy and Peak Demand Savings (Climate Zone 6, 8, 9 and 10)

| Climate Zone | Annual Energy Usage (kWh/unit) | Mean Unit Peak Demand Savings (kW/unit) |
| --- | --- | --- |
| 6 | 201 | 0.014 |
| 7 | 240 | 0.015 |
| 8 | 333 | 0.034 |
| 9 | 485 | 0.041 |
| 10 | 592 | 0.063 |

Results from the Residential Retrofit High Impact Measure Evaluation Report [329] have been amended to reflect the new Federal code effective June 1, 2014.  An EER of 12.0 was assumed for ENERGY STAR units and an EER of 10.9 was assumed for non-ENERGY STAR units.  For consistency purposes, the efficiencies are based on a 10,000 btu/h unit with louvered sides.

Table 10 summarizes the calculated energy usage (kWh) of an ES-RAC for Climate Zone 6, 7, 8, 9 and 10 based on the updated Title 20 code requirements.  Table 10 also provides the total annual energy savings (kWh/unit) and the mean peak demand reduction (kW/unit) from replacing a standard efficiency RAC to a program qualified ENERGY STAR unit.

Table 10 Energy Star RAC Estimated Annual Energy Usage (kWh), Estimated Annual Energy and Peak Demand Savings (Climate Zone 6, 8, 9 and 10)

| Climate Zone | Annual Energy Usage (kWh/unit) | Estimated Annual Energy Savings (kWh/unit) | Estimated Energy Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| 6 | 180.90 | 18.26 | 0.0137 |
| 7 | 216.00 | 21.80 | 0.0147 |
| 8 | 299.70 | 30.24 | 0.0334 |
| 9 | 436.50 | 44.05 | 0.0402 |
| 10 | 532.80 | 53.77 | 0.0618 |

The estimated annual energy savings and demand reduction for the remaining climate zones are extrapolated by using RASS Energy Use Consumption (EUC) kWh values for RAC [172]. Table 11 provides the RASS EUCs by climate zone.

Table 11 RASS EUCs for RACs by climate zone

|  | CZ 1 | CZ 2 | CZ 3 | CZ 4 | CZ 5 | CZ 6 | CZ 7 | CZ 8 | CZ 9 | CZ 10 | CZ 11 | CZ 12 | CZ 13 | CZ 14 | CZ 15 | CZ 16 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RASS EUC for RAC | 0 | 56 | 20 | 89 | 8 | 82 | 66 | 161 | 237 | 244 | 375 | 199 | 391 | 428 | 329 | 126 |

As shown in Table 12, RASS EUCs are utilized to estimate energy savings (kWh/unit) and peak demand savings (kW/unit).  The RASS EUCs were used to determine a scaling factor calculated as Residential HIM Savings/RASS EUC.  Based on climatic parallels, energy and demand savings for Climate Zones 11, 13, 14, 15 are derived from the Climate Zone 10 Residential HIM/RASS EUC scaling factor.  Similarly, estimated energy and demand savings for Climate Zones 12 and 16 are determined via the Climate Zone 8 scaling factor.  Remaining Climate Zones 1, 2, 3, 4 and 5 are calculated in a similar manner using Climate Zone 6 as a parallel. See “RoomAC RASS Extrapolation CZ 1-16.xls” in the Attachments section for more details.

Table 12 Estimated Annual Energy Savings & Demand Reduction

| Climate Zone | Sources | Estimated Annual Energy Savings (kWh/unit) | Estimated Energy Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| 1 | RASS EUC scaling factor | 0.00 | 0.000 |
| 2 | RASS EUC scaling factor | 12.47 | 0.00938 |
| 3 | RASS EUC scaling factor | 4.45 | 0.00335 |
| 4 | RASS EUC scaling factor | 19.81 | 0.0149 |
| 5 | RASS EUC scaling factor | 1.78 | 0.00134 |
| 6 | Residential Retrofit HIM evaluation Report | 18.26 | 0.0137 |
| 7 | Residential Retrofit HIM evaluation Report | 21.80 | 0.0147 |
| 8 | Residential Retrofit HIM evaluation Report | 30.24 | 0.0334 |
| 9 | Residential Retrofit HIM evaluation Report | 44.05 | 0.0402 |
| 10 | Residential Retrofit HIM evaluation Report | 53.77 | 0.0618 |
| 11 | RASS EUC scaling factor | 82.64 | 0.09498 |
| 12 | RASS EUC scaling factor | 37.38 | 0.0412 |
| 13 | RASS EUC scaling factor | 86.16 | 0.09903 |
| 14 | RASS EUC scaling factor | 94.32 | 0.1084 |
| 15 | RASS EUC scaling factor | 72.50 | 0.0833 |
| 16 | RASS EUC scaling factor | 23.67 | 0.0261 |

The demand reduction calculations/estimates must consider the peak demand period as specific summer weekday periods delineated by climate zone, 2:00 PM to 5:00 PM as set forth in CPUC Decision D06-06-063.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:HVAC_Eff_AC load shape.  See Table 13 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 13 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential Single Family | RES | DEER:HVAC_Eff_AC |
| Residential Multi-family | RES | DEER:HVAC_Eff_AC |
| Residential Mobile Home – Double-Wide | RES | DEER:HVAC_Eff_AC |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

The base case equipment cost is the purchase price of a unit that meets minimum Federal and State of California appliance standards.  Installed costs for a 10.9 EER Room Air Conditioner, 8,000-13,999 Btu/h, with louvers is estimated at $498 [375].  As labor cost is estimated at $67.61 (see Measure Case Cost section for details), the base material cost is $430.39.

## 4.2 Measure Case Cost

The Evaluation of Energy-Efficiency Standards for Room Air Conditioners in the US [375] estimates the installed cost of a 12.0 EER Room Air Conditioner, 8,000-13,999 Btu/h, with louvers at $605.  DEER provides a Measure Material Cost for RAC-RoomAC-ES of $537.39 [215].  Therefore, the installation cost is the difference between the installed cost and the measure material cost, $67.61.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB, GMC is represented by the equation below:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

*Note: Unless stated otherwise the measure case labor and base case labor are assumed to be the same value reducing the equation to the following:

GMC 	= Measure Equipment Cost – Base Case Equipment Cost

As seen in the above equations, the gross measure cost is dependent on the installation type of the measure.

Table 14 Labor and Equipment Cost for Room Air Conditioners

| Case Classification | Unit | Labor Cost ($) | Base Case Equipment Cost ($) | Measure Equipment Cost ($) | Gross Measure Cost ($) |
| --- | --- | --- | --- | --- | --- |
| Room Air Conditioner | Unit | $67.61 | $430.39 | $537.39 | $107.00 |

In the case of ROB, the equipment being replaced is assumed to have failed in place or is past its useful life so the customer is in the situation of having to purchase new equipment.  The customer is then faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.  Since the customer will be spending money to replace equipment anyway, the gross cost for the energy efficient measure is the premium paid above the non-efficient or code baseline equipment.

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost (IMC) is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  While IMC has a straight forward definition depending on the install type, the equation does vary.  The incremental cost is only used to help determine program incentives and is not affected by the RUL and EUL-RUL periods and may differ from the cost used for reporting.

In the case of ROB the IMC and GMC end up having the same equation and hence the same value.

IMC = GMC

The incremental measure cost is the measure material cost minus the base material cost, which equates to $107.00.

# Attachments

# References

[26]

[31]

[51]

[172]

[213]

[215]

[329]

[351]

[356]

[375]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |