Work Paper SCE13HC002

Revision 2

Southern California Edison

Reflective Window Film

# At-a-Glance Summary

| Measure Codes | BE-58923 |
| --- | --- |
| Measure Description | Adding reflective window film to existing windows |
| Base Case Description | Existing windows with a DEER-specified Solar Heat Gain Coefficient (SHGC) that varies by building type, vintage, and climate zone |
| Units | Sq ft |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | DEER14: 10 years |
| Measure Installation Type | Retrofit Add-on (REA) |
| Net-to-Gross Ratio | DEER14: 0.6 Com-Default>2yrs DEER14: 0.85 Com-Default-HTR-di |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 1 | 04/29/2014 | Linda Wan, P.E./TRC Energy Services | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. |
| 2 | 1/28/2016 | Eduardo Munoz/SCE | -New template update for 2016 program year -WP effective from 1/1/2016 thru 12/31/2016 -No value modifications -Removed SCE building types |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The base case is an existing window on a commercial building, with a DEER-specified Solar Heat Gain Coefficient (SHGC) that varies by building type, vintage, and climate zone. The measure case involves adding reflective window film to the base case window. The measure case SHGC is assumed to be 0.39. The following table shows the measure that is included in this work paper.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Window with Reflective Window Film |
| Existing Condition | Existing Window |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | BE-58923 |  | Window with Reflective Window Film (0.39 SHGC) replacing Existing Window |

Eligibility Requirements

- The window film must have a minimum 5 year manufacturer’s warranty.

- The conditioned space must not be evaporatively cooled.

- The window film must be installed on single-pane glass.

- The window film must have an SHGC less than or equal to 0.39. Alternatively it can have an SHGC less than or equal to 0.47, and a visible transmittance/SHGC ratio greater than 1.3.

- If the existing window already has window film, the existing film must first be removed. The existing film must be at least 13 years old.

Documentation Requirements

- Manufacturer specifications and/or NFRC specifications are required.

- An invoice will be required to document the installation of the window film. The invoice must include the window dimensions.

- A copy of the warranty is required.

## 1.2 Technical Description

As a lower cost alternative to replacing windows, window films are commonly applied to the inside of glazing to help limit the heat gain into a space. Window films may also help reduce glare and UV transmission as well as change the exterior building appearance. Limiting the solar heat gain into a space impacts the need and run time of heating and cooling systems.

## 1.3 Installation Types and Delivery Mechanisms

The delivery methods are the following:

- Financial Support - Down-Stream Incentive – Deemed

- Financial Support - Direct Install

- Partnership - Down-Stream Incentive – Deemed

- Partnership - Direct Install

- Partnership - On-bill Finance/Loan

The incentive method is Retrofit Add-on (REA).  This is a subtype of RET that is specifically for adding a piece of equipment to existing equipment to make the overall equipment more efficient.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

The Database for Energy Efficient Resources (DEER) Version 2014 does not contain the window film measure specified in this work paper.  However, DEER 2014 does contain related measures involving the replacement of existing windows with more efficient windows. These DEER measures are used to calculate the savings for this work paper.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | Yes |
| Scaled DEER measure | Yes |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | No |
| DEER Operating Hours | No |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER14 |
| Reason for Deviation from DEER | DEER does not contain this exact measure, but DEER measures were used in the calculations. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTG kWh | NTGR Therms |
| --- | --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Commercial | Any | Any | 0.6 | 0.6 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Commercial | Any | DirInstall | 0.85 | 0.85 |

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

- Business Size – Customer must have less than ten employees

- Language – Customer’s primary language spoken is not English

- Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.”

The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties.

The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| GlazDaylt-WinFilm | Reflective Window Films & Sunscreens - Commercial | Com | BldgEnv | 10 | 3.3 |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] provides mandatory requirements in Tables 110.6-A and 110.6-B for fenestration products that are not rated in accordance with NFRC 100 or NFRC 200. Windows with window film applied are subject to these requirements.

Since BE-58923 is an REA measure, the base case is the existing window. The measure case window with window film will be affected by Title 24 if it is not rated by NFRC 200, although the measure case 0.39 SHGC already exceeds the requirements in Table 110.6-B.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Table 110.6-A Default Fenestration Product U-Factors, Table 110.6-B Default Solar Heat Gain Coefficient (SHGC) | July 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

N/A

### 1.5.1 Non-DEER Study Review

There are no applicable non-DEER studies associated with this work paper.

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

DEER 2014 measures are used in a linear interpolation/extrapolation method to estimate energy savings for a window with reflective window film that has a SHGC of 0.39; refer to Section 2.1 for details. The DEER measures used in this work paper are D03-017 through D03-024. Attachment 2 contains the data files for measures that are taken directly from the DEER 2014 READi Tool v1.0.5; see tab “DEER Calcs from READi.” These results have not been modified and are only being included in the work paper for reference.

## 2.1 Energy Savings, Peak Demand Reduction, and Gas Savings Estimation Methodology – Linear Interpolation/Extrapolation Method

The energy savings are derived from DEER 2014 using the following steps:

- From READi v1.0.5, export the savings for the following measures:

  - D03-017: North glass SHGC 15% less than required by T-24

  - D03-018: East glass SHGC 20% less than required by T-24

  - D03-019: South glass SHGC 20% less than required by T-24

  - D03-020: West glass SHGC 20% less than required by T-24

  - D03-021: North glass SHGC 20% less than required by T-24

  - D03-022: East glass SHGC 30% less than required by T-24

  - D03-023: South glass SHGC 30% less than required by T-24

  - D03-024: West glass SHGC 30% less than required by T-24

The savings are already vintage weighted, since the vintage “Ex” is selected.

- From Title 24 2005, find the required SHGC values for each climate zone. Then multiply each value by 85%, 80%, or 70%, to determine the measure case SHGC values for each of the 8 measures indicated in Step 1.

- Each measure now has 2 data points that can be used for linear interpolation/extrapolation:

- For the North orientation, these data points are:

  - 15% less than T-24 SHGC value with corresponding savings from DEER

  - 20% less than T-24 SHGC value with corresponding savings from DEER

- For South, East, and West orientations:

  - 20% less than T-24 SHGC value with corresponding savings from DEER

  - 30% less than T-24 SHGC value with corresponding savings from DEER

Use linear interpolation to determine the kWh, kW, and therm savings corresponding to a SHGC value of 0.39. Negative savings are reported as 0.

- Since DEER provides savings with units of “per 100 sq ft of window”, divide savings results by 100.

- For each measure, take the average of all 4 orientations.

- As requested by the Energy Division (ED), multiply all savings by adjustment factors to account for differences between eQUEST’s Simplified window specification input method and the Glass Type Code approach (more detail in Section 2.3). The results are the final savings for this work paper.

Full savings calculations are in Attachment 1. An example of the linear interpolation/extrapolation is provided in the tab labeled “SHGC Linear IntEx Example.”  The kWh, kW, and therm savings were calculated the same way.

## 2.2 Energy Savings, Peak Demand Reduction, and Gas Savings for Building Types Not Available in DEER 2014

The following building types were not available for the 8 DEER measures mentioned above:

- Lodging – Guest Rooms

- Retail – Multistory Large

Therefore the energy savings were weighted based on SCE’s multipliers for each building type above (Attachment 2, tab “Weights for Non-DEER Buildings”). For example, the savings for the Lodging-Guest Rooms building type were calculated as 100% of the savings from the Lodging-Motel building type.  The gas savings and peak demand reduction were calculated the same way.

## 2.3 Correction of Savings Values Reported in DEER

Based on direction from the Energy Division, the methodology used in the DEER measures referred to in this work paper needed to be verified. The DEER measures use a simplified approach of defining the glass properties in the base case and measure case calculations. The simplified approach defines a window with three window components: U-factor, SHGC and Visible Transmittance. The measure case simply shifts the SHGC to the proposed values defined in each measure while leaving the U-factor and Visible Transmittance values unchanged.

In order to verify the “Simplified Approach” used in DEER is appropriate, building simulations in eQUEST were generated using the Simplified Approach and a Glass Type Code (GTC) Approach. The GTC Approach uses detailed window specifications to define the base case and measure case windows. The detailed window specifications were generated using the WINDOW 7 software and were directly imported into eQUEST.

The calculations were performed for three building types (Office – Large, Residential – Single Family, and Retail – Small) in four climate zones (3, 9, 13 and 16) for both single pane and dual pane windows. It was determined that the Simplified approach was overestimating the energy savings, peak demand reduction and gas savings compared to the more detailed GTC Approach.

Based on these findings it was determined that the savings needed to be corrected for the overestimation of savings due to the Simplified Approach used in the DEER methodology. The following table shows the average difference found in the two approaches:

Difference between GTC and Simplified Approaches

| % Difference in Energy Savings of GTC vs. Simplified Approaches (kWh/yr) | % Difference in Peak Demand Reduction of GTC vs. Simplified Approaches (kW) | % Difference in Gas Savings of GTC vs. Simplified Approaches (Therms/yr) |
| --- | --- | --- |
| -25% | -29% | -23% |

The application of these adjustment factors is shown in Attachment 2.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | Window_Tint-Ret | Large_Office |
| Education - Primary School | Window_Tint-Ret | Small_Office |
| Education - Secondary School | Window_Tint-Ret | Small_Office |
| Education - Relocatable Classroom | Window_Tint-Ret | Small_Office |
| Education - Community College | Window_Tint-Ret | Large_Office |
| Education – University | Window_Tint-Ret | Large_Office |
| Grocery | Window_Tint-Ret | Small_Office |
| Health/Medical - Hospital | Window_Tint-Ret | Large_Office |
| Health/Medical - Nursing Home | Window_Tint-Ret | Small_Office |
| Lodging – Hotel | Window_Tint-Ret | Small_Office |
| Lodging - Guest Rooms | Window_Tint-Ret | Small_Office |
| Lodging – Motel | Window_Tint-Ret | Small_Office |
| Manufacturing - Bio/Tech | Window_Tint-Ret | Large_Office |
| Manufacturing - Light Industrial | Window_Tint-Ret | Large_Office |
| Office – Large | Window_Tint-Ret | Large_Office |
| Office – Small | Window_Tint-Ret | Small_Office |
| Restaurant - Fast-Food | Window_Tint-Ret | Small_Office |
| Restaurant - Sit-Down | Window_Tint-Ret | Small_Office |
| Retail - Multistory Large | Window_Tint-Ret | Large_Office |
| Retail - Single-Story Large | Window_Tint-Ret | Large_Office |
| Retail – Small | Window_Tint-Ret | Small_Office |
| Storage – Conditioned | Window_Tint-Ret | Large_Office |

# Section 4. Costs

## 4.1 Base Case Cost

For REA measures, the base case cost is $0.00.

## 4.2 Measure Case Cost

The DEER spreadsheet contains cost information for non-residential, single-pane, clear glass windows with window film per square foot of window. The cost is reported per square foot to be consistent with the energy savings, peak demand reduction, and therms savings. The table below shows the DEER Cost Case IDs and the associated material and labor costs.

Measure Costs

| Cost Case ID | Cost Case Description | Material Cost | Installation Labor Cost – Retrofit | Total Costs/ FMC/IMC | Normalizing Unit |
| --- | --- | --- | --- | --- | --- |
| D08-N-GlazDaylt-WinFilm-37SHGC-NonNorth | Low SHGC Window Film | $1.98 | $3.66 | $5.64 | Sq. Ft. |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| BE-58923 | REA | $5.64 | $5.64 | N/A |

# Attachments

# References

| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| --- | --- |