Work Paper SCE13HC002

Revision 1

Southern California Edison Company

Reflective Window Film

# At-a-Glance Summary

| Applicable Measure Codes: | BE-58923 |
| --- | --- |
| Measure Description: | Adding reflective window film to existing windows |
| Base Case Description: | Existing windows with a DEER-specified Solar Heat Gain Coefficient (SHGC) that varies by building type, vintage, and climate zone |
| Energy Impact Common Units: | Sq ft |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | DEER14: 10 years |
| Measure Application Type: | Retrofit Add-on (REA) |
| Net-to-Gross Ratios: | DEER14: 0.6 for any Commercial DEER14: 0.85 for Commercial Direct Install |
| Important Comments: | - Residential measures have been removed from this revision. - This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13HC002.1 | No | 04/29/2014 | Linda Wan, P.E./TRC Energy Services | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The base case is an existing window on a commercial building, with a DEER-specified Solar Heat Gain Coefficient (SHGC) that varies by building type, vintage, and climate zone. The measure case involves adding reflective window film to the base case window. The measure case SHGC is assumed to be 0.39. The following table shows the measure that is included in this work paper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| BE-58923 | Window with Reflective Window Film (0.39 SHGC) replacing Existing Window |

Implementation Requirements

- Measure BE-58923 is approved for the following building types:

Table 2 Building Types

| Sector | Building Type |  |  |
| --- | --- | --- | --- |
| Commercial | Assembly | Health/Medical - Hospital | Office – Large |
|  | Education - Community College | Health/Medical - Nursing Home | Office – Small |
|  | Education - Primary School | Industrial | Restaurant - Fast-Food |
|  | Education - Relocatable Classroom | Lodging – Guest Rooms | Restaurant - Sit-Down |
|  | Education - Secondary School | Lodging - Hotel | Retail – Multistory Large |
|  | Education - University | Lodging - Motel | Retail - Single-Story Large |
|  | Grocery | Manufacturing - Bio/Tech | Retail – Small |
|  | Food Store | Manufacturing - Light Industrial | Storage – Conditioned |
|  | Health/Medical – Clinic | Misc – Commercial | Transportation - Communication - Utilities |

- The measure is approved for installation in all PG&E and SCE climate zones.

Eligibility Requirements

- The window film must have a minimum 5 year manufacturer’s warranty.

- The conditioned space must not be evaporatively cooled.

- The window film must be installed on single-pane glass.

- The window film must have an SHGC less than or equal to 0.39. Alternatively it can have an SHGC less than or equal to 0.47, and a visible transmittance/SHGC ratio greater than 1.3.

- If the existing window already has window film, the existing film must first be removed. The existing film must be at least 13 years old.

Documentation Requirements

- Manufacturer specifications and/or NFRC specifications are required.

- An invoice will be required to document the installation of the window film. The invoice must include the window dimensions.

- A copy of the warranty is required.

## 1.2 Technical Description

As a lower cost alternative to replacing windows, window films are commonly applied to the inside of glazing to help limit the heat gain into a space. Window films may also help reduce glare and UV transmission as well as change the exterior building appearance. Limiting the solar heat gain into a space impacts the need and run time of heating and cooling systems.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this work paper and application types used by READi.

The delivery methods are the following:

- Financial Support - Down-Stream Incentive – Deemed

- Financial Support - Direct Install

- Partnership - Down-Stream Incentive – Deemed

- Partnership - Direct Install

- Partnership - On-bill Finance/Loan

The incentive method is Retrofit Add-on (REA).  This is a subtype of RET that is specifically for adding a piece of equipment to existing equipment to make the overall equipment more efficient.

## 1.4 Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

The Database for Energy Efficient Resources (DEER) Version 2014 does not contain the window film measure specified in this work paper.  However, DEER 2014 does contain related measures involving the replacement of existing windows with more efficient windows. These DEER measures are used to calculate the savings for this work paper.

The measures listed in DEER do not cover the following building types:  Food Store, Health – Clinic, Lodging – Guest Rooms, Industrial, Retail – Multistory Large, Transportation – Communication - Utilities and Misc-Commercial.  However, savings from similar building types are mapped onto these building types.

Table 3 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | Yes, scaling was used in additional to other methodology. |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this exact measure, but DEER measures were used in the calculations. |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | D03-017 North glass SHGC 15% less than required by T24 |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 4 below.

Table 4 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG kWh* | NTGR Therms* |
| --- | --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Commercial | Any | Any | 0.6 | 0.6 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Commercial | Any | DirInstall | 0.85 | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no final version of the installation rate table. To address appropriate selection of the installation rate the date of the work paper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 5 below.

Table 5 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures; however, the values will not be tracked in the work papers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the work paper, refer the Excel calculation template.

Table 6 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | BldgEnv |
| Measure Case UseSubCats | Fenestration |
| Measure Case TechGroups | Fenest |
| Measure Case TechTypes | WinFilm |
| Base Case TechGroups | Fenest |
| Base Case TechTypes | ManufWin |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] provides mandatory requirements in Tables 110.6-A and 110.6-B for fenestration products that are not rated in accordance with NFRC 100 or NFRC 200. Windows with window film applied are subject to these requirements.

Since BE-58923 is an REA measure, the base case is the existing window. The measure case window with window film will be affected by Title 24 if it is not rated by NFRC 200, although the measure case 0.39 SHGC already exceeds the requirements in Table 110.6-B.

Table 7 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Table 110.6-A Default Fenestration Product U-Factors, Table 110.6-B Default Solar Heat Gain Coefficient (SHGC) | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

There are no applicable non-DEER studies associated with this work paper.

### 1.4.4 Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 8 below identifies the value/methodology used for the measures in this work paper.

Table 8 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| GlazDaylt-WinFilm | Non-Residential | Building Envelope | Reflective Window Films & Sunscreens | 10 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

DEER 2014 measures are used in a linear interpolation/extrapolation method to estimate energy savings for a window with reflective window film that has a SHGC of 0.39; refer to Section 2.1 for details. The DEER measures used in this work paper are D03-017 through D03-024. Attachment 2 contains the data files for measures that are taken directly from the DEER 2014 READi Tool v1.0.5; see tab “DEER Calcs from READi.” These results have not been modified and are only being included in the work paper for reference.

## 2.1 Energy Savings, Peak Demand Reduction, and Gas Savings Estimation Methodology – Linear Interpolation/Extrapolation Method

The energy savings are derived from DEER 2014 using the following steps:

- From READi v1.0.5, export the savings for the following measures:

  - D03-017: North glass SHGC 15% less than required by T-24

  - D03-018: East glass SHGC 20% less than required by T-24

  - D03-019: South glass SHGC 20% less than required by T-24

  - D03-020: West glass SHGC 20% less than required by T-24

  - D03-021: North glass SHGC 20% less than required by T-24

  - D03-022: East glass SHGC 30% less than required by T-24

  - D03-023: South glass SHGC 30% less than required by T-24

  - D03-024: West glass SHGC 30% less than required by T-24

The savings are already vintage weighted, since the vintage “Ex” is selected.

- From Title 24 2005, find the required SHGC values for each climate zone. Then multiply each value by 85%, 80%, or 70%, to determine the measure case SHGC values for each of the 8 measures indicated in Step 1.

- Each measure now has 2 data points that can be used for linear interpolation/extrapolation:

- For the North orientation, these data points are:

  - 15% less than T-24 SHGC value with corresponding savings from DEER

  - 20% less than T-24 SHGC value with corresponding savings from DEER

- For South, East, and West orientations:

  - 20% less than T-24 SHGC value with corresponding savings from DEER

  - 30% less than T-24 SHGC value with corresponding savings from DEER

Use linear interpolation to determine the kWh, kW, and therm savings corresponding to a SHGC value of 0.39. Negative savings are reported as 0.

- Since DEER provides savings with units of “per 100 sq ft of window”, divide savings results by 100.

- For each measure, take the average of all 4 orientations.

- As requested by the Energy Division (ED), multiply all savings by adjustment factors to account for differences between eQUEST’s Simplified window specification input method and the Glass Type Code approach (more detail in Section 2.3). The results are the final savings for this work paper.

Full savings calculations are in Attachment 1. An example of the linear interpolation/extrapolation is provided in the tab labeled “SHGC Linear IntEx Example.”  The kWh, kW, and therm savings were calculated the same way.

## 2.2 Energy Savings, Peak Demand Reduction, and Gas Savings for Building Types Not Available in DEER 2014

The following building types were not available for the 8 DEER measures mentioned above:

- Food Store

- Health/Medical – Clinic

- Industrial

- Lodging – Guest Rooms

- Retail – Multistory Large

- Transportation – Communication – Utilities

- Misc - Commercial

Therefore the energy savings were weighted based on SCE’s multipliers for each building type above (Attachment 2, tab “Weights for Non-DEER Buildings”). For example, the savings for the Industrial building type were calculated as 50% of the savings from the Manufacturing - Light Industrial building type and 50% of the savings from the Manufacturing – Bio/Tech building type.  The gas savings and peak demand reduction were calculated the same way.

## 2.3 Correction of Savings Values Reported in DEER

Based on direction from the Energy Division, the methodology used in the DEER measures referred to in this work paper needed to be verified. The DEER measures use a simplified approach of defining the glass properties in the base case and measure case calculations. The simplified approach defines a window with three window components: U-factor, SHGC and Visible Transmittance. The measure case simply shifts the SHGC to the proposed values defined in each measure while leaving the U-factor and Visible Transmittance values unchanged.

In order to verify the “Simplified Approach” used in DEER is appropriate, building simulations in eQUEST were generated using the Simplified Approach and a Glass Type Code (GTC) Approach. The GTC Approach uses detailed window specifications to define the base case and measure case windows. The detailed window specifications were generated using the WINDOW 7 software and were directly imported into eQUEST.

The calculations were performed for three building types (Office – Large, Residential – Single Family, and Retail – Small) in four climate zones (3, 9, 13 and 16) for both single pane and dual pane windows. It was determined that the Simplified approach was overestimating the energy savings, peak demand reduction and gas savings compared to the more detailed GTC Approach.

Based on these findings it was determined that the savings needed to be corrected for the overestimation of savings due to the Simplified Approach used in the DEER methodology. The following table shows the average difference found in the two approaches:

Table 9 Difference between GTC and Simplified Approaches

| % Difference in Energy Savings of GTC vs. Simplified Approaches (kWh/yr) | % Difference in Peak Demand Reduction of GTC vs. Simplified Approaches (kW) | % Difference in Gas Savings of GTC vs. Simplified Approaches (Therms/yr) |
| --- | --- | --- |
| -25% | -29% | -23% |

The application of these adjustment factors is shown in Attachment 2.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the Window_Tint-Ret load shape.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Assembly | Large_Office | Window_Tint-Ret |
| Education - Primary School | Small_Office | Window_Tint-Ret |
| Education - Secondary School | Small_Office | Window_Tint-Ret |
| Education - Relocatable Classroom | Small_Office | Window_Tint-Ret |
| Education - Community College | Large_Office | Window_Tint-Ret |
| Education – University | Large_Office | Window_Tint-Ret |
| Grocery | Small_Office | Window_Tint-Ret |
| Food Store | Small_Office | Window_Tint-Ret |
| Health/Medical - Hospital | Large_Office | Window_Tint-Ret |
| Health/Medical - Nursing Home | Small_Office | Window_Tint-Ret |
| Health/Medical – Clinic | Small_Office | Window_Tint-Ret |
| Lodging – Hotel | Small_Office | Window_Tint-Ret |
| Lodging - Guest Rooms | Small_Office | Window_Tint-Ret |
| Lodging – Motel | Small_Office | Window_Tint-Ret |
| Manufacturing - Bio/Tech | Large_Office | Window_Tint-Ret |
| Manufacturing - Light Industrial | Large_Office | Window_Tint-Ret |
| Industrial | Large_Office | Window_Tint-Ret |
| Misc – Commercial | Small_Office | Window_Tint-Ret |
| Office – Large | Large_Office | Window_Tint-Ret |
| Office – Small | Small_Office | Window_Tint-Ret |
| Restaurant - Fast-Food | Small_Office | Window_Tint-Ret |
| Restaurant - Sit-Down | Small_Office | Window_Tint-Ret |
| Retail - Multistory Large | Large_Office | Window_Tint-Ret |
| Retail - Single-Story Large | Large_Office | Window_Tint-Ret |
| Retail – Small | Small_Office | Window_Tint-Ret |
| Storage – Conditioned | Large_Office | Window_Tint-Ret |
| Transportation - Communication – Utilities | Large_Office | Window_Tint-Ret |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

For REA measures, the base case cost is $0.00.

## 4.2 Measure Case Cost

The DEER spreadsheet contains cost information for non-residential, single-pane, clear glass windows with window film per square foot of window. The cost is reported per square foot to be consistent with the energy savings, peak demand reduction, and therms savings. Table 11 shows the DEER Cost Case IDs and the associated material and labor costs.

Table 11 Measure Costs

| Cost Case ID | Cost Case Description | Material Cost | Installation Labor Cost – Retrofit | Total Costs/ GMC/IMC | Normalizing Unit |
| --- | --- | --- | --- | --- | --- |
| D08-N-GlazDaylt-WinFilm-37SHGC-NonNorth | Low SHGC Window Film | $1.98 | $3.66 | $5.64 | Sq. Ft. |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

Per the E3, the gross measure cost is the cost to install an energy efficient measure.  This definition implies two different meanings depending on the install type.  In the case of REA, gross measure cost (GMC) means the full cost of the measure to purchase and install.

For REA, GMC is represented by the equation below:

GMC = Measure Equipment Cost + Measure Labor Cost

In the case of REA, the customer is making a decision to replace existing, working equipment before the useful life of the equipment has expired.  Since this is a discretionary choice by the consumer, the cost invoked is the full cost of the equipment and installation of the energy efficient equipment.

Refer to Table 11 for the measure equipment and labor costs.

### 4.3.2 Incremental Measure Cost

Incremental measure cost (IMC) is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  The incremental cost is only used to help determine program incentives.

For REA, there exists no base case measure comparison.  For this reason, IMC is represented by the equation below for REA install types:

IMC = Measure Equipment Cost + Measure Labor Cost

As a result, the IMC is equal to the GMC.  Refer to Table 11 for the measure equipment and labor costs.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [60] | 2005 Building Energy Efficiency Standards for Residential and Nonresidential Buildings-October 2005 |
| [213] | EUL/RUL Values Provided through Excel Spreadsheet |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |