Work Paper SCE13HC051

Revision 2

Southern California Edison

Prescriptive Window Retrofit

# At-a-Glance Summary

| Measure Codes | BE-92103 |
| --- | --- |
| Measure Description | Prescriptive Window U-0.32/SHGC 0.25 |
| Base Case Description | Standard Window U-0.67/SHGC 0.79 |
| Units | Sq. ft. |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | BS-Win: EUL 20 yrs, RUL 6.7 yrs |
| Measure Installation Type | Retrofit or Early Retirement (RET/ER) |
| Net-to-Gross Ratio | Res-Default>2: 0.55 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 6/21/12 | Jason Wang/SCE | Original work paper for 2013 PC |
| 1 | 3/28/14 | Andres Fergadiotti/SCE | Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. Estimated measure savings using ED’s recommended GLASS-TYPE-CODE methodology. Energy Analysis includes SFM and MFM building types. SFM and MFM prototypes were generated using latest MASControl software application. |
| 2 | 1/19/16 | Yun Han/SCE | - New template update for 2016 program year - WP effective from 1/1/2016 thru 12/31/2016 - No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details energy savings on energy efficiency measure consisting of retrofitting standard (2005 DEER) double pane clear glass/window assembly with high performance glass/window assembly meeting minimum 2013 Title-24 prescriptive requirements.

Base Case

The base case is a standard double-pane clear window based on 2005 DEER Update Study – Single Family Home Prototype Characteristics using 1993-2001 Vintage with typical glass/window assembly U-factor of 0.67 and Solar Heat Gain Coefficient (SHGC) of 0.79.

Measure Case

The measure case is a high performance glass/window assembly that meets the minimum 2013 Energy Standard prescriptive compliance approach for newly constructed residential buildings with maximum allowable U-factor of 0.32 and SHGC of 0.25.  The measure can be met by various alternatives - using a double and/or triple pane glass window with Low-E clear and/or tint and/or double electrochromic glass window.  Energy savings on the measure are estimated using a double pane Low-E window/glass assembly.  See Section 2 for additional performance documentation on Base case and Measure case windows.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Prescriptive Window U-0.32/SHGC 0.25 |
| Existing Condition | Standard Window U-0.67/SHGC 0.79 |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | BE-92103 |  | Prescriptive Window U-0.32/SHGC 0.25 replacing Standard Window U-0.67/SHGC 0.79 |

## 1.2 Technical Description

Prescriptive glass/window assembly with U-0.32/SHGC 0.25 replacing a Standard glass/window assembly with U-0.67/SHGC 0.79

## 1.3 Installation Types and Delivery Mechanisms

The install type is Retrofit (RET).

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |

The delivery mechanism is Down-Stream Incentive – Deemed.

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | Yes |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | Yes |
| DEER Version | DEER 2014 |
| Reason for Deviation from DEER | Change in unit from “100 sq ft” to “Square Foot” |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| BS-Win | High Performance Windows | Res | BldgEnv | 20 | 6.7 |

### 1.4.2 Codes and Standards Analysis

Per 2013 Building Energy Efficiency Standards [355], installed fenestration products, under the performance and prescriptive compliance approaches for newly constructed residential buildings, shall have an area weighted average U-factor and SHGC no greater than the applicable value in TABLE 150.1-A  and shall be determined in accordance with Sections 110.6(a)2 of the Standard with maximum U-factor of 0.32 and SHGC of 0.25 applicable to all SCE’s climate zones.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards – Table 150.1-A | July 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

N/A

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

Energy and demand savings on the measure were estimated using comparative (parametric) runs between the base case and measure case window/glass assemblies using the 2014 DEER Residential Single Family (SFM) and Multi-Family (MFM) DOE2.2 prototypes with prototypes generated using the latest MASControl software application.

Methodology for estimating energy savings is based on the GLASS-TYPE-CODE method within the eQuest/DOE2.2 energy modeling software.  The GLASS-TYPE-CODE method selects a glazing from the DOE library which contains pre-determined performance data on window/glass assemblies including center of glass and glass plus window frame heat transfer coefficient “U-factor”, Solar Heat Gain Coefficient (“SHGC”), and Visible Transmittance (“VT”) among others.  Performance parameters on both measure case and base case window/glass assemblies and selected GLASS-TYPE-CODE utilized in the energy analysis are summarized below:

Base Case Fenestration:

U-0.67/SHGC 0.79 [Double Clear]

GLASS-TYPE-CODE: 2000 [Double Clear]|U-0.57/U-0.62 (Center G./Alum w/break) | SHGC 0.76

Measure Case Fenestration:

U-0.32/SHGC 0.25 [Double Low-E (e2=0.04) Tint]

GLASS-TYPE-CODE: 2667 [Double Low-E (e2=0.04)|U-0.29/U-0.33 (Center G./Alum w/break)| SHGC 0.29

Energy savings on the measure were normalized based on total square footage of window in corresponding building (home).  For instance, the total energy and demand impacts as estimated by the building energy model were divided by the total window area as indicated by the building energy model’s LV-D report.  Based on measure characteristics and building weight documentation per climate zone, only the vintage “Built before 1978” was used in the analysis.  This assumption is based on the highest number of building stokes per IOU Weights by Climate Zone & Vintage as documented in DEER14 Database Definition-v3.xlsm per Attachment 2 in this report. Energy simulations utilize latest CTZ2010 weather data.

The measure assumes that the energy impacts on the 2nd baseline are zero (o) since the measure brings fenestration (windows) up to Title-24 prescriptive requirements.

Attachment 1 contains detail information on measure savings for both SFM and MFM building types per climate zones under SCE’s territory.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | Window_Tint-Ret | NON_RES |
| Residential Multi-family | Window_Tint-Ret | NON_RES |

# Section 4. Costs

Measure cost is expected to vary per glass/window assembly type which may include double or triple pane glass with Low-E clear or tint or double electrochromic glass.  Estimated measure cost information including Measure Material and Labor Costs are based on Residential Cost Data (RSMeans Residential Cost Data 2010 – [408]) documentation assuming high performance double-pane Low-E glass window.

Base Case cost was based on DEER05 documentation.

## 4.1 Base Case Cost

Base Case cost from DEER05 is $16.41 per square foot of window and Labor cost as $8.80 per square foot of window.

## 4.2 Measure Case Cost

Measure cost is $32.10 per square foot of window [408] and Labor cost as $8.80 per square foot of window.

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| BE-92103 | RET | $15.69 | $40.90 | $15.69 |

# Attachments

1. 2.

# References

[355]

[408]