Work Paper SCE13HC062

Revision 1

Southern California Edison Company

Upstream Residential HVAC

# At-a-Glance Summary

| Solution and Measure Codes: | Refer to Table 2 |
| --- | --- |
| Measure Description: | Refer to Table 1 |
| Base Case Description: | Refer to Table 1 |
| Units: | Per Ton for ACs and HPs Per Household for Furnaces |
| Energy Savings: | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit): | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life: | Refer to Table 8 |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratio: | 0.85 [A] |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/2014 Ex Ante Database Specification provided by the California Public Utilities Commission (CPUC) Commission Staff (CS); SCE will provide that data set separately. |

# Document Revision History

| Revision # | Revision Date | Author (Affiliation) | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 3/03/2014 | Chris Li (PG&E) | - Adapted PG&E 13-14 work paper for SCE. - Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. |
| 1 | 1/21/2015 | Ryan Cho (SCE) | - New template for 2015. - Work paper updated for the reporting period, effective 1/1/15 – 12/31/15. - Updated to align with DEER 2015. - Updated Split AC costs with the 2010-2012 ExAnte Measure Cost Study [475]. |

# Section 1: General Measure & Baseline Data

## 1.1 Measure Description & Background

Table 1: Base and Measure Cases

| Typical Base Case Description | - Residential SEER-rated split Air Conditioners, Size Range: 18 - 45 kBTU/h, SEER = 14 (EER = 12.2), EIR = 0.239, Fan W/CFM = 0.29, one-speed fan - Residential SEER-rated split Air Conditioners, Size Range: 45 - 65 kBTU/h, SEER = 14 (EER = 11.8), EIR = 0.247, Fan W/CFM = 0.29, one-speed fan - Residential SEER-rated split Heat Pumps, SEER = 14 (EER = 11.9), HSPF = 8.2 (COP = 3.48), EIR = 0.245, Fan W/CFM = 0.29, one-speed fan - Furnace AFUE 80 |
| --- | --- |
| Typical Measure Description | - Residential SEER-rated split Air Conditioners, Size Range: 18 - 65 kBTU/h, SEER = 15 (EER = 12.8), EIR = 0.232, Fan W/CFM = 0.25, one-speed fan - Residential SEER-rated split Air Conditioners, Size Range: 18 - 65 kBTU/h, SEER = 16 (EER = 12.5), EIR = 0.238, Fan W/CFM = 0.27, two-speed fan - Residential SEER-rated split Air Conditioners, Size Range: 18 - 65 kBTU/h, SEER = 17 (EER = 13.3), EIR = 0.223, Fan W/CFM = 0.27, two-speed fan - Residential SEER-rated split Air Conditioners, Size Range: 18 - 65 kBTU/h, SEER = 18 (EER = 14), EIR = 0.209, Fan W/CFM = 0.27, two-speed fan - Residential SEER-rated split Air Conditioners, Size Range: 18 - 45 kBTU/h, SEER = 17.4, one-speed fan, evaporative cooled condenser - Residential SEER-rated split Heat Pumps, SEER = 15 (EER = 12.8), HSPF = 8.7 (COP = 3.68), EIR = 0.232, Fan W/CFM = 0.25, one-speed fan - Residential SEER-rated split Heat Pumps, SEER = 16 (EER = 12.5), HSPF = 9 (COP = 3.57), EIR = 0.238, Fan W/CFM = 0.27, two-speed fan - Residential SEER-rated split Heat Pumps, SEER = 17 (EER = 13.3), HSPF = 9.4 (COP = 3.74), EIR = 0.223, Fan W/CFM = 0.27, two-speed fan - Residential SEER-rated split Heat Pumps, SEER = 18 (EER = 14), HSPF = 9.7 (COP = 3.86), EIR = 0.209, Fan W/CFM = 0.27, two-speed fan - Efficient Residential Gas Furnace - AFUE 96 - Efficient Residential Gas Furnace - AFUE 97 |

Basecase and measure descriptions shown above are directly from DEER 2015, READi v2.1.0

Table 2: Measures and Codes

| Solution Code | Measure Code | Measure Name |
| --- | --- | --- |
| AC-67409 | S8807 | <45kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner |
| AC-57261 | S8808 | <45kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner |
| AC-87703 | S8809 | <45kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner |
| AC-96378 | S8811 | <45kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner |
| AC-96912 | S8810 | <45kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner |
| AC-76820 | SSSS1 | 45-65kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner |
| AC-92308 | SSSS2 | 45-65kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner |
| AC-95844 | SSSS3 | 45-65kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner |
| AC-98514 | SSSS4 | 45-65kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner |
| AC-90503 | SSSS5 | 45-65kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner |
| AC-95310 | S8803 | 15 SEER Heat Pump (HSPF = 8.7) |
| AC-50116 | S8804 | 16 SEER Heat Pump (HSPF = 9.0) |
| AC-72417 | S8805 | 17 SEER Heat Pump (HSPF = 9.4) |
| AC-78956 | S8806 | 18 SEER Heat Pump (HSPF = 9.7) |
| N/A | S8813 | Efficient Residential Gas Furnace – AFUE 97 |
| N/A | S8812 | Efficient Residential Gas Furnace – AFUE 96 |

Catalog Description

The measures presented on this workpaper are not in the rebate catalog, they are promoted under our upstream residential HVAC program.

Program Restrictions and Guidelines

The rebate is upstream provided to the distributor at the time of sale upon receipt of sales data.  This is not a direct install program.

Terms and Conditions:

Only those Air Conditioning and Heat Pump systems covered by AHRI 210/240 2008 are included.  Efficiency values must be certified by Air-conditioning Heating and Refrigeration Institute (AHRI) as verified by the AHRI certification number.  For split systems the certification will specify an evaporator coil model and may specify additional components.  In some cases a furnace is specified in the certificate.  In this case since the furnace is specified as part of the AC system it will not be eligible for a separate incentive.

Natural Gas furnaces shall be those tested using the DOE procedures Code of Federal Regulations, 10 CFF Part 430, Subpart B, Appendix N (Standard) which incorporates ANSI/ASHRAE Standard 103.   The efficiency is verified by the AHRI certification number.  Propane and oil fired furnaces are not eligible.

Eligibility Requirements:

The purpose of the Upstream Program is to encourage HVAC Distributors to stock premium efficiency equipment.  This premium efficient equipment is significantly more efficient than the standard efficient equipment, which is mandated by California Title 20 appliance standards and California Title 24 standards. Table C-3 the incentives are paid to the Distributor who may do what they deem best with the incentive.  The program does not control or restrict the distributor’s use of the funding.

Implementation Requirements:

The region will be defined by postal zip codes and incentives will only be paid on systems:

- That are being installed in those zip codes

- For which the applicant has documented the purchase as a replacement on burnout.

- That meets the WHPA working group’s efficiency tiers.

Table 3: WHPA efficiency tiers

| Tier 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Air Conditioners |  | Heat Pumps |  | Gas Furnaces |
|  | Split System | Packaged | Split Air Source | Packaged | AFUE |
| Efficiency | 17 SEER, 13 EER | 15 SEER, 12 EER | 17 SEER, 13 EER, 9 HSPF | 15 SEER, 12 EER, 8 HSPF | 96% AFUE gas and propane furnaces, gas and propane boilers, oil furnaces and hot water boilers |
| Tier 3 |  |  |  |  |  |
|  | Air Conditioners |  | Heat Pumps |  | Gas Furnaces |
|  | Split System | Packaged | Split Air Source | Packaged | AFUE |
| Efficiency | 18 SEER, 13 EER | 16 SEER, 12 EER | 18 SEER, 13 EER, 9 HSPF | 16 SEER, 12 EER, 9 HSPF | 97% AFUE gas and propane furnaces, gas and propane boilers, oil furnaces and hot water boilers |

Note: The tiers and equipment are based on WHPA tiers which represent what is produced and sold in the market which can be different than DEER or Consortium for Energy Efficiency (CEE) values.

## 1.2 Technical Description

Packaged HVAC

A single-package Air Conditioner or Heat Pump system consists of a single cabinet housing containing a condensing unit, a compressor, and an indoor fan/coil. The heating section (if one is included) may be either gas-fired or incorporated into the refrigeration circuit as a heat pump.  Single-package units are typically installed on the rooftop or on the ground outside of the house.  A benefit of package units is that there is no need for field-installed refrigerant piping, minimizing labor costs and the possibility of contaminating the system with dirt, metal, oxides or non-condensing gases.

Split-System HVAC

A split system consists of two major parts in separate housings: a remote compressor/condenser unit, and an indoor coil. The two system components are connected by a pair of refrigerant lines. A forced air furnace is usually installed with the indoor evaporator coil to permit deliver of either heating or cooling.    In the case of a heat pump, the air handler is used with the indoor coil in condensing mode for heating and in evaporator mode for cooling.  Typical locations for the indoor section are in the attic, a closet with either inside or outside access, or in the garage.   The outdoor compressor/condenser unit is usually located on the ground outside of the house.

The evaporatively cooled condenser is a split system AC that is covered by AHRI 210/240 but does not receive an SEER rating.  Its EER rating is used for the program and is verified using the AHRI certification number.

Central Natural Gas Furnaces

Natural gas burning, forced-air furnaces provide heat to the conditioned space by passing indoor air through a heat exchanger.  A blower fan pulls cool air from inside the dwelling through the return air ducts and forces it through the furnace heat exchanger heating it by up to 50 degrees Fahrenheit.  The combustion gases from the furnace are vented outside through flue connected to the combustion unit near the heat exchanger. The AFUE 90 and higher furnaces use two heat exchangers which lower the temperature of the combustion gases to where the moisture condenses and drained in a code approved manner.  These condensing furnaces use plastic flue piping making them easy to identify.

## 1.3 Application Types and Delivery Mechanisms

See Appendices A and B for definitions of application types and delivery mechanisms.

The measures outlined in this work paper are for Replace on Burnout (ROB) applications.

The delivery and incentive mechanisms for measures contained in this work paper is Upstream Programs / Up-Stream incentive.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Energy and gas savings for these measures are downloaded from the DEER2015 READi tool (v2.1.0) directly except the furnaces are modified to reflect the units of “per household”.

Table 4: DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Referenced versions of DEER and READi | DEER 2015, READi v2.1.0 |
| Summary of deviation from DEER | The furnaces are modified to reflect the units of “per household”. |
| DEER measures scaled? | No |
| DEER eQUEST prototypes used? | No |
| DEER operating hours used? | No |

Net-to-Gross Ratio

DEER2011 did not provide an M&V NTG for upstream residential HVAC measures, the closest NTG that would match this upstream residential HVAC program is the NTG of 0.85 for nonresidential upstream HVAC program.  On February 28, 2014, Katie Wu from the Energy Division approved the use of 0.85 for the upstream residential program [A].

The NTG value is based on using an ED approved NTG of 0.85 from DEER2011. The relevant NTG values for the measures in this work paper are in the table below.

Table 5: Net-to-Gross Ratio

| NTGR ID | Description | Sector | BldgType | ProgDelivID | NTG |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mHVAC-DX-up | All package and split system AC & HP replacements | Res | Any | PreRebUp | 0.85 |

Installation Rate

The IR values were obtained using the DEER READi tool. The relevant IR values for the measures in this work paper are in the table below.

Table 6: Installation Rate

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Technology Fields

The Technology Fields were obtained from the Ex Ante Database Specification. The relevant Use Category, Use Sub-category, Technology Group, and Technology Type values for the measures in this work paper are in the table below.

Table 7: Technology Fields

| Classification | Air Conditioner | Heat Pump | Furnace |
| --- | --- | --- | --- |
| Measure Case UseCategory | HVAC | HVAC | HVAC |
| Measure Case UseSubCats | Space Cooling | Space Heating and Cooling | Space Heating |
| Measure Case TechGroups | dX AC Equipment | dx HP Equipment | Space Heating Equipment |
| Measure Case TechTypes | SEER Rated Split System AC | SEER Rated Split System HP | Gas Furnace |
| Base Case TechGroups | dX AC Equipment | dx HP Equipment | Space Heating Equipment |
| Base Case TechTypes | SEER Rated Split System AC | SEER Rated Split System HP | Gas Furnace |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

Table 8: EUL and RUL

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| HV-ResHP | High Efficiency Heat Pump | RES | HVAC | 15 | 5 |
| HV-ResAC | High Efficiency Air Conditioner | RES | HVAC | 15 | 5 |
| HV-ResEvapAC | Evap Cooled Split Systems Air Conditioner | RES | HVAC | 15 | 5 |
| HV-EffFurn | High Efficiency Gas Furnace | RES | HVAC | 20 | 6.7 |
| HV-ResHP | High Efficiency Heat Pump | RES | HVAC | 15 | 5 |

### 1.4.2 Codes and Standards Analysis

Title 20:

The measures in this workpaper fall under Title 20 of the California Energy Regulations. Under the Title 20 regulation (CEC-400-2014-009-CMF), the following is required by Section 1605.1. (c) (1), “Central Air Conditioners.  The EER, SEER, COP, and HSPF, as applicable, of all central air conditioners shall be not less than the applicable values shown in Tables C-2, C-3, C-4, C-5, and C-6.” The efficiency requirements for air-cooled split-systems and packaged air conditioners less than 65,000 Btu/h (5.4 tons) are identified in Table C-2 (for single-phase power) as being minimum 14.0 SEER. Also, residential and commercial split air-conditioners have additional EER code requirements above and below 45 kBtu/h. [422]. For furnaces, under section 1605.1(B) on Table E-6, the minimum standard for gas furnaces with less than 225,000 Btu/h is 80% AFUE.

Table 9: T20 Codes

| Title 20 Std. Description | Base or Measure Case | Value | Units | Code Source or Reference |
| --- | --- | --- | --- | --- |
| Single package air-cooled air conditioners (<65,000 Btuh) | Base | 14.0 | SEER | Table C-2, Title 20, 2014 |
| Single package air-cooled heat pumps (<65,000 Btuh) | Base | 14.0 | SEER | Table C-2, Title 20, 2014 |
| Evaporatively-cooled air conditioners (<65,000 Btuh) | Base | 12.1 | EER | Table C-4, Title 20, 2014 |
| Natural Gas Furnaces | Base | 80% | AFUE | Table E-6, Title 20, 2014 |

Title 24:

The Title 24 Building Energy Efficiency Standards do not specify requirements for air conditioning units less than 65,000 Btu/h (5.4 tons), other than stating that such equipment must comply with Title 20 requirements [422].

Federal Standards:

A new federal rule sets energy efficiency standards for residential HVAC equipment, including split-system air conditioners under 65,000 Btu/h, at 14.0 SEER in the southwest region that includes California. However, this new rule does not take effect until January 1, 2015 [368]. Federal Regional Standards for furnaces, are incorporated into Federal DOE (10 CFR Part 430) Energy Regulations. Under this regulation, regional standards are scheduled for to change the AFUE for non-weatherized furnaces on May 1, 2013 and on January 1, 2015 for weatherized furnaces and central air conditioners and heat pumps.  Most residential type furnaces are installed inside the house and are classified  as non-weatherized for which the minimum AFUE is 80% for units less than 225,000 Btu/h in input capacity sold in the warm regions of the country.

Table 10: Federal Standards

| Federal Std. Description | Base or Measure Case | Value | Units | Code Source or Reference |
| --- | --- | --- | --- | --- |
| Central Air Conditioners | Base | 14.0 SEER, 12.2 or 11.7 EER | Per System | CFR Part 430 |
| Central furnaces | Base | 80% AFUE | Per System | CFR Part 430 |

Table 11: Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Section 110.1 requires air conditioners under 65 kBtu/h to comply with Title 20 | 7/1/2014 |
| Title 20 (2014) | Section 1605.1 requires air-cooled split-systems and packaged air conditioners under 65 kBtu/h to have a minimum SEER of 14 and additional EER code requirements above and below 45 kBtu/h for split air conditioners only. | 1/1/2015 for AC and HP |
|  | Section 1605.1 Tables C-4, and E-6 | 5/1/2013 for Gas furnaces |
| Federal Codes (DOE / EPA) | Federal DOE Energy Regulations, CFR part 430 A recent federal rule requires split-system and packaged air conditioners to have a minimum SEER of 14. | 1/1/2015 for AC and HP |
|  | Federal standards require a minimum furnace AFUE of 80%. | 5/1/13 for Gas furnaces |

### 1.4.3 Non-DEER Study Review

There were no M&V or other studies which were referenced for these measures.

# Section 2: Calculation Methodology

Energy and gas savings for these measures are downloaded from the DEER2015 READi tool (v2.1.0) directly except the furnaces are modified to reflect the units of “per household”. See the savings calculation spreadsheet for Gas Furnaces Calculation [B].

The following table contains data files for measures taken directly from or created with the DEER READi Tool. These results have not been modified and are being included in the workpaper for reference.

Table 12: READi Tool Outputs

| Solution Code | Measure Code | Measure Name | READI Results |
| --- | --- | --- | --- |
| AC-67409 | S8807 | <45kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner |  |
| AC-57261 | S8808 | <45kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner |  |
| AC-87703 | S8809 | <45kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner |  |
| AC-96378 | S8811 | <45kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner |  |
| AC-96912 | S8810 | <45kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner |  |
| AC-76820 | SSSS1 | 45-65kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner |  |
| AC-92308 | SSSS2 | 45-65kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner |  |
| AC-95844 | SSSS3 | 45-65kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner |  |
| AC-98514 | SSSS4 | 45-65kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner |  |
| AC-90503 | SSSS5 | 45-65kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner |  |
| AC-95310 | S8803 | 15 SEER Heat Pump (HSPF = 8.7) |  |
| AC-50116 | S8804 | 16 SEER Heat Pump (HSPF = 9.0) |  |
| AC-72417 | S8805 | 17 SEER Heat Pump (HSPF = 9.4) |  |
| AC-78956 | S8806 | 18 SEER Heat Pump (HSPF = 9.7) |  |
| N/A | S8813 | Efficient Residential Gas Furnace – AFUE 97 |  |
| N/A | S8812 | Efficient Residential Gas Furnace – AFUE 96 |  |

# Section 3: Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Table 13: Building Types and Load Shapes

| Building Type | Load Shape | E3 Alt. Building Type |
| --- | --- | --- |
| Residential Single Family (SCE) | DEER:HVAC_Split-Package_AC DEER:HVAC_Split-Package_HP | RES |
| Residential Multi-Family (SCE) | DEER:HVAC_Split-Package_AC DEER:HVAC_Split-Package_HP | RES |
| Residential Mobile Home – Double-Wide (SCE) | DEER:HVAC_Split-Package_AC DEER:HVAC_Split-Package_HP | RES |
| Residential (PG&E) | DEER:HVAC_Split-Package_AC | RES |

# Section 4: Base Case & Measure Costs

## 4.1 Base Case Cost

The 2010-2012 Ex-ante Measure Cost Study [475] was consulted and the equipment base case costs are estimated based on split-system DX (residential and commercial) equipment price estimates from this study. All costs are noted as $ per measure unit, for air conditioners/heat pumps this unit is per ton and for furnaces, the unit is per household.  Note that WO17 did not contain data for all split system heat pump equipment, so the other DEER 2008 data [316] was used on the heat pumps only for consistency.

For split-system DX, predicted prices and incremental costs are systematically lower compared to current and previous DEER estimates by 10-30 percent according this report.

## 4.2 Measure Case Cost

The 2010-2012 Ex-ante Measure Cost Study [475] was consulted and the equipment measure case costs are estimated based on split-system DX (residential and commercial) equipment price estimates from this study. All costs are noted as $ per measure unit, for air conditioners/heat pumps this unit is per ton and for furnaces, the unit is per household.  Note that WO17 did not contain data for all split system heat pump equipment, so the other DEER 2008 data [316] was used on the heat pumps only for consistency.

For split-system DX, predicted prices and incremental costs are systematically lower compared to current and previous DEER estimates by 10-30 percent according this report.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost (GMC)

For ROB, the equipment being replaced is assumed to have failed in place or is past its useful life. The customer is faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.  Therefore, gross measure cost (GMC) means the cost premium required to install the energy efficient measure over a less efficient piece of equipment. GMC is represented by the equation below:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Code Base Case Equipment Cost + Code Base Case Labor Cost)

Table 14 Gross Measure Costs for Commercial Units

| Measure Name | Measure Case Material Cost/ton | Measure Case Labor Cost/ton | Base Case Material Cost/ton | Base Case Labor Cost/ton | Gross Measure Cost |
| --- | --- | --- | --- | --- | --- |
| <45kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner | $607.10 | $220.92 | $514.98 | $220.92 | $92.13 |
| <45kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner | $699.23 | $220.92 | $514.98 | $220.92 | $184.25 |
| <45kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner | $791.36 | $220.92 | $514.98 | $220.92 | $276.38 |
| <45kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner | $883.48 | $220.92 | $514.98 | $220.92 | $368.51 |
| <45kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner | $1159.86 | $220.92 | $514.98 | $448.01 | $644.89 |
| 45-65kBtu/hr 15 SEER (12.72 EER) Split-System Air Conditioner | $607.10 | $220.92 | $514.98 | $220.92 | $92.13 |
| 45-65kBtu/hr 16 SEER (11.61 EER) Split-System Air Conditioner | $699.23 | $220.92 | $514.98 | $220.92 | $184.25 |
| 45-65kBtu/hr 17 SEER (12.28 EER) Split-System Air Conditioner | $791.36 | $220.92 | $514.98 | $220.92 | $276.38 |
| 45-65kBtu/hr 18 SEER (13.37 EER) Split-System Air Conditioner | $883.48 | $220.92 | $514.98 | $220.92 | $368.51 |
| 45-65kBtu/hr 17.4 SEER (15.1 EER) Evap-Cooled Split-System Air Conditioner | $1159.86 | $220.92 | $514.98 | $448.01 | $644.89 |
| 15 SEER Heat Pump (HSPF = 8.7) | $1070.00 | $448.01 | $933 | $448.01 | $137.00 |
| 16 SEER Heat Pump (HSPF = 9.0) | $1207.00 | $448.01 | $933 | $448.01 | $274.00 |
| 17 SEER Heat Pump (HSPF = 9.4) | $1344.00 | $448.01 | $933 | $448.01 | $411.00 |
| 18 SEER Heat Pump (HSPF = 9.7) | $1481.00 | $448.01 | $933 | $448.01 | $548.00 |
| Efficient Residential Gas Furnace – AFUE 97 | * | * | * | * | * |
| Efficient Residential Gas Furnace – AFUE 96 | * | * | * | * | * |

* For Gas Furnaces, please see Gas Furnaces Calculations [B].

### 4.3.2 Incremental Measure Cost (IMC)

For ROB, the incremental measure cost (IMC) is the same as the gross measure cost.  See Section 4.3.1.

# Attachments

- SCE Savings:

- PG&E Savings:

# References

[316]

[368]

[422]

[475]

[A] Email from Katie Wu on the approval of using the 0.85 NTG for upstream residential HVAC.

[B] Attachment 3 – Gas Furnaces Calculations

[C] Attachment 4 – PG&E Calculation Template

# Appendix A: Application Types

This table compares the application types in SCE’s systems with those in DEER.

| SCE Application (Program) Type | DEER Application Type | Savings |  | Cost |  | Life |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL | 1st BL | 2nd BL |
| New Construction (NEW) | New Construction (Nc) | Above Code or Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob), Normal Replacement (NR) | Above Code or Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Customer Existing | Above Code or Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Customer Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Customer Existing | N/A | Full Cost | N/A | EUL | 0 |

# Appendix B: Delivery Mechanisms

A delivery mechanism is a delivery method paired with an incentive method. SCE’s delivery methods include:

- Appliance Turn-in and Recycling

- Audit/Information

- Commissioning

- Financial Support

- Innovative Design

- Midstream Programs

- Partnership

- Upstream Programs

The following table describes the incentive methods.

| Incentive Method | Description |
| --- | --- |
| Direct Install | The utility program performs an assessment of the customer’s facility, provides recommendations, and implements energy efficiency measures for free. |
| Down-Stream Incentive - Deemed | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The utility program provides customers with energy efficient equipment for free. |
| Mid-Stream Incentive | The utility program offers buydowns and incentives to third parties (typically retailers, distributors, and contractors), who then stock, promote, lower prices on, and/or sell energy efficient equipment. Contractors install energy efficiency equipment, sometimes using specified quality procedures, at the customer’s property. |
| On-bill Finance - loan | Customers can finance energy efficiency projects at 0% interest and repay the loan through their monthly utility bill. |
| Testing Services / Other | The utility program performs free testing services or assessments of the customer’s facility and provides information and recommendations for potential energy efficiency measures. |
| Up-Stream Buy Down, Up-Stream Incentive | The utility program offers buydowns and incentives to vendors (typically manufacturers and distributors), who then manufacture, stock, promote, lower prices on, and/or sell energy efficient equipment. There is some overlap between the mid-stream and up-stream approaches. |