Work Paper SCE13LG017

Revision 2

Southern California Edison

Interior Integral, Non-Dimmable (Screw-in) CFLs

# At-a-Glance Summary

| Measure Codes | LT-10566, LT-46312, LT-48984, LT-71032, LT-13421, LT-56121, LT-89941, LT-38194, LT-12918, LT-44329, LT-28401, LT-44133, LT-52345, LT-78965, LT-38498, LT-89584, LT-90135, LT-69840, LT-79693, LT-86739, LT-77458, LT-68581, LT-81933, LT-91878, LT-75322, LT-74990, LT-62706, LT-99557, LT-75726, LT-97219, LT-59300, LT-73056, LT-55230, LT-71988, LT-62375, LT-87274, LT-75856, LT-73388 |
| --- | --- |
| Measure Description | Interior integral non-dimmable (screw-in) compact fluorescent lamps (CFL) |
| Base Case Description | Incandescent lamp |
| Units | Per lamp |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | Commercial: 10,000 / hours of use Residential: 3.5 years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Refer to Net to Gross Ratio Table |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 4/12/12 | Jenny Roehm (PECI) | Original work paper for 2013 Program Cycle |
| 0 | 4/12/12 | Brian V. O’Keefe (SCE) | Used DEER 2011 where available for measure savings source. |
| 1 | 4/15/14 | Linda Wan, P.E. (TRC Energy Services) | - Updated work paper template. - Updated solution codes. - Added technical description. - Updated code analysis for Title 24 2013. - Added installation rate. - Added Non-DEER Study Review. - Updated energy savings and DEER measures. |
| 1 | 4/15/14 | Jason Wang (SCE) | - Work paper updated for reporting period, effective 7/1/14 – 12/31/14. |
| 2 | 9/18/14 | Ryan Cho (SCE) | - Updated for DEER2016 HOU, CDF, NTG, EUL - Updated WP name - Removed measures that are not in use - Removed Misc – Commerical and added Office - Small - Updated for Costs - Updated WP template and Calculation template - Removed the Midstream Program (Tom Pasker) - 100% of measure are directly from DEER - Reporting period, effective 1/1/16 - Split out residential and commercial measures; added LT-86739, LT-77458, LT-68581, LT-81933, LT-91878, LT-75322, LT-74990, LT-62706, LT-99557, LT-75726, LT-97219, LT-59300, LT-73056, LT-55230, LT-71988, LT-62375, LT-87274, LT-75856, LT-73388 |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Interior integral non-dimmable (screw-in) compact fluorescent lamp (The lamp type can be Spiral, Spiral with Reflector, Integral A, and Globe) |
| Existing Condition | N/A |
| Code/Standard | Incandescent lamp |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | LT-10566 | N/A | 12 Watt Integral A CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-46312 | N/A | 14 Watt Integral A CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-48984 | N/A | 18 Watt Integral A CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-71032 | N/A | 19 Watt Integral A CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-13421 | N/A | 20 Watt Integral A CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-56121 | N/A | 14 Watt Integral Globe CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-89941 | N/A | 19 Watt Integral Globe CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-38194 | N/A | 9 Watt Integral Globe CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-12918 | N/A | 15 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-44329 | N/A | 23 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-28401 | N/A | 16 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-44133 | N/A | 18 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-52345 | N/A | 25 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-78965 | N/A | 26 Watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-38498 | N/A | 15 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-89584 | N/A | 32 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-90135 | N/A | 40 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-69840 | N/A | 42 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-79693 | N/A | 55 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-86739 | N/A | 12 Watt Integral A CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-77458 | N/A | 14 Watt Integral A CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-68581 | N/A | 18 Watt Integral A CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-81933 | N/A | 19 Watt Integral A CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-91878 | N/A | 20 Watt Integral A CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-75322 | N/A | 14 Watt Integral Globe CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-74990 | N/A | 19 Watt Integral Globe CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-62706 | N/A | 9 Watt Integral Globe CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-99557 | N/A | 15 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-75726 | N/A | 23 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-97219 | N/A | 16 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-59300 | N/A | 18 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-73056 | N/A | 25 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-55230 | N/A | 26 Watt Integral Spiral with Reflector CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-71988 | N/A | 15 Watt Integral Spiral CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-62375 | N/A | 32 Watt Integral Spiral CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-87274 | N/A | 40 Watt Integral Spiral CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-75856 | N/A | 42 Watt Integral Spiral CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-73388 | N/A | 55 Watt Integral Spiral CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |

Implementation Requirements

This measure is eligible in the following building types:

- Residential Single Family

- Office - Small

## 1.2 Technical Description

Integral CFLs contain the lamp, ballast, and screw-in base together as a single unit.  As a result, when the CFL burns out, the entire unit is replaced [406].  The specialty bulbs involved in this measure are the following shapes and sizes [407]:

- Integral A – Has the look of an incandescent bulb but has the efficiency of a spiral bulb;

- Integral globe – Most commonly used in bathroom vanity bars and ceiling pendants;

- Integral spiral – Most popular type of CFL;

- Integral spiral with reflector – Most commonly used in directional lighting, specifically in recessed ceiling fixtures; and

## 1.3 Installation Types and Delivery Mechanisms

The delivery method is:

Up-Stream Incentive / Up-Stream Buy Down

The upstream strategy is used to stimulate sales of energy efficient lighting. The lighting measures are promoted and tracked separately from other point of service (POS) rebate measures.  Lighting incentives and promotions influence customers to purchase energy-saving lighting products at retail outlets and install them in homes and small businesses.

The install type is:

Replace on Burnout (ROB)

ROB measures replace existing equipment with more energy efficient equipment when the existing equipment has failed or passed its useful life.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | N/A |
| DEER Measure IDs Used | Yes |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mCFL | CFL-screw in, All. | Com | Any | PreRebUp | 0.54 |
| Res-sAll-mCFL-up | CFL-screw in,All. | Com | Any | PreRebUp | 0.9 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFL-Com | CFL Lamps - Indoor- Commercial - 10,000 Rated Hours | Com | Lighting | 10,000 / HOU | N/A |
| ILtg-CFL-Res | “2015 Uncertain Measures – WP Updates” Guidance | Res | Lighting | 3.5 | N/A |

### 1.4.2 Codes and Standards Analysis

This work paper uses ED-specified wattages and wattage ratios to determine code case wattages; these satisfy or exceed any energy consumption requirements mentioned below.

Title 24 (2013) Building Energy Efficiency Standards [355] includes standards for both non-residential and residential buildings.  Section 150.0 (k) 7 and Section 150.0 (k) 8 pertain to residential luminaries.  Both Sections call out that installed luminaries need to be high efficacy lighting as defined by Table 150.0-A or Table 150.0-B for lighting systems not listed in Table 150.0-A.  According to Table 150.0-A, “GU-24 sockets for compact fluorescent lamps” are classified as high efficacy.  Thus, the measures outlined in this work paper satisfy the requirement as a high efficacy lighting technology.

Title 20 (2015) Appliance Efficiency Regulations [493] includes standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California.  Table K-5 lists the minimum efficacy (lumens/watt) requirements for medium base compact fluorescent lamps. It is assumed that the measures listed in DEER 2016 comply with the 2015 Title 20, Table K-5.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards Section 150.0 (k) | July 1, 2014 |
| Title 20 (2015) | Table K-5 Standards for Medium Base Compact Fluorescent Lamps | January 8, 2007 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

The DEER READI tool and the WO17 study [475] were consulted for the values used in this workpaper, and there are no applicable non-DEER studies associated with this workpaper.

## 1.6 Data Quality and Future Data Needs

The savings and other values such as NTG, EUL, and GSIA values for this workpaper are downloaded from the DEER READI tool, and the Work Order 17 – the Ex Ante Measure Cost Study is utilized for the costs.

# Section 2. Calculation Methodology

Measures in this work paper are directly from the DEER READI tool. The following table indicates which measures are taken directly from or created with the DEER READI tool.

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| LT-10566 | 12 Watt Integral A CFL | C-In-CFLscw-A(12w)-dWP30 |
| LT-46312 | 14 Watt Integral A CFL | C-In-CFLscw-A(14w)-dWP35 |
| LT-48984 | 18 Watt Integral A CFL | C-In-CFLscw-A(18w)-dWP46 |
| LT-71032 | 19 Watt Integral A CFL | C-In-CFLscw-A(19w)-dWP48 |
| LT-13421 | 20 Watt Integral A CFL | C-In-CFLscw-A(20w)-dWP51 |
| LT-56121 | 14 Watt Integral Globe CFL | C-In-CFLscw-Glb(14w)-dWP35 |
| LT-89941 | 19 Watt Integral Globe CFL | C-In-CFLscw-Glb(19w)-dWP48 |
| LT-38194 | 9 Watt Integral Globe CFL | C-In-CFLscw-Glb(9w)-dWP23 |
| LT-12918 | 15 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl-1(15w)-dWP38 |
| LT-44329 | 23 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl-1(23w)-dWP59 |
| LT-28401 | 16 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl(16w)-dWP41 |
| LT-44133 | 18 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl(18w)-dWP46 |
| LT-52345 | 25 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl(25w)-dWP64 |
| LT-78965 | 26 Watt Integral Spiral with Reflector CFL | C-In-CFLscw-Refl(26w)-dWP66 |
| LT-38498 | 15 Watt Integral Spiral CFL | C-In-CFLscw(15w)-dWP38 |
| LT-89584 | 32 Watt Integral Spiral CFL | C-In-CFLscw(32w)-dWP82 |
| LT-90135 | 40 Watt Integral Spiral CFL | C-In-CFLscw(40w)-dWP102 |
| LT-69840 | 42 Watt Integral Spiral CFL | C-In-CFLscw(42w)-dWP107 |
| LT-79693 | 55 Watt Integral Spiral CFL | C-In-CFLscw(55w)-dWP141 |
| LT-86739 | 12 Watt Integral A CFL | R-In-CFLscw-A(12)-dWP18 |
| LT-77458 | 14 Watt Integral A CFL | R-In-CFLscw-A(14)-dWP21 |
| LT-68581 | 18 Watt Integral A CFL | R-In-CFLscw-A(18)-dWP27 |
| LT-81933 | 19 Watt Integral A CFL | R-In-CFLscw-A(19)-dWP28 |
| LT-91878 | 20 Watt Integral A CFL | R-In-CFLscw-A(20)-dWP30 |
| LT-75322 | 14 Watt Integral Globe CFL | R-In-CFLscw-Glb(14)-dWP21 |
| LT-74990 | 19 Watt Integral Globe CFL | R-In-CFLscw-Glb(19)-dWP28 |
| LT-62706 | 9 Watt Integral Globe CFL | R-In-CFLscw-Glb(9)-dWP13 |
| LT-99557 | 15 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl-1(15)-dWP28 |
| LT-75726 | 23 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl-1(23)-dWP43 |
| LT-97219 | 16 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl(16)-dWP30 |
| LT-59300 | 18 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl(18)-dWP33 |
| LT-73056 | 25 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl(25)-dWP47 |
| LT-55230 | 26 Watt Integral Spiral with Reflector CFL | R-In-CFLscw-Refl(26)-dWP48 |
| LT-71988 | 15 Watt Integral Spiral CFL | R-In-CFLscw(15)-dWP22 |
| LT-62375 | 32 Watt Integral Spiral CFL | R-In-CFLscw(32)-dWP47 |
| LT-87274 | 40 Watt Integral Spiral CFL | R-In-CFLscw(40)-dWP59 |
| LT-75856 | 42 Watt Integral Spiral CFL | R-In-CFLscw(42)-dWP62 |
| LT-73388 | 55 Watt Integral Spiral CFL | R-In-CFLscw(55)-dWP81 |

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | DEER:Indoor_CFL_Ltg | RES |
| Office – Small | DEER:Indoor_CFL_Ltg | NON_RES |

# Section 4. Costs

## 4.1 Base Case Cost

All of the cost values are from the latest ex ante database through the READI v2.3.0 except for SCE13LG017_02_B001 and SCE13LG017_02_B002. These new SCE cost IDs are created and the DEER2016-Costs_Ltg-CFL-Incand_8-31-2015-DRAFT.xls [B] is utilized for developing material cost with the assumption of 150 watt Incandescent as a basecase measure. Due to the range limit in the workbook, 150 watt Incandescent would produce the best available material cost for the basecase measure. For a complete breakdown of base case costs, please refer to Attachment 1 [A].

## 4.2 Measure Case Cost

All of the cost values are from the latest ex ante database through the READI v2.3.0. For a complete breakdown of measure case costs, please refer to Attachment 1 [A].

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

For a complete breakdown of full and incremental measure costs, please refer to Attachment 1 [A].

# Attachments

# References

| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| --- | --- |
| [406] | Efficient Lighting Using Compact Fluorescent Lamps and Fixtures |
| [407] | CFL Sizes and Shapes |
| [475] | 2010-2012 WO017 Ex Ante Measure Cost Study Final Report |
| [493] | 2015 Appliance Efficiency Regulations (Title 20) |
| [A] | Attachment 1 |
| [B] | Attachment 2 |