Work Paper SCE13LG017

Revision 1

Southern California Edison Company

Upstream Interior Integral, Non-Dimmable (Screw-in) CFLs and Circlines

# At-a-Glance Summary

| Applicable Measure Codes: | LT-42938, LT-59849, LT-69598, LT-71834, LT-82934, LT-91044, LT-32354, LT-43526, LT-56472, LT-67593, LT-30988, LT-24394, LT-96705, LT-10566, LT-24746, LT-32875, LT-47948, LT-54837, LT-59873, LT-90870, LT-10112, LT-23829, LT-60194, LT-49877, LT-84743, LT-46312, LT-56121, LT-67591, LT-94387, LT-76584, LT-24326, LT-58432, LT-69431, LT-38498, LT-39874, LT-12918, LT-69807, LT-73859, LT-89505, LT-95069, LT-15926, LT-28401, LT-50988, LT-48984, LT-58901, LT-68142, LT-68743, LT-44133, LT-84330, LT-71032, LT-89941, LT-91324, LT-99875, LT-79878, LT-29426, LT-13421, LT-28134, LT-38321, LT-34543, LT-18546, LT-25143, LT-89732, LT-78432, LT-69921, LT-50789, LT-40794, LT-37456, LT-30916, LT-59843, LT-49382, LT-57843, LT-69442, LT-60988, LT-44329, LT-89877, LT-78410, LT-89321, LT-94312, LT-97423, LT-77098, LT-28575, LT-10954, LT-83213, LT-71984, LT-61823, LT-52345, LT-39670, LT-49287, LT-54382, LT-21645, LT-69566, LT-78965, LT-49762, LT-23832, LT-34328, LT-48345, LT-54954, LT-69599, LT-39243, LT-45435, LT-35098, LT-28353, LT-14845, LT-87534, LT-78167, LT-28463, LT-18787, LT-28721, LT-39292, LT-31048, LT-18436, LT-87392, LT-29706, LT-60824, LT-79521, LT-89584, LT-90435, LT-58194, LT-50862, LT-61498, LT-78491, LT-84910, LT-90135, LT-18924, LT-20984, LT-68732, LT-76231, LT-58463, LT-48924, LT-58464, LT-69840, LT-69483, LT-40285, LT-10292, LT-98404, LT-78457, LT-28714, LT-38473, LT-83740, LT-48332, LT-59341, LT-69547, LT-79693, LT-81722, LT-89019, LT-67382, LT-78194, LT-87948, LT-94317, LT-18499, LT-28749, LT-60944, LT-40938, LT-30924, LT-48594, LT-56954, LT-69405, LT-89876, LT-79413, LT-18486, LT-29785, LT-38194, LT-48148, LT-18844, LT-96432 |
| --- | --- |
| Measure Description: | Interior integral non-dimmable (screw-in) compact fluorescent lamps (CFL) |
| Base Case Description: | Incandescent lamp |
| Energy Impact Common Units: | Per lamp |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | Commercial: 3.3 years Residential: 9.7 years |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Upstream: 0.54 Midstream: 0.7 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Work paper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG017.0 | No | 4/12/12 | Jenny Roehm, PECI | Original work paper for 2013 PC |
|  |  |  | Brian V. O’Keefe/SCE | Used DEER 2011 where available for measure savings source. |
| SCE13LG017.1 | Yes | 4/15/14 | Linda Wan, P.E./TRC Energy Services | -Updated work paper template. -Updated solution codes. -Added technical description. -Updated code analysis for Title 24 2013. -Added installation rate. -Added Non-DEER Study Review. -Updated energy savings and DEER measures. |
|  | No | 4/15/14 | Jason Wang/SCE/EAS | Work paper updated for reporting period, effective 7/1/14 – 12/31/14. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is an interior integral non-dimmable (screw-in) compact fluorescent lamp (CFL). The lamp type can be:

- Spiral

- Spiral UL-Rated

- Spiral with Reflector

- Candelabra

- Integral A

- Globe

The measure case wattages range from 7 to 55 Watts.

The base case is an incandescent lamp.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-87948 | 7 Watt Candelabra CFL |
| LT-94317 | 7 Watt Integral A CFL |
| LT-18499 | 7 Watt Integral Globe CFL |
| LT-28749 | 7 Watt Integral Spiral CFL |
| LT-60944 | 7 Watt Integral Spiral UL-Rated CFL |
| LT-40938 | 7 Watt Integral Spiral with Reflector CFL |
| LT-30924 | 8 Watt Candelabra CFL |
| LT-48594 | 8 Watt Integral A CFL |
| LT-56954 | 8 Watt Integral Globe CFL |
| LT-69405 | 8 Watt Integral Spiral CFL |
| LT-89876 | 8 Watt Integral Spiral UL-Rated CFL |
| LT-79413 | 8 Watt Integral Spiral with Reflector CFL |
| LT-18486 | 9 Watt Candelabra CFL |
| LT-29785 | 9 Watt Integral A CFL |
| LT-38194 | 9 Watt Integral Globe CFL |
| LT-48148 | 9 Watt Integral Spiral CFL |
| LT-18844 | 9 Watt Integral Spiral UL-Rated CFL |
| LT-96432 | 9 Watt Integral Spiral with Reflector CFL |
| LT-42938 | 10 Watt Candelabra CFL |
| LT-59849 | 10 Watt Integral A CFL |
| LT-69598 | 10 Watt Integral Globe CFL |
| LT-71834 | 10 Watt Integral Spiral CFL |
| LT-82934 | 10 Watt Integral Spiral UL-Rated CFL |
| LT-91044 | 10 Watt Integral Spiral with Reflector CFL |
| LT-32354 | 11 Watt Candelabra CFL |
| LT-43526 | 11 Watt Integral A CFL |
| LT-56472 | 11 Watt Integral Globe CFL |
| LT-67593 | 11 Watt Integral Spiral CFL |
| LT-30988 | 11 Watt Integral Spiral UL-Rated CFL |
| LT-24394 | 11 Watt Integral Spiral with Reflector CFL |
| LT-96705 | 12 Watt Candelabra CFL |
| LT-10566 | 12 Watt Integral A CFL |
| LT-24746 | 12 Watt Integral Globe CFL |
| LT-32875 | 12 Watt Integral Spiral CFL |
| LT-47948 | 12 Watt Integral Spiral UL-Rated CFL |
| LT-54837 | 12 Watt Integral Spiral with Reflector CFL |
| LT-59873 | 13 Watt Candelabra CFL |
| LT-90870 | 13 Watt Integral A CFL |
| LT-10112 | 13 Watt Integral Globe CFL |
| LT-23829 | 13 Watt Integral Spiral CFL |
| LT-60194 | 13 Watt Integral Spiral UL-Rated CFL |
| LT-49877 | 13 Watt Integral Spiral with Reflector CFL |
| LT-84743 | 14 Watt Candelabra CFL |
| LT-46312 | 14 Watt Integral A CFL |
| LT-56121 | 14 Watt Integral Globe CFL |
| LT-67591 | 14 Watt Integral Spiral CFL |
| LT-94387 | 14 Watt Integral Spiral UL-Rated CFL |
| LT-76584 | 14 Watt Integral Spiral with Reflector CFL |
| LT-24326 | 15 Watt Candelabra CFL |
| LT-58432 | 15 Watt Integral A CFL |
| LT-69431 | 15 Watt Integral Globe CFL |
| LT-38498 | 15 Watt Integral Spiral CFL |
| LT-39874 | 15 Watt Integral Spiral UL-Rated CFL |
| LT-12918 | 15 Watt Integral Spiral with Reflector CFL |
| LT-69807 | 16 Watt Candelabra CFL |
| LT-73859 | 16 Watt Integral A CFL |
| LT-89505 | 16 Watt Integral Globe CFL |
| LT-95069 | 16 Watt Integral Spiral CFL |
| LT-15926 | 16 Watt Integral Spiral UL-Rated CFL |
| LT-28401 | 16 Watt Integral Spiral with Reflector CFL |
| LT-50988 | 18 Watt Candelabra CFL |
| LT-48984 | 18 Watt Integral A CFL |
| LT-58901 | 18 Watt Integral Globe CFL |
| LT-68142 | 18 Watt Integral Spiral CFL |
| LT-68743 | 18 Watt Integral Spiral UL-Rated CFL |
| LT-44133 | 18 Watt Integral Spiral with Reflector CFL |
| LT-84330 | 19 Watt Candelabra CFL |
| LT-71032 | 19 Watt Integral A CFL |
| LT-89941 | 19 Watt Integral Globe CFL |
| LT-91324 | 19 Watt Integral Spiral CFL |
| LT-99875 | 19 Watt Integral Spiral UL-Rated CFL |
| LT-79878 | 19 Watt Integral Spiral with Reflector CFL |
| LT-29426 | 20 Watt Candelabra CFL |
| LT-13421 | 20 Watt Integral A CFL |
| LT-28134 | 20 Watt Integral Globe CFL |
| LT-38321 | 20 Watt Integral Spiral CFL |
| LT-34543 | 20 Watt Integral Spiral UL-Rated CFL |
| LT-18546 | 20 Watt Integral Spiral with Reflector CFL |
| LT-25143 | 22 Watt and 32 Watt T9 Circline CFL |
| LT-89732 | 22 Watt Candelabra CFL |
| LT-78432 | 22 Watt Integral A CFL |
| LT-69921 | 22 Watt Integral Globe CFL |
| LT-50789 | 22 Watt Integral Spiral CFL |
| LT-40794 | 22 Watt Integral Spiral UL-Rated CFL |
| LT-37456 | 22 Watt Integral Spiral with Reflector CFL |
| LT-30916 | 22 Watt T9 Circline CFL |
| LT-59843 | 23 Watt Candelabra CFL |
| LT-49382 | 23 Watt Integral A CFL |
| LT-57843 | 23 Watt Integral Globe CFL |
| LT-69442 | 23 Watt Integral Spiral CFL |
| LT-60988 | 23 Watt Integral Spiral UL-Rated CFL |
| LT-44329 | 23 Watt Integral Spiral with Reflector CFL |
| LT-89877 | 24 Watt Candelabra CFL |
| LT-78410 | 24 Watt Integral A CFL |
| LT-89321 | 24 Watt Integral Globe CFL |
| LT-94312 | 24 Watt Integral Spiral CFL |
| LT-97423 | 24 Watt Integral Spiral UL-Rated CFL |
| LT-77098 | 24 Watt Integral Spiral with Reflector CFL |
| LT-28575 | 25 Watt Candelabra CFL |
| LT-10954 | 25 Watt Integral A CFL |
| LT-83213 | 25 Watt Integral Globe CFL |
| LT-71984 | 25 Watt Integral Spiral CFL |
| LT-61823 | 25 Watt Integral Spiral UL-Rated CFL |
| LT-52345 | 25 Watt Integral Spiral with Reflector CFL |
| LT-39670 | 26 Watt Candelabra CFL |
| LT-49287 | 26 Watt Integral A CFL |
| LT-54382 | 26 Watt Integral Globe CFL |
| LT-21645 | 26 Watt Integral Spiral CFL |
| LT-69566 | 26 Watt Integral Spiral UL-Rated CFL |
| LT-78965 | 26 Watt Integral Spiral with Reflector CFL |
| LT-49762 | 26 Watt T9 Circline CFL |
| LT-23832 | 27 Watt Candelabra CFL |
| LT-34328 | 27 Watt Integral A CFL |
| LT-48345 | 27 Watt Integral Globe CFL |
| LT-54954 | 27 Watt Integral Spiral CFL |
| LT-69599 | 27 Watt Integral Spiral UL-Rated CFL |
| LT-39243 | 27 Watt Integral Spiral with Reflector CFL |
| LT-45435 | 28 Watt Candelabra CFL |
| LT-35098 | 28 Watt Integral A CFL |
| LT-28353 | 28 Watt Integral Globe CFL |
| LT-14845 | 28 Watt Integral Spiral CFL |
| LT-87534 | 28 Watt Integral Spiral UL-Rated CFL |
| LT-78167 | 28 Watt Integral Spiral with Reflector CFL |
| LT-28463 | 30 Watt Candelabra CFL |
| LT-18787 | 30 Watt Integral A CFL |
| LT-28721 | 30 Watt Integral Globe CFL |
| LT-39292 | 30 Watt Integral Spiral CFL |
| LT-31048 | 30 Watt Integral Spiral UL-Rated CFL |
| LT-18436 | 30 Watt Integral Spiral with Reflector CFL |
| LT-87392 | 32 Watt and 40 Watt T9 Circline CFL |
| LT-29706 | 32 Watt Candelabra CFL |
| LT-60824 | 32 Watt Integral A CFL |
| LT-79521 | 32 Watt Integral Globe CFL |
| LT-89584 | 32 Watt Integral Spiral CFL |
| LT-90435 | 32 Watt Integral Spiral UL-Rated CFL |
| LT-58194 | 32 Watt Integral Spiral with Reflector CFL |
| LT-50862 | 32 Watt T9 Circline CFL |
| LT-61498 | 40 Watt Candelabra CFL |
| LT-78491 | 40 Watt Integral A CFL |
| LT-84910 | 40 Watt Integral Globe CFL |
| LT-90135 | 40 Watt Integral Spiral CFL |
| LT-18924 | 40 Watt Integral Spiral UL-Rated CFL |
| LT-20984 | 40 Watt Integral Spiral with Reflector CFL |
| LT-68732 | 40 Watt T5 Circline CFL |
| LT-76231 | 40 Watt T9 Circline CFL |
| LT-58463 | 42 Watt Candelabra CFL |
| LT-48924 | 42 Watt Integral A CFL |
| LT-58464 | 42 Watt Integral Globe CFL |
| LT-69840 | 42 Watt Integral Spiral CFL |
| LT-69483 | 42 Watt Integral Spiral UL-Rated CFL |
| LT-40285 | 42 Watt Integral Spiral with Reflector CFL |
| LT-10292 | 45 Watt Candelabra CFL |
| LT-98404 | 45 Watt Integral A CFL |
| LT-78457 | 45 Watt Integral Globe CFL |
| LT-28714 | 45 Watt Integral Spiral CFL |
| LT-38473 | 45 Watt Integral Spiral UL-Rated CFL |
| LT-83740 | 45 Watt Integral Spiral with Reflector CFL |
| LT-48332 | 55 Watt Candelabra CFL |
| LT-59341 | 55 Watt Integral A CFL |
| LT-69547 | 55 Watt Integral Globe CFL |
| LT-79693 | 55 Watt Integral Spiral CFL |
| LT-81722 | 55 Watt Integral Spiral UL-Rated CFL |
| LT-89019 | 55 Watt Integral Spiral with Reflector CFL |
| LT-67382 | 55 Watt T8 Circline CFL |
| LT-78194 | 55 Watt T9 Circline CFL |

Implementation Requirements

This measure is eligible in the following building types:

- Residential Single Family

- Miscellaneous Commercial

## 1.2 Technical Description

Integral CFLs contain the lamp, ballast, and screw-in base together as a single unit.  As a result, when the CFL burns out, the entire unit is replaced [406].  The specialty bulbs involved in this measure are the following shapes and sizes [407]:

- Candelabra – More likely to be used for decorative reasons and has a slimmer shape that can fit into tighter fixtures;

- Integral A – Has the look of an incandescent bulb but has the efficiency of a spiral bulb;

- Integral globe – Most commonly used in bathroom vanity bars and ceiling pendants;

- Integral spiral – Most popular type of CFL;

- Integral spiral UL-rated – Has been through performance testing and meets the Underwriters Laboratories (UL) requirements;

- Integral spiral with reflector – Most commonly used in directional lighting, specifically in recessed ceiling fixtures; and

- Circline – Circular lamp that is packaged with the electronic ballast and also contains a screw base.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this work paper and application types used by READi.

The delivery methods are:

- Upstream Programs – Up-Stream Buy Down

The upstream strategy is used to stimulate sales of energy efficient lighting. The lighting measures are promoted and tracked separately from other point of service (POS) rebate measures.  Lighting incentives and promotions influence customers to purchase energy-saving lighting products at retail outlets and install them in homes and small businesses.

- Midstream Programs / Mid-Stream Incentive

A key portion of the Residential Lighting Incentive Program educates customers to use the products correctly while understanding their benefits. The POS program element provides maximum ease for customers to participate. It features discounted ENERGY STAR® qualified products, and introduces new and advanced lighting technologies to the market.

  - Only the circline measures are offered mid-stream.

The install type is:

- Replace on Burnout (ROB)

ROB measures replace existing equipment with more energy efficient equipment when the existing equipment has failed or passed its useful life.

## 1.4 Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

### Most measures are directly from DEER, while some measures are scaled using the READi tool.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | Yes |
| DEER Building Prototypes Used | No |
| Deviation from DEER | Some DEER measures are used directly, while other measures are scaled using the READi tool. |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | Com-Lighting-InGen_CFLratio0357_CFLscw-7w |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGRs for this work paper are shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mCFL | CFL-screw in, All. | Com | Any | PreRebUp | 0.54 |
| Res-sAll-mCFL-up | CFL-screw-in, All. | Res | Any | PreRebUp | 0.54 |
| All-Default<=2yrs | All other EEM with no evaluated NTGR; new technology in program for 2 or fewer years | Any | Any | Any | 0.7 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no final version on the installation rate table. To address appropriate selection of the installation rate the date of the work paper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below:

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Com-CFL-SCE | Non-Res CFL; Non-Upstream Program; Annual Installation Rate | Com | Any | Any | 0.61 |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1.00 |

Com-CFL-SCE is used for all Mid-stream measures, and Def-GSIA is used for all Upstream measures.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the work papers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the work paper. For a full set of values associated with the measures in the work paper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this work paper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | InGen |
| Measure Case TechGroups | Ltg_Screwin |
| Measure Case TechTypes | CFLint_lamp |
| Base Case TechGroups | Ltg_Screwin |
| Base Case TechTypes | Incan_lamp |

### 1.4.2 Codes and Standards Analysis

Title 24 (2013) Building Energy Efficiency Standards [355] includes standards for both non-residential and residential buildings.  Section 150.0 (k) 7 and Section 150.0 (k) 8 pertain to residential luminaries.  Both Sections call out that installed luminaries need to be high efficacy lighting as defined by Table 150.0-A or Table 150.0-B for lighting systems not listed in Table 150.0-A.  According to Table 150.0-A, “GU-24 sockets for compact fluorescent lamps” are classified as high efficacy.  Thus, the measures outlined in this work paper satisfy the requirement as a high efficacy lighting technology.

Title 20 (2014) Appliance Efficiency Regulations [422] includes standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California.  Table K-5 lists the minimum efficacy (lumens/watt) requirements for medium base compact fluorescent lamps. It is assumed that the measures listed in DEER 2014 comply with the 2014 Title 20, Table K-3.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards Section 150.0 (k) | July 1, 2014 |
| Title 20 (2014) | Table K-5 Standards for Medium Base Compact Fluorescent Lamps | January 8, 2007 |

### 1.4.3 Non-DEER Study Review

There are no applicable non-DEER studies associated with this work paper.

### 1.4.4 Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. DEER rates the life of a CFL as 10,000 hours. The EUL is determined by dividing 10,000 hours by the DEER annual operating hours for the building type. The residential EUL is further multiplied by a switching degradation factor of 0.523. Table 7 below identifies the values used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFL-Com | Commercial | Lighting | CFL replacing Incandescent Lamp | 3.2 | N/A |
| ILtg-CFL-Res | Residential | Lighting | CFL replacing Incandescent Lamp | 9.7 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

Most measures in this work paper are directly from the DEER 2014 READi tool. However, the following measures were not available from READi and were therefore created as new scaled measures in READi:

- Commercial 45-Watt CFL

- Residential 40-Watt CFL

- Residential 40-Watt Reflector CFL

Table 8 contains a data file with all the READi exports. These results have not been modified and are only being included in the workpaper for reference.

Table 8 READi Tool Outputs

| Solution Code | Measure Name | READi Results |
| --- | --- | --- |
| Multiple solution codes | Multiple measures |  |

The methodology used by READi to calculate energy savings is described in the Energy Division (ED) Workpaper Disposition for Lighting Retrofits (Attachment 3, [385]). Given the measure case wattage, the base case wattage is calculated by multiplying the measure case wattage by a wattage ratio:

- Residential Non-Reflector CFL: 3.47

- Residential Reflector CFL: 4.09

- Non-residential CFL: 3.57

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Miscellaneous Commercial | NON_RES | DEER:Indoor_CFL_Ltg |
| Residential Single Family | RES | DEER:Indoor_CFL_Ltg |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

Base case costs are from DEER 2008 [215]. For measures that do not have an exact match in DEER, the closest values are used.  For example, DEER does not have a 24.29-Watt incandescent, so the 40-Watt incandescent is used. See Table 10 below for sample base case costs.

Table 10 Sample DEER 2008 Base Case Costs

| Base Case Cost Description | Commercial & Residential Cost Case ID | Program Delivery Strategies | Base Case Material Cost |
| --- | --- | --- | --- |
| Incandescent, medium base, 40W | D08-NE-ILtg-Inc-40W | All Program Delivery Strategies | $0.64 |
| Incandescent, medium base, reflector, 60W | D08-NE-ILtg-Inc-Rfl-60W | All Program Delivery Strategies | $3.54 |
| Incandescent, medium base, globe, 40W | D08-NE-ILtg-Inc-Glb-40W | All Program Delivery Strategies | $4.90 |

The base case labor cost is $5.89, from DEER 2008.

## 4.2 Measure Case Cost

Measure case costs are from DEER 2008. For measures that do not have an exact match in DEER, the closest values are used.  For example, DEER does not have a 12-Watt CFL, so the 13-Watt CFL is used.

SCE believes that the DEER 2008 measure case costs are “Participant Costs” which are determined using the following equation:

Participant Cost = Gross Measure Cost – Rebates to Customer – Direct Install Labor and Material Costs

Therefore, to determine the actual measure case cost, the upstream incentive should be added to the DEER value. This work paper directly uses the DEER costs without modification because the upstream incentives are unknown. As a result, the costs in this work paper should not be directly used as inputs for the E3 calculator.

Table 11 shows a sample of the measure costs.

Table 11 Sample DEER 2008 Measure Case Costs

| Measure Case Cost Description | Commercial Cost Case ID | Residential Cost Case ID | Program Delivery Strategies | Measure Case Material Cost |
| --- | --- | --- | --- | --- |
| 7 Watt Integral Spiral CFL | D08-NE-ILtg-CFL-Int-7wCFLsSMg7w-Rpl | D08-RE-InLt-CFL-Int-7W | Upstream Prescriptive Rebates/Incentives - Single Pack | $1.15 |
| 15 Watt Integral Spiral with Reflector CFL | D08-NE-ILtg-CFL-Int-15wCFLsrSMg15w-Rpl | D08-RE-InLt-CFL-Rfl-15W | Upstream Prescriptive Rebates/Incentives - Single Pack | $5.94 |
| 9 Watt Integral Globe CFL | D08-NE-ILtg-CFL-Int-Glb-9wCFLsSMg9w-Rpl | D08-RE-InLt-CFL-Glb-9W | Upstream Prescriptive Rebates/Incentives - Single Pack | $4.90 |

Circline costs are from an online retailer.  Table 12 is a summary of the material costs for circlines.

Table 12 Circline Cost

| Measure Case Cost Description | Measure Case Material Cost |
| --- | --- |
| 22W, 8” Diameter, Cool White T9 Circline Bulb | $7.49 |
| 32W, 12” Diameter, Cool White T9 Circline Bulb | $8.99 |
| 40W, 16” Diameter, Cool White T9 Circline Bulb | $11.49 |
| 40W, 12” Diameter, Cool White T5 Circline Bulb | $24.99 |

The measure case labor cost is $5.89, from DEER 2008.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

The gross measure cost is the cost to install an energy efficient measure.  In the case of ROB, the equipment being replaced/installed is assumed to have failed in place or is past its useful life. In this scenario, the customer is in the situation of having to purchase new equipment.  The customer is faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.  Because the customer will be spending money to replace their equipment anyway, the gross cost for the energy efficient measure is the premium paid above the non-efficient or code baseline equipment.

The gross measure cost (∆cost/unit) is defined as the difference in costs between the base case and the measure equipment.  Installation labor costs are assumed to be the same.

X = Base Case Cost = $0.64

Y = Measure Equipment Cost = $1.15

∆cost/unit = Y- X = $1.15 – $0.64 = $0.51

For a complete breakdown of base, measure, and gross/incremental measure costs, please refer to Attachment 2.

### 4.3.2 Incremental Measure Cost

For ROB measures, the IMC is represented by the equation below:

IMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

*Note: The measure case labor and base case labor are the same value, reducing the equation to the following:

IMC 	= Measure Equipment Cost – Base Case Equipment Cost

In the case of ROB, the IMC and GMC are the same.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [213] | EUL/RUL Values Provided through Excel Spreadsheet |
| [215] | Revised DEER Measure Cost Summary |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [385] | Workpaper Disposition for Lighting Retrofits |
| [406] | Efficient Lighting Using Compact Fluorescent Lamps and Fixtures |
| [407] | CFL Sizes and Shapes |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |