Work Paper SCE13LG019

Revision 1

Southern California Edison Company

Energy Star Ceiling Fan with CFLs

# At-a-Glance Summary

| Applicable Measure Codes: | AC-90878, LT-89679, LT-90632 |
| --- | --- |
| Measure Description: | Energy Star Ceiling Fan |
| Base Case Description: | Non-Energy Star Ceiling Fan w/Incandescent Lamps |
| Energy Impact Common Units: | Fan |
| Energy Savings : | Refer to Calculation Template Example: Res SF, CZ6: 33.43 kWh, 0.01211 kW, -0.51980 Therms |
| Gross Measure Cost ($/unit) | Refer to Calculation Template Example: Res SF, CZ6: $275.37 |
| Measure Incremental Cost ($/unit): | Refer to Calculation Template Example: Res SF, CZ6: $46.55 |
| Effective Useful Life (years): | 16 |
| Measure Application Type: | RET |
| Net-to-Gross Ratios: | Downstream: 0.55, Direct Install: 0.85 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG019.0 | No | 4/16/12 | Jared Kruzek/AESC | Original work paper 2013 PC |
| SCE13LG019.1 | Yes | 4/17/14 | Yun Han/SCE | - New WP template w/updated IEs - Work paper updated for reporting period, effective 7/1/2014-12/31/2014 - Code language update |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the retrofit of a ceiling fan with incandescent lamps to an Energy Star Ceiling Fan with CFLs.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| AC-90878 | Energy Star Ceiling Fan replacing Ceiling Fan |
| LT-89679 | Energy Star Ceiling Fan (Common Area) replacing Ceiling Fan |
| LT-90632 | Energy Star Ceiling Fan (Dwelling Area) replacing Ceiling Fan |

The measures in this work paper apply to residential applications in SCE climate zones and include the following building types:  Residential – Single Family, Residential – Multi Family, and Residential – Double Wide Mobile Home.

To qualify for lighting savings the existing fan fixture must have incandescent bulbs. The existing fan fixture must also not already be an Energy Star fixture.

## 1.2 Technical Description

The measures in this work paper are replacing the whole ceiling fan, not just the lamps installed. The energy savings includes savings from lighting as well as having premium efficiency motors in the new ceiling fans.

## 1.3 Measure Application Type

The delivery methods that are available for these measures are:

- Financial Support / Down-Stream Incentive – Deemed

- Financial Support – Direct Install

The program/install type for the above measures is:

- Retrofit (RET)

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this type of measure |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | InGen |
| Measure Case TechGroups | Non-DEER |
| Measure Case TechTypes | Non-DEER |
| Base Case TechGroups | Non-DEER |
| Base Case TechTypes | Non-DEER |

### 1.4.2 Codes and Standards Analysis

According to Title 20 2014 [422], there are no energy efficiency standards or energy design standards for ceiling fans and ceiling fan light kits.

Title 24 2013 [355] states - “In residential kitchens, the wattage of electrical boxes finished with a blank cover or where no electrical equipment has been installed, and where the electrical box can be used for a luminaire or a surface mounted ceiling fan, shall be calculated as 180 watts of low efficacy lighting per electrical box.”

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 150.0k1(c) | July 1, 2014 |
| Title 20 (2012) | 2012 Appliance Efficiency Regulations 1605.1(d) | January 1, 2007 |

### 1.4.3 Non-DEER Study Review

Estimated Energy Star Ceiling Fan Replacement Savings of the Statewide Investor Owned Utility Ceiling Fan Study [372] was used for energy and demand savings. The data was taken from page 24, Table 13, which shows the operating hours of the lighting and the fan for the baseline and the Energy-Star model Ceiling Fans.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value, the READI v1.0.5 [] tool was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFLfix-Res | Residential | Indoor Lighting | CFL Fixtures | 16 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

The energy and demand savings are from Table 13 of the Estimated Energy Star Ceiling Fan Replacement Savings of the Statewide Investor Owned Utility Ceiling Fan Study [372].  It assumes that the premium efficiency motors provide a 20% energy savings over the existing standard efficiency motors and the CFLs provide a 60% energy savings over the existing incandescent light bulbs.  The results of this study apply to residential single and multifamily dwellings located in all climate zones throughout California.

Table 8 Residential Energy Savings for Energy Star Ceiling Fans with CFLs

|  |  | Load | Run Time | Annual Energy |
| --- | --- | --- | --- | --- |
| Statewide |  | (kW) | (hours) | (kWh) |
|  | Lighting | .132 | 329 | 43.43 |
|  | Fans | .038 | 897 | 34.09 |
| Energy-Star |  |  |  |  |
|  | Lighting | .053 | 329 | 17.44 |
|  | Fans | .030 | 897 | 26.91 |
| Savings |  |  |  |  |
|  | Lighting | .079 |  | 25.99 |
|  | Fans | .008 |  | 7.18 |
| Total |  |  |  | 33.17 |

The energy and demand savings for the lighting and fans are calculated separately. The savings are combined for each of the three core measures, but interactive effects and coincident diversity factors are only applied to the lighting portion. Interactive effects and the diversity factors vary by building type, and the total savings will vary by building type as well. It is assumed that the ceiling fan is running continuously during the DEER peak period (concurrently with the residential HVAC unit) and that the lighting is operating for a fraction of time during the DEER peak period.

Interactive effects have been incorporated for the lighting measures only. It is assumed that for each building type an AC unit is serving the space where the fan will be installed and the more efficient lights will reduce the cooling load. Since the study referenced above does not take into account interactive effects, they have been added on top of the savings derived from that study.  As an example, for the common area lighting measure in climate zone 6 in a Residential – Multi-family building type, energy interactive effects of 1.0037, demand interactive effect of 1.193 and a coincident diversity factor of 0.043 provided in DEER 2014 are applied to the savings:

Total Energy Savings = Lighting Energy Savings * Energy Interactive Effects + Fan Energy Savings

Total Energy Savings = 25.99*1.0037 + 7.18

Total Energy Savings = 33.27 kWh

Total Demand Reduction = Lighting Demand Savings*Coincident Diversity Factor * Demand Interactive Effects + Fan Demand Savings

Total Demand Reduction = 0.079 kW * 0.043 * 1.193 + .008 kW

Total Demand Reduction = 0.01205 kW

Full list of energy savings and demand reduction for other building types and climate zones are in the attachment [].

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential Single Family | RES | DEER:Indoor_CFL_Ltg |
| Residential Multi-family | RES | DEER:Indoor_CFL_Ltg |
| Residential Mobile Home - Double-Wide | RES | DEER:Indoor_CFL_Ltg |

# Section 4. Base Case & Measure Costs

Direct Install measures use program tracking systems for measure costs. Full list of costs with climate zone multipliers applied can be found in the attachment [B].

## 4.1 Base Case Cost

The total base case cost of a Ceiling Fan for a RET measure is $255.34. The base case equipment cost of $222.34 was averaged from 6 different non-Energy Star brands of various models.  Please see the Ceiling Fan Prices Summary in the attachments section [,].  The base labor cost used for this work paper is $33.00 and was established using RSMeans [406] for a ceiling fan with 2,960 CFM fan.

## 4.2 Measure Case Cost

The total measure cost of a Ceiling Fan is $308.85. The measure equipment cost of $275.85 was averaged from 10 different Energy Star brands of various models.  Please see the Ceiling Fan Prices Summary attachments [C,D] for more information.  The measure labor cost was assumed to be equal to the base case labor cost of $33.00.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

In the case of RET, the customer is making a conscious decision to replace existing, working equipment before the useful life of the equipment has expired. Since this is a discretionary choice by the consumer, the cost invoked is the full cost of the energy efficient equipment and installation.

For RET, GMC is represented by the equation below. Measure cost and reference are listed in Section 4.2.

GMC = Measure Equipment Cost + Measure Labor Cost

GMC = $275.85 + $33.00

GMC = $308.85

### 4.3.2 Incremental Measure Cost

For RET measures, there exists a theoretical base case that the measure can be compared in cost. Because a base case exists for RET, IMC is represented by the equation below.

IMC = Measure Equipment Cost – Base Case Equipment Cost

IMC = $275.85 - $222.34

IMC = $53.51

# Attachments

1 2 3

# References

[31]

[351]

[355]

[372]

[406]

[422]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

http://deeresources.com/index.php/deer2013-update-for-2014-codes

Attachment 1 - Calculation Template 2015 v3.xlsm

Attachment 2 – Ceiling Fan Prices Summary.xls

Attachment 3 – Ceiling Fan Prices PDF.xls