Work Paper SCE13LG020

Revision 1

Southern California Edison Company

Wall Mounted Occupancy Sensors – Multifamily and Hospitality

# At-a-Glance Summary

| Applicable Measure Codes: | Residential: LT-60778, LT-79435, LT-78685, LT-60934 Commercial: LT-44912, LT-50932 |
| --- | --- |
| Measure Description: | Wall-mounted occupancy sensor controlling a lighting system in a mobile home, multi-family building, or commercial lodging guest room. |
| Base Case Description: | Lighting system without automated controls. |
| Energy Impact Common Units: | Sensor |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | EUL: 8 years, EUL ID: ILtg-OccSens |
| Measure Application Type: | Retrofit Add-on (REA) |
| Net-to-Gross Ratios: | NTGR: Nonres: 0.6 Nonres Direct Install: 0.6 Nonres Direct Install Hard-to-Reach: 0.89 Res: 0.55 Res Direct Install: 0.85 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG020.0 | No | 4/25/12 | Andres Fergadiotti/SCE (DES) | Updated to 2013-14 work paper template. |
| SCE13LG020.1 | Yes | 4/15/14 | Siobhan McCabe/TRC Energy Services | - Updated work paper template. - Updated calculation template to modify occupancy sensor and interactive effects factors. |
|  | No | 6/16/14 | Jason Wang/SCE (EAS) | - Work paper updated for reporting period, effective 7/1/14 – 12/31/14. - Non-res savings updated for ED disposition |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is a wall-mounted occupancy sensor installed on a lighting system; the type and quantity of controlled lighting fixtures vary by area type. The base case assumes that no occupancy sensor is installed.

Table 1 shows several examples of the areas that qualify for each measure:

Table 1 Measure Names

| Solution Code | Measure Name | Area Description |
| --- | --- | --- |
| Residential Market Sector |  |  |
| LT-60778 | Occupancy Sensor (Res Living Area) Control | Kitchens, Living Rooms, etc. |
| LT-79435 | Occupancy Sensor (Res Bathroom) Control | Bathrooms |
| LT-78685 | Occupancy Sensor (Res Common Area) Control | Laundry Rooms, Clubhouses, etc. |
| LT-60934 | Occupancy Sensor (Res Dwelling Area) Control | Average of Living and Bathroom |
| Non Residential Market Sector |  |  |
| LT-44912 | Occupancy Sensor (Non Res Bathroom) Control | Hotel/Motel Guest Room Restrooms |
| LT-50932 | Occupancy Sensor (Non Res Dwelling Area) Control | Hotel/Motel Guest Rooms |

Eligibility Requirements

- Only wall-mounted sensors are eligible. Ceiling-mounted sensors and integrated sensors in fixtures are not eligible.

- There may be additional eligibility requirements based upon contractual agreements with entities receiving incentives as part of this program.

Implementation Requirements

- This measure is approved for installation in:

  - Living, bathroom, common, and dwelling areas of Residential Multi-family and Mobile Home - Double-Wide buildings.

  - Restrooms and guest rooms of hotels and motels.

- All SCE climate zones are eligible.

Documentation Requirements

The programs offering the measures in this work paper may require submitted paid invoices for approval.

## 1.2 Technical Description

A lighting occupancy sensor is a device that automatically turns lights on when a person enters into a room, and turns lights off when a specified period of time has passed after an area is vacated. Energy savings are achieved by reducing the typical run time of the lights.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

This program is offered via:

- Financial Support - Down-Stream Incentive – Deemed

- Financial Support - Direct Install

- Midstream Programs / Mid-Stream Incentive

The install type of these measures is Retrofit Add-on (REA). This is a subtype of RET that is specifically for adding a piece of equipment to an existing system or equipment to make the overall system or equipment more efficient.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

This specific measure is not included in the 2014 Database for Energy Efficient Resources (DEER) READi tool. The DEER version 2005 [26] does include a wall mounted occupancy sensor measure, Measure ID D03-856 (Run ID CALC00AVOCC01). However, the DEER occupancy sensor assumes a building type of “All Commercial” which is different than that of the measures in this work paper. This work paper assumes that an occupancy sensor is installed in a multi-family, mobile home, or lodging guest room building type. These differing building types result in different savings values. Table 2 indicates the DEER difference summary.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | Different building type (e.g., All Commercial vs. Residential) |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mOccSens | Occupancy Sensors | Com | Any | PreRebDown | 0.6 |
| NonRes-sAll-mLtgCtrl | Lighting controls | Com | Any | DirInstall | 0.6 |
| NonRes-sAll-mLtgCtrl-htr | Lighting controls | Com | Any | DirInstall | 0.89 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the defaulted NTG values, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no version of the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | Controls |
| Measure Case TechGroups | NonDEER |
| Measure Case TechTypes | OccSensor |
| Base Case TechGroups | NonDEER |
| Base Case TechTypes | NonDEER |

### 1.4.2 Codes and Standards Analysis

The measures in this work paper are not affected by the California Appliance Efficiency Regulations – Title 20 (2014) [422].

Title 24 (2013) [355]

Section 150.0(k) Residential Lighting (see excerpt below) requires high efficacy fixtures in bathrooms, laundry rooms, utility rooms, and garages. Occupancy or vacancy sensors are optional for bathrooms but required for laundry rooms, garages and utility rooms. Other areas, including multi-family common areas, have the option of installing either high efficacy fixtures or controls. Since the measures in this work paper are Retrofit Add-on, they are not affected by this code.

Section 150.0(k) Residential Lighting 5. Lighting in Bathrooms. Lighting installed in bathrooms shall meet the following requirements: - A minimum of one high efficacy luminaire shall be installed in each bathroom; and - All other lighting installed in each bathroom shall be high efficacy or controlled by vacancy sensors. 6. Lighting in Garages, Laundry Rooms, and Utility Rooms. Lighting installed in attached and detached garages, laundry rooms, and utility rooms shall be high efficacy luminaires and controlled by vacancy sensors. 7. Lighting other than in Kitchens, Bathrooms, Garages, Laundry Rooms, and Utility Rooms. Lighting installed in rooms or areas other than in kitchens, bathrooms, garages, laundry rooms, and utility rooms shall be high efficacy, or shall be controlled by either dimmers or vacancy sensors. EXCEPTION 1 to Section 150.0(k)7: Luminaires in closets less than 70 square feet. EXCEPTION 2 to Section 150.0(k)7: Lighting in detached storage buildings less than 1,000 square feet located on a residential site. 12. Interior Common Areas of Low-rise Multi-Family Residential Buildings. - In a low-rise multifamily residential building where the total interior common area in a single building equals 20 percent or less of the floor area, permanently installed lighting for the interior common areas in that building shall be high efficacy luminaires or controlled by an occupant sensor. - In a low-rise multifamily residential building where the total interior common area in a single building equals more than 20 percent of the floor area, permanently installed lighting in that building shall: - Comply with the applicable requirements in Sections 110.9, 130.0, 130.1, 140.6 and 141.0; and - Lighting installed in corridors and stairwells shall be controlled by occupant sensors that reduce the lighting power in each space by at least 50 percent. The occupant sensors shall be capable of turning the light fully On and Off from all designed paths of ingress and egress.

Section 130.1(c) Shut-OFF Controls (see excerpt below) states that hotel/motel guest rooms must have captive card key controls, occupancy sensors or other automatic controls. The exception is an emergency egress light that may be controlled separately and does not need an automatic shut-off. Since the measures in this work paper are Retrofit Add-on, they are not affected by this code.

Section 130.1(c) Shut-OFF Controls 8. Hotel motel guest rooms shall have captive card key controls, occupancy sensing controls, or automatic controls such that, no longer than 30 minutes after the guest room has been vacated, lighting power is switched off. EXCEPTION to Section 130.1(c)8: One high efficacy luminaire as defined in TABLE 150.0-A or 150.0-B that is switched separately and where the switch is located within 6 feet of the entry door.

This workpaper assumes that all base case lighting fixtures are either linear fluorescent or compact fluorescent, both of which classify as high efficacy lighting.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | §130.1(c)8, §150.0(k)5,6,7,12 | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

The following non-DEER studies are referenced in this work paper:

- Occupancy Sensor Nightlight Wall Switch for Hotel Guest Room Bathrooms [383]

- Lighting Energy Savings Opportunities in Hotel Guestrooms [381]

### 1.4.4 Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted.  Table 77 below identifies the value/methodology used for the measures in this work paper. DEER EUL Iltg-OccSens is for the Non-Residential market sector, but this work paper also uses it for the Residential sector because the technology life is expected to be the similar regardless of sector.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-OccSens | Non-Residential | Indoor Lighting | Occupancy Sensors | 8 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

## Energy Savings Methodology

The energy savings for the measures in this work paper are broken out by market sector. The residential sector uses standard fixtures and wattages from the Table of Standard Fixture Wattages. The non-residential sector follows the same approach for guest rooms but uses Emerging Technologies study results for the bathroom area.

### Residential Market Sector

Controlled Wattage Assumptions for Living and Common Areas

For living and common areas, it is assumed that the occupancy sensor controls four linear fluorescent T8 fixtures. The fixture code, type, wattage, and total controlled wattage are shown in Table 8:

Table 8 Living and Common Area Occupancy Controlled Lighting

| Fixture Code* | Number of Controlled Fixtures | Fixture Type* | Fixture Wattage (W)* |
| --- | --- | --- | --- |
| F42ILL-R | 4 | 2-lamp, 48”, 32-Watt T8 | 52 |
|  |  | Total Controlled Wattage: | 208 |

*Obtained from Appendix B Table of Standard Fixture Wattages [382].

Controlled Wattage Assumptions for Bathrooms

For bathrooms, it is assumed that the occupancy sensor controls the vanity and ceiling fan lighting shown in Table 10:

Table 10 Bathroom Occupancy Controlled Lighting

| Fixture Code* | Location | Number of Controlled Fixtures | Fixture Type* | Fixture Wattage (W)* |
| --- | --- | --- | --- | --- |
| CFQ13/1 | Vanity | 4 | 1-lamp, 13-Watt, quad, compact fluorescent** | 17 |
| CFQ18/2 | Ceiling Fan | 1 | 2-lamp, 18-Watt, quad, compact fluorescent | 45 |
|  |  |  | Total Controlled Wattage: | 113 |

*Obtained from Appendix B Table of Standard Fixture Wattages [382].

**The vanity fixture would have used the wattage of a 4-lamp, 13-Watt CFL fixture, but this fixture type was not in Appendix B.

Controlled Wattage Assumptions for Dwelling Areas

No controlled wattage is determined for dwelling areas because the dwelling area energy savings are calculated as the average of the living area and bathroom area energy savings.

Occupancy Sensor Percentage Time Off (PTO)

For all areas, a percentage time off value of 20% is used.

DEER 2005 contains an occupancy sensor measure where a wall box occupancy sensor controls three 4-foot 2-lamp fluorescent fixtures with 34 watt, T-8 lamps and electronic ballasts [26]. This fixture setup represents areas similar to office and retail spaces. DEER 2005 also provides assumptions for operating hours and how they are affected by occupancy sensors. The following table and equation show how the DEER assumptions were used to determine the PTO value for this work paper.

Table 8 DEER 2005 Occupancy Sensor Measure Assumptions

| Scenario | Annual Hours of Operation |
| --- | --- |
| Normal building hours of operation (60 hours/week, 50 weeks/year) | 3000 |
| Base case: Occupants manually switch off the lighting 15% of the time | 2550 |
| Measure Case: Occupancy sensors switch off lighting 1050 hours/year | 1950 |

The PTO is calculated as follows:

PTO %=Base Case Hours-Measure Case [Hours]Normal hours of operation [Hours]

=2550-19503000

=20%

PTO=PTOkWh=PTOkW=PTOtherms

The 20% PTO is used for all residential areas because it is assumed that an occupant will leave the bathroom lights on just as often as in the living room or kitchen.

Energy Savings

The energy savings were calculated as shown in the following example for the Residential Living Area measure, Multi-family building type, climate zone 6:

Energy Savings kWh=Controlled Wattage W*HOU Hours*Interactive Effects kWhkWh*PTO[%]1000 WattkW

=208 W*541 Hours*1.00 kWhkWh*20%1000 WattkW

=22.51 kWh

A summary of the values used for each residential area is shown in Table 9:

Table 9 Calculation Inputs for Residential Measures

| Area | Controlled Wattage (W) | Annual Hours/year | Source | kWh Interactive Effects | PTO |
| --- | --- | --- | --- | --- | --- |
| Residential Living Area (Kitchens, Living Rooms, etc.) | 208 | 541 | Operating Schedule: Res – Interior – Dwelling | Varies by climate zone | 20% |
| Residential Common Area (Laundry Rooms, Clubhouses, etc.) | 113 | 7474 | Operating Schedule: Res – Interior – Common | Varies by climate zone | 20% |
| Residential Bathroom | 208 | 541 | Operating Schedule: Res – Interior – Dwelling | Varies by climate zone | 20% |
| Residential Dwelling Area* | N/A | N/A | N/A | N/A | N/A |

### *The dwelling area values are “N/A” because the dwelling area energy savings are directly calculated as the average of the living area and bathroom area energy savings.

### Non-Residential Market Sector: Guest Room Dwelling Area

Controlled Wattage Assumptions

For guest room dwelling areas, it is assumed that the occupancy sensor controls CFL fixtures and lamps. A typical guestroom layout from the LBNL study “Lighting Energy Savings Opportunities in Hotel Guestrooms” [381] is shown in Figure 1. It is assumed that the entry-way light and the floor lamp/torchiere will be controlled by the occupancy sensor while the table lamps will be manually switched on/off by the occupant. The bathroom light is addressed in another section of this work paper.

Figure 1 Layout of a Typical Guest Room with Fixture Locations

The CFL fixture codes, types, wattages, and total controlled wattage are assumed to be those shown in Table 10:

Table 10 Guest Room Occupancy Controlled Lighting

| Fixture Code* | Location | Number of Controlled Fixtures | Fixture Type* | Fixture Wattage (W)* |
| --- | --- | --- | --- | --- |
| CFQ18/2 | Entry-Way Light | 1 | 2-lamp, 18-Watt, quad, compact fluorescent | 45 |
| CFQ18/4 | Floor Lamp/ Torchiere | 1 | 4-lamp, 18-Watt, quad, compact fluorescent | 90 |
|  |  |  | Total Controlled Wattage: | 135 |

*Obtained from Appendix B Table of Standard Fixture Wattages [382].

Occupancy Sensor Percentage Time Off (PTO)

The Energy Division’s (ED’s) 2013‒2014 Workpaper Disposition for Occupancy Sensor Lighting Controls (Attachment 2, [384]) provided occupancy sensor energy savings (kWh, kW, therms) on a “per kW” basis for several building types. These were multiplied by the controlled wattage of 135 W to obtain savings. The Lodging – Motel building type was used.

The energy savings were calculated as shown in the following example for climate zone 6:

Energy Savings kWh=Controlled Wattage W*ED Disposition SavingskWhkW1000 WattkW

=135 W*626 kWh1000 WatthourkWh

=84.51 kWh

### Non-Residential Market Sector: Guest Room Bathroom Area

The Emerging Technologies (ET) report “ET 04.06 Occupancy Sensor Nightlight Wall Switch for Hotel Guest Room Bathrooms” [383], is used for the source of kWh and kW savings. The ET study took physical measurements in several hotels and saw potential savings for wall sensors installed in guest room bathrooms. The results of the study are summarized in Table 11. Since the results for bathrooms are taken directly from the ET report, interactive effects are not used.

Table 11 ET 04.06 Study Results

| ET 04.06 Measure | Energy Savings (kWh/yr) | Demand Reduction (Peak kW) |
| --- | --- | --- |
| Wall Switch with Occupancy Sensor installed in Hotel/Motel Guest Room Bathrooms | 163 | 0.0159 |

## Complete energy savings calculations are in the Calculation Template (Attachment 1).

## Demand Reduction Methodology

### Residential Market Sector

Using a 20% PTO, the demand reduction was calculated as shown in the following example for the Residential Living Area measure, Multi-family building type, climate zone 6:

Demand Reduction kW=Controlled Wattage W*Interactive Effects kWkW*CDF kWkW*PTO[%]1000 WattkW

=208 W*1.19kWkW*0.043*20%1000 WattkW

=0.00213 kW

The dwelling area demand reduction is calculated as the average of the living area and bathroom area demand reduction.

### Non-Residential Market Sector: Guest Room Dwelling Area

The demand reduction was calculated as shown in the following example for climate zone 6:

Demand Reduction kW=Controlled Wattage W*ED Disposition SavingskWkW1000 WattkW

=135 W*0.092kWkW1000 WattkW

=0.01242 kW

### Non-Residential Market Sector: Guest Room Bathroom Area

As shown in Table 11, the demand reduction is 0.0159 kW.

## Complete demand reduction calculations are in the Calculation Template (Attachment 1).

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available. Therefore, the closest load shape chosen for this measure is the Occupancy Sensor load shape. See Table 12 for a list of all Building Types and Load Shapes. See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 12 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential– Multifamily | Misc._Commercial | Occupancy Sensor |
| Residential– Mobile Home-Double Wide | Misc._Commercial | Occupancy Sensor |
| Lodging - Guestrooms | Hotel_Motel | Occupancy Sensor |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

For Retrofit and Retrofit Add-on measures (REA) the base case cost is assumed to be zero because these are discretionary modifications (retrofit) to the customers’ existing equipment.  The alternative is to make no changes to their existing system.

## 4.2 Measure Case Cost

Because the base case cost is $0/unit, the measure cost is the gross cost. See Section 4.3.1 below for discussion of the measure cost that includes material and labor.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

The gross measure cost is defined as the cost (equipment and labor) incurred to install the energy efficient measure. DEER 2008 contains cost data for occupancy sensors [215]. Although the DEER measure is not exactly the same as any of the measures in this work paper (the quantity and type of controlled fixtures are different), the DEER costs are used.

Table 13 DEER2008 Gross Costs Summary

| Measure | Cost Case Description | DEER08 Cost Case ID | Material Cost | Labor Cost | Gross Measure Cost |
| --- | --- | --- | --- | --- | --- |
| Occupancy Sensor | Assume control 3 2-lamp fixtures w/T8 34W EL Ballast | D08-NE-ILtg-OccSens | $47.15 | $44.12 | $91.27* |

*This cost was comparable to RSMeans Electrical Cost Data 2010 for an automatic wall switch, line number 26 61 13.10 0150.  On page 293, the material and labor cost for an automatic wall switch is $82.35; with overhead the total cost is $97.00 [230].

### 4.3.2 Incremental Measure Cost

For REA, the IMC is the same as the GMC.

# Attachments

# References

| [26] | 2004-2005 Database for Energy Efficiency Resources (DEER) Update Study - Final Report - Itron Inc. - Dec. 2005 |
| --- | --- |
| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| [49] | DEER Database |
| [213] | EUL/RUL Values Provided through Excel Spreadsheet |
| [215] | Revised DEER Measure Cost Summary |
| [230] | 2010 RS Means Electrical Cost Data, 33 Annual Edition |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [381] | Lighting Energy Savings Opportunities in Hotel Guestrooms: Results from a Research Study at the Redondo Beach Crowne Plaza |
| [382] | Appendix B: Table of Standard Fixture Wattages and Sample Lighting Table |
| [383] | Occupancy Sensor Nightlight Wall Switch for Hotel Guest Room Bathrooms |
| [384] | Workpaper Disposition for Lighting Occupancy Sensor Controls |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |