Work Paper SCE13LG025

Revision 2

Southern California Edison Company

Occupancy Sensors, Wall or Ceiling Mounted, Wired, Battery, or Self Powered Wireless

# At-a-Glance Summary

| Applicable Measure Codes: | LT-58209, LT-79132, LT-98724, LT-81984, LT-36254, LT-20964, LT-86543, LT-13779, LT-43578, LT-38102, LT-10867, LT-30078, LT-54667, LT-53465, LT-61726, LT-56854, LT-21331, LT-65677, LT-32443, LT-68999 |
| --- | --- |
| Measure Description: | Wired, Battery-Powered, Self-Powered, Wall or Ceiling Mounted Occupancy Sensors |
| Base Case Description: | No sensors |
| Energy Impact Common Units: | Per kW controlled |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 8 |
| Measure Application Type: | REA |
| Net-to-Gross Ratios: | Deemed: 0.6, Direct Install: 0.89 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG025.0 | No | 04/03/12 | Margaret Pigman/PL Energy | -Original work paper draft for 2013-2014 program cycle. -Updated version of 2010-2012 work paper WPSCNRLG0025. |
| SCE13LG025.1 | Yes | 06/21/13 | Yun Han/ SCE | Updated: - Occupancy time off to ED’s 13-14 work paper disposition - EUL reference to DEER 2011 w/EUL ID - T24 code language to 2013 From 10-12 WP, added: - AB and 3-way switch >500 - Wall Mounted Lighting Sensor >500 - Ceiling Mounted Lighting Sensor >500 - SCs LT-58209, 13779, 43578, 98724, 38102, 54667, 53465 |
| SCE13LG025.2 | Yes | 06/02/14 | Yun Han/SCE | - New template - Some measure common units changed to per kW controlled - Merged SCE13LG107 - WP updated for reporting period, effective 7/1/14-12/31/14 |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the energy and demand impacts of wall or ceiling mounted occupancy sensors in a non-residential building type.  This work paper considers twenty separate measures, shown in Table 1, involving the addition of wall or ceiling mounted occupancy sensors to interior lighting systems that are either wired, battery-powered, or self-powered. The list in Table 1 contains the same measures installed in different room locations and/or applications. This was done on purpose to accommodate various measure configurations within a program’s purchase order and thus different vendor costing.

The sensors are installed to control lighting systems that were previously under manually controlled.  For sensors that control a group of fixtures that draws more than 500 Watts, the base case is ten 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts. According to the SPC Table of Standard fixture Wattages [220], a fixture of this type draws 59 Watts; therefore, the group of ten fixtures draws 590 Watts.

For sensors that control a group of fixtures that draw less than 500 Watts, the base case is three 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts. According to the SPC Table of Standard Fixtures Wattages [220], a fixture of this type draws 59 Watts; therefore the group of three fixtures draws 177 Watts. Measures that do not specify the wattage controlled are assumed to control less than 500 Watts.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-58209 | Wall-Mounted Occupancy Sensor <500 Watts Control per Sensor |
| LT-79132 | Ceiling Mounted Occupancy Sensor <500 Watts Control per Sensor |
| LT-98724 | Wall- or Ceiling-Mounted Occupancy Sensor <500 Watts Control per Sensor |
| LT-81984 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor Control per Sensor |
| LT-36254 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor <500 Watts Control per Sensor |
| LT-20964 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor Watts Control per Sensor |
| LT-86543 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor <500 Watts Control per Sensor |
| LT-13779 | Wall Mounted Lighting Sensor ≥500 Watts Control per Sensor |
| LT-43578 | Ceiling Mounted Lighting Sensor ≥500 Watts Control per Sensor |
| LT-38102 | Wall- or Ceiling-Mounted Occupancy Sensor ≥500 Watts Control per Sensor |
| LT-10867 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor ≥500 Watts Control per Sensor |
| LT-30078 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor ≥500 Watts Control per Sensor |
| LT-54667 | Wall Mounted Lighting Sensor AB Switch ≥500 Watts Control per Sensor |
| LT-53465 | Wall Mounted Lighting Sensor 3 Way Switch ≥500 Watts Control per Sensor |
| LT-61726 | Occupancy Sensor (Small Area) Control per kW Controlled |
| LT-56854 | Occupancy Sensor (Large Area) Control per kW Controlled |
| LT-21331 | Ceiling Mounted Occupancy Sensor (Large Area) Control per kW Controlled |
| LT-65677 | Wall Mounted Occupancy Sensor (Large Area) Control per kW Controlled |
| LT-32443 | Wall Mounted Occupancy Sensor AB Switch (Large Area) Control per kW Controlled |
| LT-68999 | Wall Mounted Occupancy Sensor 3 Way Switch (Large Area) Control per kW Controlled |

The measures in this work paper apply to all Non-Residential building types in SCE climate zones. To qualify for incentives, the customer cannot have an existing occupancy sensor.

## 1.2 Technical Description

A lighting occupancy sensor is a device that automatically turns lights on when a person enters into a room and turns lights off when a specified period of time has passed after an area is vacated.  Thus, energy savings are achieved by reducing the typical run time of the lights.

## 1.3 Measure Application Type

The delivery / incentive mechanisms methods are:

- Financial Support / Down-stream Incentive – Deemed

- Financial Support / Direct Install

- Midstream Programs – Mid-Stream Incentive

The install type is Retrofit Add-on (REA).  This is a subtype of RET that is specifically for adding a piece of equipment to an existing system or piece of equipment to make the overall system or equipment more efficient.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

These measures are included in READI v.2.0.1. However, the measures are limited to certain building types and the common units are expressed in “per kW controlled”.  ED’s Lighting Control Disposition [] provides savings of all the DEER and ED Weighted building types expressed in “per Sensor” which is used for certain measures in this work paper.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not include battery or self-powered sensors, does not specify base case fixtures, and not all building types available. |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mOccSens | Occupancy Sensors | Com | Any | PreRebDown | 0.60 |
| NonRes-sAll-mLtgCtrl-htr | Lighting controls | Com | Any | DirInstall | 0.89 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.60 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor Lighting Controls |
| Measure Case TechGroups | Non-DEER |
| Measure Case TechTypes | hard-wired occupancy sensor, Infrared Sensor |
| Base Case TechGroups | Non-DEER |
| Base Case TechTypes | Non-DEER |

### 1.4.2 Codes and Standards Analysis

Occupancy sensors are not affected by Title 24 2013 [355] due to the following:

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire Modifications-in-Place

Also, if an occupancy sensor is required by or used to comply with code, then it cannot be used to claim energy and demand savings. Therefore, this measure is only eligible in buildings built before July 1, 2014 in which the lighting fixtures are not being replaced.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 141.0(b) | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

No studies were used for this work paper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-OccSens | Non-Residential | Lighting | Occupancy sensors | 8 | 2.67 |

# Section 2. Energy Savings & Demand Reduction Calculations

This work paper considers twenty measures involving the addition of wall- or ceiling-mounted occupancy sensors to interior lighting systems. The different measure types and controlled wattages are shown in Table 8.

Table 8 Measure Types

| Solution Code | Measure Name | Controlled Fixtures | Controlled Wattage |
| --- | --- | --- | --- |
| LT-58209 | Wall-Mounted Occupancy Sensor <500 Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-79132 | Ceiling Mounted Occupancy Sensor <500 Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-98724 | Wall- or Ceiling-Mounted Occupancy Sensor <500 Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-81984 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-36254 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor <500 Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-20964 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-86543 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor <500 Watts Control per Sensor | (3) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 177 W |
| LT-13779 | Wall Mounted Lighting Sensor ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-43578 | Ceiling Mounted Lighting Sensor ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-38102 | Wall- or Ceiling-Mounted Occupancy Sensor ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-10867 | Wall- or Ceiling-Mounted Battery-Powered Wireless Lighting Sensor ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-30078 | Wall- or Ceiling-Mounted Self-Powered Wireless Lighting Sensor ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-54667 | Wall Mounted Lighting Sensor AB Switch ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-53465 | Wall Mounted Lighting Sensor 3 Way Switch ≥500 Watts Control per Sensor | (10) 4’ 2-lamp T8 fixtures (F42ILL) with electronic ballasts | 590 W |
| LT-61726 | Occupancy Sensor (Small Area) Control per kW Controlled | N/A | N/A |
| LT-56854 | Occupancy Sensor (Large Area) Control per kW Controlled | N/A | N/A |
| LT-21331 | Ceiling Mounted Occupancy Sensor (Large Area) Control per kW Controlled | N/A | N/A |
| LT-65677 | Wall Mounted Occupancy Sensor (Large Area) Control per kW Controlled | N/A | N/A |
| LT-32443 | Wall Mounted Occupancy Sensor AB Switch (Large Area) Control per kW Controlled | N/A | N/A |
| LT-68999 | Wall Mounted Occupancy Sensor 3 Way Switch (Large Area) Control per kW Controlled | N/A | N/A |

All savings in this work paper are provided by the Energy Division’s 2013‒2014 Workpaper Disposition for Occupancy Sensor Lighting Controls [A].

To calculate energy savings and demand reduction for ED weighted building types, the Workpaper Disposition provides the occupancy reduction factor for kWh and lighting peak demand reduction factor for kW reduction. These factors are used with the building operating hours, interactive effects, and wattage controlled to come up with the energy savings and demand reduction.

The DEER building types use the “per kW Controlled” value from READI 2.0.1 [] with the wattage controlled to obtain the energy savings and demand reduction.

This workpaper has two types of measures, one expressed in “per kW controlled” and the other in “per sensor”.  In cases where the measure is expressed in “per kW controlled”, the wattage controlled is not factored into the equation.

The non-DEER building types are weighted with other building types as described in the SCE calculation template [C].

Table 9 shows the building types and the source of the occupancy factors used.

Table 9 Occupancy Sensor Percent Time Off by Market Sector

| Building Type | Source | Occupancy Sensor Time Off | Fraction of Usage Reduction |
| --- | --- | --- | --- |
| Agricultural | Non-DEER | Average | Average |
| Assembly | ED Weighted | 20% | 8% |
| Education - Primary School | DEER | 12% | 6% |
| Education - Secondary School | DEER | 11% | 5% |
| Education - Relocatable Classroom | DEER | 10% | 4% |
| Education - Community College | DEER | 11% | 6% |
| Education - University | DEER | 13% | 6% |
| Grocery | ED Weighted | 4% | 2% |
| Food Store | Non-DEER | Average | Average |
| Health/Medical - Hospital | DEER | 11% | 8% |
| Health/Medical - Nursing Home | DEER | 13% | 9% |
| Health/Medical - Clinic | Non-DEER | Average | Average |
| Lodging - Hotel | DEER | 40% | 10% |
| Lodging - Guest Rooms | Non-DEER) | Average | Average |
| Lodging - Motel | DEER | 36% | 7% |
| Manufacturing - Bio/Tech | ED Weighted | 14% | 11% |
| Manufacturing - Light Industrial | ED Weighted | 15% | 11% |
| Industrial | Non-DEER | Average | Average |
| Misc - Commercial | Non-DEER | Average | Average |
| Office - Large | DEER | 30% | 19% |
| Office - Small | DEER | 31% | 19% |
| Restaurant - Fast-Food | ED Weighted | 15% | 14% |
| Restaurant - Sit-Down | ED Weighted | 16% | 16% |
| Retail - Multistory Large | ED Weighted | 3% | 1% |
| Retail - Single-Story Large | ED Weighted | 2% | 1% |
| Retail - Small | ED Weighted | 3% | 1% |
| Storage - Conditioned | ED Weighted | 16% | 6% |
| Storage - Unconditioned | ED Weighted | 16% | 6% |
| Warehouse - Refrigerated | ED Weighted | 15% | 19% |
| Transportation - Communication - Utilities | Non-DEER | Average | Average |

## Energy Savings Methodology for ED Weighted Building Type

## Per kW Controlled

The annual energy savings per kW Controlled is the product of the annual hours of use, energy interactive effects (EIE), and occupancy sensor percent time off (PTO) as shown in Equation 1.

Equation 1

The energy savings vary by solution code, building type, and climate zone because of different values for operating hours, percent time off, and interactive effects. For example, Equation 2 shows the calculation for a sensor controlling less than 500 Watts in an Assembly building located in climate zone 6. The operating hours and interactive effects values were taken from DEER 2014.

Equation 2

## Per Sensor

The annual energy savings is the product of the annual hours of use, energy interactive effects (EIE), occupancy sensor percent time off (PTO), and controlled kW as shown in Equation 1.

Equation 1

The energy savings vary by solution code, building type, and climate zone because of different values for controlled wattage, operating hours, percent time off, and interactive effects. For example, Equation 2 shows the calculation for a sensor controlling less than 500 Watts in an Assembly building located in climate zone 6. The operating hours and interactive effects values were taken from DEER 2013.

Equation 2

## Demand Reduction Methodology for ED Weighted Building Type

## Per kW Controlled

The demand reduction per Watt Controlled is the product of the interactive effects (DIE) and lighting peak demand reduction factor as shown in Equation 5.

Equation 5

As with the energy savings, the demand reduction varies by solution code, building type, and climate zone because of different values for interactive effects and peak demand reduction. For example, Equation 6 shows the calculation for a sensor controlling less than 500 Watts in an Assembly building type located in climate zone 6.

Equation 6

## Per Sensor

The demand reduction per sensor is the product of the controlled kW, interactive effects (DIE), and lighting peak demand reduction factor as shown in Equation 3.

Equation 3

As with the energy savings, the demand reduction varies by solution code, building type, and climate zone because of different values for controlled wattage, interactive effects, and peak demand reduction. For example, Equation 4 shows the calculation for a sensor controlling less than 500 Watts in an Assembly building type located in climate zone 6.

Equation 4

The calculation spreadsheet attached at the end of this document [] shows the calculation for all the permutations of solution code, building type, and climate zone.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the Occupancy Sensor load shape.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | Agricultural | Occupancy Sensor |
| Assembly | Misc._Commercial | Occupancy Sensor |
| Education - Primary School | K_thru_12_School | Occupancy Sensor |
| Education - Secondary School | K_thru_12_School | Occupancy Sensor |
| Education - Relocatable Classroom | K_thru_12_School | Occupancy Sensor |
| Education - Community College | K_thru_12_School | Occupancy Sensor |
| Education - University | K_thru_12_School | Occupancy Sensor |
| Grocery | Misc._Commercial | Occupancy Sensor |
| Food Store | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Hospital | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Nursing Home | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Clinic | Misc._Commercial | Occupancy Sensor |
| Lodging - Hotel | Hotel_Motel | Occupancy Sensor |
| Lodging - Guest Rooms | Hotel_Motel | Occupancy Sensor |
| Lodging - Motel | Hotel_Motel | Occupancy Sensor |
| Manufacturing - Bio/Tech | Industrial | Occupancy Sensor |
| Manufacturing - Light Industrial | Industrial | Occupancy Sensor |
| Industrial | Industrial | Occupancy Sensor |
| Misc - Commercial | Misc._Commercial | Occupancy Sensor |
| Office - Large | Large_Office | Occupancy Sensor |
| Office - Small | Small_Office | Occupancy Sensor |
| Restaurant - Fast-Food | Misc._Commercial | Occupancy Sensor |
| Restaurant - Sit-Down | Misc._Commercial | Occupancy Sensor |
| Retail - Multistory Large | Large_Retail_Store | Occupancy Sensor |
| Retail - Single-Story Large | Large_Retail_Store | Occupancy Sensor |
| Retail - Small | Small_Retail_Store | Occupancy Sensor |
| Storage - Conditioned | Misc._Commercial | Occupancy Sensor |
| Storage - Unconditioned | Misc._Commercial | Occupancy Sensor |
| Transportation - Communication - Utilities | Trans_Comm_Util | Occupancy Sensor |
| Warehouse – Refrigerated | Misc._Commercial | Occupancy Sensor |

# Section 4. Base Case & Measure Costs

Direct Install measures use program tracking systems for measure costs. Full list of costs with climate zone multipliers applied can be found in the attachment [C].

## 4.1 Base Case Cost

For this measure category, the base case cost is assumed to be zero because a piece of equipment is being added to an existing system to make the overall system more efficient.

## 4.2 Measure Case Cost

## Per kW Controlled

The measure cost is the sum of the material cost and the installation labor cost. The DEER08 Revised Measure Cost Summary reports costs based on the size of the space served and per kW controlled [215], as shown in Table 11.

Table 11 Measure Costs for Wired Occupancy Sensors

| Space Size | DEER Cost Case ID | Material Cost per kW | Installation Cost per kW | Measure Cost per kW |
| --- | --- | --- | --- | --- |
| Large | D08-NE-ILtg-OccSens-LgSpace | $110.81 | $110.56 | $221.37 |
| Small | D08-NE-ILtg-OccSens-SmSpace | $234.29 | $110.56 | $344.85 |

The material cost of wireless occupancy sensors and switches/dimmers as one package were taken from various online vendors and averaged [].  The material cost of self-powered occupancy sensors and switches/dimmers as one package was taken from the actual manufacturer’s website. The material cost was divided by 0.59kW for large and 0.177kW for small to normalize the cost to “per kW controlled”. The labor was taken from the DEER 2008 cost spreadsheet [215] as shown in Table 12.

Table 12 Measure Costs for Wireless Occupancy Sensors

| Space Size | Measure | Material Cost per kW | Installation Cost per kW | Measure Cost per kW |
| --- | --- | --- | --- | --- |
| Large | Battery-Powered Sensor with Switch | $347.03 | $110.56 | $457.59 |
| Large | Self-Powered Sensor with Switch | $466.07 | $110.56 | $576.63 |
| Small | Battery-Powered Sensor with Switch | $1156.78 | $110.56 | $1267.34 |
| Small | Self-Powered Sensor with Switch | $1553.56 | $110.56 | $1664.12 |

## Per Sensor

The costs of the sensors Per Sensor takes the costs from Per kW and factors in the kW controlled.

Table 13 Measure Cost per Sensor

| Space Size | Measure | Material Cost per Sensor | Labor Cost per Sensor | Total Cost per Sensor |
| --- | --- | --- | --- | --- |
| Small | Wired | $41.47 | $19.57 | $61.04 |
| Large | Wired | $65.38 | $65.23 | $130.61 |
| Small | Battery-Powered | $204.75 | $19.57 | $224.31 |
| Large | Battery-Powered | $204.75 | $65.23 | $269.98 |
| Small | Self-Powered | $274.98 | $19.57 | $294.55 |
| Large | Self-Powered | $274.98 | $65.23 | $340.21 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

Gross Measure Cost is equal to the Measure Costs shown in Section 4.2.

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is equal to the Measure Costs shown in Section 4.2.

# Attachments

1. 2. 3.

# References

[31]

[213]

[215]

[220]

[351]

[355]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

Attachment 2 - 2013-2014_LightingControl_Disposition-6Sep2013

http://deeresources.com/index.php/deer2013-update-for-2014-codes

Attachment 1- Calculation Template 2015 v3.xlsm

Attachment 3 - Cost.xlsx