Work Paper SCE13LG031

Revision 1

Southern California Edison Company

Linear Fluorescent Dimming Ballasts

# At-a-Glance Summary

| Applicable Measure Codes: | LT-19967, LT-29506, LT-87351, LT-98321 |
| --- | --- |
| Measure Description: | Dimming ballasts with occupancy sensors, 20% tuning, and daylight harvesting |
| Base Case Description: | No controls |
| Energy Impact Common Units: | Ballast |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | DEER: 8 years EUL, 2.7 years RUL |
| Measure Application Type: | RET, ROB |
| Net-to-Gross Ratios: | Downstream: 0.6, Midstream: 0.6, Direct Install: 0.89 |
| Important Comments: | N/A |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE15LG031 Rev 0 | No | 04/11/12 | Jared Kruzek/AESC | Original work paper for 2013 PC |
| SCE15LG031 Rev 1 | Yes | 01/29/14 | Yun Han/SCE | - Work paper updated for reporting period, effective 7/1/2014 – 12/31/2014. - New interactive effects and code language - Updated savings calculator with new occupancy factors; updated savings |

# Section 1. General Measure & Baseline Data

## Measure Description & Background

This work paper documents the energy savings associated with retrofitting T8 and T5 linear fluorescent fixtures with dimming ballasts and various control strategies. These measures save energy and reduce electric demand by controlling the operating hours and power output of linear fluorescent fixtures.

The baselines for these measures are T8 and T5 linear fluorescent fixtures with non-dimming ballasts and without automatic controls.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-87351 | Dimming Ballasts - 20% Tuning |
| LT-98321 | Dimming Ballasts - 20% Tuning + Daylight Harvesting |
| LT-19967 | Dimming Ballasts - 20% Tuning + Occupancy Sensor |
| LT-29506 | Dimming Ballasts - 20% Tuning + Occupancy Sensor + Daylight Harvesting with Side Lighting Photo Controls |

The measures in this work paper apply to all SCE climate zones in the specific building types below.  To qualify for incentives:

- The existing fixtures must be T8 or T5 linear fluorescent fixtures with non-dimming ballasts. Existing T12 fixtures do not qualify.

- Existing controls must be bi-level switching with no other controls.

- The building type must fall into one of the categories below. This work paper only addresses these building types as these are typical buildings types where dimming ballasts are implemented and have been effective in saving energy.

  - Education – Primary: Elementary schools and religions education facilities

  - Education – Secondary: Middle schools, junior high and high schools

  - Education – Relocatable: Mobile classroom

  - Education – Community College: Community colleges, career or vocational training, adult education buildings used for classroom instruction

  - Education – University: College and university buildings used for classroom instruction

  - Office – Large: Government office, bank, city hall, social service, collectively over 100,000 square feet.

  - Office – Small: Daycare or preschool, church, contractor or sales office, collectively under 100,000 square feet

  - Retail – Multistory Large: Department stores

  - Retail – Single-story Large: Big box retail stores, dealerships or showrooms, rental centers

  - Retail – Small: Stores located in strip malls, studio, gallery

  - Miscellaneous Commercial

## 1.2 Technical Description

The definition of each control strategy is as follows:

Dimming Ballasts: Controllable ballasts allow the light and power level of a lamp to be modulated based on area needs.

20% Tuning: A permanent reduction in light output to avoid over-illumination.

Occupancy sensors: Occupancy sensors shut off lights when nobody is present.

Daylight Harvesting and Daylight Harvesting with Side Lighting Photo Controls: Daylight harvesting uses photocells that sense natural sunlight to reduce the area lights at proper levels. Savings value applies to entire day lit area, not just on primary side lit daylight area.

## 1.3 Measure Application Type

The delivery methods that are available for these measures are:

- Financial Support  / Down-Stream Incentive – Deemed

- Partnerships / Down-Stream Incentive – Deemed

- Financial Support / Direct Install

- Partnership / Direct Install

- Midstream Programs / Mid-Stream Incentive

The program/install type for the above measures is:

- Retrofit (RET) – Direct Install

- Replace on Burn-out (ROB) – Downstream & Midstream

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not include combinations of tuning, daylight harvesting, and occupancy sensing |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mLtgCtrl-htr | Lighting controls | Com | Any | DirInstall | 0.89 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | All | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |
| All-Default<=2yrs | All other EEM with no evaluated NTGR; new technology in program for 2 or fewer years | All | Any | All | 0.7 |
| NonRes-sAll-mLtgCtrl | Lighting controls | Com | Any | PreRebDown | 0.6 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template. (In the event that the READi IDs do not support the technology in this workpaper simply indicate “Non-DEER”.)

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor Lighting Controls |
| Measure Case TechGroups | Non-DEER |
| Measure Case TechTypes | Non-DEER |
| Base Case TechGroups | Non-DEER |
| Base Case TechTypes | Non-DEER |

### 1.4.2 Codes and Standards Analysis

In Title 24 2013 [355], Section 141.0(b)2 states:

Lighting System Alterations shall meet the applicable requirements in TABLE 141.0-E and the

following:

- Lighting System Alterations include alterations where an existing lighting system is modified, luminaires are replaced, or luminaires are disconnected from the circuit, removed and reinstalled, whether in the same location or installed elsewhere.

EXCEPTION 1 to Section 141.0(b)2Iii: Alterations that qualify as a Luminaire Modification-in-

Place.

EXCEPTION 2 to Section 141.0(b)2Iii: Portable luminaires, luminaires affixed to moveable

partitions, and lighting excluded in accordance to Section 140.6(a)3.

Luminaire Modifications-in-Place shall meet the applicable requirements in TABLE 141.0-F and the

following:

- To qualify as a Luminaire Modification-in-Place, luminaires shall only be modified by one or more of the following methods:

  - Replacing lamps and ballasts with like type or quantity in a manner that preserves the original luminaire listing.

  - Changing the number or type of light source in a luminaire including: socket renewal, removal or relocation of sockets or lampholders, and/or related wiring internal to the luminaire including the addition of safety disconnecting devices.

  - Changing the optical system of a luminaire in part or in whole.

  - Replacement of whole luminaires one for one in which the only electrical modification involves disconnecting the existing luminaire and reconnecting the replacement luminaire.

- Luminaire Modifications-In-Place shall include only alterations to lighting system meeting the

following conditions:

  - Luminaire Modifications-in-Place shall not be part of or the result of any general remodeling or renovation of the enclosed space in which they are located.

  - Luminaire Modifications-in-Place shall not cause, be the result of, or involve any changes to the panelboard or branch circuit wiring, including line voltage switches, relays, contactors, dimmers and other control devices, providing power to the lighting system.

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition

of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire

Modifications-in-Place

Installation of the controls does not trigger alteration since the controls do not alter the existing lighting system. The controls also do not trigger modification in place due to the exception. Installation of a dimmable ballast does not trigger modification in place since it requires replacing lamps AND ballast.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 141.0(b)2Iiii | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

Daylight Harvesting savings is from various studies compiled together by HMG Consulting [373].

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value, the READI v1.0.4 [] tool was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-OccSens | Non-Residential | Lighting | Occupancy Sensors | 8 | 2.67 |
| GlazDaylt-Dayltg | Non-Residential | Lighting | Daylighting - controls | 8 | 2.67 |

# Section 2. Energy Savings & Demand Reduction Calculations

These measures save energy and reduce peak demand by reducing the power output of the fixtures with dimming ballasts and daylight harvesting. Additional savings are achieved by reducing operating hours of fixtures with occupancy sensors.

20% tuning is a process where the power level of the ballast is reduced by 20%. This is done when the light level from a fixture(s) is greater than required for a given area and can be reduced by reducing the power draw of the ballast. The 20% tuning for this paper is assumed to be a permanent change to the output level either done at the factory or in the field when the ballasts are being installed. Savings are achieved from tuning since the power draw (i.e. the kW) of each ballast is reduced.

Four measures were evaluated in this work paper. The first measure involves the installation of dimming ballasts with 20% tuning. 20% tuning reduces the maximum power output to 80% of the ballast’s capability. The second measure combines the 20% ballast tuning with daylight harvesting (DH). Additional savings are achieved from reducing the light levels based on available daylight. The third measure combines the 20% ballast tuning with occupancy sensors (OS). Occupancy sensors provide further savings by reducing the operating hours of the fixtures. The final measure combines all three control strategies, 20% tuning, daylight harvesting, and occupancy sensors. To account for interactive effects of the modifications, savings are calculated based on the assumption that the 20% tuning will be implemented first, followed by daylight harvesting, and lastly occupancy sensors.

The Dimmable Ballast Tool in the SCE IDSM Online Application Tool (OAT) v4.65 was used to quantify the energy and demand savings, defined in terms of kWh/ballast and kW/ballast respectively, for these measures []. This tool is a preferred calculation tool for the Statewide Customized Offering, and allows with the user to identify a baseline fixture and then calculate savings based on the proposed measure: dimming ballast with 20% tuning, dimming ballast with 20% tuning and daylight harvesting, dimming ballast with 20% tuning and occupancy sensors, or dimming ballast with 20% tuning, daylighting harvesting and occupancy sensors. The tool uses ED specified kWh and kW occupancy sensor reduction factors by climate zones from the latest ED Disposition [] without interactive effects.   The interactive effects are factored in at the end from the list of all savings which can be found in the attachment [].

The building types were limited to educational, office, retail and miscellaneous commercial as these are the targeted building types for the Dimming Ballast Tool.  The area types for each building type were selected as a representative sample so that average energy and demand savings could be calculated for each building type. Because occupancy sensor savings vary by area type, a variety of area types were needed in order to calculate a meaningful average savings. The savings associated with 20% tuning and 20% tuning with daylight harvesting are not affected by room type. The energy and demands savings from each building type were then averaged in order to find an overall average savings for each measure.

The Dimmable Ballast Tool produces results in terms of kWh/year and kW peak demand for the base and measure cases.  Because the tool was run for one fixture, which includes only one ballast, results are presented in specific energy (kWh/ballast) and specific demand (kW/ballast).  An average baseline and measure case specific energy and specific power was calculated for each measure and broken out by building type.

Specific energy savings (kWh/ballast) for each building type were calculated as the difference between the building type average base case specific energy and the building type average measure case specific energy.  Specific power savings were calculated as the difference between the building type base case specific power and the building type measure case specific power. This was done for each of the four measures and for all building types. Table 8 shows the energy and demand savings for the 20% Tuning measure by building type in climate zone 6. Savings for other measures by building type can be found in the attachment [D].

Table 8 20% Tuning Energy and Demand Savings

| 20% Tuning | kWh | kW |
| --- | --- | --- |
| Education - Primary School | 29.40 | 0.01531 |
| Education - Secondary School | 30.86 | 0.01532 |
| Education - Relocatable Classroom | 32.47 | 0.01496 |
| Education - Community College | 32.95 | 0.01515 |
| Education - University | 32.12 | 0.01495 |
| Misc - Commercial | 37.15 | 0.01472 |
| Office - Large | 36.52 | 0.01552 |
| Office - Small | 34.82 | 0.01494 |
| Retail - Multistory Large | 46.39 | 0.01450 |
| Retail - Single-Story Large | 57.83 | 0.01449 |

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DayLt & Controls load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Education – Primary School | K-thru_12_School | DayLt & Controls |
| Education – Secondary School | K-thru_12_School | DayLt & Controls |
| Education – Relocatable Classroom | School | DayLt & Controls |
| Education – Community College | College_University | DayLt & Controls |
| Education – University | College_University | DayLt & Controls |
| Misc- Commercial | Misc._Commercial | DayLt & Controls |
| Office – Large | Large_Office | DayLt & Controls |
| Office – Small | Small_Office | DayLt & Controls |
| Retail – Multistory Large | Large_Retail_Store | DayLt & Controls |
| Retail – Single-story Large | Large_Retail_Store | DayLt & Controls |
| Retail -Small | Small_Retail_Store | DayLt & Controls |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

The dimming ballast is the only equipment in the four measure cases that constitutes a retrofit. For this reason, only a base case cost was calculated for the ballast. The occupancy sensors and daylighting controls are add-on features and as such, the base case cost is assumed to be zero. For the ballast base case cost, no information was available in the DEER database or in RSMeans []. In order to establish a base equipment cost, an average cost per non-dimmable ballast was found from various online sources for an assortment of ballast types and manufacturers []. The average base case cost is $22.13. The labor cost is taken from DEER database. The cost of retrofitting 1 T8 lamp and ballast was used as a conservative approach since DEER does not break out the labor of installing lamp and ballast separately. The table below shows the base case cost for each measure. Since only the ballast contributes to the base case cost, the cost is the same for all measures.

Table 10 Base Case Costs

| Measure name | Base Case Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- |
| Dimming Ballasts – 20% Tuning | $22.13 | $26.51 | $48.64 |
| Dimming Ballasts - 20% Tuning + Daylight Harvesting | $22.13 | $26.51 | $48.64 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor | $22.13 | $26.51 | $48.64 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor + Daylight Harvesting with Side Lighting Photo Controls | $22.13 | $26.51 | $48.64 |

## 4.2 Measure Case Cost

A gross measure cost was found for the dimming ballasts, daylighting controls, and occupancy sensors. In order to calculate a measure cost for dimming ballasts, several online sources were consulted and averaged [] since no information was available in the DEER database or in RSMeans [E]. The average cost for dimming ballast is $70.14. The labor cost of installing the ballast used the same cost of installing the baseline ballast of $26.51.

For occupancy sensors, the DEER Database was consulted. The cost per sensor is $47.15 for material and $44.12 for labor, which gives a measure cost of $91.27 per sensor [49]. According to DEER, each sensor controls three fixtures.  Typically, one ballast serves one fixture. Therefore, the cost per ballast is $30.43.

Since the DEER database did not have cost information available for daylighting controls, 2012 RSMeans was consulted [E]. The total cost per sensor is $131.00 for material and $79.00 for labor, which gives a total measure cost of $210.00 per sensor. RSMeans does not give an estimate of how many fixtures each sensor controls. The 2013-2014 Title 24 Codes and Standards Enhancement (CASE) study on Controllable Lighting [374] includes a cost analysis on daylighting controls. The study assumes the typical number of fixtures served by daylighting controls sensors for three spaces: office, open office and reception.  For an office, the study assumes that five (5) sensors are required for five (5) fixtures. For an open office, the study assumes that two (2) sensors are required for sixteen (16)  fixtures. For a reception area, the study assumes that one (1) sensor is required for four (4) fixtures. To be conservative, the measure cost for daylighting controls assumes one (1) sensor serves eight (8) fixtures, similar to the CASE study’s assumption for open offices.  Therefore, the cost per ballast for each daylight harvesting sensor is $26.26.

Table 11 shows the individual costs of the measures.

Table 11 Individual Costs

| Measure Name | Measure Case Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- |
| Dimming Ballasts – 20% Tuning | $70.14 | Cancels out | $70.14 |
| Daylight Harvesting | $131/8 = $16.38 | $79/8 = $9.88 | $26.26 |
| Occupancy Sensor | $47.15/3 = $15.72 | $44.12/3 = $14.71 | $30.43 |

Table 12 shows the measure case cost per measure. The cost varies for each measure depending on the included equipment.

Table 12 Measure Case Costs

| Measure Name | Measure Case Cost | Labor | Total Cost |
| --- | --- | --- | --- |
| Dimming Ballasts – 20% Tuning | $70.14 | $26.51 | $96.65 |
| Dimming Ballasts - 20% Tuning + Daylight Harvesting | $70.14 + $16.38 = $86.52 | $26.51 + $9.88 = $36.39 | $122.91 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor | $70.14 + $15.72 = $85.86 | $26.51 + $14.71 = $41.22 | $127.08 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor + Daylight Harvesting with Side Lighting Photo Controls | $70.14 + $15.72 + $16.38 = $102.24 | $26.51 + $14.71 + $9.88 = $51.10 | $153.34 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

The gross measure cost (GMC) for RET is represented by the equation below:

GMC = Measure Equipment Cost + Measure Labor Cost

GMC for RET is the same as the measure case cost as shown in Table 12.

The GMC for ROB is represented by the equation below:

GMC 	= Measure Equipment Cost – Base Case Equipment Cost

GMC for ROB is the difference between the measure cost and base case cost as shown in Table 14.

Table 14 Gross Measure Costs for ROB

| Measure Name | Base Case Cost | Measure Case Cost | Gross Measure Cost |
| --- | --- | --- | --- |
| Dimming Ballasts – 20% Tuning | $48.64 | $96.65 | $48.01 |
| Dimming Ballasts - 20% Tuning + Daylight Harvesting | $48.64 | $122.91 | $74.27 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor | $48.64 | $127.08 | $78.44 |
| Dimming Ballasts - 20% Tuning + Occupancy Sensor + Daylight Harvesting with Side Lighting Photo Controls | $48.64 | $153.34 | $104.70 |

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the same as the GMC for ROB as shown in Table 14 of Section 4.3.1.

# Attachments

1. 2. 3. 4.5.

# References

[31]

[351]

[355]

[373]

[374]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

http://deeresources.com/index.php/deer2013-update-for-2014-codes

Attachment 2 - Dimmable Ballast Energy Savings per System Retrofit.xls

Attachment 3 - Occ Sensor Disposition.xlsx

Attachment 1 - Calculation Template 2015 v0.xlsm

2012 RSMeans Electrical Cost Data. 35th Annual Edition.

Attachment 4  – Non Dimming Ballast Cost Documentation

Attachment 5 – Dimming Ballast Cost Documentation