Work Paper SCE13LG076

Revision 2

Southern California Edison Company

Integrated Linear Fluorescent Occupancy Sensor

# At-a-Glance Summary

| Applicable Measure Codes: | LT-76101, LT-41007 |
| --- | --- |
| Measure Description: | Integrated occupancy sensor |
| Base Case Description: | No occupancy sensor |
| Energy Impact Common Units: | Fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 8 years |
| Measure Application Type: | Retrofit Add-on (REA) |
| Net-to-Gross Ratios: | Downstream: 0.60, Direct Install: 0.89, Default: 0.60 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG076.0 | No | 05/24/12 | Sara Richardson /EMCOR Energy Services | Changed install type from RET to REA, updated NTGR, Calculations were revised to include updated DEER hours, IE and CD factors, updated costs according to REA guidelines. |
| SCE13LG076.1 | Yes | 10/17/13 | Jack Melnyk/SCE | Savings updated to ED disposition savings |
| SCE13LG076.2 | Yes | 05/06/14 | Yun Han/SCE | - New WP template. - Work paper updated for reporting period, effective 7/1/2014-12/31/2014 - Code language updated - Added Mid-Stream delivery method |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the E3 calculator input values used to forecast the impacts of fixture integrated occupancy sensors controlling less than 150W and greater than 150W.

The base case fixture for ≥150W is equipped with four 46” 54W T5 HO lamps and electronic ballast without occupancy controls.  The fixture consumes 234 Watts (F44PHL/2) which is obtained from the 2013 SPC Table of Standard Fixture Wattages [431].

The base case fixture for <150W is equipped with two 48” 32W T8 lamps and electronic ballast without occupancy controls.  The fixture consumes 59 Watts (fixture code F42ILL) [431].

The measure case for both cases is to install an integrated occupancy sensor on the base case fixture. One occupancy sensor will control one fixture. Table 1 shows the measures that are included in this work paper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-76101 | Integrated Occupancy Sensor (fixture wattage ≥ 150 Watts) |
| LT-41007 | Integrated Occupancy Sensor (fixture wattage < 150 Watts) |

This work paper is only eligible for fixtures that do not already have an occupancy sensor. The measures apply to all building types and climate zones in an interior space. The customer may be required to present an invoice documenting the installation of measure case equipment.

## 1.2 Technical Description

An integrated occupancy sensor is installed on the fixture itself, in contrast to typical occupancy sensors that are installed on walls or ceilings.

## 1.3 Measure Application Type

The delivery methods for this work paper are:

- Financial Support – Down-stream Incentive - Deemed

- Financial Support – Direct Install

- Midstream Programs – Mid-Stream Incentive

The program/install type for the measures is:

- Retrofit Add-on (REA)

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

These measures are included in READI v.1.0.5. However, the measures are limited to certain building types and the common units are expressed in “per kW controlled”.  ED’s Lighting Control Disposition [] provides savings of all the DEER and ED Weighted building types expressed in both “per kW” and “per Sensor” which is used for this work paper.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain all building types, not the right common unit |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | Occupancy Sensor Pack-200 SF (D03-003) Occupancy Sensor Pack-1000 SF (D03-004) |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mOccSens | Occupancy Sensors | Com | Any | PreRebDown | 0.60 |
| NonRes-sAll-mLtgCtrl-htr | Lighting controls | Com | Any | DirInstall | 0.89 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.60 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor Lighting Controls |
| Measure Case TechGroups | Non-DEER |
| Measure Case TechTypes | hard-wired occupancy sensor |
| Base Case TechGroups | Non-DEER |
| Base Case TechTypes | Non-DEER |

### 1.4.2 Codes and Standards Analysis

Occupancy sensors are not affected by Title 24 2013 [355] due to the following:

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire Modifications-in-Place

Also, if an occupancy sensor is required by or used to comply with code, then it cannot be used to claim energy and demand savings. Therefore, this measure is only eligible in buildings built before 2014 in which the lighting fixtures are not being replaced.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 141.0(b) | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

No studies were used for this work paper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-OccSens | Non-Residential | Lighting | Occupancy sensors | 8 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

This work paper contains two measures as shown in Table 8 with the measure type and controlled wattages.

Table 8 Measure Types

| Solution Code | Measure Name | Controlled Fixtures | Controlled Wattage |
| --- | --- | --- | --- |
| LT-76101 | Integrated Occupancy Sensor (fixture wattage ≥ 150 Watts) | 46” 4-lamp 54W T5 HO with Electronic Ballast (F44PHL/2) | 234 |
| LT-41007 | Integrated Occupancy Sensor (fixture wattage < 150 Watts) | 48” 2-lamp 32W T8 with Electronic Ballast (F42ILL) | 59 |

All savings in this work paper are provided by the Energy Division’s 2013‒2014 Workpaper Disposition for Occupancy Sensor Lighting Controls [A].

To calculate energy savings and demand reduction for ED weighted building types, the Workpaper Disposition provides the occupancy reduction factor for kWh and lighting peak demand reduction factor for kW reduction. These factors are used with the building operating hours, interactive effects, and wattage controlled to come up with the energy savings and demand reduction.

The DEER building types use the “per kW Controlled” value from READI 1.0.5 [] with the wattage controlled to obtain the energy savings and demand reduction.

The non-DEER building types are weighted with other building types as described in the SCE calculation template [C].

Table 9 shows the building types and the source of the occupancy factors used.

Table 9 Occupancy Sensor Percent Time Off by Market Sector

| Building Type | Source | Occupancy Sensor Time Off | Fraction of Usage Reduction |
| --- | --- | --- | --- |
| Agricultural | Non-DEER | Average | Average |
| Assembly | ED Weighted | 0.1991 | 0.0773 |
| Education - Primary School | DEER | READI | READI |
| Education - Secondary School | DEER | READI | READI |
| Education - Relocatable Classroom | DEER | READI | READI |
| Education - Community College | DEER | READI | READI |
| Education - University | DEER | READI | READI |
| Grocery | ED Weighted | 0.0418 | 0.0203 |
| Food Store | Non-DEER | Average | Average |
| Health/Medical - Hospital | DEER | READI | READI |
| Health/Medical - Nursing Home | DEER | READI | READI |
| Health/Medical - Clinic | Non-DEER | Average | Average |
| Lodging - Hotel | DEER | READI | READI |
| Lodging - Guest Rooms | Non-DEER | Average | Average |
| Lodging - Motel | DEER | READI | READI |
| Manufacturing - Bio/Tech | ED Weighted | 0.1402 | 0.1141 |
| Manufacturing - Light Industrial | ED Weighted | 0.1520 | 0.1124 |
| Industrial | Non-DEER | Average | Average |
| Misc - Commercial | Non-DEER | Average | Average |
| Office - Large | DEER | READI | READI |
| Office - Small | DEER | READI | READI |
| Restaurant - Fast-Food | ED Weighted | 0.1500 | 0.1374 |
| Restaurant - Sit-Down | ED Weighted | 0.1555 | 0.1585 |
| Retail - Multistory Large | ED Weighted | 0.0315 | 0.0074 |
| Retail - Single-Story Large | ED Weighted | 0.0207 | 0.0085 |
| Retail - Small | ED Weighted | 0.0300 | 0.0140 |
| Storage - Conditioned | ED Weighted | 0.1600 | 0.0578 |
| Storage - Unconditioned | ED Weighted | 0.1600 | 0.0578 |
| Warehouse - Refrigerated | ED Weighted | 0.1500 | 0.1892 |
| Transportation - Communication - Utilities | Non-DEER | Average | Average |

## Energy Savings Methodology for ED Weighted Building Type

## Per Sensor

The annual energy savings is the product of the annual hours of use, energy interactive effects (EIE), occupancy sensor percent time off (PTO), and controlled kW as shown in Equation 1.

Equation 1

The energy savings vary by solution code, building type, and climate zone because of different values for controlled wattage, operating hours, percent time off, and interactive effects. For example, Equation 2 shows the calculation for a sensor controlling less than 150 Watts in an Assembly building located in climate zone 6. The operating hours and interactive effects values were taken from DEER 2013.

Equation 2

## Demand Reduction Methodology for ED Weighted Building Type

## Per Sensor

The demand reduction per sensor is the product of the controlled kW, interactive effects (DIE), and lighting peak demand reduction factor as shown in Equation 3.

Equation 3

As with the energy savings, the demand reduction varies by solution code, building type, and climate zone because of different values for controlled wattage, interactive effects, and peak demand reduction. For example, Equation 4 shows the calculation for a sensor controlling less than 150 Watts in an Assembly building type located in climate zone 6.

Equation 4

The calculation spreadsheet attached at the end of this document [] shows the calculation for all the permutations of solution code, building type, and climate zone.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the Occupancy Sensor load shape.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | Agricultural | Occupancy Sensor |
| Assembly | Misc._Commercial | Occupancy Sensor |
| Education - Primary School | K_thru_12_School | Occupancy Sensor |
| Education - Secondary School | K_thru_12_School | Occupancy Sensor |
| Education - Relocatable Classroom | K_thru_12_School | Occupancy Sensor |
| Education - Community College | K_thru_12_School | Occupancy Sensor |
| Education - University | K_thru_12_School | Occupancy Sensor |
| Grocery | Misc._Commercial | Occupancy Sensor |
| Food Store | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Hospital | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Nursing Home | Misc._Commercial | Occupancy Sensor |
| Health/Medical - Clinic | Misc._Commercial | Occupancy Sensor |
| Lodging - Hotel | Hotel_Motel | Occupancy Sensor |
| Lodging - Guest Rooms | Hotel_Motel | Occupancy Sensor |
| Lodging - Motel | Hotel_Motel | Occupancy Sensor |
| Manufacturing - Bio/Tech | Industrial | Occupancy Sensor |
| Manufacturing - Light Industrial | Industrial | Occupancy Sensor |
| Industrial | Industrial | Occupancy Sensor |
| Misc - Commercial | Misc._Commercial | Occupancy Sensor |
| Office - Large | Large_Office | Occupancy Sensor |
| Office - Small | Small_Office | Occupancy Sensor |
| Restaurant - Fast-Food | Misc._Commercial | Occupancy Sensor |
| Restaurant - Sit-Down | Misc._Commercial | Occupancy Sensor |
| Retail - Multistory Large | Large_Retail_Store | Occupancy Sensor |
| Retail - Single-Story Large | Large_Retail_Store | Occupancy Sensor |
| Retail - Small | Small_Retail_Store | Occupancy Sensor |
| Storage - Conditioned | Misc._Commercial | Occupancy Sensor |
| Storage - Unconditioned | Misc._Commercial | Occupancy Sensor |
| Transportation - Communication - Utilities | Trans_Comm_Util | Occupancy Sensor |
| Warehouse – Refrigerated | Misc._Commercial | Occupancy Sensor |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

For REA measures there is no base case cost.

## 4.2 Measure Case Cost

The DEER 2008 spreadsheet [215] contains cost information for both occupancy sensors in a large space and a small space. The cost of the sensors per kW Controlled is converted to per Fixture by multiplying the cost by kW controlled.

Table 11 Gross Measure Cost

| Measure | Cost Case ID | Material Cost/kW Controlled | Installation Labor Cost – Retrofit/kW Controlled | kW Controlled | Cost per Fixture (Material + Labor) |
| --- | --- | --- | --- | --- | --- |
| Integrated Occupancy Sensor (fixture wattage ≥ 150 Watts) | D08-NE-ILtg-OccSens-LgSpace | $110.81 | $110.56 | 0.234 | $51.80 |
| Integrated Occupancy Sensor (fixture wattage < 150 Watts) | D08-NE-ILtg-OccSens-SmSpace | $234.29 | $110.56 | 0.059 | $20.34 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For REA measures, the Gross Measure Cost is the Measure Case Cost as shown in Table 11.

### 4.3.2 Incremental Measure Cost

For REA measures, the Incremental Measure Cost is the same as Gross Measure Cost.

# Attachments

1. 2.

# References

[31]

[213]

[215]

[351]

[355]

[431]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

Attachment 2 - 2013-2014_LightingControl_Disposition-6Sep2013

http://deeresources.com/index.php/deer2013-update-for-2014-codes

Attachment 1- Calculation Template 2015 v3.xlsm