Work Paper SCE13LG052

Revision 1

Southern California Edison

LED Channel Letter Signage (Red)

# At-a-Glance Summary

| Measure Codes | LT-74003, LT-68873, LT-10063, LT-48213, LT-98431, LT-67567 |
| --- | --- |
| Measure Description | Red LED Channel Letter Signage |
| Base Case Description | Red Neon/Incandescent Channel Letter Signage |
| Units | Per foot |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | LED-sign: EUL 16yrs, RUL 5.3yrs |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Com-Default>2yrs: 0.6 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/06/12 | James Gowen/Matrix | Updated work paper to new template. |
| 1 | 02/02/16 | Yun Han/SCE | - New template update for 2016 program year - WP effective from 1/1/2016 thru 12/31/2016 - Removed SCE building types - No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the E3 Calculator inputs for replacing incandescent or red neon channel letter sign lighting with red light emitting diode (LED) lighting for both interior and exterior applications.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Red LED Channel Letter Signage |
| Existing Condition | Red Neon/Incandescent Channel Letter Signage |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-74003 |  | <2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage |
|  |  | LT-68873 |  | <2 ft Channel Letter Signage (Red) (24 Hour) LED replacing Neon Channel Letter Signage |
|  |  | LT-10063 |  | <2 ft Channel Letter Signage (Red) (Dusk to Dawn) LED replacing Neon Channel Letter Signage |
|  |  | LT-48213 |  | >2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage |
|  |  | LT-98431 |  | >2 ft Channel Letter Signage (Red) (24 Hour) LED replacing Neon Channel Letter Signage |
|  |  | LT-67567 |  | >2 ft Channel Letter Signage (Red) (Dusk to Dawn) LED replacing Neon Channel Letter Signage |

This measure only applies to red LED systems because there is minimal to negative savings for other LED light colors.

## 1.2 Technical Description

Channel letter signs are individual internally illuminated letters and graphics. The signs are comprised of a U-shaped channel and an acrylic face inside of which the light source is typically neon and sometimes incandescent. In this measure, the base case neon and/or incandescent source is to be replaced with an LED system.

## 1.3 Installation Types and Delivery Mechanisms

The Installation Type is Replace on Burnout (ROB).

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |

The delivery methods for all Solution Codes are:

- Financial Support/Down-Stream Incentive – Deemed

Solution Codes LT-74003, LT-10063, LT-48213, and LT-67567 also include:

- Mid-stream Program/Mid-stream Incentive

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | N/A |
| Reason for Deviation from DEER | DEER does not contain this type of measure. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| LED-sign | LED Open Signs | Com | Lighting | 16 | 5.3 |

### 1.4.2 Codes and Standards Analysis

Title 24 2008 Section 148 [65] contains codes related to externally/internally illuminated signs for indoor and outdoor:

Mandatory measures

The Standards require that indoor and outdoor sign lighting be automatically controlled so that it is turned off during daytime hours and during other times when it is not needed. In brief, the mandatory sign lighting requirements include:

- Automatic shutoff controls,

- Dimming controls, and

- Demand responsive controls for electronic message centers

Sign Lighting Power Requirements

Sign Lighting Standards apply to both indoor and outdoor signs and contain two different prescriptive compliance options: Specific technology and watts per square foot approaches. The watt per square foot approach specifies a maximum lighting power that can be installed, expressed in W/ft² of sign area. The specific technology approach specifies that the signs shall be illuminated with efficient lighting sources (electronic ballasts, high efficacy lamps, efficient power supplies and efficient transformers).

For this measure, the LED channel letter signs will require controls to comply with Title 24. These signs already comply with the Sign Lighting Power Requirements because LEDs are specified as an efficient lighting source.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2008) | 2008 Non-Residential Compliance Manual, Section 148 | January 1, 2010 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

### 1.5.1 ET06.12 - LED Channel Letter Lighting

- Author: Vireak Ly

- Completed 12/16/2008

- Commercial Market covered

- Techniques used: Monitoring/measurement

## 1.6 Data Quality and Future Data Needs

Updated study is needed for colors other than Red.

# Section 2. Calculation Methodology

The annual energy savings is the product of the demand reduction per linear foot of sign (W/ft) and the annual sign operating hours. The annual operating hours are based on DEER exterior lighting measures, which have an annual run-time of 4,100 hours [49]. Exterior lighting hours of operation are a very good indicator of exterior sign hours of operation. For exterior measures (LT-10063 and LT-67567), energy interactive effects are not included in the energy savings calculation. The exterior annual energy savings is calculated as shown in Equation 1.

Equation 1

Annual Energy savings (kWh / ft- year)

= (Demand Reduction [kW/ft of sign]) x (Annual Operating Hours)

Equation 2 shows an example calculation for exterior lighting:

Equation 2

Annual Energy savings = (0.01716 kW/ft) x (4100 hrs/yr) =70.36 kWh/ft-yr

Interior Signs:

The annual energy savings is the product of the demand reduction per linear foot of sign (W/ft) and the annual operating hours and energy interactive effects. The interior annual energy savings are calculated as shown in

Equation 3.

Equation 3

Annual Energy savings (kWh/ft- year)

= (Demand Reduction [kW/ft of sign]) x (Annual Operating Hours) x (Energy Interactive Effects)

The annual operating hours and energy interactive effects vary with building type for interior applications. Equation 4 shows an example calculation for the Grocery market sector in climate zone 6, for a one-foot sign:

Equation 4

Annual Energy Savings = 0.01716 kW × 4,910 hours/year × 1.0137 = 85.41 kWh/ft-yr

## Demand Reduction

The demand reduction estimates use the “per linear foot” wattages obtained from major channel letter lighting product manufacturers. Table below shows the average values from manufacturer materials.

Manufacturer Data

| Technology | Wattage | Source |
| --- | --- | --- |
| Neon | 5.318 | GE Lumination |
| Neon | 4.2 | Sloan LED |
| Neon | 4.2 | LSI Industries |
| LED | 0.87 | GE Lumination |
| LED | 1.8 | Sloan LED |
| LED | 1.2 | LSI Industries |
| LED | 1.2 | Permlight |
| Neon Average |  | 4.57 |
| LED Average |  | 1.27 |

The demand reduction per linear foot of neon replaced is the difference between the neon and LED wattages, as shown in Equation 5.

Equation 5

Demand Reduction (W/ foot) = (Base Case W/ft) – (LED W/ft)

= (4.57 W/ft) – (1.27 W/ft) = 3.30 Watts/ft. or 0.003 kW/ft.

The entire length of the sign is obtained by:

- Measuring the width of each individual letter without measuring the distance between letters, and

- Adding up the measurements of each individual letter to get the length of the entire sign being replaced.

The average length of neon per foot of letter is dependent on many variables, such as how long the neon stroke length is for each letter, how often the letter occurs, and how wide the letter is. The stroke length per letter is estimated using a simple LED alphanumeric display module. The height of the letter is assumed to be two units high and one unit wide. Figure below shows a few examples of how the stroke length is estimated.

Example Stroke Length of 5, 4, and 5, Respectively

The stroke length per letter width is calculated by dividing the stroke length for each letter by its width. All letters are assumed to have one unit width except the letter “I.”

Table below shows the weighted stroke length per letter width and average stroke length per letter length.

Average Stroke Length per Letter Length

| Letter | Letter Frequency* | Stroke Length per Letter** | Letter Width | Stroke Length per Width | Weighted Stroke Length per Letter Width |
| --- | --- | --- | --- | --- | --- |
| A | 8.17% | 6 | 1 | 6 | 0.490 |
| B | 1.49% | 7 | 1 | 7 | 0.104 |
| C | 2.78% | 4 | 1 | 4 | 0.111 |
| D | 4.25% | 5 | 1 | 5 | 0.213 |
| E | 12.70% | 5 | 1 | 5 | 0.635 |
| F | 2.23% | 4 | 1 | 4 | 0.089 |
| G | 2.02% | 6 | 1 | 6 | 0.121 |
| H | 6.09% | 5 | 1 | 5 | 0.305 |
| I | 6.97% | 2 | 0.5 | 4 | 0.279 |
| J | 0.15% | 4 | 1 | 4 | 0.006 |
| K | 0.77% | 5 | 1 | 5 | 0.039 |
| L | 4.03% | 3 | 1 | 3 | 0.121 |
| M | 2.41% | 8 | 1 | 8 | 0.192 |
| N | 6.75% | 6 | 1 | 6 | 0.405 |
| O | 7.51% | 6 | 1 | 6 | 0.450 |
| P | 1.93% | 5 | 1 | 5 | 0.096 |
| Q | 0.10% | 6 | 1 | 6 | 0.006 |
| R | 5.99% | 6 | 1 | 6 | 0.359 |
| S | 6.33% | 6 | 1 | 6 | 0.380 |
| T | 9.06% | 4 | 1 | 4 | 0.362 |
| U | 2.76% | 5 | 1 | 5 | 0.138 |
| V | 0.98% | 4 | 1 | 4 | 0.039 |
| W | 2.36% | 8 | 1 | 8 | 0.189 |
| X | 0.15% | 4 | 1 | 4 | 0.006 |
| Y | 1.97% | 3 | 1 | 3 | 0.059 |
| Z | 0.07% | 4 | 1 | 4 | 0.003 |
| Average Stroke Length per Letter Length |  |  |  |  | 5.20 |

*Relative frequencies of letters in the English language reference:

http://en.wikipedia.org/wiki/Letter_frequencies

The demand reduction per foot of letter signage is determined by multiplying the demand reduction per linear foot of neon replaced by the average number of feet per letter, as shown in

Equation 6.

Equation 6

Gross Demand Reduction (W/ft of sign) =Demand Reduction (W/ft neon) x (Avg. feet/letter width)

Where:

- Demand reduction per linear foot of neon replaced is 3.30 W/ft neon, as shown in Equation 5.

- Average number of feet of neon per letter was calculated as 5.20 ft, as shown in Table above.

The actual calculation is shown in Equation 7:

Equation 7

Gross Demand Reduction (W/ft of sign) = (3.30 W/ft neon) x (5.20 ft neon/letter width)

=17.2 W/ft of sign or 0.017 kW/ft of sign

Exterior Signs:

The gross demand reduction is used only to determine the annual energy savings. The peak demand reduction is zero, as these exterior lighting applications are on at night and have a peak coincidence factor of 0.0. Channel letter signs do not have to be on to convey a message. This is unlike direct-view signs, such as message boards, which have to be on to convey a message.

Interior Signs:

Coincident peak demand savings takes into account the degree to which the demand savings coincides with peak demand. It is the product of the gross demand savings, coincident diversity factor (CDF), and demand interactive effects (DIE), as shown in Equation 8.

Equation 8

Coincident Peak demand savings [kW/unit] = Gross Demand Reduction × (CDF) × (DIE)

Equation 9 shows the calculation for the Restaurant- Sit-Down market sector for a one-foot sign,

Equation 9

Coincident Peak demand savings [kW/unit] = 0.017 kW/foot × (0.8) × (1.23)

= 0.017 kW/foot or 17 Watts/ft

A complete list of energy savings and demand reduction for other building types and dusk-to-dawn/24 hour operations can be found in the Calculations Template [A].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Exterior Load Shape | Exterior E3 Alternate Building Type | Interior Load Shape | Interior E3 Alternate Building Type |
| --- | --- | --- | --- | --- |
| Assembly | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Community College | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Primary School | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Secondary School | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - University | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Grocery | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Hospital | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Hotel | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Motel | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Large | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Small | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Multistory Large | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Single-Story Large | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Small | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Conditioned | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Unconditioned | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | Outdoor Lt | Misc._Commercial | DEER:Indoor_Non-CFL_Ltg | NON_RES |

# Section 4. Costs

## 4.1 Base Case Cost

Base case not available in this revision. Rev-up from previous program year. Will be available in next revision.

## 4.2 Measure Case Cost

As the overall size of a channel letter grows, at some point it becomes necessary to add a second, parallel row of LEDs within the channel to provide for consistent illumination across the face of the letter. While the letter size at which this occurs varies somewhat due to a number of factors, two feet is generally considered the rough break point at which the number of LEDs per letter effectively doubles. For this reason, it’s appropriate to present material costs both for letters less than two feet and those two feet or higher.

The cost shown in the Table below was derived by taking the average cost of contractors [B] and engineering estimates [C]. Note that contractor naming was abbreviated as their bids are considered confidential.

Measure Cost

| Contractor | Measure Name | Total Cost Per Letter |
| --- | --- | --- |
| M.A. Inc. | <2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage | $217.00 |
| A.S | <2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage | $205.00 |
|  | Average Cost < 2 ft | $211.00 |
| M.A. Inc. | >2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage | $539.33 |
| A.S | >2 ft Channel Letter Signage (Red) LED replacing Neon Channel Letter Signage | $386.67 |
|  | Average Cost > 2 ft | $463.00 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-74003 | ROB | $211.00 | $211.00 | N/A |
| LT-68873 | ROB | $211.00 | $211.00 | N/A |
| LT-10063 | ROB | $211.00 | $211.00 | N/A |
| LT-48213 | ROB | $463.00 | $463.00 | N/A |
| LT-98431 | ROB | $463.00 | $463.00 | N/A |
| LT-67567 | ROB | $463.00 | $463.00 | N/A |

# Attachments

1. 2. 3.  4.  5.

# References

[49]

[65]

[A] Attachment 1 – Calculation Template v2.1.xlsm

[B] Attachment 2 – LED Channel Letter Signage Estimate 1.pdf

[C] Attachment 3 – LED Channel Letter Signage Estimate 2.pdf

[D] Attachment 4 – LED Channel Letter Signage Super Flux Module Series.pdf

[E] Attachment 5 – LED Channel Letter Signage Report.pdf