Work Paper SCE13LG087

Revision 1

Southern California Edison Company

Fluorescent Fixture to Fluorescent Fixture Retrofit

# At-a-Glance Summary

| Applicable Measure Codes: | Refer to Excel Calc. Sheet |
| --- | --- |
| Measure Description: | T5 or T8 Linear Fluorescent Fixture |
| Base Case Description: | T12, T8, or T5 Linear Fluorescent Fixture |
| Energy Impact Common Units: | Per Fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | 15 years or less |
| Measure Application Type: | Early Retirement (RET), Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Downstream T5&T8 Commercial: 0.7, rest 0.6, Midstream: 0.6, Direct Install T8: 0.89, rest 0.85 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG087.0 | No | 06/05/12 | Yun Han/SCE | - Updated to latest WP v0.1 from WPSCNRLG0087.8 - Created 117 new measures in READI - Costs added for DI program |
|  |  |  | Brian V. O’Keefe | - Updated to latest WP v0.4 and calculation template v2.2 |
| SCE13LG087.1 | Yes | 06/30/14 | Yun Han/SCE | - Removed duplicate measure LT-49832 - NTGs updated - Work paper updated for reporting period, effective 7/1/2014-12/31/2014 - Code baseline updated to ED’s May 30th 2014 disposition - Cost updated to the correct references and fixed “per-lamp” costs |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of existing linear fluorescent lamp(s) and ballast(s) with more efficient linear fluorescent lamp(s) and ballast(s).

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-61312 | (1) 24in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) |
| LT-46109 | (1) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) |
| LT-18943 | (1) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent |
| LT-33633 | (1) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent |
| LT-83912 | (1) 36in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent |
| LT-18642 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T5 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) |
| LT-12877 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) |
| LT-94378 | (1) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) |
| LT-20394 | (1) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-30495 | (1) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent |
| LT-79688 | (1) 46in (1) Programmed Start Tandem 2 Lamp Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-94841 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent |
| LT-30612 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) |
| LT-50439 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent |
| LT-39815 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) |
| LT-40596 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent |
| LT-82210 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) |
| LT-43895 | (1) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent |
| LT-50998 | (1) 48in (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-89585 | (1) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) |
| LT-14830 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent |
| LT-98664 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) |
| LT-40989 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-67100 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) |
| LT-71023 | (1) 48in Reduced 25 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent |
| LT-69594 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent |
| LT-27597 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) |
| LT-50963 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-84012 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) |
| LT-89708 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent |
| LT-91014 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-10436 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-24444 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-85932 | (1) 48in Reduced 28 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent |
| LT-58535 | (1) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent |
| LT-20983 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent |
| LT-58109 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) |
| LT-48392 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-78895 | (2) 22in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent |
| LT-59482 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent |
| LT-83726 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-90213 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent |
| LT-54637 | (2) 24in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent |
| LT-60969 | (2) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent |
| LT-91429 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-19002 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent |
| LT-26889 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-32000 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent |
| LT-73521 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-50395 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent |
| LT-25668 | (2) 34in (1) Premium Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent |
| LT-98333 | (2) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent |
| LT-43018 | (2) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent |
| LT-73491 | (2) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) |
| LT-68385 | (2) 46in (1) Instant Start Ballast - Normal Light Output w/ Dimming T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-69232 | (2) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-78696 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-89685 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-19843 | (2) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent |
| LT-60902 | (2) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent |
| LT-93723 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-37676 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-39875 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-49843 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-78548 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-58359 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-19832 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Dimming T8 Linear Fluorescent replacing (1) 96in HO T12 Linear Fluorescent |
| LT-38503 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ DR Dimming Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-18545 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-29436 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-68384 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-40696 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-18409 | (2) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent |
| LT-19210 | (2) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-24981 | (2) 48in (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-90075 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-73849 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-38546 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-49810 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-49587 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-20796 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-58327 | (2) 48in (2) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-98587 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-15425 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 RLO Linear Fluorescent |
| LT-21054 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-60495 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) |
| LT-10511 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-25867 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-32845 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-45291 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-57432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-87326 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-70093 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) |
| LT-29908 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-69821 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-78193 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-83945 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-90462 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-32645 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent |
| LT-29023 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-30613 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-40921 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-50734 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-60432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-71214 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-89122 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-18934 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent |
| LT-24985 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-30765 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-43532 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-57486 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-65784 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-79584 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-93333 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-85346 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-93656 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-88743 | (2) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent |
| LT-19844 | (2) 96in (1) Instant Start Ballast - Normal Light Output - HO T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-94443 | (2) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |
| LT-94323 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in HO T12 Linear Fluorescent |
| LT-27685 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-80796 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-10494 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-24162 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-39678 | (2) U-Tube (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent |
| LT-40789 | (3) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 24in T12 Linear Fluorescent |
| LT-38675 | (3) 46in (2) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-23738 | (3) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent |
| LT-75850 | (3) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent |
| LT-49588 | (3) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-50379 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-60092 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-49683 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-50968 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-79821 | (3) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent |
| LT-38219 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-89032 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) |
| LT-32378 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-83248 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-40056 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) |
| LT-37543 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-10154 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent |
| LT-23675 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-34867 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-40982 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-18592 | (3) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-35845 | (3) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-59462 | (4) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent |
| LT-45382 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent |
| LT-29478 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 36in T12 Linear Fluorescent |
| LT-50488 | (4) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-38493 | (4) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent |
| LT-84940 | (4) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent |
| LT-68578 | (4) 48in (1) Instant Start Ballast - Reduced Light Output (Kit) T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-89871 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-99866 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-85958 | (4) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent |
| LT-59103 | (4) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent |
| LT-78327 | (4) 48in (2) Instant Start Ballast - Reduced Light Output (New Fixture) T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-20483 | (4) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-89782 | (4) 48in (2) Instant Start Ballast - Reduced Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent |
| LT-45842 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-36789 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) |
| LT-45732 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-95485 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-57859 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) |
| LT-68712 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) |
| LT-59655 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent |
| LT-19235 | (4) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |
| LT-59685 | (6) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |
| LT-39058 | (6) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-91623 | (6) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent |
| LT-69544 | (6) 48in (2) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |
| LT-49785 | (6) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent |
| LT-18743 | (8) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |
| LT-54329 | (8) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent |

## 1.2 Technical Description

Linear fluorescent lamps and ballasts replacements in this workpaper are expressed in “fixture” common units, although the actual fixtures are not being replaced.

## 1.3 Measure Application Type

The delivery method that is available for these measures are:

- Financial Support - Down-Stream Incentive – Deemed, in which the incentive is paid directly to the customer

- Financial Support – Direct Install, in which the measure is installed at the customer’s facility by program contractors

- Midstream Programs – Mid-stream Incentive, in which the incentive is paid to the distributor

The install types are:

- Retrofit (RET) for direct install

- Replace-on-Burnout (ROB) for Downstream and Midstream

For direct install measures SCE is currently claiming full customer savings for the RUL as the measures are being implemented as early retirement measures.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain specific base case and measure equipment |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mT5T8-dn | T5 and T8 lamps | Com | Any | PreRebDown | 0.7 |
| NonRes-sAll-mT8-dir | T8 lamps | Com | Any | DirInstall | 0.89 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.6 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Com-LF-SCE | Non-Res Linear Fluorescent fixture; Annual Installation Rate | Com | Any | NonUpStrm | 0.95 |
| MFm-LF-SCE | Linear Fluorescent fixture; Annual Installation Rate; Multi-family | Res | Mfm | NonUpStrm | 0.77 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | InGen, InCommon |
| Measure Case TechGroups | Ltg_Lmp+Blst |
| Measure Case TechTypes | LF_LmpBlst |
| Base Case TechGroups | Ltg_Lmp+Blst |
| Base Case TechTypes | LF_LmpBlst |

### 1.4.2 Codes and Standards Analysis

In Title 24 2013 [355], Section 141.0(b)2 states:

Lighting System Alterations shall meet the applicable requirements in TABLE 141.0-E and the

following:

- Lighting System Alterations include alterations where an existing lighting system is modified, luminaires are replaced, or luminaires are disconnected from the circuit, removed and reinstalled, whether in the same location or installed elsewhere.

EXCEPTION 1 to Section 141.0(b)2Iii: Alterations that qualify as a Luminaire Modification-in-

Place.

EXCEPTION 2 to Section 141.0(b)2Iii: Portable luminaires, luminaires affixed to moveable

partitions, and lighting excluded in accordance to Section 140.6(a)3.

Luminaire Modifications-in-Place shall meet the applicable requirements in TABLE 141.0-F and the

following:

- To qualify as a Luminaire Modification-in-Place, luminaires shall only be modified by one or more of the following methods:

  - Replacing lamps and ballasts with like type or quantity in a manner that preserves the original luminaire listing.

  - Changing the number or type of light source in a luminaire including: socket renewal, removal or relocation of sockets or lampholders, and/or related wiring internal to the luminaire including the addition of safety disconnecting devices.

  - Changing the optical system of a luminaire in part or in whole.

  - Replacement of whole luminaires one for one in which the only electrical modification involves disconnecting the existing luminaire and reconnecting the replacement luminaire.

- Luminaire Modifications-In-Place shall include only alterations to lighting system meeting the

following conditions:

  - Luminaire Modifications-in-Place shall not be part of or the result of any general remodeling or renovation of the enclosed space in which they are located.

  - Luminaire Modifications-in-Place shall not cause, be the result of, or involve any changes to the panelboard or branch circuit wiring, including line voltage switches, relays, contactors, dimmers and other control devices, providing power to the lighting system.

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition

of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire

Modifications-in-Place

Title 20 2014 [422] includes regulations to fluorescent lamp ballasts, replacement fluorescent lamp ballasts, and lamps.

2012 Federal Standards for General Service Fluorescent Lamps issued by Department of Energy effective July 14, 2012 contains Energy Conservation Standards that applies to various linear fluorescent lamp types [].

This workpaper uses standard and code wattage values given by the ED Disposition [C] which are assumed to be within the guidelines of T24, T20, and NEMA standards. T24 Modifications-in-Place is triggered due to lamp & ballast replacements and de-lamping.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 141.0(b)2Iiii | July 1, 2014 |
| Title 20 (2014) | 2014 Appliance Efficiency Regulations | July 1, 2014 |
| NEMA (2012) | Federal standards for general service fluorescent lamps issued by DOE | July 14, 2012 |

### 1.4.3 Non-DEER Study Review

N/A.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436] was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Lfluor-Elec, ILtg-T5 | Non-Residential | Lighting | Linear Fluorescent with Electronic Ballast | EUL varies by bldg type EUL = Rated Life of Ballast (70,000 hours) / Annual usage for building type (usage provided by DEER) OR 15 years (whichever is less). | RUL = (1/3)EUL |

See the attachment [] for full list of EULs and RULs for different building types.

# Section 2. Energy Savings & Demand Reduction Calculations

Table 8 lists the base and measure wattages and provided by Energy Division’s Workpaper Disposition for Lighting Retrofits (30May2014 []).

Table 8 List of Base and Measure Wattages

| Solution Code | Measure name | Measure | Customer Average | Code | 1st Baseline Delta | 2nd Baseline Delta |
| --- | --- | --- | --- | --- | --- | --- |
| LT-61312 | (1) 24in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) | 34 | 56 | 56 | 22 | 22 |
| LT-46109 | (1) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) | 33 | 56 | 56 | 23 | 23 |
| LT-18943 | (1) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent | 14 | 28 | 28 | 14 | 14 |
| LT-33633 | (1) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent | 26 | 46 | 46 | 20 | 20 |
| LT-83912 | (1) 36in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent | 27 | 46 | 46 | 19 | 19 |
| LT-18642 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T5 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) | 48 | 66 | 66 | 18 | 18 |
| LT-12877 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) | 46 | 66 | 66 | 20 | 20 |
| LT-94378 | (1) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | 64 | 72 | 64 | 8 | 0 |
| LT-20394 | (1) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 62 | 72 | 62 | 10 | 0 |
| LT-30495 | (1) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 33 | 43 | 33 | 10 | 0 |
| LT-79688 | (1) 46in (1) Programmed Start Tandem 2 Lamp Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 59 | 72 | 59 | 13 | 0 |
| LT-94841 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent | 49 | 54 | 54 | 5 | 5 |
| LT-30612 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) | 106 | 116 | 116 | 10 | 10 |
| LT-50439 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent | 51 | 54 | 54 | 3 | 3 |
| LT-39815 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) | 109 | 116 | 116 | 7 | 7 |
| LT-40596 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 31 | 43 | 31 | 12 | 0 |
| LT-82210 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | 54 | 72 | 60 | 18 | 6 |
| LT-43895 | (1) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 25 | 41 | 31 | 16 | 6 |
| LT-50998 | (1) 48in (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 41 | 72 | 60 | 31 | 19 |
| LT-89585 | (1) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | 31 | 43 | 31 | 12 | 0 |
| LT-14830 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | 27 | 31 | 31 | 4 | 4 |
| LT-98664 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) | 27 | 31 | 31 | 4 | 4 |
| LT-40989 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) | 27 | 31 | 31 | 4 | 4 |
| LT-67100 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) | 45 | 59 | 59 | 14 | 14 |
| LT-71023 | (1) 48in Reduced 25 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 28 | 43 | 31 | 15 | 3 |
| LT-69594 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | 28 | 31 | 31 | 3 | 3 |
| LT-27597 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) | 27 | 31 | 31 | 4 | 4 |
| LT-50963 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) | 27 | 31 | 31 | 4 | 4 |
| LT-84012 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) | 53 | 59 | 59 | 6 | 6 |
| LT-89708 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | 24 | 31 | 31 | 7 | 7 |
| LT-91014 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 31 | 59 | 59 | 28 | 28 |
| LT-10436 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 28 | 59 | 59 | 31 | 31 |
| LT-24444 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 37.4 | 59 | 59 | 21.6 | 21.6 |
| LT-85932 | (1) 48in Reduced 28 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 27 | 43 | 31 | 16 | 4 |
| LT-58535 | (1) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent | 58 | 76 | 76 | 18 | 18 |
| LT-20983 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent | 49 | 62 | 55 | 13 | 6 |
| LT-58109 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) | 98 | 123 | 109 | 25 | 11 |
| LT-48392 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 57 | 123 | 109 | 66 | 52 |
| LT-78895 | (2) 22in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 34 | 56 | 56 | 22 | 22 |
| LT-59482 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 33 | 56 | 56 | 23 | 23 |
| LT-83726 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 33 | 72 | 59 | 39 | 26 |
| LT-90213 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent | 33 | 112 | 112 | 79 | 79 |
| LT-54637 | (2) 24in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 31 | 56 | 56 | 25 | 25 |
| LT-60969 | (2) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 29 | 56 | 56 | 27 | 27 |
| LT-91429 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 35 | 72 | 60 | 37 | 25 |
| LT-19002 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | 35 | 60 | 60 | 25 | 25 |
| LT-26889 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 31 | 72 | 60 | 41 | 29 |
| LT-32000 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | 31 | 60 | 60 | 29 | 29 |
| LT-73521 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 31 | 72 | 60 | 41 | 29 |
| LT-50395 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | 31 | 60 | 60 | 29 | 29 |
| LT-25668 | (2) 34in (1) Premium Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent | 48 | 66 | 66 | 18 | 18 |
| LT-98333 | (2) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent | 46 | 76 | 76 | 30 | 30 |
| LT-43018 | (2) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent | 46 | 66 | 66 | 20 | 20 |
| LT-73491 | (2) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) | 54 | 123 | 109 | 69 | 55 |
| LT-68385 | (2) 46in (1) Instant Start Ballast - Normal Light Output w/ Dimming T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 72 | 144 | 108 | 72 | 36 |
| LT-69232 | (2) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 117 | 144 | 118 | 27 | 1 |
| LT-78696 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 64 | 72 | 64 | 8 | 0 |
| LT-89685 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 64 | 115 | 64 | 51 | 0 |
| LT-19843 | (2) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent | 106 | 116 | 116 | 10 | 10 |
| LT-60902 | (2) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent | 109 | 116 | 116 | 7 | 7 |
| LT-93723 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 64 | 115 | 89 | 51 | 25 |
| LT-37676 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 79 | 144 | 90 | 65 | 11 |
| LT-39875 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 79 | 144 | 90 | 65 | 11 |
| LT-49843 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 54 | 115 | 89 | 61 | 35 |
| LT-78548 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 54 | 115 | 89 | 61 | 35 |
| LT-58359 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 54 | 144 | 112 | 90 | 58 |
| LT-19832 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Dimming T8 Linear Fluorescent replacing (1) 96in HO T12 Linear Fluorescent | 70 | 112 | 80 | 42 | 10 |
| LT-38503 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ DR Dimming Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 59 | 144 | 108 | 85 | 49 |
| LT-18545 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 54 | 115 | 89 | 61 | 35 |
| LT-29436 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 54 | 144 | 108 | 90 | 54 |
| LT-68384 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 54 | 115 | 89 | 61 | 35 |
| LT-40696 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 59 | 144 | 112 | 85 | 53 |
| LT-18409 | (2) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent | 54 | 62 | 60 | 8 | 6 |
| LT-19210 | (2) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 59 | 72 | 60 | 13 | 1 |
| LT-24981 | (2) 48in (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 65 | 123 | 109 | 58 | 44 |
| LT-90075 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 54 | 72 | 70 | 18 | 16 |
| LT-73849 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 54 | 144 | 112 | 90 | 58 |
| LT-38546 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 54 | 72 | 70 | 18 | 16 |
| LT-49810 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 45 | 72 | 59 | 27 | 14 |
| LT-49587 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48 | 115 | 83 | 67 | 35 |
| LT-20796 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48 | 144 | 108 | 96 | 60 |
| LT-58327 | (2) 48in (2) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 54 | 115 | 89 | 61 | 35 |
| LT-98587 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 48 | 112 | 112 | 64 | 64 |
| LT-15425 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 RLO Linear Fluorescent | 48 | 102 | 102 | 54 | 54 |
| LT-21054 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 45 | 59 | 59 | 14 | 14 |
| LT-60495 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) | 45 | 59 | 59 | 14 | 14 |
| LT-10511 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) | 45 | 59 | 59 | 14 | 14 |
| LT-25867 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 67 | 89 | 89 | 22 | 22 |
| LT-32845 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 67 | 112 | 112 | 45 | 45 |
| LT-45291 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 67 | 89 | 89 | 22 | 22 |
| LT-57432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 67 | 112 | 112 | 45 | 45 |
| LT-87326 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 53 | 59 | 59 | 6 | 6 |
| LT-70093 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) | 53 | 59 | 59 | 6 | 6 |
| LT-29908 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) | 53 | 59 | 59 | 6 | 6 |
| LT-69821 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 53 | 89 | 89 | 36 | 36 |
| LT-78193 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 53 | 112 | 112 | 59 | 59 |
| LT-83945 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 53 | 89 | 89 | 36 | 36 |
| LT-90462 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 53 | 112 | 112 | 59 | 59 |
| LT-32645 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 44 | 59 | 59 | 15 | 15 |
| LT-29023 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 70 | 89 | 89 | 19 | 19 |
| LT-30613 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 70 | 112 | 112 | 42 | 42 |
| LT-40921 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 70 | 89 | 89 | 19 | 19 |
| LT-50734 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 70 | 112 | 112 | 42 | 42 |
| LT-60432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 54 | 74 | 59 | 20 | 5 |
| LT-71214 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 53 | 120 | 89 | 67 | 36 |
| LT-89122 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 53 | 168 | 112 | 115 | 59 |
| LT-18934 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 54 | 80 | 59 | 26 | 5 |
| LT-24985 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 53 | 120 | 89 | 67 | 36 |
| LT-30765 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 54 | 168 | 112 | 114 | 58 |
| LT-43532 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 54.3 | 89 | 89 | 34.7 | 34.7 |
| LT-57486 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 54.3 | 112 | 112 | 57.7 | 57.7 |
| LT-65784 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 50 | 89 | 89 | 39 | 39 |
| LT-79584 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 50 | 112 | 112 | 62 | 62 |
| LT-93333 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 51 | 89 | 89 | 38 | 38 |
| LT-85346 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 64 | 89 | 89 | 25 | 25 |
| LT-93656 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 64 | 112 | 112 | 48 | 48 |
| LT-88743 | (2) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent | 90 | 122 | 122 | 32 | 32 |
| LT-19844 | (2) 96in (1) Instant Start Ballast - Normal Light Output - HO T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 160 | 210 | 160 | 50 | 0 |
| LT-94443 | (2) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 109 | 246 | 219 | 137 | 110 |
| LT-94323 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in HO T12 Linear Fluorescent | 98 | 207 | 109 | 109 | 11 |
| LT-27685 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 98 | 123 | 109 | 25 | 11 |
| LT-80796 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 160 | 210 | 160 | 50 | 0 |
| LT-10494 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 60 | 72 | 60 | 12 | 0 |
| LT-24162 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 60 | 72 | 60 | 12 | 0 |
| LT-39678 | (2) U-Tube (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | 52 | 72 | 60 | 20 | 8 |
| LT-40789 | (3) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 24in T12 Linear Fluorescent | 47 | 62 | 62 | 15 | 15 |
| LT-38675 | (3) 46in (2) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 83 | 144 | 89 | 61 | 6 |
| LT-23738 | (3) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent | 172 | 179 | 179 | 7 | 7 |
| LT-75850 | (3) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent | 176 | 179 | 179 | 3 | 3 |
| LT-49588 | (3) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 83 | 144 | 89 | 61 | 6 |
| LT-50379 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 78 | 120 | 89 | 42 | 11 |
| LT-60092 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 83 | 144 | 89 | 61 | 6 |
| LT-49683 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 78 | 120 | 89 | 42 | 11 |
| LT-50968 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 83 | 144 | 89 | 61 | 6 |
| LT-79821 | (3) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 78 | 115 | 89 | 37 | 11 |
| LT-38219 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 68 | 89 | 89 | 21 | 21 |
| LT-89032 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) | 68 | 89 | 89 | 21 | 21 |
| LT-32378 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) | 68 | 89 | 89 | 21 | 21 |
| LT-83248 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 75 | 89 | 89 | 14 | 14 |
| LT-40056 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) | 75 | 89 | 89 | 14 | 14 |
| LT-37543 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) | 75 | 89 | 89 | 14 | 14 |
| LT-10154 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 65 | 89 | 89 | 24 | 24 |
| LT-23675 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 92 | 112 | 112 | 20 | 20 |
| LT-34867 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 74 | 112 | 112 | 38 | 38 |
| LT-40982 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 97 | 112 | 112 | 15 | 15 |
| LT-18592 | (3) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 167 | 210 | 167 | 43 | 0 |
| LT-35845 | (3) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 167 | 210 | 167 | 43 | 0 |
| LT-59462 | (4) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent | 61 | 112 | 112 | 51 | 51 |
| LT-45382 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent | 87 | 122 | 122 | 35 | 35 |
| LT-29478 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 36in T12 Linear Fluorescent | 87 | 162 | 112 | 75 | 25 |
| LT-50488 | (4) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 54 | 123 | 109 | 69 | 55 |
| LT-38493 | (4) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent | 214 | 234 | 234 | 20 | 20 |
| LT-84940 | (4) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent | 218 | 234 | 234 | 16 | 16 |
| LT-68578 | (4) 48in (1) Instant Start Ballast - Reduced Light Output (Kit) T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 102 | 123 | 112 | 21 | 10 |
| LT-89871 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 102 | 123 | 112 | 21 | 10 |
| LT-99866 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 102 | 160 | 112 | 58 | 10 |
| LT-85958 | (4) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent | 112 | 216 | 175 | 104 | 63 |
| LT-59103 | (4) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 94 | 207 | 167 | 113 | 73 |
| LT-78327 | (4) 48in (2) Instant Start Ballast - Reduced Light Output (New Fixture) T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 102 | 144 | 112 | 42 | 10 |
| LT-20483 | (4) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 102 | 144 | 112 | 42 | 10 |
| LT-89782 | (4) 48in (2) Instant Start Ballast - Reduced Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 102 | 144 | 112 | 42 | 10 |
| LT-45842 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 90 | 112 | 112 | 22 | 22 |
| LT-36789 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) | 90 | 98 | 98 | 8 | 8 |
| LT-45732 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) | 90 | 98 | 98 | 8 | 8 |
| LT-95485 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 98 | 112 | 112 | 14 | 14 |
| LT-57859 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) | 98 | 102 | 102 | 4 | 4 |
| LT-68712 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) | 98 | 102 | 102 | 4 | 4 |
| LT-59655 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 86 | 112 | 112 | 26 | 26 |
| LT-19235 | (4) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 219 | 246 | 219 | 27 | 0 |
| LT-59685 | (6) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 175 | 246 | 175 | 71 | 0 |
| LT-39058 | (6) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 156 | 210 | 182 | 54 | 26 |
| LT-91623 | (6) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent | 175 | 216 | 182 | 41 | 7 |
| LT-69544 | (6) 48in (2) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 162 | 246 | 175 | 84 | 13 |
| LT-49785 | (6) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 142 | 210 | 175 | 68 | 33 |
| LT-18743 | (8) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 188 | 246 | 224 | 58 | 36 |
| LT-54329 | (8) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 188 | 246 | 224 | 58 | 36 |

## 2.1 Energy Savings Estimation Methodologies

The energy savings estimates are calculated as follows:

The following is a 1st baseline Retrofit Install Type sample energy savings calculation for (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) for Agricultural building type in Climate Zone 6.

EnergySavingskWhUnit×Year=[72 ×3264.34-54 ×3204]×1.058371,000WatthourskWh

EnergySavingskWhUnit×Year=65.64

The following is a 1st baseline Replace on Burnout and 2nd baseline Retrofit Install Type sample energy savings calculation for (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) for Agricultural building type in Climate Zone 6.

EnergySavingskWhUnit×Year=60-54×3204×1.058371,000WatthourskWh

EnergySavingskWhUnit×Year=20.35

## 2.2 Demand Reduction Estimation Methodologies

The demand reduction estimates are calculated as follows:

The following is a 1st baseline Retrofit Install Type sample demand reduction calculation for (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) for Agricultural building type in Climate Zone 6.

Demand ReductionkWUnit=[72×0.790709-54 ×0.773105]×1.10821,000WatthourskWh

Demand ReductionkWUnit=0.01683

The following is a 1st baseline Replace on Burnout and 2nd baseline Retrofit Install Type sample demand reduction calculation for (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) for Agricultural building type in Climate Zone 6.

Demand ReductionkWUnit=60-54×0.773105×1.10821,000WatthourskWh

Demand ReductionkWUnit=0.00514

Energy savings and demand reduction for other measures in different building types and climate zones can be found in the attachment [B].

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_Non-CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Assembly | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Primary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Secondary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Relocatable Classroom | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Community College | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - University | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Grocery | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Food Store | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Hospital | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Nursing Home | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Clinic | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Hotel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Guest Rooms | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Motel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Bio/Tech | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Light Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Misc - Commercial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office - Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office - Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Fast-Food | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Sit-Down | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Multistory Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Single-Story Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Conditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Unconditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Transportation - Communication - Utilities | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Warehouse - Refrigerated | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Residential Multi-family | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Residential Mobile Home – Double-Wide | NON_RES | DEER:Indoor_Non-CFL_Ltg |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

Base and Measure Case Costs were taken either from DEER, Statewide Lighting Cost Study (# reference), Lighting Cost Updates (# reference w/”A”), or from an online store. Cost and labor values can be found in the attachments section []. Note for some of the measures, the base cost reference and the measure cost reference are the same reference. For these measures, the closest cost available was the same for both the measure and case. This implies that there is little cost difference between the two options.

There are a number of measures that contained additional features not addressed in any of the three cost references, excluding online stores. Those features included reflectors, dimming, and A/B switching. If the fixture was present on either DEER or either of the Statewide Lighting Cost Studies, the base fixture cost was adjusted by either a multiplier for reflectors or an additive for control type features (dimming and A/B switching).

The multiplier for reflectors was determined by using the “Revised DEER Measure Cost Summary (05_30_2008) Revised (06_02_2008).xls”. This file contains a number of linear fluorescent measures that have both reflectors and no reflectors for the same fixture. The measure and labor cost for those fixtures were ratioed together and then averaged. The actual calculations can be seen in the cost attachment. The multiplier for material is 2.05 and the multiplier for labor is 1.18.

The additive cost for dimming and A/B switching was determined by referring to a control measure from the DEER costing also found in “Revised DEER Measure Cost Summary (05_30_2008) Revised (06_02_2008).xls”. To approximate the addition cost of control features the DEER measure and labor cost for “Assume control 3 2-lamp fixtures w/T8 34W EL Ballast”, DEER Cost Case ID D08-NE-ILtg-OccSens, was added to the measure. The additional material cost is $47.15 and the additional labor cost is $44.12.

Table 10 Base/Measure Cost Reference

| Solution Code | Measure Name | Measure Cost Reference | Base Cost Reference | Note |
| --- | --- | --- | --- | --- |
| LT-61312 | (1) 24in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) | 59 | A2.2 | 2 Lamp Fixture Divided by Two |
| LT-46109 | (1) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent (per lamp) | 65 | A2.2 | 2 Lamp Fixture Divided by Two |
| LT-18943 | (1) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 24in T12 Linear Fluorescent | 62 | A2.1 | None |
| LT-33633 | (1) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent | 86 | A1.1 | None |
| LT-83912 | (1) 36in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent | 88 | A1.1 | None |
| LT-18642 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T5 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) | 79 | A1.2 | 2 Lamp Fixture Divided by Two |
| LT-12877 | (1) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 36in T12 Linear Fluorescent (per lamp) | 94 | A1.2 | 2 Lamp Fixture Divided by Two |
| LT-94378 | (1) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | 46in28wT5PSEl64w | 48in2g32wT8ISNEl59w | 2 Lamp Fixture Divided by Two |
| LT-20394 | (1) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 46in54wT5HOPSEl62w | 48in2g32wT8ISNEl59w | None |
| LT-30495 | (1) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 46in28wT5PSEl33w | 48in2g32wT8ISNEl31w | None |
| LT-79688 | (1) 46in (1) Programmed Start Tandem 2 Lamp Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 46in54wT5HOT2PSEl59w | 48in2g32wT8ISNEl59w | None |
| LT-94841 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent | A8 | A5 | None |
| LT-30612 | (1) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) | A8 | A5 | 2 Lamp Fixture Divided by Two |
| LT-50439 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent | A8 | A5 | None |
| LT-39815 | (1) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 46in T5 Linear Fluorescent (per lamp) | A8 | A5 | 2 Lamp Fixture Divided by Two |
| LT-40596 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl31w | 48in2g32wT8ISNEl31w | None |
| LT-82210 | (1) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | 48in2g32wT8ISNEl59w | 48in2g32wT8ISNEl59w | 2 Lamp Fixture Divided by Two |
| LT-43895 | (1) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | 48in2g32wT8RSREl27w | 48in2g32wT8ISNEl31w | None |
| LT-50998 | (1) 48in (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 48in2g32wT8ISVEl41w | 48in2g32wT8ISNEl59w | None |
| LT-89585 | (1) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent (per lamp) | A11.2 | 48in2g32wT8ISNEl59w | 2 Lamp Fixture Divided by Two |
| LT-14830 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | A9 | 48in2g32wT8ISNEl31w | None |
| New 1 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) | A9 | 48in2g32wT8ISNEl31w | None |
| New 2 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) | A9 | 48in2g32wT8ISNEl31w | None |
| LT-67100 | (1) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) | 1 | 48in2g32wT8ISNEl59w | 2 Lamp Fixture Divided by Two |
| LT-71023 | (1) 48in Reduced 25 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | A9 | 48in2g32wT8ISNEl31w | None |
| LT-69594 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | A10 | 48in2g32wT8ISNEl31w | None |
| New 3 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Common Area) | A10 | 48in2g32wT8ISNEl31w | None |
| New 4 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (Dwelling Area) | A10 | 48in2g32wT8ISNEl31w | None |
| LT-84012 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent (per lamp) | 9 | 48in2g32wT8ISNEl59w | 2 Lamp Fixture Divided by Two |
| LT-89708 | (1) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 48in T8 Linear Fluorescent | A10 | 48in2g32wT8ISNEl31w | None |
| LT-91014 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | A10 | 48in2g32wT8ISNEl59w | None |
| LT-10436 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | A10 | 48in2g32wT8ISNEl59w | None |
| LT-24444 | (1) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | A10 | 48in2g32wT8ISNEl59w | None |
| LT-85932 | (1) 48in Reduced 28 Watt T8 Linear Fluorescent replacing (1) 48in T12 Linear Fluorescent | A10 | 48in2g32wT8ISNEl31w | None |
| LT-58535 | (1) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent | Online Retailer | Online Retailer | Cost from Online Retailer |
| LT-20983 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent | 96in59wT8T2ISREl49w | 96in59wT8T2ISREl49w | None |
| LT-58109 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) | 96in59wT8ISREl98w | 96in59wT8ISREl98w | 2 Lamp Fixture Divided by Two |
| LT-48392 | (1) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 96in59wT8T2ISREl49w | 96in59wT8ISREl98w | None |
| LT-78895 | (2) 22in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 59 | A2.2 | None |
| LT-59482 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 65 | A2.2 | None |
| LT-83726 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 65 | 48in2g32wT8ISNEl59w | None |
| LT-90213 | (2) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent | 65 | A2.4 | None |
| LT-54637 | (2) 24in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 65 | A2.2 | Reflector Multiplier Applied |
| LT-60969 | (2) 24in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 24in T12 Linear Fluorescent | 66 | A2.2 | None |
| LT-91429 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A6 | A3 | None |
| LT-19002 | (2) 24in F17 (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | A6 | A3 | None |
| LT-26889 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A7 | A3 | None |
| LT-32000 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | A7 | A3 | None |
| New 18 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A7 | A3 | Reflector Multiplier Applied |
| New 19 | (2) 24in F17 (1) Premium Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T8 U-Tube Fluorescent | A7 | A3 | Reflector Multiplier Applied |
| LT-25668 | (2) 34in (1) Premium Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent | 79 | A1.2 | None |
| LT-98333 | (2) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (1) 72in T12 Linear Fluorescent | 92 | Online Retailer | None |
| LT-43018 | (2) 36in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 36in T12 Linear Fluorescent | 94 | A1.2 | None |
| LT-73491 | (2) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent (per lamp) | A8.2 | A5.2 | 2 Lamp Fixture Divided by Two |
| LT-68385 | (2) 46in (1) Instant Start Ballast - Normal Light Output w/ Dimming T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | A8.2 | A5.2 | Control Function Additive |
| LT-69232 | (2) 46in (1) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 46in54wT5HOPSEl117w | 48in2g32wT8ISREl102w | None |
| LT-78696 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 46in28wT5PSEl64w | 48in2g32wT8ISNEl59w | None |
| LT-89685 | (2) 46in (1) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 46in28wT5PSEl64w | 48in2g32wT8ISNEl89w | None |
| LT-19843 | (2) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent | A8.2 | A5.2 | None |
| LT-60902 | (2) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 46in T5 Linear Fluorescent | A8.2 | A5.2 | None |
| LT-93723 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISHEl65w | 48in2g32wT8ISNEl89w | None |
| LT-37676 | (2) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISHEl65w | 48in2g32wT8ISREl102w | None |
| LT-39875 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISREl102w | None |
| LT-49843 | (2) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISNEl89w | None |
| LT-78548 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISNEl89w | Control Function Additive |
| LT-58359 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-19832 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Dimming T8 Linear Fluorescent replacing (1) 96in HO T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 96in59wT8T2ISREl49w | Control Function Additive |
| LT-38503 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ DR Dimming Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-18545 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNElrfl59w | 48in2g32wT8ISNEl89w | Control Function Additive |
| LT-29436 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors and A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNElrfl59w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-68384 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNElrfl59w | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-40696 | (2) 48in (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNElrfl59w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-18409 | (2) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (1) 96in T12 Linear Fluorescent | 48in1g32wT8ISREl52w | 96in59wT8T2ISREl49w | None |
| LT-19210 | (2) 48in (1) NEMA Premium High Efficiency Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | A11.2 | 48in2g32wT8ISNEl59w | None |
| LT-24981 | (2) 48in (1) Premium Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 96in59wT8ISREl98w | None |
| LT-90075 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISNEl59w | None |
| LT-73849 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISREl102w | None |
| LT-38546 | (2) 48in (1) Premium Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISNEl59w | Control Function Additive |
| LT-49810 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISNEl59w | None |
| LT-49587 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISNEl89w | None |
| LT-20796 | (2) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl54w | 48in2g32wT8ISREl102w | None |
| LT-58327 | (2) 48in (2) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl59w | 48in2g32wT8ISNEl89w | None |
| LT-98587 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 48in3g32wT8PISNElrfl54w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-15425 | (2) 48in (2) Premium Instant Start Ballast - Reduced Light Output w/ Reflector T8 Linear Fluorescent replacing (4) 48in T8 RLO Linear Fluorescent | 48in3g32wT8PISNElrfl54w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-21054 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 1 | 48in2g32wT8ISNEl59w | None |
| New 5 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) | 1 | 48in2g32wT8ISNEl59w | None |
| New 6 | (2) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) | 1 | 48in2g32wT8ISNEl59w | None |
| LT-25867 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | None |
| LT-32845 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | None |
| LT-45291 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-57432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | Reflector Multiplier Applied |
| LT-87326 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISNEl59w | None |
| New 7 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Common Area) | 9 | 48in2g32wT8ISNEl59w | None |
| New 8 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent (Dwelling Area) | 9 | 48in2g32wT8ISNEl59w | None |
| LT-69821 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | None |
| LT-78193 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISREl102w | None |
| LT-83945 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-90462 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISREl102w | Reflector Multiplier Applied |
| LT-32645 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 48in T8 Linear Fluorescent | 10 | 48in2g32wT8ISNEl59w | None |
| LT-29023 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | None |
| LT-30613 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | None |
| LT-40921 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-50734 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast - Very High Light Output w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | Reflector Multiplier Applied |
| LT-60432 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISNEl59w | None |
| LT-71214 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | None |
| LT-89122 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISREl102w | None |
| LT-18934 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (2) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISNEl59w | Reflector Multiplier Applied |
| LT-24985 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-30765 | (2) 48in Reduced 28 Watt (1) Instant Start Ballast w/ Reflectors T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 9 | 48in2g32wT8ISREl102w | Reflector Multiplier Applied |
| LT-43532 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | None |
| LT-57486 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | None |
| LT-65784 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | None |
| LT-79584 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISREl102w | None |
| New 17 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 9 | 48in2g32wT8ISNEl89w | Reflector Multiplier Applied |
| LT-85346 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISNEl89w | None |
| LT-93656 | (2) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 11 | 48in2g32wT8ISREl102w | None |
| LT-88743 | (2) 72in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent | Online Retailer | Online Retailer | Cost from Online Retailer |
| LT-19844 | (2) 96in (1) Instant Start Ballast - Normal Light Output - HO T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 146 | 96in59wT8ISNEl167w | None |
| LT-94443 | (2) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 96in59wT8ISREl98w | 96in59wT8ISNEl219w | None |
| LT-94323 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in HO T12 Linear Fluorescent | 96in59wT8ISREl98w | 96in59wT8ISREl98w | None |
| LT-27685 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 96in59wT8ISREl98w | 96in59wT8ISREl98w | None |
| LT-80796 | (2) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 96in59wT8ISREl98w | 96in59wT8ISNEl167w | None |
| LT-10494 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A3 | A3 | None |
| LT-24162 | (2) U-Tube (1) Instant Start Ballast - Normal Light Output w/ Reflectors T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A3 | A3 | Reflector Multiplier Applied |
| LT-39678 | (2) U-Tube (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) T12 U-Tube Fluorescent | A3 | A3 | None |
| LT-40789 | (3) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 24in T12 Linear Fluorescent | 69 | A2.3 | None |
| LT-38675 | (3) 46in (2) Programmed Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in3g32wT8PISNEl83w | 48in2g32wT8ISREl102w | None |
| LT-23738 | (3) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent | A8.3 | A5.3 | None |
| LT-75850 | (3) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (3) 46in T5 Linear Fluorescent | A8.3 | A5.3 | None |
| LT-49588 | (3) 48in (1) Instant Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISHEl93w | 48in2g32wT8ISREl102w | None |
| LT-50379 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl89w | 48in2g32wT8ISNEl89w | None |
| LT-60092 | (3) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl89w | 48in2g32wT8ISREl102w | None |
| LT-49683 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl89w | 48in2g32wT8ISNEl89w | Control Function Additive |
| LT-50968 | (3) 48in (1) Instant Start Ballast - Normal Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISNEl89w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-79821 | (3) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T12 Linear Fluorescent | 48in2g32wT8ISREl78w | 48in2g32wT8ISNEl89w | None |
| LT-38219 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 3 | 48in2g32wT8ISNEl89w | None |
| New 9 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) | 3 | 48in2g32wT8ISNEl89w | None |
| New 10 | (3) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) | 3 | 48in2g32wT8ISNEl89w | None |
| LT-83248 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 12 | 48in2g32wT8ISNEl89w | None |
| New 11 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Common Area) | 12 | 48in2g32wT8ISNEl89w | None |
| New 12 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent (Dwelling Area) | 12 | 48in2g32wT8ISNEl89w | None |
| LT-10154 | (3) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 48in T8 Linear Fluorescent | 13 | 48in2g32wT8ISNEl89w | None |
| LT-23675 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 14 | 48in2g32wT8ISREl102w | None |
| LT-34867 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 12 | 48in2g32wT8ISREl102w | None |
| LT-40982 | (3) 48in Reduced 28 Watt (1) Programmed Start Ballast - Very High Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 14 | 48in2g32wT8ISREl102w | None |
| LT-18592 | (3) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 96in59wT8ISNEl167w | 96in59wT8ISNEl167w | None |
| LT-35845 | (3) 96in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 96in59wT8ISNEl167w | 96in59wT8ISNEl167w | None |
| LT-59462 | (4) 24in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 24in T12 Linear Fluorescent | 74 | A2.4 | None |
| LT-45382 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (2) 72in T12 Linear Fluorescent | 103 | Online Retailer | None |
| LT-29478 | (4) 36in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 36in T12 Linear Fluorescent | 103 | A1.4 | None |
| LT-50488 | (4) 46in (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 108 | 96in59wT8ISREl98w | None |
| LT-38493 | (4) 46in Reduced 49 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent | A8.4 | A5.4 | None |
| LT-84940 | (4) 46in Reduced 51 Watt (1) Instant Start Ballast - Normal Light Output T5 Linear Fluorescent replacing (4) 46in T5 Linear Fluorescent | A8.4 | A5.4 | None |
| LT-68578 | (4) 48in (1) Instant Start Ballast - Reduced Light Output (Kit) T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 96in59wT8ISREl98w | None |
| LT-89871 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 96in59wT8ISREl98w | None |
| LT-99866 | (4) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 48in2g32wT8ISREl102w | None |
| LT-85958 | (4) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent | 45 | 48in2g32wT8ISNEl175w | None |
| LT-59103 | (4) 48in (1) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (2) 96in T12 Linear Fluorescent | 46 | 96in59wT8ISREl98w | None |
| LT-78327 | (4) 48in (2) Instant Start Ballast - Reduced Light Output (New Fixture) T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 48in2g32wT8ISREl102w | None |
| LT-20483 | (4) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 48in2g32wT8ISREl102w | None |
| LT-89782 | (4) 48in (2) Instant Start Ballast - Reduced Light Output w/ A/B Switching T8 Linear Fluorescent replacing (4) 48in T12 Linear Fluorescent | 48in2g32wT8ISREl102w | 48in2g32wT8ISREl102w | Control Function Additive |
| LT-45842 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 5 | 48in2g32wT8ISREl102w | None |
| New 13 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) | 5 | 96in59wT8ISREl98w | None |
| New 14 | (4) 48in Reduced 25 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) | 5 | 96in59wT8ISREl98w | None |
| LT-95485 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 15 | 48in2g32wT8ISREl102w | None |
| New 15 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Common Area) | 15 | 48in2g32wT8ISREl102w | None |
| New 16 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent (Dwelling Area) | 15 | 48in2g32wT8ISREl102w | None |
| LT-59655 | (4) 48in Reduced 28 Watt (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 48in T8 Linear Fluorescent | 16 | 48in2g32wT8ISREl102w | None |
| LT-19235 | (4) 96in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 96in59wT8ISNEl219w | 96in59wT8ISNEl219w | None |
| LT-59685 | (6) 48in (1) Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 48in2g32wT8ISNEl175w | 96in59wT8ISNEl219w | None |
| LT-39058 | (6) 48in (1) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 48in2g32wT8ISREl156w | 96in59wT8ISNEl167w | None |
| LT-91623 | (6) 48in (1) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (6) 48in T12 Linear Fluorescent | 48in3g32wT82PISNElrfl162w | 48in2g32wT8ISNEl175w | None |
| LT-69544 | (6) 48in (2) Premium Instant Start Ballast - Normal Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 48in3g32wT82PISNEl162w | 96in59wT8ISNEl219w | None |
| LT-49785 | (6) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (3) 96in T12 Linear Fluorescent | 48in3g32wT82PISREl142w | 96in59wT8ISNEl167w | None |
| LT-18743 | (8) 48in (2) Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 48in2g32wT8ISREl204w | 96in59wT8ISNEl219w | None |
| LT-54329 | (8) 48in (2) Premium Instant Start Ballast - Reduced Light Output T8 Linear Fluorescent replacing (4) 96in T12 Linear Fluorescent | 48in3g32wT82PISREl188w | 96in59wT8ISNEl219w | None |

*These measures are per lamp measures derived from a 2 lamp fixture. The cost values have been divided by two to approximate the savings.

## 4.2 Measure Case Cost

See Table 10 for the Measure Case Cost reference.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For RET 1st baseline, GMC is represented by the equation below:

GMC = Measure Equipment Cost + Measure Labor Cost

For RET 2nd baseline and ROB, GMC is represented by the equation below:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the same as Gross Measure Cost of ROB as shown in Section 4.3.1.

All the costs referenced in this work paper can be found in the worksheet [B].

# Attachments

1.

2.

3.

# References

[31]

[351]

[355]

[422]

[436]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

http://www.ncaee.org/modules/info/files/files_4adcd38174d01.pdf

Attachment 1 – Calculation Template v4.xlsx

Attachment 2 – 2013-2014_LightingRetrofit_Disposition-30May2014.xlsx

Attachment 3 – Cost – Final Development