Work Paper SCE13LG075

Revision 2

Southern California Edison

CFL Plug in Lamps

For Work Paper Reviewer Use Only

List all major comments that occurred during the review. This table may only be removed during management review.

| Major Comment | Reviewer Name | Date | Outcome/Resolution |
| --- | --- | --- | --- |
| The new template is not utilized. Please update the entire WP using the new template provided for DEER 2015 update. Use this, Statewide Work Paper Template v0 SCE 09-11-15.docx which is located at WP template folder. | RC |  | Updated |
| There are several issues with this WP that are need to be correctly updated. See the comments below. | RC |  | Corrected |
| Please delete the old calculation template and attach the new calculation template to this WP. | RC |  | New calculation template added |
| Template – GSIA_ID_SFm column is not filled out | RC |  | Added |
|  |  |  | Modified units throughout from “lamp” to “fixture” |

# At-a-Glance Summary

| Measure Codes | LT-71839; LT-81723; LT-28364; LT-48738; LT-32837; LT-51093; LT-86596; LT-68853; LT-81934; LT-19481; LT-84715 |
| --- | --- |
| Measure Description | Residential plug in CFL lamps in fixtures such as torchieres, desk, table, and floor lamps with wattages ranging from 13 Watts to 70 Watts |
| Base Case Description | Incandescent lamps in fixtures such as torchieres, desk, table, and floor lamps with higher wattages than the measure case wattage |
| Units | Per fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | DEER2014: 16 years - ILtg-CFLfix-Res per referenced EUL ID in DEER/READI Measure Definitions |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | DEER2014/2016: Res-sAll-mCFL-up: NTG = 0.9 Res-Default>2: NTG= 0.55 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| SCE13LG075.0 | 04/11/2012 | James Gowen/Matrix | -Original work paper draft for 2013-2014 program cycle. -Updated version of 2010-2012 work paper WPSCRELG0075. |
| SCE13LG075.1 | 04/16/2014 | Linda Wan/TRC Energy Services | -Updated work paper template -Added technical description, installation rate, READi Technology Fields. -Updated energy savings calculations with new READi data. -Updated cost to include labor. |
| SCE13LG075.1 | 04/16/2014 | Jason Wang/SCE | Work paper updated for reporting period, effective 7/1/14 – 12/31/14. |
| SCE13LG075.2 | 09/10/2015 | Andres Fergadiotti/SCE | -Updated work paper “Calculation Template v6.0 08-17-15” - Updated work paper (word) template – “Statewide Work Paper Template v0 SCE 09-11-15” - Updated impacts per latest DEER/READI v2.2.0 - Updated NTG and EUL per latest DEER database |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of existing incandescent/halogen lamps with compact fluorescent (CFL) lamps in residential plug-in fixtures.

The measure case is a residential CFL plug-in fixture such as a torchiere, desk, table, or floor lamp with CFL wattage ranging from 13 to 70 Watts. The base case is an incandescent fixture such as a torchiere, desk, table, or floor lamp with a higher wattage than the measure case. The following table shows all of the measures contained within this work paper.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Residential CFL plug-in lamp fixture |
| Existing Condition | Residential Incandescent lamp fixture |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-71839 |  | 13 Watt CFL replacing a higher wattage source |
|  |  | LT-81723 |  | 14 Watt CFL replacing a higher wattage source |
|  |  | LT-28364 |  | 18 Watt CFL replacing a higher wattage source |
|  |  | LT-48738 |  | 19 Watt CFL replacing a higher wattage source |
|  |  | LT-32837 |  | 22 Watt CFL replacing a higher wattage source |
|  |  | LT-51093 |  | 23 Watt CFL replacing a higher wattage source |
|  |  | LT-86596 |  | 26 Watt CFL replacing a higher wattage source |
|  |  | LT-68853 |  | 36 Watt CFL replacing a higher wattage source |
|  |  | LT-81934 |  | 55 Watt CFL replacing a higher wattage source |
|  |  | LT-19481 |  | 65 Watt CFL replacing a higher wattage source |
|  |  | LT-84715 |  | 70 Watt CFL replacing a higher wattage source |

The measures contained in this work paper are for Residential Single Family and Multi-family buildings located in SCE climate zones only.

Per latest 2013-2015 Solutions Directory, there are currently no eligibility requirements, implementation requirements, or other program restrictions guidelines for these solution codes.

## 1.2 Technical Description

For this work paper, “plug in lamp” refers to a portable light fixture such as a torchiere, desk, table, or floor lamp that can be plugged into an electrical wall outlet. The light bulb used in this type of fixture can be compact fluorescent (CFL) or incandescent.

## 1.3 Installation Types and Delivery Mechanisms

The measure install type is Replace-on-Burnout (ROB).  ROB measures replace existing equipment with more energy efficient equipment on failure or close to failure of the existing equipment.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.  The delivery methods are the following:

- Upstream Programs – Up-Stream Buy Down

The upstream strategy is used to stimulate sales of energy efficient lighting. The lighting measures are promoted and tracked separately from other point of sale rebate measures. Lighting incentives and promotions influence customers to purchase energy-savings lighting products at retail outlets and install them in homes and small businesses.

The following measures are offered through the upstream program:

  - 13 Watt CFL replacing a higher wattage source

  - 14 Watt CFL replacing a higher wattage source

  - 18 Watt CFL replacing a higher wattage source

  - 19 Watt CFL replacing a higher wattage source

  - 23 Watt CFL replacing a higher wattage source

Note that measure offering under the Upstream Programs exclude some of the measures included in this workpaper specifically 22 Watt Plug-in Lamp CFL and those above 23 Watt Plug-in Lamps CFL.

- Financial Support – Exchange/Replacement

The Lamp Exchange Program consists of local events where residential customers can bring old, standard efficiency plug in lighting products such as torchieres, desk, table, and floor lamps to exchange for new energy efficient CFL fixtures at no cost. For the lamp exchange program, base case lamps can be halogen or incandescent floor, table, or desk lamps.

The following measures are offered through the exchange program:

  - 13 Watt CFL replacing a higher wattage source

  - 18 Watt CFL replacing a higher wattage source

  - 22 Watt CFL replacing a higher wattage source

  - 26 Watt CFL replacing a higher wattage source

  - 36 Watt CFL replacing a higher wattage source

  - 55 Watt CFL replacing a higher wattage source

  - 65 Watt CFL replacing a higher wattage source

  - 70 Watt CFL replacing a higher wattage source

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

All measures and measures impacts documented in this workpaper were taken from DEER/READi tool version 2.2.0

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | Yes |
| DEER Version | DEER 2016, READI v2.2.0 |
| Reason for Deviation from DEER | N/A |
| DEER Measure IDs Used | Refer to DEER/READI Data Section |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Res-sAll-mCFL-up | CFL-screw in, All | Res | Any | PreRebUp | 0.90 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |
| MFm-IntCF-SCE | Interior CFL; Annual Installation Rate; Multi-Family | Res | MFm | NonUpStrm | 0.72 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFLfix-Res | CFL Fixtures – Indoor – Residential | Res | Lighting | 16 | N/A |

### 1.4.2 Codes and Standards Analysis

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2014) [A] | Section 1605.1(n) Luminaires and Torchieres | July 1, 2014 / Portable lighting fixtures (2010), year the regulations take effect. |

Title 20 Appliance Efficiency Regulations [A] includes standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California. Section 1605.3(n)(4)(A) addresses portable luminaires manufactured on or after January 1, 2010:

Section 1605.3(n)(3)(A)

- Portable luminaires manufactured on or after January 1, 2010 shall meet one or more of the following requirements:

(i) Be equipped with a dedicated fluorescent lamp socket connected to a high frequency electronic ballast contained within the portable luminaire;

(ii) Be equipped with one or more GU‐24 line‐voltage sockets and not rated for use with incandescent lamps of any type, including line voltage or low voltage;

(iii) Be an LED luminaire or a portable luminaire with an LED light engine with integral heat sink, and comply with the minimum requirements shown in Table N‐2;

(iv) Be equipped with an E12, E17, or E26 screw‐based socket and be prepackaged and sold together with one screw‐based compact fluorescent lamp or screw-based LED lamp for each screw‐based socket on the portable luminaire. The compact fluorescent or LED lamps which are prepackaged with the portable luminaire shall be fully compatible with the luminaire controls, meaning that portable luminaires having a dimmer control shall be prepackaged with dimmable compact fluorescent or LED lamps, and portable luminaires having 3‐way controls shall be prepackaged with 3‐way compact fluorescent or LED lamps. The compact fluorescent lamps which are prepackaged with the luminaires shall also meet the minimum energy efficiency levels established by ENERGY STAR® for compact fluorescent lamps in effect on December 31, 2008. The LED lamps required to be packaged with the luminaire shall comply with the minimum requirements shown in Table N‐2;

(v) Be equipped with one or more single‐ended, non‐screw based halogen lamp sockets (line or low voltage), a dimmer control or high low control, and be rated for a maximum of 100W.

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

None relevant studies were reviewed for this work paper update.

## 1.6 Data Quality and Future Data Needs

Additional cost data may help to better inform and support this workpaper.

# Section 2. Calculation Methodology

All measures and measures impacts documented in this workpaper were directly taken from DEER/READi tool version 2.2.0

The following table contains data file with all the READi exports informing CFL fixtures measures.  The savings come from the columns beginning with “ACustWB.”  Data additionally includes Measure Definitions from DEER/READI.

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| Multiple solution codes | Multiple measures |  |

The methodology used by READi to calculate energy savings is described in the Energy Division (ED) Workpaper Disposition for Lighting Retrofits (Attachment 3, [385]). Given the measure case wattage, the base case wattage is calculated by multiplying the measure case wattage by a wattage ratio of 2.48 for Residential Non-Reflector CFLs as applied and documented in the 2015 Uncertain Measures - Attachment 4.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-Family | DEER:Indoor_CFL_Ltg | RES |

# Section 4. Costs

## 4.1 Base Case Cost

ROB measures assume a single base case cost for the entire life of the measure. Since no cost information on subject measures is currently available in CPUC Work Order 17 [B], costs are taken from lamp fixture suppliers and/or manufacturers and can be found in the costs spreadsheet (Attachment 2).  Note that Incandescent-to-CFL equivalency table was used to match base case wattage with the measure case.

The labor cost for all plug-in lamp fixtures is $5.75 for the base case.  Labor cost was determined using labor rate documentation from Work Order 17 [B].

## 4.2 Measure Case Cost

Since no cost information on subject measure is currently available in Work Order 17 [B], the measure case costs are taken from lamp fixture suppliers and/or manufacturers and can be found in the costs spreadsheet.  The Full and Incremental Cost table provide sample of the measure case cost data.  If a measure was not covered by the DEER costs spreadsheet, the cost of the closest wattage and measure type was used.

The labor cost for all plug-in lamp fixtures is $5.75 for the measure case.   Labor cost was determined using labor rate documentation from Work Order 17 [B].

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Sample full and incremental measure cost indicated below.

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| 13 Watt Plug-in Lamp CFL | ROB | $2.90 | $48.75 | N/A |
| 36 Watt Plug-in Lamp CFL | ROB | $49.50 | $140.50 | N/A |

# Attachments

# References

| [385] | Workpaper Disposition for Lighting Retrofits |
| --- | --- |

[A] 	2015 Appliance Efficiency Regulations (Title 20)

[B]     CPUC Work Order # 17 for Measure Costs