Work Paper SCE13LG075

Revision 1

Southern California Edison Company

CFL Plug in Lamps

# At-a-Glance Summary

| Applicable Measure Codes: | LT-71839; LT-81723; LT-28364; LT-48738; LT-32837; LT-51093; LT-86596; LT-68853; LT-81934; LT-19481; LT-84715 |
| --- | --- |
| Measure Description: | Residential plug in CFL fixtures such as torchieres, desk, table, and floor lamps with wattages ranging from 13 Watts to 70 Watts |
| Base Case Description: | Incandescent fixtures such as torchieres, desk, table, and floor lamps with higher wattages than the measure case wattage |
| Energy Impact Common Units: | Per fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | DEER2014: 16 years |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | DEER2014: CFL-screw in, All. = 0.54 DEER2014: All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years = 0.55 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG075.0 | No | 04/11/2012 | James Gowen/Matrix | -Original work paper draft for 2013-2014 program cycle. -Updated version of 2010-2012 work paper WPSCRELG0075. |
| SCE13LG075.1 | Yes | 4/16/2014 | Linda Wan/TRC Energy Services | -Updated work paper template -Added technical description, installation rate, READi Technology Fields. -Updated energy savings calculations with new READi data. -Updated cost to include labor. |
|  | No | 4/16/14 | Jason Wang/SCE | Work paper updated for reporting period, effective 7/1/14 – 12/31/14. |

# Section 1. General Measure & Baseline Data

This work paper details the replacement of existing incandescent lamps (portable light fixtures, e.g. table lamps) with compact fluorescent (CFL) lamps.

## 1.1 Measure Description & Background

The measure case is a residential CFL plug in lamp such as a torchiere, desk, table, or floor lamp with CFL wattage ranging from 13 to 70 Watts. The base case is an incandescent lamp such as a torchiere, desk, table, or floor lamp with a higher wattage than the measure case. Table 1 shows all of the measures contained within this work paper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-71839 | 13 Watt Plug-in Lamp CFL |
| LT-81723 | 14 Watt Plug-in Lamp CFL |
| LT-28364 | 18 Watt Plug-in Lamp CFL |
| LT-48738 | 19 Watt Plug-in Lamp CFL |
| LT-32837 | 22 Watt Plug-in Lamp CFL |
| LT-51093 | 23 Watt Plug-in Lamp CFL |
| LT-86596 | 26 Watt Plug-in Lamp CFL |
| LT-68853 | 36 Watt Plug-in Lamp CFL |
| LT-81934 | 55 Watt Plug-in Lamp CFL |
| LT-19481 | 65 Watt Plug-in Lamp CFL |
| LT-84715 | 70 Watt Plug-in Lamp CFL |

The measures contained in this work paper are for Single Family and Multi-family residential buildings located in SCE climate zones only.

## 1.2 Technical Description

For this work paper, “plug in lamp” refers to a portable light fixture such as a torchiere, desk, table, or floor lamp that can be plugged into an electrical wall outlet. The light bulb used in this type of fixture can be compact fluorescent (CFL) or incandescent.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this work paper and application types used by READi.

The delivery methods are the following:

- Upstream Programs – Up-Stream Buy Down

The upstream strategy is used to stimulate sales of energy efficient lighting. The lighting measures are promoted and tracked separately from other point of sale rebate measures. Lighting incentives and promotions influence customers to purchase energy-savings lighting products at retail outlets and install them in homes and small businesses.

The following measures are offered through the upstream program:

  - 13 Watt Plug-in Lamp CFL

  - 14 Watt Plug-in Lamp CFL

  - 18 Watt Plug-in Lamp CFL

  - 19 Watt Plug-in Lamp CFL

  - 23 Watt Plug-in Lamp CFL

- Financial Support – Exchange/Replacement

The Lamp Exchange Program consists of local events where residential customers can bring old, standard efficiency plug in lighting products such as torchieres, desk, table, and floor lamps to exchange for new energy efficient CFL fixtures at no cost. For the lamp exchange program, base case lamps can be halogen or incandescent floor, table, or desk lamps.

The following measures are offered through the exchange program:

  - 13 Watt Plug-in Lamp CFL

  - 18 Watt Plug-in Lamp CFL

  - 22 Watt Plug-in Lamp CFL

  - 26 Watt Plug-in Lamp CFL

  - 36 Watt Plug-in Lamp CFL

  - 55 Watt Plug-in Lamp CFL

  - 65 Watt Plug-in Lamp CFL

  - 70 Watt Plug-in Lamp CFL

The measure install type is Replace-on-Burnout (ROB).  ROB measures replace existing equipment with more energy efficient equipment on failure or close to failure of the existing equipment.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Several of these measures are included in DEER 2014. For those that are not in DEER, the READi tool version 1.0.5 was used to determine savings by scaling existing DEER measures.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | Yes |
| DEER Building Prototypes Used | No |
| Deviation from DEER | Some measures are from DEER, and others are scaled. |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | Res-Lighting-InGen_CFLratio0347_CFLscw-13w |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-sAll-mCFL-up | CFL-screw in, All | Res | Any | PreRebUp | 0.54 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment (Attachment 1). This value is obtained from the support table available in READi. Currently there is no final version on the installation rate table. To address appropriate selection of the installation rate the date of the work paper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| MFm-IntCF-SCE | Interior CFL; Annual Installation Rate; Multi-Family | Res | MFm | NonUpStrm | 0.72 |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Since the Calculation Template can only accommodate one GSIA value per measure, 0.72 is used for all measures.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the work papers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the work paper. For a full set of values associated with the measures in the work paper refer the Excel calculation template (Attachment 1).

Table 5 READi Tech IDs

| READi Field Name | Values included in this work paper |
| --- | --- |
| Measure Case UseCategory | Lighting (Lighting) |
| Measure Case UseSubCats | Indoor General Lighting (InGen) |
| Measure Case TechGroups | Lighting – Plug-in (Ltg_PlugIn) |
| Measure Case TechTypes | CFL Fixture (CFL_fixt) |
| Base Case TechGroups | Lighting – Plug-in (Ltg_PlugIn) |
| Base Case TechTypes | Incandescent Lamp (Incan_lamp) |

### 1.4.2 Codes and Standards Analysis

Title 20 Appliance Efficiency Regulations [356] includes standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California. Section 1605.3(n)(4)(A) addresses portable luminaires manufactured on or after January 1, 2010:

Section 1605.3(n)(4)(A) - Portable Luminaires. - Portable luminaires manufactured on or after January 1, 2010 shall meet one or more of the following requirements: - Be equipped with a dedicated fluorescent lamp socket connected to a high frequency electronic ballast contained within the portable luminaire; - Be equipped with one or more GU-24 line-voltage sockets and not rated for use with incandescent lamps of any type, including line voltage or low voltage; - Be an LED luminaire or a portable luminaire with an LED light engine with integral heat sink, and comply with the minimum requirements shown in Table N-3; - Be equipped with an E12, E17, or E26 screw-based socket and be prepackaged and sold together with one screw-based compact fluorescent lamp or screw-based LED lamp for each screw-based socket on the portable luminaire. The compact fluorescent or LED lamps which are prepackaged with the portable luminaire shall be fully compatible with the luminaire controls, meaning that portable luminaires having a dimmer control shall be prepackaged with dimmable compact fluorescent or LED lamps, and portable luminaires having 3-way controls shall be prepackaged with 3-way compact fluorescent or LED lamps. The compact fluorescent lamps which are prepackaged with the luminaires shall also meet the minimum energy efficiency levels established by ENERGY STAR® for compact fluorescent lamps in effect on December 31, 2008. The LED lamps required to be packaged with the luminaire shall comply with the minimum requirements shown in Table N-3; - Be equipped with one or more single-ended, non-screw based halogen lamp sockets (line or low voltage), a dimmer control or high low control, and be rated for a maximum of 100W.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2012) | Section 1605.3 (n) Luminaires and Torchieres | January 1, 2010 |

### 1.4.3 Non-DEER Study Review

There are no applicable non-DEER studies associated with this work paper.

### 1.4.4 Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFLfix-Res | Residential | Indoor Lighting | CFL Fixtures | 16 | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

Most measures in this work paper are directly from the DEER 2014 READi tool. However, the following measures were not available from READi and were therefore created as new scaled measures in READi:

- Residential 36-Watt CFL

- Residential 65-Watt CFL

- Residential 70-Watt CFL

Table 8 contains a data file with all the READi exports. These results have not been modified and are only being included in the workpaper for reference. The savings come from the columns beginning with “ACustWB.”

Table 8 READi Tool Outputs

| Solution Code | Measure Name | READi Results |
| --- | --- | --- |
| Multiple solution codes | Multiple measures |  |

The methodology used by READi to calculate energy savings is described in the Energy Division (ED) Workpaper Disposition for Lighting Retrofits (Attachment 3, [385]). Given the measure case wattage, the base case wattage is calculated by multiplying the measure case wattage by a wattage ratio of 3.47 for Residential Non-Reflector CFLs.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Residential Single Family | RES | DEER:Indoor_CFL_Ltg |
| Residential Multi-Family | RES | DEER:Indoor_CFL_Ltg |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

ROB measures assume a single base case cost for the entire life of the measure.  Table 10 shows a sample of the base case cost data. Costs are taken from DEER 2008 and can be found in the costs spreadsheet (Attachment 2). If a measure was not covered by the DEER costs spreadsheet, the cost of the closest wattage and measure type is used.

The labor cost is $5.89 for the base case.

## 4.2 Measure Case Cost

The measure case costs are taken from DEER 2008 and can be found in the costs spreadsheet.  Table 10 shows a sample of the measure case cost data.  If a measure was not covered by the DEER costs spreadsheet, the cost of the closest wattage and measure type is used.

The labor cost is $5.89 for the measure case.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB type measures, GMC is represented by the equation below:

GMC	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

*Note: Unless stated otherwise the measure case labor and base case labor are assumed to be the same value reducing the equation to the following:

GMC = Measure Equipment Cost – Base Case Equipment Cost

Costs are taken from DEER 2008 and can be found in the costs spreadsheet. If a measure was not covered by the DEER costs spreadsheet, the cost of the closest wattage and measure type is used. Table 9 shows a sample of the calculation of the gross measure cost.

Table 10 Gross Measure Cost

| Measure | Measure Case Cost ID | Measure Cost | Base Case Cost ID | Base Case Material Cost | GMC |
| --- | --- | --- | --- | --- | --- |
| 13 Watt Plug-in Lamp CFL | D08-RE-ILtg-CFL-Tbl-20W-Rpl-Prim | $50.43 | D08-RE-ILtg-Inc-Tbl-60W | $44.24 | $6.19 |
| 36 Watt Plug-in Lamp CFL | D08-RE-ILtg-CFL-Tbl-32W-Rpl-Prim | $63.20 | D08-RE-ILtg-Inc-Tbl-120W | $58.05 | $5.15 |

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  For ROB, the IMC is the same value as GMC.

# Attachments

1.

2.

3.

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [213] | EUL/RUL Values Provided through Excel Spreadsheet |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [356] | 2012 Appliance Efficiency Regulations (Title 20) |
| [385] | Workpaper Disposition for Lighting Retrofits |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |