Work Paper SCE13LG099

Revision 1

Southern California Edison Company

Interior Integral Non Dimmable (Screw-in) CFLs

# At-a-Glance Summary

| Applicable Measure Codes: | See Table 1 |
| --- | --- |
| Measure Description: | Screw-in compact fluorescent lamps (CFLs) |
| Base Case Description: | Various low efficacy lamps including incandescent, HID, and circular fluorescent lamps |
| Energy Impact Common Units: | Lamp |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | Varies; see Table 7 |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Varies; see Table 3 |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG099.0 | No | 5/30/12 | Brian V. O’Keefe/SCE | -Original work paper draft for 2013-2014 program cycle. -Updated version of 2010-2012 work paper WPSCNRLG0099. |
|  |  |  | Alfredo Gutierrez/SCE | New measures added to original draft. |
| SCE13LG099.1 | Yes | 3/18/14 | Sara Richardson/TRC Energy Services, Jason Wang/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14.-Updated solution codes -Revised calculations -Revised costs -Added midstream |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This workpaper details the replacement of various low efficacy lamps with screw-in compact fluorescent lamps (CFLs). Table 1 shows all of the measures and their corresponding base cases contained within this workpaper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-49284 | >=27watts Integral Spiral with Reflector CFL replacing Incandescent |
| LT-84675 | >=27watts Interior Spiral CFL replacing Incandescent |
| LT-48789 | 11 Watt Interior Spiral CFL replacing Incandescent |
| LT-13420 | 13 Watt Integral Spiral CFL replacing Incandescent |
| LT-53389 | 14 Watt Interior Spiral CFL replacing Incandescent |
| LT-40123 | 14 Watt Interior Spiral with Reflector CFL replacing Incandescent |
| LT-27012 | 14-26 watts Integral Spiral with Reflector CFL replacing Incandescent |
| LT-56098 | 14-26 watts Interior Spiral CFL replacing Incandescent |
| LT-55729 | 14-28 watt Integral Spiral with Reflector CFL replacing Incandescent |
| LT-87678 | 15 Watt Integral Spiral CFL replacing Incandescent |
| LT-91827 | 16 Watt Integral Spiral CFL replacing Incandescent |
| LT-12909 | 17 Watt Integral Spiral CFL replacing Incandescent |
| LT-12938 | 18 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| LT-90212 | 18 Watt Integral Spiral CFL replacing 2 x 32 Watt Circular T12 Fluorescent |
| LT-24374 | 18 Watt Integral Spiral CFL replacing Incandescent |
| LT-29809 | 20 Watt Integral Spiral CFL replacing Incandescent |
| LT-47968 | 20 Watt Integral Spiral with Reflector CFL replacing Incandescent |
| LT-38873 | 23 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| LT-59824 | 23 Watt Integral Spiral CFL replacing 200 Watt Incandescent |
| LT-34354 | 23 Watt Integral Spiral CFL replacing Incandescent |
| LT-87321 | 23 Watt Interior Spiral with Reflector CFL replacing Incandescent |
| LT-70896 | 25 Watt Interior Spiral CFL replacing Incandescent |
| LT-59621 | 26 Watt Integral Spiral CFL replacing Incandescent |
| LT-43543 | 26 Watt Integral Spiral with Reflector CFL replacing Incandescent |
| LT-97466 | 28 Watt Integral Spiral CFL replacing Incandescent |
| LT-78466 | 32 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| LT-10933 | 32 Watt Integral Spiral CFL replacing 200 Watt Incandescent |
| LT-68569 | 32 Watt Integral Spiral CFL replacing Incandescent |
| LT-53737 | 36 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| LT-65867 | 42 Watt Integral Spiral CFL replacing 100 Watt HID |
| LT-69522 | 42 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| LT-17865 | 42 Watt Integral Spiral CFL replacing 500 Watt Incandescent |
| LT-38821 | 5-13 watts Integral Spiral CFL replacing Incandescent |
| LT-65394 | 5-13 watts Integral Spiral with Reflector CFL replacing Incandescent |
| LT-58329 | 55 Watt Integral Spiral CFL replacing 200 Watt Incandescent |
| LT-39673 | 7 Watt Integral Spiral CFL replacing Incandescent |
| LT-40785 | 9 Watt Integral Spiral CFL replacing Incandescent |
| LT-45573 | >=27watts Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-10932 | 11 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-42049 | 13 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-29743 | 13 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-62221 | 14 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-30896 | 14 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-78881 | 14-26 watts Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-63599 | 15 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-71924 | 15 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-49567 | 16 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-45002 | 18 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-89923 | 18 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-81721 | 19 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-79790 | 20 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-91534 | 20 Watt Integral Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-19354 | 23 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-45264 | 23 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-67283 | 25 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-89342 | 26 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-92843 | 26 Watt Integral Spiral with Reflector (Dwelling Area) CFL replacing Incandescent |
| LT-24832 | 27 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-75625 | 32 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-25483 | 36 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-14429 | 42 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-95436 | 55 Watt Integral Spiral (Dwelling Area) CFL replacing Incandescent |
| LT-78374 | >=27watts Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-80193 | 11 Watt Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-35634 | 13 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-98467 | 13 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-10967 | 14 Watt Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-29406 | 14 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-39825 | 14-26 watts Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-53498 | 15 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-40396 | 15 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-52647 | 16 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-58924 | 18 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-68597 | 18 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-78195 | 19 Watt Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-46756 | 20 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-89532 | 20 Watt Integral Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-60905 | 23 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-88532 | 23 Watt Interior Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-98143 | 25 Watt Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-78532 | 26 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-66545 | 26 Watt Integral Spiral with Reflector (Common Area) CFL replacing Incandescent |
| LT-10371 | 27 Watt Interior Spiral (Common Area) CFL replacing Incandescent |
| LT-58657 | 32 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-82345 | 36 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-38925 | 42 Watt Integral Spiral (Common Area) CFL replacing Incandescent |
| LT-20534 | 55 Watt Integral Spiral (Common Area) CFL replacing Incandescent |

Eligibility Requirements

The proposed lamps must be on the Energy Star qualified list and must be a one to one replacement.

Implementation Requirements

- Dwelling Area measures are eligible for Residential Single Family, Multi-family, and Mobile Home building types.

- Common Area measures are eligible for Residential Multi-family and Mobile Home building types.

- Measures that are not indicated as Dwelling Area or Common Area are eligible for all Non-residential building types.

Documentation Requirements

Direct install programs that use the actual customer existing wattages found in the field must track these wattages along with the measure used.

## 1.2 Technical Description

The measure involves the installation of a screw in compact fluorescent lamp with integrated ballast.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The measure install type is Replace on Burnout (ROB).

All measures are available through these delivery methods:

- Financial Support – Direct Install

- Financial Support – Giveaway

- Midstream Programs/Mid-Stream Incentive

Measure LT-55729 “14-28 watt Integral Spiral with Reflector CFL replacing Incandescent” is additionally available through:

- Financial Support – Downstream Incentive – Deemed

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

All of these measures are available in DEER 2014. However, savings values from READi do not exactly match savings that are calculated manually using the DEER methodology (for SCE weighted building types). In addition, DEER savings for Common Area measures are normalized to a “per-DeltaWatt” basis. For consistency, all savings are calculated using SCE’s calculation template, which uses the latest DEER interactive effects and operating hours which were released in February 2014.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER methodology was used. |
| DEER Version | DEER14 |
| DEER Run ID and Measure Name (Sample) | C-In-CFLscw-Refl(28w)-dWP71 |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for the measures is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| Measures using this NTGR | NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- | --- |
| NonRes Direct Install | NonRes-sAll-mCFL-dir | CFLs | Com | Any | DirInstall | 0.8 |
| Res Direct Install | Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Res Giveaway | Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |
| NonRes Giveaway | Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
|  | Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | All | 0.6 |
|  | Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |
| NonRes Midstream, Res Midstream | All-Default<=2yrs | All other EEM with no evaluated NTGR; new technology in program for 2 or fewer years | All | Any | All | 0.7 |
| NonRes Downstream (LT-55729 only) | NonRes-sAll-mCFL-dn | CFLs | Com | Any | PreRebDown | 0.53 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| Measures | GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- | --- |
| NonRes Direct Install, NonRes Downstream (LT-55729) | Com-CFL-SCE | Non-Res CFL; Non-Upstream Program; Annual Installation Rate | Com | Any | NonUpStrm | 0.61 |
| NonRes Giveaway, NonRes Midstream, Res Giveaway, Res Midstream | Def-GSIA | Default GSIA values | Any | Any | Any | 1.0 |
| Res Direct Install | MFm-IntCF-SCE | Interior CFL; Annual Installation Rate; Multi-family | Res | MFm | NonUpStrm | 0.72 |

*Denotes that the column is taken from the DEER GSIA Table.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor General Lighting |
| Measure Case TechGroups | Lighting - Lamps |
| Measure Case TechTypes | Integral CFL (screw-in) |
| Base Case TechGroups | Lighting - Lamps |
| Base Case TechTypes | Incandescent Lamp, Linear Fluorescent Lamp, HID Lamp with Ballast |

### 1.4.2 Codes and Standards Analysis

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2014) | Tables K-5, K-6, K-10, K-11 | January 1, 2011 |

This work paper uses ED-specified wattages and wattage ratios to determine code case wattages; these satisfy or exceed any energy consumption requirements mentioned below.

Title 24 (2013) does not apply to screw-in CFLs or any other general service lamps.

The Title 20 (2014) Appliance Efficiency Regulations [422] include standards for medium base CFLs (Table K-5), federally-regulated general service incandescent lamps (Table K-6), and state-regulated general service lamps (Table K-10, Table K-11).

### 1.4.3 Non-DEER Study Review

No Non-DEER studies were reviewed as a part of this workpaper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFL-Com | Non-Residential | Indoor Lighting | CFL Lamps - 10,000 Hour | EUL = Rated Life of Lamp (10,000 hours) / DEER Annual Occupancy Sensor Operating Hours | N/A |
| ILtg-CFL-Res | Residential | Indoor Lighting | CFL Lamps - 10,000 Hour | EUL = Rated Life of Lamp (10,000 hours) * Switching Degradation Factor (0.523) / DEER Annual Operating Hours | N/A |

# Section 2. Energy Savings & Demand Reduction Calculations

All measures in this work paper were addressed in the May 30 Energy Division Lighting Disposition (Attachment 3) and are available from DEER 2014. However, savings values from READi do not exactly match savings that are calculated manually using the DEER methodology (for SCE weighted building types). In addition, DEER savings for Common Area measures are normalized to a “per-DeltaWatt” basis. For consistency, all savings in this work paper are calculated using SCE’s calculation template, which uses the latest DEER interactive effects and operating hours which were released in February of 2014.

All measure cases wattages are from the May 30 disposition. Base case wattages are either determined by using wattage ratios or values explicitly stated in the disposition (e.g. 150 Watt Incandescent or 100 Watt HID). See Table 8 for an example of the wattages used.

Table 8 Example DEER IDs and Wattages

| Measure | READi MeasureID | Measure Wattage (W) | Wattage Ratio | Base Wattage (W) |
| --- | --- | --- | --- | --- |
| 11 Watt Interior Spiral CFL replacing Incandescent | C-In-CFLscw(11w)-dWP28 | 11 | 3.57 | 39.27 |
| 42 Watt Integral Spiral CFL replacing 100 Watt HID | C-In-CFLscw(42w)-dWP58 | 42 | N/A | 100 |
| 13 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Incandescent | R-In-CFLscw-Refl(13w)-dWP40 | 13 | 4.09 | 53.17 |
| 11 Watt Interior Spiral (Dwelling Area) CFL replacing Incandescent | R-In-CFLscw(11w)-dWP27 | 11 | 3.47 | 38.17 |
| 11 Watt Interior Spiral (Common Area) CFL replacing Incandescent | R-InCmn-CFLscw(11w)-dWP27 | 11 | 3.47 | 38.17 |

See Attachment 1 for all savings.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | NON_RES | DEER:Indoor_CFL_Ltg |
| Assembly | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Primary School | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Secondary School | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Relocatable Classroom | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Community College | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - University | NON_RES | DEER:Indoor_CFL_Ltg |
| Grocery | NON_RES | DEER:Indoor_CFL_Ltg |
| Food Store | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Hospital | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Nursing Home | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Clinic | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging - Hotel | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging - Guest Rooms | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging - Motel | NON_RES | DEER:Indoor_CFL_Ltg |
| Manufacturing - Bio/Tech | NON_RES | DEER:Indoor_CFL_Ltg |
| Manufacturing - Light Industrial | NON_RES | DEER:Indoor_CFL_Ltg |
| Industrial | NON_RES | DEER:Indoor_CFL_Ltg |
| Misc - Commercial | NON_RES | DEER:Indoor_CFL_Ltg |
| Office - Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Office - Small | NON_RES | DEER:Indoor_CFL_Ltg |
| Restaurant - Fast-Food | NON_RES | DEER:Indoor_CFL_Ltg |
| Restaurant - Sit-Down | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail - Multistory Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail - Single-Story Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail - Small | NON_RES | DEER:Indoor_CFL_Ltg |
| Storage - Conditioned | NON_RES | DEER:Indoor_CFL_Ltg |
| Storage - Unconditioned | NON_RES | DEER:Indoor_CFL_Ltg |
| Transportation - Communication - Utilities | NON_RES | DEER:Indoor_CFL_Ltg |
| Warehouse - Refrigerated | NON_RES | DEER:Indoor_CFL_Ltg |
| Residential Single Family | RES | DEER:Indoor_CFL_Ltg |
| Residential Multi-family | RES | DEER:Indoor_CFL_Ltg |
| Residential Mobile Home - Double-Wide | RES | DEER:Indoor_CFL_Ltg |

# Section 4. Base Case & Measure Costs

For Direct Install measures, SCE directly utilizes one or more contractors as part of the program.  The actual cost can vary by contractor, the date when the work occurred, and by the volume of business.  Contractor costs are confidential information and are based upon contractually agreed upon pricing as established in their purchase order with SCE; therefore, the SCE program tracking system is the only source for this data.  The data within the workpaper are not the contractor values but instead represent an approximate example of the costs that may be incurred. The cost data supplied is for informational purposes only and are not intended for reporting purposes.

## 4.1 Base Case Cost

For ROB measures, the base case cost for some sample measures is listed in Table 10. Except where noted below, these data are extracted from the 2008 DEER Measure Cost Summary [215].

Table 10 Base Case Cost

| Solution Code | Base Case DEER ID | Base Case Cost | Base Case Labor |
| --- | --- | --- | --- |
| LT-78374 | D08-NE-ILtg-Inc-100W | $0.68 | $5.89 |
| LT-49284 | D08-NE-ILtg-Inc-Rfl-100W | $3.54 | $5.89 |
| LT-80193 | D08-NE-ILtg-Inc-40W | $0.64 | $5.89 |
| LT-10932 | D08-NE-ILtg-Inc-Rfl-50W | $3.54 | $5.89 |
| LT-39825 | D08-NE-ILtg-Inc-60W | $0.68 | $5.89 |
| LT-55729 | D08-NE-ILtg-Inc-Rfl-60W | $3.54 | $5.89 |
| LT-52647 | D08-NE-ILtg-Inc-Rfl-75W | $3.54 | $5.89 |
| LT-12938 | D08-NE-ILtg-Inc-150W | $2.47 | $5.89 |
| LT-90212 | Internet* (2x32 W Circular T12) | $20.32 | $5.89 |
| LT-29809 | D08-NE-ILtg-Inc-75W | $0.68 | $5.89 |
| LT-65867 | 100wMHR128w-Rpl | $132.51 | $18.04 |
| LT-69522 | D08-NE-ILtg-Inc-150W | $2.47 | $5.89 |
| LT-17865 | Internet* (500 W Incandescent) | $5.69 | $5.89 |

*DEER does not contain the cost of these items; therefore, an online cost was used, see Attachment 2.

## 4.2 Measure Case Cost

Measure case cost for some sample measures is listed in Table 14. Except where noted below, these data are extracted from the 2008 DEER Measure Cost Summary [215]. Refer to Attachment 2 for the extrapolation methodology.

Table 11 Measure Case Cost

| Solution Code | Measure Case DEER ID | Measure Case Cost | Measure Case Labor |
| --- | --- | --- | --- |
| LT-78374 | D08-RE-InLt-CFL-Int-28W | $10.31 | $5.89 |
| LT-49284 | DEER Extrapolated from D08-NE-OLtg-CFL-Int-25wCFLsrSMg25w-Rpl | $15.51 | $5.89 |
| LT-80193 | D08-RE-InLt-CFL-Int-11W | $6.32 | $5.89 |
| LT-10932 | D08-RE-InLt-CFL-Int-11W | $6.32 | $5.89 |
| LT-39825 | D08-RE-InLt-CFL-Int-18W | $7.97 | $5.89 |
| LT-55729 | D08-NE-ILtg-CFL-Int-18wCFLsrSMg18w-Rpl | $12.02 | $5.89 |
| LT-52647 | D08-RE-InLt-CFL-Rfl-16W | $11.60 | $5.89 |
| LT-12938 | D08-NE-ILtg-CFL-Int-18wCFLsSMg18w-Rpl | $7.97 | $5.89 |
| LT-90212 | D08-NE-ILtg-CFL-Int-18wCFLsSMg18w-Rpl | $7.97 | $5.89 |
| LT-29809 | D08-NE-ILtg-CFL-Int-20wCFLsSMg20w-Rpl | $8.44 | $5.89 |
| LT-65867 | DEER Extrapolated from D08-NE-ILtg-CFL-Int-40wCFLsSMg40w-Rpl | $13.58 | $5.89 |
| LT-69522 | DEER Extrapolated from D08-NE-ILtg-CFL-Int-40wCFLsSMg40w-Rpl | $13.58 | $5.89 |
| LT-17865 | DEER Extrapolated from D08-NE-ILtg-CFL-Int-40wCFLsSMg40w-Rpl | $13.58 | $5.89 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

Gross Measure Cost is the cost to install an energy efficient measure. This definition implies two different meanings depending on the install type. It can either mean the full cost of the measure as in the case of RET or it can mean the cost premium required to install the energy efficient measure over a less efficient piece of equipment as in the case of ROB.

For ROB, GMC is represented by the equation below:

GMC = (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

Refer to Table 15 for the Gross Measure Cost.

Table 12 Gross Measure Cost

| Solution Code | Measure Name | Gross Measure Cost |
| --- | --- | --- |
| LT-78374 | >=27watts Integral Spiral (Common Area) CFL | $9.63 |  |
| LT-49284 | >=27watts Integral Spiral with Reflector CFL | $11.97 |  |
| LT-80193 | 11 Watt Integral Spiral (Common Area) CFL | $5.68 |  |
| LT-10932 | 11 Watt Integral Spiral (Dwelling Area) CFL | $2.78 |  |
| LT-39825 | 14-26 watts Integral Spiral (Common Area) CFL | $7.29 |  |
| LT-55729 | 14-28W Integral Spiral with Reflector CFL | $8.48 |  |
| LT-52647 | 16 Watt Integral Spiral with Reflector (Common Area) | $8.06 |  |
| LT-12938 | 18 Watt Integral Spiral CFL replacing 150 Watt Incandescent | $5.50 |  |
| LT-90212 | 18 Watt Integral Spiral CFL | ($8.30) |  |
| LT-29809 | 20 Watt Integral Spiral CFL | $7.76 |  |
| LT-65867 | 42 Watt Integral Spiral CFL replacing 100 Watt HID | ($118.93) |  |
| LT-69522 | 42 Watt Integral Spiral CFL replacing 150 Watt Incandescent | $11.11 |  |
| LT-17865 | 42 Watt Integral Spiral CFL replacing 500 Watt Incandescent | $7.89 |  |

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  For ROB measures, IMC is the same as the Gross Measure Cost. See Table 12 for the Incremental Measure Cost.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [215] | Revised DEER Measure Cost Summary |
| [351] | Energy Efficiency Policy Manual-Version 5 |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |
| [436] | 2014 DEER EUL Table - Update |

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |