Work Paper SCE13LG085

Revision 3

Southern California Edison Company

Interior Compact Fluorescent Fixture

# At-a-Glance Summary

| Applicable Measure Codes: | LT-17083, LT-83701, LT-33489, LT-26133, LT-84901, LT-39256, LT-79504, LT-93432, LT-19423, LT-60765, LT-68432, LT-40994, LT-80989, LT-89854, LT-97876, LT-24867, LT-39075, LT-73649, LT-81657, LT-59687, LT-23209, LT-45765, LT-59876, LT-79523, LT-87943, LT-35476, LT-49660, LT-62735, LT-49593, LT-50587, LT-67654, LT-78765, LT-85876, LT-90987, LT-19143, LT-27559, LT-34841, LT-72843, LT-38978, LT-45343, LT-56473, LT-65832, LT-78567, LT-87453, and LT-90776. |
| --- | --- |
| Measure Description: | Replacement of various low efficacy fixtures (i.e. <100 Watt lamp base) with compact fluorescent fixtures (i.e. 70 Watt Interior Fixture CFL) in various target sectors. |
| Base Case Description: | The specific measures containing wattage ranges (i.e., <100 Watt lamp base case) are assumed to have a metal halide base case while all other measures for multi-family housing (Common and Dwelling area) are assumed to have an incandescent base case. |
| Energy Impact Common Units: | Per Fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | EUL of 12 years for Non-Residential measures and EUL of 16 years for residential measures. |
| Measure Application Type: | Early Retirement (RET) and Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Downstream Com 0.53, Ag/Ind 0.6, Res 0.55, Midstream Com/Ag/Ind 0.6, Res 0.55, DirectInstall Com 0.8, Ag/Ind/Res 0.85 |
| Important Comments: | This workpaper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG085.0 | Yes | 04/09/12 | Kristine Falletta/PECI Brian V. O’Keefe/SCE | Original work paper template for 2013 PC Updates from original draft: - Extracted measures directly from READi Tool for the dwelling areas - Updated to the newest calculation template, version 2.2 - Updated measure names to reflect new WRR values |
| SCE13LG085.1 | Yes | 07/30/13 | Yun Han/SCE | - Updated range measures to lowest value based on ED’s disposition - T24 code language updated to 2013 - Delivery & Incentive Mechanism language corrected - Deleted unused NTGs |
| SCE13LG085.2 | Yes | 12/04/13 | Yun Han/SCE | - Added 10 missing measures from Rev 0 - Updated cost section that showed IMC in GMC section - Removed Residential Building Types from Non-Res measures |
| SCE13LG085.3 | Yes | 03/28/14 | David Pruitt/PL Energy | - New template for 2015 program year - Used measure Wattages from the ED lighting disposition table dated 30 May, 2014 - Included the new READI fields. |
|  |  | 08/04/14 | Jack Melnyk/SCE | - WRR of CFL Fixtures updated from 3.47 to 3.53, including measure names - Updated 64W Interior Common Area measure SC from LT-27559 to LT-72843 - Adjusted CFL Fixture savings based on DEER operating hours from Motel Corridor to Common Areas for Common Area measures - Removed READI savings and used DEER methodology for Dwelling measures - Added code baseline for HID measures - Workpaper updated for reporting period, effective 7/1/2014-12/31/2014 |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of various low efficacy fixtures (i.e. <100 Watt lamp base case) with compact fluorescent fixtures (i.e. 70 Watt Interior Fixture CFL) in various target sectors.  Table 1 shows the list of core measures included in this work paper by solution code.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-17083 | Up to 70 Watt Interior Fixture CFL replacing less than 100 Watt lamp base case |
| LT-83701 | Up to 128 Watt Interior Fixture CFL replacing 101 - 175 Watt lamp base case |
| LT-33489 | Up to 192 Watt Interior Fixture CFL replacing 176 - 399 Watt lamp base case |
| LT-26133 | Up to 244 Watt (Tier 1) Interior Fixture CFL replacing greater than 400 Watt lamp base case |
| LT-84901 | Up to 360 Watt (Tier 2) Interior Fixture CFL replacing greater than 400 Watt lamp base case |
| LT-79504 | 16 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 56.48 |
| LT-19423 | 22 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 77.66 |
| LT-68432 | 23 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 81.19 |
| LT-80989 | 25 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 88.25 |
| LT-97876 | 26 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 91.78 |
| LT-39075 | 27 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 95.31 |
| LT-81657 | 30 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 105.9 |
| LT-23209 | 32 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 112.96 |
| LT-59876 | 36 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 127.08 |
| LT-87943 | 39 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 137.67 |
| LT-49660 | 40 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 141.2 |
| LT-49593 | 44 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 155.32 |
| LT-67654 | 46 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 162.38 |
| LT-85876 | 52 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 183.56 |
| LT-19143 | 54 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 190.62 |
| LT-34841 | 55 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 194.15 |
| LT-38978 | 64 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 225.92 |
| LT-56473 | 69 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 243.57 |
| LT-78567 | 72 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 254.16 |
| LT-90776 | 92 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 324.76 |
| LT-39256 | 16 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 56.48 |
| LT-93432 | 22 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 77.66 |
| LT-50765 | 23 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 81.19 |
| LT-40994 | 25 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 88.25 |
| LT-89854 | 26 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 91.78 |
| LT-24867 | 27 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 95.31 |
| LT-73649 | 30 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 105.9 |
| LT-59687 | 32 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 112.96 |
| LT-45765 | 36 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 127.08 |
| LT-79523 | 39 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 137.67 |
| LT-35476 | 40 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 141.2 |
| LT-62735 | 44 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 155.32 |
| LT-50587 | 46 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 162.38 |
| LT-78765 | 52 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 183.56 |
| LT-90987 | 54 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 190.62 |
| LT-27559 | 55 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 194.15 |
| LT-72843 | 64 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 225.92 |
| LT-45343 | 69 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 243.57 |
| LT-65832 | 72 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 254.16 |
| LT-87453 | 92 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 324.76 |

The specific measures containing wattage ranges (i.e., <100 Watt lamp base case) are assumed to have a metal halide base case while all other measures for multi-family housing (Common and Dwelling area) are assumed to have an incandescent base case. The residential multifamily and mobile home measures assume either an interior common area or indoor dwelling area.  Each measure described in this work paper applies to either nonresidential buildings or multifamily residential buildings.  Single family residential buildings are not included in this work paper.

## 1.2 Technical Description

This work paper details the replacement of various low efficacy fixtures (i.e. <100 Watt lamp base case) with compact fluorescent fixtures (i.e. 70 Watt Interior Fixture CFL) in various target sectors.

A replacement of Incandescent and/or HID lamps with CFLs that have similar lumen output results in measurable demand and energy savings.

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The delivery methods are Financial Support - Down-Stream Incentive – Deemed and Financial Support - Direct Install for both residential and nonresidential applications.

Midstream Programs – Mid-Stream Incentive is for Nonresidential measures only.

The measure install type is RET for direct install and ROB for the other delivery methods.

## 1.4 MEASURE and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

The specific measures using wattage ranges (<100 Watt lamp base case, for example) for the base cases are not included in the Database for Energy Efficient Resources (DEER) Version 2011.4.01 [49], although the database contains similar interior lighting retrofit measures (Com-Lighting InGen_CFLratio0353_CFLfixt-28W). The measure ID given is for a 70 Watt CFL; however, DEER uses an incandescent base case, while this work paper assumes a metal halide base case. Also, the wattage levels included in this work paper exceed what DEER provides.

For measures not using ranges, DEER does include similar measures in the Database for Energy Efficient Resources (DEER) Version 2011.4.01 (Com-Lighting-InGen_CFLratio0353_CFLfixt-28W). This work paper utilizes the standard DEER operating hours to model residential multifamily dwelling areas, but uses other DEER building types to approximate the operating hours for residential multifamily common areas.

The differences from DEER are summarized in Table 2 below.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | Yes |
| Scaled DEER Measure | Yes |
| DEER Building Prototypes Used | Yes |
| Deviation from DEER | For the multifamily measures this workpaper assumes different operating hours for common areas. For the nonresidential measures this workpaper assumes a metal halide base case. These base cases are more conservative than the DEER WRR |
| DEER Version | DEER 2011 |
| DEER Run ID and Measure Name (Sample) | Com-Lighting-InGen_CFLratio0353_CFLfixt-28W |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mCFL-dir | CFLs | Com | Any | DirInstall | 0.8 |
| Ind-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |
| Res-Default-HTG-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| NonRes-sAll-mCFL-dn | CFLs | Com | Any | PreRebDown | 0.53 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | All | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | All | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.6 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | All | 0.55 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the residential and non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer to the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor General Lighting, Indoor Common Area Lighting |
| Measure Case TechGroups | Lighting – Fixtures |
| Measure Case TechTypes | CFL Fixture |
| Base Case TechGroups | Lighting – Fixtures |
| Base Case TechTypes | HID Fixture, Fixture |

### 1.4.2 Codes and Standards Analysis

This measure is not addressed in the Title 20 2014Appliance Efficiency Regulations [422].

In Section 140.0(b)2I ii and iii of California’s Title 24 2013 Non-Residential Building Energy Efficiency Standards [355], the Alteration codes and standards language states:

Lighting System Alterations shall meet the applicable requirements in TABLE 141.0-E and the following:

- Lighting System Alterations include alterations where an existing lighting system is modified, luminaires are replaced, or luminaires are disconnected from the circuit, removed and reinstalled, whether in the same location or installed elsewhere.

EXCEPTION 1 to Section 141.0(b)2Iii: Alterations that qualify as a Luminaire Modification-in-

Place.

EXCEPTION 2 to Section 141.0(b)2Iii: Portable luminaires, luminaires affixed to moveable partitions, and lighting excluded in accordance to Section 140.6(a)3.

Luminaire Modifications-in-Place shall meet the applicable requirements in TABLE 141.0-F and the following:

- To qualify as a Luminaire Modification-in-Place, luminaires shall only be modified by one or more of the following methods:

  - Replacing lamps and ballasts with like type or quantity in a manner that preserves the original luminaire listing.

  - Changing the number or type of light source in a luminaire including: socket renewal, removal or relocation of sockets or lampholders, and/or related wiring internal to the luminaire including the addition of safety disconnecting devices.

  - Changing the optical system of a luminaire in part or in whole.

  - Replacement of whole luminaires one for one in which the only electrical modification involves disconnecting the existing luminaire and reconnecting the replacement luminaire.

- Luminaire Modifications-In-Place shall include only alterations to lighting system meeting the following conditions:

  - Luminaire Modifications-in-Place shall not be part of or the result of any general remodeling or renovation of the enclosed space in which they are located.

  - Luminaire Modifications-in-Place shall not cause, be the result of, or involve any changes to the panelboard or branch circuit wiring, including line voltage switches, relays, contactors, dimmers and other control devices, providing power to the lighting system.

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition

of occupancy or vacancy sensors

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Building Energy Efficiency Standards, Section 140.0(b) | January 1, 2014 |
| Title 20 (2014) | N/A | N/A |

### 1.4.3 Non-DEER Study Review

No non-DEER studies or data were reviewed for this Workpaper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFLfix-Com | Non-Residential | Indoor Lighting | CFL Fixtures | 12 | 4 |
| ILtg-CFLfix-Res | REsidential | Indoor Lighting | CFL Fixtures | 16 | 5.3 |

# Section 2. Energy Savings & Demand Reduction Calculations

Table 8 contains measures that utilize the 3.53 Wattage Reduction Ratio (WRR) as directed by the May 30, 2014 ED Disposition [].

Table 8  Measures with WRR

| Solution Code | Measure Name |
| --- | --- |
| LT-79504 | 16 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 56.48 |
| LT-19423 | 22 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 77.66 |
| LT-68432 | 23 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 81.19 |
| LT-80989 | 25 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 88.25 |
| LT-97876 | 26 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 91.78 |
| LT-39075 | 27 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 95.31 |
| LT-81657 | 30 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 105.9 |
| LT-23209 | 32 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 112.96 |
| LT-59876 | 36 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 127.08 |
| LT-87943 | 39 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 137.67 |
| LT-49660 | 40 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 141.2 |
| LT-49593 | 44 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 155.32 |
| LT-67654 | 46 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 162.38 |
| LT-85876 | 52 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 183.56 |
| LT-19143 | 54 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 190.62 |
| LT-34841 | 55 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 194.15 |
| LT-38978 | 64 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 225.92 |
| LT-56473 | 69 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 243.57 |
| LT-78567 | 72 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 254.16 |
| LT-90776 | 92 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 324.76 |
| LT-39256 | 16 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 56.48 |
| LT-93432 | 22 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 77.66 |
| LT-50765 | 23 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 81.19 |
| LT-40994 | 25 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 88.25 |
| LT-89854 | 26 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 91.78 |
| LT-24867 | 27 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 95.31 |
| LT-73649 | 30 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 105.9 |
| LT-59687 | 32 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 112.96 |
| LT-45765 | 36 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 127.08 |
| LT-79523 | 39 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 137.67 |
| LT-35476 | 40 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 141.2 |
| LT-62735 | 44 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 155.32 |
| LT-50587 | 46 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 162.38 |
| LT-78765 | 52 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 183.56 |
| LT-90987 | 54 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 190.62 |
| LT-27559 | 55 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 194.15 |
| LT-72843 | 64 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 225.92 |
| LT-45343 | 69 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 243.57 |
| LT-65832 | 72 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 254.16 |
| LT-87453 | 92 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 324.76 |

The non-dwelling/non-common area measures are categorized into five different wattage levels based on average common lighting replacements which are retrofitted using Compact Fluorescent Fixtures.  For the purpose of calculating the energy savings for measures with ranges, it is assumed that the base case is always a standard metal halide. Table 9 shows the base case and measure wattage levels for the five measures discussed.  The assumed base case fixture wattage was taken from the 2013 SPC Standard Fixture Watts [431].  The CFL fixture replacement wattage was based on ED disposition [A].

Table 9 Base Case and Measure Wattage

| Solution Code | Description | Assumed Base Case Standard MH Wattage, including ballast (Watts) | Code Baseline | CFL Fixture Replacement Wattage based on ED Disposition (Watts) |
| --- | --- | --- | --- | --- |
| LT-17083 | Up to 70 Watt Interior Fixture CFL replacing less than 100 Watt lamp base case | 95W (based on a 70W MH lamp) – MH70/1 | 95W (based on a 70W MH lamp) – MH70/1 | 70 |
| LT-83701 | Up to 128 Watt Interior Fixture CFL replacing 101 - 175 Watt lamp base case | 190W (based on a 150W MH lamp) – MH150/1 | 190W* (based on a 150W PSMH lamp) – PSMH150/1 | 128 |
| LT-33489 | Up to 192 Watt Interior Fixture CFL replacing 176 - 399 Watt lamp base case | 295W (based on a 250W MH lamp) – MH250/1 | 232W* (based on a 200W PSMH lamp) – PSMH200/1 | 192 |
| LT-26133 | Up to 244 Watt (Tier 1) Interior Fixture CFL replacing greater than 400 Watt lamp base case | 458W (based on a 400W MH lamp) – MH400/1 | 400W* (based on a 350W PSMH lamp) – PSMH350/1 | 244 |
| LT-84901 | Up to 360 Watt (Tier 2) Interior Fixture CFL replacing greater than 400 Watt lamp base case | 458W (based on a 400W MH lamp) – MH400/1 | 456W* (based on a 400W PSMH lamp) – PSMH400/1 | 360 |

*This is the wattage for a code case fixture that is lumen equivalent to the base case fixture. This value differs from the May 30 disposition value.

The example below shows the energy savings calculation for replacing a 95W metal halide with a 70W CFL fixture for the Agricultural building type in CZ6.

Equation 1

1st baseline

2nd baseline

The demand difference (Watts per unit) is simply the difference between the electric demand of the base unit and the electric demand of the energy efficient unit multiplied by the coincident diversity factor and demand interactive effect as shown in Equation 2.  Similar to the energy savings, the demand difference is calculated for both the 1st baseline period and 2nd baseline period for retrofit measures.

The example below shows the demand reduction calculation for replacing a 95W metal halide with a 70W CFL fixture for Agricultural building type in CZ6.

Equation 2

1st baseline

2nd baseline

The complete list of energy and demand savings calculations can be found in the attached calculation spreadsheet [].

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_Non-CFL_Ltg load shape.  See Table 10 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 10 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | NON_RES | DEER:Indoor_CFL_Ltg |
| Assembly | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Primary School | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Secondary School | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Relocatable Classroom | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - Community College | NON_RES | DEER:Indoor_CFL_Ltg |
| Education - University | NON_RES | DEER:Indoor_CFL_Ltg |
| Grocery | NON_RES | DEER:Indoor_CFL_Ltg |
| Food Store | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Hospital | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Nursing Home | NON_RES | DEER:Indoor_CFL_Ltg |
| Health/Medical - Clinic | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging – Hotel | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging - Guest Rooms | NON_RES | DEER:Indoor_CFL_Ltg |
| Lodging – Motel | NON_RES | DEER:Indoor_CFL_Ltg |
| Manufacturing - Bio/Tech | NON_RES | DEER:Indoor_CFL_Ltg |
| Manufacturing - Light Industrial | NON_RES | DEER:Indoor_CFL_Ltg |
| Industrial | NON_RES | DEER:Indoor_CFL_Ltg |
| Misc - Commercial | NON_RES | DEER:Indoor_CFL_Ltg |
| Office – Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Office – Small | NON_RES | DEER:Indoor_CFL_Ltg |
| Restaurant - Fast-Food | NON_RES | DEER:Indoor_CFL_Ltg |
| Restaurant - Sit-Down | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail - Multistory Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail – Single-Story Large | NON_RES | DEER:Indoor_CFL_Ltg |
| Retail – Small | NON_RES | DEER:Indoor_CFL_Ltg |
| Storage - Conditioned | NON_RES | DEER:Indoor_CFL_Ltg |
| Storage - Unconditioned | NON_RES | DEER:Indoor_CFL_Ltg |
| Transportation - Communication – Utilities | NON_RES | DEER:Indoor_CFL_Ltg |
| Warehouse – Refrigerated | NON_RES | DEER:Indoor_CFL_Ltg |
| Residential – Multi-Family | RES | DEER:Indoor_CFL_Ltg |
| Residential – Mobile Home | RES | DEER:Indoor_CFL_Ltg |

# Section 4. Base Case & Measure Costs

Direct Install measures use program tracking systems for measure costs.

DEER 2011 base case and measure cost values are unchanged from DEER 2008. All costs with climate zone multipliers applied can be found in Attachment 1.

## 4.1 Base Case Cost

For the non-residential measures, the base cost is referenced from DEER 2008 [215].

For the measures with no ranges (incandescent base case), DEER 2008 [215] incremental cost data was extrapolated in order to back calculate the base case equipment cost [.  DEER08 cost data only provides fixture cost for outdoor fixtures which were used as a proxy to the indoor fixtures in this work paper.

Table 11 Base Case Cost

| Solution Code | Measure Name | Base Equipment Cost | Base Labor Cost | Base Case Cost |
| --- | --- | --- | --- | --- |
| LT-17083 | Up to 70 Watt Interior Fixture CFL replacing less than 100 Watt lamp base case | $127.35* | $18.04 | $145.39 |
| LT-83701 | Up to 128 Watt Interior Fixture CFL replacing 101 - 175 Watt lamp base case | $137.37 | $72.17 | $209.54 |
| LT-33489 | Up to 192 Watt Interior Fixture CFL replacing 176 - 399 Watt lamp base case | $152.84* | $72.17 | $225.01 |
| LT-26133 | Up to 244 Watt (Tier 1) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $164.40* | $72.17 | $236.57 |
| LT-84901 | Up to 360 Watt (Tier 2) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $164.40* | $72.17 | $236.57 |
| LT-39256 | 16 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 56.48 | $19.22 | $18.04 | $37.26 |
| LT-79504 | 16 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 56.48 | $19.22 | $18.04 | $40.23 |
| LT-93432 | 22 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 77.66 | $22.19 | $18.04 | $40.73 |
| LT-19423 | 22 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 77.66 | $22.19 | $18.04 | $41.71 |
| LT-50765 | 23 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 81.19 | $22.69 | $18.04 | $41.84 |
| LT-68432 | 23 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 81.19 | $22.69 | $18.04 | $42.71 |
| LT-40994 | 25 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 88.25 | $23.67 | $18.04 | $44.19 |
| LT-80989 | 25 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 88.25 | $23.67 | $18.04 | $45.18 |
| LT-89854 | 26 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 91.78 | $23.80 | $18.04 | $47.16 |
| LT-97876 | 26 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 91.78 | $23.80 | $18.04 | $48.64 |
| LT-24867 | 27 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 95.31 | $24.66 | $18.04 | $49.14 |
| LT-39075 | 27 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 95.31 | $24.66 | $18.04 | $51.11 |
| LT-73649 | 30 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 105.9 | $26.15 | $18.04 | $52.11 |
| LT-81657 | 30 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 105.9 | $26.15 | $18.04 | $55.07 |
| LT-59687 | 32 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 112.96 | $27.14 | $18.04 | $56.06 |
| LT-23209 | 32 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 112.96 | $27.14 | $18.04 | $56.56 |
| LT-45765 | 36 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 127.08 | $29.12 | $18.04 | $61.01 |
| LT-59876 | 36 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 127.08 | $29.12 | $18.04 | $63.48 |
| LT-79523 | 39 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 137.67 | $30.60 | $18.04 | $64.97 |
| LT-87943 | 39 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 137.67 | $30.60 | $18.04 | $74.86 |
| LT-35476 | 40 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 141.2 | $31.10 | $18.04 | $37.26 |
| LT-49660 | 40 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 141.2 | $31.10 | $18.04 | $40.23 |
| LT-62735 | 44 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 155.32 | $33.07 | $18.04 | $40.73 |
| LT-49593 | 44 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 155.32 | $33.07 | $18.04 | $41.71 |
| LT-50587 | 46 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 162.38 | $34.06 | $18.04 | $41.84 |
| LT-67654 | 46 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 162.38 | $34.06 | $18.04 | $42.71 |
| LT-78765 | 52 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 183.56 | $37.03 | $18.04 | $44.19 |
| LT-85876 | 52 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 183.56 | $37.03 | $18.04 | $45.18 |
| LT-90987 | 54 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 190.62 | $38.02 | $18.04 | $47.16 |
| LT-19143 | 54 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 190.62 | $38.02 | $18.04 | $48.64 |
| LT-27559 | 55 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 194.15 | $38.52 | $18.04 | $49.14 |
| LT-34841 | 55 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 194.15 | $38.52 | $18.04 | $51.11 |
| LT-72843 | 64 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 225.92 | $42.97 | $18.04 | $52.11 |
| LT-38978 | 64 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 225.92 | $42.97 | $18.04 | $55.07 |
| LT-45343 | 69 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 243.57 | $45.44 | $18.04 | $56.06 |
| LT-56473 | 69 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 243.57 | $45.44 | $18.04 | $56.56 |
| LT-65832 | 72 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 254.16 | $46.93 | $18.04 | $61.01 |
| LT-78567 | 72 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 254.16 | $46.93 | $18.04 | $63.48 |
| LT-87453 | 92 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 324.76 | $56.82 | $18.04 | $64.97 |
| LT-90776 | 92 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 324.76 | $56.82 | $18.04 | $74.86 |

*DEER Costs

## 4.2 Measure Case Cost

The costs of the CFL fixtures (with ranges in the measure name) were based on talking with various manufacturers [See attachments , ,]. For all other measures, the gross measure cost was taken (extrapolated) from DEER 2008 [215] and was used to derive the measure equipment cost. The cost of the labor was based on the cost of labor for installing the base case metal halide fixtures from DEER 2008 [215].  The complete Gross Measure Cost (GMC) including calculations along with the DEER defined climate zone multipliers can be found in the attachments.

Table 12 Measure Case Cost

| Solution Code | Measure Name | Measure Equipment Cost | Measure Labor Cost | Measure Case Cost |
| --- | --- | --- | --- | --- |
| LT-17083 | Up to 70 Watt Interior Fixture CFL replacing less than 100 Watt lamp base case | $111.00 | $18.04 | $129.04 |
| LT-83701 | Up to 128 Watt Interior Fixture CFL replacing 101 - 175 Watt lamp base case | $310.00 | $72.17 | $382.17 |
| LT-33489 | Up to 192 Watt Interior Fixture CFL replacing 176 - 399 Watt lamp base case | $310.00 | $72.17 | $382.17 |
| LT-26133 | Up to 244 Watt (Tier 1) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $225.00 | $72.17 | $297.17 |
| LT-84901 | Up to 360 Watt (Tier 2) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $385.00 | $72.17 | $457.17 |
| LT-39256 | 16 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 56.48 | $21.52 | $18.04 | $39.56 |
| LT-79504 | 16 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 56.48 | $21.52 | $18.04 | $42.86 |
| LT-93432 | 22 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 77.66 | $24.81 | $18.04 | $43.41 |
| LT-19423 | 22 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 77.66 | $24.81 | $18.04 | $44.50 |
| LT-50765 | 23 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 81.19 | $25.36 | $18.04 | $45.05 |
| LT-68432 | 23 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 81.19 | $25.36 | $18.04 | $45.60 |
| LT-40994 | 25 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 88.25 | $26.46 | $18.04 | $47.25 |
| LT-80989 | 25 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 88.25 | $26.46 | $18.04 | $48.34 |
| LT-89854 | 26 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 91.78 | $27.01* | $18.04 | $50.54 |
| LT-97876 | 26 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 91.78 | $27.01* | $18.04 | $52.18 |
| LT-24867 | 27 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 95.31 | $27.56 | $18.04 | $52.73 |
| LT-39075 | 27 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 95.31 | $27.56 | $18.04 | $54.92 |
| LT-73649 | 30 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 105.9 | $29.20 | $18.04 | $56.02 |
| LT-81657 | 30 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 105.9 | $29.20 | $18.04 | $59.31 |
| LT-59687 | 32 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 112.96 | $30.30 | $18.04 | $60.41 |
| LT-23209 | 32 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 112.96 | $30.30 | $18.04 | $60.96 |
| LT-45765 | 36 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 127.08 | $32.49 | $18.04 | $65.89 |
| LT-59876 | 36 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 127.08 | $32.49 | $18.04 | $68.63 |
| LT-79523 | 39 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 137.67 | $34.14 | $18.04 | $70.28 |
| LT-87943 | 39 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 137.67 | $34.14 | $18.04 | $81.25 |
| LT-35476 | 40 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 141.2 | $34.69 | $18.04 | $39.56 |
| LT-49660 | 40 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 141.2 | $34.69 | $18.04 | $42.86 |
| LT-62735 | 44 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 155.32 | $36.88 | $18.04 | $43.41 |
| LT-49593 | 44 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 155.32 | $36.88 | $18.04 | $44.50 |
| LT-50587 | 46 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 162.38 | $37.98 | $18.04 | $45.05 |
| LT-67654 | 46 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 162.38 | $37.98 | $18.04 | $45.60 |
| LT-78765 | 52 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 183.56 | $41.27 | $18.04 | $47.25 |
| LT-85876 | 52 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 183.56 | $41.27 | $18.04 | $48.34 |
| LT-90987 | 54 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 190.62 | $42.36 | $18.04 | $50.54 |
| LT-19143 | 54 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 190.62 | $42.36 | $18.04 | $52.18 |
| LT-27559 | 55 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 194.15 | $42.91 | $18.04 | $52.73 |
| LT-34841 | 55 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 194.15 | $42.91 | $18.04 | $54.92 |
| LT-72843 | 64 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 225.92 | $47.85 | $18.04 | $56.02 |
| LT-38978 | 64 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 225.92 | $47.85 | $18.04 | $59.31 |
| LT-45343 | 69 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 243.57 | $50.59 | $18.04 | $60.41 |
| LT-56473 | 69 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 243.57 | $50.59 | $18.04 | $60.96 |
| LT-65832 | 72 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 254.16 | $52.23 | $18.04 | $65.89 |
| LT-78567 | 72 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 254.16 | $52.23 | $18.04 | $68.63 |
| LT-87453 | 92 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 324.76 | $63.20 | $18.04 | $70.28 |
| LT-90776 | 92 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 324.76 | $63.20 | $18.04 | $81.25 |

*DEER Costs

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For Retrofit, 1st baseline, the Gross Measure Cost is the Measure Case Cost as shown in Table 12.

For the Retrofit 2nd Baseline and ROB measures, the Gross Measure Cost is defined as follows:

GMC 	= Measure Equipment Cost – Base Case Equipment Cost

The GMC values are shown below in Table 13.

Table 13 Gross Measure Cost

| Solution Code | Measure Name | Base Equipment Cost | Measure Equipment Cost | Gross Measure Cost |
| --- | --- | --- | --- | --- |
| LT-17083 | Up to 70 Watt Interior Fixture CFL replacing less than 100 Watt lamp base case | $127.35 | $111.00 | $16.35 |
| LT-83701 | Up to 128 Watt Interior Fixture CFL replacing 101 - 175 Watt lamp base case | $137.37 | $310.00 | -$172.63 |
| LT-33489 | Up to 192 Watt Interior Fixture CFL replacing 176 - 399 Watt lamp base case | $152.84 | $310.00 | -$157.16 |
| LT-26133 | Up to 244 Watt (Tier 1) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $164.40 | $225.00 | -$60.60 |
| LT-84901 | Up to 360 Watt (Tier 2) Interior Fixture CFL replacing greater than 400 Watt lamp base case | $164.40 | $385.00 | -$220.60 |
| LT-39256 | 16 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 56.48 | $19.22 | $21.52 | -$2.30 |
| LT-79504 | 16 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 56.48 | $22.19 | $24.81 | -$2.62 |
| LT-93432 | 22 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 77.66 | $22.69 | $25.36 | -$2.68 |
| LT-19423 | 22 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 77.66 | $23.67 | $26.46 | -$2.79 |
| LT-50765 | 23 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 81.19 | $23.80 | $27.01 | -$3.21 |
| LT-68432 | 23 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 81.19 | $24.66 | $27.56 | -$2.89 |
| LT-40994 | 25 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 88.25 | $26.15 | $29.20 | -$3.05 |
| LT-80989 | 25 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 88.25 | $27.14 | $30.30 | -$3.16 |
| LT-89854 | 26 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 91.78 | $29.12 | $32.49 | -$3.38 |
| LT-97876 | 26 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 91.78 | $30.60 | $34.14 | -$3.54 |
| LT-24867 | 27 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 95.31 | $31.10 | $34.69 | -$3.59 |
| LT-39075 | 27 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 95.31 | $33.07 | $36.88 | -$3.81 |
| LT-73649 | 30 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 105.9 | $34.06 | $37.98 | -$3.91 |
| LT-81657 | 30 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 105.9 | $37.03 | $41.27 | -$4.24 |
| LT-59687 | 32 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 112.96 | $38.02 | $42.36 | -$4.34 |
| LT-23209 | 32 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 112.96 | $38.52 | $42.91 | -$4.40 |
| LT-45765 | 36 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 127.08 | $42.97 | $47.85 | -$4.88 |
| LT-59876 | 36 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 127.08 | $45.44 | $50.59 | -$5.15 |
| LT-79523 | 39 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 137.67 | $46.93 | $52.23 | -$5.31 |
| LT-87943 | 39 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 137.67 | $56.82 | $63.20 | -$6.38 |
| LT-35476 | 40 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 141.2 | $19.22 | $21.52 | -$2.30 |
| LT-49660 | 40 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 141.2 | $22.19 | $24.81 | -$2.62 |
| LT-62735 | 44 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 155.32 | $22.69 | $25.36 | -$2.68 |
| LT-49593 | 44 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 155.32 | $23.67 | $26.46 | -$2.79 |
| LT-50587 | 46 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 162.38 | $23.80 | $27.01 | -$3.21 |
| LT-67654 | 46 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 162.38 | $24.66 | $27.56 | -$2.89 |
| LT-78765 | 52 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 183.56 | $26.15 | $29.20 | -$3.05 |
| LT-85876 | 52 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 183.56 | $27.14 | $30.30 | -$3.16 |
| LT-90987 | 54 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 190.62 | $29.12 | $32.49 | -$3.38 |
| LT-19143 | 54 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 190.62 | $30.60 | $34.14 | -$3.54 |
| LT-27559 | 55 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 194.15 | $31.095 | $34.69 | -$3.59 |
| LT-34841 | 55 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 194.15 | $33.07 | $36.88 | -$3.81 |
| LT-72843 | 64 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 225.92 | $34.06 | $37.98 | -$3.91 |
| LT-38978 | 64 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 225.92 | $37.03 | $41.27 | -$4.24 |
| LT-45343 | 69 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 243.57 | $38.02 | $42.36 | -$4.34 |
| LT-56473 | 69 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 243.57 | $38.52 | $42.91 | -$4.40 |
| LT-65832 | 72 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 254.16 | $42.97 | $47.85 | -$4.88 |
| LT-78567 | 72 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 254.16 | $45.44 | $50.59 | -$5.15 |
| LT-87453 | 92 Watt Interior Fixture (Common Area) CFL replacing Incandescent Average Watts = 324.76 | $46.93 | $52.23 | -$5.31 |
| LT-90776 | 92 Watt Interior Fixture (Dwelling Area) CFL replacing Incandescent Average Watts = 324.76 | $56.82 | $63.20 | -$6.38 |

*DEER Costs

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the same as Gross Measure Cost as shown in Section 4.3.1, Table 13.

# Attachments

1.  2. 3. 4.5.6.

# References

[31]

[49]

[215]

[351]

[355]

[422]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

[] Attachment 2- May 30, 2014 ED Disposition

[] Attachment 1 – Calculation Template 2015 v4.xlsm

[] Attachment 3 – Cost.xlsx

[] Attachment 4 – Vendor 1 Cost.pdf

[] Attachment 5 – Vendor 2 Cost.pdf

[] Attachment 6 – Vendor 3 Cost.pdf