Work Paper SCE13LG086

Revision 2

Southern California Edison

Interior Linear Fluorescent Fixture

For Work Paper Reviewer Use Only

List all major comments that occurred during the review. This table may only be removed during management review.

| Major Comment | Reviewer Name | Date | Outcome/Resolution |
| --- | --- | --- | --- |

# At-a-Glance Summary

| Measure Codes | LT-41199, LT-12866, LT-84912, LT-26100, LT-55943, LT-92448, LT-38287, LT-10232, LT-38754, LT-39676 |
| --- | --- |
| Measure Description | linear fluorescent fixtures (T5, or T5HO) |
| Base Case Description | The existing interior lights being replaced must be incandescent, mercury vapor, standard metal halide or high pressure sodium. |
| Units | Per fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | EUL varies by building type. Refer to Section 1.4.1 |
| Measure Installation Type | Early Retirement (ER) and Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Refer to Section 1.4.1 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/24/12 | Peter Davi/SCE | Updated to latest WP template v0.01 from WPSCNRLG0086 |
| 0 | 04/24/12 | Brian V. O’keefe/SCE | Changes from original draft: - 24 Hour measure removed from the workpaper - Revised the RUDD costing method to a DEER approach |
| 1 | 07/08/14 | David Pruitt/PL Energy | - Used measure wattages from the ED lighting disposition table dated 30 May, 2014 - Included the new READI fields |
| 1 | 07/08/14 | Jack Melnyk/SCE | - Added Midstream Programs-Midstream Incentive - Updated for reporting period, effective 7/1/2014 – 12/31/2014 |
| 2 | 09/15/15 | Ryan Cho/SCE | - Updated for DEER2016 HOU, CDF, NTG, EUL - Added HTR NTG & Implementation language - Removed following building types: Agricultural, Food Store, Health/Medical – Clinic, Industrial, Misc – Commercial, Transportation – Communication - Utilities - Updated for Costs - Updated WP template and Calculation template - All measure are directly from DEER/READI - Reporting period, effective 1/1/16 |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | linear fluorescent fixtures (T5, or T5HO) |
| Existing Condition | Existing incandescent, mercury vapor, standard metal halide, or high pressure sodium interior lighting fixtures |
| Code/Standard | Metal Halide Fixture (Upto 175W) Pulse Start Metal Halide Fixture (> 175W) |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | LT-41199 | N/A | Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case |
| N/A | N/A | LT-12866 | N/A | Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case |
| N/A | N/A | LT-84912 | N/A | Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case |
| N/A | N/A | LT-26100 | N/A | Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case |
| N/A | N/A | LT-55943 | N/A | 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case |
| N/A | N/A | LT-92448 | N/A | Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case |
| N/A | N/A | LT-38287 | N/A | Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case |
| N/A | N/A | LT-10232 | N/A | (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor |
| N/A | N/A | LT-28754 | N/A | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID |
| N/A | N/A | LT-39676 | N/A | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor |

Eligibility Requirements

In all cases, the replacement linear fluorescent fixtures must be a lower wattage than the existing base case and all base cases and measure cases must fall into the wattage classifications shown in Measures and Codes Table shown above.

Documentation Requirements

For direct install measures, the program must track the installed and removed equipment to the degree specified in the contracts set up with SCE.

For all fixture replacement measures, if the measure is limited to projects not covered by Title 24 Section 146, require the submission of pre and post lighting construction documents that clearly identify all enclosed spaces and which fixtures have been replaced.

## 1.2 Technical Description

This work paper details the replacement of existing incandescent, mercury vapor, standard metal halide, or high pressure sodium interior lighting fixtures with more efficient linear fluorescent fixtures (T5, or T5HO).

## 1.3 Installation Types and Delivery Mechanisms

The delivery method is:

Financial Support – Direct Install

Financial Support – Downstream Incentive – Deemed

Midstream Programs – Mid-Stream Incentives

The install type is:

Replace on Burnout (ROB)

ROB measures replace existing equipment with more energy efficient equipment when the existing equipment has failed or passed its useful life. ROB is for all other delivery methods.

Retrofit (RET)

RET measures for direct install only.

The Non Residential Direct Install program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is a form which information for measures installed in all building types related to the Non Residential Direct Install program.  The PLF contains the following fields: Service Account Address, Measures Proposed/Installed, Product Make/Model, Install Locations (detailed to define separate spaces/floors, as well as specific locations within the space including but not limited to:  Bathrooms, Hallways, Meeting Rooms, Offices, Warehouse, etc.).

Customers are solicited to participate primarily through field visits.  Contractors conduct energy consultation and provide recommendation that can help eligible customers use less energy.  If the customer agrees, the Direct Install contractor will help them complete an authorization form and schedule an installation appointment.

SCE can also provide photos to show measure functionality and a sample close up photos to substantiate the measure base case where applicable.  These photos would be part of the required project package.  The project package is identified with the Service Account Number and attached to SCE’s SMART database (SCE Project Tracking System) at each specific project level. The above described information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | N/A |
| DEER Measure IDs Used | Yes |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mLFHB-Deemed | Nonresidential Linear Fluorescent: high bay applications; deemed; all delivery mechanisms | NonRes | Any | Deemed | 0.65 |
| NonRes-sAll-mLFHBT5-Deemed | Nonresidential Linear Fluorescent: T5 lamps; high bay applications; deemed; all delivery mechanisms | NonRes | Any | Deemed | 0.65 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | NonRes | Any | DirInstall | 0.85 |
| NonRes-sAll-mLFDL-Deemed | Nonresidential Linear Fluorescent: delamping; deemed; all delivery mechanisms except upstream | NonRes | Any | Deemed/NonUpstream | 0.65 |
| NonRes-sAll-mLFOth-Deemed | Nonresidential Linear Fluorescent: measures not listed elsewhere; all delivery mechanisms | NonRes | Any | Deemed | 0.6 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive NTG values listed in NTG Table above.

- Business Size – Customer must have less than ten employees

- Language – Customer’s primary language spoken is not English

- Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego CountyThe “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:“Notes on OMB CSA designations:The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.” The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties. The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Com-LF-All | Non-Res Linear Fluorescent fixture; Annual Installation Rate | Any | Any | Any | 0.934 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-T5 | Non-Residential | Indoor Lighting | Linear Fluorescents | EUL varies by building type EUL = Rated Life of Ballast (70,000 hours) / Annual usage for building type (usage provided by DEER) OR 15 years (whichever is less). | RUL value is a flat 1/3 of the EUL value |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] defines an alteration as “any change to a building's water-heating system, space-conditioning system, lighting system, or envelope that is not an addition. Alteration is also any change that is regulated by Part 6 to an outdoor lighting system that is not an addition. Alteration is also any change that is regulated by Part 6 to signs located either indoors or outdoors” [355].

Further, Subchapter 6 (b) 2. I. v. states that “Any lighting alteration that increases the installed lighting power in an enclosed space shall meet the requirements of Sections 110.9, 130.0, 130.1, 130.4, 140.3(c) and 140.6 [355].

Title 20 2015 Appliance Efficiency Regulations [493] include an Energy Efficiency Standard for Metal Halide Luminaires.  Probe-start ballasts metal halide luminaires rated at least partially within the range of 150 to 500 watts are not eligible. Additionally, metal halide luminaires that are not probe-start ballasts must comply with Section 1605.3(n)(2)(A) and 1605.3(n)(2)(B) as applicable.

Effective July 14th, 2012, multiple T12 lamp types will be impacted by the new US Department of Energy standards set for general service fluorescent lamps. Accordingly, T12 fixtures are being phased out as an eligible baseline lighting technology for the purpose of calculating energy savings for lighting retrofit projects. In accordance with this requirement, all T12 fixtures are not eligible as Base Case for this measure.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards Section 150.0 (k) | July 1, 2014 |
| Title 20 (2015) | Table K-5 Standards for Medium Base Compact Fluorescent Lamps | January 1, 2016 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

The DEER READI tool and the WO17 study [475] were consulted for the values used in this workpaper, and there are no applicable non-DEER studies associated with this workpaper.

## 1.6 Data Quality and Future Data Needs

The savings and other values such as NTG, EUL, and GSIA values for this workpaper are downloaded from the DEER READI tool, and the Work Order 17 – the Ex Ante Measure Cost Study is utilized for the costs.

# Section 2. Calculation Methodology

These results have not been modified and are being included in the workpaper for reference.

The following table indicates which measures are taken directly from or created with the DEER READI tool.

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| LT-41199 | Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO(62w)-dWP66 |
| LT-12866 | Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO(117w)-dWP73 |
| LT-84912 | Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(179w)-dWP116-dWC109 |
| LT-26100 | Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(234w)-dWP224-dWC166 |
| LT-55943 | 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(351w)-dWP107-dWC105 |
| LT-92448 | Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO(585w)-dWP495-dWC233 |
| LT-38287 | Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(234w)-dWP170-dWC166 |
| LT-10232 | (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(234w)-dwP221-dwC131 |
| LT-28754 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(351w)-dwP105 |
| LT-39676 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor | C-In-LFFixt-T5-46in-54w+El-PS-HLO-1(351w)-dwP429-dwC105 |

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Primary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Secondary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Community College | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - University | DEER:Indoor_CFL_Ltg | NON_RES |
| Grocery | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Hospital | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Hotel | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Motel | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Multistory Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Single-Story Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Conditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Unconditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | DEER:Indoor_CFL_Ltg | NON_RES |

# Section 4. Costs

## 4.1 Base Case Cost

Base case costs are developed from the 2010-2012 WO017 Ex Ante Measure Cost Study Final Report [475]. For a complete breakdown of base case costs, please refer to Attachment 2 [B].

## 4.2 Measure Case Cost

Measure case costs are developed from the 2010-2012 WO017 Ex Ante Measure Cost Study Final Report [475]. For a complete breakdown of measure case costs, please refer to Attachment 2 [B].

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

For a complete breakdown of full and incremental measure costs, please refer to Attachment 1 [A].

# Attachments

# References

| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| --- | --- |
| [475] | 2010-2012 WO017 Ex Ante Measure Cost Study Final Report |
| [493] | 2015 Appliance Efficiency Regulations (Title 20) |
| [A] | Attachment 1 |
| [B] | Attachment 2 |