Work Paper SCE13LG086

Revision 1

Southern California Edison Company

Interior Linear Fluorescent Fixture

# At-a-Glance Summary

| Applicable Measure Codes: | LT-41199, LT-12866, LT-84912, LT-26100, LT-55943, LT-92448, LT-38287, LT-10232, LT-38754, LT-39676 |
| --- | --- |
| Measure Description: | This work paper documents the E3 Calculator input values for replacing existing interior lighting fixtures with more efficient linear fluorescent fixtures (T8, T5, or T5HO). |
| Base Case Description: | The existing interior lights being replaced must be incandescent, mercury vapor, standard metal halide or high pressure sodium. |
| Energy Impact Common Units: | Per fixture |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | EUL varies by building type. EUL = 14.3 for Grocery and Food Store; EUL = 13.3 for Health / Medical – Hospital; EUL = 15 for all other building types. |
| Measure Application Type: | Early Retirement (ER) and Replace on Burnout (ROB). |
| Net-to-Gross Ratios: | NTG = 0.6, 0.7, 0.85, or 0.9 depending on Sector and Program Delivery type. |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG086.0 | Yes | 04/24/12 | Peter Davi/SCE | - Updated to latest WP template v0.01 from WPSCNRLG0086 |
|  |  |  | Brian V. O’Keefe/SCE | Changes from original draft: - 24 Hour measure removed from the workpaper. - Revised the RUDD costing method to a DEER approach. |
| SCE13LG086.1 | Yes | 07/08/14 | David Pruitt/PL Energy | - Used measure Wattages from the ED lighting disposition table dated 30 May, 2014. - Included the new READI fields. |
|  |  |  | Jack Melnyk/SCE | -Added Midstream Programs-Midstream Incentive -Work paper updated for reporting period, effective 7/1/2014 – 12/31/2014 |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The Measure is the replacement of existing interior incandescent/HID lighting fixtures with more efficient linear fluorescent fixtures (T8, T5, or T5HO).

The Basecase is existing interior lights which are being replaced.  These fixtures must be incandescent, mercury vapor, standard metal halide, or high pressure sodium.

This work paper documents the E3 Calculator input values for replacing existing interior incandescent/HID lighting fixtures with more efficient linear fluorescent fixtures (T8, T5, or T5HO). Table 1 below shows the 10 measure classifications that the existing base case wattage can fall into.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-41199 | Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case |
| LT-12866 | Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case |
| LT-84912 | Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case |
| LT-26100 | Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case |
| LT-55943 | 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case |
| LT-92448 | Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case |
| LT-38287 | Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case |
| LT-10232 | (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor |
| LT-28754 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID |
| LT-39676 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor |

In all cases, the replacement linear fluorescent fixtures must be a lower wattage than the existing base case and all base cases and measure cases must fall into the wattage classifications shown in Table 1.

For direct install measures, the program must track the installed and removed equipment to the degree specified in the contracts set up with SCE.

For all fixture replacement measures, if the measure is limited to projects not covered by Title 24 Section 146, require the submission of pre and post lighting construction documents that clearly identify all enclosed spaces and which fixtures have been replaced.

## 1.2 Technical Description

This work paper details the replacement of existing incandescent, mercury vapor, standard metal halide, or high pressure sodium interior lighting fixtures with more efficient linear fluorescent fixtures (T8, T5, or T5HO).

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The delivery methods that are available for these measures are:

- Financial Support - Down-Stream Incentive – Deemed, in which the incentive is paid directly to the customer

- Financial Support – Direct Install, in which the measure is installed at the customer’s facility by program contractors.

- Midstream Programs - Mid-Stream Incentive, in which the incentive is paid to the distributor

- The install types are:

- Retrofit (RET) for direct install

- Replace-on-Burnout (ROB) for all other delivery methods

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

The Database for Energy Efficient Resources (DEER) 2011 v4.01 contains linear fluorescent measures. However, due to the way these measures are configured, as being available for range of wattages, the measures in DEER are not applicable.  This is summarized in Table 2 DEER Difference Summary below.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this type of measure. |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mT5T8-dn | T5 and T8 lamps | Com | Any | PreRebDown | 0.7 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.6 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Com-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Com-LF-SCE | Non-Res Linear Fluorescent fixture; Annual Installation Rate | Com | Any | NonUpStrm | 0.95 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex-ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor General Lighting |
| Measure Case TechGroups | Lighting – Fixtures |
| Measure Case TechTypes | Linear Fluorescent Fixture |
| Base Case TechGroups | Lighting – Fixtures |
| Base Case TechTypes | HID Fixture |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] defines an alteration as “any change to a building's water-heating system, space-conditioning system, lighting system, or envelope that is not an addition. Alteration is also any change that is regulated by Part 6 to an outdoor lighting system that is not an addition. Alteration is also any change that is regulated by Part 6 to signs located either indoors or outdoors” [355].

Further, Subchapter 6 (b) 2. I. v. states that “Any lighting alteration that increases the installed lighting power in an enclosed space shall meet the requirements of Sections 110.9, 130.0, 130.1, 130.4, 140.3(c) and 140.6 [355].

Title 20 2014 Appliance Efficiency Regulations [422] include an Energy Efficiency Standard for Metal Halide Luminaires.  Probe-start ballasts metal halide luminaires rated at least partially within the range of 150 to 500 watts are not eligible. Additionally, metal halide luminaires that are not probe-start ballasts must comply with Section 1605.3(n)(2)(A) and 1605.3(n)(2)(B) as applicable.

Effective July 14th, 2012, multiple T12 lamp types will be impacted by the new US Department of Energy standards set for general service fluorescent lamps. Accordingly, T12 fixtures are being phased out as an eligible baseline lighting technology for the purpose of calculating energy savings for lighting retrofit projects. In accordance with this requirement, all T12 fixtures are not eligible as Base Case for this measure.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance Manual, Sections 130 and 140.6 | July 1, 2014 |
| Title 20 (2014) | Section 1605.3(n) Energy Efficiency Standard for Metal Halide Luminaires | July 1, 2014 |

### 1.4.3 Non-DEER Study Review

No non-DEER studies or data were reviewed for this Workpaper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2015 program cycle extension on www.deeresources.com. The DEER documentation “Summary of EUL-RUL Analysis for the April 2008 Update to DEER” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, EUL_Summary_10-1-08.xls [213], was consulted. Table 7 DEER14 EUL Value/Methodologybelow identifies the value/methodology used for the measures in this work paper.

There is no unique EUL_ID for the different EUL values as indicated below.  Therefore, ILtg-Lfluor-Elec is used for all measures.

Table 8 identifies the actual EUL and RUL for all of the building types in this work paper.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Lfluor-Elec | Non-Residential | Indoor Lighting | Linear Fluorescents | EUL varies by building type EUL = Rated Life of Ballast (70,000 hours) / Annual usage for building type (usage provided by DEER) OR 15 years (whichever is less). | RUL value is a flat 1/3 of the EUL value |

Table 8 DEER08 EUL by Building Type

| Building Type | Operating Hours | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- |
| Agricultural | 3600.0 | 15.0 | 5.00 |
| Assembly | 2610.0 | 15.0 | 5.00 |
| Education - Primary School | 2140.0 | 15.0 | 5.00 |
| Education - Secondary School | 2280.0 | 15.0 | 5.00 |
| Education - Relocatable Classroom | 2480.0 | 15.0 | 5.00 |
| Education - Community College | 2420.0 | 15.0 | 5.00 |
| Education - University | 2350.0 | 15.0 | 5.00 |
| Grocery | 4910.0 | 14.3 | 4.75 |
| Food Store | 4910.0 | 14.3 | 4.75 |
| Health/Medical - Hospital | 5260.0 | 13.3 | 4.44 |
| Health/Medical - Nursing Home | 4160.0 | 15.0 | 5.00 |
| Health/Medical - Clinic | 4160.0 | 15.0 | 5.00 |
| Lodging - Hotel | 1950.0 | 15.0 | 5.00 |
| Lodging - Guest Rooms | 1550.0 | 15.0 | 5.00 |
| Lodging - Motel | 1550.0 | 15.0 | 5.00 |
| Manufacturing - Bio/Tech | 3530.0 | 15.0 | 5.00 |
| Manufacturing - Light Industrial | 3220.0 | 15.0 | 5.00 |
| Industrial | 3220.0 | 15.0 | 5.00 |
| Misc - Commercial | 3600.0 | 15.0 | 5.00 |
| Office - Large | 2640.0 | 15.0 | 5.00 |
| Office - Small | 2590.0 | 15.0 | 5.00 |
| Restaurant - Fast-Food | 4840.0 | 14.5 | 4.82 |
| Restaurant - Sit-Down | 4830.0 | 14.5 | 4.83 |
| Retail - Multistory Large | 3380.0 | 15.0 | 5.00 |
| Retail - Single-Story Large | 4270.0 | 15.0 | 5.00 |
| Retail - Small | 3380.0 | 15.0 | 5.00 |
| Storage - Conditioned | 3420.0 | 15.0 | 5.00 |
| Storage - Unconditioned | 3420.0 | 15.0 | 5.00 |
| Transportation - Communication - Utilities | 3600.0 | 15.0 | 5.00 |

# Section 2. Energy Savings & Demand Reduction Calculations

To document savings for this work paper, measure case and base cases were selected that represent an appropriate replacement and are near the upper bound of the acceptable replacement wattage to be conservative in the saving estimates as these measures will represent a large majority of similar replacements.

Base Case

For each measure, a Metal Halide or Mercury Vapor lamp and fixture were selected in the appropriate ranges or specified measures wattage used in the energy savings estimates.  The lamps were chosen in the middle to lower section of the range (if applicable) to be conservative.  The fixture wattage is taken from Appendix B of the 2013 SPC Program Manual [431].  The base cases used in the calculations are shown in Table 9. The SPC Program Manual did not include 350W metal halide fixtures, so those wattages were extrapolated from the other fixture wattages.

The base case Wattage values for a number of the following measures were updated using the “2013-2014_LightingRetrofit_Disposition-30May2014” table [A] provided by the Energy Division.  The values that have been updated with the table are noted with an asterisk (*).

Code Base Case

Title 20 prohibits traditional probe-start ballast metal halide fixtures from 150 to 500W, so pulse-start metal halide fixtures were chosen as the code base case. These values are also taken from Appendix B of the 2013 SPC Program Manual and shown in Table 99. Not all base cases are affected by Title 20.

The code case Wattage values for a number of the following measures were updated using the “2013-2014_LightingRetrofit_Disposition-30May2014” table [A] provided by the Energy Division.

Table 9 Base Case Selection for Savings Estimates

| Solution Code | Measure | Base Case Fixture | Fixture Base Case Code | Fixture Base Case Wattage | Code Base Case Fixture | Fixture Code Base Code | Fixture Code Case Wattage |
| --- | --- | --- | --- | --- | --- | --- | --- |
| LT-41199 | Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case | Metal Halide, 100 Watts | HIDfixt-MH-100W | 128 | Metal Halide, 100 Watts | HIDfixt-MH-100W | 128 |
| LT-12866 | Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case | Metal Halide, (1) 175W lamp | MH-175W | 215* | Pulse Start Metal Halide, (1) 175W lamp | HIDfixt-PSMH-175 | 208* |
| LT-84912 | Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case | Metal Halide, (1) 250W lamp | MH250/1 | 295 | Pulse Start Metal Halide, (1) 200W lamp | HIDfixt-PSMH-200w | 232* |
| LT-26100 | Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | Metal Halide, (1) 400W lamp | MH400/1 | 458 | Pulse Start Metal Halide, 350 Watts | HIDfixt-PSMH-350w | 400 |
| LT-55943 | 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | Metal Halide, (1) 400W lamp | MH400/1 | 458 | Pulse Start Metal Halide, 400 Watts | HIDfixt-PSMH-400w | 456 |
| LT-92448 | Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case | Metal Halide, (1) 750W lamp | MH750/1 | 850* | Pulse Start Metal Halide, 750 Watts | HIDfixt-PSMH-750w | 818 |
| LT-38287 | Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case | Metal Halide, (1) 350W lamp, extrapolated | N/A | 400* | Pulse Start Metal Halide, (1) 320W Lamp, Pulse Start CWA Ballast | PSMH-320w | 365* |
| LT-10232 | (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor | Mercury Vapor, (1) 400W lamp | MV400/1 | 455 | Pulse Start Metal Halide, 250 Watts | HIDfixt-PSMH-250w | 288* |
| LT-28754 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID | Pulse Start Metal Halide, (1) 400W lamp, Pulse Start CWA Ballast | MH400PS/1 | 456 | Pulse Start Metal Halide, 400 Watts | HIDfixt-PSMH-400w | 456 |
| LT-39676 | (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor | Mercury Vapor, (1) 700W lamp | MV700/1 | 780 | Pulse Start Metal Halide, 400 Watts | HIDfixt-PSMH-400w | 456 |

*This is the wattage for a code case fixture that is lumen equivalent to the base case fixture. This value differs from the May 30 disposition value.

Measure Case

The measure cases were taken from the Table of Standard Fixture Wattages [431] and are shown in Table 10.

The measure case Wattage values for one of the following measures were updated using the “2013-2014_LightingRetrofit_Disposition-30May2014” table [A] provided by the Energy Division.

Table 10 Measure Case Selection for Savings Estimates

| Measure Description Classifications | Measure Case Selection | Fixture Code | Fixture Wattage |
| --- | --- | --- | --- |
| Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case | (1) 46" T5HO with IS Ballast | F41PHL | 62 |
| Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case | (2) 46" T5HO with IS Ballast | F42PHL | 117 |
| Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case | (3) 46" T5HO with (2) IS Ballast | F43PHL/2 | 179 |
| Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | (4) 46" T5HO with (2) IS Ballast | F44PHL/2 | 234 |
| 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | (6) 46" T5HO with (3) IS Ballast | F46PHL/3 | 351 |
| Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case | Fluorescent, (8) 46", T5/HO lamp, (4) Instant Start Ballast | F48PHL/4 | 468* |
| Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case | (4) 46" T5HO with (2) IS Ballast | F44PHL/2 | 234 |
| (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor | (4) 46" T5HO with (2) IS Ballast | F44PHL/2 | 234 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID | (6) 46" T5HO with (3) IS Ballast | F46PHL/3 | 351 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor | (6) 46" T5HO with (3) IS Ballast | F46PHL/3 | 351 |

*This value comes from the previous revision of this work paper (Revision 0) and differs from the May 30 disposition value. Since the disposition document only addresses code baseline and not the measure case, it is unclear why the disposition’s recommended measure case wattages differ from those in Revision 0 of this work paper. Therefore, the disposition value is not used.

“The methodology of calculating energy savings for Non-CFL lamps follows a simple formula that captures wattage level changes, annual hours of use, and interactive effects on other end-uses.  For the most part, the methodology for calculating the energy savings for the Non-CFL interior lights is the same as for interior CFL lamps.” [26, page 3-8]

Equation 1 illustrates the energy savings estimation methodologies used to calculate Non-CFL interior lighting measures.

Equation 1

Equation 2 illustrates a step-by-step sample calculation shown for the Education – Primary School building type in climate zone 6 using the methodology shown in Equation 1 for Up to 128 Watt Interior Fixture T5 Linear Fluorescent  replacing 101-175 Watt lamp base case. The Energy Savings values use the same methodologies for all market sectors as derived here for the Education – Primary School market sector and can also be found in the Calculation Template [E].

1st baseline

Energy Savings268.81 kWh savingsunit∙year=[215x2135.2-(117x1950  )]x(1.1641)1,000 Watt hours /kWh		Equation 2

2nd baseline

Energy Savings206.57 kWh savingsunit∙year=91X(1950)x(1.1641)1,000 Watt hours /kWh 			Equation 3

“The methodology of calculating demand reduction for Non-CFL lamps follows a simple formula that captures Wattage level changes, interactive effects on other end-uses, and coincidence factor for peak demand. For the most part, the methodology for calculating the demand reduction for the Non-CFL interior lights is the same as for interior CFL lamps.” [26, page 3-6]

Equation 3 illustrates the peak demand reduction estimation method used.

Equation 4

Equation 4 illustrates a step-by-step sample calculation is shown for the Education – Primary School market sector in climate zone 6 using the methodology illustrated in Equation 3. The Demand Reduction values use the same methodologies for all market sectors as derived here for the Education – Primary School market sector and can also be found in the Calculation Template [E].

1ST baseline

Peak Demand Savings0.08776 kW savingsunit=[215x0.61448-117x0.55113]x(1.2977)unit	Equation 5

2nd baseline

Peak Demand Savings0.06508 kW savingsunit=0.091kWunitx0.55113x(1.2977)unit	Equation 6

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_Non-CFL_Ltg load shape.  See Table 11 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 11 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Assembly | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Primary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Secondary School | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Relocatable Classroom | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - Community College | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Education - University | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Grocery | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Food Store | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Hospital | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Nursing Home | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Health/Medical - Clinic | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Hotel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Guest Rooms | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Lodging - Motel | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Bio/Tech | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Manufacturing - Light Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Industrial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Misc - Commercial | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office - Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Office - Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Fast-Food | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Restaurant - Sit-Down | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Multistory Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Single-Story Large | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Retail - Small | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Conditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Storage - Unconditioned | NON_RES | DEER:Indoor_Non-CFL_Ltg |
| Transportation - Communication - Utilities | NON_RES | DEER:Indoor_Non-CFL_Ltg |

# Section 4. Base Case & Measure Costs

Direct Install measures use program tracking systems for measure costs.

## 4.1 Base Case Cost

The base case and code base case costs are taken from the DEER 2008 Cost Summary Spreadsheet [B]. Costs for the 350W and 750W metal halide fixtures and the 750W pulse-start metal halide fixture were extrapolated from other costs in their respective categories. It is assumed that the labor cost is the same in the base and code base cases. The mercury vapor costs are taken from online retailers and are the sum of fixture and lamp costs [C]. The 700 Watt mercury vapor cost was interpolated between the 400 Watt and 1000 Watt costs; those were the closest wattages available from online retailers.

Table 12 Base and Code Case Costs

| Measure | Base Case Fixture | Base Case Equipment Cost | Code Base Case Fixture | Code Base Case Equipment Cost | Labor Cost |
| --- | --- | --- | --- | --- | --- |
| Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case | Metal Halide, (1) 100W lamp | $132.51 | Metal Halide, (1) 100W lamp | $132.51 | $18.04 |
| Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case | Metal Halide, (1) 175W lamp | $137.67 | Pulse Start Metal Halide, (1) 175W lamp, Pulse Start CWA Ballast | $135.34 | $72.17 |
| Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case | Metal Halide, (1) 250W lamp | $152.84 | Pulse Start Metal Halide, (1) 200W lamp | $139.67 | $72.17 |
| Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | Metal Halide, (1) 400W lamp | $164.40 | Pulse Start Metal Halide, (1) 350W lamp, Pulse Start CWA Ballast | $165.64 | $72.17 |
| 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | Metal Halide, (1) 400W lamp | $164.40 | Pulse Start Metal Halide, (1) 400W lamp, Pulse Start CWA Ballast | $203.54 | $72.17 |
| Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case | Metal Halide, (1) 750W lamp | $214.33 | Pulse Start Metal Halide, (1) 750W lamp, Pulse Start CWA Ballast | $292.98 | $72.17 |
| Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case | Metal Halide, (1) 350W lamp, extrapolated | $161.41 | Pulse Start Metal Halide, (1) 320W Lamp, Pulse Start CWA Ballast | $178.87 | $72.17 |
| (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor | Mercury Vapor, (1) 400W lamp | $266.34 | Pulse Start Metal Halide, (1) 250W Lamp, Pulse Start CWA Ballast | $148.33 | $72.17 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID | Pulse Start Metal Halide, (1) 400W lamp, Pulse Start CWA Ballast | $203.54 | Pulse Start Metal Halide, (1) 400W lamp, Pulse Start CWA Ballast | $203.54 | $72.17 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor | Mercury Vapor, (1) 700W lamp | $421.74 | Pulse Start Metal Halide, (1) 400W lamp, Pulse Start CWA Ballast | $203.54 | $72.17 |

## 4.2 Measure Case Cost

The measure case costs are taken from the DEER 2008 Cost Summary Spreadsheet [B]. DEER only contained costs for a 4 and 6 lamp T5 measure so the other lamp quantities were extrapolated from those cost case IDs, 46in54wT5HO2PSEl234w-Rpl and 46in54wT5HO3PSEl351w-Rpl.  Labor cost was derived from the DEER Cost Summary Spreadsheet as well. See Attachment 2 [D] for the cost derivation.

Table 13 Measure Costs

| Measure Name | Measure case Description | DEER Cost ID | Measure Cost | Labor Cost |
| --- | --- | --- | --- | --- |
| Up to 64 Watt Interior Fixture T5 Linear Fluorescent replacing less than 100 Watt lamp base case | (1) 46" T5HO with IS Ballast | extrapolated | $90.48 | $18.04 |
| Up to 128 Watt Interior Fixture T5 Linear Fluorescent replacing 101 - 175 Watt lamp base case | (2) 46" T5HO with IS Ballast | extrapolated | $129.00 | $72.17 |
| Up to 192 Watt Interior Fixture T5 Linear Fluorescent replacing 176 - 399 Watt lamp base case | (3) 46" T5HO with (2) IS Ballast | extrapolated | $167.51 | $72.17 |
| Up to 244 Watt (Tier 1) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | (4) 46" T5HO with (2) IS Ballast | 46in54wT5HO2PSEl234w-Rpl | $206.03 | $72.17 |
| 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | (6) 46" T5HO with (3) IS Ballast | 46in54wT5HO3PSEl351w-Rpl | $283.06 | $72.17 |
| Up to 600 Watt Interior Fixture T5 Linear Fluorescent replacing greater than 400 Watt lamp base case | (8) 46" T5HO with (4) IS Ballast | extrapolated | $360.09 | $72.17 |
| Up to 244 Watt Interior Fixture T5 Linear Fluorescent replacing 350 Watt lamp base case | (4) 46" T5HO with (2) IS Ballast | 46in54wT5HO2PSEl234w-Rpl | $206.03 | $72.17 |
| (4) 46in (2) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Mercury Vapor | (4) 46" T5HO with (2) IS Ballast | 46in54wT5HO2PSEl234w-Rpl | $206.03 | $72.17 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 400 Watt Pulse Start HID | (6) 46" T5HO with (3) IS Ballast | 46in54wT5HO3PSEl351w-Rpl | $283.06 | $72.17 |
| (6) 46in (3) Programmed Start Ballast - Normal Light Output - HO T5 Linear Fluorescent replacing 700 Watt Mercury Vapor | (6) 46" T5HO with (3) IS Ballast | 46in54wT5HO3PSEl351w-Rpl | $283.06 | $72.17 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For RET (1st Baseline), the gross measure cost is the full measure cost:

GMC 	= (Measure Equipment Cost + Measure Labor Cost)

For RET (2nd Baseline), the gross measure cost is:

GMC 	= (Measure Equipment Cost – Code Equipment Cost)

For ROB, the equipment being replaced is assumed to have failed in place or is past its useful life. The customer is faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.  Therefore, gross measure cost (GMC) means the cost premium required to install the energy efficient measure over a less efficient piece of equipment. GMC is represented by the equation below:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

= (Measure Equipment Cost – Code Equipment Cost)

*Note: Unless stated otherwise the measure case labor and base case labor are assumed to be the same value. For this work paper, the measure and base case (code base case) costs are different.

Table 14 includes a sample calculation of GMC. See the cost spreadsheet [D] for the rest of the calculations. They are also in the calculations template [E].

Table 14 Incremental and Gross Measure Cost

| Measure Name | Code Base Equipment Cost | Code Base Labor Cost | Measure Equipment Cost | Measure Labor Cost | GMC (1st Baseline) | GMC (2nd Baseline) | GMC (ROB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 245 to 360 Watt (Tier 2) Interior Fixture T5 Linear Fluorescent replacing 400 Watt lamp base case | $203.54 | $72.17 | $283.06 | $72.17 | $355.23 | $79.52 | $79.52 |

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure. The incremental cost is only used to help determine program incentives.

For ROB, the IMC is the same value as GMC.

For RET, the IMC is represented by the equation below:

IMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Code Base Case Equipment Cost + Code Base Case Labor Cost)

= (Measure Equipment Cost – Code Base Case Equipment Cost)

*Note: Unless stated otherwise the measure case labor and base case labor are assumed to be the same value. For this work paper, the measure and base case (code base case) costs are different.

See the cost spreadsheet [D] for the rest of the calculations. They are also in the calculations template [E].

# Attachments

1.

2.

3.

4.

# References

[26]

[31]

[213]

[220]

[351]

[355]

[422]

[A] Attachment 4 - 2013-2014_LightingRetrofit_Disposition-30May2014

[B]  Revised DEER Measure Cost Summary (05_30_2008) Revised (06_02_2008) spreadsheet, www.deeresources.com for 09-11 Planning Reporting

[C] Attachment 3 – Mercury Vapor Cost Screenshots.docx

[D] Attachment 2 – LG086 Wattages and Costs.xlsx

[E] Attachment 1 – Calculation Template v2.2-SCELG086.xlsm

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |