Work Paper SCE13LG099

Revision 2

Southern California Edison

Interior Integral Non Dimmable (Screw-in) CFLs

# At-a-Glance Summary

| Measure Codes | LT-38821, LT-13420, LT-56098, LT-84675, LT-78466, LT-10933, LT-68569, LT-53737, LT-65394, LT-27012, LT-55729, LT-49284, LT-62221, LT-45002, LT-79790, LT-19354, LT-30896, LT-89923, LT-71924, LT-45264, LT-10967, LT-68597, LT-40396, LT-88532 |
| --- | --- |
| Measure Description | Spiral CFL and Spiral CFL w/Reflector |
| Base Case Description | Various low efficacy lamps including incandescent, HID, and Circline |
| Units | Lamp |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | Commercial: 10,000 / hours of use Residential: 3.5 years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Refer to Net to Gross Ratio Table |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 5/30/12 | Brian V. O’Keefe/SCE | - Original work paper draft for 2013-2014 program cycle - Updated version of 2010-2012 work paper WPSCNRLG0099 - New Measures added to original draft |
| 1 | 3/18/14 | Sara Richardson/TRC Energy Services, Jason Wang/SCE | - Work paper updated for the reporting period, effective 7/1/14 – 12/31/14.-Updated solution codes - Revised calculations - Revised costs - Added midstream |
| 2 | 9/23/15 | Ryan Cho (SCE) | - Updated for DEER2016 HOU, CDF, NTG, EUL - Added HTR NTG & Implementation language - Removed Agricultural, Food Store, Health/Medical – Clinic, Industrial, Misc – Commercial, Transportation – Communication - Utilities - Updated for Costs - Updated WP template and Calculation template - Removed the Midstream Program (Tom Pasker) - 100% of measure are directly from DEER - Reporting period, effective 1/1/16 - Removed several measures due to lack of use. See Section 1.1 for the remaining measures. |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Spiral CFL and Spiral CFL w/Reflector |
| Existing Condition | Various low efficacy lamps including incandescent, HID, and Circline |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | LT-38821 | N/A | 5-13 watts Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-13420 | N/A | 13 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-56098 | N/A | 14-26 watts Interior Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-84675 | N/A | >=27watts Interior Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-78466 | N/A | 32 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| N/A | N/A | LT-10933 | N/A | 32 Watt Integral Spiral CFL replacing 200 Watt Incandescent |
| N/A | N/A | LT-68569 | N/A | 32 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-53737 | N/A | 36 Watt Integral Spiral CFL replacing 150 Watt Incandescent |
| N/A | N/A | LT-65394 | N/A | 5-13 watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-27012 | N/A | 14-26 watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-55729 | N/A | 14-28 watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-49284 | N/A | >=27watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts |
| N/A | N/A | LT-62221 | N/A | 14 Watt Interior Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-45002 | N/A | 18 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-79790 | N/A | 20 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-19354 | N/A | 23 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-30896 | N/A | 14 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-89923 | N/A | 18 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-71924 | N/A | 15 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-45264 | N/A | 23 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-10967 | N/A | 14 Watt Interior Spiral (Common Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts |
| N/A | N/A | LT-68597 | N/A | 18 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-40396 | N/A | 15 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |
| N/A | N/A | LT-88532 | N/A | 23 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts |

Eligibility Requirements

The proposed lamps must be on the Energy Star qualified list and must be a one to one replacement.

Implementation Requirements

- Dwelling Area measures are eligible for Residential Single Family, Multi-family, and Mobile Home building types.

- Common Area measures are eligible for Residential Multi-family and Mobile Home building types.

- Measures that are not indicated as Dwelling Area or Common Area are eligible for all Non-residential building types.

Documentation Requirements

Direct install programs that use the actual customer existing wattages found in the field must track these wattages along with the measure used.

## 1.2 Technical Description

The measure involves the installation of a screw in compact fluorescent lamp with integrated ballast.

## 1.3 Installation Types and Delivery Mechanisms

The delivery method is:

Financial Support – Direct Install

Financial Support – Giveaway

Giveaway measures are available for the solution codes listed: LT-49284, LT-84675, LT-27012, LT-56098, LT-55729, LT-38821, LT-65394, LT-62221, LT-30896, LT-10967

Financial Support – Downstream Incentive – Deemed

Downstream measures are available for the solution codes listed: LT-55729

The install type is:

Replace on Burnout (ROB)

ROB measures replace existing equipment with more energy efficient equipment when the existing equipment has failed or passed its useful life.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | N/A |
| DEER Measure IDs Used | C-In-CFLscw-Refl(28w)-dWP71 |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| NonRes-sAll-mCFL-All | Nonresidential CFLs: deemed; all delivery mechanisms | Com | Any | DeemedNonUpstream | 0.6 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes residential measures that are offered via direct install activities into hard-to-reach (HTR) customer homes.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer homes as HTR. The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria.

SCE’s Multi-Family Energy Efficiency Rebate (MFEER) program addresses the ongoing concern with “split incentives”, where the residents are not the owners of the property, so they lack incentive to improve their energy usage. Similarly, the property owners do not live on-site and pay higher utility expenses due to inefficient appliances, thus lack any incentive to upgrade. The MFEER is designed to drive this customer segment toward participation by offering property owners a variety of energy efficiency measures and services.  The MFEER program will offer and track measure installations in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums.  Measures offered via direct install activities in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums will receive the HTR NTG.  Other measures in the MFEER program will receive default NTG (NTGR_ID: Res-Default>2), unless otherwise specified in DEER.

This work paper also includes commercial measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

o	Business Size – Customer must have less than ten employees

o	Language – Customer’s primary language spoken is not English

o	Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.”

The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties.

The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |
| Com-CFL-All | Non-Res CFL; Non-Upstream Program; Annual Installation Rate | Com | Any | Any | 0.691 |
| MFm-IntCFL-All | Interior CFL; Annual Installation Rate; Multi-family | Res | MFm | Any | 0.798 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-CFL-Com | CFL Lamps - Indoor- Commercial - 10,000 Rated Hours | Com | Lighting | 10,000 / HOU | N/A |
| ILtg-CFL-Res | “2015 Uncertain Measures – WP Updates” Guidance | Res | Lighting | 3.5 | N/A |

### 1.4.2 Codes and Standards Analysis

This work paper uses ED-specified wattages and wattage ratios to determine code case wattages; these satisfy or exceed any energy consumption requirements mentioned below.

Title 24 (2013) Building Energy Efficiency Standards [355] includes standards for both non-residential and residential buildings.  Section 150.0 (k) 7 and Section 150.0 (k) 8 pertain to residential luminaries.  Both Sections call out that installed luminaries need to be high efficacy lighting as defined by Table 150.0-A or Table 150.0-B for lighting systems not listed in Table 150.0-A.  According to Table 150.0-A, “GU-24 sockets for compact fluorescent lamps” are classified as high efficacy.  Thus, the measures outlined in this work paper satisfy the requirement as a high efficacy lighting technology.

Title 20 (2015) Appliance Efficiency Regulations [493] includes standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California.  Table K-5 lists the minimum efficacy (lumens/watt) requirements for medium base compact fluorescent lamps. It is assumed that the measures listed in DEER 2016 comply with the 2015 Title 20, Table K-5.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Building Energy Efficiency Standards Section 150.0 (k) | July 1, 2014 |
| Title 20 (2015) | Table K-5 Standards for Medium Base Compact Fluorescent Lamps | January 1, 2016 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

The DEER READI tool and the WO17 study [475] were consulted for the values used in this workpaper, and there are no applicable non-DEER studies associated with this workpaper.

## 1.6 Data Quality and Future Data Needs

The savings and other values such as NTG, EUL, and GSIA values for this workpaper are downloaded from the DEER READI tool, and the Work Order 17 – the Ex Ante Measure Cost Study is utilized for the costs.

# Section 2. Calculation Methodology

Measures in this work paper are directly from the DEER READI tool.

The following table indicates which measures are taken directly from or created with the DEER READI tool.

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| LT-38821 | 5-13 watts Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw(13w)-dWP33 |
| LT-13420 | 13 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw(13w)-dWP33 |
| LT-56098 | 14-26 watts Interior Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw(18w)-dWP46-1 |
| LT-84675 | >=27watts Interior Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw(28w)-dWP71 |
| LT-78466 | 32 Watt Integral Spiral CFL replacing 150 Watt Incandescent | C-In-CFLscw(32w)-dWP118 |
| LT-10933 | 32 Watt Integral Spiral CFL replacing 200 Watt Incandescent | C-In-CFLscw(32w)-dWP168 |
| LT-68569 | 32 Watt Integral Spiral CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw(32w)-dWP82 |
| LT-53737 | 36 Watt Integral Spiral CFL replacing 150 Watt Incandescent | C-In-CFLscw(36w)-dWP114 |
| LT-65394 | 5-13 watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw-Refl(13w)-dWP33 |
| LT-27012 | 14-26 watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw-Refl(18w)-dWP46 |
| LT-55729 | 14-28 watt Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw-Refl(18w)-dWP46 |
| LT-49284 | >=27watts Integral Spiral with Reflector CFL replacing Com CFL Base Case, Total Watts = 3.57 x Msr Watts | C-In-CFLscw-Refl(28w)-dWP71 |
| LT-62221 | 14 Watt Interior Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts | R-In-CFLscw(14)-dWP21 |
| LT-45002 | 18 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts | R-In-CFLscw(18)-dWP27 |
| LT-79790 | 20 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts | R-In-CFLscw(20)-dWP30 |
| LT-19354 | 23 Watt Integral Spiral (Dwelling Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts | R-In-CFLscw(23)-dWP34 |
| LT-30896 | 14 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-In-CFLscw-Refl(14)-dWP26 |
| LT-89923 | 18 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-In-CFLscw-Refl(18)-dWP33 |
| LT-71924 | 15 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-In-CFLscw-Refl-1(15)-dWP28 |
| LT-45264 | 23 Watt Interior Spiral with Reflector (Dwelling Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-In-CFLscw-Refl-1(23)-dWP43 |
| LT-10967 | 14 Watt Interior Spiral (Common Area) CFL replacing Non-Refl CFL Base Case, Total Watts = 2.48 x Msr Watts | R-InCmn-CFLscw(14)-dWP21 |
| LT-68597 | 18 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-InCmn-CFLscw-Refl(18)-dWP33 |
| LT-40396 | 15 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-InCmn-CFLscw-Refl-1(15)-dWP28 |
| LT-88532 | 23 Watt Interior Spiral with Reflector (Common Area) CFL replacing Reflector CFL Base Case, Total Watts = 2.86 x Msr Watts | R-InCmn-CFLscw-Refl-1(23)-dWP43 |

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Primary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Secondary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Community College | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - University | DEER:Indoor_CFL_Ltg | NON_RES |
| Grocery | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Hospital | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Hotel | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Motel | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Multistory Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Single-Story Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Conditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Unconditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | DEER:Indoor_CFL_Ltg | NON_RES |
| Residential Single Family | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-family | DEER:Indoor_CFL_Ltg | RES |
| Residential Mobile Home - Double-Wide | DEER:Indoor_CFL_Ltg | RES |

# Section 4. Costs

## 4.1 Base Case Cost

All of the cost values are from the latest ex ante database through the READI v2.3.0 except for SCE13LC099_02_B001 through SCE13LC099_02_B013. This new SCE cost ID is created and the DEER2016-Costs_Ltg-CFL-Incand_8-31-2015-DRAFT.xls [B] and WO017 Lighting Cost Calculator (Inc, CFL, LED, HID, LF).xlsx are utilized for developing material cost. For a complete breakdown of base case costs, please refer to Attachment 1 [A].

## 4.2 Measure Case Cost

All of the cost values are from the latest ex ante database through the READI v2.3.0 except for SCE13LC099_02_M001. For a complete breakdown of measure case costs, please refer to Attachment 1 [A].

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

For a complete breakdown of full and incremental measure costs, please refer to Attachment 1 [A].

# Attachments

1.

2.

3.

# References

| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| --- | --- |
| [475] | 2010-2012 WO017 Ex Ante Measure Cost Study Final Report |
| [493] | 2015 Appliance Efficiency Regulations (Title 20) |
| [A] | Attachment 1 |
| [B] | Attachment 2 |
| [C] | Attachment 3 |