Work Paper SCE13LG102

Revision 1

Southern California Edison

Exterior Induction Lighting

# At-a-Glance Summary

| Measure Codes | See Section 1.1 |
| --- | --- |
| Measure Description | Efficient hardwired induction lighting fixture |
| Base Case Description | Existing exterior HID lighting fixture |
| Units | Fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | OLtg-MH – 15 Years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | 0.6 (DEER NTGR ID: Com-Default>2yrs) |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 04/02/2012 | Akane Karasawa/AESC | Original work paper for 2013 PC |
| 1 | 09/25/2015 | Andres Fergadiotti/SCE | - Updated workpaper templates - Updated Building Types - Updated EUL/NTG per latest DEER documentation - Updated operating hours per DEER - Update Labor Cost - Created Cost IDs per internal guidance, e.g., 2016 WP Update Guide |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the E3 Calculator input values for replacing an existing exterior lighting fixture with a more efficient hardwired induction lighting fixture. Table 1 summarizes the core measure descriptions and their solution codes.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Exterior Fixture Induction |
| Existing Condition | Exterior Fixture HID |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-10663 |  | Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case |
|  |  | LT-60651 |  | Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case |
|  |  | LT-21067 |  | Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case |
|  |  | LT-58832 |  | Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case |
|  |  | LT-78695 |  | Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case |

The existing exterior fixture being replaced must be an incandescent, mercury vapor, standard probe-start metal halide, or high-pressure sodium fixture.  Existing pulse start metal halide fixtures and T12 fixtures do not qualify. Interior fixtures do not qualify for these measures. In order to qualify for the 400 watt base case category, the fixtures must be installed at a height of over 12’ above the ground level. In all cases, the replacement induction fixture must be of lower wattage than the existing base case fixture and all base cases and measure cases must fall into the wattage classifications shown in Measures and Codes table above. The measures apply to all non-residential building types.

## 1.2 Technical Description

This measure replaces an existing exterior lighting fixture with a more efficient hardwired induction lighting fixture.  Induction lighting is essentially a fluorescent light without electrodes or filaments, the items that frequently cause other bulbs to burn out quickly. Thus, many induction lighting units have an extremely long life of up to 100,000 hours.

Induction lamps are ideally suited for high-ceiling applications where the lamps are difficult, costly or hazardous to access. They are also ideally suited for such applications where the advantages of fluorescent lighting are sought but a light source is needed that can start and operate efficiently in extremely cold temperatures.  As a result, induction lighting is a suitable for a wide range of applications including but not limited to outdoor area and security fixtures.

## Installation Types and Delivery Mechanisms

The program/install type for these measures is Replace-on-Burnout (ROB)

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.  The delivery methods that are available for these measures are:

- Financial Support - Down-Stream Incentive – Deemed

- Partnership – Down-Stream Incentive – Deemed

- Midstream Programs – Mid-Stream Incentive

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

The Database for Energy Efficient Resources DEER2016 (READi v2.3.0) [49] does not include exterior induction fixture measures.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | N/A |
| Scaled DEER measure | N/A |
| DEER Base Case | N/A |
| DEER Measure Case | N/A |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | N/A |
| DEER Version | N/A |
| Reason for Deviation from DEER | Not in DEER |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| OLtg-MH | HID Lighting – Metal Halide | Com | Lighting | 15 | 5 |

### 1.4.2 Codes and Standards Analysis

This measure is governed by Federal Energy Conservation Program for Consumer Products, 10 CFR Part 430, Federal Appliance Standards, national policy act of “16 CFR Part 305 (2005)”, and Title-20 “Appliance Efficiency Code”[51], which mandates the display of federal code on all products.

Title 20 [422]: Appliance Efficiency Regulations include standards for both federally regulated appliances and non-federally-regulated appliances. The standards within these regulations apply to appliances that are sold or offered for sale in California.

Title 20 Appliance Efficiency Regulations also include an Energy Efficiency Standard for Metal Halide Luminaires.  Probe-start ballasts metal halide luminaires rated at least partially within the range of 150 to 500 watts are not eligible. Additionally, metal halide luminaires that are not probe-start ballasts must comply with Section 1605.3(n)(2)(A) and 1605.3(n)(2)(B) as applicable.

Title 24 [355]:

Section 141.0(b)2J states:

Alterations to existing outdoor lighting systems shall meet the following requirements:

- Alterations that increase the connected lighting load in a lighting application listed in TABLE 140.7-A or 140.7-B shall meet the applicable requirements of Sections 130.0, 130.2, 130.4, and 140.7; and

- In alterations that replace 10 percent or more of the luminaires in a lighting application listed in TABLE 140.7-A or 140.7-B, the altered luminaires shall meet the applicable requirements of Sections 130.0, 130.2, 130.4; and

- In alterations that replace more than 50 percent of the luminaires in a lighting application listed in TABLE 140.7-A or 140.7-B, the lighting in that application shall meet the applicable requirements of Sections 130.0, 130.2, 130.4, and 140.7.

Section 140.7 contains lighting power requirements that apply to all outdoor lighting.

Section 130.2(c)3 states that:

All installed outdoor lighting, where the bottom of the luminaire is mounted 24 feet or less above the ground, shall be controlled with automatic lighting controls that meet all of the following requirements:

- Shall be motion sensors or other lighting control systems that automatically controls lighting in accordance with item B in response to the area being vacated of occupants.; and

- Shall be capable of automatically reducing the lighting power of each luminaire by at least 40 percent but not exceeding 80 percent, or provide continuous dimming through a range that includes 40 percent through 80 percent, and

- Shall employ auto-ON functionality when the area becomes occupied, and

- No more than 1,500 watts of lighting power shall be controlled together.

EXCEPTION 1 to Section 130.2(c)3: Lighting for Outdoor Sales Frontage, Outdoor Sales Lots, and Outdoor Sales Canopies complying with Section 130.2(c)4.

EXCEPTION 2 to Section 130.2(c)3: Lighting for Building Facades, Ornamental Hardscape and Outdoor Dining complying with Section 130.2(c)5.

EXCEPTION 3 to Section 130.2(c)3: Outdoor lighting, where luminaire rated wattage is determined in accordance with Section 130.0(c), and which meet one of the following conditions:

- Pole-mounted luminaires with a maximum rated wattage of 75 watts each; or

- Non-pole mounted luminaires each with a maximum rated wattage of 30 watts each; or

- Linear lighting with a maximum wattage of 4 watts per linear foot of luminaire.

EXCEPTION 4 to Section 130.2(c)3: Applications listed as Exceptions to Section 140.7(a) shall not be required to meet the requirements of Section 130.2(c)3.

This work paper triggers Title 24 lighting alterations, requiring lighting controls and lighting power requirements, unless the application meets the exception criteria.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Section 1607. Marking of Appliances, Section 1605.3(n) Energy Efficiency Standard for Metal Halide Luminaires | January 1, 2016 |
| Title 24 (2013) | Section 140.7, 141.0, 130.2 | July 1, 2014 |
| US DOE Standards (2009) | DOE 10 CFR Part 30 | July 14th, 2012 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

The fixture wattage for measures listed under this workpaper were taken from Appendix B of the 2009 SPC Program Manual [14].

## 1.6 Data Quality and Future Data Needs

Cost documentation will be evaluated and potentially updated in new version of the workpaper.

# Section 2. Calculation Methodology

To document savings for this work paper, measure case and base cases were selected that represent an appropriate replacement and are near the upper bound of the acceptable replacement wattage to be conservative in the saving estimates as these measures will represent a large majority of similar replacements.

For each of the measure classifications, a high pressure sodium lamp and fixture were selected in the appropriate ranges used in the energy savings estimates. The lamps were chosen in the middle to lower section of the range to be conservative. The fixture wattage is taken from Appendix B of the 2009 SPC Program Manual [14]. The base cases used in the calculations are shown in following table.

Base Case Selection for Savings Estimates

| Measure Description Classifications | Base Case Selection | Fixture Code | Fixture Wattage |
| --- | --- | --- | --- |
| Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case | High Pressure Sodium, (1) 70W lamp | HPS70/1 | 95 |
| Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case | High Pressure Sodium, (1) 100W lamp | HPS100/1 | 138 |
| Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case | High Pressure Sodium, (1) 150W lamp | HPS150/1 | 188 |
| Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case | High Pressure Sodium, (1) 250W lamp | HPS250/1 | 295 |
| Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case | High Pressure Sodium, (1) 400W lamp | HPS400/1 | 465 |

Note: Although the lamp wattages chosen for each measure may not fall within the range of a specific measure, the fixture wattage does fall within the range.

The measure case fixture wattages were taken from online manufacturer data [Attachment 2] and are shown in following table.

Measure Case Selection for Savings Estimates

| Measure Description Classifications | Measure Case Selection | Lamp Wattage | Fixture Wattage |
| --- | --- | --- | --- |
| Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case | 40 Watt Cobra Head | 40 | 44 |
| Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case | 70 Watt Cobra Head | 70 | 78 |
| Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case | 100 Watt Cobra Head | 100 | 111 |
| Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case | 150 Watt Cobra Head | 150 | 165 |
| Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case | 200 Watt Cobra Head | 200 | 215 |

The methodology of calculating energy savings for induction fixtures follows a simple formula that captures wattage level changes, hours of daily use and interactive affects. Since these fixtures are being installed outside the conditioned space, interactive affects can be ignored as the fixture’s reduction in thermal energy output will have no effect on an HVAC system.

The following example shows the annual electric kWh savings for the measure “Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case.”

The following example shows the demand reduction estimates for the measure “Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case” in Assembly building type, Climate Zone 6.

Demand ReductionkWUnit=51×.2211,000WatthourskWh=0.011271 kWUnit

A complete list of savings for other measures in this work paper can be found in the attachment [Attachment 1].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | Misc._Commercial | Outdoor LT |
| Education - Primary School | Misc._Commercial | Outdoor LT |
| Education - Secondary School | Misc._Commercial | Outdoor LT |
| Education - Relocatable Classroom | Misc._Commercial | Outdoor LT |
| Education - Community College | Misc._Commercial | Outdoor LT |
| Education - University | Misc._Commercial | Outdoor LT |
| Grocery | Misc._Commercial | Outdoor LT |
| Health/Medical - Hospital | Misc._Commercial | Outdoor LT |
| Health/Medical - Nursing Home | Misc._Commercial | Outdoor LT |
| Lodging - Hotel | Misc._Commercial | Outdoor LT |
| Lodging - Guest Rooms | Misc._Commercial | Outdoor LT |
| Lodging - Motel | Misc._Commercial | Outdoor LT |
| Manufacturing - Bio/Tech | Misc._Commercial | Outdoor LT |
| Manufacturing - Light Industrial | Misc._Commercial | Outdoor LT |
| Office - Large | Misc._Commercial | Outdoor LT |
| Office - Small | Misc._Commercial | Outdoor LT |
| Restaurant - Fast-Food | Misc._Commercial | Outdoor LT |
| Restaurant - Sit-Down | Misc._Commercial | Outdoor LT |
| Retail - Multistory Large | Misc._Commercial | Outdoor LT |
| Retail - Single-Story Large | Misc._Commercial | Outdoor LT |
| Retail - Small | Misc._Commercial | Outdoor LT |
| Storage - Conditioned | Misc._Commercial | Outdoor LT |
| Storage - Unconditioned | Misc._Commercial | Outdoor LT |
| Warehouse - Refrigerated | Misc._Commercial | Outdoor LT |

# Section 4. Costs

## 4.1 Base Case Cost

For retrofit measures, there exists a theoretical base case that the measure can be compared to in cost. For this measure, a high pressure sodium fixture was selected as a base case fixture in the appropriate wattage ranges as described in Section 2. Therefore, Base Case Costs were calculated using an average of the corresponding high pressure sodium fixture cost taken from multiple online retailers [Attachment 3] and summarized in following table below.

The labor cost for these measures was approximated as $98.00 per non-residential indoor HID (exterior) fixture labor costs listed in the 2010 RSMeans Electrical Cost Data [A].  It was assumed that the labor hours required to retrofit indoor HID fixtures are approximately the same as retrofitting outdoor HID fixtures.

Base Case Cost

| Solution Code | Description | Base Cost | Labor | Total Base Cost |
| --- | --- | --- | --- | --- |
| LT-10663 | Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case | $107.80 | $98.00 | $205.80 |
| LT-60651 | Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case | $154.91 | $98.00 | $252.91 |
| LT-21067 | Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case | $175.39 | $98.00 | $273.39 |
| LT-58832 | Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case | $266.40 | $98.00 | $364.40 |
| LT-78695 | Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case | $266.77 | $98.00 | $364.77 |

## 4.2 Measure Case Cost

The Measure Costs were taken from multiple online retailers [Attachment 3] and averaged.

Measure Cost

| Solution Code | Description | Measure Cost | Labor | Total Measure Cost |
| --- | --- | --- | --- | --- |
| LT-10663 | Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case | $291.51 | $98.00 | $389.51 |
| LT-60651 | Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case | $408.80 | $98.00 | $506.80 |
| LT-21067 | Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case | $467.83 | $98.00 | $565.83 |
| LT-58832 | Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case | $505.02 | $98.00 | $603.02 |
| LT-78695 | Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case | $580.23 | $98.00 | $678.23 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  For ROB, the IMC is the same value as GMC. The incremental cost is only used to help determine program incentives.  In the case of ROB, the equipment being replaced is assumed to have failed in place or is past its useful life. The customer is faced with either purchasing standard efficiency or code baseline equipment versus energy efficient equipment.  Therefore, gross measure cost (GMC) means the cost premium required to install the energy efficient measure over a less efficient piece of equipment.

The gross (or incremental) measure cost for the measures are shown in table below.

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| Up to 70 Watt Exterior Fixture Induction replacing less than or equal to 100 Watt fixture base case | ROB | $183.71 | $183.71 | N/A |
| Up to 100 Watt Exterior Fixture Induction replacing 101 - 175 Watt fixture base case | ROB | $253.88 | $253.88 | N/A |
| Up to 120 Watt Exterior Fixture Induction replacing 176 - 200 Watt fixture base case | ROB | $292.44 | $292.44 | N/A |
| Up to 180 Watt Exterior Fixture Induction replacing 201 - 399 Watt fixture base case | ROB | $238.63 | $238.63 | N/A |
| Up to 250 Watt Exterior Fixture Induction replacing 400 Watt fixture base case | ROB | $313.46 | $313.46 | N/A |

# Attachments

1.

2.

3.

# References

[14]

[49]

[51]

[215]

[355]

[422]

[A] RSMeans Electrical Cost Data 2010, 33 Annual Edition, by John H. Chiang (Editor)