Work Paper SCE13LG103

Revision 2

Southern California Edison Company

Interior LED Downlight Fixtures

# At-a-Glance Summary

| Applicable Measure Codes: | LT-61219, LT-68701, LT-16307, LT-89884 |
| --- | --- |
| Measure Description: | LED Downlights less than 15 Watts |
| Base Case Description: | PAR30 incandescent or halogen between 40 and 100 Watts |
| Energy Impact Common Units: | Lamp |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | Varies by building type |
| Measure Application Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | Default>2yrs:0.60 commercial 0.55 residential HTR-di 0.85 commercial/residential |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13LG103.0 | No | 03/30/12 | Peter Davi/SCE | - Original WP from WPSCNRLG0103.1 - Updated to latest WP template v0.1 - Updated Energy Star measures list as of April 2012 - Updated all Energy Savings Calculations in reference to new measures from Energy Star LED Product List |
|  |  |  | Brian V. O’Keefe/SCE | - Added Residential – Double Wide Mobile - Updated the EUL values for fixtures. |
| SCE13LG103.1 | Yes | 12/11/13 | Brian V. O’Keefe/SCE | - Updated measure wattage to be in line with the ED lighting retrofit workpaper disposition. |
| SCE13LG103.2 | Yes | 04/17/14 | Yun Han/SCE | - New WP template - Work paper updated for reporting period, effective 7/1/14-12/31/14 - Updated NTG - Code language update - Operating hours, IEs, load shape updated to CFL - Added Mid-stream and Direct Install delivery method - Retrofit install type and NTG for DI added - Added 2 new measures - Common Area and Dwelling Area for Res building types |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of pendant and recessed incandescent or halogen lamps that are between 40 and 100 watts, with LED down light modules that are less than 15 watts. The measures are shown in Table 1. It is assumed that the LED down light fixtures are replacing PAR30 lamps.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| LT-61219 | ≤ 15 Watt Down Light (Res) LED Fixture |
| LT-68701 | ≤ 15 Watt Down Light (Non Res) LED Fixture |
| LT-16307 | ≤ 15 Watt Down Light (Common Area) LED Fixture |
| LT-89884 | ≤ 15 Watt Down Light (Dwelling Area) LED Fixture |

Measures in this work paper must be on Energy Star’s qualified product list [] to receive incentives.

## 1.2 Technical Description

Downlights are part of a fixture in a 4- or 6-inch can installed flush with the ceiling. LED downlights are typically sold as a retrofit-kit which doesn’t require installation of new fixture housing. They simply replace the existing lamp and its trim.

## 1.3 Measure Application Type

The delivery method that is available for these measures is:

- Financial Support / Down-Stream Incentive – Deemed

- Financial Support/ Direct Install

- Midstream Programs/Mid-Stream Incentive

The program/install type for the above measures is:

- Replace on Burnout (ROB)

- Retrofit (RET) for Direct Install Only

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | Yes |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain LED fixture technology as measures |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID | NTG* |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.60 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.60 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.60 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Com-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTG-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Note that for the direct install delivery mechanism, a distinction between hard to reach and non-hard to reach markets will be made on a project by project basis. This work paper shows the NTG associated with a hard to reach direct install delivery mechanism and the residential/non-residential defaulted NTG value, where in fact, a measure offered through direct install and is not “hard to reach” will receive a default NTG value.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measue Case UseCategory | Lighting |
| Measure Case UseSubCats | Indoor General Lighting, Indoor Common Area Lighting |
| Measure Case TechGroups | Lighting – Lamps |
| Measure Case TechTypes | LED Lamp |
| Base Case TechGroups | Lighting - Lamps |
| Base Case TechTypes | Incandescent Lamp |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] Section 150.0(k)1 contains codes related to Residential lighting which includes a minimum 50 percent of total rated wattage in kitchen to be high efficacy, and non-high efficacy lighting to be controlled by vacancy sensors in certain areas for new construction. The measures in this work paper for Residential building types are not affected by this code.

Title 24 2013 Section 141.0(b)2 contains codes related to Nonresidential lighting as shown below. The measures in this work paper do change the light source in a luminaire and replace the optical system of a luminaire, which triggers Modifications-in-Place. Triggering Modifications-in-Place requires mandatory control provisions in Section 130.1(a)(b)(c)(d) for each enclosed space that includes Area, Shut-off, Multi-level, and if applicable, Daylighting Controls.

Lighting System Alterations shall meet the applicable requirements in TABLE 141.0-E and the following: - Lighting System Alterations include alterations where an existing lighting system is modified, luminaires are replaced, or luminaires are disconnected from the circuit, removed and reinstalled, whether in the same location or installed elsewhere. EXCEPTION 1 to Section 141.0(b)2Iii: Alterations that qualify as a Luminaire Modification-in- Place. EXCEPTION 2 to Section 141.0(b)2Iii: Portable luminaires, luminaires affixed to moveable partitions, and lighting excluded in accordance to Section 140.6(a)3. Luminaire Modifications-in-Place shall meet the applicable requirements in TABLE 141.0-F and the following: - To qualify as a Luminaire Modification-in-Place, luminaires shall only be modified by one or more of the following methods: - Replacing lamps and ballasts with like type or quantity in a manner that preserves the original luminaire listing. - Changing the number or type of light source in a luminaire including: socket renewal, removal or relocation of sockets or lampholders, and/or related wiring internal to the luminaire including the addition of safety disconnecting devices. - Changing the optical system of a luminaire in part or in whole. - Replacement of whole luminaires one for one in which the only electrical modification involves disconnecting the existing luminaire and reconnecting the replacement luminaire. - Luminaire Modifications-In-Place shall include only alterations to lighting system meeting the following conditions: - Luminaire Modifications-in-Place shall not be part of or the result of any general remodeling or renovation of the enclosed space in which they are located. - Luminaire Modifications-in-Place shall not cause, be the result of, or involve any changes to the panelboard or branch circuit wiring, including line voltage switches, relays, contactors, dimmers and other control devices, providing power to the lighting system. EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire Modifications-in-Place

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | 2013 Non-Residential Compliance manual, Section 141.0(b)2 | July 1, 2014 |
| Energy Star | Light Fixtures for Consumers | April 1, 2012 |

### 1.4.3 Non-DEER Study Review

ET07.15 – Recessed LED Downlights [416] report was used to qualify the technologies in this workpaper.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The ED December 2013 disposition [] provides the RUL of 5,500 hours for the existing technology. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the Energy Division’s Workpaper Disposition for Lighting Retrofits [B] was consulted. Table 7 below identifies the value/methodology used for the measures in this work paper. Residential fixtures have a maximum rated EUL of 16 years and 12 years for Nonresidential. Refer to the calculation template for EUL values.

Table 7 DEER14 EUL Value/Methodology

| READi EUL ID | Market | Enduse | Measure | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Com-LED-20000hr | Non-Residential | Lighting | LED Lighting | Varies by building type EUL = Rated Life of Lamp (20,000 hrs) / Annual Usage | RUL = (5,500/ Annual Usage)/3 |
| ILtg-Res-LED-20000hr | Residential | Lighting | LED Lighting | Varies by building type EUL = Rated Life of Lamp (20,000 hrs) / Annual Usage | RUL = (5,500/ Annual Usage)/3 = 3.4 |

# Section 2. Energy Savings & Demand Reduction Calculations

The original measure case was determined to be 11.64W by averaging LED light fixtures 15W and under [] obtained from ENERGY STAR® Commercial LED Product List [A].  ED’s May 2014 disposition [] recommends 10W to be used for the measure wattage with a PAR30 lamp Wattage Reduction Ratio (WRR) of 3.42 to calculate the base case as shown in Table 8.

Table 8 Delta Watts

| Measure Name | Measure Watts | WRR | Base Watts | Delta Watts |
| --- | --- | --- | --- | --- |
| ≤ 15 Watt Down Light (Res) LED Fixture | 10 | 3.42 | 34.2 | 24.2 |
| ≤ 15 Watt Down Light (Non Res) LED Fixture | 10 | 3.42 | 34.2 | 24.2 |
| ≤ 15 Watt Down Light (Common Area) LED Fixture | 10 | 3.42 | 34.2 | 24.2 |
| ≤ 15 Watt Down Light (Dwelling Area) LED Fixture | 10 | 3.42 | 34.2 | 24.2 |

## 2.1 Energy Savings Estimation Methodologies

The energy savings estimates are calculated as follows:

The following is a sample energy savings calculation for a ≤15 Watt Downlight (Non Res) LED Fixture in an Agricultural building type in Climate Zone 6.

1st baseline

EnergySavingskWhUnit×Year=34.2x2708-10.0x2674×1.057371,000WatthourskWh

EnergySavingskWhUnit×Year=69.65

2nd baseline

EnergySavingskWhUnit×Year=24.2×2674.0×1.057371,000WatthourskWh

EnergySavingskWhUnit×Year=68.42

## 2.2 Demand Reduction Estimation Methodologies

The demand reduction estimates are calculated as follows:

The following is a sample demand reduction calculation for a ≤15 Watt Downlight (Non Res) LED Fixture in an Agricultural building type in Climate Zone 6.

1st baseline

Demand ReductionkWUnit=34.2x0.665278-10.0x0.651778×1.102721,000WatthourskWh

Demand ReductionkWUnit=0.0179

2nd baseline

Demand ReductionkWUnit=24.2×0.651778×1.102721,000WatthourskWh

Demand ReductionkWUnit=0.01739

Non-residential savings use the reduced occupancy sensor operating hours due to code. Residential savings use the full residential operating hours. Please refer to the attachment [] for a full list of savings.

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available.  Therefore, the closest load shape chosen for this measure is the DEER:Indoor_ CFL_Ltg load shape.  See Table 9 for a list of all Building Types and Load Shapes.  See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | DEER:Indoor_CFL_Ltg | NON_RES |
| Assembly | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Primary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Secondary School | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - Community College | DEER:Indoor_CFL_Ltg | NON_RES |
| Education - University | DEER:Indoor_CFL_Ltg | NON_RES |
| Grocery | DEER:Indoor_CFL_Ltg | NON_RES |
| Food Store | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Hospital | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | DEER:Indoor_CFL_Ltg | NON_RES |
| Health/Medical - Clinic | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Hotel | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | DEER:Indoor_CFL_Ltg | NON_RES |
| Lodging - Motel | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | DEER:Indoor_CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | DEER:Indoor_CFL_Ltg | NON_RES |
| Industrial | DEER:Indoor_CFL_Ltg | NON_RES |
| Misc - Commercial | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Office - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | DEER:Indoor_CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Multistory Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Single-Story Large | DEER:Indoor_CFL_Ltg | NON_RES |
| Retail - Small | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Conditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Storage - Unconditioned | DEER:Indoor_CFL_Ltg | NON_RES |
| Transportation - Communication - Utilities | DEER:Indoor_CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | DEER:Indoor_CFL_Ltg | NON_RES |
| Residential Single Family | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-family | DEER:Indoor_CFL_Ltg | RES |
| Residential Mobile Home - Double-Wide | DEER:Indoor_CFL_Ltg | RES |
| Residential Single Family (Common Area) | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-family (Common Area) | DEER:Indoor_CFL_Ltg | RES |
| Residential Mobile Home - Double-Wide (Common Area) | DEER:Indoor_CFL_Ltg | RES |
| Residential Single Family (Dwelling Area) | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-family (Dwelling Area) | DEER:Indoor_CFL_Ltg | RES |
| Residential Mobile Home - Double-Wide (Dwelling Area) | DEER:Indoor_CFL_Ltg | RES |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

Base case cost of $9.51 from the PAR30 45-55 Watt range is taken from the statewide costs [] developed in response to the ED LED Disposition. The labor cost is $5.89 per lamp from DEER 2008 [215] for installing a screw-in CFL lamp. The total cost is shown in Table 10.

Table 10 Base Case Cost

| Measure Name | Base Equipment Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- |
| ≤ 15 Watt Down Light (Res) LED Fixture | $9.51 | $5.89 | $15.40 |
| ≤ 15 Watt Down Light (Non Res) LED Fixture | $9.51 | $5.89 | $15.40 |
| ≤ 15 Watt Down Light (Common Area) LED Fixture | $9.51 | $5.89 | $15.40 |
| ≤ 15 Watt Down Light (Dwelling Area) LED Fixture | $9.51 | $5.89 | $15.40 |

## 4.2 Measure Case Cost

Measure case cost of $45.23 is also taken from the statewide costs [F], using the same labor as the base case, shown in Table 11.

Table 11 Measure Case Cost

| Measure Name | Measure Equipment Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- |
| ≤ 15 Watt Down Light (Res) LED Fixture | $45.23 | $5.89 | $51.12 |
| ≤ 15 Watt Down Light (Non Res) LED Fixture | $45.23 | $5.89 | $51.12 |
| ≤ 15 Watt Down Light (Common Area) LED Fixture | $45.23 | $5.89 | $51.12 |
| ≤ 15 Watt Down Light (Dwelling Area) LED Fixture | $45.23 | $5.89 | $51.12 |

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For RET 1st baseline, GMC is the Measure Case Cost as shown in Table 11.

For ROB and RET 2nd baseline, GMC is represented by the equation below and shown in Table 12.

GMC 	= Measure Equipment Cost – Base Case Equipment Cost

Table 12 Gross Measure Cost

| Measure Name | Measure Equipment Cost | Base Equipment Cost | Gross Measure Cost |
| --- | --- | --- | --- |
| ≤ 15 Watt Down Light (Res) LED Fixture | $45.23 | $9.98 | $35.72 |
| ≤ 15 Watt Down Light (Non Res) LED Fixture | $45.23 | $9.98 | $35.72 |
| ≤ 15 Watt Down Light (Common Area) LED Fixture | $45.23 | $9.98 | $35.72 |
| ≤ 15 Watt Down Light (Dwelling Area) LED Fixture | $45.23 | $9.98 | $35.72 |

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the same as Gross Measure Cost as shown in Table 12.

# Attachments

1. 2.3. 4. 5.

# References

[31]

[215]

[351]

[355]

[416]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |

http://www.energystar.gov/certified-products/detail/light_fixtures

Attachment 2 – 2013-2014_LightingRetrofit_Disposition-14December2013.xlsx

Attachment 3– LED Average Energy Consumption (Measure).xls

Attachment 4 – 2013-2014_LightingRetrofit_Disposition-30May2014.xlsx

Attachment 1 – Calculation Template 2015 v3.xlsm

Attachment 5 – IMC Analysis 052912-mam.xls