Work Paper SCE13LG106

Revision 4

Southern California Edison

MR16, PAR20, PAR30, PAR38, A, Candelabra, Globe Lamps, and Residential LED Lamp Giveaway

# At-a-Glance Summary

| Measure Codes | Refer to Excel Calculation Attachment |
| --- | --- |
| Measure Description | LED MR16, PAR20/30/38, Candelabra, Globe, A19 lamps |
| Base Case Description | Incandescent/halogen/CFL MR16, PAR20/30/38, Candelabra, Globe, A19 lamps |
| Units | Per lamp |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | ILtg-Com-LED-20000hr, ILtg-Res-LED-20000hr, ILtg-Com-LED-15000hr, ILtg-Res-LED-15000hr |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Com-Default>2yrs – 0.6, Com-Default-HTR-di – 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 05/02/12 | Yun Han/SCE | - Original work paper from WPSCNRLG0106.2 - Updated to latest WP template v0.1 - Added PAR20 measures Used DEER recommended wattage ratios for all measures to calculate baseline |
| 1 | 09/05/13 | Jason Wang/SCE | - Per 3/1/13 ED Lighting Retrofit Disposition, the measure case wattages were adjusted to more conservative values. - Updated code to T20 2012 and T24 2013 - Added in the solution codes for PAR20 measures. - Added EUL_ID of “Non-DEER” Added new measures from PG&E, including candelabras |
| 2 | 02/20/14 | Yun Han/SCE | - New WP template - SCE13LG079.0 merged to this WP (Added LT-10754 8 Watt Giveaway LED) - Work paper updated for reporting period, effective 7/1/14 – 12/31/14 - Savings updated based on 5/30/14 ED disposition - Added A-lamp measures exclusively for hotel/motel (LT-30499, LT-74612) Changed Non-CFL to CFL factors, including load shape (per D.12-05-015, Attachment A, page 9) |
| 3 | 9/10/15 | Yun Han/SCE | - New template for 2016 program year - Navigant LED Study and WO17 used for costs - Added new measures: - 25W to ≥31W A LED - 20W to ≥22W PAR30 LED - <3W, ≥3W to ≤5W Candelabra LED - 1W - 10W Globe LED - Removed existing measures: - ≥25W A (LT-28046, LT-72265, LT-10844) - ≥20W PAR30 (LT-72786, LT-61725, LT-77038) - Updated Energy Star requirements to V1.1 - Work paper updated for reporting period, effective 1/1/15 – 12/31/15 |
| 4 | 7/19/16 | Ryan Cho/SCE | - Prior workpaper template had incorrect EUL cap for res lamps, although correct in the text. Workpaper template updated for correct EUL values for RES. |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper documents the E3 Calculator inputs for replacing conventional MR-16, PAR20, PAR30, and PAR38 halogen, candelabra/globe incandescent, and A-type incandescent/halogen/CFL lamps with Energy Star certified LED lamps [488]. The LED lamps shall generate equal or greater lumen output at lower power consumption than their base case counterparts or provide equal or greater center beam candle power for directional lamps to achieve energy savings.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | LED integral lamps – MR, PAR, A, Globe, Candelabra |
| Existing Condition | Incandescent/Halogen/CFL lamps – MR, PAR, A, Globe, Candelabra |
| Code/Standard | Performance based, not light type |
| Industry Standard Practice | N/A |

There are 3 tables below with a list of measures intended to be used for the programs specified.

The following measures are used in the Down-Stream Deemed Program.

Measures and Codes for Down-Stream Deemed Delivery Method

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-12834 |  | < 10 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-10965 |  | 10 Watt to 30 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-30499 |  | < 10 Watt A-Lamp (Hotel/Motel) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-74612 |  | 10 Watt to 30 Watt A-Lamp (Hotel/Motel) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89075 |  | < 8 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-91612 |  | 8 Watt to 12 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-65785 |  | < 15 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-89789 |  | 15 Watt to 21 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-90344 |  | < 17 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-92133 |  | 17 Watt to 25 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-19877 |  | < 6 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-54654 |  | 6 Watt to 10 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-59284 |  | <3 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-82451 |  | ≥3 Watt to ≤5 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-89192 |  | <3 Watt Globe LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-92728 |  | 3 Watt to 10 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |

The following measure is used in the Giveaway Program.

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-10754 |  | 8 Watt Giveaway LED |

The following measures are used in the Direct Install, Mid-stream Incentive, and Up-Stream Buy-Down Programs.

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-45662 |  | < 8 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-22929 |  | 8 Watt to < 9 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-54980 |  | 9 Watt to < 10 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-64121 |  | 10 Watt to < 11 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-78423 |  | 11 Watt to < 12 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89542 |  | 12 Watt to < 13 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-90243 |  | 13 Watt to < 14 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-10795 |  | 14 Watt to < 15 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-56673 |  | 15 Watt to < 16 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-78681 |  | 16 Watt to < 17 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-95419 |  | 17 Watt to < 18 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-32283 |  | 18 Watt to < 19 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-43162 |  | 19 Watt to < 20 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-62527 |  | 20 Watt to < 21 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-55248 |  | 21 Watt to < 22 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-92044 |  | 22 Watt to < 23 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-39756 |  | 23 Watt to < 24 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-26164 |  | 24 Watt to < 25 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-96337 |  | 25 Watt to < 26 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-94735 |  | 26 Watt to < 27 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-99541 |  | 27 Watt to < 28 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-85121 |  | 28 Watt to < 29 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-91660 |  | 29 Watt to < 30 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-98603 |  | 30 Watt to < 31 Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-64495 |  | >= 31Watt A-Lamp LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-33509 |  | < 8 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-67884 |  | 8 Watt to < 9 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-78625 |  | 9 Watt to < 10 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-55814 |  | 10 Watt to < 11 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-61249 |  | 11 Watt to < 12 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-40712 |  | 12 Watt to < 13 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-41701 |  | 13 Watt to < 14 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-27084 |  | 14 Watt to < 15 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89729 |  | 15 Watt to < 16 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-95724 |  | 16 Watt to < 17 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-16665 |  | 17 Watt to < 18 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-45182 |  | 18 Watt to < 19 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-22361 |  | 19 Watt to < 20 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-27872 |  | 20 Watt to < 21 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-98958 |  | 21 Watt to < 22 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-90897 |  | 22 Watt to < 23 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-75523 |  | 23 Watt to < 24 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-61474 |  | 24 Watt to < 25 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-97939 |  | 25 Watt to < 26 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89928 |  | 26 Watt to < 27 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-91862 |  | 27 Watt to < 28 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-71438 |  | 28 Watt to < 29 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-52009 |  | 29 Watt to < 30 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-90996 |  | 30 Watt to < 31 Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-60757 |  | >= 31Watt A-Lamp (Common Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-49398 |  | < 8 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-53867 |  | 8 Watt to < 9 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-91742 |  | 9 Watt to < 10 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-10495 |  | 10 Watt to < 11 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-63721 |  | 11 Watt to < 12 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-29876 |  | 12 Watt to < 13 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-72183 |  | 13 Watt to < 14 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-37254 |  | 14 Watt to < 15 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-28893 |  | 15 Watt to < 16 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-50240 |  | 16 Watt to < 17 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-73324 |  | 17 Watt to < 18 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-14853 |  | 18 Watt to < 19 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-95896 |  | 19 Watt to < 20 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-13541 |  | 20 Watt to < 21 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-85821 |  | 21 Watt to < 22 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-84121 |  | 22 Watt to < 23 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-90125 |  | 23 Watt to < 24 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-48765 |  | 24 Watt to < 25 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-65895 |  | 25 Watt to < 26 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-99339 |  | 26 Watt to < 27 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-99007 |  | 27 Watt to < 28 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-86724 |  | 28 Watt to < 29 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-86853 |  | 29 Watt to < 30 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-63022 |  | 30 Watt to < 31 Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-71236 |  | >= 31Watt A-Lamp (Dwelling Area) LED Replacing A19 Basecase, Total Watts = 2.96 x Msr Watts |
|  |  | LT-20730 |  | < 6 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-49725 |  | 6 Watt to < 7 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-10919 |  | 7 Watt to < 8 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-87178 |  | 8 Watt to < 9 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-89267 |  | 9 Watt to < 10 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-90684 |  | >= 10 Watt PAR20 LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-65597 |  | < 6 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-13017 |  | 6 Watt to < 7 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-18948 |  | 7 Watt to < 8 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-29846 |  | 8 Watt to < 9 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-30941 |  | 9 Watt to < 10 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-29462 |  | >= 10 Watt PAR20 (Common Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-39956 |  | < 6 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-32654 |  | 6 Watt to < 7 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-49596 |  | 7 Watt to < 8 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-46460 |  | 8 Watt to < 9 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-56172 |  | 9 Watt to < 10 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-65498 |  | >= 10 Watt PAR20 (Dwelling Area) LED Replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts |
|  |  | LT-29560 |  | < 10 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-38391 |  | 10 Watt to < 11 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-38887 |  | 11 Watt to < 12 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-49889 |  | 12 Watt to < 13 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-85840 |  | 13 Watt to < 14 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-13840 |  | 14 Watt to < 15 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-98682 |  | 15 Watt to < 16 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-76561 |  | 16 Watt to < 17 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-82047 |  | 17 Watt to < 18 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-47333 |  | 18 Watt to < 19 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-53339 |  | 19 Watt to < 20 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-90260 |  | 20 Watt to < 21 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-55011 |  | 21 Watt to < 22 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-95269 |  | >= 22 Watt PAR30 LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-58543 |  | < 10 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-56977 |  | 10 Watt to < 11 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-23336 |  | 11 Watt to < 12 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-67054 |  | 12 Watt to < 13 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-73654 |  | 13 Watt to < 14 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-15208 |  | 14 Watt to < 15 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-93469 |  | 15 Watt to < 16 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-74244 |  | 16 Watt to < 17 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-35298 |  | 17 Watt to < 18 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-55621 |  | 18 Watt to < 19 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-78587 |  | 19 Watt to < 20 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-51475 |  | 20 Watt to < 21 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-92598 |  | 21 Watt to < 22 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-55950 |  | >= 22 Watt PAR30 (Common Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-86204 |  | < 10 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-87965 |  | 10 Watt to < 11 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-67912 |  | 11 Watt to < 12 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-98765 |  | 12 Watt to < 13 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-39240 |  | 13 Watt to < 14 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-15476 |  | 14 Watt to < 15 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-87961 |  | 15 Watt to < 16 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-27168 |  | 16 Watt to < 17 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-54713 |  | 17 Watt to < 18 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-46919 |  | 18 Watt to < 19 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-62358 |  | 19 Watt to < 20 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-83317 |  | 20 Watt to < 21 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-60352 |  | 21 Watt to < 22 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-79247 |  | >= 22 Watt PAR30 (Dwelling Area) LED Replacing PAR30 Basecase, Total Watts = 3.42 x Msr Watts |
|  |  | LT-22743 |  | < 12 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-71886 |  | 12 Watt to < 13 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-19711 |  | 13 Watt to < 14 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-69480 |  | 14 Watt to < 15 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-34275 |  | 15 Watt to < 16 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-67816 |  | 16 Watt to < 17 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-27996 |  | 17 Watt to < 18 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-71514 |  | 18 Watt to < 19 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-43354 |  | 19 Watt to < 20 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-95006 |  | 20 Watt to < 21 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-20374 |  | 21 Watt to < 22 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-21907 |  | 22 Watt to < 23 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-48709 |  | 23 Watt to < 24 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-43942 |  | 24 Watt to < 25 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-65525 |  | 25 Watt to < 26 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-59745 |  | 26 Watt to < 27 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-58476 |  | >= 27 Watt PAR38 LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-68754 |  | < 12 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-76587 |  | 12 Watt to < 13 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-12326 |  | 13 Watt to < 14 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-89034 |  | 14 Watt to < 15 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-91034 |  | 15 Watt to < 16 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-91663 |  | 16 Watt to < 17 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-67849 |  | 17 Watt to < 18 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-73876 |  | 18 Watt to < 19 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-21719 |  | 19 Watt to < 20 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-28020 |  | 20 Watt to < 21 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-90289 |  | 21 Watt to < 22 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-89886 |  | 22 Watt to < 23 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-44444 |  | 23 Watt to < 24 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-98624 |  | 24 Watt to < 25 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-89165 |  | 25 Watt to < 26 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-25103 |  | 26 Watt to < 27 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-56386 |  | >= 27 Watt PAR38 (Common Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-93897 |  | < 12 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-10395 |  | 12 Watt to < 13 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-43500 |  | 13 Watt to < 14 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-15012 |  | 14 Watt to < 15 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-66727 |  | 15 Watt to < 16 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-97963 |  | 16 Watt to < 17 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-98642 |  | 17 Watt to < 18 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-31995 |  | 18 Watt to < 19 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-35132 |  | 19 Watt to < 20 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-17721 |  | 20 Watt to < 21 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-97480 |  | 21 Watt to < 22 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-20575 |  | 22 Watt to < 23 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-91264 |  | 23 Watt to < 24 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-39507 |  | 24 Watt to < 25 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-21854 |  | 25 Watt to < 26 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-16127 |  | 26 Watt to < 27 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-41095 |  | >= 27 Watt PAR38 (Dwelling Area) LED Replacing PAR38 Basecase, Total Watts = 3.81 x Msr Watts |
|  |  | LT-87923 |  | 6 Watt to < 7 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-70734 |  | 7 Watt to < 8 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-49384 |  | 8 Watt to < 9 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-50786 |  | 9 Watt to < 10 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-67244 |  | 10 Watt to < 11 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-39798 |  | 11 Watt to < 12 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-49980 |  | >= 12 Watt MR16 LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-34578 |  | 6 Watt to < 7 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-50832 |  | 7 Watt to < 8 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-78066 |  | 8 Watt to < 9 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-77649 |  | 9 Watt to < 10 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-50434 |  | 10 Watt to < 11 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-60980 |  | 11 Watt to < 12 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-83345 |  | >= 12 Watt MR16 (Common Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-34687 |  | 6 Watt to < 7 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-89037 |  | 7 Watt to < 8 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-18539 |  | 8 Watt to < 9 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-78912 |  | 9 Watt to < 10 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-98882 |  | 10 Watt to < 11 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-90345 |  | 11 Watt to < 12 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-27314 |  | >= 12 Watt MR16 (Dwelling Area) LED Replacing MR16 Basecase, Total Watts = 4.24 x Msr Watts |
|  |  | LT-76109 |  | < 1 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-79284 |  | 1 Watt to < 2 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-48077 |  | 2 Watt to < 3 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-48560 |  | 3 Watt to < 4 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-48123 |  | >= 4 Watt Candelabra LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-69087 |  | < 1 Watt Candelabra (Common Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-93364 |  | 1 Watt to < 2 Watt Candelabra (Common Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-78570 |  | 2 Watt to < 3 Watt Candelabra (Common Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-39568 |  | 3 Watt to < 4 Watt Candelabra (Common Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-78489 |  | >= 4 Watt Candelabra (Common Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-28818 |  | < 1 Watt Candelabra (Dwelling Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-63685 |  | 1 Watt to < 2 Watt Candelabra (Dwelling Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-34547 |  | 2 Watt to < 3 Watt Candelabra (Dwelling Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-90388 |  | 3 Watt to < 4 Watt Candelabra (Dwelling Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-70545 |  | >= 4 Watt Candelabra (Dwelling Area) LED Replacing Candelabra Basecase, Total Watts = 7.35 x Msr Watts |
|  |  | LT-68897 |  | <2 Watt Globe LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-53611 |  | 2 Watt to <3 Watt Globe LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-86190 |  | 3 Watt to <4 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-88455 |  | 4 Watt to <5 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-58216 |  | 5 Watt to <6 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-85655 |  | 6 Watt to <7 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-90058 |  | 7 Watt to <8 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-53409 |  | 8 Watt to <9 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-50075 |  | 9 Watt to <10 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-83519 |  | >=10 Watt Globe LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-96466 |  | <2 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-70904 |  | 2 Watt to <3 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-71033 |  | 3 Watt to <4 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-97203 |  | 4 Watt to <5 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-55416 |  | 5 Watt to <6 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-67497 |  | 6 Watt to <7 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-94532 |  | 7 Watt to <8 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-63427 |  | 8 Watt to <9 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-93464 |  | 9 Watt to <10 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-66631 |  | >=10 Watt Globe (Common Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-83851 |  | <2 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-58750 |  | 2 Watt to <3 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 7.47 x Msr Watts |
|  |  | LT-84053 |  | 3 Watt to <4 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-94864 |  | 4 Watt to <5 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-88989 |  | 5 Watt to <6 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-56614 |  | 6 Watt to <7 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-62359 |  | 7 Watt to <8 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-96669 |  | 8 Watt to <9 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-57018 |  | 9 Watt to <10 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |
|  |  | LT-62691 |  | >=10 Watt Globe (Dwelling Area) LED Replacing Globe Basecase, Total Watts = 4.94 x Msr Watts |

Measures in this work paper must be on Energy Star’s qualified product list [489] to receive incentives and are available for all Climate Zones and Building Types.

The Midstream Lighting Program provides incentives on certain pre-approved energy-efficient products at the point of sale in order to decrease electricity consumption in SCE’s service territory.  SCE reimburses the distributor a pre-authorized incentive amount for each qualifying product sold to a qualifying purchaser for which the distributor has provided a point of sale discount in the amount of such incentive, subject to program requirements described in the midstream program agreement.  The distributor provides sales data including make & model to verify the wattage of the measures. An inspector will also visit the site where these measures were installed to verify the installation.

The Non Residential Direct Install program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is a form which information for measures installed in all building types related to the Non Residential Direct Install program.  The PLF contains the following fields: Service Account Address, Measures Proposed/Installed, Product Make/Model, Install Locations (detailed to define separate spaces/floors, as well as specific locations within the space including but not limited to:  Bathrooms, Hallways, Meeting Rooms, Offices, Warehouse, etc.).

## 1.2 Technical Description

The lamps in this workpaper come in variety of bases based on the type. A-lamps and all PAR lamps come in a standard E26 screw-base, Globe and Candelabra in E12/E26, and MR16/MRx16 in GU5.3/GU10 which match the baseline counterpart. They are used in various applications that match their base.

## 1.3 Installation Types and Delivery Mechanisms

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Up-Stream Programs | See Up-Stream Incentive and Up-Stream Buy Down in the Incentive Method table. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Mid-Stream Incentive | The program gives a financial incentive to a midstream market actor, such as a retailer or contractor, to encourage the promotion of efficient measures. The incentive may or may not be passed on to the end-use customer. |
| Up-Stream Incentive | The program gives a financial incentive to an upstream market actor, such as a manufacturer or distributor, to encourage the manufacture, provision, or distribution of an efficient measure. The incentive may or may not be passed on to the end-use customer. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |

The delivery method for the LED Giveaway is:

- Financial Support / Giveaway

The delivery method for Dwelling Area, and Common Area is:

- Financial Support / Direct Install

The delivery method for Hotel/Motel is:

- Financial Support / Down-Stream Incentive

The delivery methods for the rest of the measures are:

- Financial Support / Direct Install

- Financial Support / Down-Stream Incentive – Deemed

- Midstream Programs / Mid-Stream Incentive

- Upstream Programs / Up-Stream Incentive

## 1.4 Measure Parameters

### 1.4.1 DEER Data

Most measures in this work paper use output from the READI tool. Measures that do not have Measure IDs use the approved work paper template to manually calculate energy savings. Also for certain measures, READI does not provide savings for all building types. These measures are also manually calculated using the approved work paper template.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | Yes |
| DEER Base Case | Yes/No |
| DEER Measure Case | Yes/No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI 2.3.0 |
| Reason for Deviation from DEER | DEER does not contain the new wattages added in this revision |
| DEER Measure IDs Used | C-In-LED-A19(4w)-dWP7 |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.60 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.60 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.60 |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes Multi-family measures that are offered via direct install activities into hard-to-reach (HTR) customer homes.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer homes as HTR. The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria.

SCE’s Multi-Family Energy Efficiency Rebate (MFEER) program addresses the ongoing concern with “split incentives”, where the residents are not the owners of the property, so they lack incentive to improve their energy usage. Similarly, the property owners do not live on-site and pay higher utility expenses due to inefficient appliances, thus lack any incentive to upgrade. The MFEER is designed to drive this customer segment toward participation by offering property owners a variety of energy efficiency measures and services.  The MFEER program will offer and track measure installations in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums.  Measures offered via direct install activities in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums will receive the HTR NTG.  Other measures in the MFEER program will receive default NTG (NTGR_ID: Res-Default>2), unless otherwise specified in DEER.

This work paper includes Commercial measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

The rated life for most LED lamps is 20,000 hours. Globes up to 2 Watts and all Candelabras are rated for 15,000 hours. The maximum EUL for Nonresidential is 12 years and 16 for Residential.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Com-LED-20000hr | LED Lamp - Indoor- Commercial | Com | Lighting | Varies by building type EUL = Rated Life of Lamp / Annual Usage | N/A |
| ILtg-Res-LED-20000hr | LED lamp - Indoor - Residential | Res | Lighting | Varies by building type EUL = Rated Life of Lamp / Annual Usage | N/A |
| ILtg-Com-LED-15000hr | LED Lamp - Indoor- Commercial - Small wattage Globe, Any Candle shape | Com | Lighting | Varies by building type EUL = Rated Life of Lamp / Annual Usage | N/A |
| ILtg-Res-LED-15000hr | LED lamp - Indoor - Residential - small wattage Globe, Any Candle shape | Res | Lighting | Varies by building type EUL = Rated Life of Lamp / Annual Usage | N/A |

### 1.4.2 Codes and Standards Analysis

Title 20: According to Title 20 2014 [493], Section 1605.1(k) – Lamps section lists the minimum requirements for incandescent reflector lamps, medium base compact fluorescent lamps, and general service incandescent lamps which affect the measures in this work paper.

Energy Star: Energy Star issued a document, ENERGY STAR Program Requirements for Integral LED Lamps – Partner Commitments Version 1.1 [488] which details the Energy Star requirements for Integral LED Lamps. The requirements in the document consist of two parts; general requirement for all LED lamps and lamp type specific requirement. Those requirements are the minimum requirements for an integral LED lamp to be adopted by any SCE energy efficiency program.

Although code exists, this work paper uses recommended wattage reduction ratios from the Energy Division’s Workpaper Disposition for Lighting Retrofits [433] to calculate the base case wattage. Therefore, Title 20 code does not apply.

Replacing lamps do not trigger Title 24 2013 [355].

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Section 1605.1(k) | January 1, 2016 |
| Energy Star | Version 1.1 | September 30, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

Three Emerging Technology studies were conducted for the measures included in this work paper. Studies include A-19, MR16, and screw-in directional lamps.

### 1.5.1 LED MR16 Lamps

- Emerging Technology Evaluation, Vireak Ly, August 2009

- NonRes and Res

- Integrated Sphere testing, lumen depreciation, light distribution

- Technology Validation

### 1.5.2 LED Screw-in Directional Lamps

- Emerging Technology Evaluation, Vireak Ly, December 2009

- NonRes and Res

- Integrated Sphere testing, lumen depreciation, light distribution

- Technology Validation

### 1.5.3 LED A-19 Lamps

- Emerging Technology Evaluation, Yun Han, December 2010

- NonRes and Res

- Integrated Sphere testing

- Technology Validation

## 1.6 Data Quality and Future Data Needs

The energy savings and total resource cost could be better defined with updates to the market saturation of the baseline technologies and costs. The offer structure should also be updated to tier the measures into different efficacies so that the most efficient measure is able to claim the most savings.

# Section 2. Calculation Methodology

The base case wattage is calculated based on the Wattage Reduction Ratio (WRR) multiplied by the measure wattage. Measures with wattages that are not in READI uses the WRR to calculate the baseline wattage for energy savings and demand reduction.

Wattage Reduction Ratio

| Lamp Type | WRR |
| --- | --- |
| A-19 | 2.96 |
| MR16 | 4.24 |
| PAR20 | 4.7 |
| PAR30 | 3.42 |
| PAR38 | 3.81 |
| Candelabra | 7.35 |
| Globe up to 2 Watts | 7.47 |
| Globe 3 Watts or greater | 4.94 |

The energy savings estimates are calculated as follows:

The following is a sample energy savings calculation for 8 Watt to 12 Watt PAR20 (Common Area) LED replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts in Doublewide Mobile Home in Climate Zone 6.

EnergySavingskWhUnit×Year=[8×4.7-8]×6142×1.031,000WatthourskWh

EnergySavingskWhUnit×Year=187.25

The demand reduction estimates are calculated as follows:

The following is a sample demand reduction calculation for 8 Watt to 12 Watt PAR20 (Common Area) LED replacing PAR20 Basecase, Total Watts = 4.7 x Msr Watts in Doublewide Mobile Home in Climate Zone 6.

Demand ReductionkWUnit=[8×4.7-8]×.043×1.171,000WatthourskWh

Demand ReductionkWUnit=0.00149

The following table indicates a sample of which measures are taken directly from or created with the DEER READI tool. A full list of measures and IDs can be found in the attachments [Attachment 1].

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| LT-10395 | 12 Watt to < 13 Watt PAR38 (Dwelling Area) LED | R-In-LED-PAR38(12w)-dWP33 |
| LT-10495 | 10 Watt to < 11 Watt A-Lamp (Dwelling Area) LED | R-In-LED-A19(10w)-dWP19 |
| LT-10754 | 8 Watt Giveaway LED | R-In-LED-A19(8w)-dWP15 |
| LT-10795 | 14 Watt to < 15 Watt A-Lamp LED | R-In-LED-A19(14w)-dWP27 |
| LT-10919 | 7 Watt to < 8 Watt PAR20 LED | R-In-LED-PAR20(7w)-dWP25 |
| LT-10965 | 10 Watt to 30 Watt A-Lamp LED | R-In-LED-A19(10w)-dWP19 |
| LT-12834 | < 10 Watt A-Lamp LED | R-In-LED-A19(4w)-dWP7 |
| LT-13541 | 20 Watt to < 21 Watt A-Lamp (Dwelling Area) LED | R-In-LED-A19(20w)-dWP39 |
| LT-13840 | 14 Watt to < 15 Watt PAR30 LED | R-In-LED-PAR30(14w)-dWP33 |
| LT-14853 | 18 Watt to < 19 Watt A-Lamp (Dwelling Area) LED | R-In-LED-A19(18w)-dWP35 |
| LT-15012 | 14 Watt to < 15 Watt PAR38 (Dwelling Area) LED | R-In-LED-PAR38(14w)-dWP39 |
| LT-15476 | 14 Watt to < 15 Watt PAR30 (Dwelling Area) LED | R-In-LED-PAR30(14w)-dWP33 |
| LT-16127 | 26 Watt to < 27 Watt PAR38 (Dwelling Area) LED | R-In-LED-PAR38(26w)-dWP73 |
| LT-17721 | 20 Watt to < 21 Watt PAR38 (Dwelling Area) LED | R-In-LED-PAR38(20w)-dWP56 |
| LT-18539 | 8 Watt to < 9 Watt MR16 (Dwelling Area) LED | R-In-LED-MR16(8w)-dWP25 |
| LT-19711 | 13 Watt to < 14 Watt PAR38 LED | R-In-LED-PAR38(13w)-dWP36 |
| LT-19877 | < 6 Watt MR16 LED | R-In-LED-MR16(3w)-dWP9 |
| LT-20374 | 21 Watt to < 22 Watt PAR38 LED | R-In-LED-PAR38(21w)-dWP59 |
| LT-20575 | 22 Watt to < 23 Watt PAR38 (Dwelling Area) LED | R-In-LED-PAR38(22w)-dWP61 |

Savings for all building types and climate zones can be found in the Attachments section [attachment 1].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Community College | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Primary School | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Secondary School | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - University | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Grocery | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Hospital | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Hotel | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Motel | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Small | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Multistory Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Single-Story Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Small | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Conditioned | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Unconditioned | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | DEER:Indoor_Non-CFL_Ltg | NON_RES |

# Section 4. Costs

Most costs for LED lamps were provided by Navigant as part of a study on LEDs. Several were interpolated or extrapolated from the Navigant data. The California LED Workpaper Update Study [C] recommends using 25 percentile utilizing CA specific data.

The base case costs are weighted differently for each lamp category between CFL and Incandescent lamp types. CFL costs are taken from READI. Costs not available from READI have been interpolated. Incandescent costs are calculated from WO17 workbook [Attachment 2]. The base case wattages are mapped to individual LED wattages using a table from the Energy Star Calculator.

All cost data can be found in the attachment [Attachment 3].

## 4.1 Base Case Cost

Table below shows a sample of base costs in this WP. A full list of savings in different climate zones can be found in the attachment [Attachment 1].

Base Case Cost

| Solution Code | Measure Name | Equipment Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- | --- |
| LT-12834 | < 10 Watt A-Lamp LED | $3.21 | $5.75 | $8.97 |
| LT-10965 | 10 Watt to 30 Watt A-Lamp LED | $3.50 | $5.75 | $9.26 |
| LT-45662 | < 8 Watt A-Lamp LED | $3.28 | $5.75 | $9.04 |
| LT-22929 | 8 Watt to < 9 Watt A-Lamp LED | $3.35 | $5.75 | $9.10 |
| LT-54980 | 9 Watt to < 10 Watt A-Lamp LED | $3.47 | $5.75 | $9.23 |
| LT-64121 | 10 Watt to < 11 Watt A-Lamp LED | $3.50 | $5.75 | $9.26 |
| LT-78423 | 11 Watt to < 12 Watt A-Lamp LED | $3.54 | $5.75 | $9.29 |
| LT-89542 | 12 Watt to < 13 Watt A-Lamp LED | $3.57 | $5.75 | $9.33 |
| LT-90243 | 13 Watt to < 14 Watt A-Lamp LED | $3.64 | $5.75 | $9.39 |
| LT-10795 | 14 Watt to < 15 Watt A-Lamp LED | $3.71 | $5.75 | $9.46 |

## 4.2 Measure Case Cost

Table below shows a sample of measure costs in this WP. A full list of savings in different climate zones can be found in the attachment [Attachment 1].

Measure Case Cost

| Solution Code | Measure Name | Equipment Cost | Labor Cost | Total Cost |
| --- | --- | --- | --- | --- |
| LT-12834 | < 10 Watt A-Lamp LED | $7.79 | $5.75 | $13.54 |
| LT-10965 | 10 Watt to 30 Watt A-Lamp LED | $8.06 | $5.75 | $13.81 |
| LT-45662 | < 8 Watt A-Lamp LED | $7.79 | $5.75 | $13.54 |
| LT-22929 | 8 Watt to < 9 Watt A-Lamp LED | $10.31 | $5.75 | $16.06 |
| LT-54980 | 9 Watt to < 10 Watt A-Lamp LED | $9.12 | $5.75 | $14.87 |
| LT-64121 | 10 Watt to < 11 Watt A-Lamp LED | $8.06 | $5.75 | $13.81 |
| LT-78423 | 11 Watt to < 12 Watt A-Lamp LED | $12.66 | $5.75 | $18.41 |
| LT-89542 | 12 Watt to < 13 Watt A-Lamp LED | $12.48 | $5.75 | $18.24 |
| LT-90243 | 13 Watt to < 14 Watt A-Lamp LED | $15.97 | $5.75 | $21.72 |
| LT-10795 | 14 Watt to < 15 Watt A-Lamp LED | $21.54 | $5.75 | $27.30 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-12834 | ROB | $4.58 | $4.58 | N/A |
| LT-10965 | ROB | $4.55 | $4.55 | N/A |
| LT-45662 | ROB | $4.51 | $4.51 | N/A |
| LT-22929 | ROB | $6.96 | $6.96 | N/A |
| LT-54980 | ROB | $5.64 | $5.64 | N/A |
| LT-64121 | ROB | $4.55 | $4.55 | N/A |
| LT-78423 | ROB | $9.12 | $9.12 | N/A |
| LT-89542 | ROB | $8.91 | $8.91 | N/A |
| LT-90243 | ROB | $12.33 | $12.33 | N/A |
| LT-10795 | ROB | $17.83 | $17.83 | N/A |

# Attachments

1. 2. 3.

# References

[355]

[493]

[433]

[488]

[489]

[A] http://www.energystar.gov/sites/default/files/asset/document/light_bulb_calculator_1.xlsx

[B] http://www.energystar.gov/buildings/sites/default/uploads/files/light_fixture_ceiling_fan_calculator.xlsx?4349-303e

[C] http://www.energydataweb.com/cpucFiles/pdaDocs/1359/LED%20Study_Draft%20Final%20Report_20150828_Clean.pdf