Work Paper SCE13LG108

Revision 3

Southern California Edison

Exterior LED Wall Pack Fixtures

For Work Paper Reviewer Use Only

List all major comments that occurred during the review. This table may only be removed during management review.

| Major Comment | Reviewer Name | Date | Outcome/Resolution |
| --- | --- | --- | --- |
| E.g. Please remove measure LT-12345 (LD123) from this work paper because it is no longer eligible for incentives. | Reviewer 1 | 6/1/15 | E.g. Comment incorporated. LT-12345 was removed. |

# At-a-Glance Summary

| Measure Codes | See Section 1 |
| --- | --- |
| Measure Description | LED exterior wall packs |
| Base Case Description | Metal Halide (MH), Pulse Start Metal Halide (PSMH), Compact Fluorescent (CFL), and High Pressure Sodium (HPS) exterior wall packs |
| Units | Fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | OLtg-Com-LED-50000hr – 12 years OLtg-Res-LED-50000hr – 16 years |
| Measure Installation Type | Replace on Burnout (ROB) and Retrofit (RET) |
| Net-to-Gross Ratio | Res-Default>2: 0.55 Com-Default>2yrs, Ind-Default>2yrs, Agric-Default>2yrs: 0.6 Res-Default-HTR-di, Com-Default-HTR-di, Ind-Default-HTR-di, Agricult-Default-HTR-di: 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 3/30/2012 | Ajay Wadhera/SCE | Updated Work paper to new template 2013 v0.1 from original WPSCNRLG0108.0 for the 2013-2014 bridge period |
|  |  | Brian V. O’Keefe/SCE | -Added ROB for downstream deemed, included cost section update -Updated the operating hours for the residential measures to reflector exterior res hours. |
| 1 | 8/1/2013 | Jason Wang/SCE | - Updated measure ranges per 3/1/13 ED Lighting Retrofit Disposition titled “2013-2014_LightingRetrofit_Disposition-1March2013.docx.” - Added Title 24 2013 code, which added a lighting control requirement. |
| 2 | 4/17/2014 | Jason Wang/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Adjusted wattages based on May 30, 2014 ED disposition -Updated costs based on ED disposition |
| 3 | 09/22/2015 | Yun Han/SCE | - Added new measures for Direct Install Program - Included program implementation language - Added Hard-to-Reach language for HTR NTG - Updated with Calculation Template v6.0 - Updated with latest DEER (READi v2.3.0) measure impacts |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |
| 0 | CS |  |  |  |  |
| 0 | Cal TF |  |  |  |  |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of Metal Halide (MH), Pulse Start Metal Halide (PSMH), Compact Fluorescent (CFL), and High Pressure Sodium (HPS) exterior wall packs with LED exterior wall packs. The measures are broken out into the wattage ranges as shown in the following tables.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Wall Pack LED |
| Existing Condition | Wall Pack HID – MH; PSMH; CFL; HPS |
| Code/Standard | N/A |
| Industry Standard Practice | PSMH |

Lists of measures indicated in table below are used in the Down-Stream Deemed and Direct Install Programs.

Measures and Codes - 1

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-79024 |  | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt Metal Halide |
|  |  | LT-29069 |  | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide |
|  |  | LT-38903 |  | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS |
|  |  | LT-17893 |  | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS |
|  |  | LT-37685 |  | 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL |

List of measures indicated in following table (Measures and Codes – 2) are used in the Multifamily Energy Efficiency Rebate’s (MFEER) Direct Install and Commercial Direct Install Programs. Pulse Start & Metal Halide measures are for both Early Retirement (ER) and Replace on Burnout (ROB).

The MFEER program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is an Excel spreadsheet with a tab for measures installed in dwelling areas and another for measures installed in common area locations.  The PFL contains the following fields: Apartment Address, Measure Installed, Product Make/Model, Install Locations (detailed to Kitchen, Bathroom, Hallway, Living Room, Bedroom, Dining Room, Laundry Room, and Porch/Patio), as well as Common Area locations.  This information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

The Non Residential Direct Install program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is a form which information for measures installed in all building types related to the Non Residential Direct Install program.  The PLF contains the following fields: Service Account Address, Measures Proposed/Installed, Product Make/Model, Install Locations (detailed to define separate spaces/floors, as well as specific locations within the space including but not limited to:  Bathrooms, Hallways, Meeting Rooms, Offices, Warehouse, etc.).

Customers are solicited to participate primarily through field visits.  Contractors conduct energy consultation, provide recommendation that can help eligible customers use less energy.  If the customer agrees, the Direct Install contractor will help them complete an authorization form and schedule an installation appointment.

SCE can also provide photos to show measure functionality and a sample close up photos to substantiate the measure base case where applicable.  These photos would be part of the required project package.  The project package is identified with the Service Account Number and attached to SCE’s SMART database at each specific project level. The above described information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

Measures and Codes - 2

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-60772 |  | 13-30 Watt Wall Pack LED replacing 70 Watt Metal Halide |
|  |  | LT-92080 |  | 31-50 Watt Wall Pack LED replacing 100 Watt Metal Halide |
|  |  | LT-66647 |  | 51-90 Watt Wall Pack LED replacing 150/175 Watt Metal Halide |
|  |  | LT-74456 |  | 91-115 Watt Wall Pack LED replacing 250 Watt Metal Halide |
|  |  | LT-64309 |  | 13-30 Watt Wall Pack LED replacing 70 Watt Pulse Start Metal Halide |
|  |  | LT-94750 |  | 31-50 Watt Wall Pack LED replacing 100 Watt Pulse Start Metal Halide |
|  |  | LT-53425 |  | 51-90 Watt Wall Pack LED replacing 150 Watt Pulse Start Metal Halide |
|  |  | LT-53959 |  | 91-115 Watt Wall Pack LED replacing 175 Watt Pulse Start Metal Halide |

Products must be listed on Design Lights Consortium’s (DLC) Qualified Products List (QPL) [486] under the category “Outdoor Wall-mounted Area Luminaires”.

The measures in this work paper applies to all building types in all SCE Climate Zones.

## 1.2 Technical Description

Wall packs are exterior lighting fixtures that are commonly used in walkways, parking areas, building facades, garages, and other outdoor spaces. They have photocell control and operate from dusk to dawn.

## Installation Types and Delivery Mechanisms

The Installation Types applied to these measures are Early Retirement (ER) or Retrofit (RET) and Replace on Burnout (ROB).

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.  Delivery mechanism applied to these measures are the following:

- Financial Support/Partnership – Down-Stream Incentive - Deemed

- Financial Support/Partnership – Direct Install

- Midstream Programs – Mid-stream Incentive

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

This workpaper includes two set of measures: Wall Pack LED lighting systems replacing HIDs with impacts that were directly taken from DEER/READI and Wall Pack LED lighting systems replacing specific Metal Halide (PSMH and MH) which are not included in DEER

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | DEER does not contain all the PSMH & MH wattages in READI. |
| DEER Measure IDs Used | C-Out-LEDFixt-WM-Ext-4(40w)-dWP32 |

Measures not in READI were created using “Create New Scaled Measure” in READI. The measures are exported and attached.

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.6 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | DirInstall | 0.85 |
| Ind-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | DirInstall | 0.85 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer homes.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer homes as HTR. The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria.

SCE’s Multi-Family Energy Efficiency Rebate (MFEER) program addresses the ongoing concern with “split incentives”, where the residents are not the owners of the property, so they lack incentive to improve their energy usage. Similarly, the property owners do not live on-site and pay higher utility expenses due to inefficient appliances, thus lack any incentive to upgrade. The MFEER is designed to drive this customer segment toward participation by offering property owners a variety of energy efficiency measures and services.  The MFEER program will offer and track measure installations in both common and dwelling areas of qualifying multifamily properties.  Measures implemented via direct install in both common and dwelling areas of qualifying multifamily properties will receive the HTR NTG.  Other measures in the MFEER program will receive default NTG (NTGR_ID: Res-Default>2), unless otherwise specified in DEER.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

o	Business Size – Customer must have less than ten employees

o	Language – Customer’s primary language spoken is not English

o	Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.”

The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties.

The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| OLtg-Com-LED-50000hr | LED Fixture - Outdoor- Commercial | Com | Lighting | 12 | For CFLs, the Base Case EUL is 12 (Non-res) or 16 (Res) years/3. The RUL is 4 (Non-res) or 5.3 (Res) years. For MH and HPS, Base Case EUL is maximum of 15 years and (70,000/4,100)/3. The RUL is 5.7 years. |
| OLtg-Res-LED-50000hr | LED Fixture - Outdoor- Residential | Res | Lighting | 16 |  |

### 1.4.2 Codes and Standards Analysis

Title 20 2014 [422]: California Energy Commission (CEC) Title 20 defines that a “LED luminaire” is a complete lighting unit consisting of an LED light source, meaning one or more LED lamps or LED light engines, together with parts to distribute light, to position and protect the light source, and to connect the light source to the electrical power. Title 20 does not provide efficiency requirements for the LED fixtures in this work paper.

Title 24 2013 [355]: Title 24 Section 130.2(c)5D states that outdoor wall mounted luminaires having a bilaterally symmetric distribution as described in the IES Handbook (typically referred to as “wall packs”) where the bottom of the lumnaire is mounted 24 feet or less above the ground shall comply with the applicable requirements in Section 130.2(c)3.

Section 130.2(c)3 states that:

All installed outdoor lighting, where the bottom of the luminaire is mounted 24 feet or less above the ground, shall be controlled with automatic lighting controls that meet all of the following requirements:

- Shall be motion sensors or other lighting control systems that automatically controls lighting in accordance with item B in response to the area being vacated of occupants.; and

- Shall be capable of automatically reducing the lighting power of each luminaire by at least 40 percent but not exceeding 80 percent, or provide continuous dimming through a range that includes 40 percent through 80 percent, and

- Shall employ auto-ON functionality when the area becomes occupied, and

- No more than 1,500 watts of lighting power shall be controlled together.

EXCEPTION 1 to Section 130.2(c)3: Lighting for Outdoor Sales Frontage, Outdoor Sales Lots, and Outdoor Sales Canopies complying with Section 130.2(c)4.

EXCEPTION 2 to Section 130.2(c)3: Lighting for Building Facades, Ornamental Hardscape and Outdoor Dining complying with Section 130.2(c)5.

EXCEPTION 3 to Section 130.2(c)3: Outdoor lighting, where luminaire rated wattage is determined in accordance with Section 130.0(c), and which meet one of the following conditions:

- Pole-mounted luminaires with a maximum rated wattage of 75 watts each; or

- Non-pole mounted luminaires each with a maximum rated wattage of 30 watts each; or

- Linear lighting with a maximum wattage of 4 watts per linear foot of luminaire.

EXCEPTION 4 to Section 130.2(c)3: Applications listed as Exceptions to Section 140.7(a) shall not be required to meet the requirements of Section 130.2(c)3.

Section 140.7 contains lighting power requirements that apply to all outdoor lighting.

Section 149 contains requirements related to additions, alterations, and repairs to existing buildings that will be nonresidential, high-rise residential, and hotel/motel occupancies and to existing outdoor lighting for these occupancies and to internally and externally illuminated signs.

Lighting controls will be required for this measure unless the application meets the exception criteria in Title 24 Section 130.2(c)3 or Section 140.7(a).  Following table shows the code analysis summary for these measure types.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Sections 130.2(c), 140.7, 149 | July 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

No relevant studies were reviewed for this work paper.

## 1.6 Data Quality and Future Data Needs

Workpaper cost to be re-evaluated and potentially update in new version update.

# Section 2. Calculation Methodology

The measure, base, and code case wattages for the measures in table below are from the May 30 Energy Division disposition [Attachment 2].  These measures impacts as indicated in table below are available in DEER2016/READI for both Commercial and Residential sectors.  Workpaper impacts on these measures were taken directly from DEER/READI.

Measure, Base, and Code Case Wattages

| DEER Measure ID* | Measure Name | Measure Case Wattage (W) | Base Case Wattage (W) | Code Case Wattage (W) |
| --- | --- | --- | --- | --- |
| C-Out-LEDFixt-WM-Ext-4(40w)-dWP32 | 20 to 40 Watt Wall Pack LED replacing up to 175 Metal Halide | 40 | 72 | 72 |
| C-Out-LEDFixt-WM-Ext(80w)-dWP215-dWC152 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide | 80 | 295 | 232 |
| C-Out-LEDFixt-WM-Ext-4(40w)-dWP6-2 | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS | 40 | 46 | 46 |
| C-Out-LEDFixt-WM-Ext(80w)-dWP215 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS | 80 | 295 | 295 |
| C-Out-LEDFixt-WM-Ext-4(40w)-dWP6-1 | 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL | 40 | 46 | 46 |

*All of these Measure IDs also have a residential version, e.g. “R-Out-LEDFixt-WM-Ext-4(40w)-dWP32.”

This workpaper includes Wall Pack LED lighting systems replacing HIDs with impacts that were directly taken from DEER/READI.  The following table indicates which measures are taken directly from or created with the DEER READI tool.

READI Data Used

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| LT-79024, LT-29069, LT-38903, LT-17893, LT-37685 | See Attachment 1 | See Attachment 1 |

Direct Install Measures

The measure, base, and code case wattages for the measures in following table are for Direct Install Programs only.  The base (MH) and code (PSMH) wattages are based on lumen equivalency [Attachment 3]. The lumen values used were from a lighting catalog which also factored in the fixture efficiency of the wall-pack fixture provided by an IES file [Attachment 4].

The measure wattages also based on lumen equivalency are derived from a list of products qualified on Design Lights Consortium. The lumens of the measures were mapped to the lumens of the base case which resulted in a range of measure wattages. To be conservative, the measure wattages use the highest value from its lumen range [Attachment 3].

Measure, Base, and Code Case Wattages for Direct Install

| Solution Code | Measure name | Measure Case Wattage (W) | Base Case Wattage (W) | Code Case Wattage (W) |
| --- | --- | --- | --- | --- |
| LT-60772 | LED Wall Pack up to 30 Watts replacing 70 Watt Metal Halide | 30 | 95 | 95 |
| LT-92080 | LED Wall Pack up to 50 Watts replacing 100 Watt Metal Halide | 50 | 128 | 128 |
| LT-66647 | LED Wall Pack up to 90 Watts replacing 150/175 Watt Metal Halide | 90 | 215 | 190 |
| LT-74456 | LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide | 115 | 295 | 208 |
| LT-64309 | LED Wall Pack up to 30 Watts replacing 70 Watt Pulse Start Metal Halide/Metal Halide | 30 | 95 | N/A |
| LT-94750 | LED Wall Pack up to 50 Watts replacing 100 Watt Pulse Start Watt Metal Halide/Metal Halide | 50 | 128 | N/A |
| LT-53425 | LED Wall Pack up to 90 Watts replacing 150 Watt Pulse Start Watt Metal Halide | 90 | 190 | N/A |
| LT-53959 | LED Wall Pack up to 115 Watts replacing 175 Watt Pulse Start Watt Metal Halide | 115 | 208 | N/A |

Retrofit 1st Baseline Energy Savings

Example below shows an LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide:

Energy Savings kWhyr= 295-115x 4,1001000 W/kW

Energy Savings kWhyr= 738 kWh/year

Retrofit 1st Baseline Demand Reduction

DEER 2016 provides a peak coincidence factor for exterior lighting.

DemandReductionkWUnit=295×.226-(115×.221)1,000WatthourskWh

DemandReductionkWUnit=0.04126

Retrofit 2nd Baseline Energy Savings & Replace on Burnout 1st Baseline Energy Savings

Example below shows an LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide. The code baseline is a 175 Watt Pulse Start Metal Halide with a connected 208 Watts. Triggering code will require each luminaire to reduce lighting power by at least 40% when the space is not occupied (15% occupancy factor used). Fixtures under 30 Watts are not affected by code.

Energy Savings kWhyr= 208-115x 4,100-[208-115x4100x0.6x.15]1000 W/kW

Energy Savings kWhyr= 347 kWh/year

Retrofit 2nd Baseline Demand Reduction & Replace on Burnout 1st Baseline Demand Reduction

DemandReductionkWUnit=(208-115)× .221-((208-115)×.221× .6× .15)1,000WatthourskWh

DemandReductionkWUnit=0.01870

Full list of savings can be found in the attachment [Attachment 1].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | Outdoor Lt | Misc._Commercial |
| Education - Primary School | Outdoor Lt | Misc._Commercial |
| Education - Secondary School | Outdoor Lt | Misc._Commercial |
| Education - Relocatable Classroom | Outdoor Lt | Misc._Commercial |
| Education - Community College | Outdoor Lt | Misc._Commercial |
| Education - University | Outdoor Lt | Misc._Commercial |
| Grocery | Outdoor Lt | Misc._Commercial |
| Health/Medical - Hospital | Outdoor Lt | Misc._Commercial |
| Health/Medical - Nursing Home | Outdoor Lt | Misc._Commercial |
| Lodging - Hotel | Outdoor Lt | Misc._Commercial |
| Lodging - Guest Rooms | Outdoor Lt | Misc._Commercial |
| Lodging - Motel | Outdoor Lt | Misc._Commercial |
| Manufacturing - Bio/Tech | Outdoor Lt | Misc._Commercial |
| Manufacturing - Light Industrial | Outdoor Lt | Misc._Commercial |
| Office - Large | Outdoor Lt | Misc._Commercial |
| Office - Small | Outdoor Lt | Misc._Commercial |
| Restaurant - Fast-Food | Outdoor Lt | Misc._Commercial |
| Restaurant - Sit-Down | Outdoor Lt | Misc._Commercial |
| Retail - Multistory Large | Outdoor Lt | Misc._Commercial |
| Retail - Single-Story Large | Outdoor Lt | Misc._Commercial |
| Retail - Small | Outdoor Lt | Misc._Commercial |
| Storage - Conditioned | Outdoor Lt | Misc._Commercial |
| Storage - Unconditioned | Outdoor Lt | Misc._Commercial |
| Warehouse - Refrigerated | Outdoor Lt | Misc._Commercial |
| Residential Single Family | Outdoor Lt | Misc._Commercial |
| Residential Multi-family | Outdoor Lt | Misc._Commercial |
| Residential Mobile Home - Double-Wide | Outdoor Lt | Misc._Commercial |

# Section 4. Costs

## 4.1 Base Case Cost

The base costs were taken from online sources [Attachment 3]. The labor cost was taken from RSMeans [358] 26-56-13-1100.

Base Case Cost

| Solution Code | Measure Name | Base Cost | Labor Cost | Total Base |
| --- | --- | --- | --- | --- |
| LT-79024 | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt Metal Halide | $124.93 | $98.00 | $222.93 |
| LT-29069 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide | $160.74 | $98.00 | $258.74 |
| LT-38903 | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS | $124.93 | $98.00 | $222.93 |
| LT-17893 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS | $160.74 | $98.00 | $258.74 |
| LT-37685 | 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL | $86.15 | $98.00 | $184.15 |
| LT-60772 | LED Wall Pack up to 30 Watts replacing 70 Watt Metal Halide | $124.93 | $98.00 | $222.93 |
| LT-92080 | LED Wall Pack up to 50 Watts replacing 100 Watt Metal Halide | $160.74 | $98.00 | $258.74 |
| LT-66647 | LED Wall Pack up to 90 Watts replacing 150/175 Watt Metal Halide | $164.44 | $98.00 | $262.44 |
| LT-74456 | LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide | $171.05 | $98.00 | $269.05 |
| LT-64309 | LED Wall Pack up to 30 Watts replacing 70 Watt Pulse Start Metal Halide | $124.93 | $98.00 | $222.93 |
| LT-94750 | LED Wall Pack up to 50 Watts replacing 100 Watt Pulse Start Watt Metal Halide | $160.74 | $98.00 | $258.74 |
| LT-53425 | LED Wall Pack up to 90 Watts replacing 150 Watt Pulse Start Watt Metal Halide | $164.44 | $98.00 | $262.44 |
| LT-53959 | LED Wall Pack up to 115 Watts replacing 175 Watt Pulse Start Watt Metal Halide | $171.05 | $98.00 | $269.05 |

## 4.2 Measure Case Cost

Measure costs were provided by Navigant as part of a study on LEDs. The California LED Workpaper Update Study [A, Attachment 5] recommends using 25 percentile utilizing CA specific data. The labor cost was taken from RSMeans [358] 26-56-13-1100.

Measure Case Cost

| Solution Code | Measure Name | Measure Cost | Labor Cost | Total Measure |
| --- | --- | --- | --- | --- |
| LT-79024 | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt Metal Halide | $176.34 | $98.00 | $274.34 |
| LT-29069 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide | $325.28 | $98.00 | $423.28 |
| LT-38903 | 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS | $176.34 | $98.00 | $274.34 |
| LT-17893 | 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS | $325.28 | $98.00 | $423.28 |
| LT-37685 | 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL | $176.34 | $98.00 | $274.34 |
| LT-60772 | LED Wall Pack up to 30 Watts replacing 70 Watt Metal Halide | $155.90 | $98.00 | $253.90 |
| LT-92080 | LED Wall Pack up to 50 Watts replacing 100 Watt Metal Halide | $242.32 | $98.00 | $340.32 |
| LT-66647 | LED Wall Pack up to 90 Watts replacing 150/175 Watt Metal Halide | $357.36 | $98.00 | $455.36 |
| LT-74456 | LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide | $661.71 | $98.00 | $759.71 |
| LT-64309 | LED Wall Pack up to 30 Watts replacing 70 Watt Pulse Start Metal Halide | $155.90 | $98.00 | $253.90 |
| LT-94750 | LED Wall Pack up to 50 Watts replacing 100 Watt Pulse Start Watt Metal Halide | $242.32 | $98.00 | $340.32 |
| LT-53425 | LED Wall Pack up to 90 Watts replacing 150 Watt Pulse Start Watt Metal Halide | $357.36 | $98.00 | $455.36 |
| LT-53959 | LED Wall Pack up to 115 Watts replacing 175 Watt Pulse Start Watt Metal Halide | $661.71 | $98.00 | $759.71 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| 20 to 40 Watt Wall Pack LED replacing up to 175 Watt Metal Halide | RET | $51.41 | $274.34 | $51.41 |
| 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide | RET | $164.54 | $423.28 | $164.54 |
| 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS | RET | $51.41 | $274.34 | $51.41 |
| 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS | RET | $164.54 | $423.28 | $164.54 |
| 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL | RET | $90.19 | $274.34 | $90.19 |
| LED Wall Pack up to 30 Watts replacing 70 Watt Metal Halide | RET | $30.97 | $253.90 | $30.97 |
| LED Wall Pack up to 50 Watts replacing 100 Watt Metal Halide | RET | $81.58 | $340.32 | $81.58 |
| LED Wall Pack up to 90 Watts replacing 150/175 Watt Metal Halide | RET | $192.92 | $455.36 | $192.92 |
| LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide | RET | $490.66 | $759.71 | $490.66 |
| LED Wall Pack up to 30 Watts replacing 70 Watt Pulse Start Metal Halide | RET | $30.97 | $253.90 | $30.97 |
| LED Wall Pack up to 50 Watts replacing 100 Watt Pulse Start Watt Metal Halide | RET | $81.58 | $340.32 | $81.58 |
| LED Wall Pack up to 90 Watts replacing 150 Watt Pulse Start Watt Metal Halide | RET | $192.92 | $455.36 | $192.92 |
| LED Wall Pack up to 115 Watts replacing 175 Watt Pulse Start Watt Metal Halide | RET | $490.66 | $759.71 | $490.66 |
| 20 to 40 Watt Wall Pack LED replacing up to 175 Watt Metal Halide | ROB | $51.41 | $51.41 | N/A |
| 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt Metal Halide | ROB | $164.54 | $164.54 | N/A |
| 20 to 40 Watt Wall Pack LED replacing up to 175 Watt HPS | ROB | $51.41 | $51.41 | N/A |
| 41 to 80 Watt Wall Pack LED replacing 176 to 250 Watt HPS | ROB | $164.54 | $164.54 | N/A |
| 20 to 40 Watt Wall Pack LED replacing up to 126 Watt CFL | ROB | $90.19 | $90.19 | N/A |
| LED Wall Pack up to 30 Watts replacing 70 Watt Metal Halide | ROB | $30.97 | $30.97 | N/A |
| LED Wall Pack up to 50 Watts replacing 100 Watt Metal Halide | ROB | $81.58 | $81.58 | N/A |
| LED Wall Pack up to 90 Watts replacing 150/175 Watt Metal Halide | ROB | $192.92 | $192.92 | N/A |
| LED Wall Pack up to 115 Watts replacing 250 Watt Metal Halide | ROB | $490.66 | $490.66 | N/A |
| LED Wall Pack up to 30 Watts replacing 70 Watt Pulse Start Metal Halide | ROB | $30.97 | $30.97 | N/A |
| LED Wall Pack up to 50 Watts replacing 100 Watt Pulse Start Watt Metal Halide | ROB | $81.58 | $81.58 | N/A |
| LED Wall Pack up to 90 Watts replacing 150 Watt Pulse Start Watt Metal Halide | ROB | $192.92 | $192.92 | N/A |
| LED Wall Pack up to 115 Watts replacing 175 Watt Pulse Start Watt Metal Halide | ROB | $490.66 | $490.66 | N/A |

# Attachments

- Consolidated

READI Residential

READI Commercial

NewMeasuresResidential

NewMeasuresCommercial

- 3. 4.5.

# References

[355]

[358]

[422]

[486]

[A] http://www.energydataweb.com/cpucFiles/pdaDocs/1359/LED%20Study_Draft%20Final%20Report_20150828_Clean.pdf