Work Paper SCE13LG109

Revision 1

Southern California Edison

Exterior LED Lamp Replacement

# At-a-Glance Summary

| Measure Codes | LT-37176, LT-64932, LT-60894, LT-59328, LT-89305, LT-59089, LT-98579, LT-89203, LT-98182, LT-21786, LT-50213, LT-49986, LT-49807, LT-35653, LT-89912, LT-70912 |
| --- | --- |
| Measure Description | Outdoor LED lamps: A-lamp, MR16, PAR30, PAR38 |
| Base Case Description | LED Base Case Total Watts = WRR x Msr Watts Typical base case lamp types A-lamp: Incandescent or CFL MR16, PAR30, PAR38: Halogen |
| Units | Lamp |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | OLtg-Res-LED-20000hr: 16 years OLtg-Res-LED-20000hr-Cmn: 5.9 years |
| Measure Installation Type | Replace on Burnout (ROB) |
| Net-to-Gross Ratio | Res-sAll-MLtgLED-Deemed: 0.6 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 5/25/2012 | Alvaro Mendoza (SCE) | - Updated from original work paper WPSCNRLG0109.0 to latest work paper template v0.1 - Used DEER recommended wattage ratios, EUL, and NTG |
|  |  | Brian V. O’Keefe (SCE) | - Added Double-Wide Mobile Home - Updated the exterior res operating hours. - Replaced the calculation attachment |
| 1 | 10/01/2015 | Jason Wang (SCE) | - Updated using DEER2016 HOU, CDF - Removed non-Multifamily measures, based on historical measure usage. All non-residential building types are removed. - Revised measure wattage values using the Energy Star Certified Lamps list. |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Outdoor LED lamps: A-lamp, MR16, PAR30, PAR38 |
| Existing Condition | N/A |
| Code/Standard | LED Base Case Total Watts = WRR x Msr Watts Typical base case lamp types A-lamp: Incandescent or CFL MR16, PAR30, PAR38: Halogen |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-37176 |  | Up to 10 Watt Exterior A-Lamp (Common Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts |
|  |  | LT-64932 |  | Up to 10 Watt Exterior A-Lamp (Dwelling Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts |
|  |  | LT-60894 |  | Up to 30 watts Exterior A-Lamp (Common Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts |
|  |  | LT-59328 |  | Up to 30 watts Exterior A-Lamp (Dwelling Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts |
|  |  | LT-89305 |  | Up to 6 Watt Exterior MR16 (Common Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts |
|  |  | LT-59089 |  | Up to 6 Watt Exterior MR16 (Dwelling Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts |
|  |  | LT-98579 |  | Up to 10 Watt Exterior MR16 (Common Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts |
|  |  | LT-89203 |  | Up to 10 Watt Exterior MR16 (Dwelling Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts |
|  |  | LT-98182 |  | Up to 15 Watt Exterior PAR30 (Common Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts |
|  |  | LT-21786 |  | Up to 15 Watt Exterior PAR30 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts |
|  |  | LT-50213 |  | Up to 21 Watt Exterior PAR30 (Common Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts |
|  |  | LT-49986 |  | Up to 21 Watt Exterior PAR30 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts |
|  |  | LT-49807 |  | Up to 17 Watt Exterior PAR38 (Common Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts |
|  |  | LT-35653 |  | Up to 17 Watt Exterior PAR38 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts |
|  |  | LT-89912 |  | Up to 25 Watt Exterior PAR38 (Common Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts |
|  |  | LT-70912 |  | Up to 25 Watt Exterior PAR38 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts |

Eligibility Requirements

- Eligible products must be UL certified, market-available, and rated for outdoor use.

- Eligible lamps shall generate equal or greater luminous output at lower power consumption than their base case counterparts or provide equal or greater center beam candle power for directional lamps to achieve energy savings.

- Eligible lamps must be on the Energy Star Qualified LED Lamp List and feature the DOE’s LED Lighting Facts label.

## 1.2 Technical Description

A light-emitting diode (LED) is a semiconductor device that produces visible light when an electrical current is passed through it. A heat sink absorbs the heat produced by the process. LEDs are directional light sources, and their lamp type is used to shape the light output. LEDs can be more efficient than incandescent (including halogen) and CFL lighting.

## 1.3 Installation Types and Delivery Mechanisms

These measures are offered through the Multifamily Energy Efficiency Rebate Progam.

The install type is ROB.

The delivery mechanism is Financial Support: Down-Stream Incentive – Deemed.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| New Construction (NEW/NC) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |
| Retrofit First Baseline Only (REF) | Above Customer Existing | N/A | EUL | N/A |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Appliance Turn-in and Recycling | The program motivates customers, through financial incentives, to recycle appliances that are functional but inefficient. This prevents the continued use of those appliances, by both the current owner and potential future owners. |
| Audit - Information - Testing Services | The program performs a free assessment of a customer’s facility and provides the customer with information and guidance on energy efficiency opportunities. |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |
| Up-Stream Programs | See Up-Stream Incentive in the Incentive Method Descriptions table. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The program provides customers with energy efficiency equipment or services for free. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |
| On-bill Finance – Loan (OBF) | The program offers financing for the cost of an efficient measure as part of the utility bill. This can be an add-on option to an existing program or can serve as an organizing principle for its own program. |
| Up-Stream Incentive Up-Stream Buy Down | The program gives a financial incentive to an upstream market actor (manufacturer or distributor) to encourage the manufacture, provision, or distribution of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2016, READI v2.3.0 |
| Reason for Deviation from DEER | DEER does not have measures using the wattages in this work paper. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-sAll-MLtgLED-Deemed | Residential LED: replacing CFL or incandescent lamps; deemed; all delivery mechanisms except upstream | Res | Any | DeemedNonUpstream | 0.6 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| OLtg-Res-LED-20000hr | LED lamp - Outdoor- Residential | Res | Lighting | 20,000 hours / 935 hours/year = 21.4, capped at 16 years | N/A |
| OLtg-Res-LED-20000hr-Cmn | LED lamp - Outdoor- Residential Common Area | Res | Lighting | 20,000 hours / 3,390 hours/year = 5.9 years | N/A |

### 1.4.2 Codes and Standards Analysis

Title 20: California Energy Commission (CEC) Title 20 [493] defines that a “LED luminaire” is a complete lighting unit consisting of an LED light source, meaning one or more LED lamps or LED light engines, together with parts to distribute light, to position and protect the light source, and to connect the light source to the electrical power.

It further specified that the test methods for LED luminaires using LED lamps and light engines are California Joint Appendix JA8 – 2008, “Testing of Light Emitting Diode Light Sources,” or IES LM79-08, “Approved Method: Electrical and Photometric Measurements of Solid-State Lighting Products,” at manufacturer’s option. Title 20, Table N-3, also outlines the Minimum Requirements for Portable LED Luminaires, and Portable Luminaires with LED Light Engines with Integral Heat Sink.

This work paper uses recommended wattage ratios from the Energy Division’s Workpaper Disposition for Integral LED Lamp Replacements (May 14, 2012) (Attachment 3) to calculate the base case wattage.  Therefore, code requirements are not used in the energy savings calculation.

Title 24: These measures do not fall under Title 24 of the California Energy Regulations [355].

Energy Star: The ENERGY STAR Program Requirements for Lamps – Partner Commitments Version 1.1 (Attachment 4) details the performance requirements for LED Lamps. Eligible LED lamps must meet these requirements.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Table N-3 | July 1, 2015 |
| Energy Star | Program Requirements for Lamps v1.1 | August 28, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

No studies were references in the development of this work paper.

## 1.6 Data Quality and Future Data Needs

Since the energy savings are based on DEER methodology, no future data needs are anticipated.

# Section 2. Calculation Methodology

The measures in this work paper are defined by wattage ranges, so the Energy Star Certified Light Bulbs list (Attachment 2) was used to determine an average measure wattage value within each range that would be used for energy savings calculations. The following tables shows the average measure wattages, the associated wattage reduction ratio (WRR) for each lamp type, and the base wattage (measure watts * WRR).

| Solution Code | Measure Name | Average Measure Watts | WRR | Base Watts |
| --- | --- | --- | --- | --- |
| LT-37176 | Up to 10 Watt Exterior A-Lamp (Common Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts | 8 | 2.96 | 23.68 |
| LT-64932 | Up to 10 Watt Exterior A-Lamp (Dwelling Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts | 8 | 2.96 | 23.68 |
| LT-60894 | Up to 30 watts Exterior A-Lamp (Common Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts | 12 | 2.96 | 35.52 |
| LT-59328 | Up to 30 watts Exterior A-Lamp (Dwelling Area) LED replacing LED Base Case Total Watts = 2.96 x Msr Watts | 12 | 2.96 | 35.52 |
| LT-89305 | Up to 6 Watt Exterior MR16 (Common Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts | 6 | 4.24 | 25.44 |
| LT-59089 | Up to 6 Watt Exterior MR16 (Dwelling Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts | 6 | 4.24 | 25.44 |
| LT-98579 | Up to 10 Watt Exterior MR16 (Common Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts | 7 | 4.24 | 29.68 |
| LT-89203 | Up to 10 Watt Exterior MR16 (Dwelling Area) LED replacing LED Base Case Total Watts = 4.24 x Msr Watts | 7 | 4.24 | 29.68 |
| LT-98182 | Up to 15 Watt Exterior PAR30 (Common Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts | 12 | 3.42 | 41.04 |
| LT-21786 | Up to 15 Watt Exterior PAR30 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts | 12 | 3.42 | 41.04 |
| LT-50213 | Up to 21 Watt Exterior PAR30 (Common Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts | 16 | 3.42 | 54.72 |
| LT-49986 | Up to 21 Watt Exterior PAR30 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts | 16 | 3.42 | 54.72 |
| LT-49807 | Up to 17 Watt Exterior PAR38 (Common Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts | 16 | 3.81 | 60.96 |
| LT-35653 | Up to 17 Watt Exterior PAR38 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts | 16 | 3.81 | 60.96 |
| LT-89912 | Up to 25 Watt Exterior PAR38 (Common Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts | 19 | 3.81 | 72.39 |
| LT-70912 | Up to 25 Watt Exterior PAR38 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.81 x Msr Watts | 19 | 3.81 | 72.39 |

The Wattage Reduction Ratios are from the Energy Division’s Workpaper Disposition for Integral LED Lamp Replacements (May 14, 2012) (Attachment 3):

| Lamp Type | WRR |
| --- | --- |
| A19 | 2.96 |
| MR16 | 4.24 |
| PAR20 | 4.7 |
| PAR30 | 3.42 |
| PAR38 | 3.81 |

Energy Savings

The following parameters were used to calculate energy savings:

| Area type | Annual hours of operation (HOU) | Coincident demand factor (CDF) | kWh and therm interactive effects (IEs) |
| --- | --- | --- | --- |
| Dwelling Area | 935 | 0.021 | 1 (no interactive effects for outdoors) |
| Common Area | 3,390 | 0.021 | 1 (no interactive effects for outdoors) |

The energy savings calculation equation is:

EnergySavingskWh=Base kW-Measure kW*HOU*(kWh IE)

The demand reduction calculation equation is:

DemandReductionkW=Base kW-Measure kW*CDF*(kW IE)

Example

LT-21786: Up to 15 Watt Exterior PAR30 (Dwelling Area) LED replacing LED Base Case Total Watts = 3.42 x Msr Watts, Multifamily building type, climate zone 6

EnergySavingskWh=0.012*3.42-0.012 kW*935*1=27.15 kWh

DemandReductionkW=0.012*3.42-0.012kW*0.021*1=0.00061 kW

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Mobile Home - Double-Wide | Outdoor Lt | Misc._Commercial |
| Residential Multi-family | Outdoor Lt | Misc._Commercial |

# Section 4. Costs

Most costs for LED lamps were provided by Navigant as part of a study on LEDs. Several were interpolated or extrapolated from the Navigant data. The California LED Workpaper Update Study [A] recommends using 25 percentile utilizing CA specific data.

The base case costs are split into 40% CFL and 60% incandescent. CFL costs are taken from READI. Costs not available from READI have been interpolated. Incandescent costs are calculated from WO17 workbook (Attachment 6). The base case wattages are mapped to individual LED wattages using a table from the Energy Star Calculator.

All cost data can be found in the Attachment 5.

## 4.1 Base Case Cost

Table below shows a sample of base costs in this WP. A full list of savings in different climate zones can be found in the attachment (Attachment 1).

Base Case Cost

| Solution Code | Measure Name | Equipment Cost | Labor Cost |
| --- | --- | --- | --- |
| LT-37176 | Up to 10 Watt Exterior A-Lamp (Common Area) LED | $3.06 | $5.75 |
| LT-64932 | Up to 10 Watt Exterior A-Lamp (Dwelling Area) LED | $3.06 | $5.75 |
| LT-60894 | Up to 30 watts Exterior A-Lamp (Common Area) LED | $3.28 | $5.75 |
| LT-59328 | Up to 30 watts Exterior A-Lamp (Dwelling Area) LED | $3.28 | $5.75 |
| LT-89305 | Up to 6 Watt Exterior MR16 (Common Area) LED | $7.44 | $5.75 |
| LT-59089 | Up to 6 Watt Exterior MR16 (Dwelling Area) LED | $7.44 | $5.75 |
| LT-98579 | Up to 10 Watt Exterior MR16 (Common Area) LED | $7.50 | $5.75 |
| LT-89203 | Up to 10 Watt Exterior MR16 (Dwelling Area) LED | $7.50 | $5.75 |
| LT-98182 | Up to 15 Watt Exterior PAR30 (Common Area) LED | $7.89 | $5.75 |
| LT-21786 | Up to 15 Watt Exterior PAR30 (Dwelling Area) LED | $7.89 | $5.75 |
| LT-50213 | Up to 21 Watt Exterior PAR30 (Common Area) LED | $8.28 | $5.75 |
| LT-49986 | Up to 21 Watt Exterior PAR30 (Dwelling Area) LED | $8.28 | $5.75 |
| LT-49807 | Up to 17 Watt Exterior PAR38 (Common Area) LED | $8.28 | $5.75 |
| LT-35653 | Up to 17 Watt Exterior PAR38 (Dwelling Area) LED | $8.28 | $5.75 |
| LT-89912 | Up to 25 Watt Exterior PAR38 (Common Area) LED | $8.46 | $5.75 |
| LT-70912 | Up to 25 Watt Exterior PAR38 (Dwelling Area) LED | $8.46 | $5.75 |

## 4.2 Measure Case Cost

Table below shows a sample of measure costs in this WP. A full list of savings in different climate zones can be found in the attachment (Attachment 1).

Measure Case Cost

| Solution Code | Measure Name | Equipment Cost | Labor Cost |
| --- | --- | --- | --- |
| LT-37176 | Up to 10 Watt Exterior A-Lamp (Common Area) LED | $10.31 | $5.75 |
| LT-64932 | Up to 10 Watt Exterior A-Lamp (Dwelling Area) LED | $10.31 | $5.75 |
| LT-60894 | Up to 30 watts Exterior A-Lamp (Common Area) LED | $12.48 | $5.75 |
| LT-59328 | Up to 30 watts Exterior A-Lamp (Dwelling Area) LED | $12.48 | $5.75 |
| LT-89305 | Up to 6 Watt Exterior MR16 (Common Area) LED | $9.86 | $5.75 |
| LT-59089 | Up to 6 Watt Exterior MR16 (Dwelling Area) LED | $9.86 | $5.75 |
| LT-98579 | Up to 10 Watt Exterior MR16 (Common Area) LED | $16.50 | $5.75 |
| LT-89203 | Up to 10 Watt Exterior MR16 (Dwelling Area) LED | $16.50 | $5.75 |
| LT-98182 | Up to 15 Watt Exterior PAR30 (Common Area) LED | $33.32 | $5.75 |
| LT-21786 | Up to 15 Watt Exterior PAR30 (Dwelling Area) LED | $33.32 | $5.75 |
| LT-50213 | Up to 21 Watt Exterior PAR30 (Common Area) LED | $29.74 | $5.75 |
| LT-49986 | Up to 21 Watt Exterior PAR30 (Dwelling Area) LED | $29.74 | $5.75 |
| LT-49807 | Up to 17 Watt Exterior PAR38 (Common Area) LED | $35.12 | $5.75 |
| LT-35653 | Up to 17 Watt Exterior PAR38 (Dwelling Area) LED | $35.12 | $5.75 |
| LT-89912 | Up to 25 Watt Exterior PAR38 (Common Area) LED | $34.11 | $5.75 |
| LT-70912 | Up to 25 Watt Exterior PAR38 (Dwelling Area) LED | $34.11 | $5.75 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| NEW/NC |  |  |  |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |
| REF | (MEC + MLC) – (BEC + BLC) | MEC + MLC | N/A |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-37176 | ROB | $7.25 | $7.25 | N/A |
| LT-64932 | ROB | $7.25 | $7.25 | N/A |
| LT-60894 | ROB | $9.20 | $9.20 | N/A |
| LT-59328 | ROB | $9.20 | $9.20 | N/A |
| LT-89305 | ROB | $2.41 | $2.41 | N/A |
| LT-59089 | ROB | $2.41 | $2.41 | N/A |
| LT-98579 | ROB | $9.00 | $9.00 | N/A |
| LT-89203 | ROB | $9.00 | $9.00 | N/A |
| LT-98182 | ROB | $25.43 | $25.43 | N/A |
| LT-21786 | ROB | $25.43 | $25.43 | N/A |
| LT-50213 | ROB | $21.46 | $21.46 | N/A |
| LT-49986 | ROB | $21.46 | $21.46 | N/A |
| LT-49807 | ROB | $26.84 | $26.84 | N/A |
| LT-35653 | ROB | $26.84 | $26.84 | N/A |
| LT-89912 | ROB | $25.65 | $25.65 | N/A |
| LT-70912 | ROB | $25.65 | $25.65 | N/A |

# Attachments

# References

[355]

[493]

[A] http://www.energydataweb.com/cpucFiles/pdaDocs/1359/LED%20Study_Draft%20Final%20Report_20150828_Clean.pdf