Work Paper SCE13LG111

Revision 1

Southern California Edison

LED High and Low Bay Fixtures

# At-a-Glance Summary

| Measure Codes | Refer to Excel Calculation Attachment |
| --- | --- |
| Measure Description | LED High/Low Bay Fixtures |
| Base Case Description | Metal Halide/Pulse Start Metal Halide/Gen. 2 T8 High/Low Bay Fixtures |
| Units | Per fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | ILtg-Com-LED-50000hr – 12 years |
| Measure Installation Type | Replace on Burnout (ROB), Retrofit or Early Retirement (RET/ER) |
| Net-to-Gross Ratio | Com-Default>2yrs, Ind-Default>2yrs, Agric-Default>2yrs – 0.6 Com-Default-HTR-di, Ind-Default-HTR-di, Agricult-Default-HTR-di – 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 6/26/15 | Yun Han | - New work paper based on PGECOLTG178R0 - Effective 7/1/14 – 12/31/14 |
| 1 | 10/1/15 | Yun Han | - New template for 2016 program year - Added Metal Halide measures for Early Retirement and Replace on Burnout |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This workpaper details the replacement of Pulse-Start Metal Halide (PSMH) or T8 Linear Fluorescent High Bay and Low Bay fixtures with LED High-Bay and Low-Bay Fixtures. Tables below list the measures and their wattage ranges.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | LED High/Low-Bay Fixtures |
| Existing Condition | PSMH/T8 High/Low-Bay Fixtures |
| Code/Standard | N/A |
| Industry Standard Practice | PSMH |

Measures and Codes

| Measure Codes |  | Measure Name |
| --- | --- | --- |
| SCE | PG&E |  |
| LT-20785 | LD101 | LED High/Low Bay 40 to 131 Watts Replacing 175W PSMH |
| LT-47551 | LD102 | LED High/Low Bay >131 to 160 Watts Replacing 200W PSMH |
| LT-99280 | LD103 | LED High/Low Bay >160 to 187 Watts Replacing 250W PSMH |
| LT-36878 | LD104 | LED High/Low Bay >187 to 220 Watts Replacing 320W PSMH |
| LT-70713 | LD105 | LED High/Low Bay >220 to 262 Watts Replacing 350W PSMH |
| LT-28599 | LD106 | LED High/Low Bay >262 to 280 Watts Replacing 400W PSMH |
| LT-79419 | LD107 | LED High/Low Bay >280 to 320 Watts Replacing 450W PSMH |
| LT-22805 | LD108 | LED High/Low Bay >320 to 500 Watts Replacing 750W PSMH |
| LT-68702 | LD109 | LED High/Low Bay >500 to 750 Watts Replacing 1000W PSMH |
| LT-35334 | LD111 | LED High/Low Bay 40 to 131 Watts Replacing 4-Lamp VHLO 2nd Gen. T8 |
| LT-68154 | LD112 | LED High/Low Bay >131 to 160 Watts Replacing 6-Lamp VHLO 2nd Gen. T8 |
| LT-54307 | LD113 | LED High/Low Bay >160 to 220 Watts Replacing 8-Lamp VHLO 2nd Gen. T8 |
| LT-55561 |  | LED High/Low Bay 40 to 131 Watts Replacing 175W MH (175W PSMH) |
| LT-97550 |  | LED High/Low Bay >131 to 160 Watts Replacing 250W MH (200W PSMH) |
| LT-71583 |  | LED High/Low Bay >160 to 187 Watts Replacing 320W MH (250W PSMH) |
| LT-86205 |  | LED High/Low Bay >187 to 220 Watts Replacing 350W MH (320W PSMH) |
| LT-79060 |  | LED High/Low Bay >220 to 262 Watts Replacing 400W MH (350W PSMH) |
| LT-63775 |  | LED High/Low Bay >262 to 320 Watts Replacing 450W MH (400W PSMH) |
| LT-96353 |  | LED High/Low Bay >320 to 500 Watts Replacing 750W MH (750W PSMH) |
| LT-98618 |  | LED High/Low Bay >500 to 750 Watts Replacing 1000W MH (1000W PSMH) |

Products must be listed on Design Lights Consortium’s (DLC) qualified product list (QPL) to receive incentives [486]. Self-ballasted screw-based lamps do not qualify.

These measures are allowed in all Commercial building types in all SCE climate zones.

## 1.2 Technical Description

Low-bay fixture mounting heights vary between 8 to 20 feet while a high-bay generally varies from over 20 feet up to 40 feet depending on the system wattage. Title 24 [355] defines the cut-off of high and low bay to be 25 feet. High-bay aisle fixtures are grouped in the same category as the regular high-bay lighting fixture. The spread of the light (zonal lumen density) is the only difference between the two.

Requirements for DLC’s LED high-bay and low-bay applications include:

- ≥5,000 lumens

- Min. 80 lm/W

- ≤5,700K CCT

- ≥70 CRI

- 35,000 hr L70

- ≥30%: 20-50° (High and Low-Bay)

- ≥30%: 0-20°, ≥50%: 20-50° (High-Bay Aisle)

## 1.3 Installation Types and Delivery Mechanisms

The delivery methods and install types for the measures in this work paper are:

- Replace on Burn-out (ROB)

  - Financial Support: Down-Stream Incentive - Deemed

  - Midstream Programs: Midstream Incentive

- Retrofit (RET) or Early Retirement (ER)

  - Financial Support: Direct Install

Direct Install Early Retirement Implementation Plan

The Non Residential Direct Install program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is a form which information for measures installed in all building types related to the Non Residential Direct Install program.  The PLF contains the following fields: Service Account Address, Measures Proposed/Installed, Product Make/Model, Install Locations (detailed to define separate spaces/floors, as well as specific locations within the space including but not limited to:  Bathrooms, Hallways, Meeting Rooms, Offices, Warehouse, etc.).

Customers are solicited to participate primarily through field visits.  Contractors conduct energy consultation and provide recommendation that can help eligible customers use less energy.  If the customer agrees, the Direct Install contractor will help them complete an authorization form and schedule an installation appointment.

SCE can also provide photos to show measure functionality and a sample close up photos to substantiate the measure base case where applicable.  These photos would be part of the required project package.  The project package is identified with the Service Account Number and attached to SCE’s SMART database (SCE Project Tracking System) at each specific project level. The above described information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Mid-Stream Programs | See Mid-Stream Incentive in the Incentive Method Descriptions table. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |
| Mid-Stream Incentive Mid-Stream Buy Down | The program gives a financial incentive to a midstream market actor (distributor, vendor, or retailer) to encourage the promotion of efficient measures. Buy Down means that the incentive is required to be passed down to the end-use customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | N/A |
| Reason for Deviation from DEER | DEER does not contain this type of measure. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Com-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Com | Any | Any | 0.6 |
| Ind-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ind | Any | Any | 0.6 |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | Any | 0.6 |
| Com-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Com | Any | Any | 0.85 |
| Ind-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ind | Any | Any | 0.85 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | Any | 0.85 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

- Business Size – Customer must have less than ten employees

- Language – Customer’s primary language spoken is not English

- Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Com-LED-50000hr | LED Fixture - Indoor- Commercial | Com | Lighting | 12 | 4 |

### 1.4.2 Codes and Standards Analysis

Title 24 2013 [355] Section 141.0(b)2 contains codes related to Nonresidential lighting as shown below. The measures in this work paper do replace luminaires which triggers Lighting System Alterations. Triggering Alteration requires mandatory control provisions in Section 130.1(a)(b)(c)(d) for each enclosed space that includes Area, Shut-off, Multi-level, and if applicable, Daylighting Controls.

Lighting System Alterations shall meet the applicable requirements in TABLE 141.0-E and the following:

- Lighting System Alterations include alterations where an existing lighting system is modified, luminaires are replaced, or luminaires are disconnected from the circuit, removed and reinstalled, whether in the same location or installed elsewhere.

EXCEPTION 1 to Section 141.0(b)2Iii: Alterations that qualify as a Luminaire Modification-in-

Place.

EXCEPTION 2 to Section 141.0(b)2Iii: Portable luminaires, luminaires affixed to moveable partitions, and lighting excluded in accordance to Section 140.6(a)3.

Luminaire Modifications-in-Place shall meet the applicable requirements in TABLE 141.0-F and the following:

- To qualify as a Luminaire Modification-in-Place, luminaires shall only be modified by one or more of the following methods:

  - Replacing lamps and ballasts with like type or quantity in a manner that preserves the original luminaire listing.

  - Changing the number or type of light source in a luminaire including: socket renewal, removal or relocation of sockets or lampholders, and/or related wiring internal to the luminaire including the addition of safety disconnecting devices.

  - Changing the optical system of a luminaire in part or in whole.

  - Replacement of whole luminaires one for one in which the only electrical modification involves disconnecting the existing luminaire and reconnecting the replacement luminaire.

- Luminaire Modifications-In-Place shall include only alterations to lighting system meeting the following conditions:

  - Luminaire Modifications-in-Place shall not be part of or the result of any general remodeling or renovation of the enclosed space in which they are located.

  - Luminaire Modifications-in-Place shall not cause, be the result of, or involve any changes to the panelboard or branch circuit wiring, including line voltage switches, relays, contactors, dimmers and other control devices, providing power to the lighting system.

EXCEPTION to Section 141.0(b)2Iiii2. Circuit modifications strictly limited to the addition of occupancy or vacancy sensors and class two lighting controls are permitted for Luminaire

Modifications-in-Place

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Section 141.0(b)2Iii Lighting System Alterations | July 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

No studies were used in the development of this work paper.

## 1.6 Data Quality and Future Data Needs

No future data needs are currently anticipated.

# Section 2. Calculation Methodology

The fixture performance in the high-bay and low-bay categories of the DLC list was analyzed to justify the wattage equivalency assumptions. The 418 fixtures in these categories were analyzed for equivalency to common base case fixtures.

Rather than compare the fixtures based solely on lumen output, the quantity of lumens in the 20° - 50° zone was also considered.  Many Pulse Start Metal Halide HID and linear fluorescent fixtures commonly over-illuminate the area directly beneath the fixture (0° - 20°) simply because they lack the ability to direct light to where it is needed most further from nadir.  Pulse Start Metal Halide HID and linear fluorescent sources may achieve a higher average illuminance than an LED source (and have a correspondingly higher lumen output); however they achieve similar minimum illuminance because the LED fixture may be able to do a better job of directing light out to the edges of the illuminated space.

This analysis compares fixtures based on the lumen output in the 20° - 50° range to ignore the hot spot of light that may appear directly under a fixture, and in recognition of the fact that customers are often happy with the light output of LED fixtures with lower light output than the Pulse Start Metal Halide HID or linear fluorescent fixtures replaced.

The lumens in the 20° - 50° range was calculated from the DLC list based on the measured light output of each fixture multiplied by the % of lumens in that range, labeled on the DLC spreadsheet at ZL-HBLB: 20-50 or ZL-HBA: 20-50. The lumen output for base case fixtures in the 20° - 50° range was calculated from the zonal lumen summary tables of manufacturer photometric reports.  These values were corrected for lamp lumens and ballast factor based on industry standards for Pulse-Start Metal Halide Lamps and F32T8 2nd generation linear fluorescent lamps.

The LED products equivalent in lumen output in the 20° - 50° range to the base case fixtures were grouped as well as possible, given the limitation of varying LED fixture performance. The base cases for Pulse Start Metal Halide HID and 2nd generation linear fluorescent fixtures were compared separately to the group of DLC-approved fixtures that would best replace them based on photopic lumens in the 20° - 50° zones. Measure codes were created by setting maximum and minimum wattage and minimum fixture lumen output values. Product eligibility will be determined by comparing products to those 3 values as found in the DLC list.

The base case wattages were determined based on the 12 most common high-bay and low-bay fixture configurations which include PSMH and 2nd Gen. T8 VHLO LF lamps. The system Watts are from the Standard Fixture Wattages table [431].   These are considered equivalent to the code baselines.

The LED range was created based on lumen equivalence in the 20°-50° zone of the baseline. The measure wattage was determined by using the largest value within the wattage range of each measure.

Analysis of energy savings can be found in the attachment [Attachment 2].

Measure Names

| Measure Code | Measure Name | Fixture Code | Base System Watts | Minimum Fixture Lumens | LED Watts |
| --- | --- | --- | --- | --- | --- |
| LT-20785 | LED High/Low Bay 40 to 131 Watts Replacing 175W PSMH | MH175PS/1 | 0.208 | 6,200 | 0.131 |
| LT-47551 | LED High/Low Bay >131 to 160 Watts Replacing 200W PSMH | MH200PS/1 | 0.232 | 9,600 | 0.160 |
| LT-99280 | LED High/Low Bay >160 to 187 Watts Replacing 250W PSMH | MH250PS/1 | 0.288 | 11,200 | 0.187 |
| LT-36878 | LED High/Low Bay >187 to 220 Watts Replacing 320W PSMH | MH320PS/1 | 0.365 | 12,900 | 0.220 |
| LT-70713 | LED High/Low Bay >220 to 262 Watts Replacing 350W PSMH | MH350PS/1 | 0.400 | 15,800 | 0.262 |
| LT-28599 | LED High/Low Bay >262 to 280 Watts Replacing 400W PSMH | MH400PS/1 | 0.456 | 21,600 | 0.280 |
| LT-79419 | LED High/Low Bay >280 to 320 Watts Replacing 450W PSMH | MH450PS/1 | 0.506 | 23,900 | 0.320 |
| LT-22805 | LED High/Low Bay >320 to 500 Watts Replacing 750W PSMH | MH750PS/1 | 0.818 | 32,300 | 0.500 |
| LT-68702 | LED High/Low Bay >500 to 750 Watts Replacing 1000W PSMH | MH1000PS/1 | 1.080 | 43,400 | 0.750 |
| LT-35334 | LED High/Low Bay 40 to 131 Watts* Replacing 4-Lamp VHLO 2nd Gen. T8 | F44ILL/2-V | 0.152 | 6,200 | 0.131 |
| LT-68154 | LED High/Low Bay >131 to 160 Watts Replacing 6-Lamp VHLO 2nd Gen. T8 | F46ILL/2-V | 0.226 | 9,600 | 0.160 |
| LT-54307 | LED High/Low Bay >160 to 220 Watts* Replacing 8-Lamp VHLO 2nd Gen. T8 | F44ILL/2-V x 2 | 0.304 | 11,200 | 0.220 |
| LT-55561 | LED High/Low Bay 40 to 131 Watts Replacing 175W MH (175W PSMH) | MH175/1 | 0.215 | 6,200 | 0.131 |
| LT-97550 | LED High/Low Bay >131 to 160 Watts Replacing 250W MH (200W PSMH) | MH250/1 | 0.295 | 9,600 | 0.160 |
| LT-71583 | LED High/Low Bay >160 to 187 Watts Replacing 320W MH (250W PSMH) | MH320/1 | 0.365 | 11,200 | 0.187 |
| LT-86205 | LED High/Low Bay >187 to 220 Watts Replacing 350W MH (320W PSMH) | MH350/1 | 0.4 | 12,900 | 0.220 |
| LT-79060 | LED High/Low Bay >220 to 262 Watts Replacing 400W MH (350W PSMH) | MH400/1 | 0.458 | 15,800 | 0.262 |
| LT-63775 | LED High/Low Bay >262 to 320 Watts Replacing 450W MH (400W PSMH) | MH450/1 | 0.506 | 21,600 | 0.280 |
| LT-96353 | LED High/Low Bay >320 to 500 Watts Replacing 750W MH (750W PSMH) | MH750/1 | 0.85 | 32,300 | 0.500 |
| LT-98618 | LED High/Low Bay >500 to 750 Watts Replacing 1000W MH (1000W PSMH) | MH1000/1 | 1.08 | 43,400 | 0.750 |

*SCE wattages vary slightly relative to the PGE wattages in PGECOLTG178 R0 for LED High/Low Bay 40 to 131 Watts Replacing 4-Lamp VHLO 2nd Gen. T8 and LED High/Low Bay >160 to 187 Watts Replacing 8-Lamp VHLO 2nd Gen. T8

## 2.1 Energy Savings Estimation Methodologies

The following is a sample energy savings calculation of RET 1st baseline for LED High/Low Bay 40 to 131 Watts replacing 175W PSMH in an Assembly building type in Climate Zone 6.

EnergySavingskWhUnit×Year=208×1670×1.1-131×1620×1.11,000WatthourskWh

EnergySavingskWhUnit×Year=148.65

The following is a sample energy savings calculation of RET 2nd baseline  and ROB 1st baseline for LED High/Low Bay 48 to 131 Watts replacing 175W PSMH in an Assembly building type in Climate Zone 6.

EnergySavingskWhUnit×Year=208-131×1620×1.11,000WatthourskWh

EnergySavingskWhUnit×Year=137.21

## 2.2 Demand Reduction Estimation Methodologies

The following is a sample demand reduction calculation of RET 1st baseline for LED High/Low Bay 40 to 131 Watts replacing 175W PSMH in an Assembly building type in Climate Zone 6.

DemandReductionkWUnit=208×0.492×1.23-[131×0.48×1.23]1,000WatthourskWh

DemandReductionkWUnit=0.04853

The following is a sample demand reduction calculation of RET 2nd baseline  and ROB 1st baseline for LED High/Low Bay 48 to 131 Watts replacing 175W PSMH in an Assembly building type in Climate Zone 6. This calculation uses the occupancy sensor coincident diversity factor for the chosen building type to calculate the energy savings due to code.

DemandReductionkWUnit=208-131×0.48×1.231,000WatthourskWh

DemandReductionkWUnit=0.04546

Savings for other measures in different building types and climate zones can be found in the attachment [Attachment 1].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Assembly | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Community College | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Primary School | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Relocatable Classroom | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - Secondary School | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Education - University | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Grocery | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Guest Rooms | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Hospital | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Hotel | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Bio/Tech | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Manufacturing - Light Industrial | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Lodging - Motel | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Health/Medical - Nursing Home | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Office - Small | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Fast-Food | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Restaurant - Sit-Down | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Multistory Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Single-Story Large | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Retail - Small | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Conditioned | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Storage - Unconditioned | DEER:Indoor_Non-CFL_Ltg | NON_RES |
| Warehouse - Refrigerated | DEER:Indoor_Non-CFL_Ltg | NON_RES |

# Section 4. Costs

## 4.1 Base Case Cost

The base case equipment and labor costs are calculated from the WO17 workbook [Attachment 2].

Base Cost

| Measure | Base Equipment Cost | Base Labor Cost | Total Base Cost |
| --- | --- | --- | --- |
| LT-20785 | $208.29 | $148.89 | $357.18 |
| LT-47551 | $213.94 | $148.89 | $362.83 |
| LT-99280 | $217.55 | $148.89 | $366.44 |
| LT-36878 | $265.08 | $148.89 | $413.97 |
| LT-70713 | $269.19 | $148.89 | $418.08 |
| LT-28599 | $276.16 | $148.89 | $425.05 |
| LT-79419 | $280.06 | $148.89 | $428.95 |
| LT-22805 | $302.15 | $148.89 | $451.04 |
| LT-68702 | $323.05 | $148.89 | $471.94 |
| LT-35334 | $230.19 | $173.10 | $403.28 |
| LT-68154 | $278.66 | $173.10 | $451.76 |
| LT-54307 | $327.14 | $173.10 | $500.24 |
| LT-55561 | $206.96 | $148.89 | $355.85 |
| LT-97550 | $217.03 | $148.89 | $365.92 |
| LT-71583 | $221.74 | $148.89 | $370.63 |
| LT-86205 | $265.95 | $148.89 | $414.84 |
| LT-79060 | $271.87 | $148.89 | $420.76 |
| LT-63775 | $278.21 | $148.89 | $427.10 |
| LT-96353 | $302.79 | $148.89 | $451.68 |
| LT-98618 | $321.16 | $148.89 | $470.05 |

## 4.2 Measure Case Cost

Measure costs were provided by Navigant as part of a study on LEDs. The California LED Workpaper Update Study [A, Attachment 3] recommends using 25 percentile utilizing CA specific data. The same base labor cost is used for the measure labor cost.

Measure Cost

| Measure | Measure Equipment Cost | Measure Labor Cost | Total Measure Cost |
| --- | --- | --- | --- |
| LT-20785 | $255.35 | $148.89 | $404.24 |
| LT-47551 | $309.00 | $148.89 | $457.89 |
| LT-99280 | $525.37 | $148.89 | $674.26 |
| LT-36878 | $488.79 | $148.89 | $637.68 |
| LT-70713 | $588.93 | $148.89 | $737.82 |
| LT-28599 | $490.78 | $148.89 | $639.67 |
| LT-79419 | $955.03 | $148.89 | $1,103.91 |
| LT-22805 | $1,028.15 | $148.89 | $1,177.04 |
| LT-68702 | $1,492.35 | $148.89 | $1,641.24 |
| LT-35334 | $255.35 | $173.10 | $428.45 |
| LT-68154 | $309.00 | $173.10 | $482.10 |
| LT-54307 | $484.31 | $173.10 | $657.40 |
| LT-55561 | $255.35 | $148.89 | $404.24 |
| LT-97550 | $309.00 | $148.89 | $457.89 |
| LT-71583 | $525.37 | $148.89 | $674.26 |
| LT-86205 | $488.79 | $148.89 | $637.68 |
| LT-79060 | $588.93 | $148.89 | $737.82 |
| LT-63775 | $722.90 | $148.89 | $871.79 |
| LT-96353 | $1,028.15 | $148.89 | $1,177.04 |
| LT-98618 | $1,492.35 | $148.89 | $1,641.24 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |
| RET/ER | (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs - Retrofit

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-20785 | RET | $47.06 | $404.24 | $47.06 |
| LT-47551 | RET | $95.06 | $457.89 | $95.06 |
| LT-99280 | RET | $307.81 | $674.26 | $307.81 |
| LT-36878 | RET | $223.72 | $637.68 | $223.72 |
| LT-70713 | RET | $319.74 | $737.82 | $319.74 |
| LT-28599 | RET | $214.62 | $639.67 | $214.62 |
| LT-79419 | RET | $674.97 | $1,103.91 | $674.97 |
| LT-22805 | RET | $726.00 | $1,177.04 | $726.00 |
| LT-68702 | RET | $1,169.30 | $1,641.24 | $1,169.30 |
| LT-35334 | RET | $25.17 | $428.45 | $25.17 |
| LT-68154 | RET | $30.34 | $482.10 | $30.34 |
| LT-54307 | RET | $157.17 | $657.40 | $157.17 |
| LT-55561 | RET | $48.39 | $404.24 | $48.39 |
| LT-97550 | RET | $91.97 | $457.89 | $91.97 |
| LT-71583 | RET | $303.63 | $674.26 | $303.63 |
| LT-86205 | RET | $222.84 | $637.68 | $222.84 |
| LT-79060 | RET | $317.05 | $737.82 | $317.05 |
| LT-63775 | RET | $444.69 | $871.79 | $444.69 |
| LT-96353 | RET | $725.36 | $1,177.04 | $725.36 |
| LT-98618 | RET | $1,171.19 | $1,641.24 | $1,171.19 |

Full and Incremental Costs – Replace on Burnout

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-20785 | ROB | $47.06 | $47.06 | N/A |
| LT-47551 | ROB | $95.06 | $95.06 | N/A |
| LT-99280 | ROB | $307.81 | $307.81 | N/A |
| LT-36878 | ROB | $223.72 | $223.72 | N/A |
| LT-70713 | ROB | $319.74 | $319.74 | N/A |
| LT-28599 | ROB | $214.62 | $214.62 | N/A |
| LT-79419 | ROB | $674.97 | $674.97 | N/A |
| LT-22805 | ROB | $726.00 | $726.00 | N/A |
| LT-68702 | ROB | $1,169.30 | $1,169.30 | N/A |
| LT-35334 | ROB | $25.17 | $25.17 | N/A |
| LT-68154 | ROB | $30.34 | $30.34 | N/A |
| LT-54307 | ROB | $157.17 | $157.17 | N/A |
| LT-55561 | ROB | $48.39 | $48.39 | N/A |
| LT-97550 | ROB | $91.97 | $91.97 | N/A |
| LT-71583 | ROB | $303.63 | $303.63 | N/A |
| LT-86205 | ROB | $222.84 | $222.84 | N/A |
| LT-79060 | ROB | $317.05 | $317.05 | N/A |
| LT-63775 | ROB | $444.69 | $444.69 | N/A |
| LT-96353 | ROB | $725.36 | $725.36 | N/A |
| LT-98618 | ROB | $1,171.19 | $1,171.19 | N/A |

# Attachments

1. 2. 3.

# References

[355]

[431]

[486]

[A] http://www.energydataweb.com/cpucFiles/pdaDocs/1359/LED%20Study_Draft%20Final%20Report_20150828_Clean.pdf