Work Paper SCE13PR003

Revision 2

Southern California Edison

Agricultural Pump System Overhaul

# At-a-Glance Summary

| Measure Codes | PR-15002, PR-21099, PR-78421, PR-89445, PR-93334, PR-51322, PR-13447, PR-50497, PR-14088, PR-58004 |
| --- | --- |
| Measure Description | The overhaul of existing inefficient agricultural pumps with motors less than or equal to 25 horsepower (hp) and motors greater than 25 horsepower (hp) to 50 horsepower (hp). |
| Base Case Description | Existing agricultural pumps |
| Units | Per hp |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | ProcDist-PumpOver-SubWell: EUL 6.5yrs, RUL 2.16yrs ProcDist-PumpOver-SubBstr: EUL 8.3yrs, RUL 2.76yrs ProcDist-PumpOver-CentBstr: EUL 12.7yrs, RUL 4.23yrs ProcDist-PumpOver-TurbBstr: EUL 9.3yrs, RUL 3.10yrs ProcDist-PumpOver-TurbWell: EUL 6.8yrs, RUL 2.26yrs |
| Measure Installation Type | Retrofit Add-on (REA) |
| Net-to-Gross Ratio | Agric-Default>2yrs: 0.6 NonRes-sAll-mRCx-dn: 0.8 Agricult-Default-HTR-di: 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 6/1/12 | Sabarish Vinod/Lincus, Inc | - Original work paper template for 2013-2014 PC |
| 1 | 8/13/14 | Alfredo Gutierrez/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Added NTG for commissioning to the work paper -Added up to 50 HP measures -Updated DEER Peak Definitions, and adjusted the peak factor from 65% to 38% -Fixed costing for PR-93334 (Submersible Well Pump) -Updated measure names for existing solution codes to be less than 25hp. |
| 2 | 1/27/16 | Yun Han/SCE | - New template update for 2016 program year - WP effective from 1/1/2016 thru 12/31/2016 - Removed SCE building type - Changed Ag to MLi - No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the overhaul of existing inefficient agricultural pumps with motors less than 25 horsepower (hp) and motors greater than 25 horsepower (hp) to 50 horsepower (hp). The measure is a pump that is overhauled for improved efficiency to better match the design of the pump to the actual system operating conditions. Doing so will improve the overall plant efficiency (OPE) of the pump. The base case is an existing pump that is operating inefficiently. Inefficiency can be caused by general wear and tear or pump operation that has deviated   from the original design conditions. Operators typically look for large reductions in flow rate or discharge pressure delivered by the pump to gauge the condition of the equipment. While these techniques do not provide an accurate description of the pump these are good indicators that the pump is in need of repair.

The energy savings are calculated on a per horsepower (hp) basis. Each estimate takes into account: the pump normal operation point, pumping pressures, water flow rate, type of pump, electric motor nameplate horsepower rating, pumping application, pump test history, overall plant efficiency, and other pump and site dependent information.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Turbine/Centrifugal/Submersible Booster/Well Pump System Overhaul Maintenance with motors less than or equal to 25 horsepower (hp) and motors greater than 25 horsepower (hp) to 50 horsepower (hp) |
| Existing Condition | Existing Inefficient Agricultural Pump |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

The following table provides a brief overview of the measures included in this work paper.

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | PR-15002 |  | < 25 HP Turbine Booster Pump System Overhaul Maintenance |
|  |  | PR-21099 |  | < 25 HP Turbine Well Pump System Overhaul Maintenance |
|  |  | PR-78421 |  | < 25 HP Centrifugal Booster Pump System Overhaul Maintenance |
|  |  | PR-89445 |  | < 25 HP Submersible Booster Pump System Overhaul Maintenance |
|  |  | PR-93334 |  | < 25 HP Submersible Well Pump System Overhaul Maintenance |
|  |  | PR-51322 |  | 25-50 HP Turbine Booster Pump System Overhaul Maintenance |
|  |  | PR-13447 |  | 25-50 HP Turbine Well Pump System Overhaul Maintenance |
|  |  | PR-50497 |  | 25-50 HP Centrifugal Booster Pump System Overhaul Maintenance |
|  |  | PR-14088 |  | 25-50 HP Submersible Booster Pump System Overhaul Maintenance |
|  |  | PR-58004 |  | 25-50 HP Submersible Well Pump System Overhaul Maintenance |

Only pumps <= 50hp qualify for this rebate. The measure should include one of the following:

- Replacing pump bowl assembly/impeller

- Adjusting bowl and impeller on a deep well pump. Field adjustments include: a) for semi-open impellers, all impellers in the bowl assembly must be running in close proximity (0.003 – 0.007 in) to the next lower bowl. Thus, careful adjustment in the field is required. Shaft stretch determines the final position of the impellers. Also, it directly varies with discharge head. Therefore, it has to be set to a proper specification to perform well at a given discharge head. Multistage units may require that the impellers be trimmed (reduction in diameter) to obtain proper fitting and clearance in the assembly bowl. For enclosed impellers, with a principal seal that is parallel to the centerline of the shaft, a close axial adjustment is not necessary. Therefore, this type of impeller is suited for operation under variable head conditions. Capacity and horsepower requirements can be controlled by raising the impeller until the skirts are out of the wear rings.

- Trimming the existing impeller on a booster pump

Only the following pump types may qualify for an incentive in this work paper:

- Submersible Well

- Submersible Booster

- Centrifugal Booster

- Turbine Booster

- Turbine Well

## 1.2 Technical Description

The program provides well trained pump test staff that test participants’ pumps and make recommendations about how the pump’s efficiency could be improved. The program targets pump owners in agricultural and municipal settings for any pump that moves potable water, especially high load factor customers. Participants request a pump test and SCE pump test staff visit the pump site and conduct a test that can last about three hours. The pump tester then sends written results to the pump owner that discusses the efficiency of the pump and makes recommendations on how the efficiency could be improved. The program addresses a variety of pump types, including turbine well, turbine booster, submersible well, submersible booster, and centrifugal booster pumps. This process evaluation study focused on three specific pump improvements; replacing the bowl and impeller, adjusting the bowl and impeller on a deep well pump and trimming the impeller on a booster pump [124]. It is important to remember that the work would differ based on the pump application type, e.g. “Well pump application” or “booster pump application.” In general after a pump efficiency test is conducted, the pump needs to be pulled, cleaned, disassembled, and inspected by a pump company. Then the pump company provides improvement recommendations for replacement and / or repairs in most cases there is a combination of both.

The OPE is made up of three components the major contributing factor to the overall plant efficiency is the bowl and impeller assembly which does the mechanical work output. The other two contributing factors are motor efficiency, and system losses which make up a very small percent of the OPE. OPE recommendations (improved OPE %) assume keeping the same motor efficiency, and the system efficiency little to no change (i.e. fixed values).

Figure 1 Inefficient vs. Efficient Plant

## 1.3 Installation Types and Delivery Mechanisms

The measure installation type is Retrofit Add-on (REA). Customers typically only make minimum modifications to their pumps to maintain and prolong their operation. The complete overhaul of a pump before the end of its useful life is typically only performed in the presence of an energy efficiency incentive.

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Retrofit Add-on (REA) | Above Customer Existing | N/A | EUL | N/A |

The delivery methods for these measures are:

- Financial Support/ Downstream Incentive – Deemed

- Financial Support / Direct Install

- Partnership / Direct Install

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |
| Partnership | The program implements projects through a partnership between the utility and an institutional, government, or community-based organization. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | No |
| DEER Measure Case | No |
| DEER Building Types | Yes |
| DEER Operating Hours | No |
| DEER eQUEST Prototypes | No |
| DEER Version | N/A |
| Reason for Deviation from DEER | DEER does not contain this type of measure. |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |
| NonRes-sAll-mRCx-dn | Retro-commissioning services, electric & nat. gas measures | Com | Any | PreRebDown | 0.8 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer facilities.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer facilities as HTR and also states that two criteria are sufficient to identify HTR customers if one of the criteria met is the geographic criteria.

SCE’s Commercial Direct Install program delivers free and low cost energy efficiency hardware retrofits through installation contractors to reduce peak demand and energy savings for small and medium commercial customers.  The barriers for customer participation include limited capital resources, lack of expertise and understanding of the understanding of the benefits of energy efficiency, a suspicion of the “free offer” and its legitimacy, and language and cultural barriers.  The program also addresses the ongoing concern with “split incentives”, where the customer is not the owner of the property, and therefore, lack incentive to improve their energy usage.  SCE’s Commercial Direct Install program will track the following three (3) customer data points to identify direct install activities in HTR customer facilities.  If geography and business size criteria are satisfied, SCE will identify the customer as HTR.  If geography and language criteria are satisfied, SCE will identify the customer as HTR.  Other measures in the Commercial Direct Install program will receive default NTG (NTGR_ID: Com-Default>2), unless otherwise specified in DEER.

o	Business Size – Customer must have less than ten employees

o	Language – Customer’s primary language spoken is not English

o	Geography – Businesses in areas other than the United States Office of Management and Budget (OMB) Combined Statistical Areas (CSA) of the San Francisco Bay Area,  the Greater Los Angeles Area and the Greater Sacramento Area  or the OBM metropolitan statistical areas or San Diego County

The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria:

“Notes on OMB CSA designations:

The OMB has designated a 12-county CSA titled the San Jose-San Francisco-Oakland, CA Combined Statistical Area which includes the nine counties of Alameda, Contra Costa, Marin, Napa, San Francisco, San Mateo, Santa Clara, Solano, and Sonoma which border the San Francisco Bay plus the three counties of San Joaquin, Santa Cruz, and San Benito that are economically tied to the nine counties that that border the San Francisco Bay.”

The OMB definition of this CSA includes Los Angeles, Orange, San Bernardino, Riverside and Ventura counties.

The OMB definition of this CSA includes Sacramento, Yolo, El Dorado, Placer, Sutter, Yuba, and Nevada counties.”

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ProcDist-PumpOver-SubWell | Submersible Well Pump System Overhaul | Ag | Pumping | 6.5 | 2.16 |
| ProcDist-PumpOver-SubBstr | Submersible Booster Pump System Overhaul | Ag | Pumping | 8.3 | 2.76 |
| ProcDist-PumpOver-CentBstr | Centrifugal Booster Pump System Overhaul | Ag | Pumping | 12.7 | 4.23 |
| ProcDist-PumpOver-TurbBstr | Turbine Booster Pump System Overhaul | Ag | Pumping | 9.3 | 3.10 |
| ProcDist-PumpOver-TurbWell | Turbine Well Pump System Overhaul | Ag | Pumping | 6.8 | 2.26 |

### 1.4.2 Codes and Standards Analysis

There are no code requirements within the 2013 Title 24 [355] and the 2014 Title 20 [422] which apply to the measures within this work paper.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | July 1, 2014 |
| Title 20 (2014) | N/A | July 1, 2014 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

This work paper uses program data for the Agricultural Pumping Program to establish the savings and cost for the measures contained within this work paper. The original data set used can be found in the attachments section.

Additionally, to establish the EUL, the Process Evaluation Report for the SCE Agricultural Energy Efficiency Program [124] was used. The EUL can be found on page “E-8” in the process evaluation report.

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

The following variables must be estimated to quantify the savings resulting from overhauling pumps with motors that are less than or equal to 25 hp and 25 hp to 50 hp:

- Pump size

- Annual hours of operation

- Motor loading

- Baseline Operating Plant Efficiency (OPE)

- Post Operating Plant Efficiency

The Southern California Edison (SCE) pump test database was used extensively to obtain average values of the variables listed above. Data for four years were used in estimating the averages [Attachment 2]. As more data becomes available the averages provided in this work paper may need to be revised for accuracy. Where data is missing or limited, average savings values were used to fill in the data.

The data was disaggregated according to the following pump types for each climate zone within SCE’s service territory:

- Submersible Well

- Submersible Booster

- Centrifugal Booster

- Turbine Well

- Turbine Booster

The following table shows a sample breakdown of average values from CZ10 for the less than 25hp measure. Average value calculations for the other SCE climate zones are provided in the Attachments section (Attachment 2 and Attachment 3).

Estimation of Variables for CZ 10

| Pump Type | Average Annual Hours of Operation (AOH) | Average Pump Size (Nominal Motor hp) | Average Baseline OPE (%) | Average Proposed OPE (%) | Average Motor Loading |
| --- | --- | --- | --- | --- | --- |
| Centrifugal Booster | 3,204.78 | 13.62 | 41.23 | 58.48 | 85% |
| Submersible Booster | 1,897.56 | 15.49 | 47.78 | 56.87 | 90% |
| Submersible Well | 3,426.63 | 10.80 | 38.85 | 53.51 | 109% |
| Turbine Booster | 2,364.63 | 14.32 | 53.00 | 61.20 | 90% |
| Turbine Well | 3,750.51 | 11.78 | 42.70 | 58.57 | 71% |

Equation 1

kWh Savings=Avg. Pump Size hpx 0.746kWhp x Avg. Motor Loading x Avg. AOH x (1-OPEBaseOPEPost)

Equation 2

kW Savings=kWh SavingsAvg. AOH

Average Savings per Pump Type

| Pump Type | kWh Savings (kWh/yr) | Avg. kW Savings |
| --- | --- | --- |
| Centrifugal Booster | 8,164.22 | 2.55 |
| Submersible Booster | 3,154.34 | 1.66 |
| Submersible Well | 8,244.33 | 2.41 |
| Turbine Booster | 3,046.14 | 1.29 |
| Turbine Well | 6,340.66 | 1.69 |

Equation 3

Gross Unit Annual Electricity Savings kWhunit=kWh SavingsAvg. Pump Size

Equation 4

Gross Peak Demand ReductionkWunit=kW Savings xPeak Adjustment FactorAvg. Pump Size

Peak kW Adjustment Factor

The DEER peak-defined period was revised to capture the three-day heat wave within the 16 CEC Title 24 climate zones as defined in the EE Policy Manual (Version 4). Table below exhibits the DEER peak-defined period by climate zone.

DEER Peak Period by Climate Zone

| CZ | Start Date |  | Weekday | Peak T | Ave T |
| --- | --- | --- | --- | --- | --- |
|  | Month | Day |  |  |  |
| CZ01 | Sep | 16 | Wed | 81 | 59.8 |
| CZ02 | Jul | 8 | Wed | 103 | 75.9 |
| CZ03 | Jul | 8 | Wed | 91 | 69.2 |
| CZ04 | Sep | 1 | Tue | 99 | 77.5 |
| CZ05 | Sep | 8 | Tue | 87 | 64.8 |
| CZ06 | Sep | 1 | Tue | 102 | 77.1 |
| CZ07 | Sep | 1 | Tue | 90 | 73.9 |
| CZ08 | Sep | 1 | Tue | 105 | 79.8 |
| CZ09 | Sep | 1 | Tue | 107 | 86.6 |
| CZ10 | Sep | 1 | Tue | 109 | 86.3 |
| CZ11 | Jul | 8 | Wed | 113 | 88.3 |
| CZ12 | Jul | 8 | Wed | 109 | 82.4 |
| CZ13 | Jul | 8 | Wed | 108 | 86.7 |
| CZ14 | Aug | 26 | Wed | 105 | 86.8 |
| CZ15 | Aug | 25 | Tue | 112 | 97.5 |
| CZ16 | Jul | 8 | Wed | 90 | 78.8 |

In order to confirm if the pumps operate during the peak-defined period, five years of billing data were analyzed for Time of Use (TOU) accounts where SCE tested a water pump. The five-year billing data list one billing period (one summer month) for each of five years – (2009-2013) [Attachment 4]. The data validates the distribution of when the pumps operate during the on-peak period (also called load factor). One billing period per year was selected as with an August read date. If an August read date could not be obtained, then a July read date was used.

The five-year billing data makes up over 16,000 records, which include three different customer types (Agriculture, Irrigation, and Water Supply) and twenty different TOU tariff rates. In order to determine a reasonable peak kW adjustment factor, the following steps were used to arrive at more appropriate data set:

- Remove all billing data yielding a zero “Usage kWh”

- Remove all billing data less than 5 “Max kW” and billing data that yielding a zero “Max kW” usage

- Remove all billing data less than 5 “On Peak kW” and billing data that yielding a zero “On Peak kW” usage

- Remove all billing data yielding a “Bill Period Load Factor” less than 10%. Any load factors less than 10% is assumed to operate infrequently during the month.

- Remove all billing data yielding an “On Peak Load Factor” less than 8.3% (30 min/day run). Any “On Peak Load Factor” operating less than 30 minutes per day is assumed to operate infrequently during the month.

- Remove all billing data yielding a “Max kW to On Peak kW Ratio” less than 3.0. Any “Max kW to On Peak kW Ratio less than 3.0 is assumed to operate infrequently during the month

Where,

Usage kWh = Total kWh usage during the billing period

Max kW = Maximum demand usage value (kW) for that billing period

On Peak kW = Read off of the customer database system

Bill Period Load Factor = (Usage kWh/month) / (Max kW * 24 hrs/day * Billing Days/ month)

On Peak Load Factor = On Peak kWh / (6 peak operating hours * On Peak Days)

On Peak Days = [(Billing days per month) * (5 weekdays / 7 weekdays)]

Max kW to On Peak kW Ratio = Max kW / On Peak kW

Using the steps listed above, the data set indicated that the peak kW adjustment factor for all customer types and tariff rates amounted to 38%.

Sample calculation for a centrifugal booster pump overhaul measure in climate zone 10:

kWh Savings=13.62x 0.746kWhp x 85% x 3204.78 x 1-41.23%58.48%= 8,164.22 kWh/yr

kW Savings=8,164.223204.78=2.55 kW

Gross Unit Annual Electricity Savings kWhunit=8164.2213.62=599.43kWhunit

Gross Peak Demand ReductionkWunit=2.55 x 38%13.62=0.07108kWunit

Savings Estimates

| Measure Description | CZ | Gross Unit Annual Electricity Savings (kWh/unit) | Gross Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| Centrifugal Booster Pump System Overhaul (<25hp) | 10 | 599.43 | 0.07108 |
| Submersible Booster Pump System Overhaul (<25hp) | 10 | 203.64 | 0.04078 |
| Submersible Well Pump System Overhaul (<25hp) | 10 | 763.36 | 0.08465 |
| Turbine Booster Pump System Overhaul (<25hp) | 10 | 212.72 | 0.03418 |
| Turbine Well Pump System Overhaul (<25hp) | 10 | 538.26 | 0.05454 |

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Manufacturing – Light Industrial | Ag & Water Pumping | Agricultural |

# Section 4. Costs

## 4.1 Base Case Cost

For this measure category, the base case cost is assumed to be zero as these are discretionary add-ons (REA) to the customers’ existing equipment. Their alternative is to make no changes to their existing system.

## 4.2 Measure Case Cost

For this deemed measure, cost data was obtained from existing program data. The cost information for each climate zone is given in the table below. The cost is normalized based on kWh savings. This is based on past program results for this overhaul measure (Attachment 4).

Equation 5

Cost/kWh $kWh=Measure CostkWh

2010-2011 Measure Cost per Unit by Climate Zone

| Climate Zone | kWh | Gross Measure cost | Cost/kWh |
| --- | --- | --- | --- |
| 6 | 1,417,180 | $479,596.93 | $0.34 |
| 8 | 1,345,477 | $381,211.60 | $0.28 |
| 9 | 4,230,852 | $1,716,363.21 | $0.41 |
| 10 | 5,155,257 | $5,532,045.87 | $1.07 |
| 13 | 10,573,106 | $6,434,805.90 | $0.61 |
| 14 | 1,642,449 | $1,575,155.34 | $0.96 |
| 15 | 221,504 | $102,601.44 | $0.46 |
| 16 | 328,166 | $352,103.01 | $1.07 |

Using this cost estimate above, per unit cost information was derived using kWh savings and average pump size using the following equations:

Equation 6

Cost of Retrofit $=CostkWhx kWh Savings

Equation 7

Measure Cost $unit=Cost of Retrofit $Average Pump Size hp

For example, for < 25hp Centrifugal Booster Pump System Overhaul in CZ 10,

Cost of Retrofit $=$1.07CostkWhx 8164.22=$8,735.72

Equation 8

Gross Measure Cost $unit=$8,735.7213.62=641.39$unit

The table below provides an example of typical costs involved in overhauling pumps (PR-78421).

Measure Cost

| Measure Description | CZ | Average Pump Size (Nominal motor hp) | kWh Savings (kWh/yr) | Cost/kWh | Cost of Retrofit ($) | Measure Cost($/unit) |
| --- | --- | --- | --- | --- | --- | --- |
| Centrifugal Booster Pump System Overhaul (<25hp) | 6 | 14.25 | 8,694.06 | $0.34 | $2,955.98 | $207.44 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 8 | 11.24 | 7,562.48 | $0.28 | $2,117.50 | $188.39 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 9 | 13.08 | 5,143.65 | $0.41 | $2,108.90 | $161.23 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 10 | 13.62 | 8,164.22 | $1.07 | $8,735.72 | $641.39 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 13 | 13.15 | 3,379.06 | $0.61 | $2,061.23 | $156.75 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 14 | 14.22 | 6,148.32 | $0.96 | $5,902.39 | $415.08 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 15 | 13.00 | 8,144.07 | $0.46 | $3,746.27 | $288.17 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 16 | 10.70 | 3,293.90 | $1.07 | $3,524.47 | $329.39 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| REA | MEC + MLC | MEC + MLC | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Climate Zone | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| PR-78421 | 6 | $207.44 | $207.44 | N/A |
| PR-78421 | 8 | $188.39 | $188.39 | N/A |
| PR-78421 | 9 | $161.23 | $161.23 | N/A |
| PR-78421 | 10 | $641.39 | $641.39 | N/A |
| PR-78421 | 13 | $156.75 | $156.75 | N/A |
| PR-78421 | 14 | $415.08 | $415.08 | N/A |
| PR-78421 | 15 | $288.17 | $288.17 | N/A |
| PR-78421 | 16 | $329.39 | $329.39 | N/A |

# Attachments

1. 2.  *Savings for measures less than 25 hp.

3.   4.   5.

# References

[124]

[355]

[422]