Work Paper SCE13PR003

Revision 1

Southern California Edison Company

Agricultural Pump System Overhaul

# At-a-Glance Summary

| Applicable Measure Codes: | PR-15002, PR-21099, PR-78421, PR-89445, PR-93334, PR-51322, PR-13447, PR-50497, PR-14088, PR-58004 |
| --- | --- |
| Measure Description: | The overhaul of existing inefficient agricultural pumps with motors less than or equal to 25 horsepower (hp) and motors greater than 25 horsepower (hp) to 50 horsepower (hp). |
| Base Case Description: | Existing agricultural pumps |
| Energy Impact Common Units: | Per hp |
| Energy Savings : | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life (years): | See table 7 [A] |
| Measure Application Type: | Retrofit Add On (REA) |
| Net-to-Gross Ratios: | 0.6 (Agric-Default>2yrs) 0.8 (NonRes-sAll-mRCx-dn) |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/14 CPUC Ex Ante Database Specification; SCE will provide that data set separately. |

# Document Revision History

| Workpaper and Revision # | Tech. Revision | MM/DD/YY | Author/Affiliation | Summary of Changes |
| --- | --- | --- | --- | --- |
| SCE13PR003.0 | Yes | 6/1/2012 | Sabarish Vinod/Lincus, Inc | - Original work paper template for 2013-2014 PC |
| SCE13PR003.1 | No | 8/13/2014 | Alfredo Gutierrez/SCE | -Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. -Added NTG for commissioning to the work paper -Added up to 50 HP measures -Updated DEER Peak Definitions, and adjusted the peak factor from 65% to 38% -Fixed costing for PR-93334 (Submersible Well Pump) -Updated measure names for existing solution codes to be less than 25hp. |

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the overhaul of existing inefficient agricultural pumps with motors less than 25 horsepower (hp) and motors greater than 25 horsepower (hp) to 50 horsepower (hp). The measure is a pump that is overhauled for improved efficiency to better match the design of the pump to the actual system operating conditions. Doing so will improve the overall plant efficiency (OPE) of the pump. The base case is an existing pump that is operating inefficiently. Inefficiency can be caused by general wear and tear or pump operation that has deviated   from the original design conditions. Operators typically look for large reductions in flow rate or discharge pressure delivered by the pump to gauge the condition of the equipment. While these techniques do not provide an accurate description of the pump these are good indicators that the pump is in need of repair.

The energy savings are calculated on a per horsepower (hp) basis. Each estimate takes into account: the pump normal operation point, pumping pressures, water flow rate, type of pump, electric motor nameplate horsepower rating, pumping application, pump test history, overall plant efficiency, and other pump and site dependent information.

The following table provides a brief overview of the measures included in this work paper.

Table 1 Measure Names

| Solution Code | Measure name |
| --- | --- |
| PR-15002 | < 25 HP Turbine Booster Pump System Overhaul Maintenance |
| PR-21099 | < 25 HP Turbine Well Pump System Overhaul Maintenance |
| PR-78421 | < 25 HP Centrifugal Booster Pump System Overhaul Maintenance |
| PR-89445 | < 25 HP Submersible Booster Pump System Overhaul Maintenance |
| PR-93334 | < 25 HP Submersible Well Pump System Overhaul Maintenance |
| PR-51322 | 25-50 HP Turbine Booster Pump System Overhaul Maintenance |
| PR-13447 | 25-50 HP Turbine Well Pump System Overhaul Maintenance |
| PR-50497 | 25-50 HP Centrifugal Booster Pump System Overhaul Maintenance |
| PR-14088 | 25-50 HP Submersible Booster Pump System Overhaul Maintenance |
| PR-58004 | 25-50 HP Submersible Well Pump System Overhaul Maintenance |

Only pumps <= 50hp qualify for this rebate. The measure should include one of the following:

- Replacing pump bowl assembly/impeller

- Adjusting bowl and impeller on a deep well pump. Field adjustments include: a) for semi-open impellers, all impellers in the bowl assembly must be running in close proximity (0.003 – 0.007 in) to the next lower bowl. Thus, careful adjustment in the field is required. Shaft stretch determines the final position of the impellers. Also, it directly varies with discharge head. Therefore, it has to be set to a proper specification to perform well at a given discharge head. Multistage units may require that the impellers be trimmed (reduction in diameter) to obtain proper fitting and clearance in the assembly bowl. For enclosed impellers, with a principal seal that is parallel to the centerline of the shaft, a close axial adjustment is not necessary. Therefore, this type of impeller is suited for operation under variable head conditions. Capacity and horsepower requirements can be controlled by raising the impeller until the skirts are out of the wear rings.

- Trimming the existing impeller on a booster pump

Only the following pump types may qualify for an incentive in this work paper:

- Submersible Well

- Submersible Booster

- Centrifugal Booster

- Turbine Booster

- Turbine Well

Specialty pumps with pre-determined low load factors such as fire pumps and storm water pumps do not qualify for an incentive. Industrial pumps do not qualify for an incentive. This work paper only considers cases where existing pumps that are overhauled before they break down. New pump curves clearly showing the design conditions should be submitted with the application.

## 1.2 Technical Description

The program provides well trained pump test staff that test participants’ pumps and make recommendations about how the pump’s efficiency could be improved. The program targets pump owners in agricultural and municipal settings for any pump that moves potable water, especially high load factor customers. Participants request a pump test and SCE pump test staff visit the pump site and conduct a test that can last about three hours. The pump tester then sends written results to the pump owner that discusses the efficiency of the pump and makes recommendations on how the efficiency could be improved. The program addresses a variety of pump types, including turbine well, turbine booster, submersible well, submersible booster, and centrifugal booster pumps. This process evaluation study focused on three specific pump improvements; replacing the bowl and impeller, adjusting the bowl and impeller on a deep well pump and trimming the impeller on a booster pump [124]. It is important to remember that the work would differ based on the pump application type, e.g. “Well pump application” or “booster pump application.” In general after a pump efficiency test is conducted, the pump needs to be pulled, cleaned, disassembled, and inspected by a pump company. Then the pump company provides improvement recommendations for replacement and / or repairs in most cases there is a combination of both.

The OPE is made up of three components the major contributing factor to the overall plant efficiency is the bowl and impeller assembly which does the mechanical work output. The other two contributing factors are motor efficiency, and system losses which make up a very small percent of the OPE. OPE recommendations (improved OPE %) assume keeping the same motor efficiency, and the system efficiency little to no change (i.e. fixed values).

Figure 1 Inefficient vs. Efficient Plant

## 1.3 Measure Application Type

Note: See Appendix A for a comparison of the application types used by and incorporated into SCE systems versus the application types available in the newest revision of DEER 2014. Appendix A will serve as a translation between the outputs of this workpaper and application types used by READi.

The delivery methods for these measures are:

- Financial Support/ Downstream Incentive – Deemed

- Commissioning / Down-Stream Incentive – Deemed

- Financial Support / Direct Install

- Partnership / Direct Install

The measure installation type is Retrofit Add-on (REA). Customers typically only make minimum modifications to their pumps to maintain and prolong their operation. The complete overhaul of a pump before the end of its useful life is typically only performed in the presence of an energy efficiency incentive.

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

The DEER 2014 READI Tool, version 2.0.2 does not contain agricultural pumping measures. As such, DEER was not used to determine the savings and cost of these measures.

Table 2 DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Modified DEER Methodology | No |
| Scaled DEER Measure | No |
| DEER Building Prototypes Used | No |
| Deviation from DEER | DEER does not contain this type of measure. |
| DEER Version | N/A |
| DEER Run ID and Measure Name (Sample) | N/A |

Net to Gross

The NTG value was obtained from the “DEER2011_NTGR_2012-05-16.xls” on the DEER website as required by Version 5 of the California Public Utilities Commission (CPUC) Energy Efficiency Policy Manual [351]. The relevant NTGR for this measure is shown in Table 3 below.

Table 3 Net-to-Gross Ratio

| NTGR_ID* | Description* | Sector* | BldgType* | ProgDelivID* | NTG* |
| --- | --- | --- | --- | --- | --- |
| Agric-Default>2yrs | All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years | Ag | Any | All | 0.6 |
| NonRes-sAll-mRCx-dn | Retro-commissioning services, electric & nat. gas measures | Com | Any | PreRebDown | 0.8 |
| Agricult-Default-HTR-di | All other EEM with no evaluated NTGR; direct install to hard-to-reach only. | Ag | Any | DirInstall | 0.85 |

*Denotes that the column is taken from the DEER NTG Table.

Installation Rate

The installation rate (IR) is identified in the calculation attachment. This value is obtained from the support table available in READi. Currently there is no versioning on the installation rate table. To address appropriate selection of the installation rate the date of the workpaper will serve as the last date checked for updated IR values. The installation rate varies by end use, sector, technology, application, and delivery method. The relevant IR values for this measure are shown in Table 4 below.

Table 4 Installation Rate

| GSIA_ID* | Description* | Sector* | BldgType* | ProgDelivID* | GSIAValue* |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

*Denotes that the column is taken from the DEER GSIA Table.

Spillage Rate

Spillage rate will also be applied to measures however the values will not be tracked in the workpapers. The spillage rate will be tracked in an external table to be supplied to the Energy Division.

READi Technology Fields

To support the development of the ED ex ante tables, select fields from the ex-ante database will be identified in the workpaper. For a full set of values associated with the measures in the workpaper refer the Excel calculation template.

Table 5 READi Tech IDs

| READi Field Name | Values included in this workpaper |
| --- | --- |
| Measure Case UseCategory | Irrigation |
| Measure Case UseSubCats | Farming Irrigation |
| Measure Case TechGroups | Irrigation Equipment |
| Measure Case TechTypes | Irrigation System |
| Base Case TechGroups | Irrigation Equipment |
| Base Case TechTypes | Irrigation System |

### 1.4.2 Codes and Standards Analysis

There are no code requirements within the 2013 Title 24 [355] and the 2014 Title 20 [422] which apply to the measures within this work paper.

Table 6 Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | N/A |
| Title 20 (2014) | N/A | N/A |

### 1.4.3 Non-DEER Study Review

This work paper uses program data for the Agricultural Pumping Program to establish the savings and cost for the measures contained within this work paper. The original data set used can be found in the attachments section.

Additionally, to establish the EUL, the Process Evaluation Report for the SCE Agricultural Energy Efficiency Program [124] was used. The EUL can be found on page “E-8” in the process evaluation report.

### 1.4.4  Measure and Base Case Effective Useful Life

DEER14 update documentation provides EUL and RUL information to be used for the 2013-14 program cycle on www.deeresources.com. The DEER documentation “DEER2014-EUL-table-update_2014-02-05.xlsx” provides the RUL value as a flat 1/3 of the EUL value. The RUL value will only be applied to the first baseline period for retrofit measures that have applicable code that will affect the energy savings. In all other installation types and retrofit with no applicable code that affects the energy savings, the RUL is not applicable to either the first or second baseline period.

To obtain the EUL value the DEER14 update documentation, “DEER2014-EUL-table-update_2014-02-05.xlsx” [436], was consulted, however, these specific measures are not included in DEER EUL summary. Therefore, a study involving pump overhaul measures was consulted [124]. This study did not contain a EUL value for the submersible booster pump. Hence, a weighted average was calculated for this based on the following weightings: 12% centrifugal booster pumps, 14% submersible well pumps, 31% turbine booster pumps, and 43% turbine well pumps.

Table 7 below identifies the value/methodology used for the measures in this work paper.

Table 7 EUL Value/Methodology

| READi EUL ID | Description | Sector | Enduse | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ProcDist-PumpOver-SubWell | Submersible Well Pump System Overhaul | Ag | Pumping | 6.5 | 2.16 |
| ProcDist-PumpOver-SubBstr | Submersible Booster Pump System Overhaul | Ag | Pumping | 8.3 | 2.76 |
| ProcDist-PumpOver-CentBstr | Centrifugal Booster Pump System Overhaul | Ag | Pumping | 12.7 | 4.23 |
| ProcDist-PumpOver-TurbBstr | Turbine Booster Pump System Overhaul | Ag | Pumping | 9.3 | 3.10 |
| ProcDist-PumpOver-TurbWell | Turbine Well Pump System Overhaul | Ag | Pumping | 6.8 | 2.26 |

# Section 2. Energy Savings & Demand Reduction Calculations

The following variables must be estimated to quantify the savings resulting from overhauling pumps with motors that are less than or equal to 25 hp and 25 hp to 50 hp:

- Pump size

- Annual hours of operation

- Motor loading

- Baseline Operating Plant Efficiency (OPE)

- Post Operating Plant Efficiency

The Southern California Edison (SCE) pump test database was used extensively to obtain average values of the variables listed above. Data for four years were used in estimating the averages [Attachment 2]. As more data becomes available the averages provided in this work paper may need to be revised for accuracy. Where data is missing or limited, average savings values were used to fill in the data.

The data was disaggregated according to the following pump types for each climate zone within SCE’s service territory:

- Submersible Well

- Submersible Booster

- Centrifugal Booster

- Turbine Well

- Turbine Booster

The following table shows a sample breakdown of average values from CZ10 for the less than 25hp measure. Average value calculations for the other SCE climate zones are provided in the Attachments section (Attachment 2 and Attachment 3).

Table 8 Estimation of Variables for CZ 10

| Pump Type | Average Annual Hours of Operation (AOH) | Average Pump Size (Nominal Motor hp) | Average Baseline OPE (%) | Average Proposed OPE (%) | Average Motor Loading |
| --- | --- | --- | --- | --- | --- |
| Centrifugal Booster | 3,204.78 | 13.62 | 41.23 | 58.48 | 85% |
| Submersible Booster | 1,897.56 | 15.49 | 47.78 | 56.87 | 90% |
| Submersible Well | 3,426.63 | 10.80 | 38.85 | 53.51 | 109% |
| Turbine Booster | 2,364.63 | 14.32 | 53.00 | 61.20 | 90% |
| Turbine Well | 3,750.51 | 11.78 | 42.70 | 58.57 | 71% |

Equation 1

kWh Savings=Avg. Pump Size hpx 0.746kWhp x Avg. Motor Loading x Avg. AOH x (1-OPEBaseOPEPost)

Equation 2

kW Savings=kWh SavingsAvg. AOH

Table 9 Average Savings per Pump Type

| Pump Type | kWh Savings (kWh/yr) | Avg. kW Savings |
| --- | --- | --- |
| Centrifugal Booster | 8,164.22 | 2.55 |
| Submersible Booster | 3,154.34 | 1.66 |
| Submersible Well | 8,244.33 | 2.41 |
| Turbine Booster | 3,046.14 | 1.29 |
| Turbine Well | 6,340.66 | 1.69 |

Equation 3

Gross Unit Annual Electricity Savings kWhunit=kWh SavingsAvg. Pump Size

Equation 4

Gross Peak Demand ReductionkWunit=kW Savings xPeak Adjustment FactorAvg. Pump Size

Peak kW Adjustment Factor

The DEER peak-defined period was revised to capture the three-day heat wave within the 16 CEC Title 24 climate zones as defined in the EE Policy Manual (Version 4). Table below exhibits the DEER peak-defined period by climate zone.

Table 10 DEER Peak Period by Climate Zone

|  | Start Date |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| CZ | Month | Day | Weekday | Peak T | Ave T |
| CZ01 | Sep | 16 | Wed | 81 | 59.8 |
| CZ02 | Jul | 8 | Wed | 103 | 75.9 |
| CZ03 | Jul | 8 | Wed | 91 | 69.2 |
| CZ04 | Sep | 1 | Tue | 99 | 77.5 |
| CZ05 | Sep | 8 | Tue | 87 | 64.8 |
| CZ06 | Sep | 1 | Tue | 102 | 77.1 |
| CZ07 | Sep | 1 | Tue | 90 | 73.9 |
| CZ08 | Sep | 1 | Tue | 105 | 79.8 |
| CZ09 | Sep | 1 | Tue | 107 | 86.6 |
| CZ10 | Sep | 1 | Tue | 109 | 86.3 |
| CZ11 | Jul | 8 | Wed | 113 | 88.3 |
| CZ12 | Jul | 8 | Wed | 109 | 82.4 |
| CZ13 | Jul | 8 | Wed | 108 | 86.7 |
| CZ14 | Aug | 26 | Wed | 105 | 86.8 |
| CZ15 | Aug | 25 | Tue | 112 | 97.5 |
| CZ16 | Jul | 8 | Wed | 90 | 78.8 |

In order to confirm if the pumps operate during the peak-defined period, five years of billing data were analyzed for Time of Use (TOU) accounts where SCE tested a water pump. The five-year billing data list one billing period (one summer month) for each of five years – (2009-2013) [Attachment 4]. The data validates the distribution of when the pumps operate during the on-peak period (also called load factor). One billing period per year was selected as with an August read date. If an August read date could not be obtained, then a July read date was used.

The five-year billing data makes up over 16,000 records, which include three different customer types (Agriculture, Irrigation, and Water Supply) and twenty different TOU tariff rates. In order to determine a reasonable peak kW adjustment factor, the following steps were used to arrive at more appropriate data set:

- Remove all billing data yielding a zero “Usage kWh”

- Remove all billing data less than 5 “Max kW” and billing data that yielding a zero “Max kW” usage

- Remove all billing data less than 5 “On Peak kW” and billing data that yielding a zero “On Peak kW” usage

- Remove all billing data yielding a “Bill Period Load Factor” less than 10%. Any load factors less than 10% is assumed to operate infrequently during the month.

- Remove all billing data yielding an “On Peak Load Factor” less than 8.3% (30 min/day run). Any “On Peak Load Factor” operating less than 30 minutes per day is assumed to operate infrequently during the month.

- Remove all billing data yielding a “Max kW to On Peak kW Ratio” less than 3.0. Any “Max kW to On Peak kW Ratio less than 3.0 is assumed to operate infrequently during the month

Where,

Usage kWh = Total kWh usage during the billing period

Max kW = Maximum demand usage value (kW) for that billing period

On Peak kW = Read off of the customer database system

Bill Period Load Factor = (Usage kWh/month) / (Max kW * 24 hrs/day * Billing Days/ month)

On Peak Load Factor = On Peak kWh / (6 peak operating hours * On Peak Days)

On Peak Days = [(Billing days per month) * (5 weekdays / 7 weekdays)]

Max kW to On Peak kW Ratio = Max kW / On Peak kW

Using the steps listed above, the data set indicated that the peak kW adjustment factor for all customer types and tariff rates amounted to 38%.

Sample calculation for a centrifugal booster pump overhaul measure in climate zone 10:

kWh Savings=13.62x 0.746kWhp x 85% x 3204.78 x 1-41.23%58.48%= 8,164.22 kWh/yr

kW Savings=8,164.223204.78=2.55 kW

Gross Unit Annual Electricity Savings kWhunit=8164.2213.62=599.43kWhunit

Gross Peak Demand ReductionkWunit=2.55 x 38%13.62=0.07108kWunit

Table 11 Savings Estimates

| Measure Description | CZ | Gross Unit Annual Electricity Savings (kWh/unit) | Gross Peak Demand Reduction (kW/unit) |
| --- | --- | --- | --- |
| Centrifugal Booster Pump System Overhaul (<25hp) | 10 | 599.43 | 0.07108 |
| Submersible Booster Pump System Overhaul (<25hp) | 10 | 203.64 | 0.04078 |
| Submersible Well Pump System Overhaul (<25hp) | 10 | 763.36 | 0.08465 |
| Turbine Booster Pump System Overhaul (<25hp) | 10 | 212.72 | 0.03418 |
| Turbine Well Pump System Overhaul (<25hp) | 10 | 538.26 | 0.05454 |

# Section 3. Load Shapes

The difference between the base case load shape and the measure load shape would be the most appropriate load shape; however, only end-use profiles are available. Therefore, the closest load shape chosen for this measure is the Ag & Water Pumping load shape. See Table 12 for a list of all Building Types and Load Shapes. See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 12 Building Types and Load Shapes

| Building Type | E3 Alt. Building Type | Load Shape |
| --- | --- | --- |
| Agricultural | Agricultural | Ag & Water Pumping |

# Section 4. Base Case & Measure Costs

## 4.1 Base Case Cost

For this measure category, the base case cost is assumed to be zero as these are discretionary add-ons (REA) to the customers’ existing equipment. Their alternative is to make no changes to their existing system.

## 4.2 Measure Case Cost

For this deemed measure, cost data was obtained from existing program data. The cost information for each climate zone is given in Table 13 below. The cost is normalized based on kWh savings. This is based on past program results for this overhaul measure (Attachment 4).

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

Per the E3, the Gross Measure Cost (GMC) is the cost to install an energy efficient measure. This definition implies two different meanings depending on the install type. In the case of RET and REA, GMC means the full cost of the measure to purchase and install. In the case of ROB and NEW, GMC means the cost premium required to install the energy efficient measure over a less efficient piece of equipment. Being that RET, REA, ROB, and NEW have different definitions, there is a clear distinction between the equations for the various install types.

For RET and REA, GMC is represented by the equation below:

GMC = Measure Equipment Cost + Measure Labor Cost

For this deemed measure, cost data was obtained from existing program data. The cost information for each climate zone is given in the table below. The cost is normalized based on kWh savings. This is based on past program results for this overhaul measure (Attachment 4).

Equation 5

Cost/kWh $kWh=Gross Measure CostkWh

Table 13 2010-2011 Gross Measure Cost per Unit by Climate Zone

| Climate Zone | kWh | Gross Measure cost | Cost/kWh |
| --- | --- | --- | --- |
| 6 | 1,417,180 | $479,596.93 | $0.34 |
| 8 | 1,345,477 | $381,211.60 | $0.28 |
| 9 | 4,230,852 | $1,716,363.21 | $0.41 |
| 10 | 5,155,257 | $5,532,045.87 | $1.07 |
| 13 | 10,573,106 | $6,434,805.90 | $0.61 |
| 14 | 1,642,449 | $1,575,155.34 | $0.96 |
| 15 | 221,504 | $102,601.44 | $0.46 |
| 16 | 328,166 | $352,103.01 | $1.07 |

Using this cost estimate above, per unit cost information was derived using kWh savings and average pump size using the following equations:

Equation 6

Cost of Retrofit $=CostkWhx kWh Savings

Equation 7

Gross Measure Cost $unit=Cost of Retrofit $Average Pump Size hp

For example, for < 25hp Centrifugal Booster Pump System Overhaul in CZ 10,

Cost of Retrofit $=$1.07CostkWhx 8164.22=$8,735.72

Equation 8

Gross Measure Cost $unit=$8,735.7213.62=641.39$unit

The table below provides an example of typical costs involved in overhauling pumps.

Table 14 Gross Measure Cost

| Measure Description | CZ | Average Pump Size (Nominal motor hp) | kWh Savings (kWh/yr) | Cost/kWh | Cost of Retrofit ($) | Gross Measure Cost($/unit) |
| --- | --- | --- | --- | --- | --- | --- |
| Centrifugal Booster Pump System Overhaul (<25hp) | 6 | 14.25 | 8,694.06 | $0.34 | $2,955.98 | $207.44 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 8 | 11.24 | 7,562.48 | $0.28 | $2,117.50 | $188.39 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 9 | 13.08 | 5,143.65 | $0.41 | $2,108.90 | $161.23 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 10 | 13.62 | 8,164.22 | $1.07 | $8,735.72 | $641.39 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 13 | 13.15 | 3,379.06 | $0.61 | $2,061.23 | $156.75 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 14 | 14.22 | 6,148.32 | $0.96 | $5,902.39 | $415.08 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 15 | 13.00 | 8,144.07 | $0.46 | $3,746.27 | $288.17 |
| Centrifugal Booster Pump System Overhaul (<25hp) | 16 | 10.70 | 3,293.90 | $1.07 | $3,524.47 | $329.39 |

## Please see the attachments section for the full list of measure costs for this work paper.

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost (IMC) is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure. While IMC has a straight forward definition, depending on the install type the equation does vary. The incremental cost is only used to help determine program incentives. It is not affected by the first and second baseline periods and may differ from the cost used for cost effectiveness calculations.

However, for this measure, the base case is an existing pump that is not operating efficiently. Therefore, the base case equipment cost is truly $0.00 for this measure. For this measure, the IMC will be equal to the GMC.

# Attachments

*Savings for measures less than 25 hp.

# References

[31]

[124]

[351]

[355]

[422]

[436]

# Appendix A – SCE/ED Application Types

| SCE Program Type | ED Application Type | 1st Baseline Savings | 2nd Baseline Savings | 1st Baseline Cost | 2nd Baseline Cost | 1st Baseline Life | 2nd Baseline Life |
| --- | --- | --- | --- | --- | --- | --- | --- |
| New | New Construction (Nc) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob)/Normal Replacement (NR) | Above Code/Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Cust. Existing | Above Code/Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Cust. Existing | N/A | Full Cost | N/A | EUL | 0 |