Work Paper SCE13WH001

Revision 3

Southern California Edison Company

Heat Pump Water Heater

# At-a-Glance Summary

| Solution and Measure Codes: | WH-18930 |
| --- | --- |
| Measure Description: | Heat Pump Water Heater, 40 gal, EF ≥ 2.0 |
| Base Case Description: | Electric Storage Water Heater, 40 gal, EF = 0.948 |
| Units: | Water Heater |
| Energy Savings: | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit): | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life: | 10 years (EUL_ID: WtrHt-HtPmp) |
| Measure Application Type: | Replace-on-Burnout (ROB) |
| Net-to-Gross Ratio: | 0.55 (NTGR_ID: Res-Default>2) |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/2014 Ex Ante Database Specification provided by the California Public Utilities Commission (CPUC) Commission Staff (CS); SCE will provide that data set separately. |

# Document Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 05/01/2012 | Akane Karasawa (AESC) | Original work paper for 2013 PC |
| 1 | 06/10/2014 | Jay Bhakta (SCE), Jason Wang (SCE) | Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. |
| 2 | 12/18/2014 | Jay Bhakta (SCE), Jason Wang (SCE) | Work paper updated to reflect code changes effective 4/15/15 - Base Case efficiency and Measure Name changed to incorporate new efficiency (EF=0.948) - DEER 2015 Impact Values used for savings |
| 3 | 01/28/2016 | Jay Bhakta (SCE) | -New template update for 2016 program year -WP effective from 1/1/2016 thru 12/31/2016 -No value modifications |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is a 40 gallon heat pump water heater (HPWH) with an energy factor (EF) ≥ 2.0. The base case measure is a 40 gallon electric storage water heater (ESWH) with an EF = 0.948.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | EF ≥ 2.0 40 Gallon Heat Pump Water Heater |
| Existing Condition | 40 Gallon Electric Storage Water Heater, EF = 0.948 |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
| N/A | N/A | WH-18930 | N/A | EF ≥ 2.0 40 Gallon Heat Pump Water Heater replacing 40 Gallon Electric Storage Water Heater, EF = 0.948 |

Measure Requirements

- “Instantaneous” and “tank less” water heaters do not qualify.

- The measure case heat pump water heater must have an EF of 2.0 or greater.

- The water heater storage capacity must be 40 gallons or greater.

- Only residential-style electric storage water heaters qualify for this rebate. A list of qualifying residential water heaters is at: www.sce.com/appliances. Qualifying models are from the Energy Star product list, which requires a minimum EF of 2.0.

Market Applicability

This measure is applicable to single-family, multi-family, and double-wide mobile home residential building types. The most significant barrier to water heater retrofit is the existing nature of water heater replacement. Two-thirds of consumers replace their water heaters due to the sudden failure of their existing water heater. When a water heater suddenly fails, most consumers purchase replacements that are the cheapest and most readily available model that are also easy to install. These prevailing attitudes do not encourage consumers to make the extra effort to find more advanced, energy-efficient technologies that are now available on the market.

## 1.2 Technical Description

Conventional electric-resistance water heaters usually consist of a glass-lined steel tank with foam insulation. Energy efficient units have greater amount of insulation. Located at the base and top end of the tank are two electrical heating elements. Cold water enters the base of the tank and is heated by the lower electrical heating element. The water then rises to the top portion of the tank. This is where the hot water is drawn for consumption. During periods of high demand, the electrical heating element located at the top end of the tank can be turned on to provide additional water heating.

Heat pump water heaters heat water using a heat pump, allowing them to achieve much higher efficiency (energy factor), compared to electric-resistance water heaters. Similar to electric-resistance water heaters, most heat pump water heaters consists of a glass-lined steel tank with foam insulation. Heat pump waters water heaters are typically equipped with supplemental electric-resistance elements for periods of high demand.

## 1.3 Application Types and Delivery Mechanism

The delivery method for the above measure is:

- Financial Support / Down-Stream Incentive –Deemed

The program/install type for the above measure is:

- Replace on Burnout (ROB)

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Down-Stream Incentive | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. Such an incentive may be deemed or customized. |

## 1.4  Measure Parameters

### 1.4.1 DEER Data

This work paper uses data from DEER 2015, which lists data for a HPWH with EF = 2.0 replacing a base case ESWH with EF = 0.948.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | No |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | No |
| DEER Operating Hours | No |
| DEER eQUEST Prototypes | No |
| DEER Version | DEER 2015, READI v2.1.0 |
| Reason for Deviation from DEER | DEER contains this measure but is modified |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The Installation Rate (IR)  values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1.0 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| WtrHt-HtPmp | Heat Pump Water Heater | Res | SHW | 10 | N/A |

### 1.4.2 Codes and Standards Analysis

The measures in this work paper are considered small water heaters. A small water heater can be defined as an ESWH with an input of 12 kW or less, or a HPWH rated at 24 amps or less.  The following codes apply to the measures in this work paper:

Title 20 (2014) [422]

Section 1605.1(f)(2) provides minimum Energy Factor requirements for ESWHs and HPWHs:

Title 24 (2013) [355]

These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards – Code of Federal Regulations [393]

The Code of Federal Regulations, 10 CFR 430.32(d) provides requirements for ESWHs but not HPWHs. Title 20 follows the federal code.

Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | N/A | July 1, 2014 |
| Title 20 (2014) | Section 1605.1(f)(2) | April 16, 2015 |
| Code of Federal Regulations | 10 CFR 430.32(d) | April 16, 2015 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case

## 1.5.1 Non-DEER Study Review

For the residential application of this measure, data from the California Residential Saturation Survey (RASS) [195] indicates that approximately 7.1% of residential dwellings in SCE territory use electricity for domestic water heating.

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

DEER 2015 has a measure which replaces an ESWH (EF = 0.948) with a HPWH (EF = 2.0).  The new code efficiency formula for a 40 gal ESWH was used to determine the base case efficiency:

EF = 0.960-(0.0003xV) where V= volume, gal

EF = 0.960-(0.0003x40) = 0.948

Energy savings are determined by comparing the annual energy usage of a heat pump water heater HPWH (EF = 2.0) with an ESWH (EF = 0.948) base case electric storage water heater.  Savings values were taken directly from DEER 2015 [49] using the READI Tool v2.1.0 and are included Table 8 below.  These results have not been modified and are only being included in the workpaper for reference.

DEER and READi Tool Outputs

| Measure Code | Measure Name | READI Data |
| --- | --- | --- |
| WH-18930 | EF ≥ 2.0 40 Gallon Heat Pump Water Heater replacing 40 Gallon Electric Storage Water Heater, EF = 0.948 |  |

### Since these measures are ROB, the code or “Std” baseline savings from DEER 2015 are used.  The Calculation Template (Attachment 1) shows these savings values.

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | HeatPump_WtrHt-RC | Residential |
| Residential Multi-Family | HeatPump_WtrHt-RC | Residential |
| Residential Mobile Home - Double-Wide | HeatPump_WtrHt-RC | Residential |

# Section 4. Costs

## 4.1 Base Case Cost

Base case costs are from 2010-2012 WO017 [475]: $420.83 for an ESWH with EF = 0.92 with labor costs of $313.12.  The material and labor costs for “Small Electric Storage Water Heater 40 Gal; EF = 0.92; Recov Eff = 0.98” were used instead of costs for a 40 gal ESWH of EF = .948

## 4.2 Measure Case Cost

The measure case costs are also from 2010-2012 WO017 [475]: $1761.23 for an HPWH with EF = 2.0 with labor costs of $467.69.  The material and labor costs for a “Heat pump water heater, 40 gallons, 4.0 kw, 2.0 EF, 240 volt” are used.

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| WH-18930 | ROB | $1,494.88 | $1,494.88 | N/A |

# Attachments

# References

| [49] | DEER Database |
| --- | --- |
| [195] | California Statewide Residential Appliance Saturation Study (RASS) Web site |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [393] | Code of Federal Regulations Title 10 - Energy |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |
| [475] | 2010-2012 WO017 Ex Ante Measure Cost Study Final Report (pgs 88 & 356) |