Work Paper SCE13WH001

Revision 2

Southern California Edison Company

Heat Pump Water Heater

# At-a-Glance Summary

| Solution and Measure Codes: | WH-18930 |
| --- | --- |
| Measure Description: | Heat Pump Water Heater, 40 gal, EF ≥ 2.0 |
| Base Case Description: | Electric Storage Water Heater, 40 gal, EF = 0.948 |
| Units: | Water Heater |
| Energy Savings: | Refer to Excel Calculation Attachment |
| Gross Measure Cost ($/unit): | Refer to Excel Calculation Attachment |
| Measure Incremental Cost ($/unit): | Refer to Excel Calculation Attachment |
| Effective Useful Life: | 10 years (EUL_ID: WtrHt-HtPmp) |
| Measure Application Type: | Replace-on-Burnout (ROB) |
| Net-to-Gross Ratio: | 0.55 (NTGR_ID: Res-Default>2) |
| Important Comments: | This work paper document does not contain a data set in conformance with the 4/1/2014 Ex Ante Database Specification provided by the California Public Utilities Commission (CPUC) Commission Staff (CS); SCE will provide that data set separately. |

# Document Revision History

| Revision # | Revision Date | Author (Affiliation) | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 05/01/2012 | Akane Karasawa (AESC) | Original work paper for 2013 PC |
| 1 | 06/10/2014 | Jay Bhakta (SCE), Jason Wang (SCE) | Work paper updated for the reporting period, effective 7/1/14 – 12/31/14. |
| 2 | 12/18/2014 | Jay Bhakta (SCE), Jason Wang (SCE) | Work paper updated to reflect code changes effective 4/15/15 - Base Case efficiency and Measure Name changed to incorporate new efficiency (EF=0.948) - DEER 2015 Impact Values used for savings |

# Section 1: General Measure & Baseline Data

## 1.1 Measure Description & Background

The measure case is a 40 gallon heat pump water heater (HPWH) with an energy factor (EF) ≥ 2.0. The base case measure is a 40 gallon electric storage water heater (ESWH) with an EF = 0.948.

Table 1 Measure Names

| Solution Code | Measure Name |
| --- | --- |
| WH-18930 | EF ≥ 2.0 40 Gallon Heat Pump Water Heater replacing 40 Gallon Electric Storage Water Heater, EF = 0.948 |

Measure Requirements

- “Instantaneous” and “tank less” water heaters do not qualify.

- The measure case heat pump water heater must have an EF of 2.0 or greater.

- The water heater storage capacity must be 40 gallons or greater.

- Only residential-style electric storage water heaters qualify for this rebate. A list of qualifying residential water heaters is at: www.sce.com/appliances. Qualifying models are from the Energy Star product list, which requires a minimum EF of 2.0.

Market Applicability

This measure is applicable to single-family, multi-family, and double-wide mobile home residential building types. The most significant barrier to water heater retrofit is the existing nature of water heater replacement. Two-thirds of consumers replace their water heaters due to the sudden failure of their existing water heater. When a water heater suddenly fails, most consumers purchase replacements that are the cheapest and most readily available model that are also easy to install. These prevailing attitudes do not encourage consumers to make the extra effort to find more advanced, energy-efficient technologies that are now available on the market.

## 1.2 Technical Description

Conventional electric-resistance water heaters usually consist of a glass-lined steel tank with foam insulation. Energy efficient units have greater amount of insulation. Located at the base and top end of the tank are two electrical heating elements. Cold water enters the base of the tank and is heated by the lower electrical heating element. The water then rises to the top portion of the tank. This is where the hot water is drawn for consumption. During periods of high demand, the electrical heating element located at the top end of the tank can be turned on to provide additional water heating.

Heat pump water heaters heat water using a heat pump, allowing them to achieve much higher efficiency (energy factor), compared to electric-resistance water heaters. Similar to electric-resistance water heaters, most heat pump water heaters consists of a glass-lined steel tank with foam insulation. Heat pump waters water heaters are typically equipped with supplemental electric-resistance elements for periods of high demand.

## 1.3 Application Types and Delivery Mechanism

See Appendices A and B for definitions of application types and delivery mechanisms.

The delivery method for the above measure is:

- Financial Support / Down-Stream Incentive –Deemed

The program/install type for the above measure is:

- Replace on Burnout (ROB)

## 1.4  Measure and Base Case Cost Effectiveness Data

### 1.4.1 DEER Measure and Base Case Analysis

This work paper uses data from DEER 2015, which lists data for a HPWH with EF = 2.0 replacing a base case ESWH with EF = 0.948.

Table 2: DEER Difference Summary

| DEER Difference Summary Table |  |
| --- | --- |
| Referenced versions of DEER and READI | DEER 2015, READI v2.1.0 |
| Summary of deviation from DEER | DEER contains this measure |
| DEER measures scaled? | No – not necessary |
| DEER eQUEST prototypes used? | No |
| DEER operating hours used? | No |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

Table 3: Net-to-Gross Ratio

| NTGR ID | Description | Sector | BldgType | ProgDelivID | NTG |
| --- | --- | --- | --- | --- | --- |
| Res-Default>2 | All other EEM with no evaluated NTGR; existing EEM with same delivery mechanism for more than 2 years | Res | Any | Any | 0.55 |

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

Table 4: Installation Rate

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1.0 |

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Technology Fields

The Technology Fields were obtained from the Ex Ante Database Specification. The relevant Use Category, Use Sub-category, Technology Group, and Technology Type values for the measures in this work paper are in the table below.

Table 5: Technology Fields

| Classification | Value |
| --- | --- |
| Measure Case UseCategory | Service and Domestic Hot Water |
| Measure Case UseSubCats | Water Heating |
| Measure Case TechGroups | Water Heating Equipment |
| Measure Case TechTypes | EF Rated Heat Pump Water Heater |
| Base Case TechGroups | Water Heating Equipment |
| Base Case TechTypes | EF Rated Storage Water Heater |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

Table 6: EUL and RUL

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| WtrHt-HtPmp | Heat Pump Water Heater | Res | Water Heating | 10 | N/A |

### 1.4.2 Codes and Standards Analysis

The measures in this work paper are considered small water heaters. A small water heater can be defined as an ESWH with an input of 12 kW or less, or a HPWH rated at 24 amps or less.  The following codes apply to the measures in this work paper:

Title 20 (2014) [422]

Section 1605.1(f)(2) provides minimum Energy Factor requirements for ESWHs and HPWHs:

Title 24 (2013) [355]

These measures do not fall under Title 24 of the California Energy Regulations.

Federal Standards – Code of Federal Regulations [393]

The Code of Federal Regulations, 10 CFR 430.32(d) provides requirements for ESWHs but not HPWHs. Title 20 follows the federal code.

Table 7: Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Code of Federal Regulations | 10 CFR 430.32(d) | April 16, 2015 |
| Title 20 (2014) | Section 1605.1(f)(2) | April 16, 2015 |

### 1.4.3 Non-DEER Study Review

For the residential application of this measure, data from the California Residential Saturation Survey (RASS) [195] indicates that approximately 7.1% of residential dwellings in SCE territory use electricity for domestic water heating.

# Section 2: Calculation Methodology

DEER 2015 has a measure which replaces an ESWH (EF = 0.948) with a HPWH (EF = 2.0).  The new code efficiency formula for a 40 gal ESWH was used to determine the base case efficiency:

EF = 0.960-(0.0003xV) where V= volume, gal

EF = 0.960-(0.0003x40) = 0.948

Energy savings are determined by comparing the annual energy usage of a heat pump water heater HPWH (EF = 2.0) with an ESWH (EF = 0.948) base case electric storage water heater.  Savings values were taken directly from DEER 2015 using the READI Tool v2.1.0 and are included Table 8 below.  These results have not been modified and are only being included in the workpaper for reference.

Table 8 DEER and READi Tool Outputs

| Solution Code | Measure Name | DEER Outputs |
| --- | --- | --- |
| WH-18930 | EF ≥ 2.0 40 Gallon Heat Pump Water Heater replacing 40 Gallon Electric Storage Water Heater, EF = 0.948 |  |

### Since these measures are ROB, the code or “Std” baseline savings from DEER 2015 are used.  The Calculation Template (Attachment 1) shows these savings values.

# Section 3: Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below. See the KEMA report [31] for a more thorough discussion regarding the load shapes for this measure.

Table 9: Building Types and Load Shapes

| Building Type | Load Shape | E3 Alt. Building Type |
| --- | --- | --- |
| Residential Single Family | HeatPump_WtrHt-RC | Residential |
| Residential Multi-Family | HeatPump_WtrHt-RC | Residential |
| Residential Mobile Home - Double-Wide | HeatPump_WtrHt-RC | Residential |

# Section 4: Base Case & Measure Costs

## 4.1 Base Case Cost

Base case costs are from 2010-2012 WO017 [475]: $420.83 for an ESWH with EF = 0.92 with labor costs of $313.12.  The material and labor costs for “Small Electric Storage Water Heater 40 Gal; EF = 0.92; Recov Eff = 0.98” were used instead of costs for a 40 gal ESWH of EF = .948

## 4.2 Measure Case Cost

The measure case costs are also from 2010-2012 WO017 [475]: $1761.23 for an HPWH with EF = 2.0 with labor costs of $467.69.  The material and labor costs for a “Heat pump water heater, 40 gallons, 4.0 kw, 2.0 EF, 240 volt” are used.

## 4.3 Gross and Incremental Measure Cost

### 4.3.1 Gross Measure Cost

For ROB measures, the Gross Measure Cost (GMC) is:

GMC 	= (Measure Equipment Cost + Measure Labor Cost) –

(Base Case Equipment Cost + Base Case Labor Cost)

= ($1,761.23+$467.68) – ($420.83+313.12)

= $1,494.88

### 4.3.2 Incremental Measure Cost

Incremental Measure Cost is the premium cost to install an energy efficient measure over a standard efficiency measure or code baseline measure.  For ROB, the IMC is the same value as GMC. The incremental cost is only used to help determine program incentives.

# Attachments

# References

| [31] | Load Shape Update Initiative - KEMA / JJ Hirsch and Assoc. / Itron Inc. - November 17, 2006 |
| --- | --- |
| [49] | DEER Database |
| [195] | California Statewide Residential Appliance Saturation Study (RASS) Web site |
| [355] | 2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings (Title 24) |
| [393] | Code of Federal Regulations Title 10 - Energy |
| [422] | 2014 Appliance Efficiency Regulations (Title 20) |
| [436] | 2014 DEER EUL Table - Update |
| [475] | 2010-2012 WO017 Ex Ante Measure Cost Study Final Report (pgs 88 & 356) |

# Appendix A: Application Types

This table compares the application types in SCE’s systems with those in DEER.

| SCE Application (Program) Type | DEER Application Type | Savings |  | Cost |  | Life |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL | 1st BL | 2nd BL |
| New Construction (NEW) | New Construction (Nc) | Above Code or Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Replace on Burnout (ROB) | Replace on Burnout (Rob), Normal Replacement (NR) | Above Code or Standard | N/A | Incremental Cost | N/A | EUL | 0 |
| Retrofit (RET) | Early Replacement (ER) | Above Customer Existing | Above Code or Standard | Full Cost | Incremental Cost | RUL | EUL-RUL |
| Retrofit – First Baseline Only (REF) | Early Replacement RUL (ErRul) | Above Customer Existing | N/A | Full Cost | N/A | EUL | 0 |
| Retrofit Add-on (REA) | N/A | Above Customer Existing | N/A | Full Cost | N/A | EUL | 0 |

# Appendix B: Delivery Mechanisms

A delivery mechanism is a delivery method paired with an incentive method. SCE’s delivery methods include:

- Appliance Turn-in and Recycling

- Audit/Information

- Commissioning

- Financial Support

- Innovative Design

- Midstream Programs

- Partnership

- Upstream Programs

The following table describes the incentive methods.

| Incentive Method | Description |
| --- | --- |
| Direct Install | The utility program performs an assessment of the customer’s facility, provides recommendations, and implements energy efficiency measures for free. |
| Down-Stream Incentive - Deemed | The customer installs qualifying energy efficient equipment and submits an incentive application to the utility program. Upon application approval, the utility program pays an incentive to the customer. |
| Exchange - Replacement | The utility program holds events where customers can trade functional equipment for similar but more energy efficient equipment, free of charge. |
| Giveaway | The utility program provides customers with energy efficient equipment for free. |
| Mid-Stream Incentive | The utility program offers buydowns and incentives to third parties (typically retailers, distributors, and contractors), who then stock, promote, lower prices on, and/or sell energy efficient equipment. Contractors install energy efficiency equipment, sometimes using specified quality procedures, at the customer’s property. |
| On-bill Finance - loan | Customers can finance energy efficiency projects at 0% interest and repay the loan through their monthly utility bill. |
| Testing Services / Other | The utility program performs free testing services or assessments of the customer’s facility and provides information and recommendations for potential energy efficiency measures. |
| Up-Stream Buy Down, Up-Stream Incentive | The utility program offers buydowns and incentives to vendors (typically manufacturers and distributors), who then manufacture, stock, promote, lower prices on, and/or sell energy efficient equipment. There is some overlap between the mid-stream and up-stream approaches. |