Work Paper SCE17LG115

Revision 0

Southern California Edison

Residential LED Interior Fixtures

# At-a-Glance Summary

| Measure Codes | Refer to Excel Calculation Attachment |
| --- | --- |
| Measure Description | Residential LED Interior Fixtures |
| Base Case Description | CFL/Halogen/Incandescent Fixtures using A19 screw-in lamps or integrated modules |
| Units | Per fixture |
| Energy Savings | Refer to Excel Calculation Attachment |
| Full Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Incremental Measure Cost ($/unit) | Refer to Excel Calculation Attachment |
| Effective Useful Life | ILtg-Res-LED-50000hr: 16 years EUL, 5.33 years RUL |
| Measure Installation Type | Replace on Burnout (ROB), Retrofit or Early Retirement (RET/ER) (Pursuant to POE Draft Resolution E-4818) |
| Net-to-Gross Ratio | All-Default<=2yrs: 0.7, Res-Default-HTR-di: 0.85 |
| Important Comments | This work paper has a complementary Ex Ante Database data set that will be provided in a separate submission to the California Public Utilities Commission (CPUC). |

# Revision History

| Rev | Date | Author | Summary of Changes |
| --- | --- | --- | --- |
| 0 | 11/1/16 | Don Ly | - New WP for the 2017 cycle - Modified from SCE13LG115 rev. 1 - Updated Costs for 2017 using PGECOLTG139 Cost Method - Updated WRR per 2016 Lighting Disposition |

# Commission Staff and Cal TF Comments

| Rev | Party | Submittal Date | Comment Date | Comments | WP Developer Response |
| --- | --- | --- | --- | --- | --- |
| 0 | CS |  |  |  |  |
| 0 | Cal TF |  |  |  |  |

Cal TF website: http://www.caltf.org/

# Section 1. General Measure & Baseline Data

## 1.1 Measure Description & Background

This work paper details the replacement of Residential interior CFL/Incandescent/Halogen light fixtures using A19 lamps with interior LED fixtures ranging from 15 to 41 Watts. The list of measures is shown in the tables below.

Base, Standard, and Measure Cases

| Case | Description of Typical Scenario |
| --- | --- |
| Measure | Residential LED interior fixture |
| Existing Condition | Residential incandescent/halogen/CFL interior fixture |
| Code/Standard | N/A |
| Industry Standard Practice | N/A |

For the purpose of this workpaper, the term Residential LED Interior Fixtures is used to describe all the interior fixture products that are used for the Multifamily EE Rebate’s (MFEER) Direct Install Program. The fixtures include Ceiling Mounted Downlights, Vanity, and Wall Sconces. Due to how the Multifamily Direct Install program implements the projects, the three different types of fixtures are reported as a single measure – Residential Interior LED Fixtures with one Watt increments.

Measures and Codes

| Measure Codes |  |  |  | Measure Name |
| --- | --- | --- | --- | --- |
| SCG | SDG&E | SCE | PG&E |  |
|  |  | LT-52657 |  | 15-16 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-88237 |  | 16-17 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-94776 |  | 17-18 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-51718 |  | 18-19 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-67611 |  | 19-20 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-93376 |  | 20-21 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-92639 |  | 21-22 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-58127 |  | 22-23 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-98384 |  | 23-24 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-51055 |  | 24-25 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-67077 |  | 25-26 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-94978 |  | 26-27 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-74554 |  | 27-28 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-72547 |  | 28-29 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-55125 |  | 29-30 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-63872 |  | 30-31 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-54591 |  | 31-32 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-95714 |  | 32-33 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-59066 |  | 33-34 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-69011 |  | 34-35 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-52455 |  | 35-36 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-52123 |  | 36-37 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89839 |  | 37-38 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-89969 |  | 38-39 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-66138 |  | 39-40 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-74352 |  | 40-41 Watt Interior Fixture (Res) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-86433 |  | 15-16 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-63468 |  | 16-17 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-82362 |  | 17-18 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-62400 |  | 18-19 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-85567 |  | 19-20 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-52787 |  | 20-21 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-64406 |  | 21-22 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-52989 |  | 22-23 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-50521 |  | 23-24 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-94646 |  | 24-25 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-62270 |  | 25-26 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-81294 |  | 26-27 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-65604 |  | 27-28 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-75954 |  | 28-29 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-81626 |  | 29-30 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-77888 |  | 30-31 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-55457 |  | 31-32 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-56395 |  | 32-33 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-77022 |  | 33-34 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-78422 |  | 34-35 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-87501 |  | 35-36 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-60264 |  | 36-37 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-91976 |  | 37-38 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-70281 |  | 38-39 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-85365 |  | 39-40 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-72749 |  | 40-41 Watt Interior Fixture (Dwelling Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-91239 |  | 15-16 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-99048 |  | 16-17 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-88901 |  | 17-18 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-69343 |  | 18-19 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-64738 |  | 19-20 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-65272 |  | 20-21 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-63670 |  | 21-22 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-68477 |  | 22-23 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-54057 |  | 23-24 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-97316 |  | 24-25 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-54259 |  | 25-26 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-83431 |  | 26-27 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-59195 |  | 27-28 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-95180 |  | 28-29 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-64204 |  | 29-30 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-66875 |  | 30-31 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-58864 |  | 31-32 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-82897 |  | 32-33 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-90373 |  | 33-34 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-75088 |  | 34-35 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-70945 |  | 35-36 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-59932 |  | 36-37 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-79692 |  | 37-38 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-70411 |  | 38-39 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-61534 |  | 39-40 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |
|  |  | LT-74886 |  | 40-41 Watt Interior Fixture (Common Area) LEDreplacingBase Case, Total Watts = 2.96 x Msr Watts |

To qualify for incentives, the residential LED interior fixture must be on the Energy Star Certified Light Fixtures Qualified Products List (QPL) [A]. Any fixtures requiring the use of A19 lamps must use LEDs and also be on the Energy Star’s QPL. Energy Star fixtures using A19 lamps must have GU-24 sockets.

The MFEER program quality control ensures correct documentation of existing measure, base case, and specification of the energy efficient product installed.  SCE requires a detailed Product Location Form (PLF) for each project submitted for rebate or incentive.  The PLF is an Excel spreadsheet with a tab for measures installed in dwelling areas and another for measures installed in common area locations.  The PFL contains the following fields: Apartment Address, Measure Installed, Product Make/Model, Install Locations (detailed to Kitchen, Bathroom, Hallway, Living Room, Bedroom, Dining Room, Laundry Room, and Porch/Patio), as well as Common Area locations.  This information is entered and tracked in the program’s tracking database.  This level of data is provided in the Participation Data that is provided to the CPUC on a quarterly basis.

This measure applies to all Residential building types including Single Family, Multi-family, and Mobile Home – Double-Wide in all SCE Climate Zones.

## 1.2 Technical Description

Ceiling Mounted Downlight Fixtures are mounted on the surface of the ceiling as opposed to the recessed downlights, in which the fixture installs flush to the ceiling. Vanity light fixtures are used in bathrooms to illuminate the area while combing, brushing, make-up, etc.  They are mounted over the mirror to reduce shadows that would otherwise come from a ceiling mounted fixture. Sconce light fixtures are installed along the walls used to provide illumination in common areas and hallways.

All three LED fixtures described typically uses two to four A19 lamps with a GU-24 socket or contains integrated LED modules instead of the A19 GU-24 lamp. These are hard wired fixtures that need estimated 30 minutes to replace.

## 1.3 Installation Types and Delivery Mechanisms

The delivery method that is available for these measures is:

- Financial Support – Direct Install

The program/install type for the above measures is:

- Retrofit or Early Retirement (RET/ER)

- Replace on Burnout (ROB)

Installation Type Descriptions

| Installation Type | Savings |  | Life |  |
| --- | --- | --- | --- | --- |
|  | 1st Baseline (BL) | 2nd BL | 1st BL | 2nd BL |
| Replace on Burnout (ROB) | Above Code or Standard | N/A | EUL | N/A |
| Retrofit or Early Replacement (RET/ER) | Above Customer Existing | Above Code or Standard | RUL | EUL-RUL |

A delivery mechanism is a delivery method paired with an incentive method. Delivery mechanisms are used by programs to obtain program participation and energy savings. For the RET install type the programs will follow the Preponderance of Evidence (POE) document requirements.

The California Public Utilities Commission is currently evaluating the documentation required to satisfy the Preponderance of Evidence (POE) requirement for measures with RET (to be renamed AR going forward) install type. Draft Resolution E-4818 is currently out for public review and includes specific language for deemed measures in Section 1.5.4. Upon adoption of the Resolution, SCE will modify the data collection activities described above, as needed, to be consistent with the final Resolution requirements.

Delivery Method Descriptions

| Delivery Method | Description |
| --- | --- |
| Financial Support | The program motivates customers, through financial incentives such as rebates or low interest loans, to implement energy efficient measures or projects. |

Incentive Method Descriptions

| Incentive Method | Description |
| --- | --- |
| Direct Install | The program implements energy efficiency measures for qualifying customers, at no cost to the customer. |

## 1.4 Measure Parameters

### 1.4.1 DEER Data

Although the fixture savings in this work paper are based on A19 lamps using WRR method, the LED A19 lamps listed in the READI tool do not provide 1 Watt increments up to the wattage range needed and also does not provide the proper EUL for a lighting fixture.

DEER Difference Summary

| DEER Item | Used for Workpaper? |
| --- | --- |
| Modified DEER methodology | No |
| Scaled DEER measure | Yes |
| DEER Base Case | Yes |
| DEER Measure Case | Yes |
| DEER Building Types | Yes |
| DEER Operating Hours | Yes |
| DEER eQUEST Prototypes | No |
| DEER Version | Lighting Disposition |
| Reason for Deviation from DEER | Took values from Lighting Disposition |
| DEER Measure IDs Used | N/A |

Net-to-Gross Ratio

The NTG values were obtained using the DEER READI tool. The relevant NTG values for the measures in this work paper are in the table below.

| NTGR ID | Description | Sector | BldgType | Measure Delivery | NTGR |
| --- | --- | --- | --- | --- | --- |
| All-Default<=2yrs | All other EEM with no evaluated NTGR; new technology in program for 2 or fewer years | All | Any | Any | 0.7 |
| Res-Default-HTR-di | All other EEM with no evaluated NTGR; direct install hard-to-reach only. | Res | Any | DirInstall | 0.85 |

Note: Direct install measures that are not hard-to-reach will use the default NTG value.

This work paper includes measures that are offered via direct install activities into hard-to-reach (HTR) customer homes.  “Final Resolution E-4700”, dated December 18, 2014, defines specific criteria to classify customer homes as HTR. The “Required Corrections to Measure Level Input Parameters Identified by Commission Staff per D.14-10-046 Order Paragraph 16”, dated November 3, 2014, includes additional clarification for the geographic criteria.

SCE’s Multi-Family Energy Efficiency Rebate (MFEER) program addresses the ongoing concern with “split incentives”, where the residents are not the owners of the property, so they lack incentive to improve their energy usage. Similarly, the property owners do not live on-site and pay higher utility expenses due to inefficient appliances, thus lack any incentive to upgrade. The MFEER is designed to drive this customer segment toward participation by offering property owners a variety of energy efficiency measures and services.  The MFEER program will offer and track measure installations in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums.  Measures offered via direct install activities in both common and dwelling areas of multifamily complexes and common areas of mobile home parks and condominiums will receive the HTR NTG.  Other measures in the MFEER program will receive default NTG (NTGR_ID: Res-Default>2), unless otherwise specified in DEER.

Spillage Rate

Spillage rates are not tracked in work papers; they are tracked in an external document which will be supplied to the Commission Staff.

Installation Rate

The IR values were obtained using the DEER READI tool. The relevant IR values for the measures in this work paper are in the table below.

| GSIA ID | Description | Sector | BldgType | ProgDelivID | GSIAValue |
| --- | --- | --- | --- | --- | --- |
| Def-GSIA | Default GSIA values | Any | Any | Any | 1 |

Effective and Remaining Useful Life

The EUL and RUL values were obtained using the DEER READI tool. DEER defines the RUL as 1/3 of the EUL value. The RUL value is only applicable to the first baseline period for an RET measure with an applicable code baseline. The relevant EUL and RUL values for the measures in this work paper are in the table below.

| EUL ID | Description | Sector | UseCategory | EUL (Years) | RUL (Years) |
| --- | --- | --- | --- | --- | --- |
| ILtg-Res-LED-50000hr | LED Fixture – Indoor – Residential | Res | Lighting | 16 | 5.33 |

### 1.4.2 Codes and Standards Analysis

Title 20 2015 [493], Section 1605.1(k) – Lamps section lists the minimum requirements for incandescent reflector lamps, medium base compact fluorescent lamps, and general service incandescent lamps which affect the measures in this work paper.

Energy Star: Energy Star issued a document, ENERGY STAR Program Requirements for Integral LED Lamps – Version 2.0 [510] which details the Energy Star requirements for Integral LED Lamps. The requirements in the document consist of two parts; general requirement for all LED lamps and lamp type specific requirement. Those requirements are the minimum requirements for an integral LED lamp to be adopted by any SCE energy efficiency program.

Title 24 2016 [496] does not affect fixture replacements in Residential building types.

Although code exists, this work paper uses recommended wattage reduction ratios from the Energy Division’s Workpaper Disposition for Lighting Retrofits (22 July2016 [D]) to calculate the base case wattage. Therefore, Title 20 code is not used. Also the disposition uses the code to update the second baseline WRR value from 2.96 to 2.42 to reflect that Title 24 Section 150 already requires high efficacy fixtures in most cases.

Code Summary

| Code | Reference | Effective Dates |
| --- | --- | --- |
| Title 20 (2015) | Section 1605.1(k) | January 1, 2015 |
| Energy Star | Version 2.0 | January 2, 2017 |

## 1.5 EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

N/A

## 1.6 Data Quality and Future Data Needs

N/A

# Section 2. Calculation Methodology

The methodology of the wattages to be used for energy savings is based on the LED wattage being offered and the Wattage Reduction Ratio (WRR). Since the baseline and proposed fixtures use A19 lamps, the WRRs provided by the Energy Division disposition [Attachment 3] will be used to calculate the baseline wattages. The previous disposition utilized a WRR of 2.96, which will be used as the first baseline and WRR of 2.42 in the latest disposition [Attachment 3] will be used as the 2nd baseline. For RET/ER retrofit calculations see below.

The 1st baseline energy estimates with WRR of 2.96 are calculated as follows:

The following is a sample energy savings calculation for a 15 Watt LED Interior Fixture in Multi-Family (Common Area) building type, Climate Zone 6.

EnergySavingskWhUnit×Year=[15×2.96-15]×6142×1.031,000WatthourskWh

EnergySavingskWhUnit×Year=185.99

The demand reduction estimates are calculated as follows:

The following is a sample demand reduction calculation for a 15 Watt LED Interior Fixture in Multi-Family (Common Area) building type, Climate Zone 6.

Demand ReductionkWUnit=[15×2.96-15]×.043×1.171,000WatthourskWh

Demand ReductionkWUnit=0.001479

The 2nd baseline energy estimates with WRR of 2.42 are calculated as follows:

The following is a sample energy savings calculation for a 15 Watt LED Interior Fixture in Multi-Family (Common Area) building type, Climate Zone 6. For the Common Area scenario READi v2.4.7 was referenced for the Hours of Usage value of 6142 when replacing a CFL light source in interior settings.

EnergySavingskWhUnit×Year=[15×2.42-15]×6142×1.031,000WatthourskWh

EnergySavingskWhUnit×Year=134.75

The demand reduction estimates are calculated as follows:

The following is a sample demand reduction calculation for a 15 Watt LED Interior Fixture in Multi-Family (Common Area) building type, Climate Zone 6.

Demand ReductionkWUnit=[15×2.42-15]×.043×1.171,000WatthourskWh

Demand ReductionkWUnit=0.001071

For ROB scenarios, the 2nd baseline will be considered as code to measure savings calculations so energy savings calculations will be as is the 2nd baseline shown above. The only difference between RET/ER and ROB is that in an ROB scenario the EUL is equal to the first baseline life. We will be considering the full EUL value as the life for this measure.

A complete list of energy savings and demand reduction for other measures in different building types and climate zones can be found in the attachment [B].

# Section 3. Load Shapes

The ideal load shape for net benefits estimates would represent the difference between the base case and measure case. The closest load shapes that are applicable to the measures in this work paper are listed in the table below.

Building Types and Load Shapes

| Building Type | Load Shape | E3 Alternate Building Type |
| --- | --- | --- |
| Residential Single Family | DEER:Indoor_CFL_Ltg | RES |
| Residential Multi-family | DEER:Indoor_CFL_Ltg | RES |
| Residential Mobile Home - Double-Wide | DEER:Indoor_CFL_Ltg | RES |

# Section 4. Costs

For Direct Install measures, SCE directly utilizes one or more contractors as part of the program. The actual cost can vary by contractor, the date in which the work occurred, and by the volume of business. Contractor costs are confidential information and are based upon contractually agreed upon pricing as established in their purchase order with SCE; therefore, the SCE program tracking system is the only source for this data.

## 4.1 Base Case Cost

The base case costs are split into 60% CFL and 40% non CFL, as per Navigant LED Study. The technique of web scraping (aka web harvesting, web crawling, web data extraction) was used to gather pricing information from the Home Depot website for base case costs.  The methodology used for measure costs applies to base case costs.  See Section 4.2 for the methodology.  The base case costs are reduced by 30% as suggested by the Navigant LED Study [Attachment 4].  The 30% reduction factor is “to account for the difference between online and typical purchase price” (page 1-3). All cost calculations can be found in Attachment 2. A labor cost of $36.13 is used based on WO017.  It is estimated a half hour for installation, using the $72.26 labor rate from WO017 [475].

Base Case Costs

| Measure | Base Equipment Cost | Base Labor Cost | Total Base Cost |
| --- | --- | --- | --- |
| LT-52657 | $47.10 | $36.13 | $83.23 |
| LT-86433 | $47.10 | $36.13 | $83.23 |
| LT-91239 | $47.10 | $36.13 | $83.23 |
| LT-99048 | $48.31 | $36.13 | $84.44 |
| LT-88237 | $48.31 | $36.13 | $84.44 |
| LT-63468 | $48.31 | $36.13 | $84.44 |
| LT-88901 | $49.52 | $36.13 | $85.65 |
| LT-94776 | $49.52 | $36.13 | $85.65 |
| LT-82362 | $49.52 | $36.13 | $85.65 |
| LT-69343 | $50.74 | $36.13 | $86.87 |
| LT-51718 | $50.74 | $36.13 | $86.87 |
| LT-62400 | $50.74 | $36.13 | $86.87 |
| LT-64738 | $51.95 | $36.13 | $88.08 |
| LT-67611 | $51.95 | $36.13 | $88.08 |
| LT-85567 | $51.95 | $36.13 | $88.08 |
| LT-65272 | $53.17 | $36.13 | $89.30 |
| LT-93376 | $53.17 | $36.13 | $89.30 |
| LT-52787 | $53.17 | $36.13 | $89.30 |
| LT-63670 | $54.38 | $36.13 | $90.51 |
| LT-92639 | $54.38 | $36.13 | $90.51 |
| LT-64406 | $54.38 | $36.13 | $90.51 |
| LT-68477 | $55.59 | $36.13 | $91.72 |
| LT-58127 | $55.59 | $36.13 | $91.72 |
| LT-52989 | $55.59 | $36.13 | $91.72 |
| LT-54057 | $56.81 | $36.13 | $92.94 |
| LT-98384 | $56.81 | $36.13 | $92.94 |
| LT-50521 | $56.81 | $36.13 | $92.94 |
| LT-97316 | $58.02 | $36.13 | $94.15 |
| LT-51055 | $58.02 | $36.13 | $94.15 |
| LT-94646 | $58.02 | $36.13 | $94.15 |
| LT-54259 | $59.24 | $36.13 | $95.37 |
| LT-67077 | $59.24 | $36.13 | $95.37 |
| LT-62270 | $59.24 | $36.13 | $95.37 |
| LT-83431 | $60.45 | $36.13 | $96.58 |
| LT-94978 | $60.45 | $36.13 | $96.58 |
| LT-81294 | $60.45 | $36.13 | $96.58 |
| LT-59195 | $61.66 | $36.13 | $97.79 |
| LT-74554 | $61.66 | $36.13 | $97.79 |
| LT-65604 | $61.66 | $36.13 | $97.79 |
| LT-95180 | $62.88 | $36.13 | $99.01 |
| LT-72547 | $62.88 | $36.13 | $99.01 |
| LT-75954 | $62.88 | $36.13 | $99.01 |
| LT-64204 | $64.09 | $36.13 | $100.22 |
| LT-55125 | $64.09 | $36.13 | $100.22 |
| LT-81626 | $64.09 | $36.13 | $100.22 |
| LT-66875 | $65.30 | $36.13 | $101.43 |
| LT-63872 | $65.30 | $36.13 | $101.43 |
| LT-77888 | $65.30 | $36.13 | $101.43 |
| LT-58864 | $66.52 | $36.13 | $102.65 |
| LT-54591 | $66.52 | $36.13 | $102.65 |
| LT-55457 | $66.52 | $36.13 | $102.65 |
| LT-82897 | $67.73 | $36.13 | $103.86 |
| LT-95714 | $67.73 | $36.13 | $103.86 |
| LT-56395 | $67.73 | $36.13 | $103.86 |
| LT-90373 | $68.95 | $36.13 | $105.08 |
| LT-59066 | $68.95 | $36.13 | $105.08 |
| LT-77022 | $68.95 | $36.13 | $105.08 |
| LT-75088 | $70.16 | $36.13 | $106.29 |
| LT-69011 | $70.16 | $36.13 | $106.29 |
| LT-78422 | $70.16 | $36.13 | $106.29 |
| LT-70945 | $71.37 | $36.13 | $107.50 |
| LT-52455 | $71.37 | $36.13 | $107.50 |
| LT-87501 | $71.37 | $36.13 | $107.50 |
| LT-59932 | $72.59 | $36.13 | $108.72 |
| LT-52123 | $72.59 | $36.13 | $108.72 |
| LT-60264 | $72.59 | $36.13 | $108.72 |
| LT-79692 | $73.80 | $36.13 | $109.93 |
| LT-89839 | $73.80 | $36.13 | $109.93 |
| LT-91976 | $73.80 | $36.13 | $109.93 |
| LT-70411 | $75.02 | $36.13 | $111.15 |
| LT-89969 | $75.02 | $36.13 | $111.15 |
| LT-70281 | $75.02 | $36.13 | $111.15 |
| LT-61534 | $76.23 | $36.13 | $112.36 |
| LT-66138 | $76.23 | $36.13 | $112.36 |
| LT-85365 | $76.23 | $36.13 | $112.36 |
| LT-74886 | $77.44 | $36.13 | $113.57 |
| LT-74352 | $77.44 | $36.13 | $113.57 |
| LT-72749 | $77.44 | $36.13 | $113.57 |

## 4.2 Measure Case Cost

The technique of web scraping was used to gather pricing information from the Home Depot website for measure case costs.  First, a small sample of products was examined between different online retailers to determine the need to include items from various retailers and the discrepancy between pricing.  Please refer to the Competitive Pricing tab in the cost spreadsheet.  Due to the competitive pricing of the same fixture from different retailers, only Home Depot data was examined in detail.

A manual process of examining reasonable cost was conducted by viewing the scatterplot of all costs and its associated rated wattages and categorizing the items into a high, medium, or low cost bin.  Note that in some cases where enough data was scraped, only Energy Star lamps and fixtures were considered in the measure case and CA Title 20 compliant lamps and fixtures were considered in the base case.

Item descriptions were also viewed to understand the reasoning of such high costs.  It was almost always found that items with high costs were associated with architectural features and/or specialty finishes.  As a result, items that fell into the high cost category was not used in the calculations of cost for the work papers because it does not appropriately reflect the approach most consumers would take to implement energy efficiency projects.  Refer to the cost spreadsheet for detailed information.  Furthermore, the latest EM&V Study from Navigant for LED costs uses the 25th percentile for the median price.

Using the low and medium cost data from Home Depot, the best-fit line or linear regression was used to determine the association between fixture wattages and cost.  Please see the cost spreadsheet for the specific linear regression equation generated for the low cost and medium cost.  Raw data points are also included in the spreadsheet.

For work paper purposes, the costs are an equal representation of the medium and low cost categories.  Therefore, the best representative association is the average of the trend line for medium cost and the trend line of the low cost.  This process is not the same as a linear regression determined from the low and medium cost items combined.  Due to the quantity in the data sampling, the items associated with the low or medium cost would influence the linear regression.  For this reason, the best representative cost comes from the average of the linear regression from the medium cost and the linear regression from the cost.  This is how cost is propagated for all the technology categories.

As with base case costs, the measure costs are also reduced by 30% to account for the bulk wholesale pricing. See Attachment 2 for more details.

Measure Case Costs

| Measure | Measure Equipment Cost | Measure Labor Cost | Total Measure Cost |
| --- | --- | --- | --- |
| LT-52657 | $59.26 | $36.13 | $95.39 |
| LT-86433 | $59.26 | $36.13 | $95.39 |
| LT-91239 | $59.26 | $36.13 | $95.39 |
| LT-99048 | $60.09 | $36.13 | $96.22 |
| LT-88237 | $60.09 | $36.13 | $96.22 |
| LT-63468 | $60.09 | $36.13 | $96.22 |
| LT-88901 | $60.91 | $36.13 | $97.04 |
| LT-94776 | $60.91 | $36.13 | $97.04 |
| LT-82362 | $60.91 | $36.13 | $97.04 |
| LT-69343 | $61.73 | $36.13 | $97.86 |
| LT-51718 | $61.73 | $36.13 | $97.86 |
| LT-62400 | $61.73 | $36.13 | $97.86 |
| LT-64738 | $62.55 | $36.13 | $98.68 |
| LT-67611 | $62.55 | $36.13 | $98.68 |
| LT-85567 | $62.55 | $36.13 | $98.68 |
| LT-65272 | $63.37 | $36.13 | $99.50 |
| LT-93376 | $63.37 | $36.13 | $99.50 |
| LT-52787 | $63.37 | $36.13 | $99.50 |
| LT-63670 | $64.19 | $36.13 | $100.32 |
| LT-92639 | $64.19 | $36.13 | $100.32 |
| LT-64406 | $64.19 | $36.13 | $100.32 |
| LT-68477 | $65.01 | $36.13 | $101.14 |
| LT-58127 | $65.01 | $36.13 | $101.14 |
| LT-52989 | $65.01 | $36.13 | $101.14 |
| LT-54057 | $65.84 | $36.13 | $101.97 |
| LT-98384 | $65.84 | $36.13 | $101.97 |
| LT-50521 | $65.84 | $36.13 | $101.97 |
| LT-97316 | $66.66 | $36.13 | $102.79 |
| LT-51055 | $66.66 | $36.13 | $102.79 |
| LT-94646 | $66.66 | $36.13 | $102.79 |
| LT-54259 | $67.48 | $36.13 | $103.61 |
| LT-67077 | $67.48 | $36.13 | $103.61 |
| LT-62270 | $67.48 | $36.13 | $103.61 |
| LT-83431 | $68.30 | $36.13 | $104.43 |
| LT-94978 | $68.30 | $36.13 | $104.43 |
| LT-81294 | $68.30 | $36.13 | $104.43 |
| LT-59195 | $69.12 | $36.13 | $105.25 |
| LT-74554 | $69.12 | $36.13 | $105.25 |
| LT-65604 | $69.12 | $36.13 | $105.25 |
| LT-95180 | $69.94 | $36.13 | $106.07 |
| LT-72547 | $69.94 | $36.13 | $106.07 |
| LT-75954 | $69.94 | $36.13 | $106.07 |
| LT-64204 | $70.76 | $36.13 | $106.89 |
| LT-55125 | $70.76 | $36.13 | $106.89 |
| LT-81626 | $70.76 | $36.13 | $106.89 |
| LT-66875 | $71.58 | $36.13 | $107.71 |
| LT-63872 | $71.58 | $36.13 | $107.71 |
| LT-77888 | $71.58 | $36.13 | $107.71 |
| LT-58864 | $72.41 | $36.13 | $108.54 |
| LT-54591 | $72.41 | $36.13 | $108.54 |
| LT-55457 | $72.41 | $36.13 | $108.54 |
| LT-82897 | $73.23 | $36.13 | $109.36 |
| LT-95714 | $73.23 | $36.13 | $109.36 |
| LT-56395 | $73.23 | $36.13 | $109.36 |
| LT-90373 | $74.05 | $36.13 | $110.18 |
| LT-59066 | $74.05 | $36.13 | $110.18 |
| LT-77022 | $74.05 | $36.13 | $110.18 |
| LT-75088 | $74.87 | $36.13 | $111.00 |
| LT-69011 | $74.87 | $36.13 | $111.00 |
| LT-78422 | $74.87 | $36.13 | $111.00 |
| LT-70945 | $75.69 | $36.13 | $111.82 |
| LT-52455 | $75.69 | $36.13 | $111.82 |
| LT-87501 | $75.69 | $36.13 | $111.82 |
| LT-59932 | $76.51 | $36.13 | $112.64 |
| LT-52123 | $76.51 | $36.13 | $112.64 |
| LT-60264 | $76.51 | $36.13 | $112.64 |
| LT-79692 | $77.33 | $36.13 | $113.46 |
| LT-89839 | $77.33 | $36.13 | $113.46 |
| LT-91976 | $77.33 | $36.13 | $113.46 |
| LT-70411 | $78.16 | $36.13 | $114.29 |
| LT-89969 | $78.16 | $36.13 | $114.29 |
| LT-70281 | $78.16 | $36.13 | $114.29 |
| LT-61534 | $78.98 | $36.13 | $115.11 |
| LT-66138 | $78.98 | $36.13 | $115.11 |
| LT-85365 | $78.98 | $36.13 | $115.11 |
| LT-74886 | $79.80 | $36.13 | $115.93 |
| LT-74352 | $79.80 | $36.13 | $115.93 |
| LT-72749 | $79.80 | $36.13 | $115.93 |

## 4.3 Full and Incremental Measure Cost

Full and Incremental Measure Cost Equations

| Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- |
|  |  | 1st Baseline | 2nd Baseline |
| ROB | (MEC + MLC) – (BEC + BLC) | (MEC + MLC) – (BEC + BLC) | N/A |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs – ROB Install Type

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-52657 | ROB | $12.17 | $12.17 | N/A |
| LT-86433 | ROB | $12.17 | $12.17 | N/A |
| LT-91239 | ROB | $12.17 | $12.17 | N/A |
| LT-99048 | ROB | $11.78 | $11.78 | N/A |
| LT-88237 | ROB | $11.78 | $11.78 | N/A |
| LT-63468 | ROB | $11.78 | $11.78 | N/A |
| LT-88901 | ROB | $11.38 | $11.38 | N/A |
| LT-94776 | ROB | $11.38 | $11.38 | N/A |
| LT-82362 | ROB | $11.38 | $11.38 | N/A |
| LT-69343 | ROB | $10.99 | $10.99 | N/A |
| LT-51718 | ROB | $10.99 | $10.99 | N/A |
| LT-62400 | ROB | $10.99 | $10.99 | N/A |
| LT-64738 | ROB | $10.60 | $10.60 | N/A |
| LT-67611 | ROB | $10.60 | $10.60 | N/A |
| LT-85567 | ROB | $10.60 | $10.60 | N/A |
| LT-65272 | ROB | $10.21 | $10.21 | N/A |
| LT-93376 | ROB | $10.21 | $10.21 | N/A |
| LT-52787 | ROB | $10.21 | $10.21 | N/A |
| LT-63670 | ROB | $9.81 | $9.81 | N/A |
| LT-92639 | ROB | $9.81 | $9.81 | N/A |
| LT-64406 | ROB | $9.81 | $9.81 | N/A |
| LT-68477 | ROB | $9.42 | $9.42 | N/A |
| LT-58127 | ROB | $9.42 | $9.42 | N/A |
| LT-52989 | ROB | $9.42 | $9.42 | N/A |
| LT-54057 | ROB | $9.03 | $9.03 | N/A |
| LT-98384 | ROB | $9.03 | $9.03 | N/A |
| LT-50521 | ROB | $9.03 | $9.03 | N/A |
| LT-97316 | ROB | $8.64 | $8.64 | N/A |
| LT-51055 | ROB | $8.64 | $8.64 | N/A |
| LT-94646 | ROB | $8.64 | $8.64 | N/A |
| LT-54259 | ROB | $8.24 | $8.24 | N/A |
| LT-67077 | ROB | $8.24 | $8.24 | N/A |
| LT-62270 | ROB | $8.24 | $8.24 | N/A |
| LT-83431 | ROB | $7.85 | $7.85 | N/A |
| LT-94978 | ROB | $7.85 | $7.85 | N/A |
| LT-81294 | ROB | $7.85 | $7.85 | N/A |
| LT-59195 | ROB | $7.46 | $7.46 | N/A |
| LT-74554 | ROB | $7.46 | $7.46 | N/A |
| LT-65604 | ROB | $7.46 | $7.46 | N/A |
| LT-95180 | ROB | $7.06 | $7.06 | N/A |
| LT-72547 | ROB | $7.06 | $7.06 | N/A |
| LT-75954 | ROB | $7.06 | $7.06 | N/A |
| LT-64204 | ROB | $6.67 | $6.67 | N/A |
| LT-55125 | ROB | $6.67 | $6.67 | N/A |
| LT-81626 | ROB | $6.67 | $6.67 | N/A |
| LT-66875 | ROB | $6.28 | $6.28 | N/A |
| LT-63872 | ROB | $6.28 | $6.28 | N/A |
| LT-77888 | ROB | $6.28 | $6.28 | N/A |
| LT-58864 | ROB | $5.89 | $5.89 | N/A |
| LT-54591 | ROB | $5.89 | $5.89 | N/A |
| LT-55457 | ROB | $5.89 | $5.89 | N/A |
| LT-82897 | ROB | $5.49 | $5.49 | N/A |
| LT-95714 | ROB | $5.49 | $5.49 | N/A |
| LT-56395 | ROB | $5.49 | $5.49 | N/A |
| LT-90373 | ROB | $5.10 | $5.10 | N/A |
| LT-59066 | ROB | $5.10 | $5.10 | N/A |
| LT-77022 | ROB | $5.10 | $5.10 | N/A |
| LT-75088 | ROB | $4.71 | $4.71 | N/A |
| LT-69011 | ROB | $4.71 | $4.71 | N/A |
| LT-78422 | ROB | $4.71 | $4.71 | N/A |
| LT-70945 | ROB | $4.32 | $4.32 | N/A |
| LT-52455 | ROB | $4.32 | $4.32 | N/A |
| LT-87501 | ROB | $4.32 | $4.32 | N/A |
| LT-59932 | ROB | $3.92 | $3.92 | N/A |
| LT-52123 | ROB | $3.92 | $3.92 | N/A |
| LT-60264 | ROB | $3.92 | $3.92 | N/A |
| LT-79692 | ROB | $3.53 | $3.53 | N/A |
| LT-89839 | ROB | $3.53 | $3.53 | N/A |
| LT-91976 | ROB | $3.53 | $3.53 | N/A |
| LT-70411 | ROB | $3.14 | $3.14 | N/A |
| LT-89969 | ROB | $3.14 | $3.14 | N/A |
| LT-70281 | ROB | $3.14 | $3.14 | N/A |
| LT-61534 | ROB | $2.75 | $2.75 | N/A |
| LT-66138 | ROB | $2.75 | $2.75 | N/A |
| LT-85365 | ROB | $2.75 | $2.75 | N/A |
| LT-74886 | ROB | $2.35 | $2.35 | N/A |
| LT-74352 | ROB | $2.35 | $2.35 | N/A |
| LT-72749 | ROB | $2.35 | $2.35 | N/A |

| Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- |
|  | 1st Baseline | 2nd Baseline |
| (MEC + MLC) – (BEC + BLC) | MEC + MLC | (MEC + MLC) – (BEC + BLC) |

MEC = Measure Equipment Cost; MLC = Measure Labor Cost

BEC = Base Case Equipment Cost; BLC = Base Case Labor Cost

Full and Incremental Costs – RET/ER Install Type

| Measure | Installation Type | Incremental Measure Cost | Full Measure Cost |  |
| --- | --- | --- | --- | --- |
|  |  |  | 1st Baseline | 2nd Baseline |
| LT-52657 | RET/ER | $12.17 | $95.39 | $12.17 |
| LT-86433 | RET/ER | $12.17 | $95.39 | $12.17 |
| LT-91239 | RET/ER | $12.17 | $95.39 | $12.17 |
| LT-99048 | RET/ER | $11.78 | $96.22 | $11.78 |
| LT-88237 | RET/ER | $11.78 | $96.22 | $11.78 |
| LT-63468 | RET/ER | $11.78 | $96.22 | $11.78 |
| LT-88901 | RET/ER | $11.38 | $97.04 | $11.38 |
| LT-94776 | RET/ER | $11.38 | $97.04 | $11.38 |
| LT-82362 | RET/ER | $11.38 | $97.04 | $11.38 |
| LT-69343 | RET/ER | $10.99 | $97.86 | $10.99 |
| LT-51718 | RET/ER | $10.99 | $97.86 | $10.99 |
| LT-62400 | RET/ER | $10.99 | $97.86 | $10.99 |
| LT-64738 | RET/ER | $10.60 | $98.68 | $10.60 |
| LT-67611 | RET/ER | $10.60 | $98.68 | $10.60 |
| LT-85567 | RET/ER | $10.60 | $98.68 | $10.60 |
| LT-65272 | RET/ER | $10.21 | $99.50 | $10.21 |
| LT-93376 | RET/ER | $10.21 | $99.50 | $10.21 |
| LT-52787 | RET/ER | $10.21 | $99.50 | $10.21 |
| LT-63670 | RET/ER | $9.81 | $100.32 | $9.81 |
| LT-92639 | RET/ER | $9.81 | $100.32 | $9.81 |
| LT-64406 | RET/ER | $9.81 | $100.32 | $9.81 |
| LT-68477 | RET/ER | $9.42 | $101.14 | $9.42 |
| LT-58127 | RET/ER | $9.42 | $101.14 | $9.42 |
| LT-52989 | RET/ER | $9.42 | $101.14 | $9.42 |
| LT-54057 | RET/ER | $9.03 | $101.97 | $9.03 |
| LT-98384 | RET/ER | $9.03 | $101.97 | $9.03 |
| LT-50521 | RET/ER | $9.03 | $101.97 | $9.03 |
| LT-97316 | RET/ER | $8.64 | $102.79 | $8.64 |
| LT-51055 | RET/ER | $8.64 | $102.79 | $8.64 |
| LT-94646 | RET/ER | $8.64 | $102.79 | $8.64 |
| LT-54259 | RET/ER | $8.24 | $103.61 | $8.24 |
| LT-67077 | RET/ER | $8.24 | $103.61 | $8.24 |
| LT-62270 | RET/ER | $8.24 | $103.61 | $8.24 |
| LT-83431 | RET/ER | $7.85 | $104.43 | $7.85 |
| LT-94978 | RET/ER | $7.85 | $104.43 | $7.85 |
| LT-81294 | RET/ER | $7.85 | $104.43 | $7.85 |
| LT-59195 | RET/ER | $7.46 | $105.25 | $7.46 |
| LT-74554 | RET/ER | $7.46 | $105.25 | $7.46 |
| LT-65604 | RET/ER | $7.46 | $105.25 | $7.46 |
| LT-95180 | RET/ER | $7.06 | $106.07 | $7.06 |
| LT-72547 | RET/ER | $7.06 | $106.07 | $7.06 |
| LT-75954 | RET/ER | $7.06 | $106.07 | $7.06 |
| LT-64204 | RET/ER | $6.67 | $106.89 | $6.67 |
| LT-55125 | RET/ER | $6.67 | $106.89 | $6.67 |
| LT-81626 | RET/ER | $6.67 | $106.89 | $6.67 |
| LT-66875 | RET/ER | $6.28 | $107.71 | $6.28 |
| LT-63872 | RET/ER | $6.28 | $107.71 | $6.28 |
| LT-77888 | RET/ER | $6.28 | $107.71 | $6.28 |
| LT-58864 | RET/ER | $5.89 | $108.54 | $5.89 |
| LT-54591 | RET/ER | $5.89 | $108.54 | $5.89 |
| LT-55457 | RET/ER | $5.89 | $108.54 | $5.89 |
| LT-82897 | RET/ER | $5.49 | $109.36 | $5.49 |
| LT-95714 | RET/ER | $5.49 | $109.36 | $5.49 |
| LT-56395 | RET/ER | $5.49 | $109.36 | $5.49 |
| LT-90373 | RET/ER | $5.10 | $110.18 | $5.10 |
| LT-59066 | RET/ER | $5.10 | $110.18 | $5.10 |
| LT-77022 | RET/ER | $5.10 | $110.18 | $5.10 |
| LT-75088 | RET/ER | $4.71 | $111.00 | $4.71 |
| LT-69011 | RET/ER | $4.71 | $111.00 | $4.71 |
| LT-78422 | RET/ER | $4.71 | $111.00 | $4.71 |
| LT-70945 | RET/ER | $4.32 | $111.82 | $4.32 |
| LT-52455 | RET/ER | $4.32 | $111.82 | $4.32 |
| LT-87501 | RET/ER | $4.32 | $111.82 | $4.32 |
| LT-59932 | RET/ER | $3.92 | $112.64 | $3.92 |
| LT-52123 | RET/ER | $3.92 | $112.64 | $3.92 |
| LT-60264 | RET/ER | $3.92 | $112.64 | $3.92 |
| LT-79692 | RET/ER | $3.53 | $113.46 | $3.53 |
| LT-89839 | RET/ER | $3.53 | $113.46 | $3.53 |
| LT-91976 | RET/ER | $3.53 | $113.46 | $3.53 |
| LT-70411 | RET/ER | $3.14 | $114.29 | $3.14 |
| LT-89969 | RET/ER | $3.14 | $114.29 | $3.14 |
| LT-70281 | RET/ER | $3.14 | $114.29 | $3.14 |
| LT-61534 | RET/ER | $2.75 | $115.11 | $2.75 |
| LT-66138 | RET/ER | $2.75 | $115.11 | $2.75 |
| LT-85365 | RET/ER | $2.75 | $115.11 | $2.75 |
| LT-74886 | RET/ER | $2.35 | $115.93 | $2.35 |
| LT-74352 | RET/ER | $2.35 | $115.93 | $2.35 |
| LT-72749 | RET/ER | $2.35 | $115.93 | $2.35 |

# Attachments

1. 2. 3. 4.

# References

[493]

[496]

[501]

[510]

[475]

[A] http://www.energystar.gov/productfinder/download/certified-light-fixtures/

[B] Attachment 1 – Calculation Template 2016 v6.7.3.xlsm

[C] Attachment 2 – LG115.0 Costs.xlsx

[D] Attachment 3 - Lighting Disposition July.docx