# Workpaper WPSCGNRWH120618A

Revision 3

Southern California Gas Company

Customer Programs Department

Faucet Aerators for Bathroom/Kitchen Sinks in Residential Buildings

Revision History

| Revision No. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 | June 18, 2012 | Original Workpaper | Chan Paek, SCG |
| 1 | December 23, 2013 | Disposition Changes & Weather Updates | Julianna Colwell, SCG |
| 2 | July 21, 2014 | Climate Zone updates & added measures | Julianna Colwell, SCG |
| 3 | December 31, 2015 | Update due to the change in GPM Baseline. The savings calculations remain since the measure is retrofit add on, however the RUL was updated. Bathroom Aerator measure of “0.5GPM” was added. | Carlos Pineda, SCG |

Table A: Measure Summary

| Measure ID | Measure Description | Pre-Existing Description | Code/Standard Description | Sector | App Type(s) | Delivery Method(s) | EUL ID/ RUL ID | NTG ID(s) | GSIA ID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| ShwFca001 | Faucet Aerator, Bathroom Sink, 0.5 GPM | 2.2 GPM flow rate with no flow restriction present | Faucet with no flow restriction device present | RES | REA | Direct Install | WtrHt-WH-Aertr/6.67 | Res-mDHWaerator | Res-LowF-FA-All |
| ShwFca002 | Faucet Aerator, Bathroom Sink, 1.0 GPM | 2.2 GPM flow rate with no flow restriction present | Faucet with no flow restriction device present | RES | REA | Direct Install | WtrHt-WH-Aertr/6.67 | Res-mDHWaerator | Res-LowF-FA-All |
| ShwFca003 | Faucet Aerator, Bathroom Sink, 1.5 GPM | 2.2 GPM flow rate with no flow restriction present | Faucet with no flow restriction device present | RES | REA | Direct Install | WtrHt-WH-Aertr/6.67 | Res-mDHWaerator | Res-LowF-FA-All |
| ShwFca004 | Faucet Aerator, Kitchen Sink, 1.5 GPM | 2.2 GPM flow rate with no flow restriction present | Faucet with no flow restriction device present | RES | REA | Direct Install | WtrHt-WH-Aertr/6.67 | Res-mDHWaerator | Res-LowF-FA-All |

Table B: Measure Summary

| Measure ID | Descriptors |  |  |  |  | Above Preexisting/ Customer-Average Savings |  |  | Above Code/ Standard Savings |  |  | Cost |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bldg Type | Bldg Vint | Bldg Loc | Bldg HVAC | Norm Unit | kWh/ unit | kW/unit | therm | kWh/ unit | kW/unit | therm | Code/ Standard ($/unit) | Measure ($/unit) | Incremental Measure ($/unit) |
| ShwFca001 | SF | Any | 9 | N/A | Each | N/A | N/A | 2.0 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca002 | SF | Any | 9 | N/A | Each | N/A | N/A | 1.5 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca003 | SF | Any | 9 | N/A | Each | N/A | N/A | 1.0 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca004 | SF | Any | 9 | N/A | Each | N/A | N/A | 5.9 | N/A | N/A | N/A | $0.00 | $7.99 | $7.99 |
| ShwFca001 | MF | Any | 9 | N/A | Each | N/A | N/A | 1.2 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca002 | MF | Any | 9 | N/A | Each | N/A | N/A | 0.9 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca003 | MF | Any | 9 | N/A | Each | N/A | N/A | 0.6 | N/A | N/A | N/A | $0.00 | $5.43 | $5.43 |
| ShwFca004 | MF | Any | 9 | N/A | Each | N/A | N/A | 4.9 | N/A | N/A | N/A | $0.00 | $7.99 | $7.99 |

Note: For the complete list of Measures, refer to the accompanying calculation spreadsheet

# Table of Contents

Revision History	i

Table A: Measure Summary	ii

Table B: Measure Summary	iii

List of Tables	v

SECTION 1 - General Measure & Baseline Data	1

1.01	Measure & Delivery Description	1

1.02	DEER Differences Analysis	1

1.03	Code Analysis	1

1.04	Measure APplication Type	3

1.05	Measure Effective Useful Life	3

1.06	Net-to-Gross Ratios for Different Program Strategies	3

1.07	Gross Realization Rate	3

1.08	Installation rate	3

1.09	Time-of-Use Adjustment Factor	4

SECTION 2 - Energy Savings & Demand Reduction Calculations	4

2.01	Load Shapes	4

2.02	Energy Savings	4

SECTION 3 - Base Case & Measure Costs	5

3.01	Base Case Cost	5

3.02	Gross Measure Cost	5

3.03	Incremental Measure Cost	5

Attachments	6

Measure Worksheet	6

Energy Efficiency Starter Kit Workpaper, SCGWP100309A Rev. 3.	6

References	7

List of Tables

Table 1: Standards for Plumbing Fittings and Fixtures and Trigger Dates	2

Table 2: Aerator Saving	5

General Measure & Baseline Data

Measure & Delivery Description

This workpaper addresses the savings associated with the installation of faucet aerators on bathroom or kitchen sinks for single, multi-family and mobile homes.

Mobile home savings are the same as Single home savings. Mobile homes will not have separate defined measures.

Measures in this workpaper include (0.5, 1.0 and 1.5) GPM aerators for bathroom sinks and 1.5 GPM aerators for kitchen sinks, each measure has a Single and Multifamily option.

Measure Application Type & Delivery Method

Measures in this workpaper apply direct install (DI) and retrofit add-on (REA) for existing faucets with no aerators and a flow rate of 2.2 GPM.

This measure is applicable to DMO, MF and SF residential buildings with water heaters performing on natural gas.

DEER Differences Analysis

The savings for this technology are not included in DEER2013, but RUL, climate zone factors, NTG Ratio, and the cost information are provided in DEER and the Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) February 22, 2013, which is provided below in the attachment section.

Code Analysis

Tittle 20 has been updated to reflect lower flow rates, the first change will take in effect on September 1st, 2015 ensuing with a second update to take effect on July 1st, 2016. The changes are shown in the table below.

Table 1: Standards for Plumbing Fittings and Fixtures and Trigger Dates

| Regulated water appliances in California under Tittle 20 | Previous CA Tittle 20 Regulation | Effective date for this standard is September 1, 2015 | Effective date for this standard is January 1, 2016 | Effective date for this standard is July 1, 2016 | Effective date for this standard is July 1, 2018 | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| Showerhead | 2.5 GPM at 80 psi |  |  | 2.0 GPM at 80 psi | 1.8 GPM at 80 psi | New tiered regulation. |
| Lavatory Faucet and aerator | 2.2 GPM at 60 psi | 1.5 GPM at 60 psi |  | 1.2 GPM at 60 psi |  | New tiered regulation. |
| Public Lavatory Faucet | 2.2 GPM at 60 psi |  | .5 GPM at 60 psi |  |  | No sell through is permitted after January 1, 2016. |
| Kitchen (sink) faucet | 2.2 GPM at 60 psi |  | 1.8GPM, optional temporary flow of 2.2 GPM, at 60psi |  |  | No sell through is permitted after January 1, 2016. |
| No Wall Mount Urinal | 0.5 GPF max |  | 0.5 GPF max |  |  | CEC standard catch up to CA Health and Safety Code standard. |
| Wall Mounter Urinal | 0.5 GPF max |  | 0.125 GPF max |  |  | No sell through is permitted after January 1, 2016. |
| Water Closet (Toilet) | 1.28 GPF max |  | 1.28 GPF max |  |  | No sell through permitted. Water closet sold or offered for sale on or after January 1, 2016, shall pass the Waste Extraction Test (Section 7.10) of ASME A112.19.2 |

Measure APplication Type

Retrofit Add On

Aerators are installed to existing faucets without an aerator and a flow rate of 2.2 GPM.

The measure is not applicable to new faucets that meet the 2016 Tittle 20 code requirement of 1.5 GPM flow rate.

Measure Effective Useful Life

The EUL (WtrHt-WH-Aertr) of the aerator provided in DEER table is 10 years; this EUL was used for all measures in the superseded workpaper revision. With the updated Tittle 20 requirement it is necessary to define both a first and second baseline for this measure.  This first baseline will have an RUL that is one-third of the faucet EUL (20 years), equivalent to 6.67 years (13*20=6.67). The second baseline RUL is zero.

This measure is an REA (Retrofit Add-on); Aerators are installed to existing faucets with a high flow rate (2.2GPM). As required by the Disposition for Water Fixtures from the California Public Utilities Commission this measure will apply the remaining useful life to the first baseline, for the second baseline there will be no savings. The assumption is that upon replacing a faucet (second baseline case) the aerator would also be removed.

The EUL (WtrHt-WH-Faucet) of faucets was found to be 20 years (NAHB Study of Life Expectancy of Home Components and Glacier Bay 20 year warranty), the remaining useful life for an aerator is 6.67 years  (13 * EUL-faucet) which will be applied to the 1st baseline. During this period the aerators will yield water and gas savings.

Net-to-Gross Ratios for Different Program Strategies

The NTGR (Res-mDHWaerator) values for this technology are provided in the Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) October 20, 2015 as follows.

NTGR for this measure in any building type other than multi-family with direct install delivery method is 0.59.

NTGR for this measure in MF (multi-family) building type with direct install delivery method is 0.65.

Gross Savings and INstallation Adjustment (GSIA)****

The gross realization rate of 0.665 is applied for this workpaper.

Table 2: GSIA

| MeasureID | GSIA ID | GSIA Type | GSIA Value | Description |
| --- | --- | --- | --- | --- |
| ShwFca001 | Res-LowF-FA-All | Annual Installation Rate | 0.665 | Residential low-flow Faucet Aerator |
| ShwFca002 | Res-LowF-FA-All | Annual Installation Rate | 0.665 | Residential low-flow Faucet Aerator |
| ShwFca003 | Res-LowF-FA-All | Annual Installation Rate | 0.665 | Residential low-flow Faucet Aerator |
| ShwFca004 | Res-LowF-FA-All | Annual Installation Rate | 0.665 | Residential low-flow Faucet Aerator |

Time-of-Use Adjustment Factor

N/A

Energy Savings & Demand Reduction Calculations

Load Shapes

N/A

Energy Savings

The savings calculations for this paper are defined in the Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) February 22, 2013. The stated Climate zone factors in Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) February 22, 2013 are out of date and should be replaced by the Climate Zone calculated factors.

Table 3 shows the savings values listed in the Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) February 22, 2013.

Values in Table 1 below represent the before of the climate zone adjustment factors being applied.

The climate zone factors are applied to Aerator Savings in Table 1, and the estimated savings for each climate zone are presented “Measure Worksheet.xlsx” in Attachment section.

The baseline flow rates can be found in DEER and Title 20.

The maximum flow rate of faucets should not be greater than 2.2 GPM at 60 psi.

Table 3: Aerator Saving

| Aerator | Estimated Savings (therm/yr/aerator) |
| --- | --- |
| 0.5 GPM, Bathroom (SF) | 2.0 |
| 1.0 GPM, Bathroom (SF) | 1.5 |
| 1.5 GPM, Bathroom (SF) | 1.0 |
| 1.5 GPM, Kitchen (SF) | 5.9 |
| 0.5 GPM, Bathroom (MF) | 1.2 |
| 1.0 GPM, Bathroom (MF) | 0.9 |
| 1.5 GPM, Bathroom (MF) | 0.6 |
| 1.5 GPM, Kitchen (MF) | 4.9 |

Base Case & Measure Costs

Base Case Cost

The base case of this REA measure is to not do anything, and therefore, the base case cost is $0.

Gross Measure Cost

These measure costs were pulled from the 2013 program cycle negotiated 3rd party contractor installation costs.

The average cost to install a Kitchen Aerator was $7.99

The average cost to install a Bathroom Aerator was $5.43

The gross measure cost is $7.99 for Kitchen Aerators and $5.43 for Bathroom Aerators.

Incremental Measure Cost

The IMC is equal to the gross measure cost; $7.99 for Kitchen Aerators and $5.43 for Bathroom Aerators.

Attachments

Measure Worksheet & Savings Calculations

Workpaper Disposition for Water Fixtures from the California Public Utilities Commission (CPUC), Energy Division (ED) February 22, 2013

Energy Efficiency Starter Kit Workpaper, SCGWP100309A Rev. 3.

Climate Zone Factors

2013/2014 Disposition Savings Calculations

References

Expected Useful Life, “DEER2014-EUL-table-update_2014-02-05.xlsx”, DEER 2014.

NAHB- Study of life Expectancy of Home Components

Glacier bay 20 Year Warranty

Climate Zone Factors