Workpaper WPSCGREWH120919A

Revision 3

Southern California Gas Company

Customer Programs Department

Tankless Water Heaters for Single Family and Multifamily Applications

Revision History

| Revision No. | Date | Description | Author |
| --- | --- | --- | --- |
| 0 | Oct. 09, 2012 | Original release | Stu Knoke (ICF) |
| 1 | May 20, 2014 | Revision due to Title 24 | Julianna Colwell (SCG) |
| 2 | Dec 08, 2014 | - Update with regards to DEER 2015 and Federal Code Standard - Update Workpaper Template - Updated Cost Information - Incorporated SF Tankless WP WPSCGREWH140122a - Added Tier 1 measure - Added Measure ID - Revised Terms and Conditions to exclude tankless water heater attached to storage tank. | Miguel Urrea (SCG) |
| 3 | Aug 08, 2016 | - Updates per Water Heater Disposition (Attachment A) - Adjust cost to include installation (Labor and Material) - Changed energy impacts to report directly from DEER - Added air quality emission requirements | Miguel Urrea (SCG) |

Measure Summary Table A

| Measure ID | Measure Description | Pre-Existing Description | Code/Standard Description | Sector | App Type(s) | Delivery Method(s) | EUL ID | NTG ID(s) | GSIA ID |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g | Tankless Water Heater ≤200 MBtu/hr (Small / Medium), Tier 1 (≥0.82 EF) | N/A | Storage Water Heater 40 Gal ≤75 MBTUh EF 0.615 | Res | ROB | PreRebDown, PreReb, PreRebup | WtrHt-Instant-Res | Res-Default >2yrs | Def-GSIA |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g | Tankless Water Heater ≤200 MBtu/hr (Small / Medium), Tier 2 (≥0.92 EF) | N/A | Storage Water Heater 40 Gal ≤75 MBTUh EF 0.615 | Res | ROB | PreRebDown, PreReb, PreRebup | WtrHt-Instant-Res | Res-Default >2yrs | Def-GSIA |

Measure Summary Table B

| Measure ID | Descriptors |  |  |  |  | Above Preexisting/ Customer-Average Savings |  |  | Above Code/ Standard Savings |  |  | Cost |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Bldg Type | Bldg Vint | Bldg Loc | Bldg HVAC | Norm Unit | kWh/ unit | kW/ unit | therm | kWh/ unit | kW/unit | therm | Code/ Standard ($/unit) | Measure ($/unit) | Incremental Measure ($/unit) |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g | SFm | Ex | SCG | Any | Each | 0 | 0 | 0 | 0 | 0 | 33.6 | $1,170.00 | $2,381.00 | $1,211.00 |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g | MFm | Ex | SCG | Any | Each | 0 | 0 | 0 | 0 | 0 | 25.4 | $1,170.00 | $2,381.00 | $1,211.00 |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g | SFm | Ex | SCG | Any | Each | 0 | 0 | 0 | -3.52 | -0.00036 | 46.5 | $1,170.00 | $2,896.00 | $1,726.00 |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g | MFm | Ex | SCG | Any | Each | 0 | 0 | 0 | -2.91 | -0.0003 | 36 | $1,170.00 | $2,896.00 | $1,726.00 |

Note: For the complete list of Measures, refer to the READI7

# Table of Contents

Revision History	i

Measure Summary Table A	ii

Measure Summary Table B	ii

List of Figures	iv

List of Tables	iv

SECTION 1 - General Measure & Baseline Data	1

1.01	Measure & Delivery Description	1

1.02	DEER Differences Analysis	3

1.03	Code Analysis	5

1.04	Measure Effective Useful Life	7

1.05	Net-to-Gross Ratios for Different Program Strategies	7

1.06	Gross Realization Rate	7

1.07	Time-of-Use Adjustment Factor	7

1.08	Gross Savings and Installation Adjustment (GSIA)	7

1.09	EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information	8

SECTION 2 - Energy Savings & Demand Reduction Calculations	8

2.01	Load Shapes	8

2.02	Energy Savings	8

SECTION 3 - Base Case & Measure Costs	11

3.02	Base Case Cost	12

3.03	Gross Measure Cost	12

3.04	Incremental Measure Cost	12

Attachments	14

List of Figures

Figure 1 -	California Counties Subject to Ultra Low NOx Requirements	6

Figure 2 -	Energy Factor Distribution for Small Tankless Water Heaters in the CEC Appliance Efficiency Database	9

List of Tables

Table 1 -	Measure Descriptors	1

Table 2 -	Delivery Methods	2

Table 3 -	Measure Application Type	2

Table 4 -	Sector and Subsector	3

Table 5 -	Code Summary	4

Table 6 -	Code Summary - NOx Requirements for AQMDs	4

Table 7 -	California Title 20 Gas Appliance Standards And Code Of Federal Regulations	6

Table 8 -	GSIA Table	6

Table 9 -	Baseline and Qualifying Measure Efficiencies for Tankless Water Heaters Replacing Storage Water Heaters	7

Table 10 -	Calculations of Annual Energy Savings for Tankless Water Heaters Replacing Storage Water Heaters	7

Table 11 -	Gross and Incremental Measure Cost for Tankless Water Heaters Replacing Storage Water Heaters	9

General Measure & Baseline Data

Measure & Delivery Description

Measure Description

Tankless water heaters have become available in recent years for a variety of applications.  Relative to a storage water heater, a tankless unit has a relatively large burner that rapidly heats water to the desired temperature.  Due to the rapid “instantaneous” heating, a tankless water heater does not require a storage tank.

Due to the relatively larger burner size, these water heating devices are capable of providing hot water on a continuous basis.  They have relatively high energy efficiency levels because standby losses from storage tanks are essentially eliminated.

The California Titles 20 and 24 standards define an instantaneous water heater to mean “a water heater that has an input rating of at least 4,000 Btu per hour per gallon of stored water”,.  All “tankless” water heaters are “instantaneous” water heaters and tankless water heaters generally have rated inputs less than 200 MBtu/hr.

Tankless water heaters are most useful in point-of-use applications, i.e., at the faucet and with no circulation loop.  They are very inefficient in applications with a circulation loop due to the temperature loss in the circulation system which causes the tankless water heater to run without water demand.  They are problematic in central systems with circulation loops which have long pipe runs from the water heater to the faucet.

The two measures are as followed:

RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g - Tankless Water Heater, ≤200 MBtu/hr (Small / Medium), Tier 1 (≥0.82 EF)

RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g - Tankless Water Heater, ≤200 MBtu/hr (Small / Medium), Tier 2 (≥0.92 EF)

The 2006 Instantaneous Water Heater Workpaper includes a more detailed technology description in its Appendix A.

Code/Standard Description

Storage water heater means a water heater that heats and stores water within the appliance at a thermostatically-controlled temperature for delivery on demand, and that has an input less than 4,000 Btu per hour per gallon of stored water. The baseline for this measure is as followed:

Small (≤75 MBtu/hr) 40 gallon storage water heater with an energy factor of 0.615 and an emission limit of 10ng/J or 15 ppm Nox @3% O2 dry.

    - Preexisting Description – NA

    - Measure Descriptors

- Measure Descriptors

| MeasureID | Use-Category | UseSubCategory | Tech Group | Tech Type | PreTech Group | PreTech Type | StdTech Group | StdTech Type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g | SHW | Heating | WaterHtg_eq | Instant_EF | NA | NA | WaterHtg_eq | Stor_EF |
| RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g | SHW | Heating | WaterHtg_eq | Instant_EF | NA | NA | WaterHtg_eq | Stor _EF |

Delivery Method

The preferred delivery method is a downstream prescriptive rebate offered to the gas customer purchasing the new tankless water heater	.

However, a midstream point-of-sale prescriptive rebate or an upstream manufacturer prescriptive rebate strategy may also be implemented.

Table 2 - display the DEER approved delivery methods.  Each IOU may select its own preferred delivery method in their ex ante database submission.

- Delivery Methods

| Code | Description |
| --- | --- |
| PreRebDown | Downstream Prescriptive Rebate |
| PreReb | Prescriptive Rebate |
| PreRebUp | Upstream Prescriptive Rebate |
| NonUpStrm | All non-upstream delivery types for deemed measures |

Measure Application Type

- Measure Application Type

| Code | Description | Comment |
| --- | --- | --- |
| ROB | Replace on Burnout | measure applied when existing equipment fails or maintenance requires replacement |

Eligibility Requirements

Test methods for measuring water heater efficiencies are referenced in the California Titles 20 and 24 standards1,2.

Minimum qualifying energy factor (EF) for small (<200 MBtu/hr) tankless water heaters replacing storage water heaters:

0.82 for Tier 1 (non-condensing)

0.92 for Tier 2 (condensing)

Minimum emission requirements per air district.

Tier 2 hot water heaters are condensing and often require flue modifications to handle the condensate.

Implementation Requirements

The rebate applies to one natural gas tankless water heater replacing one natural gas storage water heater on burnout or to new installations in existing buildings.

The rebate does not apply to new construction (NC).

This measure is applicable only to single family and multifamily residential domestic (or “service”) hot water applications with a water heater in (single family only), or attached to, each dwelling unit or in adjacent units(multifamily only). Table 4 - displays approved sectors and subsectors.

This measure is limited to single family residences with a central water heater in each dwelling unit and multifamily residences with a water heater in each dwelling unit or with a water heater serving adjacent units.

This measure includes replacing a storage water heater with a tankless water heater.

This Workpaper does not cover water heaters or hot water boilers used for commercial domestic hot water, space conditioning, industrial (process) end-use, pool, or spa applications.

- Sector and Subsector

| Sector | Subsector | Comment |
| --- | --- | --- |
| Res | SFm | Residential Single Family |
| Res | MFm | Residential Multi-family |

Documentation Requirements

The manufacturer’s name and equipment model number must be provided.

If necessary, customer must provide proof of unit efficiency (e.g., manufacturer’s equipment specification sheet).

Terms & Conditions

Only tankless water heaters as defined by the California Energy Commission qualify, and they must:

Be used primarily for domestic hot water

Be installed at the point of use

Provide hot water only when there is a hot water draw from the end use

Not be connected to an external storage tank.

Have an input rating of at least 4,000 Btu per hour per gallon of stored water.

Never be used to supply hot water to a circulation loop

DEER Differences Analysis

DEER has adopted the following measures into READI per IOU request, and they are used as is:

- Tankless Water Heater	≤200 MBtu/hr (Small / Medium), Tier 1 (≥0.82 EF)

  - RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p82EF-40g

- Tankless Water Heater	≤200 MBtu/hr (Small / Medium), Tier 2 (≥0.92 EF)

  - RG-WtrHt-SmlInst-Gas-150kBtuh-lt2G-0p92EF-40g

DEER Incremental Measure Cost

Since the efficiencies used here do not contain cost information in DEER, the incremental measure costs was developed.

The incremental measure costs used in this Workpaper are derived from Technical Support Documents (TSD) for Pool Heaters, Direct Heating Equipment and Water Heaters (EERE-2006-STD-0129).

Code Analysis

- Code Summary

| Code | Applicable Code Reference | Effective Dates |
| --- | --- | --- |
| Title 24 (2013) | Section 110.3 | 11/26/2013 |
| Title 20 (2012) | Section 1605.3(f) | 1/20/2004 |
| Code of Federal Regulations | 10 CFR 430.32(d) | 04/16/2015 |
| South Coast AQMD | Rule 1121, Rule 1146.2 | See Rule |
| Bay Area AQMD | Regulation 9, Rule 6 | See Rule |
| San Joaquin Valley APCD | Rule 4902 | See Rule |
| Sacramento Metropolitan AQMD | Rule 414 | See Rule |
| Yolo-Solano AQMD | Regulation II, Rule 2.37 | See Rule |
| Ventura County Air Pollution Control District | Rule 74.11, Rule 74.11.1 | See Rule |

- Code Summary - NOx Requirements for AQMDs

| Category | Rated heat input capacity | Rule | Emission Limit |
| --- | --- | --- | --- |
| Small Water Heater | <75,000 Btu/hr | YSAQMD-2.37, SJVUAPCD-4902 , SCAQMD-1121, BAAQMD-Reg 9-Rule 6, SMAQMD-414, SBCAPCD-352, VCAPCD-74.11 | 10ng/J or 15 ppm NOx @3% O2 dry |
| Instantaneous Water Heater*** | Unspecified | SJVUAPCD-4902, BAAQMD-Reg 9-Rule 6, VCAPCD-74.11.1, SMAQMD-414, SCAQMD-1146.2*, YSAQMD-2.37* | 14ng/J or 20 ppm NOx @3% O2 dry |

*SCAQMD does not set emission requirements for instantaneous water heaters, but any instantaneous water heaters within Type 1 water heaters (75,001 Btu/h – 400,000 Btu/h) rated input capacity must meet this requirement. YSAQMD and VCAPCD requirements are based on small and large water heaters ( < 75,000 and 75,001 – 400,000 btu/h).  Any instantaneous water heater within this rated input capacity must meet this requirement.

California Counties Subject to Ultra Low NOx Requirements

The minimum baseline efficiencies are consistent with the Code of Federal Regulations standards.

The minimum qualifying measure efficiency for Tier 2 exceeds the California Titles 20 and 24 and the Code of Federal Regulations standards.  Since it is considered standard practice to purchase storage water heaters for residential applications we are offering a Tier 1 measure of a tankless water heater listed at the Code of Federal Regulations standards of energy factor 0.82.

- California Title 20 Gas Appliance Standards and Code Of Federal Regulations

| Equipment Type | Rated Input (MBtu/hr) | Rated Volume (gal) | Efficiency Units | Minimum Efficiency | Maximum Standby Loss (Btu/hr) |
| --- | --- | --- | --- | --- | --- |
| Storage Water Heaters |  |  |  |  |  |
| Small federally-regulated | ≤ 75 | ≥ 20 and ≤ 50 | EF | 0.675-(0.0015*V) | --- |
| Small federally-regulated | ≤ 75 | > 50 and ≤ 100 | EF | 0.8012-(0.00078*V) | --- |
| Small non-federal regulated | ≤ 75 | < 20 | EF | 0.62-(0.0019*V) | --- |
| Small non-federal regulated | ≤ 75 | > 100 | EF | 0.62-(0.0019*V) | --- |
| Tankless Water Heaters |  |  |  |  |  |
| Small federally-regulated | ≤ 200 | < 2 | EF | 0.82-(0.0019*V) | --- |
| Small non-federal regulated | ≤ 50 | Unspecified | EF | 0.62-(0.0019*V) | --- |
| Small non-federal regulated | ≤ 200 | ≥ 2 | EF | 0.62-(0.0019*V) | --- |

*V is the rated volume in gallons; Q is the rated input is Btu/hr

Measure Effective Useful Life

For tankless water heaters, the EUL for WtrHt-Instant-Res of 20 years is taken from the DEER 2014 EUL Table.

Net-to-Gross Ratios for Different Program Strategies

The 2014 DEER documents recommend a net-to-gross ratio (NTGR) of 0.55 for all other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years listed as Res-Default>2.

Gross Realization Rate

Gross realization rate of 1.00 is applied to the measures in this document.

Time-of-Use Adjustment Factor

N/A

Gross Savings and Installation Adjustment (GSIA)

- GSIA Table

| GSIA ID | GSIA Type | GSIA Value | Description |
| --- | --- | --- | --- |
| Def-GSIA | Annual Installation Rate | 1 | Default GSIA Value |

EM&V, Market Potential, and Other Studies – Base Case and Measure Case Information

N/A

Energy Savings & Demand Reduction Calculations

Load Shapes

N/A

Energy Savings

Annual Gas Energy Savings.

The annual gas energy savings are based on DEER2015.

Baseline and Measure Efficiencies.

Table 9 -lists the efficiency units and efficiency values recommended for tankless water heaters.

- Baseline and Qualifying Measure Efficiencies for Tankless Water Heaters Replacing Storage Water Heaters

| Equipment Type | Rated Input (MBtu/hr) | Efficiency Units | Base Efficiency (Storage Water Heater) | Qualifying Efficiency (Tankless Water Heater) |
| --- | --- | --- | --- | --- |
| Tankless Water Heaters |  |  |  |  |
| Small, Tier 1 (non-condensing) | ≤ 200 | EF | 0.615 | 0.82 |
| Small, Tier 2 (condensing) | ≤ 200 | EF | 0.615 | 0.92 |

Energy Savings Calculation

The energy savings data was calculated in DEER 2015 Water Heater Calculator.  DEER has incorporated these measures into READI. The savings reported in READI are used for this Workpaper.

Table 10 -lists the base efficiencies, measure efficiencies, and saving values for the two measures reported in the DEER 2015 for a residential tankless water heaters replacing a 75 gallon storage water heater.

The example in Table 10  uses the SCG IOU value. The rest of the savings values can be found in READI.

- Calculations of Annual Energy Savings for Tankless Water Heaters Replacing Storage Water Heaters

| Equipment Type | Small, Tier 1 | Small, Tier 2 |
| --- | --- | --- |
| Rated Input (MBtu/hr) | ≤ 200 | ≤ 200 |
| Adjusted Values |  |  |
| Storage Base Volume (Gal) | 40 | 40 |
| Efficiency units | EF | EF |
| Base Efficiency, E3 | 0.615 | 0.615 |
| Measure Efficiency (Tankless water heater), E4 | 0.82 | 0.92 |
| Adjusted Annual Energy Savings (therms/yr/MBtuh), ∆Q3-4 (SFm) | 33.60 | 46.50 |
| Adjusted Annual Energy Savings (therms/yr/MBtuh), ∆Q3-4 (MFm) | 25.40 | 36.00 |

Base Case & Measure Costs

Cost data derived from Technical Support Documents (TSD) for Pool Heaters, Direct Heating Equipment and Water Heaters (EERE-2006-STD-0129)8.  This data includes both labor and material cost.  Condensing water heaters typically use pvc for venting flue exhaust which is cheaper and easier to install than sheet metal used for non-condensing water heaters12.  The cost difference in venting material and installation is captured in the above referenced document.  The cost data and calculations are included in an Excel file embedded as Attachment D.  Summary of cost data captured from the TSDs are included in Attachment E.

Both online vendor cost survey and the Ex Ante Measure Cost Study were taken into consideration while developing the cost data.  Vendor cost survey provided a reference point for product cost, but did not provide sufficient data for installation cost due to the various installation set-ups.  The Measure Cost Study did not appear to take ultra-low NOx production cost into consideration for gas storage and tankless water heaters.  Citing the CEC’s Appliance Database as the source for qualifying water heaters (CEC’s Appliance Database does not take air quality regulations into consideration). Air quality regulations were only mentioned in the study in reference to boiler projects.  Due to thorough investigation of water heater cost and installation set-ups the TSD were used for final cost data.  Both the Measure Cost Study and online vendor study are included in the cost data Attachment D for reference.

TSD “Pool Heaters, Direct Heating Equipment and Water Heaters (EERE-2006-STD-0129)”10 reports total installed cost (IP) for small stoage and tankless water heaters as the sum of consumer product price (CPP) and Cost to the consumer to install products (INST) (IP = CPP + INST).  The tankless water heaters are reported in those cost details.

Base Case Cost

When the customer is replacing equipment on burnout (ROB) or buying new equipment, the customer must buy a new water heater to continue operating, so the base case cost is that of a baseline (standard) water heater.  The baseline water heater is a storage water heater.

The base case costs are shown in Table 11 - below.

Gross Measure Cost

The gross measure costs include the cost of the equipment, including installation and start-up costs.

Venting material used with non-condensing water heaters are not suitable for condensing due to material properties.  Plastics used for condensing water heaters have lower vent temperatures limits and are not suitable for non-condensing water heaters.  The cost differences in these set-ups are taken into consideration in the cost analysis.  The gross measure costs are shown in Table 11 - below.

Incremental Measure Cost

The incremental measure cost (IMC) is the difference between the cost of the average baseline unit and the average measure.

The incremental measure costs are shown in Table 11 - below which is focused on tankless water heaters replacing storage water heaters for domestic hot water use.  The cost data and calculations are included in an Excel file embedded as Attachment D.

- Gross and Incremental Measure Cost for Tankless Water Heaters Replacing Storage Water Heaters

| Equipment Type | Small Tier 1 | Small Tier 2 |
| --- | --- | --- |
| Rated Input (MBtu/hr) | ≤ 200 | ≤ 200 |
| 2014 Vendor Survey Data per MBtuh |  |  |
| Base Cost – Product ($/MBtuh) | $468.00 | $468.00 |
| Base Cost – Installation ($/MBtuh) | $702.00 | $702.00 |
| Total Base Cost ($/MBtuh) | $1,170.00 | $1,170.00 |
| Gross Measure Cost – Product ($/MBtuh) | $1,109.00 | $1,936.00 |
| Gross Measure Cost – Installation ($/MBtuh) | $1,272.00 | $960.00 |
| Total Gross Measure Cost ($/MBtuh) | $2,381.00 | $2,896.00 |
| Incremental Measure Cost ($/unit) | $1,211.00 | $1,726.00 |

Attachments

Attachment A – Water Heater Disposition

Attachment B – B-REP-05-599-17A - Instantaneous (Tankless) Water Heaters

Attachment C – CEC Database

Attachment D – Tankless Water Heater Vendor Cost Data

Attachment E – Water Heater & Boiler Cost Summary

# References

(2014 Appliance Efficiency Regulations, 2014)  http://www.energy.ca.gov/2014publications/CEC-400-2014-009/CEC-400-2014-009-CMF.pdf

(2013 Building Energy Efficiency Standards for Residential and Nonresidential Buildings - Revised, 2013) http://www.energy.ca.gov/2012publications/CEC-400-2012-004/CEC-400-2012-004-CMF-REV2.pdf

(B-REP-05-599-17A - Instantaneous (Tankless) Water Heaters, 2005) Attachment B

(DOE, Docket ID: EERE-2006-STD-0129, 2010)

https://www.regulations.gov/#!docketDetail;D=EERE-2006-STD-0129

(EUL table update, 2014), http://deeresources.com/files/DEER2013codeUpdate/download/DEER2014-EUL-table-update_2014-02-05.xlsx

(DEER2011 Update Net-To-Gross table, 2012), http://deeresources.com/files/DEER2011/download/DEER2011_NTGR_2012-05-16.xls

(Remote Ex-Ante Database Interface, 2016) ftp://deeresources.com/DEER/READI_v2.4.7.zip

(Itron, Inc, 2014) http://www.calmac.org/publications/2010-2012_WO017_Ex_Ante_Measure_Cost_Study_-_Final_Report.pdf