Work Paper WPSDGENRAG0001

Sprinkler to Drip Irrigation

Revision # 1

San Diego Gas & Electric Company

Energy Efficiency Engineering

Sprinkler to Drip Irrigation

Measure Code O-C1

# At-A-Glance Summary

| Applicable Measure Code: | O-C1 Sprinkler to Drip Irrigation, Field Vegetables, Deciduous trees, Vineyard, & Citrus |
| --- | --- |
| Measure Description: | This measure replaces high pressure sprinkler irrigation with micro (or drip) irrigation system. |
| Energy Impact Common Units: | Per Acre |
| Base Case Description: | Source: DEER 2005 - D03-972, D03-973, D03-974, D03-975, D03-976, D03-977, D03-978, D03-979 High-pressure, impact- type sprinkler irrigation (50 psi or higher operating pressure). |
| Base Case Energy Consumption: | N/A |
| Measure Energy Consumption: | N/A |
| Energy Savings (Base Case – Measure) | Source: DEER 2005 D03-972, D03-973, D03-974, D03-975, D03-976, D03-977, D03-978, D03-979 |
| Costs Common Units: | Per Acre |
| Base Case Equipment Cost ($/unit): | N/A |
| Measure Equipment Cost ($/unit): | N/A |
| Measure Incremental Cost ($/unit): | Source: DEER 2005 Final Report $1,000 per Acre |
| Effective Useful Life (years): | Source: DEER 2008 20 years |
| Program Type: | Replace on Burnout (ROB) |
| Net-to-Gross Ratios: | NTGR=0.6 Source: DEER 2011 |

# At-A-Glance Measure List

# Work Paper Approvals

| Peter Ford Manager, Energy efficiency Engineering | Date |
| --- | --- |
| Charles Harmstead Supervisor, Energy Efficiency Engineering | Date |

# Document Revision History

Revision #	Date	Description	Author (Company)

| Revision 1 | 03/12/08 | Adopted from PGECO AGR111 R3.doc dated 8/28/12. Modified savings to select only coastal zone 7, representative of SDGE. Added citrus to savings calculations. Developed weighted savings for one measure using average of four crop types and weighted well, non-well savings | Charles Harmstead |
| --- | --- | --- | --- |

# Table of Contents

At-A-Glance Summary	ii

At-A-Glance Measure List	iii

Work Paper Approvals	v

Document Revision History	vi

Table of Contents	vii

List of Tables	viii

List of Figures	viii

Section 1. General Measure & Baseline Data	1

1.1	Measure Description & Background	1

1.2 DEER Differences Analysis	3

1.4 EM&V, Market Potential, and Other Studies	3

1.5 Base Cases for Savings Estimates: Existing & Above Code	3

1.6 Base Cases & Measure Effective Useful Lives	3

1.7 Net-to-Gross Ratios for Different Program Strategies	3

Section 2. Calculation Methods	4

2.1 Electric Energy Savings Estimation Methodologies	4

2.2. Demand Reduction Estimation Methodologies	5

2.3. Gas Energy Savings Estimation Methodologies	5

Section 3. Load Shapes	5

3.1 Base Case Load Shapes	6

3.2 Measure Load Shapes	6

Section 4. Base Case & Measure Costs	6

4.1 Base Cases Costs	6

4.2 Measure Costs	6

4.3 Incremental & Full Measure Costs	6

Index	7

References	8

# List of Tables

Table-1: Net-to-Gross Ratio	4

Table-2:  Annual kWh/Acre-year savings for converting from sprinkler to micro irrigation	5

Table-3:  kW/Acre savings for converting from sprinkler to micro-irrigation	5

# List of Figures

There are no figures in this work paper

# Section 1. General Measure & Baseline Data

## Measure Description & Background

This work paper documents the rationale for the “sprinkler to drip irrigation” measure as listed in the SDG&E Energy Efficiency Business  Rebate Catalog.

Catalog Description

SPRINKLER TO DRIP IRRIGATION

Requirements:

- Customer must have electricity distributed by SDG&E to the installation address.

- Must convert from a high-pressure, impact-type, sprinkler irrigation system (50 psi operating pressure or more at the sprinkler head) to a micro-irrigation system.

Exclusions:

- Not applicable to new plantings of vineyards or orchards unless a vineyard or orchard was the previous crop on the field.

- No drip tape systems.

Application Process:

- Must include an Assessor’s Parcel Map or other documentation to verify acreage

| Product Code | Description |
| --- | --- |
| O-C1 | Sprinkler to Drip Irrigation |

Program Restrictions and Guidelines

Terms and Conditions

- Customer must have electricity distributed by SDG&E to the installation address

- Must convert from a high-pressure, impact-type, sprinkler irrigation system (50 psi operating pressure or more at the sprinkler head) to a micro-irrigation system

- Not applicable to new plantings of vineyards or orchards unless a vineyard or orchard was the previous crop on the field

- No drip tape systems

The reason for the 3-rd restriction is to have a baseline for our energy calculation. If the previous crop is different from the one being planted there would not be a one to one comparison of energy needs. Other plantings might have different watering needs and it will be impossible to calculate the decrease in energy use if the previous crop is different from the one being planted.

The reason for drip tape system not being allowed for the rebate is due to shorter Effective Useful Life (EUL) of the drip tape system.

The reason that this measure is not applicable for conversion from flood irrigation is that in average the energy use will increase and a full engineering analysis is required on case by case bases to prove the energy reduction. (Based on a report from Peter Canessa)

Market Applicability

This measure is applicable to sprinkler irrigated farmland in the SDG&E service territory relying on electric water pumping to water crops.

Technical Description

This measure encourages non-residential customers to replace their high pressure, impact type, sprinkler irrigation system (50 psi operating pressure or more at the sprinkler head) to a micro-irrigation system.

- Micro-irrigation systems consist of systems of above and below ground pipelines and / or hoses, delivering water under pressure, to specialized devices which deliver it directly to plants.  The intent is to accurately supply small amounts of water on a frequent basis so as to maintain constant, comparatively high, root-zone soil moisture.  In addition, micro-irrigation provides opportunities for very precise control of fertilizer applications.  Other advantages may include reduced weed growth and diseases and increased flexibility in timing cultivation operations. Energy may be saved by converting from a sprinkler irrigation system to a micro-irrigation system by reducing operating pressure and higher potential irrigation efficiencies. Converting to a micro-irrigation system will tend to reduce the amount of required water and associated pumping.

Drip Tape: Drip tape (also commonly called “row-crop drip”) - very thin wall polyethylene tubing with a variety of emitter designs integrated with the tubing during manufacture.  Drip tape comes in a variety of diameters, flow rates, emitter spacing, and wall thicknesses.  Drip tape may be laid aboveground but is more commonly buried. Polyethylene hose may only be guaranteed for 7-10 years.  Some growers report using buried drip tape up to 10 years but the norm is more like 2 to 4 years, and then only if it is 8 mil thickness or thicker. (Based on a report from Peter Canessa)Error: Reference source not found

Definition of the five major crop types used: (Based on a report from Peter Canessa)Error: Reference source not found

- Field crops - any crop for which a single, annual crop is grown each year.

- Vegetable crops - short season fresh-vegetable crops where it is common to double-crop

- Deciduous orchards - nuts (almond, walnut, pistachio), avocado, figs, and stonefruit

- Citrus

- Grape (wine, table, or raisin)

## 1.2 DEER Differences Analysis

This workpaper matches with the DEER 2005 for climate zone 7 (Measure ID D03-972, 973, 974, 975, 976, 977, 978, and 979) sprinkler to micro irrigation measure.  DEER has savings data for well and non-well applications. Electric savings for this measure are taken as a weighted average of well and non-well energy savings from the DEER database.

This measure was not included in the DEER 2011 update; however, DEER 2005 still represents the best data currently available.  As a result, DEER 2005 values will continue to be used until better data is available.

1.3 Codes & Standards Requirements Analysis

California State Water Code requires landowners to obtain a permit or conditional waiver for runoff. One way to minimize runoff is a drip irrigation system. However, there is no code requirement for micro (drip) irrigation.

## 1.4 EM&V, Market Potential, and Other Studies

Only one reference was made to evaluation of the sprinkler to drip irrigation measure. It was a retention study completed in 2003 that confirmed the 20 year EUL.

## 1.5 Base Cases for Savings Estimates: Existing & Above Code

The base case assumes a high-pressure, impact-type, sprinkler irrigation system that has at least 50 psi operating pressure or more at the sprinkler head.

## 1.6 Base Cases & Measure Effective Useful Lives

Micro-irrigation systems are a combination of many sub-systems, including a pumping plant, filters, mainline and manifold piping, and the system of distribution tubing and emission devices.  It is assumed that the system life is that of the pumping system and main pipelines to be 20 yearsError: Reference source not found.

## 1.7 Net-to-Gross Ratios for Different Program Strategies

Table-1 below shows Net-to-Gross ratio for this measureError: Reference source not found.  The value of 0.6 comes from the DEER 2011 update and represents the default NTG for “All other EEMs with no evaluated NTGR; existing EEM in programs with same delivery mechanism for more than 2 years”, specifically for the Agricultural sector.

Table-1: Net-to-Gross Ratio

|  |  | DEER Spreadsheet |  |
| --- | --- | --- | --- |
| Program Approach | NTG | File name | Cell Number |
| ROB | 0.60 | DEER2011-NTG_IncludingCarryoversFromDEER2008_2011-12-07.xls | T150 |

# Section 2. Calculation Methods

## 2.1 Electric Energy Savings Estimation Methodologies

Electric savings in this measure are taken as the average of well and non-well energy savings from the DEER database as shown in the equation below.  The pumping energy use per acre foot of water is related to the delivery of water to the field irrigation system and the boost energy required to operate the system itself. The delivery pumping energy use is only associated with well systems. Therefore, there is a difference in energy use between well and non-well water sources. The number of well (groundwater) systems have grown in the state due to other water demands, need for reliable and flexible water supplies, and higher costs and environmental concerns with surface water conveyance and storage. The exact distribution of well to non-well systems is not known. According to a report for the California Energy Commission, the total irrigation groundwater energy requirement in an average year is 4,745 GWh and 821.8 GWh for surface water. Therefore, the distribution of the amount of agricultural pumping energy between surface (non-well) and groundwater (well) is 15% and 85%, respectively.

Electrical SavingskWhAcre=Electrical SavingsWellkWhAcre×0.85+Electrical Savingsnon−WellkWhAcre×0.15

Example of saving for climate zone 1-5, measure A266:

Electrical SavingskWhAcre=434kWhAcre×0.85+367kWhAcre×0.15=424KWhAcre

These savings are presented in kWh/Acre of land. The savings are the difference in annual energy consumed between the high pressure, impact type sprinkler irrigation system to more efficient micro irrigation system.

DEER has provided savings numbers for four crop types: citrus trees, deciduous trees, field and vegetable crops, and vineyards.  DEER does not provide saving number for climate zone 10; Table-2 provides the energy savings information from DEER for non-well and well applications.Error: Reference source not found

Table-2:  Annual kWh/Acre-year savings for converting from sprinkler to micro irrigation

## 2.2. Demand Reduction Estimation Methodologies

Demand reduction estimations for this measure are taken directly from the DEER database where they are presented in Watts/Acre of land. These values are independent of well and non-well systems since the energy savings is due to the reduction in applied water.  The savings are the difference in power demand between the high pressure, impact type, and sprinkler irrigation system to more efficient micro irrigation system.

DEER has provided KW/Acre savings for all San Diego climate zones except Z10 and four crop types, citrus trees, deciduous trees, field and vegetable crops and orchards and vineyards.  Table-3 provides demand savings from DEERError: Reference source not found. For saving numbers of climate zone 10, we used a more conservative number from climate zone 7.

Table-3:  kW/Acre savings for converting from sprinkler to micro-irrigation

Average savings across crop types is 211 kW/Acre

## 2.3. Gas Energy Savings Estimation Methodologies

There are no gas savings associated with this measure.

# Section 3. Load Shapes

Load Shapes are an important part of the life-cycle cost analysis of any energy efficiency program portfolio.  The net benefits associated with a measure are based on the amount of energy saved and the avoided cost per unit of energy saved.  For electricity, the avoided cost varies hourly over an entire year.  Thus, the net benefits calculation for a measure requires both the total annual energy savings (kWh) of the measure and the distribution of that savings over the year.  The distribution of savings over the year is represented by the measure’s load shape.  The measure’s load shape indicates what fraction of annual energy savings occurs in each time period of the year.  An hourly load shape indicates what fraction of annual savings occurs for each hour of the year.  A Time-of-Use (TOU) load shape indicates what fraction occurs within five or six broad time-of-use periods, typically defined by a specific utility rate tariff.  Formally, a load shape is a set of fractions summing to unity, one fraction for each hour or for each TOU period.  Multiplying the measure load shape with the hourly avoided cost stream determines the average avoided cost per kWh for use in the life cycle cost analysis that determines a measure’s Total Resource Cost (TRC) benefit.

## 3.1 Base Case Load Shapes

The base case load shape would be expected to follow a typical agricultural pumping end use load shape.

## 3.2 Measure Load Shapes

For purposes of the net benefits estimates in the E3 calculator, what is required is the load shape that ideally represents the difference between the base equipment and the installed energy efficiency measure.  This difference load profile is what is called the Measure Load Shape and would be the preferred load shape for use in the net benefits calculations.

The measure load shape for this measure is determined by the E3 calculator based on the applicable agricultural market sector and the pumping end-use.

# Section 4. Base Case & Measure Costs

The DEER Measure Cost Data Users Guide, version 2.01, defines the following terms:

- Retrofit (RET) – replacing a working technology prior to failure.

- Replace on Burnout (ROB) – replacing a technology at the end of its useful life.

- New Construction (NEW) – installing a technology in a new construction or major renovation project.

Sprinkler to drip irrigation conversion is suitable for ROB installations. Hence only full (Installed) cost is described in the measure cost.

## 4.1 Base Cases Costs

This measure is a replace on burnout measure where only full cost is applicable.

## 4.2 Measure Costs

DEER provides an installed cost of this measure at $1,000 per acre.Error: Reference source not found

## 4.3 Incremental & Full Measure Costs

DEER provides an installed cost of this measure at $1,000 per acre.Error: Reference source not found

Installed (Full) Cost = Measure Equipment Cost + Labor Including OH&P.

# Index

Cost	6

DEER	3, 4, 5, 6

DEER Measure Cost Data Users Guide	6

High pressure sprinkler	2

Micro-irrigation	3

New Construction	6

Pipelines	2

Replace on Burnout	6

Retrofit	6

Rootzone soil moisture	2

Runoff	3

Water pumping	2

# References

-DEER 2008, http://www.deeresources.com/deer0911planning/downloads/EUL_Summary_10-1-08.xls

-  DEER2011-NTG_IncludingCarryoversFromDEER2008_2011-12-07.xls. DEER 2011. Excel., Tab: NTGR, Row 150

- Peter Canessa report on “SPRK2MICRO”, page 20, page 1, page 8

- “2004-2005 Database for Energy Efficiency Resources (DEER) Update Study”, prepared for Southern California Edison, prepared by Itron, Inc., December 2005 (Measure ID D03-972, 973, 974, 975, 978, and 979).

- Central Valley Regional Water Quality Control Board, Fact Sheet of Conditional Waiver of Waste Discharge Requirements for Discharges from Irrigated Land, July 2006. http://www.waterboards.ca.gov/centralvalley/water_issues/irrigated_lands/general_prog_info/irrlands_disch_fact_sht.pdf

- “6th Year Retention Study of Pacific Gas & Electric Company’s 1996 and 1997 Energy Efficiency Incentives Program, Agricultural Sector Measures,” March 2003.

- http://cetehama.ucdavis.edu/Agriculture/Groundwater_Management.htm

- “California Agricultural Water Electrical Energy Requirements,” prepared for Public Interest Energy Research Program, December 2003.